SECTION 01 74 13

PROGRESS CLEANING

FINAL CLEANING

# Note to Designer:  Modify this section to address the needs of this project

- SCOPE

  - This section specifies the requirements for maintaining a clean and orderly work site during and at the completion of the Work.

  - Pay special attention to work areas that affect occupied residents’ spaces and public areas.

- RELATED DOCUMENTS

  - This section supplements Article 4.17 of the General Conditions.

  - Consult the individual sections of the specifications for cleaning of Work installed under those sections.

  - Section 01.73.29 Cutting and Patching.

  - Section 01.74.19 Construction Waste Management and Disposal.

- CLEANING DURING CONSTRUCTION

  - Conduct all cleaning and disposal operations to comply with all federal, state, and local laws, regulations, codes, ordinances and by-laws.

    - Do not burn or bury rubbish and waste materials on the site.

    - Do not dispose of volatile wastes such as mineral spirits, oil, or paint thinner in storm or sanitary drains.

    - Do not dispose of wastes into streams or waterways.

  - Wet down dry materials and rubbish to lay dust and prevent blowing dust.

  - Do not allow materials and rubbish to drop free or be thrown from upper floors, but remove by use of a material hoist or rubbish chutes.

  - Maintain the Site free from accumulations of waste, debris, and rubbish.

  - Do not leave debris in occupied units.

  - Provide on-site containers for collection of waste materials and rubbish.

  - At the end of each day, remove and legally dispose waste materials and rubbish from site.

  - Vacuum clean interior building areas when ready to receive finish painting, and continue vacuum cleaning on an as-needed basis until Substantial Completion.

  - Schedule cleaning operations so that dust and other contaminants resulting from cleaning process will not fall on wet, newly painted surfaces.

  - Disposal of materials shall be in compliance with all applicable laws, regulations, ordinances, codes, and by-laws.

- FINAL CLEANING

Modify as appropriate for the scope of your project – for example there is no need to have a contractor clean mirrors on a boiler replacement project but cleaning the area around new boilers is important, especially leaving parts and materials that may cause harm to occupants.

  - Prior to submitting a request to the Architect to certify Substantial Completion of the Work, the Contractor shall inspect all interior and exterior spaces and verify that all waste materials, rubbish, tools, equipment, machinery, and surplus materials have been removed, and that all sight-exposed surfaces are clean.  Leave the Project clean and ready for occupancy.

  - Unless otherwise specified under other sections of the Specifications, the Contractor shall perform final cleaning operations as herein specified prior to final inspection.

  - Cleaning shall include all surfaces, interior and exterior, which the Contractor has had access to, whether new or existing.

  - Employ experienced workmen or professional cleaners for final cleaning.

  - Use only cleaning materials recommended by the manufacturer of the surface to be cleaned.

  - Use cleaning materials which will not create a hazard to health or property and which will not damage surfaces.

  - All broken or defective glass caused by the Contractor's Work shall be replaced at the expense of the Contractor.

  - Remove grease, mastic, adhesive, dust, dirt, stains, labels, fingerprints, and other foreign materials from sight-exposed interior and exterior surfaces.  This includes cleaning of the Work of all finishing trades where needed, whether or not cleaning by such trades is included in their respective specifications.

  - Clean and polish all new and existing glass and plastic glazing (if any) throughout the building(s), on both sides.  Clean plastic glazing in accordance with the manufacturer's directions.  This cleaning shall be completed by qualified window cleaners at the expense of the Contractor just prior to acceptance of the Work.

  - Wash and polish all mirrors.

  - Repair, patch, and touch up marred surfaces to the specified finish, to match adjacent surfaces.

  - Polish glossy surfaces to a clear shine.

  - Do the final cleaning of resilient floors and wood floors as specified under the respective sections of the Specifications.

  - Leave all architectural metals, hardware, and fixtures in undamaged, polished conditions.

  - Leave pipe and duct spaces, plenums, furred spaces and the like clean of debris and decayable materials.

  - In cleaning items with manufacturer's finish or items previously finished by a Subcontractor, care shall be taken not to damage such manufacturer's or Subcontractor's finish.  In cleaning glass and finish surfaces, care shall be taken not to use detergents or other cleaning agents which may stain adjoining finish surfaces.  Any damage to finishes caused by cleaning operations shall be repaired at the Contractor's expense.

  - Broom clean exposed concrete surfaces and paved surfaces.  Rake clean other surfaces of grounds.

  - Ventilating systems - Replace filters and clean ducts, blowers, and coils if units were operated during construction.

  - Owner's responsibility for cleaning commences at Substantial Completion.

## END OF SECTION 01.74.13

Progress Cleaning and Final Cleaning