|  | Departmentof Youth Services |
| --- | --- |
|  | Official Policy |

This policy shall apply to DYS state employees and contracted Providers. Providers are expected to have their own policy consistent with this policy.

Applicability:

Signature:

105 CMR 480.000: Minimum Requirements for the Management of Medical or Biological Waste (State Sanitary Code, Chapter VIII), Massachusetts Department of Public Health

105 CMR 450.000: Minimum Health and Sanitation Standards for DYS Secure Residential Facilities

29 CFR 1910.1030: Occupational Exposure to Bloodborne Pathogens Standards, OSHA, US Department of Labor

DYS Policy 02.05.01, Standard Precautions DYS Policy 03.02.16 (b), Hazardous Materials

References:

February 16, 2018

Effective Date:

02.05.02

Policy#:

Medical Waste Management in Residential Locations

Policy Name:

Policy

It is DYS policy that all DYS residential locations shall have  a  Medical  Waste Management Plan specific to the building where the program is  located  describing  the types and amount of medical waste generated at a specific location, and how wastes are managed to ensure proper storage and disposal. Such plan shall be drafted by the Facility Administrator for that building or, if stand alone, the Program Director.

Such Plan shall include that all medical and biological waste, including medical sharps, shall be separately contained and disposed of in accordance with this policy and procedures and with staff using standard precautions to ensure safe containment, disposal and to prevent further hazards to youth and employees.

It is DYS policy that medical waste generated at residential locations is not treated on site. All medical waste is transported off-site at a minimum once per calendar year by a registered medical waste hauler and treated at an approved off-site treatment facility.

I	Medical Waste Management	I of 5

Procedure

- Definitions

  - The following definitions shall have the meanings assigned to them in this policy for purposes of interpreting this policy.

Blood and blood products: Discarded bulk human  blood  and  blood  products in free draining, liquid state; body fluids contaminated with visible blood; and materials saturated/dripping with blood.

Medical or biological waste: Waste that is a body fluid or substance that, only if contaminated with blood, would be considered infectious. According to the Center for Disease Control and Prevention (CDC), such waste includes medical sharps and saliva, urine, feces, vomit, teeth, tears containing visible blood. Waste that is not considered medical waste and does not require containment as described in this policy includes feminine hygiene product and teeth, saliva, vomit, urine or fecal matter that is not contaminated with visible blood.

Medical Sharps: Discarded medical articles that may cause puncture or cuts including needles and, for the purposes of this policy, shaving razors.

Medical Waste Management Plan: A plan using the DYS Medical Waste Management Plan Form that describes the types and amount of medical waste generated at a specific location, and indicates how wastes are managed to ensure proper treatment and disposal.

Residential Placement: DYS state or provider operated program where a youth resides, where youth are not allowed to leave without approval, and there is 24/7 DYS state or provider supervision at the location including but not limited to hardware  secure programs,  staff secure programs,  and programs considered  Track I and 2.

Small-scale Generator (SSG):	A waste generator that generates less than 50 pounds of medical or biological waste every 30 days.

- Terms that are defined Policy #01.01.04; "Policy Definitions" shall have the meanings assigned to them in that policy, unless a contrary meaning is clearly intended.

- Terms not defined in Policy #01.01.04 or in this policy shall have the meanings assigned to them by reasonably accepted standard dictionary definitions of American English.

- Disposal of Medical or Biological Waste other than Sharps

I      Medical Waste Management	2 of 5

- Employees shall use authorized OSHA compliant blood spill kits to clean potentially infectious materials.

- Employees shall keep medical or biological waste separate from other waste at the point of generation using red biohazard bags labeled with the words "Biohazardous Waste" or the international biohazard symbol and the word "Biohazard" to insure containers will not rip or tear or allow moisture, and then tie bags to prevent spillage.

- Employees shall follow Standard Precautions policy and procedure for safe handling and transportation within the location from the point of generation to the point of storage.

- Disposal of Medical Sharps

  - Medical sharps shall be separated from other wastes and disposed of immediately after use in red, fluorescent orange or orange-red leak-proof, rigid, puncture resistant, shatterproof containers that resist breaking under normal conditions of use and handling, and that are marked prominently with the universal biohazard warning symbol and the word "Biohazard" in a contrasting color.

- Medical sharps containers shall be considered 'full' when they reach 2/3 capacity or the manufacturer's full line. Lids on filled sharps containers must be snapped closed, taped, or otherwise sealed to prevent loss of contents prior to disposal.

- Medical sharp containers may remain at the location's health office or other secured area until full and then brought to the medical waste storage area for disposal.

- Storage of medical or biological waste

  - Once in the above described bags or containers, all medical or biological waste shall be collected and stored in 4.5CU ft. biohazardous waste boxes until transported by the medical waste hauler.

- All areas for on-site storage of containers of medical or biological waste shall be in an uncarpeted room or area with solid, cleanable, non-absorbent flooring, used exclusively for waste storage such as a closet or smaller storage room.

- Storage area shall be locked at all times.

- All on-site storage areas shall:

  - Have prominent signage indicating the space is used for the storage of regulated medical or biological waste;

  - Be designed or equipped to prevent unauthorized access;

I	Medical Waste Management	3 of 5

  - Be designed or located to protect the waste from the elements and prevent access by vermin;

  - Provide sufficient space to allow for clear separation of regulated medical or biological waste from any other waste when applicable;

  - Be maintained so there is no decaying or offensive odors, using refrigeration when necessary; and

  - Be adequate to accommodate the volume of regulated medical or biological waste generated prior to removal of waste for either waste transport off-site or on-site treatment.

- Transportation

- The medical waste plan shall include the name, address, contact number and transporter identification number of the medical waste transporter for that location.

- Location health services staff or other designated staff for stand-alone DYS residential placements that may not generated larger quantities of medical or biological waste and are not on a DYS campus may transport medical waste generated at their location to a DYS residential location within their region that has a medical waste hauler.

- Emergency Action Plan: While it is highly unlikely that the company would experience a shut-down of operations, contingencies include the use of an alternate hauler or contacting the Department of Public Health at 617-624-6000 for guidance.

- Record-keeping

- Health Services staff for each residential placement shall maintain a current record-keeping log using the forms recommended by the Massachusetts Department of Public Health that has secured, consecutively numbered pages and on its cover the name of the facility and in large print the words "Medical Waste Record-keeping Log". Such log shall specify the following:

  - Exact date of each shipment;

  - Total number of containers;

  - Type of waste;

  - Total combined weight or volume;

  - Receipt of the hauler; and

  - Name of transporter with transporter identification number.

- Stand alone residential placements that bring their waste to another DYS location, shall record the above information and, in addition:

  - the printed name and signature of the responsible person transporting;

I	Medical Waste Management	I	4 of5

  - the name and address of the location that received the waste; and

  - the name and signature of the person that received the waste.

- The record-keeping log, copies of all shipping papers with corresponding medical waste tracking forms will be kept in the location's Health Services Office and retained for a period of 3 years.

- Training

  - All new direct care employees shall receive training on Standard Precautions, blood borne pathogens and the requirements on how to dispose, store and transfer waste considered medical or biological.

- Current employees shall receive annual training on Standard Precautions, blood borne pathogens and hazardous waste requirements.

- Health Services staff who are employees of health care institutions shall receive OSHA Bloodborne Pathogen Training annually at such institutions.

OUTCOME MEASURES AND PERFORMANCE INDICATORS

- All facilities have a site-specific Medical Waste Management Plan.

- Employees shall adhere to all procedures concerning medical waste segregation, containment, labeling, collection and storage.

- All facilities shall have a medical waste record-keeping log.

- All employees shall attend required Training at the initiation of employment and annually thereafter.

I     Medical Waste Management	5 of 5