# ITEM 102.513	AIR EXCAVATION AND ROOT PRUNING	FOOT

REV. 2022.01.01 (REV. DATE TO BE REMOVED BY MASSDOT CONTRACTS)

Item 102.513 Air Excavation and Root Pruning is for the services of excavating soil with an air pressure tool in order to expose tree roots, and for associated services and materials necessary to complete the work of pruning, backfilling with existing soil, watering, mulching, and fertilizing. This item shall include the furnishing and operating the air excavating tool.

Associated Item: All references to Arborist herein shall refer to the Arborist under Item 102.55 Arborist. Arborist shall meet the requirements as specified under that Item and shall be compensated under that Item.

Trees to be air spaded shall be those shown on the plans and/or as determined necessary by the Engineer per the recommendations of the Arborist.

## REFERENCES

The standards from American National Standards Institute (ANSI): A300 (Part 8)-2013 Root Management with special attention to Section 84 shall apply to this work. If requested, the Contractor shall provide to the Engineer one copy of this reference. Provision of reference shall be incidental to this item.

## METHODS

Air excavation and pruning work shall be performed by or overseen by the Arborist.

Air excavation of soil and root pruning shall occur any time prior to equipment work within the root zone of marked trees.

Air excavation shall be done along the limit of proposed excavation. Trench shall be of sufficient width to observe and cut roots and shall be to the depth of proposed excavation.  Immediately following air excavation, roots shall be pruned.

Following pruning, roots shall immediately be fully covered with backfill and immediately watered. Roots shall continue to be watered and fertilized as directed by the Arborist.

## METHOD OF MEASUREMENT AND BASIS OF PAYMENT

Item 102.513 will be measured and paid per foot where air spading, pruning, watering, and fertilizing are performed. This item will include full compensation for all labor, equipment, materials, and incidentals required for the satisfactory completion of the work.

Arborist services shall be per Item 102.55 Arborist and compensated under that Item.