Massachusetts Department of Public Health

Bureau of Environmental Health

Environmental Toxicology Program

http://www.mass.gov/dph/beaches

May 2016

Marine and Freshwater

Beach Testing in Massachusetts

Annual Report: 2015 Season

PART ONE: THE MDPH/BEH BEACHES PROJECT	4

I.	Overview	5

II.	Background	5

A.	Beach Water Quality & Health: The Need for Testing	5

B.	Establishment of the MDPH/BEH Beaches Project	6

III.	Beach Water Quality Monitoring	7

A.	Sample collection	7

B.	Sample analysis	7

1.	The MDPH contract laboratory program	7

2.	The use of indicators	7

3.	Enterococci	8

4.	E. coli	8

5.	Laboratory Methods	8

6.	Bacterial Standards	8

C.	Reporting	9

1.	The Beaches Website	10

2.	Exceedances: Beach Closures & Public Notification	10

3.	Data Management	11

D.	Quality Assurance	11

E.	The Tier System and Frequency of Testing	11

1.	The Three Tiers	12

2.	Sanitary Surveys and Variances	12

IV.	Limitations	13

PART TWO: THE 2015 BATHING SEASON	14

I.	MDPH ACCOMPLISHMENTS	15

A.	Beaches Website/Data Management	15

B.	Trainings	15

C.	Quality Assurance	16

D.	Laboratory Program	16

E.	2014 Amendments to Bathing Beach Regulations	16

F.	Emergency Response	17

II.	MONITORING	19

A.	Results	19

1.	Marine Beaches	19

2.	Freshwater Beaches	21

B.	Analysis of Results	22

FUTURE PLANS	27

A.	Direct Web-Based Reporting	27

B.	Training and Community Outreach	27

C.	Sanitary Surveys	27

D.	USEPA Recreational Water Quality Criteria	28

III.	SUMMARY	28

ACKNOWLEDGEMENTS	30

REFERENCES	31

TABLES	34

FIGURES	99

APPENDICES	108

TABLES											41

FIGURES											93

APPENDICES										103

# PART ONE: THE MDPH/BEH BEACHES PROJECT

## Overview

There are over 1,000 marine and freshwater public and semi-public bathing beaches in Massachusetts. Depending on weather and a variety of other changing conditions, beach water can sometimes contain bacteria at levels that create health impacts such as sore throat, gastroenteritis, or even meningitis or encephalitis. Therefore, it is critical to monitor bacteria levels at beaches and ensure levels are acceptable within U.S. Environmental Protection Agency (USEPA) and state regulatory standards. In 2015, 4.5% (n=720) of all samples collected during the bathing season exceeded bacterial standards, resulting in temporary beach closures in some cases.

In Massachusetts, bathing beach water quality is regulated by the Massachusetts Department of Public Health (MDPH) under Massachusetts General Law and the Code of Massachusetts Regulations. These require that all public and semi-public bathing beaches (e.g., beaches at camps, campgrounds, hotels, condominiums, country clubs) in the state be monitored for bacterial, and on occasion environmental, contamination during the bathing beach season. The exact dates of a given bathing season vary from beach to beach and are determined by the operators of each individual beach. Some open as early as Memorial Day, but the majority of beaches begin operation when the school year ends in mid-June, and most close for the season during the week of Labor Day.

## Background

### Beach Water Quality & Health: The Need for Testing

The health risks associated with both marine and freshwater swimming have been demonstrated in numerous studies. Swimmers may ingest or come in contact with pathogens (illness-causing microorganisms), and several prospective and retrospective epidemiological studies have demonstrated an increased risk of disease among swimmers relative to non-swimmers in both marine and fresh waters that are polluted with pathogens (e.g., bacteria and viruses). One retrospective study found the relative risk of gastrointestinal (GI) illness among swimmers in polluted waters to be one to three times that of non-swimmers.

Swimming in polluted marine water can lead to gastrointestinal symptoms (e.g., nausea, vomiting, diarrhea, abdominal pain), respiratory symptoms (e.g., sore throat, cough, chest cold, runny nose, sneezing), eye and ear symptoms (e.g., irritation, earache, itchiness), dermatological symptoms (e.g., skin rash, pruritis), or constitutional symptoms (e.g., fever, chills). Several studies conducted by the USEPA and others have also associated gastrointestinal symptoms with swimming in polluted fresh waters, and more recent studies have reaffirmed that there is a significant association between swimming in contaminated water and gastrointestinal illness.

Pathogens in beach waters typically have a fecal source, and pathogens associated with human fecal matter (e.g., some strains of Escherichia coli) may enter both marine and fresh beach waters in a variety of ways. Many of these pathways involve sewage: system failures in human sewage treatment facilities, leaking sewer pipes, combined sewer overflows, illegal sewer hookups, leachate from septic systems, or discharge of sewage by boats. Other sources of pathogens in beach waters include (but are not limited to) rainfall and resulting surface water runoff (washing contaminants such as animal wastes from dogs or farms into beach water). Bathers may also contribute significantly to pathogen concentrations in recreational waters, and swimmer-to-swimmer contamination is another potential source for microbial contamination. All of these factors contribute to elevations in bacteria which can cause illness among swimmers.

### Establishment of the MDPH/BEH Beaches Project

Responding to these health concerns, state and federal regulatory agencies have worked together to establish a system to protect the public from exposure to swimming-related pathogens.

In 2000, the U.S. Congress enacted the Beaches Environmental Assessment and Coastal Health (BEACH) Act (Appendix C). The BEACH Act, which amends the Federal Water Pollution Control Act (often referred to as the Clean Water Act, or CWA) is intended to improve the quality of the nation’s coastal recreational waters. It seeks to reduce the risk of illness to users of these waters through the identification of high-risk beaches, identification and mitigation of sources of pollution, and notification/risk communication to the public. It also authorizes grants to eligible states to support these objectives.

Also in 2000, the Massachusetts legislature passed An Act Relative to Minimum Standards for Public Bathing Waters, often referred to as the Massachusetts Beaches Act (Appendix D). The Act directed MDPH and local health officials to:

(1) adopt bathing water standards protective of public health to apply to all public and semi-public bathing beaches across the state;

(2) require regular bacteria testing at all public and semi-public beaches; and

(3) notify the public when bathing standards are violated.

The Act mandated that all beaches be tested weekly except where MDPH and the local board of health stipulated a greater or lesser frequency for a specific beach. It also mandated MDPH to publish an annual report analyzing statewide bacteria testing results.

Beginning in 2001, the program has received funding from USEPA. This funding has provided significant support for MDPH efforts to

(1) develop and maintain an inventory of marine bathing beaches;

(2) compile and analyze monitoring data; and

(3) conduct assessments of those beaches identified as high-risk.

With the help of these funds, and building upon such groundwork as the beaches inventory, the web-based reporting system, and mapping layer, MDPH/BEH has established a system of routine beach monitoring oversight that has been in place since 2002.

## Beach Water Quality Monitoring

### Sample collection

Water quality samples for most public bathing beaches in Massachusetts are collected by local boards of health. On Cape Cod, a large number of beaches are sampled by the Barnstable County Department of Health and the Environment. The Massachusetts Department of Conservation and Recreation (MDCR), which operates beaches, performs its own sampling. Samples for semi-public beaches are usually collected by the beach operator, although there are some communities that collect semi-public beach samples in the course of their routine sampling of public beaches.

Sample collection is required to be in compliance with Standard Methods for the Examination of Water and Waste Water of the American Public Health Association or as approved by the USEPA. Sample collectors are to record a variety of field data at the time of sample collection, using the current Beach Sampling Field Data Form developed by MDPH/BEH (Appendix E). Water samples, with field data form attached, are submitted to a certified laboratory for analysis. The field data are later included with the corresponding laboratory results when they are submitted to MDPH/BEH (see the Reporting section).

### Sample analysis

#### The MDPH contract laboratory program

All beach water samples are required to be analyzed within six hours of collection. The laboratories who perform the analyses are generally hired by either the city or town the beach is in (most often by its board of health, but sometimes by another municipal department, e.g., recreation) or by the operator of that beach. For the 2015 season MDPH/BEH used state funds and limited federal funding to contract with a number of laboratories to analyze qualifying communities’ public marine beach samples. Since 2003, MDPH/BEH has reimbursed communities over $1,100,000 for the analysis of approximately 57,000 marine samples from over 50 communities that have taken part in the contract laboratory program.

#### The use of indicators

In the United States, most swimming-associated diseases are caused by a wide variety of pathogens associated with fecal contamination (Cabelli, 1983). Most of these pathogens are very difficult to measure directly, but water samples that contain them also contain other microorganisms which are easier to measure. These “indicator organisms” provide a reliable indication of the pathogens’ presence and quantity. By measuring these other microorganisms, which live in the same microbiologic conditions, follow the same life cycles, and occur at levels proportionate to those of the pathogens, public health officials are able to estimate the level of the pathogens in beach water samples. When the presence of one microorganism is used to indicate the presence of another, it is referred to as an “indicator”.

#### Enterococci

In its Ambient Water Quality for Bacteria – 1986, USEPA recommended that enterococci, rather than fecal or total coliforms, be used as the indicator species in marine water quality testing. Since 2000, enterococcus has been the required indicator for routine marine beach testing in Massachusetts (105 CMR 445.000). All marine beaches submitting data have used this method since 2004.

The enterococcus method detects the number of bacteria that grow under certain laboratory conditions (USEPA, 1985). It measures the concentration of bacteria from a group of species within the Streptococcus genus, some of which (e.g., Streptococcus faecalis) are typically found in human and animal intestines (USEPA, 1985). Although not all of the species detected by this method are associated with fecal contamination (USEPA, 1985), leading to false-positive results, it is prudent, for public health purposes, to treat all exceedances of the indicator level as possible public health risks. Moreover, the enterococcus method does not detect as many non-fecal species as older methods do (e.g., fecal or the total coliform), and is therefore more accurate. However, all viruses and some bacterial pathogens are not detected by this method.

#### E. coli

Escherichia coli, usually referred to as E. coli, is a species of bacteria that originates in human and animal intestines (USEPA, 1985). Certain strains of this species are enteric (i.e., intestinal) pathogens (NAS, 1977). While both the total and fecal coliform methods can detect E. coli as part of a group of organisms, the E. coli method tests specifically for the presence or absence of this one particular species. Because E. coli originates in human and animal intestines, this method is a very sensitive indicator of fecal contamination for freshwater beaches (USEPA, 1985).

#### Laboratory Methods

Enterococcus and E. coli are currently the preferred indicators for beach water quality testing and the only ones accepted in Massachusetts. The laboratory methods required for beach water analysis in Massachusetts are those specified in the most recent edition of the American Public Health Association’s Standard Methods for Examination of Water and Waste Water or as approved by the USEPA.

Currently, the required methods for enterococci, either Method 1600: Membrane Filter Test Method for Enterococcus in Water or Enterolert. Method 1600, which was approved and adopted by USEPA in 1997, enable a faster turnaround time for testing of enterococcus, making it practical for local use. Laboratories contracted by MDPH to perform public, marine beach sample analysis are required to utilize the Modified Enterococcus Method (Method 1600) or Enterolert as approved by the USEPA and the MDPH/BEH Beach Project QAPP. Both are culture-enzyme-substrate methods, approved and adopted by USEPA in 2003 for testing ambient water (Jagals et al., 2000; Federal Register, 2003).

#### Bacterial Standards

Water quality standards are guidance concentrations used by public health officials to make decisions regarding the health risks associated with swimming. These criteria are typically expressed as the concentration of an indicator in the water above which there is an unacceptable risk for adverse health effects in swimmers.

Because the correlation between indicator levels and the levels of the actual pathogens posing health concerns is strong, indicator levels allow public health officials to estimate the health risk related to swimming at a particular beach. But other site-specific factors are taken into consideration to supplement these estimates, such as recent rainfall patterns and the number of people who use the beach.

The concentration of a microorganism in water is usually reported as the number of colony forming units (cfu) of indicators present per 100 milliliters (ml) of water. Massachusetts has specific water quality standards for marine water and freshwater.

##### Marine

USEPA (1986) used the relationship between the number of cases of swimming-associated disease and the enterococcus concentration in bathing water to establish the criteria for enterococci in marine waters at 104 cfu per 100 ml for a single sample and 35 cfu per 100 ml for the geometric mean of at least five samples over a 30-day period. These standards were set such that the expected incidence of gastrointestinal illness among swimmers would be the same as it had been for the previous USEPA water quality criteria for fecal coliform (i.e., 19 illnesses per 1,000 swimmers at marine beaches). MDPH/BEH adopted these standards by regulation beginning with the 2000 bathing season, with an exception.  The geometric mean standard of 35 cfu/100 ml remains the same, but is calculated using the five most recent non-storm event samples.

##### Freshwater

As indicated in the regulations (105 CMR 445.031) (see Appendix B), the indicator organisms for freshwater bathing beaches are E. coli and enterococcus. This is based on research conducted by USEPA (Dufour, 1984; USEPA, 1986). Each freshwater beach is required to test for one of these two indicators.

For enterococcus, no sample shall exceed 61 cfu per 100 ml, and the geometric mean of the most recent five samples within the same bathing season shall not exceed 33 cfu per 100 ml. For E. coli, no sample shall exceed 235 cfu per 100 ml, and the geometric mean of the most recent five samples within the same bathing season shall not exceed 126 cfu per 100 ml. These are the standard criteria established in MDPH/BEH regulations (105 CMR 445.031). The geometric mean is calculated using non-storm event samples.

Both the E. coli and the enterococcus standards are based on studies (Dufour, 1984; USEPA, 1986) that showed that levels of E. coli and enterococcus correlated strongly with rates of swimmer-associated gastrointestinal disease in freshwaters. The values are set to a level of risk of no more than eight cases of acute gastrointestinal illness per 1,000 swimmers in freshwater beaches.

### Reporting

The laboratories performing these analyses report their results to the beach operator or board of health that has hired them. Beach operators report their results to the local board of health. Boards of health report them to MDPH/BEH.

For communities with public marine beaches, the MDPH contract laboratories report the results directly to the MDPH/BEH Beaches Website via a secure Internet connection as soon as they are generated. Data are then displayed on the Beaches website in near real-time for public notification of beach closures and test results. Some boards of health that do not use MDPH/BEH contract laboratories send their marine sampling results to MDPH/BEH staff, who then enter the data onto the beaches website.

#### The Beaches Website

In 2003, using funding provided as part of the USEPA BEACH Grant, MDPH established a web-based system designed to make up-to-date water quality information on all public marine beaches available to the public as quickly as possible. This system has two components:

(1) A series of password-protected data-entry pages through which MDPH/BEH contract laboratories enter all water quality data (along with corresponding field data) directly into one centralized database. The laboratories are required under the MDPH/BEH contract to enter these data as soon as they become available. Local boards of health also have access to this portion of the website to review laboratory and associated field data in order to most efficiently take public health action.

(2) The Beach Water Quality Locator, a public website that allows users to select a beach via a series of interactive maps of the Massachusetts coast to see if it is currently open and to view its most recent test results. Historical data for each beach are available as well.

In 2006, the MDPH beaches website was enhanced through the addition of a GIS layer to display maps of beach locations, provide graphs for both single sample and geometric mean data, and improve reliability and efficiency for data entry. These improvements allow the public to quickly find the locations of all beaches through the use of GIS maps and to view graphical and tabular historical monitoring data. In 2014, revisions to the website posting mechanism were made such that for a majority of beaches a posting will display only after two consecutive exceedances have occurred.

#### Exceedances: Beach Closures & Public Notification

When a water sample from a beach exceeds a bacterial standard (either one or two consecutive single sample violations or a geometric mean violation), Massachusetts law requires that the beach be posted. MDPH/BEH contract laboratories are required to report exceedances of bacterial water quality standards to MDPH/BEH and local boards of health as soon as analyses are completed and results are available. Beach operators are required to report exceedances to their local boards of health immediately.

Under Massachusetts law (MGL C 111, § 5S), the local board of health is required to post standard signs at the key access points to a beach immediately after, or within 24 hours of, being notified that the beach did not meet water quality standards. In addition, the board of health is required to notify MDPH/BEH within 24 hours of the posting by submitting the standard beach posting form provided by MDPH/BEH. The posting form affirms that the beach waters have been closed and that signs have been put up at that beach. MDCR is responsible for the posting of its own beaches.

For public marine beaches, up-to-date posting information can also be accessed on MDPH/BEH’s Beach Water Quality Locator website. MDPH contract laboratories enter these results into the Beaches Website as soon as they become available. When the results for a given beach exceed water quality standards, the website automatically generates a notification of that beach’s postings. These closure notifications (or “postings”) are added to the Beach Water Quality Locator webpage twice each day, at 9:30 AM and 12:30 PM. This means the web-based system allows for public notification that is as near to real-time as possible. Local health officials can view postings shortly before public notification, which gives them an opportunity to place signs at the beaches and to prepare for public inquiries that may result, depending on the most recent data. MDPH/BEH staff have provided training to local health officials on how to use the website.

In 2014, the MDPH Public Health Council approved for promulgation amendments to the bathing beaches regulations (105 CMR 445.000, Minimum Standards for Bathing Beaches). These amendments define bathing water quality as unacceptable when two samples collected on consecutive days exceed the established water quality standard. This change, consistent with practices in other Northeast states (e.g., Connecticut, New Jersey), stipulates that a posting will not be required until two samples collected on consecutive days show elevated levels of bacteria. For beaches with a history of multi-day elevated bacteria levels (i.e., one or more instances of consecutive exceedances in at least two of the last four beach seasons), postings continue to be required after a single exceedance. These amendments are discussed in greater detail in Part Two, Section I.E.

#### Data Management

Marine data, already entered via the website, are uploaded to USEPA by MDPH/BEH in fulfillment of USEPA reporting requirements under the USEPA BEACH Grant, which mandates that MDPH must electronically report to USEPA all routine marine monitoring sampling data and laboratory results, as well as beach postings, on an annual basis.

The marine data are also kept in an in-house database at MDPH/BEH for analysis and inclusion in this report.  Freshwater data (including field data) are entered into the same database. All data are validated and checked for completeness by MDPH/BEH personnel. Local boards of health and laboratories are contacted directly, as necessary, to resolve questions and discrepancies in the data.

### Quality Assurance

In 2003, MDPH/BEH completed its Quality Assurance Project Plan (QAPP), which was approved by USEPA. In 2007, the QAPP was revised to reflect changes in the Beaches Project. This update was approved by USEPA and distributed to MDPH/BEH’s contract laboratories before the 2007 beach season. The QAPP describes quality assurance and quality control mechanisms developed and related steps (including enforcement measures) taken to ensure that the state’s beach monitoring activities and the resulting data meet USEPA’s published performance criteria. It also updates details on approved laboratory methods, MDPH/BEH contacts, and website information. MDPH/BEH uses the same quality standards for its freshwater monitoring activities.

### The Tier System and Frequency of Testing

The Massachusetts and federal beach Acts require that all public and semi-public marine bathing beaches be tested weekly. However, some beaches have a history of severe pollution problems, while others have proven over time to be exceptionally clean. The former require more frequent monitoring, and the latter less frequent monitoring. For a beach that has not had a single violation for two consecutive years and for which a sanitary survey has been completed to ensure there is a low risk of future violations, weekly testing may result in unnecessarily burdening local health officials’ resources that could be more effectively used (e.g., providing increased testing at those beaches which, due to greater pollution, are known to pose a greater health risk to swimmers).

#### The Three Tiers

To address this, the USEPA BEACH Grant required the development of a tiered monitoring approach to sampling, and in 2003 MDPH/BEH developed the Public Health-Based Beach Evaluation, Classification, and Tiered Monitoring Plan. The purpose of the Plan is to facilitate the identification and clean-up of pollution problems, while allowing those beaches with more pristine records to be monitored less often than weekly. The Plan is based on a three-tier system that classifies all beaches (both marine and freshwater) according to the severity of their pollution:

Tier One includes heavily used beaches which have pollution problems. Because of the ongoing pollution concerns/violations, these beaches are generally sampled more than once a week. There are currently five Tier One beaches in Massachusetts. All five are marine beaches and are tested daily.

Tier Two includes higher-use beaches with some pollution. These beaches must be tested once per week. The majority of beaches (424 of the 529 marine and 542 of the 544 freshwater beaches) are categorized as Tier Two beaches.

Tier Three beaches are those with no known pollution problems. They are required to be tested once every two weeks or sometimes less frequently, as determined by the local board of health and MDPH/BEH through the variance process. There are 100 marine beaches and two freshwater beaches currently listed as Tier Three beaches.

Because the frequency of monitoring mandated by both federal and state law is weekly, Tier Two functions as the default, or baseline classification. If monitoring data indicate severe pollution, a beach may be reclassified as Tier One and monitored more frequently. If the data show that a beach has maintained exceptionally clean water quality, it may be reclassified as Tier Three, allowing for less frequent than weekly testing, usually one to two times a month.

#### Sanitary Surveys and Variances

For a beach to upgrade to Tier Three status, its operator must apply to the local board of health for a variance; beaches operated by state agencies must apply to MDPH. Pursuant to Massachusetts regulations (105 CMR 445.100), two requirements must be met for the variance to be issued: (1) the beach must have a proven track record of “clean” sampling (at least two full seasons of water quality data with no exceedances); and (2) MDPH/BEH’s sanitary survey form must be completed for the beach by a Registered Sanitarian, Certified Health Officer, or Registered Environmental Health Specialist. The Survey is a tool health officials can use to assess the level of pollution at a given beach and to identify all possible sources of contamination (e.g., sewage discharge, stormwater overflows, bird and animal populations). Local health officials must review sanitary surveys before approving variance applications for final approval by MDPH.

## Limitations

The ability of MDPH/BEH to provide prompt public notification of beach water quality monitoring results is limited by both the completeness and accuracy of the data reported. In addition, the use of indicator organism criteria, although strongly supported in the literature, has some uncertainties. Finally, analytical techniques that require 24 hours to generate results may potentially leave beach users at risk on the day that a sample was collected and leave beaches closed the following day when most beaches will show clean results.

Although data completeness and accuracy are inevitably reliant upon the parties and individuals involved in data collection and reporting, the electronic reporting system and public beaches website have vastly improved the accuracy and quality of marine data submitted. Another stabilizing factor is the 100% compliance Massachusetts has achieved in the use of enterococcus, the state and federally mandated indicator organism, in testing at public marine beaches reporting routine monitoring results.

In recent years, MDPH/BEH was provided data from approximately 99% of the communities with open freshwater beaches. The amount and quality of data submitted from each community, however, varied greatly. During the beach season, communities often use different monitoring techniques. Therefore, the comprehensiveness of data varies among communities. Currently, with the exception of exceedances, which are required to be reported to MDPH/BEH within 24 hours, freshwater beach data are normally reported once during the year, after the end of the beach season. As a result, MDPH/BEH personnel can only review the data for proper sample collection and testing techniques after the sampling season has ended. MDPH/BEH continues to work individually with local boards of health to reduce issues related to quality control and variability by providing guidance and resources as necessary.

Another limitation, related to the specificity of analytical methods, is that the data are indicator-, not pathogen-, specific. As a result, the data only suggest a potential for the presence of pathogens that can cause human disease. The presence or absence of specific pathogens is not directly measured. These are inherent limitations of using indicators as a measure of water quality, in Massachusetts and elsewhere. However, it does need to be emphasized that a substantial body of scientific research generally supports the use of these indicators as described earlier in this document (Cabelli, 1983; USEPA, 1986).

The criteria developed for each indicator are set at an acceptable level of risk of an adverse health effect, in this case gastrointestinal (GI) illness, rather than at a no-risk level. The indicator limits recommended by USEPA for enterococcus in marine waters are associated with a risk level of 19 GI illnesses per 1,000 swimmers (USEPA, 1986). Therefore, levels of indicators considered in compliance by the Massachusetts and national requirements do not imply absence of risk of adverse health effects for the total population at risk.

Using current indicators, it takes 24 hours to receive the results of a bathing beach water sample analysis (Wade et al., 2005). This delay can lead to the exposure of bathers to unsafe bacterial levels, as well as unnecessary closings (Wade et al., 2006) (e.g., beach closed on day of results, but by then the bacterial criteria may not be exceeded). This delay also makes it very difficult for investigators to track the contamination back to its sources, as it may dissipate before an investigation begins (SCCWRP, 2006).

# PART TWO: THE 2015 BATHING SEASON

## MDPH ACCOMPLISHMENTS

In addition to its routine monitoring activities (outlined earlier), MDPH/BEH performed a variety of more specific activities, unique to the 2015 season.

The required field data forms were distributed to local boards of health in 2015.

Beaches Website/Data Management

The beaches website was maintained and improved as needed to provide the public with the most up-to-date information as well as to present the data in an easy to use format. Links to all MDPH/BEH standardized forms were checked and the forms were made available for download via the Forms link in the Publications and Reports section on the bathing beaches website. Both local communities and laboratories were notified and given field data forms that made it easier for samplers to record conditions while in the field.

Guidance and training were provided to local boards of health, when necessary, to ensure quality assurance for data entry conducted outside of the contract laboratory program.

The beach program’s database was again updated for the 2015 beach season. Beach locations were revised as sampling points were combined at continuous, uninterrupted beaches; non-swim beaches were identified and reassigned; and new swim beaches were identified. Throughout the duration of the project, MDPH has continuously been adding new information associated with beaches, such as waterbody information, latitude/longitude coordinates, and potential pollution sources of freshwater beaches.

After the 2013 bathing season ended, extensive enhancements were made to the MDPH beach database to improve alignment with the USEPA reporting schema. Enhancements included the following:

- Analytical methods are now being reported to USEPA.

- The formatting for reporting non-detects has been improved.

- The procedure for assigning USEPA-issued identification numbers to beaches has been better aligned with USEPA specifications. This particular change to meet USEPA requirements resulted in some multiple sample location marine beaches being reported as a single beach vs. multiple beaches. This will be further explored in the discussion of 2015 marine beach results.

- A system for ensuring the reporting of any changes to beach name, coordinates, sampling frequency, or Tier has been implemented.

- The latitudes and longitudes of each beach are now being reported to USEPA.

Four trainings for local health officials were conducted prior to the beach season in 2015.

Trainings

In April 2015, MDPH gave presentations to local health officials at four seminars held by the Massachusetts Health Officers Association (MHOA) and MDPH/BEH Community Sanitation Program at geographically diverse locations across the state. MDPH staff presented a review of 2014 beach data and regulatory amendments (see Part Two, Section I.E). Staff also responded to questions from local health officials in preparation for the 2015 beach season. Informational packets were provided containing the beach sampling field data forms, sanitary survey forms, posting forms, and fact sheets. Importantly, time was allocated for health agents to provide feedback and pose any questions they had regarding forms and procedures.

In preparation for the 2015 beach season, MDPH staff personally communicated with and conducted outreach to bathing beach communities while collecting 2014 beach data. Local health officials were reminded of their responsibilities under the state’s bathing beach regulations and provided with any technical assistance or forms needed. MDPH staff also discussed deficiencies in reporting and updated the internal database based on these conversations. These efforts help enhance reporting, as nearly all communities in the state now report beach testing results on a yearly basis.

### Quality Assurance

MDPH staff conducted numerous site visits to ensure proper public notification of bacterial contamination.

Throughout the beach season MDPH staff conducted numerous inspections at selected beaches identified as being posted to ensure proper signage was present. MDPH staff also assisted local health officials and laboratories in developing their weekly sampling schedules for the 2015 beach season. MDPH helped analyze the locations and logistics with local health officials, and staff standardized field forms with beach names and the weeks when they were to be sampled so that appropriate sampling schedules were maintained (weekly, biweekly or monthly).

### Laboratory Program

54 of the 60 marine communities utilized the contract laboratory services.

Since 2003, MDPH/BEH has supported local marine communities for routine monitoring through the services of contract laboratories funded by MDPH. This support continued in 2015. The laboratories are Barnstable County Department of Health and Environment Water Quality Testing Laboratory, Biomarine, Inc., G&L Laboratories, Inc., New Bedford Health Department Laboratory, and Wampanoag Environmental Laboratory.  The laboratories funded by MDPH/BEH analyzed 4,023 marine beach samples from 54 marine beach communities during the 2015 beach season.

Laboratories fulfilled their contract requirements by promptly entering sampling data and laboratory results into the MDPH/BEH public notification website as results became available. Beach postings were automatically generated by the website when submitted samples exceeded acceptable water quality standards. Display of these postings on the public pages occurs twice per day, at 9:30 AM and 12:30 PM.

### 2014 Amendments to Bathing Beach Regulations

MDPH has collected water quality data at approximately 1,000 marine and freshwater beaches for the past 15 years. An analysis of historical data which was conducted after the 2013 beach season suggested that beaches rarely have two consecutive tests that exceed the relevant USEPA single sample water quality standard (for marine waters, 104 cfu/100 ml enterococci; for fresh waters, 235 cfu/100 ml E. coli or 61 cfu/100 ml enterococci).  For the majority of beaches, any initial exceedance is transient in nature and not reflected in the confirmatory sample that is processed the next day (i.e., 24 hours after the initial exceedance). Therefore, beach operators (primarily local health agents) have often posted an advisory sign at a beach when the current water quality meets the established standard. Posting advisories in this manner deprives residents of the benefits of swimming and other water-related exercise, creates unnecessary work and costs for the local health agents, and causes economic losses to communities and businesses connected with those beaches.  In an effort to address these concerns, MDPH proposed regulatory amendments to the bathing beaches regulations in April 2014. These amendments were approved by the MDPH Public Health Council on May 14, 2014 and went into effect on June 6, 2014. At the majority of beaches, an advisory is no longer required until two samples collected on consecutive days show elevated bacteria levels. If a retest is not collected on the day following the initial exceedance, then posting is still required until a sample confirms that the water quality standard is met. For beaches with consecutive exceedances in two or more of the last four beach seasons, postings continue to be required after each exceedance. Thus, under the regulatory amendments, the quality of bathing water is considered unacceptable based on the following evaluation of the bacteriological water quality criteria:

- Two samples of bathing water, collected on two consecutive days, that both exceed the single sample water quality standard, or one sample of bathing water that exceeds the single sample water quality standard when an additional sample is not collected on the following day; or

- One sample of bathing water that exceeds the single sample standard at beaches where, in two or more of the last four full beach seasons, samples collected on two consecutive days both exceeded the single sample water quality standard; or

- Any bathing water sample that exceeds the geomean water quality standard.

Please refer to Appendix A for a copy of the current regulations and Appendix E for a further description of the regulatory amendments.

### Emergency Response

MDPH/BEH has provided assistance in investigating potential outbreaks of water-borne parasites and illnesses. In past years, incidents requiring emergency response have included occurrences of E. coli O157:H7, giardiasis, shigellosis, and Vibrio vulnificus infection. These response actions can involve reviewing sampling results and/or medical records, or the preparation of educational materials, often in collaboration with local boards of health, other programs within MDPH/BEH (e.g., Food Protection, Community Sanitation), other MDPH bureaus (e.g., MDPH’s Bureau of Infectious Disease), or other state agencies.

MDPH/BEH has also responded to numerous occurrences of algal blooms across the state, many of which posed a potential health risk to swimmers due to their proximity to bathing beaches. In 2007, MDPH/BEH developed a protocol for responding to harmful algae blooms at freshwater bodies.  Typically, this involves performing a site visit and providing technical support and educational materials to local health officials. During the 2015 season, MDPH/BEH issued 20 algae bloom advisories for lakes, ponds, and rivers throughout Massachusetts, 6 of which had beaches. In total, 7 beaches were affected by these advisories.

## MONITORING

### Results

MDPH received data for over 1,100 sampling locations at over 1,000 marine and freshwater beaches in 2015.

220 communities reported data to MDPH in 2015.

During the 2015 bathing season, 220 communities in Massachusetts with public or semi-public, marine or freshwater beaches sent water quality data to MDPH/BEH. In total, MDPH received water quality data collected from 587 marine and 590 freshwater sampling locations at 529 marine and 544 freshwater beaches, respectively.  Due to the length of some beaches in Massachusetts, multiple sampling locations are necessary to distinguish specific areas of water quality. For the purposes of this report, a sample location is considered a single beach. In total, MDPH/BEH received results for 15,932 water samples from marine and freshwater beaches collected during the 2015 beach season. Thirty-nine communities reported only marine bathing beach data, 160 communities reported only freshwater beach data, and 21 communities reported both marine and freshwater bathing beach data (Table 1).

#### Marine Beaches

There are 60 communities with marine beaches in Massachusetts.

During the 2015 bathing season, 60 Massachusetts coastal communities with public or semi-public marine bathing beaches submitted beach monitoring data to MDPH/BEH. These communities accounted for 587 sampling locations at 529 public or semi-public marine bathing beaches.

A total of 8,050 samples were collected at marine beaches in 2015 to evaluate bacterial water quality.

A total of 8,050 water samples were collected from marine public and semi-public beaches and reported to MDPH/BEH during the 2015 bathing beach season (Table 2). Bather density data were collected as part of routine sampling. Massachusetts regulations require samples to be taken within the area of greatest bather density (105 CMR 445.000). GPS surveys of marine beaches completed by MDPH/BEH in 2003 and subsequent observations by MDPH/BEH beach inspectors confirm that samples are being taken within the areas that typically receive the highest use (greatest bather density) such as areas near main entrances and/or areas closest to parking lots. Due to the time needed to collect and analyze samples, a majority of the samples were collected at times when bather density consisted of ten or fewer individuals (Table 3). Most samples were collected before noon, when the bather load is generally low even in high-use areas.

100% of the samples collected were analyzed with the mandated indicator method.

With the passage of the Massachusetts Beaches Act in 2000, the state adopted the USEPA-recommended enterococci as the standard indicator for water quality monitoring at marine beaches. The institution of the MDPH/BEH beaches website and contract laboratory program has helped all boards of health in Massachusetts’ marine communities to adopt the use of enterococci as an indicator organism. Enterococcus was the indicator used for all water samples taken at marine beaches in 2015. The use of MDPH/BEH contract laboratories for the analysis of public marine beach water samples has played a major role in achieving uniform compliance with the MDPH/BEH regulations for marine beaches.

Nearly 100% of marine beaches were tested as required in 2015.

Eighty three percent of the marine beaches were tested daily or weekly (the minimum requirement is weekly sampling) (Table 4). The remaining marine beaches were permitted to sample less frequently because of Tier Three status.

Local health departments, independent laboratories, the National Park Service, MDCR, and semi-public operators (camps, hotels, neighborhood associations, etc.) were responsible for collecting the majority of the marine beach water samples. MDPH/BEH contract laboratories performed the majority of analyses for these samples during 2015. Four communities (Boston, Lynn, Revere, and Salisbury) have marine beaches that are solely managed by MDCR and therefore are not eligible for the contract laboratory system. MDCR coordinates with MDPH for laboratory results to be entered onto the beaches website by the laboratories they utilize. The towns of Kingston and Sandwich opted not to use MDPH/BEH contracted laboratories in 2015.  Each of these towns faxed their data to MDPH beach inspectors and these data were subsequently entered directly onto the beaches website for prompt public notification.

The total number of marine beach postings (i.e., verification to MDPH/BEH that a sign was posted at the beach) received in 2015 was 240 (Table 5). As noted previously in Part Two, Section I.A., following the 2013 season, MDPH/BEH enhanced its beach database reporting to meet USEPA data reporting requirements due in the winter of 2013-2014. USEPA requirements included the use of USEPA-issued beach identification codes and treatment of multiple sample location beaches as a single beach if the operator made posting decisions for the entire beach based on an exceedance at any one location or a subset of locations. For beaches where the operator would close at a single location but not at other locations that tested clean, a unique USEPA-issued identification code was assigned to each sample location.

The number of marine beach postings in 2015 was 240.

As a result of the USEPA reporting requirements, MDPH/BEH reported to USEPA in February 2016 a total of 240 marine postings in 2015 (vs. 289 postings based on previous reporting requirements). The difference is primarily attributed to multiple sample location beaches managed as a single beach now being reported as a single beach posting instead of multiple location postings. For example, at Constitution Beach in East Boston, which has three sampling locations managed collectively, in previous years a three-day closure would have been reported as three separate postings that lasted for a total of nine days (three days each at three sampling locations). The new system would report this as one posting that lasted for three total days (three days at one beach).

4.8% of all samples collected exceeded the bacterial standard.

The number of postings is less than the total number of single sample exceedances (n=390). A single posting may cover multiple exceedances and exceedances occurring prior to a beach’s opening might not be posted. However, the primary reason is the regulatory amendments that were promulgated in 2014: at the majority of beaches, a single sample exceedance does not require posting if a resample collected on the day following the initial exceedance meets water quality standards. These data are discussed further in the Analysis of Results section. The percentage of exceedances for the total number of samples collected was 4.8% in 2015 (Table 6). Of the 587 public or semi-public marine beach locations, 154 (26%) incurred at least one bacterial exceedance (Table 7).

154 marine beaches had at least one exceedance.

Total rainfall was slightly below normal in Boston and slightly above normal in Chatham.

Total rainfall amounts in Massachusetts were slightly elevated for the months of June and August during the 2015 season as compared to 2014 (Tables 8 through 11). The Boston area received 9.29 inches of rain during the 2015 beach season (i.e., June through August), which is below the 10.46 inches of rain normally received in those months and more than the 8.94 inches received in 2014.  The Chatham area (Brewster, Chatham, Dennis, Eastham, Harwich, and Orleans) received slightly elevated rainfall totals during the 2015 beach season than 2014 (10.36 inches vs. 9.72 inches respectively).

Samples associated with potential pollution sources at the beach at the time of sampling had a higher rate of exceedances compared to samples for which no potential sources were noted.

As part of routine sampling, environmental observations should be recorded on a field data form and reported to MDPH/BEH. Samplers have the option of recording potential sources of pollution, as well as noting when no sources are observed. In 2015, the field forms accompanying 4,389 of the 8,050 marine samples collected (about 55%) included information as to whether potential transient pollution sources had been observed at the time of sampling.  Of those 1,132 samples, 26% recorded the presence of a specific source (e.g. birds, dogs, waste solids, trash), and 7% of those samples exceeded the bacterial standard (Table 12). The field forms for 3,257 samples specifically noted the absence of potential pollution sources. Of those, 2% exceeded the bacterial standard. The data suggest that potential bacterial sources at the time of sampling present a higher risk of bacterial exceedances.

There were a large number of samples (3,661) for which information on pollution source (or lack thereof) was not provided on field data forms. For these samples, 6.7% exceeded the bacterial standard. MDPH continues to stress the need to record these potential sources on the field data form. Although some communities have improved in this area, many still do not complete the form.

#### Freshwater Beaches

A total of 7,882 samples were collected at Massachusetts freshwater beaches in 2015.

During the 2015 bathing season, 181 Massachusetts communities with public or semi-public freshwater bathing beaches submitted beach monitoring data to MDPH/BEH. These communities provided data for 590 public or semi-public freshwater bathing beaches and collected a total of 7,882 freshwater samples (Table 2).

Routine samples are most often collected in the morning to allow time for laboratory delivery.

For bather density (Table 3), the data are similar to those for marine beaches, with a high percentage (85%) indicating low bather density (0-10 bathers on the beach) during sampling. As discussed previously, most samples are collected during non-peak bathing hours, usually between 8 AM and 12 PM. Samples at beaches are often taken in the morning to allow adequate time for delivery to and analysis at the laboratory.

Nearly 98% of freshwater beaches were tested at least weekly.

In 2015, local health officials used the approved indicator organism (either E. coli or enterococci) at 100% of freshwater beaches in Massachusetts, with the majority of beaches using the E. coli indicator. Over 98% of public and semi-public freshwater beaches in Massachusetts were tested with at least the minimum required weekly frequency in 2015 (Table 4). Two freshwater beaches have Tier Three status and sample every other week or once a month, as approved by MDPH and the local board of health. Less than two percent of freshwater beaches (n=9) either did not sample as required or did not submit data to MDPH/BEH detailing all sampling conducted for the season. In addition, there were 2 beaches in 2 towns that did not submit any data to the local board of health and/or MDPH and are considered to be out of compliance for the 2015 bathing season. These are not included in the tally of 590 freshwater beaches. As noted, communities that did not test all their beaches with the required frequency have been contacted to review regulatory requirements. Independent laboratories collected the majority of samples to be analyzed from freshwater beaches. Local health departments and MDCR collected the remainder of the samples.

4.2% of bacterial water quality samples exceeded the freshwater standard.

330 freshwater bacterial exceedances were reported in 2015.

The total number of freshwater beach exceedances detected in 2015 was 330 (4.2% of all freshwater samples collected) (Table 5). This was an increase from the number of exceedances of the fresh water quality standards (235 cfu/100 ml E. coli and 61 cfu/100 ml enterococcus) in 2014 (196 or 2.7%) (Table 6). Of the 590 public or semi-public freshwater beach locations, 158 (27%) incurred at least one bacterial exceedance (Table 7). These data are discussed further in the Analysis of Results section.

The field forms accompanying 3,495 of the 7,882 freshwater samples collected (44%) included information as to whether potential transient pollution sources had been observed to be present at the sampling location.  Of those 3,495 samples, 20% (n=689) recorded the presence of specific source (e.g. birds, dogs, algae, trash), and 9.3% of those 689 samples (n=64) exceeded the bacterial standard (Table 12). The field forms for 2,806 samples specifically noted the absence of potential pollution sources. Of those, 2.3% (n=65) exceeded the bacterial standard. As with marine beaches, there were a large number of samples (4,387) for which information on pollution sources (or lack thereof) was not provided on field data forms. For these samples, 4.6% exceeded the bacterial standard.

### Analysis of Results

In 2015, 220 marine and/or freshwater communities reported bathing beach water quality data to MDPH/BEH. MDPH/BEH received the required posting notification (or confirmed that posting was not necessary) for all but 5 of 390 single sample marine exceedances or approximately 99%. It should be noted that not all exceedances require posting. For example, if a beach is already posted because of a prior single sample or geometric mean exceedance and a follow up sample shows a continued exceedance, an additional posting notification to MDPH/BEH is not required for the follow up exceedance. Therefore a single beach posting could cover several exceedances. Local boards of health may preemptively post beaches without a bacterial exceedance and these instances are included in the total number of postings. It should also be noted that, due to regulatory amendments that were promulgated in 2014, the majority of beaches are no longer required to post after a single exceedance if a next day resample shows acceptable water quality. As a result, there were 150 occasions when postings were not necessary. The amendments had a significant impact in reducing the number of postings in 2015.

At freshwater beaches, there were 330 single sample bacterial exceedances in 2015. MDPH/BEH received posting notification (or confirmed that posting was not necessary) for all but 44 of these exceedances. This is slightly more than 2014, when there were 38 exceedances without posting notifications. Posting notifications were received/not required for 87% of exceedances at freshwater beaches in 2015, compared to 81% in 2014. Similar to marine beaches, there was also a reduction in freshwater beach postings as there were 76 occasions when postings were not necessary under the regulatory amendments.

MDPH receives the majority of the freshwater data after October 31 each year (the Massachusetts Beach Act requires submittal of data by October 31). Therefore, MDPH staff are not able to remind local health departments to submit the required posting notifications at the time of the exceedances, which may reduce the number of posting notifications received in a given year. The 44 exceedances that required postings but were missing them highlight the need for continued outreach to health departments of freshwater communities on beach water quality regulatory requirements, in particular the recent amendments. Missed postings for 5 of these 44 exceedances appear to have resulted from a misunderstanding of the revised regulations. Efforts were made by MDPH/BEH staff to obtain posting information by contacting communities both during and after the beach season to explain the regulations and by providing standardized reporting forms; both the forms and regulations are available for download from the MDPH/BEH website.

Completeness of the field data forms filled out by samplers has also increased over the years. While there are still areas for improvement, such as actively reporting the presence or absence of environmental pollution sources, Massachusetts local health officials have for the most part adhered to MDPH/BEH’s field forms. This can be seen in the wide range of potential sources of pollution noted on the field forms submitted in 2015. Prior to 2003, most noted potential sources of pollution were fairly general (i.e., outflow pipes, wildlife, and boats). Starting in 2004 and continuing in the 2015 bathing beach season, more communities began to document incidents of algae and wrack build-up on beaches and the presence of trash, birds, dogs, waste solids and fish die-offs. These notations become an important factor when the communities or MDPH/BEH need to identify possible reasons for continuously elevated bacterial levels at a particular beach that may increase potential health risks and to develop strategies to reduce these sources.

Observations made by samplers at freshwater beaches may help to explain some contributing factors to elevated indicator levels (Table 12). Of freshwater beaches that had a recorded pollution source, 9.3% exceeded public health standards, compared to 2.3% for those that actively noted an absence of observed sources.  For marine beaches, the percentage of exceedances at beaches where a pollution source was noted (7%) was also higher than those where none were noted (2%).  However, 45% of marine samples and 56% of freshwater samples were accompanied by a field data form that did not include any information on the presence or absence of pollution sources. Notification on the presence or absence of pollution sources is an area that needs improvement in order to help in the formulation of mitigation strategies.

The exceedance rate was within the historical average for marine beaches in 2015.

As shown in Table 6, from 2001 through 2015, from 2.8 to 7.0% of all marine samples collected during the summer bathing seasons exceeded the enterococcus standard, with an overall average of 5.0% exceedance across all seasons. The rate of marine beach exceedances in 2015 was 4.8% which is within the historical range. All marine communities that had at least one exceedance in 2015 appear in Figure 1. A complete listing of marine beaches sampled during the 2015 beach season, their exceedances, and postings can be found in Table 13.

Total rainfall amounts for the marine regions during the 2015 beach season (Tables 8 - 9) coincide with their historic total, with Boston and Chatham receiving near average levels of total rainfall. The Boston region received 9.29 inches of rain during beach season (i.e., June through August), which is slightly below the 10.46 inches of rain normally received in those months. The Chatham region received slightly above average rainfall during the beach season (10.36 inches received in 2015 vs. the 10.15 inches typically received). Similar rainfall totals vs. the historic average may help explain the similar exceedance rate vs. the historical average as rainfall is often a major driver of bacterial exceedances in beach water. Given that the Boston and Chatham regions received similar inches of rain in 2015 as in 2014, a comparable exceedance rate would be expected and was observed: 4.8% vs. 4.4%.  Figures 2 and 3 display rainfall information and the percentages of bacterial exceedances for the Boston and Chatham areas during the 2015 bathing season.

From 2001 through 2015, from 2.7 to 5.9% of all freshwater samples collected during the summer bathing seasons exceeded water quality standards, with an overall average exceedance rate of 3.9% across all seasons. The rate of freshwater beach exceedances in 2015 was 4.2% which is slightly higher than the historical average. All communities that experienced at least one freshwater exceedance in 2015 can be seen in Figure 4. A complete listing of freshwater beaches sampled during the 2015 beach season, their exceedances, and postings can be found in Table 14.

At freshwater beaches, the percentage of exceedances in 2015 was higher than the 2014 exceedance rate (2.7%). As shown in Tables 8 through 11, the amount of rainfall was highly variable in four areas around the state (Amherst, Ashburnham, Boston, and Chatham). As noted previously, during the 2015 bathing season both the Boston and Chatham areas received rainfall totals near the historical averages. The Amherst area’s rainfall during the bathing season was 39% higher than normal and over two inches more than in 2014.  The Ashburnham area’s rainfall during the bathing season was approximately equal to the normal amount. It should be noted that these are only four data points for a large geographical area and therefore localized rainfall totals could be different.

Figures 5 and 6 show the historical relationship between exceedances at marine and freshwater beaches and the total amount of rainfall between June and August. For both marine and freshwater beaches, exceedances generally rise and fall with rainfall amounts, with some exceptions. As discussed previously and as expected, in 2015 the amount of rainfall and the percentage of exceedances was consistent with 2014.

Bacterial exceedances are closely tied to rain events.

Table 15 and Figure 7 show that the total number of exceedances statewide is significantly higher within 24 hours of a rain event. These rain data are based on information recorded on the field data form. In 2015, 234 of the 390 marine exceedances had corresponding rain event information, while for freshwater beaches rain event data were recorded for 285 of the 330 bacterial exceedances. Sixty four percent of marine beach exceedances and 38% of freshwater exceedances occurred within 24 hours of a rain event in 2015. Figure 7 shows the exponential drop-off in the number of exceedances as the time from rainfall increases.

Greater bather use at a beach can increase bacterial levels.

The bather load at a particular beach can affect water quality as well because humans are also sources of fecal pollution. The greater the bather density at a beach, the greater the likelihood that human sources are contributing to higher enterococcus levels. However, as in previous years, more than three-fourths of the marine beach samples (83%) and freshwater beach samples (85%) that reported bather density indicated low bather density (0-10 bathers on the beach) during sampling (Table 3). This can be attributed largely to samples being taken during off-peak hours for swimming. Samples are primarily collected before 12:00 PM so that laboratories can begin the analysis before the close of business and before the six hour holding time expires.

While the data relative to the impacts of bather density on exceedances are extremely limited, beaches staff did evaluate the data to determine if trends were apparent. For marine samples with a corresponding bather density, exceedance rates showed an increase when the bather density reached 50 bathers or greater (Table 16). The overall rate of exceedances for all marine samples (4.8%) was higher than the rates of exceedance for the three lowest bather density groupings (0-10, 10-20, and 20-50 bathers) and lower than the rate of exceedance for samples that did not have a corresponding bather density. The rate of exceedance for samples with a corresponding bather density of >50 bathers was highest of all, at 19%. For freshwater samples, the exceedance rates also showed an increase when the bather density reached 50 bathers or greater (Table 16). The overall rate for all freshwater samples (4.2%) was similar to the lowest bather density grouping and slightly higher than the rate of exceedance for samples that did not have a corresponding bather density. The highest exceedance rate was again found in the bather density grouping of >50 bathers (18.3%). As with the marine data, the rate of exceedance for the freshwater samples was highest when the bather density reached 50+ bathers. Therefore, bather density appeared to have an effect on beach water quality in 2015.

Decaying plant material has been shown to incubate bacteria and may release bacteria to the water at high tide.

The decaying plant material, or wrack line, at a beach may also be an incubator for bacteria, potentially increasing bacterial counts even outside spring tides. In addition, it has been suggested that wrack is often the subject of scavenging by wildlife and pets, which may defecate in it, further increasing its contribution to bacterial contamination (Heufelder 1988). Wrack also keeps the soil surfaces it covers in a dark, wet environment, which is conducive to bacterial growth. Researchers have found that survival of fecal coliform and enterococcal bacteria was far greater in salt water when organic debris (i.e., wrack) were present (Martin and Gruber 2005). Furthermore, they concluded that tidal flushing of wrack during high tide could easily transport elevated bacterial densities into the marine environment, thus potentially degrading the surrounding waters (Martin and Gruber 2005).

Virtually all of the Massachusetts coast line is a no-discharge zone.

Illicit boat waste can be a major source of beach closures.

Other potential sources of bacteria, which are difficult to directly measure through routine beach water sampling, have the ability to influence overall water quality. At marine beaches, illicit discharges of human waste from boats may cause significant degradation of water quality in areas where there is significant boating activity. It is generally believed that the number of illicit discharges from boats is proportional to the difficulty posed in the disposal of the wastewater; therefore there has been significant effort by many coastal communities to increase the number of locations where boat waste can safely be discharged. USEPA worked with state and local officials to designate virtually all marine waters within three miles of the Massachusetts coast as a no-discharge zone.

Sediments may also be a bacterial incubator and contribute to higher bacterial results.

Additionally, sediments may act as a sink for fecal indicators at both fresh and marine beaches. These sediments may be disturbed by tides, human activities, or stormwater runoff and potentially increase bacterial contamination.

## FUTURE PLANS

### Direct Web-Based Reporting

In 2016, MDPH/BEH contract laboratories, local boards of health, and others will continue to perform data entry to the electronic, web-based public notification website. MDPH/BEH will be working with contract laboratories and other data reporters to ensure that field data are accurately recorded via the web-based reporting system. Important information regarding recent rainfall data and the presence of transient pollution sources will be targeted. As in previous years, a history of postings will be maintained on the website to facilitate analysis of the data. This will provide more accurate recordkeeping so that trends can be analyzed in future annual reports.

MDPH conducted four trainings in the spring of 2016 for local health officials.

Training and Community Outreach

In the spring of 2016, MDPH/BEH worked in collaboration with the MDPH/BEH Community Sanitation Program and the Massachusetts Health Officers Association to provide four separate training events for local boards of health in different regions of the state. These trainings provided information on beaches and harmful algae blooms. MDPH/BEH will continue to offer sampling training and provide additional technical assistance to freshwater and marine communities where needed. MDPH/BEH will also provide assistance on the use of the MDPH posting form and the field data forms that are required to be completed each time a sample is taken.

### Sanitary Surveys

MDPH/BEH will continue to facilitate sanitary surveys in support of the Tiered Monitoring Plan and the variance process during 2016. When the Tiered Monitoring Plan is adopted at specific beaches, a “high” priority beach will receive the most frequent water quality sampling and analysis. Such a beach might be one with high bather volume, high frequency or percentage of exceedances, problematic sources of pollution, or a combination of these factors. A “medium” priority beach will be sampled once per week and will still be required to meet water quality standards. Beaches that are tiered “medium” can have any of the factors listed for “high” priority beaches but with less frequency or intensity of any of the three criteria. A “low” priority beach is one that is relatively pristine. Low-priority beaches are eligible for less frequent testing, as infrequently as every 30 days under 105 CMR 445.000, if the local health department receives a testing variance. This categorization will assist MDPH/BEH in working with local health departments in 2016 to conduct sanitary surveys that will support the Tiered Monitoring Plan. Data from the 2014 and 2015 beach seasons will be incorporated into the existing Tiered Monitoring Plan to update the published classifications. These efforts will allow MDPH/BEH and marine communities to focus on determining and alleviating pollution sources at problematic beaches, and also allowing MDPH/BEH to reduce unnecessary sampling at low-priority beaches through the variance process. MDPH/BEH will be conducting many sanitary surveys at public marine beaches in support of these efforts.

### USEPA Recreational Water Quality Criteria

In 2012, USEPA published updated recreational water quality criteria (RWQC).  The criteria consist of two elements: a geometric mean (geomean) and a statistical threshold value (STV).  The geomean and the STV represent the 50th and 90th percentile, respectively, of a water quality distribution determined through USEPA epidemiological studies.  Within the criteria, USEPA also included two supplemental values, called Beach Action Values (BAVs), which represent the 75th percentile of the water quality distribution.  USEPA described BAVs as optional values that states could use when informing the public about unsafe water quality.

In 2014, USEPA published the National Beach Guidance and Required Performance Criteria for Grants, 2014 Edition (Guidance).  The document describes required performance criteria for states that receive USEPA BEACH Act grant funding, which MDPH has received since 2001.  One of the performance criteria requires that states identify and use Beach Notification Thresholds (BNTs).  USEPA defined a BNT as a water quality value that is used to “trigger” a beach closure notification.  The RWQC includes two USEPA derived sets of BNTs- the BAVs.  If a state chooses to use BNTs other than the BAVs, USEPA requires that the state “submit a written justification to USEPA based in science, local water quality data, or monitoring experience” (USEPA 2014).

MDPH conducted several analyses of local water quality data to determine the most appropriate BNTs for public health protection in Massachusetts,.  In general, these efforts were focused on comparing the BAVs listed in the 2014 Guidance, to the BNTs currently being used in Massachusetts (i.e., single sample maximums or geometric means described in the 1986 USEPA Recreational Water Quality Criteria).  Based on a comprehensive analysis that considered 5 years of local water quality data, MDPH analyses suggest that there when applying either set of standards to protect public health, there is no difference between the two.  Based on a rigorous analysis of empirical data from over 1,000 beaches, the current BNTs appear to be as health protective as the USEPA BAVs described in the 2014 Guidance.  Absent of a compelling reason to change the existing beach water quality criteria, MDPH will continue to use the 1986 Recreational Water Quality Criteria as BNTs when making beach closure notifications.

## SUMMARY

Nearly 15,000 samples were collected at over 1,000 beaches in 2015.

This report summarizes beach monitoring and testing data from Massachusetts public and semi-public marine and freshwater bathing beaches during the 2015 season. In total, 220 communities with operating bathing beaches reported 15,932 water samples collected at over 1,000 beaches. Beach results from the 2015 season show a comparable marine exceedance rate as the 2014 beach season, while water quality data at freshwater beaches showed an increase in the percentage of exceedances in 2015 compared to 2014. Massachusetts marine communities are nearly in full compliance with the regulations with the exception of some beaches missing sampling rounds and posting notifications. This illustrates in part the success of the electronic reporting requirement through the MDPH/BEH contract laboratory system for marine beaches. This requirement has also facilitated improved compliance with the regulations by boards of health in other areas besides sample reporting. For example, 100% of the marine beach samples were tested for the correct indicator required by regulation. MDPH/BEH also achieved nearly full compliance with the posting regulation in marine communities. Massachusetts freshwater communities continue to increase their usage of the required field data form, including identifying potential environmental pollution sources.

MDPH/BEH continues to provide training and information to local communities in an effort to improve compliance with the regulations. MDPH/BEH also continues to make improvements to its public notification website to make sure that information is accessible to the public as soon as it becomes available. In addition, MDPH/BEH is continuing to focus efforts on the most vulnerable beaches through its Tiered Monitoring Plan and sanitary surveys. Finally, the regulatory amendments reduced the number of posting days in 2015 and allowed for greater public access to bathing beaches during days of acceptable water quality.

# ACKNOWLEDGEMENTS

This study would not have been possible without the efforts, cooperation, and collaboration of the local health departments in Massachusetts (including Barnstable County). MDPH/BEH also received much assistance from many local, regional, and state organizations, including MDCR, in ensuring that bathing beaches are tested and that their data are forwarded to MDPH/BEH in a timely fashion. Finally, we are grateful to USEPA for providing financial support. USEPA BEACH Grant funds have helped support efforts in areas related to public marine beach data reporting and notification, including the public notification website, laboratory analysis of routine monitoring samples, training, and enforcement activities.

# REFERENCES

Barrell RA, Hunter PR, Nichols G, 2000.  Microbiological standards for water and their relationship to health risk.  Commun Dis Public Health.  March, Volume 3, pp. 8-13.

Boehm, A. B. and S.B. Weisberg, 2005. Tidal forcing of enterococci at marine recreational beaches at fortnightly and semi-diurnal frequencies. Environmental Science and Technology, 39(14), pp. 5575-5583.

Beaches Environmental Assessment and Coastal Health Act. Federal Register 2002 21 March, 67 (55) pp. 13140-13143.

Cabelli, VJ, Dufour, A, McCabe, L, and Levin, MA, 1982, Swimming-associated gastroenteritis and water quality, American Journal of Epidemiology, Volume 115 (4) pp. 606-616.

Cabelli, VJ, 1983, Health Effects Criteria for Marine Recreational Waters, USEPA Document Number USEPA-600/1-80-031, Health Effects Research Laboratory, Office of Research and Development, United States Environmental Protection Agency, Research Triangle Park, North Carolina.

Cabelli, VJ, 1989, Swimming-associated illness and recreational water quality criteria, Water Science Technology, Volume 21 (2) pp. 13-21.

California Department of Health Services, 1997. Draft Guidance for Saltwater Recreational Areas: Assessing Microbiological Contamination and Taking Corrective Action.

California Department of Health Services, 1997.  Draft Guidance for Freshwater Recreational Areas: Assessing Microbiological Contamination and Taking Corrective Action.

CDC, 1990, Waterborne disease outbreaks, 1986-1988, Morbidity and Mortality Weekly Report, Volume 39 (SS-1), Center for Disease Control and Prevention, pp. 1-13.

CDC, 1991, Waterborne disease outbreaks, 1989-1990, Morbidity and Mortality Weekly Report, Volume 40 (SS-3), Center for Disease Control and Prevention, pp. 1-21.

CDC, 1992, Cercarial dermatitis outbreak at a state park-Delaware, 1991, Morbidity and Mortality Weekly Report, April 10, 1992, Volume 41, Number 14, Center for Disease Control and Prevention, pp. 225-228.

CDC, 1993, Surveillance for waterborne disease outbreaks-United States, 1991-2, Morbidity and Mortality Weekly Report, November 19, 1993, Volume 42 (SS-5), Center for Disease Control and Prevention, pp. 1-22.

CDC, 1996, Surveillance for waterborne disease outbreaks-United States, 1993-4, Morbidity and Mortality Weekly Report, April 12, 1996, Volume 45 (SS-1), Center for Disease Control and Prevention, pp. 1-33.

CDC, 2002, Surveillance for waterborne disease outbreaks-United States, 1999-2000, Morbidity and Mortality Weekly Report, November 22, 2002, Volume 51 (SS-8), Center for Disease Control and Prevention, pp. 1-48.

CDC, 2004, Surveillance for waterborne disease outbreaks-United States, 2001-2002, Morbidity and Mortality Weekly Report, October 22, 2004, Volume 53 (SS-08), Center for Disease Control and Prevention, pp. 1-22.

Corbett, SJ, Rubin, GL, Curry, GK, and Kleinbaum, DG, 1993, The health effects of swimming at Sydney beaches, American Journal of Public Health, Volume 83 (12) pp. 1701-1706.

Coye, MJ, and Goldoft, MG, 1989, Microbiological contamination of the ocean and human health, New Jersey Medicine, Volume 86 (7) pp. 533-538.

Dufour, AP, 1984, Health Effects Criteria for Fresh Recreational Waters, USEPA Document Number USEPA-600/1-84-004.  Health Effects Research Laboratory, Office of Research and Development, USEPA, Research Triangle Park, NC.

Evaluation of New, Rapid Microbiological Measurement Methods For Recreational Water Quality. Southern California Coastal Water Research Project. 21 February 2006.

Federal Register, 2003. Guidelines Establishing Test Procedures for the Analysis of Pollutants; Analytical Methods for Biological Pollutants in Ambient Water.  Federal Register 2003 21 July, 68 (139) pp. 43272-43283.

Gerba, Charles P., 2000. Assessment of Enteric Pathogen Shedding by Bathers during Recreational Activity and its Impact on Water Quality. Quantitative Microbiology, March 2000 Volume 2 (1) pp. 55-68.

Haile R, 1996.  A Health Effect Study of Swimmers in Santa Monica Bay.  Santa Monica Bay Restoration Project, Monterey Park, CA.

Haugland, RA. “Using Today’s Data to Close the Beach Today. Quantitative Polymerase Chain Reaction (QPCR) rapid beach closings tool.” 2005 US EPA Office of Research and Development Product Expo. 8 February 2005.

Jagals, P.; Grabow W.O.K.; Griesel M.; Jagals C.; 2000. Evaluation of Selected Membrane Filtration and Most Probable Number Methods for the Enumeration of Faecal Coliforms, Escheria coli and Enterococci in Environmental Waters. Quantitative Microbiology, June 2000 Volume 2 (2) pp. 129-140.

Massachusetts Department of Environmental Protection (MDEP) and Office of Coastal Zone Management (MCZM).  March 1997.  Stormwater Management Volume Two:  Stormwater Technical Handbook.

Massachusetts Department of Public Health Regulations, 105 CMR § 445.000, Minimum Standards for Bathing Beaches (State Sanitary Code Chapter VII).

Moore JE, Caldwell PS, Millar BC, Murphy PG, 2001.  Occurrence of Campylobacter spp. in water in Northern Ireland: implications for public health.  Ulster Med J. Nov, Volume 70, pp. 102-7.

NAS, 1977, Drinking Water and Health, Safe Drinking Water Committee, National Academy of Sciences, Washington, D.C., 1977.

Polo, F, Figueras, MJ, Inza, I, Sala, J, Fleisher, JM, Guarro, J, 1998.  Relationship between presence of Salmonella and indicators of fecal pollution in aquatic habitats.  FEMS Microbiol Lett.  March 15, Volume 160, pp. 253-6.

Prieto, MD, Lopez, B, Juanes, JA, Revilla, JA, Llorca, J, Delgado-Rodriguez, M, 2001. Recreation in Coastal Waters: Health Risks Associated with Bathing in Sea Water. Journal of Epidemiology and Community Health, June 2001, Vol. 55, pp. 442-7.

Pruss, A, 1998.  Review of epidemiological studies on health effects from exposure to recreational water.  International Journal of Epidemiology.  Vol. 27, pp. 1-9.

Schindler, PR, 2001.  Hygiene of Bathing Waters.  Gesundheitswesen.  Vol. 63, Suppl. 2, pp. S142-50.

USEPA, 1985.  Test Methods for Escherichia coli and Enterococci in Water by the Membrane Filter Procedure, USEPA Document Number USEPA-600/4-85/076, Environmental Monitoring and Support Laboratory, U.S. Environmental Protection Agency, Cincinnati, Ohio.

USEPA, 1986.  Ambient Water Quality Criteria for Bacteria - 1986, USEPA Document Number USEPA-440/5-84-002, Office of Regulations and Standards, Criteria and Standards Division, U.S. Environmental Protection Agency, Washington, DC.

USEPA, 1997.  Method 1600: Membrane Filter Test Method for Enterococci in Water.  USEPA Document Number USEPA-821-R-97-004, Office of Water, U.S. Environmental Protection Agency, Washington D.C., May 1997.

USEPA, 2007. Criteria Development Plan and Schedule: Recreational Water Quality Criteria. USEPA Document Number USEPA-823-R-08-003, Office of Water, Office of Research and Development, U.S. Environmental Protection Agency, Washington D.C., August 2007.

USEPA, 2014. National Beach Guidance and Required Performance Criteria for Grants, 2014 Edition. USEPA Document Number EPA-823-B-14-001, Office of Water, U.S. Environmental Protection Agency, Washington D.C., July 2014.

Wade, TJ, Pai, N, Eisenberg, JNS, Colford, JMJ. Do U.S. Environmental Protection Agency Water Quality Guidelines for Recreational Waters Prevent Gastrointestinal Illness? A Systematic Review and Meta-analysis. Environmental Health Perspectives, June 2003, Vol. 111 (8) pp. 1102-1109.

Wade, TJ, Dufour, AP, Calderon, RL, Beach, MJ, Sams, EA, Brenner, KP. “Protecting Swimmer Health with Same Day Water Quality Monitoring Results for Bathing Beaches.” 2005 Great Lakes Beach Association Conference. Green Bay, WI. 3 November 2005.

Wade, TJ, Calderon, RL, Sams, E, Beach, M, Brenner, KP, Williams, AH, Dufour, AP, 2006. Rapidly Measured Indicators of Recreational Water Quality Are Predictive of Swimming-Associated Gastrointestinal Illness. Environmental Health Perspectives, January 2006, Vol. 114 (1) pp. 24-28.

# TABLES

| Table 1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| MA Beaches (2015): Breakdown of communities reporting water |  |  |  |  |
| quality data for marine and/or freshwater public and semi-public |  |  |  |  |
| bathing beaches. |  |  |  |  |
|  | Type of community | # | % |  |
|  | Marine beach only | 39 | 18% |  |
|  | Freshwater beach only | 160 | 73% |  |
|  | Marine and freshwater beaches | 21 | 9% |  |
|  | Total | 220 | 100% |  |

| Table 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| MA Beaches (2015): Marine vs. freshwater testing: number of beaches, communities, and samples. |  |  |  |  |  |
|  |  | Beaches tested1 | Communities | Samples collected |  |
|  | Marine | 587 | 60 | 8,050 |  |
|  | Freshwater | 590 | 181 | 7,882 |  |
|  | Total: | 1,177 | 220 2 | 15,932 |  |
|  | 1. Beaches include both public and semi-public bathing beaches. For the purposes of this report, a sample location is considered a single beach. Also, note that this table does not include the number of beaches not tested, as data were not compiled to accurately determine this number. |  |  |  |  |
|  | 2. Twenty-one of the 60 communities containing marine beaches are also counted among the 181 communities with freshwater beaches because they contain both types of beach: see Table 1. |  |  |  |  |

| Table 3 MA Beaches (2015): Bather density at marine and freshwater public and semi-public bathing beaches at times when samples were taken. |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Marine beaches |  |  |  |
|  | Bather Density (# people) | # Samples | % |  |
|  | 0-10 | 6,664 | 83% |  |
|  | 10-20 | 304 | 4% |  |
|  | 20-50 | 129 | 2% |  |
|  | >50 | 78 | 1% |  |
|  | Not indicated | 875 | 11% |  |
|  | Total | 8,050 | 100% |  |
|  | Freshwater beaches |  |  |  |
|  | Bather Density (# people) | # Samples | % |  |
|  | 0-10 | 6,679 | 85% |  |
|  | 10-20 | 273 | 3% |  |
|  | 20-50 | 102 | 1% |  |
|  | >50 | 60 | 1% |  |
|  | Not indicated | 768 | 10% |  |
|  | Total | 7,882 | 100% |  |

| Table 4 MA Beaches (2015): Frequency of water quality testing at public and semi-public bathing beaches, grouped by beach and frequency. |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Marine Beaches |  |  |  |
|  | Test frequency | # Beaches | % |  |
|  | Daily | 12 | 2% |  |
|  | Weekly | 473 | 81% |  |
|  | Monthly | 96 | 16% |  |
|  | Biweekly | 6 | 1% |  |
|  | Total | 587 | 100% |  |
|  | Freshwater Beaches |  |  |  |
|  | Test frequency | # Beaches | % |  |
|  | Weekly | 579 | 98.1% |  |
|  | Twice per week | 2 | 0.3% |  |
|  | Monthly | 1 | 0.2% |  |
|  | Biweekly | 1 | 0.2% |  |
|  | One time | 4 | 0.7% |  |
|  | Two times | 1 | 0.2% |  |
|  | Three times | 2 | 0.3% |  |
|  | Total | 590 | 100.0% |  |

| Table 5 MA Beaches (2015): Number of exceedances and postings at marine and freshwater public and semi-public bathing beaches. |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Marine beaches |  |  |  |
|  | Exceedances, Total (Enterococcus) | 390 |  |  |
|  | Postings, Total1 | 240 |  |  |
|  | Postings, Single Sample Exceedance | 154 |  |  |
|  | Postings, Geomean Exceedances | 30 |  |  |
|  | Postings, Preemptive - Rainfall | 48 |  |  |
|  | Postings, Preemptive - Other | 7 |  |  |
|  | Permanent Closures | 1 |  |  |
|  | Freshwater beaches |  |  |  |
|  | Exceedances, Total | 330 |  |  |
|  | Exceedances, Enterococcus | 127 |  |  |
|  | Exceedances, E. Coli | 203 |  |  |
|  | Postings, Total1 | 167 |  |  |
|  | Postings, Enterococcus | 89 |  |  |
|  | Postings, E. Coli | 63 |  |  |
|  | Postings, Preemptive - Rainfall | 1 |  |  |
|  | Postings, Preemptive - Other | 1 |  |  |
|  | Postings, Algae | 13 |  |  |
|  | 1. Total postings does not necessarily equal total exceedances. Some exceedances may have occurred while the beach was already posted. In addition, many beaches are no longer required to post after an initial exceedance if a next day resample shows acceptable water quality. |  |  |  |

| Table 6 MA Beaches (2015): Number of samples in which the measured enterococcus concentration (marine beaches) or enterococcus or E.coli concentration (freshwater beaches) exceeded the respective water quality criterion at public and semi-public bathing beaches. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Year | Marine Beaches |  |  | Freshwater Beaches |  |  |  |
|  |  | Exceedances1 | Total Samples Analyzed | % | Exceedances1 | Total Samples Analyzed | % |  |
|  | 2001 | 444 | 7200 | 6.2% | 336 | 5651 | 5.9% |  |
|  | 2002 | 185 | 6686 | 2.8% | 264 | 6473 | 4.1% |  |
|  | 2003 | 320 | 7439 | 4.3% | 333 | 6480 | 5.1% |  |
|  | 2004 | 337 | 7873 | 4.3% | 267 | 7313 | 3.7% |  |
|  | 2005 | 368 | 8064 | 4.6% | 286 | 7148 | 4.0% |  |
|  | 2006 | 405 | 8367 | 4.8% | 279 | 7438 | 3.8% |  |
|  | 2007 | 253 | 7693 | 3.3% | 236 | 7977 | 3.0% |  |
|  | 2008 | 433 | 7639 | 5.7% | 325 | 7834 | 4.1% |  |
|  | 2009 | 571 | 8119 | 7.0% | 222 | 7684 | 2.9% |  |
|  | 2010 | 490 | 7919 | 6.2% | 299 | 7490 | 4.0% |  |
|  | 2011 | 481 | 8140 | 5.9% | 434 | 7612 | 5.7% |  |
|  | 2012 | 343 | 8006 | 4.3% | 200 | 7513 | 2.7% |  |
|  | 2013 | 475 | 8132 | 5.8% | 280 | 7339 | 3.8% |  |
|  | 2014 | 329 | 7516 | 4.4% | 196 | 7358 | 2.7% |  |
|  | 2015 | 390 | 8,050 | 4.8% | 330 | 7882 | 4.2% |  |
|  | Average | 388 | 7790 | 5.0% | 286 | 7279 | 3.9% |  |
|  | 1. For marine beaches, enterococcus is the indicator species. A sample is said to be in exceedance if the number of colony forming units (CFU) / 100 ml is greater than 104. For freshwater beaches, either enterococcus or E. coli can be used as indicator species. For enterococcus, a sample is said to be in exceedance if the number of CFU / 100 ml is greater than 61. For E. coli, a sample is said to be in exceedance if the number of CFU / 100 ml is greater than 235. |  |  |  |  |  |  |  |

|  | Table 7 MA Beaches (2015): Number of beaches at which at least one enterococcus sample (marine beaches) or at least one enterococcus or E. coli sample (freshwater beaches) exceeded the respective water quality criteria. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | # beaches with at | Total # beaches | % | % in previous years |  |  |  |  |
|  | least one exceedance | reporting |  | 2014 | 2013 | 2012 | 2011 | 2010 |
| Marine | 154 | 587 | 26% | 26% | 36% | 28% | 35% | 36% |
| Freshwater | 158 | 590 | 27% | 24% | 18% | 38% | 27% | 20% |

|  | Table 8 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MA Beaches (2004-2015): Rainfall during swimming season - Boston* |  |  |  |  |  |  |  |
|  | Boston |  |  |  |  |  |  |
|  | Year | Rainfall | June | July | August | Total |  |
|  | n/a | Norm for month | 3.68 | 3.43 | 3.35 | 10.46 |  |
|  | 2004 | Total | 1.95 | 3.87 | 4.38 | 10.20 |  |
|  |  | Dev From Norm | -47% | +13% | +31% | -2% |  |
|  | 2005 | Total | 1.46 | 3.37 | 2.88 | 7.71 |  |
|  |  | Dev From Norm | -60% | -2% | -14% | -26% |  |
|  | 2006 | Total | 10.09 | 3.58 | 3.20 | 16.87 |  |
|  |  | Dev From Norm | +174% | +4% | -4% | +61% |  |
|  | 2007 | Total | 2.12 | 5.26 | 0.66 | 8.04 |  |
|  |  | Dev From Norm | -42% | +53% | -80% | -23% |  |
|  | 2008 | Total | 3.46 | 6.00 | 4.47 | 13.93 |  |
|  |  | Dev From Norm | -6% | +75% | +33% | +33% |  |
|  | 2009 | Total | 3.22 | 6.90 | 3.24 | 13.36 |  |
|  |  | Dev From Norm | -13% | +101% | -3% | +28% |  |
|  | 2010 | Total | 3.18 | 2.66 | 5.75 | 11.59 |  |
|  |  | Dev From Norm | -14% | -22% | +72% | +11% |  |
|  | 2011 | Total | 4.76 | 2.04 | 7.74 | 14.54 |  |
|  |  | Dev From Norm | +29% | -41% | +131% | +39% |  |
|  | 2012 | Total | 4.71 | 3.88 | 3.08 | 11.67 |  |
|  |  | Dev From Norm | +28% | +13% | -8% | +12% |  |
|  | 2013 | Total | 10.50 | 3.61 | 1.84 | 15.95 |  |
|  |  | Dev From Norm | +185% | +5% | -45% | +52% |  |
|  | 2014 | Total | 2.62 | 4.57 | 1.75 | 8.94 |  |
|  |  | Dev From Norm | -29% | +33% | -48% | -15% |  |
|  | 2015 | Total | 5.01 | 2.09 | 2.19 | 9.29 |  |
|  |  | Dev From Norm | +36% | -39% | -35% | -11% |  |
|  | * obtained from the National Weather Service Forecast office, at |  |  |  |  |  |  |
|  | http://w2.weather.gov/climate/xmacis.php?wfo=box |  |  |  |  |  |  |

|  | Table 9 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MA Beaches (2004-2015): Rainfall during swimming season - Chatham* |  |  |  |  |  |  |  |
|  | Chatham |  |  |  |  |  |  |
|  | Year | Rainfall | June | July | August | Total |  |
|  | n/a | Norm for month | 3.44 | 3.38 | 3.33 | 10.15 |  |
|  | 2004 | Total | 1.60 | 2.48 | 5.49 | 9.57 |  |
|  |  | Dev From Norm | -53% | -27% | +65% | -6% |  |
|  | 2005 | Total | 1.61 | 3.37 | 2.99 | 7.97 |  |
|  |  | Dev From Norm | -53% | <1% | -10% | -21% |  |
|  | 2006 | Total | 9.49 | 2.97 | 2.61 | 15.07 |  |
|  |  | Dev From Norm | +176% | -12% | -22% | +48% |  |
|  | 2007 | Total | 1.38 | 2.80 | 0.35 | 4.53 |  |
|  |  | Dev From Norm | -60% | -17% | -89% | -55% |  |
|  | 2008 | Total | 1.78 | 2.85 | 1.92 | 6.55 |  |
|  |  | Dev From Norm | -48% | -16% | -42% | -35% |  |
|  | 2009 | Total | 3.55 | 6.13 | 4.14 | 13.82 |  |
|  |  | Dev From Norm | +3% | +81% | +24% | +36% |  |
|  | 2010 | Total | 1.74 | 1.78 | 4.43 | 7.95 |  |
|  |  | Dev From Norm | -49% | -47% | +33% | -22% |  |
|  | 2011 | Total | 3.61 | 3.76 | 3.51 | 10.88 |  |
|  |  | Dev From Norm | +5% | +11% | +5% | +7% |  |
|  | 2012 | Total | 1.85 | 1.30 | 1.80 | 4.95 |  |
|  |  | Dev From Norm | -46% | -62% | -46% | -51% |  |
|  | 2013 | Total | 6.94 | 4.08 | 1.44 | 12.46 |  |
|  |  | Dev From Norm | +102% | +21% | -57% | +23% |  |
|  | 2014 | Total | 2.75 | 3.72 | 3.25 | 9.72 |  |
|  |  | Dev From Norm | -20% | +10% | -2% | -4% |  |
|  | 2015 | Total | 2.76 | 2.39 | 5.21 | 10.36 |  |
|  |  | Dev From Norm | -20% | -29% | +56% | +2% |  |
|  | * obtained from the National Weather Service Forecast office, at |  |  |  |  |  |  |
|  | http://w2.weather.gov/climate/xmacis.php?wfo=box |  |  |  |  |  |  |

|  | Table 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MA Beaches (2004-2015): Rainfall during swimming season - Amherst* |  |  |  |  |  |  |  |
|  | Amherst |  |  |  |  |  |  |
|  | Year | Rainfall | June | July | August | Total |  |
|  | n/a | Norm for month | 3.81 | 3.95 | 4.1 | 11.86 |  |
|  | 2004 | Total | 2.91 | 3.89 | 3.77 | 10.57 |  |
|  |  | Dev From Norm | -24% | -2% | -8% | -11% |  |
|  | 2005 | Total | 4.42 | 2.41 | 2.81 | 9.64 |  |
|  |  | Dev From Norm | +16% | -39% | -31% | -19% |  |
|  | 2006 | Total | 6.39 | 2.83 | 3.31 | 12.53 |  |
|  |  | Dev From Norm | +68% | -28% | -19% | +6% |  |
|  | 2007 | Total | 2.59 | 5.50 | 1.12 | 9.21 |  |
|  |  | Dev From Norm | -32% | +39% | -73% | -22% |  |
|  | 2008 | Total | 6.92 | 8.20 | 2.37 | 17.49 |  |
|  |  | Dev From Norm | +82% | +108% | -42% | +47% |  |
|  | 2009 | Total | 5.38 | 9.03 | 3.52 | 17.93 |  |
|  |  | Dev From Norm | +41% | +129% | -14% | +51% |  |
|  | 2010 | Total | 3.69 | 2.98 | 2.33 | 9.00 |  |
|  |  | Dev From Norm | -3% | -25% | -43% | -24% |  |
|  | 2011 | Total | 5.84 | 1.81 | 10.05 | 17.70 |  |
|  |  | Dev From Norm | +53% | -54% | +145% | +49% |  |
|  | 2012 | Total | 4.14 | 2.45 | 5.80 | 12.39 |  |
|  |  | Dev From Norm | +9% | -38% | +41% | +4% |  |
|  | 2013 | Total | 9.44 | 4.94 | 3.51 | 17.89 |  |
|  |  | Dev From Norm | +148% | +25% | -14% | +51% |  |
|  | 2014 | Total | 2.83 | 6.94 | 4.65 | 14.42 |  |
|  |  | Dev From Norm | -26% | +76% | +13% | +22% |  |
|  | 2015 | Total | 8.10 | 4.02 | 4.37 | 16.49 |  |
|  |  | Dev From Norm | +113% | +2% | +7% | +39% |  |
|  | * Data obtained from the National Climatic Data Center's |  |  |  |  |  |  |
|  | Preliminary Record of Climatological Observations, at |  |  |  |  |  |  |
|  | http://www.ncdc.noaa.gov/IPS/coop/coop.html |  |  |  |  |  |  |

|  | Table 11 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MA Beaches (2004-2015): Rainfall during swimming season - Ashburnham* |  |  |  |  |  |  |  |
|  | Ashburnham |  |  |  |  |  |  |
|  | Year | Rainfall | June | July | August | Total |  |
|  | n/a | Norm for month | 4.06 | 4.05 | 4.28 | 12.39 |  |
|  | 2004 | Total | 1.93 | 2.90 | 5.15 | 9.98 |  |
|  |  | Dev From Norm | -52% | -28% | +20% | -19% |  |
|  | 2005 | Total | 4.43 | 5.07 | 3.37 | 12.87 |  |
|  |  | Dev From Norm | +9% | +25% | -21% | +4% |  |
|  | 2006 | Total | 8.54 | 3.55 | 4.50 | 16.59 |  |
|  |  | Dev From Norm | +110% | -12% | +5% | +34% |  |
|  | 2007 | Total | 3.76 | 6.23 | 1.32 | 11.31 |  |
|  |  | Dev From Norm | -7% | +54% | -69% | -9% |  |
|  | 2008 | Total | 4.14 | 7.80 | 3.90 | 15.84 |  |
|  |  | Dev From Norm | +2% | +93% | -9% | +28% |  |
|  | 2009 | Total | 7.20 | 7.03 | 3.66 | 17.89 |  |
|  |  | Dev From Norm | +77% | +74% | -14% | +44% |  |
|  | 2010 | Total | 3.19 | 4.97 | 3.43 | 11.59 |  |
|  |  | Dev From Norm | -21% | +23% | -20% | -6% |  |
|  | 2011 | Total | 5.64 | 1.55 | 10.23 | 17.42 |  |
|  |  | Dev From Norm | +48% | -61% | +150% | +41% |  |
|  | 2012 | Total | 3.75 | 1.39 | 5.02 | 10.16 |  |
|  |  | Dev From Norm | -8% | -66% | +17% | -18% |  |
|  | 2013 | Total | 8.32 | 3.49 | 4.70 | 16.51 |  |
|  |  | Dev From Norm | +118% | -12% | +15% | +33% |  |
|  | 2014 | Total | 1.88 | 7.77 | 2.71 | 12.36 |  |
|  |  | Dev From Norm | -51% | +97% | -34% | 0% |  |
|  | 2015 | Total | 6.99 | 3.03 | 1.88 | 11.90 |  |
|  |  | Dev From Norm | +83% | -23% | -54% | -4% |  |
|  | * Data obtained from the National Climatic Data Center's |  |  |  |  |  |  |
|  | Preliminary Record of Climatological Observations, at |  |  |  |  |  |  |
|  | http://www.ncdc.noaa.gov/IPS/coop/coop.html |  |  |  |  |  |  |

| Table 12 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| MA Beaches (2015): Number of exceedances at public and semi-public beaches which reported environmental sources of pollution. |  |  |  |  |  |  |
| Pollution Source?1 | Marine beaches |  |  | Freshwater beaches |  |  |
|  | # Exceedances | # Samples | % | # Exceedances | # Samples | % |
| Yes | 79 | 1,132 | 7.0% | 64 | 689 | 9.3% |
| Unknown | 245 | 3,661 | 6.7% | 201 | 4,387 | 4.6% |
| No | 66 | 3,257 | 2.0% | 65 | 2,806 | 2.3% |
| Total | 390 | 8,050 | 4.8% | 330 | 7,882 | 4.2% |
| 1 "Yes" indicates that a source was observed; "unknown" means that no information was recorded; "no" indicates that the field forms explicitly record an absence of pollution sources. |  |  |  |  |  |  |

| Community | Beach Name1 | Sample Location | Testing Frequency | # Tests | # Single Sample Exceedances | Minimum Exceedance | Maximum Exceedance | # Postings2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Aquinnah | Lobsterville | Sampling Point | Monthly | 4 |  |  |  |  |
| Aquinnah | Moshup Beach | Sampling Point | Monthly | 4 |  |  |  |  |
| Aquinnah | Philbin Beach | Sampling Point | Monthly | 4 |  |  |  |  |
| Aquinnah | Red Beach | Sampling Point | Monthly | 4 |  |  |  |  |
| Barnstable | Barnstable Yacht Club | Sampling Point | Weekly | 13 |  |  |  |  |
| Barnstable | Cotuit Bay Shores Association | Sampling Point | Weekly | 13 |  |  |  |  |
| Barnstable | Covell's | Sampling Point | Weekly | 16 |  |  |  |  |
| Barnstable | Craigville | Sampling Point | Weekly | 16 |  |  |  |  |
| Barnstable | Craigville Beach Club | Sampling Point | Weekly | 14 |  |  |  |  |
| Barnstable | Cross Street | Sampling Point | Weekly | 13 |  |  |  |  |
| Barnstable | Dowses | Sampling Point | Weekly | 16 |  |  |  |  |
| Barnstable | Eugenia Fortes | Sampling Point | Weekly | 15 | 2 | 110 | 228 |  |
| Barnstable | Hyannis Yacht Club | Sampling Point | Weekly | 13 |  |  |  |  |
| Barnstable | Kalmus Ocean | Sampling Point | Weekly | 16 |  |  |  |  |
| Barnstable | Kalmus Yacht | Sampling Point | Weekly | 13 |  |  |  |  |
| Barnstable | Kennedy Memorial/Veterans Beach | Sampling Point | Weekly | 16 |  |  |  |  |
| Barnstable | Keyes Beach | Sampling Point | Weekly | 18 | 2 | 194 | 340 |  |
| Barnstable | Loops | Sampling Point | Weekly | 13 |  |  |  |  |
| Barnstable | Millway | Sampling Point | Weekly | 13 |  |  |  |  |
| Barnstable | Oyster Harbors Club | Sampling Point | Weekly | 14 |  |  |  |  |
| Barnstable | Ropes | Sampling Point | Weekly | 14 | 1 | 150 | 150 | 1 |
| Barnstable | Sandy Neck | Sampling Point | Weekly | 16 |  |  |  |  |
| Barnstable | Seaside Park Improvement Association | Sampling Point | Weekly | 14 |  |  |  |  |
| Barnstable | Wianno Club (Salt-107 Seaview) | Sampling Point | Weekly | 14 |  |  |  |  |
| Beverly | Brackenbury | Sampling Point | Weekly | 13 | 1 | 187 | 187 |  |
| Beverly | Dane Street | Bathhouse | Weekly | 18 | 3 | 158 | 199 | 2 |
| Beverly | Goat Hill | Sampling Point | Weekly | 13 | 1 | 187 | 187 |  |
| Beverly | Independence Park | Sampling Point | Weekly | 17 | 3 | 189 | 213 | 1 |
| Beverly | Lynch Park | Sampling Point | Weekly | 12 |  |  |  |  |
| Beverly | Mingo | Sampling Point | Weekly | 12 |  |  |  |  |
| Beverly | Obear Park | Sampling Point | Weekly | 13 | 1 | 213 | 213 |  |
| Beverly | Rice's | Sampling Point | Weekly | 12 |  |  |  |  |
| Beverly | Sandy Point | Sampling Point | Weekly | 12 |  |  |  |  |
| Beverly | West | Sampling Point | Weekly | 12 |  |  |  |  |
| Beverly | Woodbury | Sampling Point | Weekly | 17 | 3 | 135 | 305 | 3 |
| Boston | Carson Beach (DCR) | at Bathhouse | Weekly | 17 |  |  |  |  |
| Boston | Carson Beach (DCR) | at I St. | Weekly | 17 | 1 | 111 | 111 |  |
| Boston | City Point Beach (DCR) | WWII Memorial | Weekly | 16 |  |  |  |  |
| Boston | Constitution (DCR) | Middle | Daily | 90 | 10 | 109 | 2282 | 11 |
| Boston | Constitution (DCR) | North site | Daily | 90 | 9 | 129 | 906 | 11 |
| Boston | Constitution (DCR) | Rec Center | Daily | 90 | 10 | 121 | 2310 | 11 |
| Boston | Lovell's Island (DCR) | Sampling Point | Weekly | 10 |  |  |  |  |
| Boston | M Street Beach (DCR) | Sampling Point | Weekly | 16 |  |  |  |  |
| Boston | Malibu (DCR) | Sampling Point | Daily | 90 | 12 | 109 | 2760 | 7 |
| Boston | Pleasure Bay (DCR) | Broadway | Weekly | 16 |  |  |  |  |
| Boston | Pleasure Bay (DCR) | Castle Island Playground | Weekly | 16 |  |  |  |  |
| Boston | Pleasure Bay (DCR) | South Flagpole | Weekly | 16 |  |  |  |  |
| Boston | Savin Hill (DCR) | Sampling Point | Weekly | 17 | 1 | 2600 | 2600 | 1 |
| Boston | Spectacle Island (DCR) | Sampling Point | Weekly | 10 |  |  |  |  |
| Boston | Tenean (DCR) | Sampling Point | Daily | 91 | 26 | 108 | 712 | 27 |
| Bourne | Barlows Landing | Sampling Point | Weekly | 14 |  |  |  |  |
| Bourne | Cataumet Harbor | Sampling Point | Weekly | 16 | 2 | 116 | 150 |  |
| Bourne | Cedar Point Association | Sampling Point | Weekly | 15 | 1 | 140 | 140 |  |
| Bourne | Electric Avenue | Sampling Point | Monthly | 4 |  |  |  |  |
| Bourne | Gilder Road Beach | Sampling Point | Monthly | 4 |  |  |  |  |
| Bourne | Hideaway Village Association | Sampling Point | Weekly | 14 |  |  |  |  |
| Bourne | Monument | Sampling Point | Monthly | 4 |  |  |  |  |
| Bourne | Patuisset Beach | Sampling Point | Weekly | 13 | 1 | 108 | 108 |  |
| Bourne | Pocasset Beach Improvement Association | Sampling Point | Weekly | 12 |  |  |  |  |
| Bourne | Sagamore | Sampling Point | Monthly | 4 |  |  |  |  |
| Bourne | Scraggy Neck Recreation Association | Sampling Point | Weekly | 9 |  |  |  |  |
| Bourne | Tahanto Associates, Inc. | Sampling Point | Weekly | 13 |  |  |  |  |
| Bourne | Wings Neck Trust Association (North Beach) | Sampling Point | Weekly | 15 | 1 | 582 | 582 |  |
| Bourne | Wings Neck Trust Association (South Beach) | Sampling Point | Weekly | 14 |  |  |  |  |
| Braintree | Smith Beach | Dock | Weekly | 14 | 2 | 171 | 538 | 3 |
| Brewster | Breakwater Landing | Sampling Point | Weekly | 16 |  |  |  |  |
| Brewster | Brewster Dunes | Sampling Point | Weekly | 13 |  |  |  |  |
| Brewster | Cape Cod Sea Camps (Bay) | Sampling Point | Weekly | 13 |  |  |  |  |
| Brewster | Crosby Landing | Sampling Point | Weekly | 17 | 1 | 400 | 400 | 1 |
| Brewster | Ellis Landing | Sampling Point | Weekly | 16 |  |  |  |  |
| Brewster | Ellis Landing Park Condominiums | Sampling Point | Weekly | 15 | 1 | 400 | 400 |  |
| Brewster | Halliday Acres | Sampling Point | Weekly | 10 |  |  |  |  |
| Brewster | Linnell Landing | Sampling Point | Weekly | 17 | 1 | 400 | 400 |  |
| Brewster | Mants | Sampling Point | Weekly | 16 |  |  |  |  |
| Brewster | Ocean Edge | Condos | Weekly | 15 | 1 | 246 | 246 |  |
| Brewster | Paines Creek | Sampling Point | Weekly | 16 |  |  |  |  |
| Brewster | Pilgrim Pine Acres | Sampling Point | Weekly | 10 |  |  |  |  |
| Brewster | Point of Rocks | Sampling Point | Weekly | 17 | 1 | 106 | 106 |  |
| Brewster | Saints Landing | Sampling Point | Weekly | 16 |  |  |  |  |
| Brewster | Sea Pines Condominiums | Sampling Point | Weekly | 14 |  |  |  |  |
| Brewster | Sunset Beach Association | Sampling Point | Weekly | 9 |  |  |  |  |
| Chatham | Andrew Harding Lane Beach | Sampling Point | Weekly | 12 |  |  |  |  |
| Chatham | Bucks Creek | Sampling Point | Weekly | 14 | 3 | 122 | 400 | 2 |
| Chatham | Chatham Bars Inn | Sampling Point | Weekly | 12 |  |  |  |  |
| Chatham | Cockle Cove | Sampling Point | Weekly | 12 |  |  |  |  |
| Chatham | Forest Street Beach | Sampling Point | Weekly | 13 | 1 | 110 | 110 |  |
| Chatham | Hardings East | East parking lot | Weekly | 12 |  |  |  |  |
| Chatham | Hardings West | West parking lot | Weekly | 12 |  |  |  |  |
| Chatham | Hawthorne Motel | Sampling Point | Weekly | 12 |  |  |  |  |
| Chatham | Jacknife Harbor | Sampling Point | Weekly | 12 |  |  |  |  |
| Chatham | Lighthouse | Sampling Point | Weekly | 12 |  |  |  |  |
| Chatham | Oyster Pond | Sampling Point | Weekly | 12 |  |  |  |  |
| Chatham | Pleasant Street | Sampling Point | Weekly | 14 | 2 | 106 | 110 |  |
| Chatham | Ridgevale | Sampling Point | Weekly | 12 |  |  |  |  |
| Chilmark | Great Rock Bight | Sampling Point | Weekly | 11 | 1 | 213 | 213 | 1 |
| Chilmark | Menemsha | Sampling Point | Monthly | 3 |  |  |  |  |
| Chilmark | Ocean @ Chilmark Pond Preserve | Sampling Point | Monthly | 7 |  |  |  |  |
| Chilmark | Ocean @ Lucy Vincent Beach | Sampling Point | Monthly | 5 |  |  |  |  |
| Chilmark | Ocean @ Squibnocket Beach | Sampling Point | Monthly | 4 |  |  |  |  |
| Cohasset | Black Rock | Sampling Point | Weekly | 14 | 1 | 350 | 350 |  |
| Cohasset | Sandy Beach | Sampling Point - Cohasset | Weekly | 14 |  |  |  |  |
| Danvers | Sandy Beach | West - Danvers | Weekly | 19 | 2 | 359 | 364 | 1 |
| Dartmouth | Anthony Beach | Sampling Point | Weekly | 12 |  |  |  |  |
| Dartmouth | Apponagansett Town Beach | Sampling Point | Weekly | 12 | 1 | 268 | 268 | 1 |
| Dartmouth | Bayview | Sampling Point | Weekly | 13 | 1 | 284 | 284 | 1 |
| Dartmouth | Demarest Lloyd (DCR) | Sampling Point | Weekly | 17 | 1 | 142 | 142 |  |
| Dartmouth | Hidden Bay | Sampling Point | Weekly | 12 | 1 | 500 | 500 | 1 |
| Dartmouth | Jones Town Beach North | North | Biweekly | 6 |  |  |  |  |
| Dartmouth | Jones Town Beach South | South | Weekly | 13 | 1 | 500 | 500 |  |
| Dartmouth | Moses Smith Creek | Sampling Point | Weekly | 13 | 1 | 500 | 500 |  |
| Dartmouth | Nonquitt | Sampling Point | Weekly | 12 |  |  |  |  |
| Dartmouth | Oak Hill Shores | Sampling Point | Weekly | 12 |  |  |  |  |
| Dartmouth | Round Hill | Sampling Point | Weekly | 12 |  |  |  |  |
| Dartmouth | Salter's Point East | Sampling Point | Weekly | 12 |  |  |  |  |
| Dartmouth | Salter's Point South | Sampling Point | Weekly | 12 |  |  |  |  |
| Dennis | Bayview | Sampling Point | Weekly | 15 | 1 | 166 | 166 |  |
| Dennis | Chapin Memorial | Sampling Point | Weekly | 14 |  |  |  |  |
| Dennis | Clipper Lane | Sampling Point | Weekly | 15 | 1 | 400 | 400 | 1 |
| Dennis | Cold Storage | Sampling Point | Weekly | 16 | 2 | 238 | 306 | 1 |
| Dennis | Corporation | Sampling Point | Weekly | 16 |  |  |  |  |
| Dennis | Follins Pond | Sampling Point 2 | Weekly | 16 | 2 | 112 | 394 |  |
| Dennis | Glendon Road - East | Sampling Point | Weekly | 14 |  |  |  |  |
| Dennis | Glendon Road - West | Sampling Point | Weekly | 14 |  |  |  |  |
| Dennis | Haigis | Sampling Point | Weekly | 14 |  |  |  |  |
| Dennis | Harborview | Sampling Point | Weekly | 14 |  |  |  |  |
| Dennis | Howes Street | Sampling Point | Weekly | 14 |  |  |  |  |
| Dennis | Inman Road | Sampling Point | Weekly | 14 |  |  |  |  |
| Dennis | Mayflower | Sampling Point | Weekly | 16 |  |  |  |  |
| Dennis | Raycroft | Sampling Point | Weekly | 14 |  |  |  |  |
| Dennis | Sea Street (Dennisport) | Dennisport | Weekly | 14 |  |  |  |  |
| Dennis | Sea Street (East Dennis) | East | Weekly | 14 |  |  |  |  |
| Dennis | South Village | Sampling Point | Weekly | 14 |  |  |  |  |
| Dennis | Sullivan (Depot St.) | Sampling Point | Weekly | 14 |  |  |  |  |
| Dennis | Trotting Park | Sampling Point | Weekly | 14 |  |  |  |  |
| Dennis | West Dennis - Residential | Residential | Weekly | 16 |  |  |  |  |
| Dennis | West Dennis - West | West | Weekly | 16 |  |  |  |  |
| Dennis | West Dennis - West of snack bar | West of snack bar | Weekly | 16 |  |  |  |  |
| Duxbury | Duxbury Beach @ Bath House | Sampling Point | Weekly | 15 |  |  |  |  |
| Duxbury | Landing Road | Sampling Point | Weekly | 20 | 3 | 146 | 1400 | 3 |
| Duxbury | Residents Beach (Duxbury Beach) | Sampling Point | Weekly | 15 |  |  |  |  |
| Duxbury | Shipyard Lane | Sampling Point | Weekly | 15 |  |  |  |  |
| Duxbury | West End | Sampling Point | Weekly | 15 |  |  |  |  |
| Eastham | Boat Meadow | Sampling Point | Weekly | 14 |  |  |  |  |
| Eastham | Campground | Sampling Point | Weekly | 15 | 1 | 140 | 140 |  |
| Eastham | Coast Guard (National Seashore) | Sampling Point 1 | Weekly | 11 |  |  |  |  |
| Eastham | Cole Road | Sampling Point | Weekly | 14 |  |  |  |  |
| Eastham | Cook's Brook | Sampling Point | Weekly | 14 |  |  |  |  |
| Eastham | Dyer Prince | Sampling Point | Weekly | 14 |  |  |  |  |
| Eastham | First Encounter - Spit River | Spit River | Weekly | 14 |  |  |  |  |
| Eastham | First Encounter Beach | Beach | Weekly | 14 |  |  |  |  |
| Eastham | Kingsbury | Sampling Point | Weekly | 14 |  |  |  |  |
| Eastham | Nauset Light (National Seashore) | Sampling Point 1 | Weekly | 11 |  |  |  |  |
| Eastham | S. Sunken Meadow | Sampling Point | Weekly | 14 |  |  |  |  |
| Eastham | Salt Pond | Sampling Point | Weekly | 15 | 1 | 136 | 136 | 1 |
| Eastham | Silver Springs Association | Sampling Point | Weekly | 14 |  |  |  |  |
| Eastham | Thumpertown | Sampling Point | Weekly | 15 | 1 | 304 | 304 |  |
| Eastham | Town Cove | Sampling Point | Weekly | 14 |  |  |  |  |
| Edgartown | Bend in the Road | Sampling Point | Monthly | 3 |  |  |  |  |
| Edgartown | Chappy Beach Club | Sampling Point | Monthly | 3 |  |  |  |  |
| Edgartown | Chappy Point Beach | Sampling Point | Monthly | 4 |  |  |  |  |
| Edgartown | East Beach (Chappy) | Sampling Point | Monthly | 3 |  |  |  |  |
| Edgartown | Fuller Street | Sampling Point | Monthly | 3 |  |  |  |  |
| Edgartown | Joseph Sylvia State Beach | big Bridge | Monthly | 3 |  |  |  |  |
| Edgartown | Norton Point East | East ocean | Monthly | 3 |  |  |  |  |
| Edgartown | Ocean @ Edgartown Great Pond | Sampling Point | Monthly | 4 |  |  |  |  |
| Edgartown | South Beach State Park - East | East | Monthly | 3 |  |  |  |  |
| Edgartown | South Beach State Park - Middle | Middle | Monthly | 3 |  |  |  |  |
| Edgartown | South Beach State Park - West | west | Monthly | 3 |  |  |  |  |
| Edgartown | Wasque Swim Beach | Sampling Point | Monthly | 3 |  |  |  |  |
| Essex | Clammer's Beach | Sampling Point | Monthly | 5 |  |  |  |  |
| Essex | Front Beach | Sampling Point | Monthly | 5 |  |  |  |  |
| Fairhaven | Fort Phoenix - Town Beach | Sampling Point | Weekly | 12 |  |  |  |  |
| Fairhaven | Fort Phoenix (DCR) | Sampling Point | Weekly | 17 | 1 | 188 | 188 |  |
| Fairhaven | Knollmere | Sampling Point | Weekly | 12 | 1 | 206 | 206 |  |
| Fairhaven | Manhattan Avenue | Sampling Point | Weekly | 13 | 1 | 182 | 182 |  |
| Fairhaven | Sandy Beach (Raymond Street) | Sampling Point | Weekly | 13 | 1 | 340 | 340 |  |
| Fairhaven | Seaview | Sampling Point | Weekly | 12 |  |  |  |  |
| Fairhaven | West Island Causeway | Sampling Point | Weekly | 12 |  |  |  |  |
| Fairhaven | West Island Town Beach | Sampling Point | Weekly | 12 |  |  |  |  |
| Falmouth | Acapesket Improvement Association | Sampling Point | Weekly | 14 |  |  |  |  |
| Falmouth | Bayshore Homeowners Association | Sampling Point | Weekly | 14 |  |  |  |  |
| Falmouth | Bikepath Beach (Trunk River) - East | East | Weekly | 14 |  |  |  |  |
| Falmouth | Bikepath Beach (Trunk River) - West | West | Weekly | 14 |  |  |  |  |
| Falmouth | Bowerman Beach Club | Sampling Point | Weekly | 14 |  |  |  |  |
| Falmouth | Bristol - East | East | Weekly | 14 |  |  |  |  |
| Falmouth | Bristol - West | West | Weekly | 14 |  |  |  |  |
| Falmouth | Chapoquoit | Sampling Point | Weekly | 14 |  |  |  |  |
| Falmouth | Chapoquoit Associates - Front Beach | Sampling Point | Weekly | 14 |  |  |  |  |
| Falmouth | Chapoquoit Associates - Little Beach | Sampling Point | Weekly | 14 |  |  |  |  |
| Falmouth | Falmouth Associates - 564 Surf Drive | Sampling Point | Weekly | 14 |  |  |  |  |
| Falmouth | Falmouth Heights East | East | Weekly | 14 |  |  |  |  |
| Falmouth | Falmouth Heights West | West | Weekly | 14 |  |  |  |  |
| Falmouth | Falmouth Yacht Club | Sampling Point | Weekly | 12 |  |  |  |  |
| Falmouth | Jetty Lane | Sampling Point | Weekly | 12 |  |  |  |  |
| Falmouth | Little Island Beach Preserve | Sampling Point | Weekly | 11 |  |  |  |  |
| Falmouth | Megansett | Sampling Point | Weekly | 14 |  |  |  |  |
| Falmouth | Megansett Yacht Club | Sampling Point | Weekly | 13 |  |  |  |  |
| Falmouth | Menauhant East | East | Weekly | 14 |  |  |  |  |
| Falmouth | Menauhant West | West | Weekly | 14 |  |  |  |  |
| Falmouth | Mill Road | Sampling Point | Weekly | 14 |  |  |  |  |
| Falmouth | New Silver (Silver Beach Improvement Association) | Sampling Point | Weekly | 12 |  |  |  |  |
| Falmouth | Nobska Beach Association | Sampling Point | Weekly | 14 |  |  |  |  |
| Falmouth | Old Silver (Public) - North | North | Weekly | 14 |  |  |  |  |
| Falmouth | Old Silver (Public) - South | South | Weekly | 15 | 1 | 156 | 156 |  |
| Falmouth | Old Silver 1 (Residents) | Central | Weekly | 14 |  |  |  |  |
| Falmouth | Old Silver Beach Estates Assoc. | Sampling Point | Weekly | 12 |  |  |  |  |
| Falmouth | Quisset Beach Association | Sampling Point | Weekly | 12 |  |  |  |  |
| Falmouth | Racing Beach Association | Whittemore Rd. | Weekly | 12 |  |  |  |  |
| Falmouth | Saconessett Hills Association | Sampling Point | Weekly | 14 |  |  |  |  |
| Falmouth | Seacoast Shores Associates, Inc. | Sampling Point | Weekly | 11 |  |  |  |  |
| Falmouth | Seacrest Resort | Sampling Point | Weekly | 14 |  |  |  |  |
| Falmouth | Sippewissett Highlands Trust | Sampling Point | Weekly | 14 |  |  |  |  |
| Falmouth | Stoney Beach (MBL) | Sampling Point | Weekly | 14 |  |  |  |  |
| Falmouth | Surf Drive - East | Surf Drive East | Weekly | 14 |  |  |  |  |
| Falmouth | Surf Drive - Pool | Surf Drive Pool | Weekly | 14 |  |  |  |  |
| Falmouth | Surf Drive - West | Main (concession) West | Weekly | 14 |  |  |  |  |
| Falmouth | Tides Hotel | Sampling Point | Weekly | 14 |  |  |  |  |
| Falmouth | Wild Harbour Estates | Sampling Point | Weekly | 14 |  |  |  |  |
| Falmouth | Wood Neck Beach | Sampling Point | Weekly | 14 |  |  |  |  |
| Falmouth | Wood Neck River | Sampling Point | Weekly | 14 |  |  |  |  |
| Gloucester | Cressy's | Sampling Point | Monthly | 5 |  |  |  |  |
| Gloucester | Good Harbor | Sampling Point | Weekly | 17 |  |  |  |  |
| Gloucester | Good Harbor Creek | Sampling Point | Weekly | 17 |  |  |  |  |
| Gloucester | Half Moon | Sampling Point | Monthly | 5 |  |  |  |  |
| Gloucester | Niles | Sampling Point | Monthly | 5 |  |  |  |  |
| Gloucester | Pavilion Beach | Sampling Point | Monthly | 5 |  |  |  |  |
| Gloucester | Plum Cove | Sampling Point | Weekly | 17 |  |  |  |  |
| Gloucester | Wingearsheek | Sampling Point | Weekly | 17 |  |  |  |  |
| Harwich | Atlantic Avenue | Sampling Point | Weekly | 9 | 1 | 136 | 136 |  |
| Harwich | Bank Street - Bayview Rd | Sampling Point | Monthly | 4 |  |  |  |  |
| Harwich | Brooks Road | Sampling Point | Monthly | 4 |  |  |  |  |
| Harwich | Earle Road | Sampling Point | Monthly | 4 |  |  |  |  |
| Harwich | Grey Neck | Sampling Point | Monthly | 4 |  |  |  |  |
| Harwich | Merkel Beach (Snow Inn Road) | Sampling Point | Monthly | 4 |  |  |  |  |
| Harwich | Neel Road | Sampling Point | Monthly | 4 |  |  |  |  |
| Harwich | Old Mill Point Association | Strandway-right of jetty | Weekly | 14 | 1 | 170 | 170 |  |
| Harwich | Pleasant Bay | Sampling Point | Monthly | 5 |  |  |  |  |
| Harwich | Pleasant Road | Sampling Point | Monthly | 4 |  |  |  |  |
| Harwich | Red River East | East | Monthly | 4 |  |  |  |  |
| Harwich | Red River Middle | Middle | Weekly | 14 |  |  |  |  |
| Harwich | Red River West | West | Monthly | 4 |  |  |  |  |
| Harwich | Sea Breeze Avenue | Sampling Point | Monthly | 4 |  |  |  |  |
| Harwich | Stone Horse Yacht Club | Sampling Point | Weekly | 12 | 1 | 400 | 400 |  |
| Harwich | The Belmont | Sampling Point | Weekly | 14 |  |  |  |  |
| Harwich | Wah Wah Taysee Road | Sampling Point | Monthly | 4 |  |  |  |  |
| Harwich | Wequasett Inn Resort | Sampling Point | Weekly | 14 |  |  |  |  |
| Harwich | Wychmere Harbor Beach Club | Sampling Point | Weekly | 14 |  |  |  |  |
| Harwich | Zylpha Road Beach | Sampling Point | Monthly | 4 |  |  |  |  |
| Hingham | Belair | Sampling Point | Weekly | 11 | 1 | 211 | 211 | 1 |
| Hingham | Hingham Town Beach | Sampling Point | Weekly | 12 | 2 | 142 | 301 | 1 |
| Hingham | Kimball | Sampling Point | Weekly | 11 | 1 | 754 | 754 | 1 |
| Hingham | Martin's Cove | Sampling point | Weekly | 11 | 1 | 331 | 331 | 1 |
| Hingham | North | Sampling Point | Weekly | 10 |  |  |  |  |
| Hingham | Seal Cove | Sampling Point | Weekly | 11 | 3 | 130 | 233 | 1 |
| Hingham | Wompatuck | Sampling Point | Weekly | 11 | 1 | 134 | 134 | 1 |
| Hingham | Yacht Club | Sampling Point | Weekly | 11 | 1 | 269 | 269 | 1 |
| Hull | A Street Bay Side | Sampling Point | Weekly | 12 |  |  |  |  |
| Hull | A Street Ocean | Sampling Point | Weekly | 12 |  |  |  |  |
| Hull | Darcy's | Sampling Point | Weekly | 12 |  |  |  |  |
| Hull | Edgewater | Sampling Point | Weekly | 12 |  |  |  |  |
| Hull | Gunrock | Sampling Point | Monthly | 3 |  |  |  |  |
| Hull | James Ave. | Sampling Point | Monthly | 3 |  |  |  |  |
| Hull | Kenberma | Sampling Point | Biweekly | 6 |  |  |  |  |
| Hull | Nantasket (DCR) | Bathhouse | Weekly | 16 |  |  |  |  |
| Hull | Nantasket (DCR) | North site | Weekly | 16 |  |  |  |  |
| Hull | Nantasket (DCR) | Park St. | Weekly | 16 |  |  |  |  |
| Hull | Nantasket (DCR) | Water St. | Weekly | 16 |  |  |  |  |
| Hull | Newport | Sampling Point | Monthly | 3 |  |  |  |  |
| Hull | Peddocks Island (DCR) | Sampling Point | Weekly | 10 |  |  |  |  |
| Hull | XYZ | Sampling Point | Monthly | 3 |  |  |  |  |
| Ipswich | Clark | Sampling Point | Weekly | 16 |  |  |  |  |
| Ipswich | Crane | Head Lifeguard Station | Monthly | 4 |  |  |  |  |
| Ipswich | Little Neck | Sampling Point | Weekly | 16 |  |  |  |  |
| Ipswich | Pavillion | greatest batherload | Monthly | 4 |  |  |  |  |
| Ipswich | Steep Hill | Steep Hill | Monthly | 4 |  |  |  |  |
| Kingston | Gray's | Sampling Point | Weekly | 13 | 1 | 160 | 160 | 1 |
| Kingston | Rocky Nook | Sampling Point | Weekly | 13 | 1 | 160 | 160 | 1 |
| Lynn | Kings (DCR) | Eastern Ave | Daily | 90 | 27 | 109 | 19900 | 12 |
| Lynn | Kings (DCR) | Kimball Road | Daily | 90 | 23 | 106 | 24200 | 12 |
| Lynn | Kings (DCR) | Pierce Road | Daily | 90 | 16 | 135 | 24200 | 12 |
| Manchester | Black | Sampling Point | Weekly | 16 |  |  |  |  |
| Manchester | Magnolia | Right of Bath and Tennis | Weekly | 16 |  |  |  |  |
| Manchester | Magnolia | Sampling Point 1 | Weekly | 16 |  |  |  |  |
| Manchester | Singing | Right of parking lot | Weekly | 16 |  |  |  |  |
| Manchester | Singing | Sampling Point 1 | Weekly | 16 |  |  |  |  |
| Manchester | Tuck's Point | Sampling Point | Weekly | 16 |  |  |  |  |
| Manchester | West Manchester | Sampling Point | Weekly | 18 | 2 | 213 | 253 | 2 |
| Manchester | White | Sampling Point | Weekly | 16 |  |  |  |  |
| Marblehead | Crocker Park | Sampling Point | Weekly | 12 |  |  |  |  |
| Marblehead | Devereux | Sampling Point | Weekly | 12 |  |  |  |  |
| Marblehead | Gas House | Sampling Point | Weekly | 13 | 1 | 110 | 110 |  |
| Marblehead | Grace Oliver | Sampling Point | Weekly | 13 | 1 | 3040 | 3040 |  |
| Marion | Beverly Yacht | Sampling Point | Weekly | 13 |  |  |  |  |
| Marion | Converse Point | Sampling Point | Weekly | 13 |  |  |  |  |
| Marion | Dexter Lane | Sampling Point | Weekly | 14 | 1 | 352 | 352 |  |
| Marion | Island Wharf | Sampling Point | Weekly | 13 |  |  |  |  |
| Marion | Oakdale Avenue | Sampling Point | Weekly | 13 |  |  |  |  |
| Marion | Piney Point | Sampling Point | Weekly | 14 | 1 | 312 | 312 |  |
| Marion | Planting Island | Sampling Point | Monthly | 4 |  |  |  |  |
| Marion | Silver Shell | North Jetty | Weekly | 13 |  |  |  |  |
| Marion | Silver Shell | South Jetty | Weekly | 13 |  |  |  |  |
| Marion | Tabor Academy | Sampling Point 1 | Weekly | 13 |  |  |  |  |
| Marion | Tabor Academy | Sampling Point 2 | Weekly | 13 |  |  |  |  |
| Marshfield | Brant Rock | Sampling Point | Weekly | 12 |  |  |  |  |
| Marshfield | Fieldston | Hartford Rd (Fieldston) | Weekly | 12 |  |  |  |  |
| Marshfield | Green Harbor | Sampling Point | Weekly | 12 |  |  |  |  |
| Marshfield | Rexhame | Sampling Point | Weekly | 16 |  |  |  |  |
| Marshfield | Sunrise | 9th Road - Foster Ave. | Weekly | 12 |  |  |  |  |
| Mashpee | Callies Beach | Sampling Point | Monthly | 4 |  |  |  |  |
| Mashpee | Mashpee Neck Road (Town Landing) | Sampling Point | Weekly | 14 |  |  |  |  |
| Mashpee | Maushup Village | Sampling Point | Weekly | 14 |  |  |  |  |
| Mashpee | Popponesset (Beach Road) | Sampling Point | Weekly | 17 | 3 | 120 | 400 | 1 |
| Mashpee | Popponesset (Bluff Ave) | Sampling Point | Weekly | 15 | 1 | 400 | 400 |  |
| Mashpee | Popponesset (New Seabury Inn) | Sampling Point | Weekly | 15 | 1 | 132 | 132 |  |
| Mashpee | Seconsett Island Causeway | Sampling Point | Monthly | 4 |  |  |  |  |
| Mashpee | South Cape Beach (DCR) | Sampling Point 1 | Weekly | 17 | 1 | 150 | 150 |  |
| Mattapoisett | Antassawomak East | Sampling Point 1 | Weekly | 10 |  |  |  |  |
| Mattapoisett | Antassawomak West | Sampling Point 2 | Weekly | 10 |  |  |  |  |
| Mattapoisett | Aucoot | Sampling Point | Weekly | 13 |  |  |  |  |
| Mattapoisett | Brant Beach | Sampling Point | Weekly | 10 |  |  |  |  |
| Mattapoisett | Crescent | Sampling Point | Weekly | 10 |  |  |  |  |
| Mattapoisett | Harbor Beach 1 (North) | Sampling Point | Weekly | 10 |  |  |  |  |
| Mattapoisett | Harbor Beach 2 (South) | Sampling Point | Weekly | 10 |  |  |  |  |
| Mattapoisett | Holly Woods 1 | Sampling Point | Weekly | 10 |  |  |  |  |
| Mattapoisett | Holly Woods 2 | Sampling Point | Weekly | 10 |  |  |  |  |
| Mattapoisett | Leisure Shores | Sampling Point | Weekly | 10 |  |  |  |  |
| Mattapoisett | Mattapoisett Shores Association | Sampling Point | Weekly | 11 | 1 | 196 | 196 |  |
| Mattapoisett | Mattapoisett Town Beach | Sampling Point | Weekly | 15 | 2 | 124 | 306 | 1 |
| Mattapoisett | Ned's Point | Sampling Point | Weekly | 13 |  |  |  |  |
| Mattapoisett | Peases Point I (East) | Sampling Point | Weekly | 10 |  |  |  |  |
| Mattapoisett | Peases Point II (West) | Sampling Point | Weekly | 10 |  |  |  |  |
| Mattapoisett | Point Connett | Sampling Point | Weekly | 10 |  |  |  |  |
| Mattapoisett | Shining Tides Reservation | Sampling Point | Weekly | 16 | 2 | 154 | 236 | 2 |
| Nahant | Black Rock | Sampling Point | Weekly | 16 | 2 | 130 | 364 | 2 |
| Nahant | Canoe | Sampling Point | Weekly | 15 |  |  |  |  |
| Nahant | Forty Steps Beach | Sampling Point | Weekly | 15 |  |  |  |  |
| Nahant | Nahant Beach (DCR) | Flagpole | Weekly | 16 |  |  |  |  |
| Nahant | Nahant Beach (DCR) | N. of bathhouse | Weekly | 16 | 1 | 223 | 223 |  |
| Nahant | Nahant Beach (DCR) | Parking section 9 | Weekly | 16 |  |  |  |  |
| Nahant | Nahant Beach (DCR) | South site | Weekly | 16 |  |  |  |  |
| Nahant | Short | Sampling Point | Weekly | 15 |  |  |  |  |
| Nahant | Tudor | Sampling Point | Weekly | 16 | 1 | 226 | 226 | 1 |
| Nantucket | 40th Pole | Sampling Point | Weekly | 11 |  |  |  |  |
| Nantucket | Children's | Sampling Point | Weekly | 13 | 1 | 140 | 140 |  |
| Nantucket | Cisco | Sampling Point | Monthly | 3 |  |  |  |  |
| Nantucket | Cliffside Beach | Sampling Point | Monthly | 3 |  |  |  |  |
| Nantucket | Cliffside Beach Club | Sampling Point | Weekly | 12 | 1 | 410 | 410 |  |
| Nantucket | Dionis | Sampling Point | Weekly | 11 |  |  |  |  |
| Nantucket | Jetties | Sampling Point | Weekly | 12 |  |  |  |  |
| Nantucket | Madaket | Sampling Point | Weekly | 11 |  |  |  |  |
| Nantucket | Miacomet | Sampling Point | Weekly | 11 |  |  |  |  |
| Nantucket | Sconset | Sampling Point | Monthly | 3 |  |  |  |  |
| Nantucket | Sewerbeds | Sampling Point | Weekly | 11 |  |  |  |  |
| Nantucket | Surfside 1 | Sampling Point | Weekly | 10 | 1 | 190 | 190 |  |
| Nantucket | Surfside 2 | Sampling Point | Monthly | 3 |  |  |  |  |
| Nantucket | Washing Pond | Sampling Point | Weekly | 11 |  |  |  |  |
| Nantucket | Washington Street | Sampling Point | Weekly | 11 |  |  |  |  |
| Nantucket | Wauwinet Bayside | Sampling Point | Weekly | 11 |  |  |  |  |
| Nantucket | Wauwinet Oceanside | Sampling Point | Weekly | 11 |  |  |  |  |
| New Bedford | 400 Beach | North | Weekly | 15 |  |  |  | 1 |
| New Bedford | 400 Beach | South | Weekly | 15 |  |  |  | 1 |
| New Bedford | Davy's Locker | Sampling Point | Weekly | 16 | 1 | 232 | 232 | 2 |
| New Bedford | J. Beach | Sampling Point | Weekly | 16 | 1 | 118 | 118 | 2 |
| New Bedford | Kids Beach | Sampling Point | Weekly | 16 | 1 | 132 | 132 | 2 |
| New Bedford | O'Tools | Sampling Point | Weekly | 17 | 2 | 184 | 210 | 3 |
| New Bedford | Squid | Sampling Point | Weekly | 17 | 2 | 148 | 500 | 3 |
| New Bedford | Tabor Park South | Sampling Point | Weekly | 15 |  |  |  | 1 |
| New Bedford | Tower Beach | Tower 1 | Weekly | 16 | 1 | 500 | 500 | 2 |
| New Bedford | Tower Beach | Tower 4 | Weekly | 16 | 1 | 500 | 500 | 2 |
| Newbury | Plum Island | Sampling Point | Monthly | 4 |  |  |  |  |
| Newburyport | Plum Island - 55th St. | 55th street | Weekly | 16 |  |  |  |  |
| Newburyport | Plum Island - end of island 1 | end of island 1 | Weekly | 15 |  |  |  |  |
| Newburyport | Plum Island - end of island 2 | end of island 2 | Weekly | 16 |  |  |  | 1 |
| Newburyport | Plum Island Point | Plum Island Point | Weekly | 16 |  |  |  |  |
| Oak Bluffs | Eastville Town Beach - Harbor side | Sampling Point | Monthly | 3 |  |  |  |  |
| Oak Bluffs | Eastville Town Beach - Lagoon side | Sampling Point | Weekly | 11 |  |  |  |  |
| Oak Bluffs | Inkwell Beach | Sampling Point | Weekly | 11 |  |  |  |  |
| Oak Bluffs | Joseph Sylvia State Beach - Big Bridge | Big Bridge | Monthly | 3 |  |  |  |  |
| Oak Bluffs | Joseph Sylvia State Beach - Sound | Sound | Monthly | 3 |  |  |  |  |
| Oak Bluffs | Marinelli (Jetty) Beach | Sampling Point | Weekly | 11 |  |  |  |  |
| Oak Bluffs | Medeiros Cove (Sailing Camp) | Sampling Point | Weekly | 11 |  |  |  |  |
| Oak Bluffs | Pay Beach | Sampling Point | Weekly | 11 |  |  |  |  |
| Orleans | Little Inn at Pleasant Bay | Sampling Point | Weekly | 9 |  |  |  |  |
| Orleans | Meeting House Pond | Sampling Point | Weekly | 14 |  |  |  |  |
| Orleans | Nauset | Sampling Point | Weekly | 17 | 1 | 150 | 150 |  |
| Orleans | Quanset Harbor Club Association | Sampling Point | Weekly | 10 |  |  |  |  |
| Orleans | Skaket Beach | Sampling Point | Weekly | 17 | 1 | 242 | 242 |  |
| Orleans | Skaket Beach Condominiums | Sampling Point | Weekly | 13 | 1 | 288 | 288 |  |
| Plymouth | Plymouth Beach - 1 | Sampling Point 1 | Weekly | 16 |  |  |  |  |
| Plymouth | Plymouth Beach - 3 | Sampling Point 3 | Weekly | 16 |  |  |  |  |
| Plymouth | Plymouth Beach - 5 | Sampling Point 5 | Weekly | 16 |  |  |  |  |
| Plymouth | White Horse - Full Sail | Full Sail | Weekly | 16 | 1 | 213 | 213 |  |
| Plymouth | White Horse - Hill Top | Hill Top | Weekly | 17 | 1 | 146 | 146 | 1 |
| Provincetown | 333 Commercial Street | Sampling Point | Weekly | 14 |  |  |  |  |
| Provincetown | 451 Commercial Street | Sampling Point | Weekly | 14 |  |  |  |  |
| Provincetown | 593 Commercial Street | Sampling Point | Weekly | 14 |  |  |  |  |
| Provincetown | 637 Commercial Street | Sampling Point | Weekly | 14 |  |  |  |  |
| Provincetown | Atkins Lane | Sampling Point | Weekly | 14 |  |  |  |  |
| Provincetown | Atlantic Avenue | Sampling Point | Weekly | 14 |  |  |  |  |
| Provincetown | Court Street | Sampling Point | Weekly | 14 |  |  |  |  |
| Provincetown | Herring Cove (National Seashore) | Sampling Point 1 | Weekly | 11 |  |  |  |  |
| Provincetown | Johnson Street | Sampling Point | Weekly | 14 |  |  |  |  |
| Provincetown | Kendal Lane | Sampling Point | Weekly | 14 |  |  |  |  |
| Provincetown | Provincetown Inn East | Sampling Point | Weekly | 15 | 1 | 312 | 312 |  |
| Provincetown | Provincetown Inn Rotary | Sampling Point | Monthly | 4 |  |  |  |  |
| Provincetown | Race Point (National Seashore) | Sampling Point 1 | Weekly | 11 |  |  |  |  |
| Provincetown | Ryder Street | Sampling Point Middle | Weekly | 14 |  |  |  |  |
| Provincetown | Town Landing - Breakwater | Sampling Point | Weekly | 17 | 3 | 108 | 628 | 1 |
| Provincetown | Town Landing - Snail Road | Sampling Point | Weekly | 16 | 2 | 168 | 676 | 1 |
| Provincetown | Town Landing West of Coast Guard | Sampling Point | Weekly | 14 |  |  |  |  |
| Provincetown | West End Lot | Sampling Point | Weekly | 14 |  |  |  |  |
| Provincetown | Winston Ave | Sampling Point | Weekly | 15 | 1 | 672 | 672 | 1 |
| Quincy | Avalon | Sampling Point | Weekly | 12 |  |  |  |  |
| Quincy | Broady (Baker) | Sampling Point | Weekly | 14 | 3 | 512 | 4360 | 1 |
| Quincy | Chikatawbot | Sampling Point | Weekly | 12 |  |  |  |  |
| Quincy | Delano Ave. | Sampling Point | Weekly | 13 | 1 | 121 | 121 | 1 |
| Quincy | Edgewater | Sampling Point | Weekly | 12 |  |  |  |  |
| Quincy | Germantown Firestation | Sampling Point | Weekly | 14 | 3 | 399 | 3970 | 1 |
| Quincy | Heron | Sampling Point | Weekly | 12 |  |  |  |  |
| Quincy | Merrymount | Sampling Point | Weekly | 12 |  |  |  |  |
| Quincy | Mound | Sampling Point | Weekly | 12 |  |  |  |  |
| Quincy | Nickerson | Sampling Point | Weekly | 12 |  |  |  |  |
| Quincy | Orchard Street | Sampling Point | Weekly | 12 |  |  |  |  |
| Quincy | Parkhurst | Sampling Point | Weekly | 12 |  |  |  |  |
| Quincy | Rhoda | Sampling Point | Weekly | 15 | 2 | 243 | 4790 | 1 |
| Quincy | Wollaston @ Channing Street (DCR) | Channing Street | Daily | 90 | 13 | 109 | 5172 | 19 |
| Quincy | Wollaston @ Milton Street (DCR) | Sampling Point | Daily | 90 | 18 | 105 | 4106 | 22 |
| Quincy | Wollaston @ Rice Road (DCR) | Sampling Point | Daily | 90 | 8 | 134 | 11200 | 20 |
| Quincy | Wollaston @ Sachem Street (DCR) | Sampling Point | Daily | 90 | 12 | 106 | 2760 | 19 |
| Revere | Revere (DCR) | at state police | Weekly | 18 |  |  |  |  |
| Revere | Revere (DCR) | Beach St. | Weekly | 18 | 1 | 269 | 269 |  |
| Revere | Revere (DCR) | Carey Circle | Weekly | 18 | 2 | 114 | 119 |  |
| Revere | Revere (DCR) | Oak Island St. | Weekly | 18 | 3 | 113 | 346 |  |
| Revere | Short (DCR) | Sampling Point | Weekly | 19 | 3 | 117 | 546 | 1 |
| Rockport | Back | Sampling Point | Weekly | 12 |  |  |  |  |
| Rockport | Cape Hedge | Sampling Point | Monthly | 3 |  |  |  |  |
| Rockport | Front Beach | Sampling Point | Weekly | 12 |  |  |  |  |
| Rockport | Long - Gloucester | Gloucester | Weekly | 12 |  |  |  |  |
| Rockport | Long - North | North | Weekly | 12 |  |  |  |  |
| Rockport | Old Garden | Sampling Point | Monthly | 3 |  |  |  |  |
| Rockport | Pebble | Sampling Point | Monthly | 3 |  |  |  |  |
| Salem | Camp Naumkeag | Sampling Point | Weekly | 13 |  |  |  |  |
| Salem | Children's Island - Back | Back | Weekly | 9 | 2 | 146 | 259 | 2 |
| Salem | Children's Island - Dock | Dock | Weekly | 8 |  |  |  |  |
| Salem | Children's Island - Wally | Wally | Weekly | 9 |  |  |  |  |
| Salem | Collins Cove | Sampling Point | Weekly | 13 |  |  |  |  |
| Salem | Dead Horse | Sampling Point | Biweekly | 7 |  |  |  |  |
| Salem | Forest River - Pioneer | Sampling Point | Weekly | 13 |  |  |  |  |
| Salem | Forest River - Point | Sampling Point | Biweekly | 7 |  |  |  |  |
| Salem | Juniper Point | Sampling Point | Weekly | 14 | 1 | 233 | 233 |  |
| Salem | Ocean Avenue | Sampling Point | Weekly | 13 | 1 | 216 | 216 | 1 |
| Salem | Osgood | Sampling Point | Weekly | 9 |  |  |  | 1 |
| Salem | Steps | Sampling Point | Biweekly | 7 |  |  |  |  |
| Salem | Waikiki Beach (Winter Island) | Sampling Point | Biweekly | 7 |  |  |  |  |
| Salem | Willow Avenue | Sampling Point | Weekly | 14 | 2 | 110 | 173 | 1 |
| Salem | Willows Pier | Sampling Point | Weekly | 13 | 1 | 132 | 132 | 1 |
| Salisbury | Salisbury Beach (DCR) | Sampling Point | Weekly | 16 |  |  |  |  |
| Salisbury | Salisbury Beach North End (DCR) | Star of the Sea | Weekly | 17 | 1 | 340 | 340 |  |
| Sandwich | Carleton Shores | Sampling Point | Weekly | 13 |  |  |  |  |
| Sandwich | East Sandwich | Sampling Point | Weekly | 11 |  |  |  |  |
| Sandwich | Mill Creek | Sampling Point | Weekly | 11 |  |  |  |  |
| Sandwich | Scusset (DCR) | Sampling Point | Weekly | 16 |  |  |  |  |
| Sandwich | Torrey Beach Community Association | Sampling Point | Weekly | 9 |  |  |  |  |
| Sandwich | Town Neck (Hemispheres) | Sampling Point | Weekly | 11 |  |  |  |  |
| Sandwich | Town Neck Beach | End of Boardwalk | Weekly | 11 |  |  |  |  |
| Scituate | Bassings Beach | Sampling Point | Weekly | 14 | 1 | 733 | 733 |  |
| Scituate | Egypt | Sampling Point | Weekly | 13 | 1 | 134 | 134 | 1 |
| Scituate | Humarock | Sampling Point | Weekly | 12 |  |  |  |  |
| Scituate | Minot | Sampling Point | Weekly | 13 | 1 | 134 | 134 | 1 |
| Scituate | Peggotty | Sampling Point | Weekly | 17 | 2 | 554 | 3130 | 3 |
| Scituate | Sand Hills | Sampling Point | Weekly | 12 |  |  |  |  |
| Scituate | Scituate Lighthouse | Sampling Point | Weekly | 15 | 3 | 120 | 1439 | 3 |
| Somerset | Pierce Beach | Sampling Point | Weekly | 14 | 3 | 309 | 10000 | 1 |
| Swampscott | Eisman's | Sampling Point | Weekly | 13 | 1 | 2190 | 2190 |  |
| Swampscott | Fisherman's | Sampling Point | Weekly | 14 | 2 | 199 | 373 |  |
| Swampscott | Kings | Sampling Point | Weekly | 13 | 1 | 209 | 209 |  |
| Swampscott | Phillips | Sampling Point | Weekly | 12 |  |  |  |  |
| Swampscott | Preston | Sampling Point | Weekly | 13 | 1 | 187 | 187 |  |
| Swampscott | Whales | Sampling Point | Weekly | 13 | 1 | 1010 | 1010 |  |
| Swansea | Cedar Cove Club | Sampling Point | Weekly | 12 |  |  |  |  |
| Swansea | Coles River Club off Harbor Rd | Sampling Point | Weekly | 12 |  |  |  |  |
| Swansea | Leeside | Sampling Point | Weekly | 12 | 2 | 155 | 158 | 1 |
| Swansea | Sandy Beach | Sampling Point - Swansea | Weekly | 13 | 1 | 135 | 135 | 1 |
| Swansea | Swansea Town Beach | Sampling Point | Weekly | 13 | 1 | 238 | 238 | 1 |
| Tisbury | Hilman's Point | Sampling Point | Monthly | 4 |  |  |  |  |
| Tisbury | Mink Meadows | Sampling Point | Weekly | 12 |  |  |  |  |
| Tisbury | Owen Little Way | Sampling Point | Weekly | 22 | 6 | 110 | 314 | 4 |
| Tisbury | Owen Park | Sampling Point | Monthly | 3 |  |  |  |  |
| Tisbury | Sound @ Wilfred's Pond Preserve | Sampling Point | Monthly | 4 |  |  |  |  |
| Tisbury | Tashmoo Beach | Sampling Point | Monthly | 3 |  |  |  |  |
| Tisbury | Tashmoo Cut | Sampling Point | Weekly | 13 | 1 | 2000 | 2000 |  |
| Tisbury | Vineyard Harbor Motel | Sampling Point | Weekly | 12 |  |  |  |  |
| Truro | Ballston | Sampling Point | Monthly | 4 |  |  |  |  |
| Truro | Beach Point Landing | Sampling Point | Weekly | 14 |  |  |  |  |
| Truro | Coast Guard Town | Sampling Point | Monthly | 4 |  |  |  |  |
| Truro | Cold Storage/Pond Village | Sampling Point | Monthly | 4 |  |  |  |  |
| Truro | Corn Hill | Sampling Point | Monthly | 4 |  |  |  |  |
| Truro | Cranberry Hill | Sampling Point | Weekly | 14 |  |  |  |  |
| Truro | Crow's Nest (496 Shore Rd) | Sampling Point | Weekly | 14 |  |  |  |  |
| Truro | Dune's Colony (648 Shore Rd) | Sampling Point | Weekly | 14 |  |  |  |  |
| Truro | Fisher | Sampling Point | Monthly | 4 |  |  |  |  |
| Truro | Great Hollow | Sampling Point | Monthly | 4 |  |  |  |  |
| Truro | Head of the Meadow (National Seashore) | Sampling Point 1 | Weekly | 11 |  |  |  |  |
| Truro | Head of the Meadow (Town) | Sampling Point | Monthly | 4 |  |  |  |  |
| Truro | Longnook | Sampling Point | Monthly | 4 |  |  |  |  |
| Truro | Lookout Bluff | Sampling Point | Weekly | 14 |  |  |  |  |
| Truro | Noon's Landing | Sampling Point | Weekly | 14 |  |  |  |  |
| Truro | Pamet Harbor | Sampling Point | Weekly | 14 |  |  |  |  |
| Truro | Ryder | Sampling Point | Monthly | 4 |  |  |  |  |
| Truro | Shearwater Association | Shearwater 1 | Weekly | 14 |  |  |  |  |
| Truro | Sunset Village (372 Shore Rd) | Sampling Point | Weekly | 14 |  |  |  |  |
| Wareham | Briarwood | Sampling Point | Weekly | 17 | 2 | 148 | 152 | 2 |
| Wareham | East Boulevard | Sampling Point | Weekly | 16 |  |  |  |  |
| Wareham | Forbes | Sampling Point | Weekly | 17 | 1 | 254 | 254 |  |
| Wareham | Hamilton Beach | Sampling Point | Weekly | 17 | 1 | 260 | 260 |  |
| Wareham | Indian Mound Beach | Sampling Point | Weekly | 17 | 1 | 400 | 400 |  |
| Wareham | Little Harbor | Sampling Point | Weekly | 16 |  |  |  |  |
| Wareham | North Boulevard | Sampling Point | Weekly | 17 | 1 | 400 | 400 |  |
| Wareham | Onset | Sampling Point | Weekly | 16 |  |  |  |  |
| Wareham | Parkwood | Sampling Point | Weekly | 17 | 1 | 238 | 238 |  |
| Wareham | Pinehurst | Sampling Point | Weekly | 16 |  |  |  |  |
| Wareham | Point Independence | Sampling Point | Weekly | 12 | 1 | 308 | 308 |  |
| Wareham | Riverside Avenue | Sampling Point | Weekly | 16 |  |  |  |  |
| Wareham | Shell Point | Sampling Point | Weekly | 16 |  |  |  |  |
| Wareham | Standish Shores | Sampling Point | Weekly | 10 |  |  |  |  |
| Wareham | Swift's | Sampling Point | Weekly | 17 | 2 | 122 | 400 | 1 |
| Wareham | Swift's Neck | Sampling Point | Weekly | 17 | 2 | 112 | 170 | 1 |
| Wellfleet | Burton Baker | Sampling Point | Weekly | 14 |  |  |  |  |
| Wellfleet | Cahoon Hollow | Sampling Point | Monthly | 4 |  |  |  |  |
| Wellfleet | Chequesset Yacht and Country Club | Sampling Point | Weekly | 12 |  |  |  |  |
| Wellfleet | Duck Harbor | Sampling Point | Monthly | 4 |  |  |  |  |
| Wellfleet | Indian Neck | Sampling Point | Monthly | 4 |  |  |  |  |
| Wellfleet | Maguire Landing | Sampling Point | Monthly | 4 |  |  |  |  |
| Wellfleet | Marconi (National Seashore) | Sampling Point 1 | Weekly | 11 |  |  |  |  |
| Wellfleet | Mayo | Sampling Point | Weekly | 14 |  |  |  |  |
| Wellfleet | Newcomb Hollow | Sampling Point | Monthly | 4 |  |  |  |  |
| Wellfleet | Omaha Road | Sampling Point | Monthly | 4 |  |  |  |  |
| Wellfleet | Powers Landing | Sampling Point | Monthly | 4 |  |  |  |  |
| Wellfleet | The Gut | Sampling Point | Monthly | 4 |  |  |  |  |
| Wellfleet | White Crest | Sampling Point | Monthly | 4 |  |  |  |  |
| West Tisbury | Capawok Beach | Capawok sampling point | Weekly | 10 |  |  |  |  |
| West Tisbury | Gray's Beach | Grays | Weekly | 10 |  |  |  |  |
| West Tisbury | Lambert's Cove Beach | Sampling Point | Weekly | 13 |  |  |  |  |
| West Tisbury | Long Point (Ocean) | Central | Monthly | 4 |  |  |  |  |
| West Tisbury | Naushon Beach | Naushon sampling point | Weekly | 11 |  |  |  |  |
| West Tisbury | Saltworks Beach | Saltworks | Weekly | 10 |  |  |  |  |
| West Tisbury | Tisbury Great Pond Beach | Sampling Point | Monthly | 4 |  |  |  |  |
| Westport | Baker's Beach | Sampling Point | Monthly | 3 |  |  |  |  |
| Westport | C and K Club - Howland | Sampling Point | Monthly | 3 |  |  |  |  |
| Westport | Cherry and Webb | Sampling Point | Monthly | 3 |  |  |  |  |
| Westport | East Beach | Sampling Point | Monthly | 3 |  |  |  |  |
| Westport | Elephant Rock Beach Club | Sampling Point | Monthly | 3 |  |  |  |  |
| Westport | Horseneck (DCR) | Sampling Point | Weekly | 16 |  |  |  |  |
| Westport | Spindle Rock | Sampling Point | Monthly | 3 |  |  |  |  |
| Westport | Town-Yacht | Sampling Point | Monthly | 3 |  |  |  |  |
| Weymouth | George E. Lane | Sampling Point | Weekly | 12 |  |  |  |  |
| Weymouth | Wessagusett (Old Wessagussett) | Sampling Point | Weekly | 12 |  |  |  |  |
| Winthrop | Donovans | Sampling Point | Weekly | 22 | 8 | 122 | 4160 | 4 |
| Winthrop | Grandview | Sampling Point | Weekly | 20 | 3 | 121 | 8704 | 1 |
| Winthrop | Halford | Sampling Point | Weekly | 20 |  |  |  |  |
| Winthrop | Pico | Sampling Point | Weekly | 23 | 3 | 175 | 1130 | 2 |
| Winthrop | Winthrop (DCR) | Sampling Point | Weekly | 16 |  |  |  |  |
| Winthrop | Yerrill | Sampling Point | Weekly | 20 | 1 | 221 | 221 |  |
| Yarmouth | Bass River East | East | Weekly | 16 |  |  |  |  |
| Yarmouth | Bass River West | West | Weekly | 16 |  |  |  |  |
| Yarmouth | Baxter Avenue | Sampling Point | Weekly | 16 |  |  |  |  |
| Yarmouth | Bay Road | Sampling Point | Weekly | 16 |  |  |  |  |
| Yarmouth | Bayview Street | Sampling Point | Weekly | 16 |  |  |  |  |
| Yarmouth | Colonial Acres East | East | Weekly | 16 |  |  |  |  |
| Yarmouth | Colonial Acres West | West | Weekly | 16 |  |  |  |  |
| Yarmouth | Columbus Avenue | Sampling Point | Weekly | 16 |  |  |  |  |
| Yarmouth | Englewood | Sampling Point | Weekly | 16 |  |  |  |  |
| Yarmouth | Follins Pond | Sampling Point | Weekly | 16 |  |  |  |  |
| Yarmouth | Gray's Beach | Sampling Point | Weekly | 16 |  |  |  |  |
| Yarmouth | Malfa Road | Sampling Point | Weekly | 16 |  |  |  |  |
| Yarmouth | Ocean Mist Hotel | Sampling Point | Weekly | 14 |  |  |  |  |
| Yarmouth | Parkers River East | Sampling Point | Weekly | 16 |  |  |  |  |
| Yarmouth | Parkers River West | Sampling Point | Weekly | 16 |  |  |  |  |
| Yarmouth | Seagull (Center) | East front | Weekly | 16 |  |  |  |  |
| Yarmouth | Seagull (Left) | back | Weekly | 16 |  |  |  |  |
| Yarmouth | Seagull (Right) | West front | Weekly | 16 |  |  |  |  |
| Yarmouth | Seaview Ave. Beach | Sampling Point | Weekly | 16 |  |  |  |  |
| Yarmouth | South Middle | Sampling Point | Weekly | 16 |  |  |  |  |
| Yarmouth | Thatcher Town Park | Sampling Point | Weekly | 16 |  |  |  |  |
| Yarmouth | Vernon St. | Sampling Point | Weekly | 17 | 1 | 292 | 292 |  |
| Yarmouth | Wilbur Park | Sampling Point | Weekly | 16 |  |  |  |  |
| Yarmouth | Windmill | Sampling Point | Weekly | 16 |  |  |  |  |

| Community | Beach Name1 | Sample Location | Testing Frequency | # Tests | # Single Sample Exceedances | Minimum Exceedance | Maximum Exceedance | # Postings2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abington | Island Grove Beach | Sampling Point | Weekly | 9 |  |  |  |  |
| Acton | NARA Beach | Sampling Point | Weekly | 19 | 3 | 268 | 400 | 1 |
| Agawam | Robinson Pond Beach 1 (DCR) | Sampling Point | Weekly | 18 | 3 | 123 | 176 | 2 |
| Amesbury | Camp Bauercrest | Sampling Point | Weekly | 10 |  |  |  |  |
| Amesbury | Glen Devin Condominiums | Sampling Point | Weekly | 11 | 2 | 345 | 387 | 2 |
| Amesbury | Lake Attitash - A.L.S.I.A. | Dam/Bathing area | Weekly | 12 | 1 | 461 | 461 | 1 |
| Amesbury | Lake Gardner | Sampling Point | Weekly | 12 | 1 | 308 | 308 | 1 |
| Amesbury | Tuxbury RV Resort Lagoon | Sampling Point | Weekly | 18 |  |  |  |  |
| Amherst | Puffers Pond (AKA Factory Hollow Pond) | North | Three times | 3 |  |  |  |  |
| Amherst | Puffers Pond (AKA Factory Hollow Pond) | South | Three times | 3 |  |  |  |  |
| Andover | Camp Maude Eaton | Sample Site 1 - swim 2 feet | Weekly | 12 |  |  |  |  |
| Andover | Camp Maude Eaton | Sample Site 2 - swim 4 feet | Weekly | 12 |  |  |  |  |
| Andover | Pomps Pond | Station 1 | Weekly | 13 |  |  |  |  |
| Andover | Pomps Pond | Station 2 | Weekly | 13 |  |  |  |  |
| Andover | Pomps Pond | Station 3 | Weekly | 13 | 1 | 280 | 280 |  |
| Arlington | Arlington Reservoir | Sampling Point | Weekly | 13 |  |  |  |  |
| Arlington | Medford Boat Club | Lower | Weekly | 17 |  |  |  |  |
| Arlington | Medford Boat Club | Upper | Weekly | 17 |  |  |  |  |
| Ashburnham | Camp Split Rock | Sampling Point | Two times | 2 |  |  |  |  |
| Ashburnham | Camp Wellville Beach | Sampling Point | Weekly | 7 |  |  |  |  |
| Ashburnham | Camp Winnekeag Pond | Sampling Point | Weekly | 16 |  |  |  |  |
| Ashby | Camp Lapham | Sampling point | Weekly | 8 |  |  |  |  |
| Ashby | Damon Pond Beach (DCR) | Sampling Point | Weekly | 25 | 8 | 68 | 870 | 7 |
| Ashland | Ashland Reservoir-Main Beach (DCR) | Sampling Point | Weekly | 12 |  |  |  |  |
| Ashland | Hopkinton Reservoir-Main Beach (DCR) | Sampling Point | Weekly | 18 | 2 | 99 | 99 | 2 |
| Ashland | Hopkinton Reservoir-Upper Beach (DCR) | Sampling Point | Weekly | 16 |  |  |  |  |
| Ashland | Warren Conference Center | Sampling Point | Weekly | 13 |  |  |  |  |
| Athol | Ellis Beach | Sampling Point | Weekly | 12 |  |  |  |  |
| Athol | Silver Pond Beach | Sampling Point | Weekly | 15 | 3 | 256 | 400 | 2 |
| Auburn | Century Sportsman's Club | Sampling Point | Weekly | 14 | 1 | 980 | 980 |  |
| Ayer | Ayer Town Beach | Sampling Point | Weekly | 16 |  |  |  |  |
| Ayer | Mirror Lake | Sampling Point | Weekly | 10 |  |  |  |  |
| Barnstable | Fair Acres Country Day School | Sampling Point | Weekly | 13 | 3 | 800 | 800 |  |
| Barnstable | Hamblin Pond | Sampling Point | Weekly | 13 |  |  |  |  |
| Barnstable | Hathaway Pond | Sampling Point | Weekly | 13 |  |  |  |  |
| Barnstable | Holly Point Assoc. | Sampling Point | Weekly | 14 |  |  |  |  |
| Barnstable | Homestead Homeowner's Association | Sampling Point | Weekly | 17 | 4 | 272 | 800 | 1 |
| Barnstable | Jimmy's Beach (Wequaquet Heights Association) | 118 Conners Rd | Weekly | 14 |  |  |  |  |
| Barnstable | Jimmy's Beach (Wequaquet Heights Association) | Jimmys Beach | Weekly | 14 |  |  |  |  |
| Barnstable | Joshua's Pond | Sampling Point | Weekly | 13 |  |  |  |  |
| Barnstable | Long Pond Farms Association | Sampling Point | Weekly | 13 |  |  |  |  |
| Barnstable | Lovell's Pond | Sampling Point | Weekly | 13 |  |  |  |  |
| Barnstable | Middle Pond | Sampling Point | Weekly | 13 |  |  |  |  |
| Barnstable | Regency Drive Owners Association | Sampling Point 1 | Weekly | 16 | 5 | 360 | 800 | 5 |
| Barnstable | Sand Shores Association | Sampling Point | Weekly | 14 |  |  |  |  |
| Barnstable | Wequaquet Estates | Sampling Point | Weekly | 13 |  |  |  |  |
| Barnstable | Wequaquet Lake Town | Sampling Point | Weekly | 17 | 4 | 312 | 800 | 1 |
| Barnstable | Wianno Club (Fresh-Crystal Lake) | Sampling Point | Weekly | 14 |  |  |  |  |
| Becket | Becket Woods Road District - Beach | Sampling Point | Weekly | 19 | 2 | 365.4 | 517.2 |  |
| Becket | Berkshire Lake Estates | Sampling Point | Weekly | 16 |  |  |  |  |
| Becket | Camp Becket - Iroquois Beach | Sampling Point | Weekly | 12 |  |  |  |  |
| Becket | Camp Becket - Main Beach | Sampling Point | Weekly | 12 |  |  |  |  |
| Becket | Camp Greylock - Jr. Beach | Sampling Point | Weekly | 10 |  |  |  |  |
| Becket | Camp Watitoh Beach | Sampling Point | Weekly | 10 |  |  |  |  |
| Becket | Center Lake Estates Beach | Sampling Point | Weekly | 16 |  |  |  |  |
| Becket | Center Pond Beach | Sampling Point | Weekly | 12 |  |  |  |  |
| Becket | Chimney Corners Camp - Beach | Sampling Point | Weekly | 12 | 1 | 344.8 | 344.8 |  |
| Becket | Crystal Pond Homeowners Assoc Beach | Sampling Point | Weekly | 14 | 1 | 816.5 | 816.5 |  |
| Becket | Indian Lake Assoc - Boulder Beach | Sampling Point | Weekly | 16 | 1 | 387.3 | 387.3 |  |
| Becket | Indian Lake Assoc - Dam Beach | Boat Dock | Weekly | 16 |  |  |  |  |
| Becket | Indian Lake Assoc - Forest Beach | Sampling Point | Weekly | 17 | 1 | 980.4 | 980.4 |  |
| Becket | Indian Lake Assoc - Niskayuna Beach | Sampling Point | Weekly | 15 |  |  |  |  |
| Becket | Mountain Grove Assoc. Beach | Sampling Point | Weekly | 7 |  |  |  |  |
| Becket | Sherwood Forest - Arrow Beach | Sampling Point | Weekly | 16 |  |  |  |  |
| Becket | Sherwood Forest - Boat Beach | Sampling Point | Weekly | 16 | 1 | 648.8 | 648.8 |  |
| Becket | Sherwood Forest - Excalibur | Sampling Point | Weekly | 17 | 1 | 325.5 | 325.5 |  |
| Becket | Sherwood Forest - Fireside Beach (Little Robin dam) | Sampling Point | Weekly | 17 | 1 | 689.3 | 689.3 |  |
| Becket | Sherwood Forest - Grassy Beach | Sampling Point | Weekly | 17 | 1 | 613.1 | 613.1 |  |
| Becket | Sherwood Forest - Lancelot Beach | Sampling Point 1 | Weekly | 16 |  |  |  |  |
| Becket | Sherwood Forest - North Beach | Sampling Point | Weekly | 16 | 3 | 260.3 | 1986.3 |  |
| Becket | Sherwood Forest - Will Scarlett | Sampling Point | Weekly | 16 |  |  |  |  |
| Becket | Sherwood Greens Road District Beach | Sampling Point | Weekly | 16 |  |  |  |  |
| Bedford | Springs Brook Park Bathing Beach | Concession Stand Area | Weekly | 12 |  |  |  |  |
| Bedford | Springs Brook Park Bathing Beach | Dock | Weekly | 12 |  |  |  |  |
| Bedford | Springs Brook Park Bathing Beach | spray pnd (was SP & Concess; b4 that, Pad) | Weekly | 12 |  |  |  |  |
| Bedford | Springs Brook Park Bathing Beach | water slide (used to be Rivulet) | Weekly | 12 |  |  |  |  |
| Belchertown | Belchertown Town Beach (Lake Arcadia) | Sampling Point | Weekly | 12 | 2 | 370 | 500 | 1 |
| Bellingham | Arcand Park Beach | Sampling Point | Weekly | 14 |  |  |  |  |
| Bellingham | Silver Lake | Sampling Point | Weekly | 14 |  |  |  |  |
| Billerica | Nutting Lake - Micozzi Beach | North | Weekly | 12 |  |  |  |  |
| Billerica | Nutting Lake - Micozzi Beach | South | Weekly | 12 | 1 | 276 | 276 |  |
| Bolton | Bolton Town Beach | Sampling Point | Weekly | 16 |  |  |  |  |
| Bolton | Camp Resolute | Day Beach AKA Camp 2 | Weekly | 16 |  |  |  |  |
| Bolton | Camp Resolute | Main Beach AKA Camp 1 | Weekly | 16 |  |  |  |  |
| Bolton | Tom Denney Nature Camp | Sampling Point | Weekly | 7 |  |  |  |  |
| Bourne | Picture Lake (Flax Pond) | Sampling Point | Weekly | 14 |  |  |  |  |
| Bourne | Queen Sewell Pond | Sampling Point | Weekly | 14 |  |  |  |  |
| Boxford | Camp Rotary | Sampling Point | Weekly | 10 |  |  |  |  |
| Boxford | Camp Stepping Stone | Sampling Point | Weekly | 7 |  |  |  |  |
| Boxford | Camp Wakanda | Sampling Point | Weekly | 12 |  |  |  |  |
| Boxford | Danvers YMCA Daycamp | Sampling Point | Weekly | 9 |  |  |  |  |
| Boxford | Stiles Pond Beach | Sampling Point | Weekly | 16 |  |  |  |  |
| Braintree | Sunset Lake | dock | Weekly | 14 | 1 | 720 | 720 | 1 |
| Brewster | Beechwood Landing/Greenland Pond | Sampling Point | Weekly | 14 |  |  |  |  |
| Brewster | Beechwood Landing/Long Pond | Sampling Point | Weekly | 14 |  |  |  |  |
| Brewster | Blueberry Pond | Sampling Point | Weekly | 14 |  |  |  |  |
| Brewster | Camp Favorite | Sampling Point | Weekly | 10 |  |  |  |  |
| Brewster | Cape Cod Sea Camps (Long Pond) | Sampling Point | Weekly | 10 |  |  |  |  |
| Brewster | Cliff Pond (DCR) | DYS | Weekly | 17 | 1 | 160 | 160 | 1 |
| Brewster | Cliff Pond (DCR) | Sampling Point | Weekly | 16 |  |  |  |  |
| Brewster | Crossroads for Kids Camp (Camp Mitton) | Crossroads for Kids Camp | Weekly | 11 | 1 | 440 | 440 |  |
| Brewster | Flax Pond (DCR) | Sampling Point | Weekly | 16 |  |  |  |  |
| Brewster | Long Pond | Sampling Point | Weekly | 16 |  |  |  |  |
| Brewster | Robinwood Homeowners Association (Owl Pond) | Sampling Point | Weekly | 13 |  |  |  |  |
| Brewster | Sheep Pond Beach | Sampling Point | Weekly | 13 |  |  |  |  |
| Brewster | Sheep Pond Landing | Sampling Point | Weekly | 16 |  |  |  |  |
| Brewster | Slough Pond | Sampling Point | Weekly | 16 |  |  |  |  |
| Brewster | Upper Mill Pond | Sampling Point | Weekly | 16 |  |  |  |  |
| Brimfield | Dean Pond Beach (DCR) | Sampling Point | Weekly | 12 |  |  |  |  |
| Brimfield | Village Green Family Campground | Sampling Point | Weekly | 19 | 2 | 350 | 500 | 1 |
| Brookfield | South Pond | Sampling Point | Weekly | 8 |  |  |  |  |
| Carver | Crystal Lake | Sampling Point | Weekly | 8 |  |  |  |  |
| Carver | John's Pond | Sampling Point | Weekly | 16 |  |  |  |  |
| Carver | Sampson's Pond | Sampling Point | Weekly | 15 |  |  |  |  |
| Charlemont | Cold River Pool (DCR) | Sampling Point | Weekly | 23 | 12 | 63 | 548 | 7 |
| Charlton | Buffumville Lake (USACE) | Sampling Point | Weekly | 18 |  |  |  |  |
| Charlton | Camp Foskett (YMCA) | Sampling Point | Weekly | 8 | 1 | 390 | 390 | 1 |
| Charlton | Camp Joslin | Sampling Point | Weekly | 12 |  |  |  |  |
| Charlton | Salem Covenant Church Camp | Sampling Point | Weekly | 12 | 1 | 410 | 410 |  |
| Chatham | Goose Pond | Sampling Point | Weekly | 12 |  |  |  |  |
| Chatham | Schoolhouse Pond | Sampling Point | Weekly | 12 |  |  |  |  |
| Chatham | White Pond | Sampling Point | Weekly | 12 |  |  |  |  |
| Chatham | White Pond @ 60 White Pond Rd. | Sampling Point | Weekly | 12 |  |  |  |  |
| Chelmsford | Freeman Lake | Middle | Weekly | 11 | 3 | 360 | 640 | 1 |
| Chelmsford | Hart Pond | Middle | Weekly | 9 |  |  |  |  |
| Chesterfield | Chesterfield Scout Reservation - BSA | Sampling Point | Weekly | 6 |  |  |  |  |
| Chesterfield | Damon Pond Association Beach | Sampling Point | Weekly | 16 |  |  |  |  |
| Chicopee | Chicopee Beach (DCR) | Sampling Point | Weekly | 19 | 3 | 65 | 411 | 3 |
| Concord | Annursnac Hill Assoc. | Sampling Point | Weekly | 16 |  |  |  |  |
| Concord | Silver Hill Assoc | Sampling Point | Weekly | 17 | 1 | 268 | 268 | 1 |
| Concord | Walden Pond - Main (DCR) | Sampling Point | Weekly | 19 | 2 | 210 | 960 | 2 |
| Concord | Walden Pond - Red Cross (DCR) | East | Weekly | 18 | 2 | 130 | 690 | 3 |
| Concord | Walden Pond - Red Cross (DCR) | West | Weekly | 18 | 2 | 100 | 440 | 2 |
| Concord | White Pond Assoc | Sampling Point | Weekly | 16 | 1 | 1060 | 1060 | 2 |
| Conway | Conway Swimming Pool | Sampling Point | Weekly | 14 |  |  |  |  |
| Dennis | Flax Pond (Yarmouth/Dennis) | Sampling Point | Weekly | 14 |  |  |  |  |
| Dennis | Princess Beach-Scargo Lake | Sampling Point | Weekly | 14 |  |  |  |  |
| Dennis | Scargo Lake | Sampling Point | Weekly | 14 |  |  |  |  |
| Douglas | Breezy Picnic Grounds | Sampling Point | Weekly | 13 |  |  |  |  |
| Douglas | Lake Manchaug Camping | Sampling Point | Weekly | 16 |  |  |  |  |
| Douglas | Wallum Lake (DCR) | Sampling Point | Weekly | 16 |  |  |  |  |
| Douglas | Wallum Lake Terrace | Sampling Point | Monthly | 4 |  |  |  |  |
| Dover | Grossman Beach | Sampling Point | Weekly | 11 |  |  |  |  |
| Dover | Powissett | Sampling Point | Weekly | 11 |  |  |  |  |
| Dracut | Fleur de Lis | Sampling Point | Weekly | 8 |  |  |  |  |
| Dracut | Grove | Sampling Point | Weekly | 8 |  |  |  |  |
| Dracut | Hilltop | Sampling Point | Weekly | 8 |  |  |  |  |
| Dracut | Mascuppic | Sampling Point | Weekly | 8 |  |  |  |  |
| Dracut | Passaconaway | Sampling Point | Weekly | 8 |  |  |  |  |
| Dracut | Richardson | Sampling Point | Weekly | 9 | 1 | 252 | 252 |  |
| Dudley | Merino Pond | Sampling Point | Weekly | 14 |  |  |  |  |
| Dunstable | YMCA beach @ Massapoag Pond | Sampling Point | Weekly | 6 |  |  |  |  |
| East Brookfield | Camp Frank A Day | Sampling Point | Weekly | 13 |  |  |  |  |
| East Brookfield | Lake Lashaway | Sampling Point | Weekly | 13 | 2 | 250 | 1700 |  |
| Eastham | Great Pond | Sampling Point | Weekly | 14 |  |  |  |  |
| Eastham | Herring Pond | Sampling Point | Weekly | 14 |  |  |  |  |
| Eastham | Jemima Pond | Sampling Point | Weekly | 14 |  |  |  |  |
| Eastham | Minister's Pond | Sampling Point | Weekly | 15 | 1 | 260 | 260 |  |
| Eastham | Nauset Haven Lakeside Condo (Minister) | Sampling Point | Weekly | 14 | 1 | 240 | 240 |  |
| Eastham | Whispering Pines Condo (Muddy Pond AKA Mill Pond) | Sampling Point | Weekly | 10 |  |  |  |  |
| Eastham | Wiley Park | Sampling Point | Weekly | 14 |  |  |  |  |
| Easton | Town Pool | End of 1st dock | Weekly | 10 |  |  |  |  |
| Egremont | Prospect Lake Park | Sampling Point | Weekly | 14 |  |  |  |  |
| Erving | Laurel Lake (DCR) | Sampling Point | Weekly | 16 |  |  |  |  |
| Essex | Camp Menorah | Sampling Point | Weekly | 9 |  |  |  |  |
| Essex | Centennial Grove | Sampling Point | Weekly | 16 |  |  |  |  |
| Falmouth | Ashumet Valley Holly Sands | Sampling Point | Weekly | 14 |  |  |  |  |
| Falmouth | Cape Cod Camp Resort | Sampling Point | Weekly | 14 |  |  |  |  |
| Falmouth | Coonamessett Pond | Sampling Point | Weekly | 14 |  |  |  |  |
| Falmouth | Grew's Pond | Sampling Point | Weekly | 14 |  |  |  |  |
| Falmouth | Jenkins Pond - Pinecrest | Sampling Point | Weekly | 14 |  |  |  |  |
| Falmouth | Lochstead Association | Sampling Point | Weekly | 14 |  |  |  |  |
| Falmouth | Mares Pond Association | Sampling Point | Weekly | 14 |  |  |  |  |
| Falmouth | Sand Pointe Shores-Rock Hollow | Sampling Point | Weekly | 14 |  |  |  |  |
| Falmouth | Sand Pointe Shores-White Cap | Sampling Point | Weekly | 14 |  |  |  |  |
| Falmouth | Shady Lane HA-Crooked Pond | Sampling Point | Weekly | 13 |  |  |  |  |
| Falmouth | Water-by Estates Association-Flax Pond | Sampling Point | Weekly | 14 |  |  |  |  |
| Florida | Manice Education Center Beach | Sampling Point | Weekly | 10 |  |  |  |  |
| Framingham | Cochituate Beach | Sampling Point | Weekly | 10 |  |  |  |  |
| Framingham | Learned Beach | Sampling Point | Weekly | 11 | 1 | 272 | 272 | 1 |
| Framingham | Washakum Beach | Sampling Point | Weekly | 11 | 2 | 360 | 1600 |  |
| Franklin | Chilson Beach | Sampling Point | Weekly | 8 |  |  |  |  |
| Freetown | Cathedral Camp | Sampling point | Weekly | 8 |  |  |  |  |
| Freetown | Long Pond AKA Town Beach | Sampling Point | Weekly | 17 | 2 | 630 | 990 | 1 |
| Gardner | Dunn Pond (DCR) | Sampling Point | Weekly | 19 | 3 | 77 | 130 | 3 |
| Gardner | Lithuanian Outing Assoc. | Sampling Point | Weekly | 16 |  |  |  |  |
| Georgetown | American Legion Park | Sampling Point | Weekly | 10 |  |  |  |  |
| Georgetown | Camp Leslie | Sampling Point | Weekly | 8 |  |  |  |  |
| Goshen | Camp Holy Cross | Sampling Point | Weekly | 10 |  |  |  |  |
| Goshen | Camp Howe | Sampling Point | Weekly | 9 |  |  |  |  |
| Goshen | Hammond Acres | Sampling Point | Weekly | 11 |  |  |  |  |
| Goshen | Upper Highland Lake - Campers Beach (DCR) | Sampling Point | Weekly | 16 |  |  |  |  |
| Goshen | Upper Highland Lake - Day use area beach (DCR) | Sampling Point | Weekly | 17 | 1 | 71 | 71 |  |
| Grafton | Silver Lake Beach | Sampling Point | Weekly | 11 |  |  |  |  |
| Great Barrington | Eisner Camp | Sampling Point | Weekly | 11 | 1 | 365.4 | 365.4 | 1 |
| Great Barrington | Lake Mansfield | Town beach | Weekly | 12 |  |  |  |  |
| Great Barrington | Seven Stones Beach (KSA) | Sampling Point | Weekly | 11 |  |  |  |  |
| Greenfield | Greenfield Municipal Bathing Beach | Sampling Point | Weekly | 20 | 2 | 288 | 410 |  |
| Groton | Baby Beach Lost Lake | Sampling Point | Weekly | 15 |  |  |  |  |
| Groton | Groton Town Beach (Sargisson Beach) | Sampling Point | Weekly | 18 | 1 | 256 | 256 | 1 |
| Groton | Grotonwood Camp | Sampling Point | Weekly | 8 |  |  |  |  |
| Halifax | Annawon Drive | Sampling Point | Weekly | 16 |  |  |  |  |
| Halifax | Halifax Beach Association | Sampling Point | Weekly | 6 |  |  |  | 3 |
| Halifax | Holmes Street | Sampling Point | Weekly | 15 |  |  |  |  |
| Halifax | Lingan Street | Sampling Point | Weekly | 5 |  |  |  | 3 |
| Halifax | Twin Lakes Condominiums | Sampling Point | Weekly | 16 |  |  |  |  |
| Hanson | Cranberry Cove (aka Camp Kiwanee) | Sampling Point | Weekly | 11 | 1 | 240 | 240 |  |
| Hanson | Ocean Ave. | Sampling Point | Weekly | 13 |  |  |  | 1 |
| Harvard | Camp Green Eyrie | Sampling Point | Weekly | 15 |  |  |  |  |
| Harvard | Harvard Town Beach | Sampling Point | Weekly | 12 |  |  |  |  |
| Harwich | Aunt Edie's Pond | Sandy Shore Way | Weekly | 14 |  |  |  |  |
| Harwich | Buck's Pond | Sampling Point | Weekly | 14 |  |  |  |  |
| Harwich | Clearwater Drive (Great Sands) | 3 | Weekly | 15 | 1 | 404 | 404 |  |
| Harwich | Hinkley's Pond | Sampling Point | Weekly | 14 |  |  |  |  |
| Harwich | John Joseph Pond at Lakeside Terrace (Great Sands) | Lakeside Terrace | Weekly | 11 |  |  |  |  |
| Harwich | John Joseph Pond at Vacation Lane (Great Sands) | 2 | Weekly | 14 |  |  |  |  |
| Harwich | Long Pond Rte 124 | Sampling Point | Weekly | 14 |  |  |  |  |
| Harwich | Long Pond-Cahoon St. | Sampling Point | Weekly | 14 |  |  |  |  |
| Harwich | Long Pond-Long Pond Drive | Sampling Point | Weekly | 14 |  |  |  |  |
| Harwich | Robbins Pond | Sampling Point | Weekly | 14 |  |  |  |  |
| Harwich | Sand Pond | Sampling Point | Weekly | 14 |  |  |  |  |
| Harwich | Seymour Pond | Sampling Point | Weekly | 14 |  |  |  |  |
| Harwich | Skinequit Pond | Sampling Point | Weekly | 14 |  |  |  |  |
| Haverhill | Plugs Pond | Sampling Point | Weekly | 11 |  |  |  |  |
| Heath | Mohawk Estates | Beach | Weekly | 10 |  |  |  |  |
| Hinsdale | Camp Ashmere Beach | Sampling Point | Weekly | 11 |  |  |  |  |
| Hinsdale | Camp Emerson Beach (Fernwood Reservoir) | Sampling Point | Weekly | 5 |  |  |  |  |
| Hinsdale | Camp Emerson Marina | Sampling Point | Weekly | 4 |  |  |  |  |
| Hinsdale | Camp Romaca | Beach | Weekly | 8 |  |  |  |  |
| Hinsdale | Camp Taconic Beach | Sampling Point | Weekly | 10 | 1 | 517.2 | 517.2 |  |
| Hinsdale | Dan Duquette Sports Academy | Sampling Point | Weekly | 9 | 1 | 344.1 | 344.1 |  |
| Holden | Camp Kinneywood Beach | Sampling Point | Weekly | 8 | 1 | 330 | 330 |  |
| Holden | Eagle Lake | Sampling Point | Weekly | 13 | 1 | 490 | 490 |  |
| Holland | Collette Drive Beach | Sampling Point | Weekly | 18 | 2 | 390 | 730 |  |
| Holland | Craig Road Beach | Sampling Point | Weekly | 16 |  |  |  |  |
| Holland | Holland Pond North | Sampling Point | Weekly | 7 |  |  |  |  |
| Holland | Lake Siog Park (USACE) | Sampling Point | Weekly | 13 |  |  |  | 1 |
| Holland | Massaconet Shores | Sampling Point | Weekly | 18 | 2 | 290 | 770 |  |
| Holliston | Pleasure Point | Sampling Point | Weekly | 13 |  |  |  |  |
| Holliston | Stoddard Park | Sampling Point | Weekly | 14 |  |  |  |  |
| Hopkinton | Sandy Beach | Left | Weekly | 9 |  |  |  |  |
| Hopkinton | Sandy Beach | Middle | Weekly | 9 |  |  |  |  |
| Hopkinton | Sandy Beach | Right | Weekly | 9 |  |  |  |  |
| Hubbardston | Comet Pond Beach (DCR) | Middle | Weekly | 14 | 1 | 1100 | 1100 | 1 |
| Hubbardston | Pinecrest Property Owners Assoc. | Sampling Point | Biweekly | 3 |  |  |  |  |
| Hubbardston | Pinecrest Property Owners Assoc. | Sampling Point | Biweekly | 2 | 1 | 120 | 120 |  |
| Hudson | Centennial Beach | Sampling Point | Weekly | 16 | 1 | 1011 | 1011 | 1 |
| Huntington | Timothy Hill Christian Camp | Beach | Once | 1 |  |  |  |  |
| Huntington | YMCA Camp Norwich Beach | Sampling Point | Once | 1 |  |  |  |  |
| Ipswich | Hood Pond-boat ramp | Sampling Point | Weekly | 16 |  |  |  |  |
| Kingston | Camp Morning Star | Sampling Point | Weekly | 9 |  |  |  |  |
| Lakeville | Clear Pond | Sampling Point | Weekly | 11 |  |  |  |  |
| Lakeville | Loon Pond | Sampling Point | Once | 1 |  |  |  |  |
| Lancaster | Camp Lowe Beach | Sampling Point | Weekly | 12 |  |  |  |  |
| Lancaster | JCC Family Camp | Sampling Point | Weekly | 10 |  |  |  |  |
| Lancaster | Lancaster Town Beach | Sampling Point | Weekly | 9 |  |  |  |  |
| Lanesborough | Sunrise Beach | Sampling Point | Weekly | 14 | 1 | 770 | 770 |  |
| Lee | Goose Pond at Leisure Lee Rd. | Sampling Point | Weekly | 16 |  |  |  |  |
| Lee | Lee Town Beach | Sampling Point | Weekly | 13 |  |  |  |  |
| Leicester | Camp Wind-in-the-Pines | Lower beach | Weekly | 11 |  |  |  |  |
| Leicester | Camp Wind-in-the-Pines | Upper beach | Weekly | 10 |  |  |  |  |
| Lenox | Lenox Town Beach (Laurel Lake) | Sampling Point | Weekly | 15 |  |  |  |  |
| Leominster | Ricker's Kindercamp | Sampling Point | Weekly | 9 |  |  |  |  |
| Lexington | Old Reservoir Swim Area #1 | Sampling Point | Weekly | 11 | 1 | 256 | 256 |  |
| Lexington | Old Reservoir Swim Area #2 | Sampling Point | Weekly | 11 | 1 | 256 | 256 |  |
| Littleton | Littleton Town Beach | Sampling Point | Weekly | 16 | 1 | 400 | 400 | 1 |
| Lowell | Merrimac River - Bath House | Sampling Point | Weekly | 15 | 5 | 260 | 10000 | 1 |
| Ludlow | Haviland Pond | Middle of pond | Weekly | 15 |  |  |  |  |
| Lunenburg | Hickory Hills (Brookview) | Sampling Point | Weekly | 16 |  |  |  |  |
| Lunenburg | Hickory Hills (Hemlock Drive) | Sampling Point | Weekly | 16 |  |  |  |  |
| Lunenburg | Hickory Hills (Island Rd.) | Sampling Point | Weekly | 16 |  |  |  |  |
| Lunenburg | Lake Whalom | Sampling Point | Weekly | 16 |  |  |  |  |
| Lunenburg | Lunenburg Town Beach | Sampling Point | Weekly | 9 |  |  |  |  |
| Lunenburg | Shady Point Campground | Sampling Point | Weekly | 18 | 2 | 400 | 400 | 1 |
| Marlborough | Henry F Collins Beach | Sampling Point | Weekly | 12 |  |  |  |  |
| Marlborough | Intercolonial Club of Marlborough | Sampling Point | Weekly | 12 |  |  |  |  |
| Marlborough | McDonald Beach | Sampling Point | Weekly | 12 |  |  |  |  |
| Marlborough | Memorial Beach | [obsolete] Left (Marl.) | Weekly | 6 | 1 | 269 | 269 | 1 |
| Marlborough | Memorial Beach | [obsolete] Right (Marl.) | Weekly | 6 | 1 | 1054 | 1054 | 1 |
| Marlborough | Memorial Beach | Middle (Marl.) | Weekly | 10 |  |  |  |  |
| Marlborough | Richard P. Sharon Beach | Sampling Point | Weekly | 12 |  |  |  |  |
| Marlborough | Roger's Beach | Sampling Point | Weekly | 12 | 1 | 933 | 933 | 1 |
| Mashpee | Attaquin | Sampling Point | Weekly | 14 |  |  |  |  |
| Mashpee | Briarwood Beach | Sampling Point | Weekly | 14 |  |  |  |  |
| Mashpee | Camp Farley - Wakeby Pond | Sampling Point | Weekly | 14 |  |  |  |  |
| Mashpee | Fells Pond (AKA Jim Pond) | Sampling Point | Weekly | 12 |  |  |  |  |
| Mashpee | John's Pond (Fred's Beach) | Tims Beach | Weekly | 10 | 1 | 720 | 720 |  |
| Mashpee | John's Pond (North) | Sampling Point | Weekly | 14 |  |  |  |  |
| Mashpee | John's Pond (Public) | Back Road | Weekly | 14 |  |  |  |  |
| Mashpee | John's Pond (Public) | Brickyard Road | Weekly | 14 |  |  |  |  |
| Mashpee | Mashpee Shores Assoc. | Sampling Point | Weekly | 11 |  |  |  |  |
| Mashpee | Santuit Pond | Bryants Neck | Weekly | 14 |  |  |  |  |
| Mashpee | Santuit Pond | Town Landing | Weekly | 14 |  |  |  |  |
| Mashpee | Santuit Pond Estate Assoc. - Santuit Pond | Sampling Point | Weekly | 14 |  |  |  |  |
| Mashpee | Trustees of the Reservation (Mashpee Pond) | Sampling Point | Weekly | 14 |  |  |  |  |
| Mashpee | Trustees of the Reservation (Wakeby Pond) | Sampling Point | Weekly | 14 |  |  |  |  |
| Medfield | Hinkley | Left | Weekly | 12 |  |  |  |  |
| Medfield | Hinkley | Right | Weekly | 12 |  |  |  |  |
| Medford | Wrights Pond | Deep End | Weekly | 9 |  |  |  |  |
| Medford | Wrights Pond | Shallow End | Weekly | 10 | 1 | 256 | 256 |  |
| Mendon | Mendon Town Beach | Sampling Point | Weekly | 15 | 4 | 340 | 1300 |  |
| Methuen | Forest Lake: Swimming Beach | Sampling Point | Weekly | 11 |  |  |  |  |
| Middleborough | Camp Avoda | Sampling Point | Weekly | 8 |  |  |  |  |
| Middleborough | Camp Yomechas | Sampling Point | Weekly | 16 |  |  |  |  |
| Middleborough | Woods Pond Cabins | Sampling Point | Weekly | 10 |  |  |  |  |
| Middleton | Thunderbridge | Sampling Point | Weekly | 17 | 2 | 380 | 720 | 1 |
| Milton | Houghton's Pond @ Bathhouse (DCR) | Sampling Point | Weekly | 16 |  |  |  |  |
| Monterey | Benedict Pond Beach (DCR) | Sampling Point | Weekly | 16 |  |  |  |  |
| Monterey | Camp Half Moon | Sampling Point | Weekly | 9 |  |  |  |  |
| Monterey | Lake Garfield | Sampling Point | Weekly | 13 | 1 | 648.8 | 648.8 |  |
| Mt. Washington | Camp Hi Rock - Bear Rock Beach | Sampling Point | Weekly | 14 |  |  |  |  |
| Mt. Washington | Camp Hi Rock - Main Beach | Sampling Point | Weekly | 14 |  |  |  |  |
| Nantucket | Miacomet Pond | Sampling Point | Weekly | 12 | 1 | 476 | 476 |  |
| Nantucket | Sesachacha Pond | Sampling Point | Weekly | 11 |  |  |  |  |
| Natick | Camp Arrowhead | Sampling Point | Weekly | 9 |  |  |  |  |
| Natick | Camp Nonesuch | Sampling Point | Weekly | 12 | 1 | 276 | 276 | 1 |
| Natick | Cochituate Lake-North Beach (DCR) | Sampling Point | Weekly | 16 |  |  |  |  |
| Natick | Memorial Beach (Dug Pond) | Diving | Weekly | 12 | 1 | 680 | 680 | 1 |
| Natick | Memorial Beach (Dug Pond) | Wading | Weekly | 11 |  |  |  |  |
| New Marlborough | Camp Wa Wa Segowea | Sampling Point | Weekly | 11 |  |  |  |  |
| Newton | Crystal Lake | Sampling Point | Weekly | 12 |  |  |  |  |
| North Adams | Windsor Lake | Sampling Point | Weekly | 25 | 3 | 365 | 461 | 1 |
| North Andover | Frye Pond Beach (DCR) | Sampling Point | Weekly | 16 |  |  |  |  |
| North Andover | Stevens Pond | Sampling Point | Weekly | 11 | 1 | 250 | 250 |  |
| North Attleborough | Falls Pond | Sampling Point | Weekly | 10 | 2 | 340 | 690 | 1 |
| North Attleborough | Whitings Pond | Sampling Point | Weekly | 11 | 8 | 350 | 2700 | 2 |
| North Brookfield | Brooks Pond | Sampling Point | Weekly | 11 |  |  |  |  |
| North Brookfield | Camp Atwater | Sampling Point | Weekly | 5 |  |  |  |  |
| Northampton | Musante Beach | Sampling Point | Weekly | 17 | 1 | 392 | 392 | 1 |
| Oakham | Lake Dean Dean Campground | Sampling Point | Weekly | 16 | 1 | 500 | 500 |  |
| Oakham | Pine Acres Resort | Sampling Point | Weekly | 14 |  |  |  |  |
| Oakham | Treasure Valley Scout | East | Weekly | 8 |  |  |  |  |
| Oakham | Treasure Valley Scout | West | Weekly | 8 |  |  |  |  |
| Orange | Camp Selah | Sampling Point | Weekly | 4 |  |  |  |  |
| Orange | Orange Town Beach | Beach | Weekly | 10 |  |  |  |  |
| Orleans | Crystal Lake | Sampling Point | Weekly | 16 |  |  |  |  |
| Orleans | Pilgrim Lake | Sampling Point | Weekly | 16 |  |  |  |  |
| Otis | Berkshire Sports Academy | Sampling Point | Weekly | 10 |  |  |  |  |
| Otis | Camp Bonnie Brae | Sampling Point | Weekly | 13 |  |  |  |  |
| Otis | Otis Reservoir Beach (DCR) | Sampling Point | Weekly | 16 |  |  |  |  |
| Otis | Otis Town Beach | Sampling Point | Weekly | 16 | 1 | 2050 | 2050 | 1 |
| Otis | Otis Woodlands Club | Beach | Weekly | 16 | 2 | 272.3 | 304.4 |  |
| Otis | Otis Woodlands Club | Picnic Grove | Weekly | 16 | 2 | 387.3 | 461.1 |  |
| Otis | Otis Woodlands Club | Weir | Weekly | 16 | 2 | 344.8 | 613.1 |  |
| Oxford | Barton Center | Sampling Point | Weekly | 9 |  |  |  |  |
| Oxford | Carbuncle Pond | Sampling Point | Weekly | 12 | 2 | 260 | 650 |  |
| Pembroke | Camp Pembroke | Sampling Point | Weekly | 5 |  |  |  |  |
| Pembroke | Furnace Colony | Sampling Point | Weekly | 14 | 1 | 1600 | 1600 | 1 |
| Pembroke | Little Sandy | Sampling Point | Weekly | 14 |  |  |  |  |
| Pembroke | Oldham | Sampling Point | Weekly | 14 |  |  |  |  |
| Pembroke | Stetson | Sampling Point | Weekly | 14 |  |  |  |  |
| Peru | Camp Danbee | Sampling Point | Weekly | 14 | 1 | 385 | 385 |  |
| Phillipston | Queen Lake Beach | North Beach | Weekly | 17 |  |  |  |  |
| Phillipston | Queen Lake Beach | South Beach | Weekly | 17 |  |  |  |  |
| Pittsfield | Camp Stevenson/Witawentin | Sampling Point | Weekly | 8 |  |  |  |  |
| Pittsfield | Camp Winadu | Sampling Point | Weekly | 10 |  |  |  |  |
| Pittsfield | Country Club of Pittsfield | Sampling Point | Weekly | 16 |  |  |  |  |
| Pittsfield | Lakeside Christian Camp | Sampling Point | Weekly | 11 |  |  |  |  |
| Pittsfield | Lulu Pond Beach (DCR) | Sampling Point | Weekly | 21 | 5 | 74 | 1733 | 5 |
| Pittsfield | Onota Lake - Controy Pavillion | Sampling Point | Weekly | 16 |  |  |  |  |
| Pittsfield | Onota Lake - Decom Beach | Sampling Point | Weekly | 16 | 1 | 400 | 400 |  |
| Pittsfield | Onota Lake - Public Beach at Burbank Park | Sampling Point | Weekly | 16 |  |  |  |  |
| Pittsfield | Pontoosuc Lake - Decom Beach | Public Beach | Weekly | 17 |  |  |  |  |
| Pittsfield | South Pond Farm | Sampling Point | Weekly | 15 |  |  |  |  |
| Pittsfield | The Pines | Pines | Weekly | 16 |  |  |  |  |
| Plainfield | Plainfield /Pond | Sampling Point | Weekly | 16 |  |  |  |  |
| Plymouth | Baird Center (Bloody Pond) | Sampling Point | Weekly | 12 |  |  |  |  |
| Plymouth | Barrett Pond (DCR) | Sampling Point | Weekly | 16 |  |  |  |  |
| Plymouth | Blueberry Hill Camp (Curlew Pond) | Sampling Point | Weekly | 12 |  |  |  |  |
| Plymouth | Camp Bournedale - Great Herring Pond | Sampling Point | Weekly | 5 |  |  |  |  |
| Plymouth | Camp Cachalot | Sampling Point | Weekly | 9 |  |  |  |  |
| Plymouth | Camp Clark YMCA - Hyles Pond | Sampling Point | Weekly | 12 |  |  |  |  |
| Plymouth | Camp Norse | Sampling Point | Weekly | 10 |  |  |  |  |
| Plymouth | Camp Squanto | Sampling Point | Weekly | 9 |  |  |  |  |
| Plymouth | Charge Pond (DCR) | Sampling Point | Weekly | 17 | 1 | 180 | 180 | 1 |
| Plymouth | Clear Pond Condos/Village | Sampling Point | Weekly | 9 |  |  |  |  |
| Plymouth | College Pond Day Use (DCR) | Sampling Point | Weekly | 18 | 2 | 110 | 680 | 1 |
| Plymouth | Curlew Pond (DCR) | Sampling Point | Weekly | 17 | 1 | 220 | 220 | 1 |
| Plymouth | Ellis Haven | Sampling Point | Weekly | 12 |  |  |  |  |
| Plymouth | Fearing Pond (DCR) | Beach 1 | Weekly | 16 |  |  |  |  |
| Plymouth | Fearing Pond (DCR) | Beach 2 | Weekly | 17 | 1 | 170 | 170 | 1 |
| Plymouth | Fresh Pond | Left (aka Mid Pond) | Weekly | 12 |  |  |  |  |
| Plymouth | Fresh Pond | Right (aka End Pond) | Weekly | 12 |  |  |  |  |
| Plymouth | Hedges Pond | Sampling Point | Weekly | 14 |  |  |  |  |
| Plymouth | Indian Head Resort | Sampling Point | Weekly | 12 |  |  |  | 1 |
| Plymouth | Morton Park | Cove (Satellite 2) | Weekly | 14 |  |  |  |  |
| Plymouth | Morton Park | Left (Satellite 1) | Weekly | 14 |  |  |  |  |
| Plymouth | Morton Park | Main (right) | Weekly | 14 |  |  |  |  |
| Plymouth | Pinewood Camp - Crew Dock | Sampling Point | Weekly | 12 |  |  |  |  |
| Plymouth | Pinewood Camp - Main Dock | Sampling Point | Weekly | 12 |  |  |  |  |
| Plymouth | Pinewood Camp - Round Pond | Sampling Point | Weekly | 12 |  |  |  |  |
| Plymouth | Pinewood Lodge Campground | Sampling Point | Weekly | 14 |  |  |  |  |
| Plymouth | Plymouth Estates | Sampling Point | Once | 1 |  |  |  |  |
| Plymouth | Sandy Pond Campground | Sampling Point | Weekly | 14 |  |  |  |  |
| Plymouth | Wind-in-the-Pines Camp | Sampling Point | Weekly | 8 |  |  |  |  |
| Randolph | Ponkapoag Pond | Sampling Point | Weekly | 12 |  |  |  |  |
| Richmond | Camp Russell | Sampling Point | Weekly | 13 |  |  |  |  |
| Richmond | Richmond Shores - East | Sampling Point | Weekly | 13 |  |  |  |  |
| Richmond | Richmond Town Beach | Sampling Point | Weekly | 13 |  |  |  |  |
| Rochester | S.P.E.N.A. Beach | Sampling Point | Weekly | 13 | 1 | 304 | 304 | 1 |
| Rockland | Hartsuff Park | Sampling Point | Weekly | 7 |  |  |  |  |
| Rowe | Rowe Town Beach at Pelham Park | Center AKA Swimming | Weekly | 15 |  |  |  |  |
| Rowe | Rowe Town Beach at Pelham Park | Right | Weekly | 15 |  |  |  |  |
| Royalston | St. Laurent Camp | Sampling Point | Weekly | 16 | 1 | 504 | 504 |  |
| Royalston | St. Laurent Camp | Sampling Point | Weekly | 16 | 4 | 124 | 644 | 4 |
| Royalston | Tully Lake Campground (USACE) | Sampling Point | Weekly | 12 |  |  |  | 2 |
| Russell | H.A. Moses Beach (Scout Reservation) | Sampling Point | Weekly | 12 |  |  |  |  |
| Rutland | Whitehall Pond Beach (DCR) | Sampling Point | Weekly | 16 |  |  |  |  |
| Sandisfield | York Lake Beach (DCR) | Sampling Point | Weekly | 18 | 2 | 67 | 111 | 2 |
| Sandwich | Camp Burgess | Sampling Point | Weekly | 9 |  |  |  |  |
| Sandwich | Camp Good News | Sampling Point | Weekly | 9 |  |  |  |  |
| Sandwich | Camp Hayward | Sampling Point | Weekly | 9 |  |  |  |  |
| Sandwich | Camp Lyndon | Sampling Point | Weekly | 8 |  |  |  |  |
| Sandwich | Dunroamin Park & Cottages | Sampling Point | Weekly | 14 |  |  |  |  |
| Sandwich | Lakefield Farms Trust | Sampling Point | Weekly | 11 |  |  |  |  |
| Sandwich | Lakewood Hills Property Owners Assoc. | Sampling Point | Weekly | 12 |  |  |  |  |
| Sandwich | Lawrence Pond Village | Sampling Point | Weekly | 17 | 3 | 456 | 800 | 1 |
| Sandwich | Peter's Pond RV Park | Main Beach/Beach Rd | Weekly | 11 |  |  |  |  |
| Sandwich | Peter's Pond RV Park | Small Beach/Cove St | Weekly | 13 | 2 | 800 | 800 | 1 |
| Sandwich | Peter's Pond Town Park 1 | Sampling Point | Weekly | 12 | 1 | 400 | 400 |  |
| Sandwich | Rolling Ridge Homeowners Assoc.-Lawrence Pond | Sampling Point | Weekly | 15 | 1 | 656 | 656 | 1 |
| Sandwich | Snake Pond | Sampling Point | Weekly | 12 | 1 | 400 | 400 |  |
| Sandwich | South Shore YMCA - Triangle Pond | Sampling Point | Weekly | 9 |  |  |  |  |
| Sandwich | Wakeby Pond | Sampling Point | Weekly | 12 | 1 | 400 | 400 |  |
| Saugus | Pearce Lake @ Breakheart (DCR) | Sampling Point | Weekly | 16 | 1 | 70 | 70 | 1 |
| Saugus | Peckham Pond @ Camp Nihan (DCR) | Sampling Point | Weekly | 19 | 2 | 75 | 423 | 3 |
| Savoy | North Pond Beach (DCR) | Sampling Point | Weekly | 18 | 2 | 68 | 224 | 2 |
| Sharon | Camp Gannett Beach | Sampling Point | Weekly | 10 |  |  |  |  |
| Sharon | Camp Wonderland Beach | Sampling Point | Weekly | 9 |  |  |  |  |
| Sharon | Community Center Beach | Sampling Point | Twice per week | 20 | 1 | 1600 | 1600 | 1 |
| Sharon | Everwood Day Camp | Sampling Point | Weekly | 10 | 1 | 960 | 960 |  |
| Sharon | Massapoag Yacht Club | Sampling Point | Weekly | 15 |  |  |  |  |
| Sharon | Sharon Town Beach | Central | Weekly | 11 |  |  |  |  |
| Sharon | Sharon Town Beach | Swimming dock | Twice per week | 20 | 1 | 580 | 580 |  |
| Sherborn | Farm Pond | Sampling Point | Weekly | 17 |  |  |  |  |
| Shrewsbury | Sunset Beach | Sampling Point | Weekly | 11 |  |  |  |  |
| Shutesbury | Lake Wyola (DCR) | Sampling Point | Weekly | 18 | 2 | 110 | 130 | 2 |
| Shutesbury | Lake Wyola Association | East | Weekly | 15 |  |  |  |  |
| Shutesbury | Lake Wyola Association | North | Weekly | 15 |  |  |  |  |
| Shutesbury | Lake Wyola Association | West | Weekly | 15 |  |  |  |  |
| Southwick | South Pond Beach - North | Sampling Point | Weekly | 10 | 1 | 410 | 410 | 1 |
| Spencer | Camp Laurel Wood | Sampling Point | Weekly | 15 |  |  |  |  |
| Spencer | Camp Marshall Beach | Sampling Point | Weekly | 14 |  |  |  |  |
| Springfield | Bass Pond | Sampling Point | Weekly | 10 |  |  |  |  |
| Springfield | Camp Wilder | Sampling Point | Weekly | 10 |  |  |  |  |
| Springfield | Five Mile Pond | Sampling Point | Weekly | 10 |  |  |  |  |
| Springfield | Paddle Club | Sampling Point | Weekly | 9 |  |  |  |  |
| Sterling | Lake Waushacum #1 | Sampling Point | Weekly | 10 |  |  |  |  |
| Stockbridge | Beachwood Assoc. | Sampling Point | Weekly | 16 |  |  |  |  |
| Stockbridge | Berkshire Count ry Day School/Eden Hill Sports Day Camp | Sampling Point | Weekly | 11 |  |  |  |  |
| Stockbridge | Camp Mah-kee-nac | Sampling Point | Weekly | 12 |  |  |  |  |
| Stockbridge | Kripalu Beach | Sampling Point | Weekly | 15 |  |  |  |  |
| Stockbridge | Stockbridge Town Beach (Stockbridge Bowl) | Sampling Point | Weekly | 16 |  |  |  |  |
| Stockbridge | Tanglewood | Sampling Point | Weekly | 11 |  |  |  |  |
| Stockbridge | White Pines Condos (Stockbridge Bowl) | Sampling Point | Weekly | 16 |  |  |  |  |
| Stoughton | Ames Pond | Sampling Point | Weekly | 9 |  |  |  |  |
| Stow | Lake Boone | Sampling Point | Weekly | 8 |  |  |  |  |
| Sturbridge | Italian-American Sporting Club | Sampling Point | Weekly | 15 |  |  |  |  |
| Sturbridge | Main Beach - Walker Pond Assoc. | Sampling Point | Weekly | 16 |  |  |  |  |
| Sturbridge | Oak Cove - Walker Pond Assoc. | Sampling Point | Weekly | 16 |  |  |  |  |
| Sturbridge | Pioneer Pond Beach (Outdoor World) | Sampling Point | Weekly | 17 |  |  |  |  |
| Sturbridge | Sturbridge Host Hotel | Sampling Point | Weekly | 17 |  |  |  |  |
| Sturbridge | Sturbridge Recreation - Cedar Pond | Sampling Point | Weekly | 9 |  |  |  |  |
| Sturbridge | The Trail at Big Alum Lake Association Beach | Sampling Point | Weekly | 10 |  |  |  |  |
| Sturbridge | Wells State Park - Walker Pond Beach (DCR) | Sampling Point | Weekly | 16 |  |  |  |  |
| Sturbridge | Yogi Bear Campground | Sampling Point | Weekly | 20 | 6 | 280 | 2420 | 1 |
| Sutton | Camp Blanchard | Sampling Point | Weekly | 12 |  |  |  |  |
| Sutton | King's Campground | Sampling Point | Weekly | 19 |  |  |  |  |
| Sutton | Marions Camp | Sampling Point | Weekly | 13 |  |  |  |  |
| Sutton | Old Holbrook Place | Sampling Point | Weekly | 16 |  |  |  |  |
| Sutton | Sutton Falls Camp | Sampling Point | Weekly | 20 | 2 | 490 | 1400 |  |
| Taunton | Watsons Pond (DCR) | Sampling Point | Weekly | 27 | 8 | 72 | 500 | 7 |
| Templeton | Beamans Pond (DCR) | Sampling Point | Weekly | 28 | 12 | 68 | 1100 | 4 |
| Templeton | Beamans Pond Campground (DCR) | Sampling Point | Weekly | 27 | 11 | 71 | 690 | 3 |
| Templeton | Templeton Fish and Game Club | Sampling Point | Weekly | 16 |  |  |  |  |
| Tolland | Camp Kinderland Beach | Sampling Point | Weekly | 13 |  |  |  |  |
| Tolland | Camp Timbertrails | Sampling Point | Weekly | 13 |  |  |  |  |
| Tolland | Tunxis Club | Sampling Point | Weekly | 16 |  |  |  |  |
| Tolland | Wildwood - Fox Den | Sampling Point | Weekly | 16 |  |  |  |  |
| Tolland | Wildwood - Lakeside | Sampling Point | Weekly | 16 |  |  |  |  |
| Tolland | Wildwood - Main Beach | Sampling Point | Weekly | 16 |  |  |  |  |
| Tolland | Wildwood - Meadow | Sampling Point | Weekly | 16 |  |  |  |  |
| Tolland | Wildwood - Otter Pond Beach | Sampling Point | Weekly | 17 | 1 | 517.2 | 517.2 |  |
| Topsfield | Hood's Pond | Sampling Point | Weekly | 10 |  |  |  |  |
| Townsend | Pearl Hill Pond Beach (DCR) | Sampling Point | Weekly | 23 | 9 | 67 | 690 | 5 |
| Tyngsborough | Berry's Grove Campground | Sampling Point | Weekly | 10 |  |  |  |  |
| Tyngsborough | Tyngsborough Town Beach | Sampling Point | Weekly | 14 | 3 | 310 | 496 |  |
| Tyringham | Tyringham Park Beach | Sampling Point | Weekly | 14 | 1 | 400 | 400 | 1 |
| Upton | Kiwanis Beach | Sampling Point | Weekly | 10 |  |  |  |  |
| Uxbridge | Fairwoods | Sampling Point | Weekly | 16 | 1 | 650 | 650 | 1 |
| Uxbridge | Pout Pond | Sampling Point | Weekly | 15 |  |  |  |  |
| Uxbridge | West Hill Park (USACE) | Sampling Point | Weekly | 20 | 2 | 345 | 1733 | 1 |
| Wales | Cordially Colony | Sampling Point | Weekly | 16 |  |  |  |  |
| Wales | Lakeland Beach | Sampling Point | Weekly | 16 |  |  |  |  |
| Wales | Sichols | Sampling Point | Weekly | 16 | 1 | 530 | 530 | 1 |
| Wales | Wales Town Beach | Sampling Point | Weekly | 16 |  |  |  |  |
| Ware | East Beach - Beaver Lake | Sampling Point | Weekly | 16 |  |  |  |  |
| Ware | South Beach - Beaver Lake | Sampling Point | Weekly | 16 |  |  |  |  |
| Ware | West Beach - Beaver Lake | Sampling Point | Weekly | 17 | 1 | 400 | 400 |  |
| Wareham | Blackmore Pond | Sampling Point | Weekly | 11 | 1 | 800 | 800 |  |
| Wareham | Maple Park Campground | Sampling Point I | Weekly | 17 |  |  |  |  |
| Wareham | Maple Park Campground | Sampling Point II | Weekly | 17 |  |  |  |  |
| Wareham | Shangri-La | Sampling Point | Weekly | 17 | 1 | 744 | 744 |  |
| Wareham | Wareham Lake Shores | Sampling Point | Weekly | 18 | 3 | 280 | 800 | 1 |
| Wareham | White Island Association | Sampling Point | Weekly | 16 |  |  |  |  |
| Warren | Comin's Pond | Sampling Point | Weekly | 15 |  |  |  |  |
| Warwick | Moores Pond Beach | Sampling Point | Weekly | 17 |  |  |  |  |
| Wayland | Wayland Town Beach | Left Shallow | Weekly | 13 |  |  |  |  |
| Wayland | Wayland Town Beach | Right Shallow | Weekly | 13 |  |  |  |  |
| Webster | Beacon Park Condominiums | Sampling Point | Weekly | 14 |  |  |  |  |
| Webster | Birch Island | Sampling Point | Weekly | 14 |  |  |  |  |
| Webster | Colonial Park | Sampling Point | Weekly | 14 |  |  |  |  |
| Webster | Indian Ranch | Sampling Point | Weekly | 17 |  |  |  |  |
| Webster | Kildeer Island (AKA Sandy Shore) | Sampling Point | Weekly | 14 |  |  |  |  |
| Webster | Lakeside | Sampling Point | Weekly | 14 | 1 | 320 | 320 |  |
| Webster | Memorial Beach | Sampling Point 1 | Weekly | 14 |  |  |  |  |
| Webster | Memorial Beach | Sampling Point 2 | Weekly | 14 |  |  |  |  |
| Webster | Nipmuc Cove | Sampling Point | Weekly | 14 |  |  |  |  |
| Webster | Treasure Island Condominiums | Sampling Point | Weekly | 14 |  |  |  |  |
| Wellesley | Morses Beach - Shallow | Sampling Point | Weekly | 12 | 1 | 324 | 324 |  |
| Wellfleet | Duck Pond | Sampling Point | Weekly | 14 |  |  |  |  |
| Wellfleet | Great Pond | Sampling Point | Weekly | 14 |  |  |  |  |
| Wellfleet | Gull Pond - Gull Pond Landing | Sampling Point | Weekly | 14 |  |  |  |  |
| Wellfleet | Gull Pond (2) - Steele Rd. | Sampling Point | Weekly | 14 |  |  |  |  |
| Wellfleet | Higgins Pond | Sampling Point | Weekly | 14 |  |  |  |  |
| Wellfleet | Long Pond | Sampling Point | Weekly | 14 |  |  |  |  |
| Wendell | Ruggles Pond (DCR) | Sampling Point | Weekly | 16 |  |  |  |  |
| Wenham | Gull Pond | Sampling Point | Weekly | 12 |  |  |  |  |
| Wenham | Pleasant Street Pond (Pleasant Pond Beach) | Sampling Point | Weekly | 16 |  |  |  |  |
| West Brookfield | Lake Wickabog - Main Beach | Center | Weekly | 16 |  |  |  |  |
| West Stockbridge | Card Pond Beach | Sampling Point | Weekly | 7 |  |  |  |  |
| West Stockbridge | Crane Lake Camp | Sampling Point | Weekly | 5 |  |  |  |  |
| West Tisbury | Ice House Pond | Sampling Point | Weekly | 16 | 1 | 74.4 | 74.4 |  |
| West Tisbury | Long Cove (fresh) | Sampling Point | Weekly | 13 |  |  |  |  |
| West Tisbury | Seth's Pond | Focus | Weekly | 13 |  |  |  |  |
| West Tisbury | Seth's Pond | Town Beach | Weekly | 19 | 3 | 61.6 | 103.6 |  |
| Westborough | Lake Chauncy Beach #1 | Sampling Point | Weekly | 10 |  |  |  |  |
| Westfield | Kingsley Beach (DCR) | Sampling Point | Weekly | 21 | 8 | 64 | 400 | 4 |
| Westfield | Lamberts Beach (DCR) | Sampling Point | Weekly | 19 | 4 | 90 | 400 | 3 |
| Westford | East Boston Camps - Boys Beach | Sampling Point | Weekly | 17 |  |  |  |  |
| Westford | East Boston Camps - Day Care | Sampling Point | Weekly | 16 |  |  |  |  |
| Westford | East Boston Camps - Girls Beach | Sampling Point | Weekly | 15 |  |  |  |  |
| Westford | Edwards Town Beach | Sampling Point | Weekly | 15 | 1 | 302 | 302 |  |
| Westford | Forge Village Beach | Sampling Point | Weekly | 12 |  |  |  |  |
| Westford | Lakeside Meadows | Sampling Point | Weekly | 16 |  |  |  |  |
| Westford | Marylou's Beach (NIA) | Sampling Point | Weekly | 16 |  |  |  |  |
| Westford | Nabnassett American Legion | Sampling Point | Weekly | 16 |  |  |  |  |
| Westford | Noble's Cove Beach | Sampling Point | Weekly | 17 | 1 | 328 | 328 |  |
| Westford | North Beach (NIA) | Sampling Point | Weekly | 16 |  |  |  |  |
| Westford | NVSR: Camp Pond | Sampling Point | Weekly | 9 |  |  |  |  |
| Westford | NVSR: Function Pond | Sampling Point | Weekly | 16 |  |  |  |  |
| Westford | Sandy Beach (NIA) | Sampling Point | Weekly | 17 | 1 | 400 | 400 |  |
| Westford | Summer Village Main Beach | Sampling Point | Weekly | 16 |  |  |  |  |
| Westminster | Camp Pineshore Bible Camp | End of dock | Weekly | 4 |  |  |  |  |
| Westminster | Crocker Pond | Sampling Point | Weekly | 23 | 1 | 400 | 400 | 1 |
| Westminster | Crow Hill Pond Beach (DCR) | Sampling Point | Weekly | 16 |  |  |  |  |
| Weston | River Day Camp | Sampling Point | Weekly | 13 | 1 | 284 | 284 | 1 |
| Weston | Valley Pond | Swim Pond | Weekly | 16 |  |  |  |  |
| Westwood | Membership Beach | Middle | Weekly | 16 |  |  |  |  |
| Westwood | Membership Beach | North | Weekly | 16 |  |  |  |  |
| Westwood | Membership Beach | South | Weekly | 15 |  |  |  |  |
| Westwood | North Beach | Middle | Weekly | 15 |  |  |  |  |
| Westwood | North Beach | North | Weekly | 15 |  |  |  |  |
| Westwood | North Beach | South | Weekly | 15 |  |  |  |  |
| Whately | Tri-Town Beach | Sampling Point | Weekly | 12 | 2 | 500 | 1000 | 1 |
| Wilbraham | Spec Pond Beach | Sampling Point | Weekly | 10 |  |  |  |  |
| Williamstown | Margaret Lindley Park | Sampling Point | Weekly | 18 | 2 | 238.2 | 307.6 | 2 |
| Wilmington | Wilmington Town Beach | Center | Weekly | 10 |  |  |  |  |
| Wilmington | Wilmington Town Beach | Right | Weekly | 10 |  |  |  |  |
| Winchendon | Lake Dennison State Park (DCR) | Campers Beach | Weekly | 16 |  |  |  |  |
| Winchendon | Lake Dennison State Park (DCR) | Day Use Area | Weekly | 18 | 2 | 64 | 100 | 1 |
| Winchester | Shannon Beach @ Upper Mystic (DCR) | Middle | Weekly | 20 | 3 | 70 | 246 | 4 |
| Winchester | Wedge Pond | Center | Weekly | 14 | 3 | 340 | 600 | 3 |
| Worcester | Bell Pond Beach | Sampling Point | Weekly | 8 | 1 | 348 | 348 |  |
| Worcester | Coes Pond Beach (Mill St.) | Sampling Point | Weekly | 9 | 2 | 408 | 460 |  |
| Worcester | Indian Lake Public Beach (Sherburne Ave) | Sampling Point | Weekly | 8 | 1 | 320 | 320 |  |
| Worcester | Lake Quinsigamond-Lake Park Beach (DCR) | Sampling Point | Weekly | 17 |  |  |  |  |
| Worcester | Lake Quinsigamond-Regatta Point Beach (DCR) | Sampling Point | Weekly | 17 | 1 | 120 | 120 | 1 |
| Worcester | Shore Park | Sampling Point | Weekly | 8 | 1 | 488 | 488 |  |
| Worcester | Smith Pond | Sampling Point | Weekly | 12 | 1 | 410 | 410 |  |
| Wrentham | Lake Pearl Restaurant | Beach Center | Weekly | 16 | 1 | 470 | 470 | 1 |
| Wrentham | Sweatt Beach | Sampling Point | Weekly | 14 | 1 | 260 | 260 | 1 |
| Yarmouth | Big Sandy Pond | Sampling Point | Weekly | 13 |  |  |  |  |
| Yarmouth | Camp Greenough - Boy Scouts | Sampling Point | Weekly | 6 |  |  |  |  |
| Yarmouth | Camp Wingate | Sampling Point | Weekly | 14 |  |  |  |  |
| Yarmouth | Dennis Pond | Sampling Point | Weekly | 16 |  |  |  |  |
| Yarmouth | Flax Pond (Yarmouth/Dennis) | Sampling Point | Weekly | 16 |  |  |  |  |
| Yarmouth | Halcyon Condos | Sampling Point | Weekly | 14 |  |  |  |  |
| Yarmouth | Horse Pond | Sampling Point | Weekly | 24 | 10 | 236 | 800 | 3 |
| Yarmouth | Little Sandy Pond | Sampling Point | Weekly | 16 |  |  |  |  |
| Yarmouth | Long Pond - Indian | Sampling Point | Weekly | 16 |  |  |  |  |
| Yarmouth | Long Pond - Lyman | Sampling Point | Weekly | 16 |  |  |  |  |

Table 15

MA Beaches (2015): Exceedances reported based on the number of days since last rainfall at public and semi-public bathing beaches.

|  | Marine beaches |  |  |  |
| --- | --- | --- | --- | --- |
|  | # Days Since Rain | # Exceedances | % |  |
|  | 0 | 234 | 63.6% |  |
|  | 1 | 37 | 10.1% |  |
|  | 2 | 23 | 6.3% |  |
|  | 3 | 26 | 7.1% |  |
|  | 4 | 11 | 3.0% |  |
|  | 5 | 7 | 1.9% |  |
|  | 6 | 14 | 3.8% |  |
|  | 7 | 6 | 1.6% |  |
|  | 8 | 2 | 0.5% |  |
|  | 9 | 2 | 0.5% |  |
|  | 10 | 5 | 1.4% |  |
|  | 10+ | 1 | 0.3% |  |
|  | Total | 368* | 100.0% |  |
| *Out of 390 bacterial exceedances. Twenty two exceedances had no corresponding rainfall information. |  |  |  |  |
|  | Freshwater beaches |  |  |  |
|  | # Days Since Rain | # Exceedances | % |  |
|  | 0 | 124 | 86.7% |  |
|  | 1 | 55 | 38.5% |  |
|  | 2 | 40 | 28.0% |  |
|  | 3 | 23 | 16.1% |  |
|  | 4 | 13 | 9.1% |  |
|  | 5 | 9 | 6.3% |  |
|  | 6 | 6 | 4.2% |  |
|  | 7 | 4 | 2.8% |  |
|  | 8 | 3 | 2.1% |  |
|  | 9 | 2 | 1.4% |  |
|  | 10 | 2 | 1.4% |  |
|  | 10+ | 4 | 2.8% |  |
|  | Total | 285* | 199.3% |  |
| *Out of 330 bacterial exceedances. Forty five exceedances had no corresponding rainfall information. |  |  |  |  |

Table 16

MA Beaches (2015): Exceedances grouped by bather density at the time of sample collection at public and semi-public bathing beaches.

| Marine beaches |  |  |  |
| --- | --- | --- | --- |
| Bather Density (# people) | # Samples | # Exceedances | % |
| 0-10 | 6,664 | 216 | 3.2% |
| 10-20 | 304 | 2 | 0.7% |
| 20-50 | 129 | 3 | 2.3% |
| >50 | 78 | 3 | 3.8% |
| Not indicated | 875 | 166 | 19.0% |
| Total | 8,050 | 390 | 4.8% |
| Freshwater beaches |  |  |  |
| Bather Density (# people) | # Samples | # Exceedances | % |
| 0-10 | 6,679 | 264 | 4.0% |
| 10-20 | 273 | 16 | 5.9% |
| 20-50 | 102 | 9 | 8.8% |
| >50 | 60 | 11 | 18.3% |
| Not indicated | 768 | 30 | 3.9% |
| Total | 7,882 | 330 | 4.2% |

# FIGURES

This page is intentionally left blank.

# APPENDICES

- Massachusetts State Regulations

- General Laws of Massachusetts

- Massachusetts Beach Act

- Federal BEACH Act

- Background Information for Amendments to Bathing Beach Regulations

- MDPH Beach Sampling Field Data Form

This page is intentionally left blank.

Appendix

A. Massachusetts State Regulations

This page is intentionally left blank.

105 CMR 445.000: MINIMUM STANDARDS FOR BATHING BEACHES (STATE SANITARY CODE, CHAPTER VII)

Section

445.001:	Purpose

445.002:	Authority

445.003:	Citation

445.004:	Scope

445.010:	Definitions

445.020:	Operation

445.030:	Bathing Water Quality

445.031:	Indicator Organisms

445.032:	Collection of Bathing Water Samples

445.033:	Laboratory Analysis and Reporting

445.034:	Bathing Beaches Operated by the Commonwealth

445.035:	Sampling and Analysis at Semi-Public Beaches

445.036:	Public Request for Testing

445.040:	Posting and Reopening Notifications

445.100:	Variance

445.101:	Variance to be in Writing

445.300:	Permit Required to Operate

445.400:	General Administration

445.500:	Severability

445.001: Purpose

The purpose of 105 CMR 445.000 is to protect the health, safety and well-being of the users of bathing beaches, to establish acceptable standards for the operation of bathing water and to establish a procedure for informing the public of any bathing water closures.

445.002: Authority

105 CMR 445.000 is adopted under the authority of M.G.L. c. 111, ss. 3,5S and 127A.

445.003: Citation

105 CMR 445.000 shall be known and may be cited as 105 CMR 445.000: Minimum Standards for Bathing Beaches (State Sanitary Code, Chapter VII).

445.004: Scope

These regulations shall apply to all public and semi-public bathing beaches.

445.010: Definitions

The words, terms or phrases listed below, for the purpose of 105 CMR 445.000, shall be defined and interpreted as follows:

Bathing Beach means the land where access to the bathing water is provided. It shall not mean a swimming pool as defined in 105 CMR 435.000: Minimum Standards for Swimming Pools (State Sanitary Code, Chapter V).

Bathing Water means fresh or salt water adjacent to any public bathing beach or semi- public bathing beach at the location where it is used for bathing and swimming purposes.

Board of Health means the appropriate and legally designated health authority of the community, or other legally constituted governmental unit within the Commonwealth having the usual powers and duties of the board of health of a city or town, or its authorized agent or representative.

Department means the Department of Public Health.

Operator means any person who

(a) alone or jointly or severally with others has legal title to a bathing beach whether or not that person has legal title or control of the bathing water; or

(b) has care, charge or control of such bathing beach as agent or lessee of the owner or an independent contractor.

Person means any individual or any partnership, corporation, firm, association or group, or the Commonwealth, or any of its agencies, authorities or departments or any political subdivisions of the Commonwealth, including municipalities or other legal entity.

Public Bathing Beach means any bathing beach open to the general public, whether or not any entry fee is charged, that permits access to bathing waters.

Semi-Public Bathing Beach means any bathing beach that has common access and/or common use by a group or organization, which includes

- any bathing beach used in connection with a hotel, motel, a manufactured home park, campground, apartment house, condominium, country club, youth club, school, camp or other similar establishment where the primary purpose of the establishment is not the operation of the bathing beach, and where admission to the use of the bathing beach is included in the fee or consideration paid or given for the primary use of the premises.

- any bathing beach used in connection with a neighborhood or residential association

- any bathing beach operated solely for the use of members and guests of an organization that maintains such a bathing beach.

Private Bathing Beach means any bathing beach not considered to be a public or semi-public bathing beach.

Sanitary Survey means a written report, conducted by a Massachusetts Registered Sanitary Engineer, Certified Health Officer or Registered Sanitarian, documenting an examination of the bathing water and contiguous land masses for the purpose of identifying actual or potential sources of microbiological or chemical contamination. The sanitary survey shall also include a description of the water circulation associated with the bathing area, the impact of bather load on the bathing beach area and any natural or artificial physical hazards.

445.020: Operation

No operator shall allow bathing or swimming in bathing water whenever in the opinion of the Board of Health or the Department the bathing water is or may be hazardous or unsafe for bathing or swimming. Bathing and swimming at public and semi-public beaches shall be limited to water areas that meet the requirements of 105 CMR 445.030. Any operator of a public or semi-public bathing beach shall comply with the requirements of 105 CMR 445.000.

- After May 15, 2010 no bathing beach shall be operated without a permanent sign posted at the entrance to each parking lot and/or each entrance to the beach. At minimum, the sign must state the dates of operation, the name and telephone number for the beach operator, permit number, and note that the beach is not monitored for bacteria outside of the specified date range.

- The bathing beach operator is responsible for providing and maintaining the sign required in 105 CMR 445.020 (A).

445.030: Bathing Water Quality

Bathing or swimming shall not be permitted in any bathing water where the quality of the water does not meet the standards established in 105 CMR 445.030(A), 445.030(B), or 445.030(C), and no bathing or swimming shall be allowed when the bathing water is determined by the Board of Health or the Department to be unfit or so subject to contamination as to constitute a menace to health. Bathing or swimming shall not be permitted in bathing waters when:

(A) Physical Quality.

(1) Sludge deposits, solid refuse, floating waste solids, oils, grease or scum are present; or

(2) There are safety hazards including, but not limited to, fast currents, sharp drop-offs or an unstable bottom in the wading area(s) or lack of water clarity.

(B) Bacteriological Quality.

(1) The results of a sanitary survey or other information indicates that sewage or other hazardous substances may be discharged into the bathing water to a degree considered by the Board of Health or the Department to be of public health significance; or

(2) Epidemiological evidence discloses the prevalence of an infectious disease or other health condition which is considered to be related to the use of the bathing water and is considered by the Board of Health or the Department to be of public health significance; or

(3) The bacteriological quality of the bathing water is unacceptable based on the standards specified in 105 CMR 445.031 and the following criteria:

(a) two samples of bathing water, collected on two consecutive days, that both exceed the single sample water quality standard, or one sample of bathing water that exceeds the single sample water quality standard when an additional sample is not collected on the following day; or

(b) one sample of bathing water that exceeds the single

sample standard at beaches where, in two or more of the last four full beach seasons, samples collected on two consecutive days both exceeded the single sample water quality standard; or

(c) any bathing water sample that exceeds the geomean water quality standard.

(C) Oil. Hazardous Materials, or Heavy Metals.

(1) Oil, hazardous materials, or heavy metals are present in excess of surface water quality standards or guidelines established by the United States Environmental Protection Agency or the Massachusetts Department of Environmental Protection.

445.031: Indicator Organisms

(A) For marine water, the indicator organism shall be Enterococci.

(1) No single Enterococci sample shall exceed 104 colonies per 100 ml. and the geometric mean of the most recent five (5) Enterococci levels within the same bathing season shall not exceed 35 colonies per 100 ml.

(B) For fresh water, the indicator organisms shall be E. coli or Enterococci.

(1) No single E. coli sample shall exceed 235 colonies per 100 ml. and the geometric mean of the most recent five E. coli samples within the same bathing season shall not exceed 126 colonies per 100 ml; or

(2) No single Enterococci sample shall exceed 61 colonies per 100 ml. and the geometric mean of the most recent five (5) Enterococci samples within the same bathing season shall not exceed 33 colonies per 100 ml.

445.032 Collection of Bathing Water Samples

(A) Location.

(1) The Board of Health, for public and semi-public bathing beaches that are not operated by the Commonwealth shall approve sampling locations at each bathing beach in its jurisdiction. (2) The Department, for bathing beaches that are operated by the Commonwealth, shall approve sampling locations at each bathing beach in its jurisdiction.

(3) Samples of bathing water shall be taken at locations within areas of greatest bather load.

(4) Additional samples shall also be obtained at any critical location subject to contamination from business developments, dwellings, streams, sewer outfall pipes or other sources.

(5) At locations where there are multiple beach operators within 500 meters of shoreline, the beach operators may designate a single sampling location, known as a surrogate sampling point, which will provide sufficient protection to public health as approved by the local Board of Health. These locations must meet the following criteria:

(a) Bathing beaches must not be physically separated from the surrogate sampling point by natural or man-made formations. These may include:

(I) embayments or peninsulas

(II) streams, rivers, or creeks

(III) jetties or other bounding structures

(IV) stormwater or combined-sewer overflow outfalls

(b) At any time the results of a bacterial test exceed the levels in 105 CMR 445.030, all beach operators using a surrogate sampling point must comply with 445.040.

(c) Each beach operator utilizing a surrogate sampling point will be equally responsible for the costs of testing, monitoring and analysis.

(d) Thirty days prior to the beginning of the beach season, the local Board of Health must notify the Department of the beach operators utilizing a surrogate sampling point, their location, and the location of the surrogate sampling point.

(e) The local Board of Health or the Department may require any or all of the beach operators to discontinue the use of surrogate sampling points at any time the bathing waters are found to be unfit, subject to contamination as to constitute a menace to public health, or do not provide sufficient protection to protect public health.

(B) Sample Collection. Samples shall be obtained in accordance with the procedures recommended by the most recent edition of the Standard Methods for the Examination of Water and Waste Water of the American Public Health Association or as approved by the United States Environmental Protection Agency.

(C) Frequency.

(1) The Board of Health, its agent, or any other authorized person shall collect the bacteriologic samples:

(a) Within the five days immediately preceding the opening of the bathing season; and

(b) At least weekly during the bathing season at a time and day approved by the Board of Health or the Department; and

(c) Prior to reopening a beach after closure due to the presence or suspected presence of any of the conditions specified in 105 CMR 445.030(B).

(2) Testing for oil, hazardous materials, or heavy metals shall only be required if the operator, the Board of Health, or the Department has information indicating possible contamination of the bathing beach or bathing waters from oil, hazardous materials or heavy metals.

(D) Field Data. Physical conditions at the time of sampling shall be noted and recorded on a form provided by the Department.

(E) Personnel. Samples shall be taken by the Board of Health, the Department, their duly authorized representatives or other qualified persons as determined by the Board of Health or the Department.

445.033: Laboratory Analysis and Reporting

(A) Laboratory Analysis. -Laboratory analysis of bathing water as required by 105 CMR 445.000 shall be conducted in accordance with the most recent edition of the Standard Methods for Examination of Water and Waste Water of the American Public Health Association or as approved by the United States Environmental Protection Agency.

(B) Reporting.

(1) Routine Reporting by Operators. Any operator or authorized agent of a public bathing beach, except public bathing beaches operated by the Commonwealth, and any operator or authorized agent of a semi-public bathing beach shall report the certified results of all testing, monitoring and analysis of bathing water to the Board of Health within five (5) days of receipt of the results from the laboratory.

(2) Reporting by Operators of Levels Exceeding the Established Standards. Any operator or authorized agent of a public or semi-public bathing beach shall immediately and in no event later than 12 hours after the results are validated report to the Board of Health the results of all testing, monitoring and analysis of bathing water found to exceed the standards established in 105 CMR 445.030.

(3) Reporting by the Board of Health. The Board of Health or its authorized agent shall report the results of all testing, monitoring and analysis of bathing water to the Department no later than October 31 of each year.

445.034 Bathing Beaches Operated by the Commonwealth

State agencies that own or operate a bathing beach shall conduct or cause to be conducted all testing, monitoring, and analysis of bathing water at such bathing beach in accordance with these regulations. If the results of such testing, monitoring and analysis are found to exceed the standards established in 105 CMR 445.030, state agencies shall immediately, and in no event later than 12 hours, report the results of such testing, monitoring and analysis to the Department and the Board of Health in the community where the bathing beach is located. All other results shall be reported to the Department no later than October 31 of each year.

445.035: Sampling and Analysis at Semi-Public Beaches

(A) The operators of semi-public bathing beaches shall pay for the costs of testing, monitoring and analysis of bathing waters adjacent to such semi-public bathing beaches.

(B) Operators of semi-public bathing beaches may enter into contractual agreements with the Board of Health to have the testing, monitoring and analysis of bathing water conducted by the Board of Health, the Department or other qualified persons as determined by the Board of Health or the Department.

445.036: Public Request for Testing

Any person may request that the Board of Health, or in the case of a bathing beach operated by the Commonwealth, the state agency or the Department, conduct testing, monitoring, and analysis of public and semi-public bathing waters when there is reasonable basis to believe that an alleged violation of 105 CMR 445.000 has occurred. The Board of Health or the Department, as appropriate, shall promptly review such requests and determine whether any such testing, monitoring, and analysis is necessary to ensure the public health and safety of bathing waters.

445.040: Posting and Reopening Notifications

(A) Posting. Whenever the bathing water quality does not meet the requirements of 105 CMR 445.030, 105 CMR 445.032, or after any significant rainstorm at a bathing beach where there has been a history of violations of the water quality requirements contained in 105 CMR 445.030, the Board of Health, its agent, or any other authorized person shall immediately, and in no event later than 24 hours, notify the Department, and post or cause to be posted, a sign, or signs, at the entrance to each parking lot and each entrance to the beach stating:

WARNING! NO SWIMMING

SWIMMING MAY CAUSE ILLNESS

and a graphic depiction of a swimmer in a red circle with a diagonal hatch mark. The sign shall also contain the reason for the warning, the date of the posting and the name and telephone number of the board of health. For conditions solely related to physical hazards, the word “injury” may be substituted for “illness” in the required notification.

(B) Reopening. Prior to reopening bathing water posted due to a violation or an assumption of a violation of the standards established in 105 CMR 445.030(B), the Board of Health, its agent, or any other authorized person shall verify that the certified results of the laboratory analysis are less than the standard specified in 105 CMR 445.031. Prior to reopening bathing water posted due to a violation or an assumption of a violation of the standards established in 105 CMR 445.030(A) or 105 CMR 445.030(C), the Board of Health, its agent, or any other authorized person shall confirm by analytic testing or other verifiable means that conditions no longer constitute a threat to human health or safety. The operator of any state operated bathing beach shall notify the Department and the Board of Health within 24 hours, or the next business day, of the reopening of the bathing water.

445.100: Variance

(A) The Board of Health may grant a variance from the provisions of 105 CMR 445.000 for any public or semi-public bathing beach not operated by the Commonwealth. The Department may grant a variance for any bathing beach operated by the Commonwealth. In granting a variance, the Board of Health and the Department shall review available epidemiological data and a written sanitary survey of the bathing beach, as provided by the operator. The survey shall include:

(1) All possible sources of contamination, both bacterial and chemical, on the watershed tributary to the bathing beach including the location and volume of:

(a) sewage and industrial wastewater discharges;

(b) storm water overflows;

(c) bird and animal populations; and

(d) commercial and agricultural drainage.

(2) The volume and quality of the diluting water, water depth, water surface area, tides and confluence of tributaries, water currents and prevailing winds.

(B) Any variance granted by the Board of Health shall specify the required bacteriological testing schedule, provided that the frequency of bacteriological testing shall not be less than once prior to the bathing season and at least every 30 days thereafter throughout the duration of the bathing season.

(C) Any variance granted by a Board of Health or the Department shall expire:

(1) at any time as determined by the Board of Health or the Department, but in no instance greater than four years, at which time the operator may apply for an extension, or

(2) at any time the results of bacterial testing exceed the levels specified in 105 CMR 445.031.

(D) No variance from the requirement of weekly testing shall be granted until the applicant provides the Board of Health or the Department with water quality data collected for at least two complete and consecutive bathing seasons.

(E) In granting a variance, the Board of Health or the Department must determine that the enforcement of 105 CMR 445.000 would not serve a significant public health purpose and that the granting of the variance will not conflict with the intent and spirit of these minimum standards. Any variance or other modification authorized to be made by these regulations may be subject to such qualification, revocation, suspension, or other expiration as the Board of Health or the Department expresses in its grant. A variance or other modification authorized to be made by this regulation may otherwise be revoked, modified, or suspended in whole or in part, only after the holder thereof has been notified in writing and has been given the opportunity to be heard.

445.101: Variance to be in Writing

(A) Any variance granted by the Board of Health or the Department shall be in writing. Any denial for a variance shall also be in writing and shall contain a brief statement of the reasons for denial. A copy of each variance shall be conspicuously posted for 30 days following its issuance and shall, while it is in effect, be available to the public at all reasonable hours in the office of the clerk of the community, or in the office of the Board of Health and in the case of a variance by the Department, at the Department.

(B) The Board of Health shall submit to the Department a notice of the intent to grant a variance. The Department shall approve, disapprove, or modify the variance within 45 days from receipt thereof. If the Department fails to comment within 45 days, its approval shall be presumed. No alteration of any requirement in these regulations shall be made under any variance until the Department approves it or 45 days has elapsed without comment, unless the Board of Health certifies in writing to the Department that an emergency exists.

445.300:   Permit - Issuance

(A)   Permit Required to Operate.  After May 28, 2010 no person shall commence the operation of, or continue to operate, a bathing beach unless the operator is the holder of a valid permit issued by the Board of Health or the Department.

(B)   Application.  By no later than April 26, 2010, any person currently operating a bathing beach desiring to continue operating said beach shall file a written application for a permit with the Board of Health, on forms prepared by the Department and obtained from the Board of Health.  Any information as required by the Board of Health and payment of any fee required by local bylaw, ordinance or regulation shall accompany the application.

(C)   Permit.  Upon receipt of a completed application form and any applicable fee, the Board of Health shall review the information to determine if the beach meets the criteria established in 105 CMR 445.000. If so, the Board of Health shall make a determination within 30 days for existing applicants or 60 days for new applicants whether to issue a permit to the operator or the proposed operator to operate a bathing beach, on a form provided by the Department.

(D)   Expiration and Renewal of Permit.

(1)   A permit shall expire no later than two years from the date issued.

(2)   A bathing beach permit may be renewed by applying at least 30 days prior to the expiration of the permit.  Renewal application forms prepared by the Department shall be obtained from the Board of Health.

(3)   Upon receipt of a completed renewal application form and any applicable fee, the Board of Health shall issue a renewal permit, provided that the conditions for operation set forth in 105 CMR 445.000 are satisfied. The Board of Health may suspend, revoke, or refuse to renew a permit to an operator who is in repeated non-compliance with 105 CMR 445.000.

(4)   If a permit expires while a timely filed application for renewal is pending, the bathing beach shall continue to operate under the expired permit until a new permit is issued or the renewal application is denied.

445.400: General Administration

The provisions of 105 CMR 400.000 shall govern the administration and enforcement of 105 CMR 445.000.

445.500: Severability

In the event that any section of 105 CMR 445.000 is found to be invalid or unconstitutional, the remaining sections shall not be affected and shall remain in full force and effect. To this end, the provisions of this regulation are hereby declared severable.

This page is intentionally left blank.

Appendix

B. General Laws of Massachusetts

This page is intentionally left blank.

GENERAL LAWS OF MASSACHUSETTS

PART I.

ADMINISTRATION OF THE GOVERNMENT

TITLE XVI.

PUBLIC HEALTH

CHAPTER 111. PUBLIC HEALTH

DUTIES OF THE DEPARTMENT OF PUBLIC HEALTH

Chapter 111: Section 5S Public bathing waters; minimum sanitation standards; testing, monitoring and analysis; regulations

Section 5S. (a) As used in this section, the following words shall have the following meanings:--

“Bathing water”, fresh or salt water adjacent to any public bathing beach or semi-public bathing beach in the commonwealth.

“Department”, the department of public health.

“Public bathing beach”, a beach open to the general public, whether or not an entry fee is charged, that permits access to bathing waters.

“Semi-public bathing beach”, a bathing beach used in connection with a hotel, motel, trailer park, campground, apartment house, condominium, country club, youth club, school, camp or similar establishment where the primary purpose of the establishment is not the operation of the bathing beach, and where admission to the use of the bathing beach is included in the fee paid for use of the premises. A semi-public bathing beach shall also include a bathing beach operated and maintained solely for the use of members and guests of an organization that maintains such a bathing beach.

(b) The department, in consultation with local health officers, shall establish minimum sanitation standards to protect bathing waters from contamination from the following: (1) sludge deposits and solid refuse; (2) floating solid, grease or scum wastes; (3) oil, hazardous material, and heavy metals; and (4) bacteria, including but not limited to, total coliform, fecal coliform and enterococci bacteria.

(c) Such standards shall establish safe levels of human exposure to such contaminants, and shall further incorporate, at a minimum, the following provisions:--

(1) An officer or an agent of a local board of health shall test, monitor and analyze all bathing waters within its municipality. Every local board of health shall report the results from all testing, monitoring and analysis of bathing waters to the department. The department shall establish such reporting requirements and shall keep public records thereof. The department shall issue an annual report on the state of beach water quality using data that has been reported to the department. The department shall make such data available to the public upon written request.

(2) The department shall determine at which sites to conduct testing and monitoring of bathing waters. The department shall consider, but not be limited to, the following factors in determining at which sites to conduct testing and monitoring of bathing waters: (i) prior testing results pursuant to this section for such bathing waters; (ii) the number of people who use the bathing beach annually; and (iii) whether the beach is located adjacent to a storm water drain, sewage, industrial and commercial wastewater discharges, or commercial, industrial and agricultural drains.

(d) The department shall determine at what frequency to conduct testing, monitoring and analysis of bathing waters. Testing, monitoring and analysis shall be conducted on at least a weekly basis during the bathing season, and at such times and under such conditions as shall be sufficient to protect public health and safety. The department may grant a variance from the weekly testing requirement for a public or semi-public bathing beach only where there is a documented history of no sources of pollution, both point and non-point, at the bathing beach, or where such pollution sources at the beach have been fully and completely remediated.

(e) The department shall require the posting of conspicuous warning signs to notify the public whenever there is a threat to human health or safety in bathing waters. Signs shall be posted at locations on the beach that are visible to the public in order to inform the public of the nature of the problem and the possibility of a threat to human health and safety. Signs shall be posted immediately after significant rainstorms at bathing beach locations where there has been a chronic history of violations of the department's minimum sanitation standards for bathing beaches after such rainstorms. When an officer or agent of a local board of health discovers a violation of such minimum sanitation standards, the officer or agent shall notify the department immediately, and in no event not later than 24 hours after such discovery. The local board of health shall also post signs immediately, and in no event not later than 24 hours after such a discovery.

(f) A person may request that a local board of health conduct testing, monitoring and analysis of bathing waters when there is a reasonable basis to believe that an alleged violation of such minimum sanitation standards established by this section has occurred. Local boards of health shall promptly review such requests and determine whether any such testing, monitoring and analysis is necessary to ensure the public health and safety in bathing waters.

(g) The owners of semi-public bathing beaches shall be required to pay for the costs of testing, monitoring and analysis of bathing waters adjacent to such semi-public bathing beaches.

(h) Local boards of health may enter into contractual agreements with owners of semi-public bathing beaches where the local board of health conducts testing, monitoring and analysis of such bathing waters.

(i) A municipality or state agency may adopt sanitation standards and testing, monitoring, and analysis requirements for bathing waters within its jurisdiction that are stricter than the standards adopted by the department. In any case where a municipality or state agency adopts such stricter standards, any warning signs required by this section shall display the results of such stricter standards relative to the standards of the department.

(j) The testing, monitoring and analysis of bathing waters that are under the control of any state agency shall be conducted by that state agency. All such state agencies shall meet the requirements set forth by this section and the regulations promulgated by the department.

(k) The department may, subject to appropriation, award competitive grants to local boards of health in the form of a 50 per cent reimbursement for the testing, monitoring and analysis of bathing waters and to otherwise carry out the provisions of this section and the regulations promulgated there under. The department shall enter into a contractual agreement with a sole provider of testing services to be utilized by any state agency, and which may be utilized by any local board of health, to comply with the provisions of this section.

The department shall also ensure that the provisions of this section and the regulations promulgated there under are implemented in a cost effective manner by encouraging, where possible, regional approaches or other cost effective means of carrying out the purposes of this section.

(l) The department shall enforce the provisions of this section in accordance with the penalty and enforcement provisions of section 127A.

This page is intentionally left blank.

Appendix

C. Massachusetts Beach Act

This page is intentionally left blank.

Chapter 248 of the Acts of 2000

AN ACT RELATIVE TO MINIMUM STANDARDS FOR PUBLIC BATHING WATERS.

Be it enacted by the Senate and House of Representatives in General Court assembled, and by the authority of the same, as follows:

SECTION 1. Chapter 111 of the General Laws is hereby amended by inserting after section 5R the following section:-

Section 5S. (a) As used in this section, the following words shall have the following meanings:-

"Bathing water", fresh or salt water adjacent to any public bathing beach or semi-public bathing beach in the commonwealth.

"Department", the department of public health.

"Public bathing beach", a beach open to the general public, whether or not an entry fee is charged, that permits access to bathing waters.

"Semi-public bathing beach", a bathing beach used in connection with a hotel, motel, trailer park, campground, apartment house, condominium, country club, youth club, school, camp or similar establishment where the primary purpose of the establishment is not the operation of the bathing beach, and where admission to the use of the bathing beach is included in the fee paid for use of the premises. A semi-public bathing beach shall also include a bathing beach operated and maintained solely for the use of members and guests of an organization that maintains such a bathing beach.

(b) The department, in consultation with local health officers, shall establish minimum sanitation standards to protect bathing waters from contamination from the following: (1) sludge deposits and solid refuse; (2) floating solid, grease or scum wastes; (3) oil, hazardous material, and heavy metals; and (4) bacteria, including but not limited to, total coliform, fecal coliform and enterococci bacteria.

(c) Such standards shall establish safe levels of human exposure to such contaminants, and shall further incorporate, at a minimum, the following provisions:-

(1) An officer or an agent of a local board of health shall test, monitor and analyze all bathing waters within its municipality. Every local board of health shall report the results from all testing, monitoring and analysis of bathing waters to the department. The department shall establish such reporting requirements and shall keep public records thereof. The department shall issue an annual report on the state of beach water quality using data that has been reported to the department. The department shall make such data available to the public upon written request.

(2) The department shall determine at which sites to conduct testing and monitoring of bathing waters. The department shall consider, but not be limited to, the following factors in determining at which sites to conduct testing and monitoring of bathing waters: (i) prior testing results pursuant to this section for such bathing waters; (ii) the number of people who use the bathing beach annually; and (iii) whether the beach is located adjacent to a storm water drain, sewage, industrial and commercial wastewater discharges, or commercial, industrial and agricultural drains.

(d) The department shall determine at what frequency to conduct testing, monitoring and analysis of bathing waters. Testing, monitoring and analysis shall be conducted on at least a weekly basis during the bathing season, and at such times and under such conditions as shall be sufficient to protect public health and safety. The department may grant a variance from the weekly testing requirement for a public or semi-public bathing beach only where there is a documented history of no sources of pollution, both point and non-point, at the bathing beach, or where such pollution sources at the beach have been fully and completely remediated.

(e) The department shall require the posting of conspicuous warning signs to notify the public whenever there is a threat to human health or safety in bathing waters. Signs shall be posted at locations on the beach that are visible to the public in order to inform the public of the nature of the problem and the possibility of a threat to human health and safety. Signs shall be posted immediately after significant rainstorms at bathing beach locations where there has been a chronic history of violations of the department's minimum sanitation standards for bathing beaches after such rainstorms. When an officer or agent of a local board of health discovers a violation of such minimum sanitation standards, the officer or agent shall notify the department immediately, and in no event not later than 24 hours after such discovery. The local board of health shall also post signs immediately, and in no event not later than 24 hours after such a discovery.

(f) A person may request that a local board of health conduct testing, monitoring and analysis of bathing waters when there is a reasonable basis to believe that an alleged violation of such minimum sanitation standards established by this section has occurred. Local boards of health shall promptly review such requests and determine whether any such testing, monitoring and analysis is necessary to ensure the public health and safety in bathing waters.

(g) The owners of semi-public bathing beaches shall be required to pay for the costs of testing, monitoring and analysis of bathing waters adjacent to such semi-public bathing beaches.

(h) Local boards of health may enter into contractual agreements with owners of semi-public bathing beaches where the local board of health conducts testing, monitoring and analysis of such bathing waters.

(i) A municipality or state agency may adopt sanitation standards and testing, monitoring, and analysis requirements for bathing waters within its jurisdiction that are stricter than the standards adopted by the department. In any case where a municipality or state agency adopts such stricter standards, any warning signs required by this section shall display the results of such stricter standards relative to the standards of the department.

(j) The testing, monitoring and analysis of bathing waters that are under the control of any state agency shall be conducted by that state agency. All such state agencies shall meet the requirements set forth by this section and the regulations promulgated by the department.

(k) The department may, subject to appropriation, award competitive grants to local boards of health in the form of a 50 per cent reimbursement for the testing, monitoring and analysis of bathing waters and to otherwise carry out the provisions of this section and the regulations promulgated there under. The department shall enter into a contractual agreement with a sole provider of testing services to be utilized by any state agency, and which may be utilized by any local board of health, to comply with the provisions of this section.

The department shall also ensure that the provisions of this section and the regulations promulgated there under are implemented in a cost effective manner by encouraging, where possible, regional approaches or other cost effective means of carrying out the purposes of this section.

(l) The department shall enforce the provisions of this section in accordance with the penalty and enforcement provisions of section 127A.

SECTION 2. The department of public health shall promulgate the regulations required by section 5S of chapter 111 of the General Laws not later than March 1, 2001.

SECTION 3. The division of local mandates, in the office of the state auditor, through the legislative review program, pursuant to the last paragraph of section 6B of chapter 11 of the General Laws, shall make a comprehensive report on sections 1 and 2 of this act. The report shall determine the financial impact on cities and towns of such sections and shall prepare a preliminary cost study and cost benefit analysis. The report shall be filed with the clerk of the House of Representatives not later than December 1, 2000.

SECTION 4. Sections 1 and 2 of this act shall take effect on February 1, 2001.

Approved August 11, 2000.

This page is intentionally left blank.

Appendix

D. Federal Beach Act

This page is intentionally left blank.

This page is intentionally left blank.

Appendix

E. Background Information for Amendments to the Bathing Beach Regulations

This page is intentionally left blank.

The Commonwealth of Massachusetts

Executive Office of Health and Human Services

Department of Public Health

250 Washington Street, Boston, MA 02108-4619

DEVAL PATRICK GOVERNOR JOHN W. POLANOWICZ SECRETARY CHERYL BARTLETT, RN COMMISSIONER

MEMORANDUM

To:	Commissioner Bartlett and Members of the Public Health Council

From:	Suzanne K. Condon, Associate Commissioner

Director, Bureau of Environmental Health

Date:	May 8, 2014

Re:	Request for Final Promulgation of Amendments to 105 CMR 445.000: Minimum Standards for Bathing Beaches (State Sanitary Code, Chapter VII)

INTRODUCTION

The purpose of this memorandum is to summarize public comments received and  responses to comments regarding proposed amendments to 105 CMR 445.000, Minimum Standards for Bathing Beaches (State Sanitary Code, Chapter VII) and to request approval for final promulgation.  The proposed amendments will modify the criteria for when bathing water is considered unacceptable based on water quality standards.  Department staff briefed the Council on the proposed amendments on April 9, 2014.

BACKGROUND

Under current regulations, bathing water quality is considered unacceptable when one sample exceeds the single sample water quality standard.  The proposed amendments would revise this standard as follows:

The bacteriological quality of bathing water will be considered unacceptable based on the following criteria:

- Two samples of bathing water, collected on two consecutive days, that both exceed the single sample water quality standard, or one sample of bathing water that exceeds the single sample water quality standard when an additional sample is not collected on the following day; or

- One sample of bathing water that exceeds the single sample standard at beaches where, in two or more of the last four full beach seasons, samples collected on two consecutive days both exceeded the single sample water quality standard; or

- Any bathing water sample that exceeds the geomean water quality standard.

PUBLIC HEARING AND COMMENT PERIOD

A public hearing was held in Boston on April 28, 2014.  Three people attended the hearing and one person provided written comments that he read at the hearing.  In all, four sets of written comments were received, including the one at the public hearing.  In addition, MDPH/BEH staff were asked by the Public Health Council to evaluate whether there may be a bacterial level at which a Day 1 exceedance at a beach could reliably predict that the Day 2 sample would also be an exceedance.  This memo addresses that question as well as other comments.

PUBLIC COMMENTS AND STAFF RESPONSES

A summary of comments received during the public comment period and staff responses to these comments are included as Attachment A.  In addition, MDPH/BEH reviewed historical beach water quality data to help address the following question raised by a PHC member at the April 14, 2014, meeting:  Based upon a review of historical data, is there a specific Day 1 bacterial level (i.e., a threshold) that could be established where a Day 2 exceedance can be reasonably expected?

Staff reviewed over 46,000 water quality samples taken during the three year period of 2011-2013.  That review revealed that for the beaches that will not be required to post until two consecutive days of exceedances, approximately 97 percent of all samples (marine and freshwater combined) met water quality standards.  At beaches proposed to post after 2 days, second day exceedances represent only 0.2% of the 46,000 samples evaluated.

When evaluating the three years of data to determine if a threshold value may exist, above which an initial exceedance is followed by a confirmatory exceedance on the next day, no clear pattern emerged.  For example, the maximum value for marine beaches was 10,100 cfu/100 ml.  The following day, water sampled at this marine beach was measured at 20 cfu/100 ml. The standard is 104 cfu/100 ml.  Three other samples that exceeded 5,000 cfu/100 ml, were correspondingly low (10 or less cfu/100 ml) the following day.  Similar results were observed at fresh water beaches.

Most public health officials involved with beach water monitoring believe that weather patterns are generally better predictors of high magnitude bacterial counts.  During high magnitude rain events sewage system overflows and runoff from urban areas contribute to increased bacterial counts.  An examination of the small amount of data where bacterial levels continued following a Day 1 exceedance supports this position. For example, following 2.8 inches of rainfall around August 27, 2013 in Southeastern Massachusetts, bacterial counts of 500 and 128 cfu/100 ml, on days one and two, respectively, were observed at Moses Smith Creek Beach, Dartmouth, MA.  Results for four beaches in New Bedford can be explained by the impacts of Tropical Storm Irene on August 29, 2011.

In these types of cases, local health officials already have the option to pre-emptively close beaches due to such concerns. DPH will work with local public health officials to ensure that consideration of these actions is uniform statewide.

CONCLUSION

Department staff have concluded that the proposed amendments do not warrant further modifications based upon comments received.

Department staff request approval for final promulgation of the amendments. Following PHC approval, the Department will file the amendments with the Secretary of the Commonwealth for publication in the Massachusetts Register.

Attachment A

RESPONSE TO COMMENTS

The public comment period for the proposed revisions to 105 CMR 445.000 was held from April 11, 2014, through April 28, 2014.   The comments received during the public comment period were as follows:

List of Commenters

The following individuals or organizations submitted written comments:

- Bruce Berman, Director of Strategy, Communications, and Programs, Save the Harbor/Save the Bay

- Cynthia A. Coffin, R.S., C.H.O, Health Agent, Bourne

- Melissa Gates, Northeast Regional Coordinator, Surfrider Foundation

- Michael J. Hornbrook, Chief Operating Officer, Massachusetts Water Resources Authority

Specific Comments

Comments received as well as a response to specific comments follow:

| Comment | Organization | Department Response |
| --- | --- | --- |
| The proposed change will reduce the number of inaccurate postings, but the vast majority of red flags will continue to fly on days when the water is clean. Requiring beach postings based on yesterday’s test results is a very imperfect way to manage a beach. | Save the Harbor/Save the Bay | We agree that the proposed amendment will significantly reduce the number of inaccurate postings at beaches across the Commonwealth. DPH does not require a flagging system at beaches as a form of notification. |
| Postings based on site specific criteria (e.g., rainfall, tide) and other predictive tools are more accurate than the current testing methodology. | Save the Harbor/Save the Bay, Massachusetts Water Resources Authority | DPH agrees that along with large data sets and recent testing results, modeling of rainfall amounts can be an effective tool. DPH has applied such a system at the annual Charles River swim event in Boston. If the model and/or recent test results suggest elevated bacteria levels, cancellation of the event is recommended. However, given that the amount of data to support modeling is not available for most beaches statewide, most local health officials would not be able to use such a protocol. |
| Due to the limitations in the current testing methodology, more resources should be invested in sanitary surveys and bacteria source tracking and less in daily testing. | Massachusetts Water Resources Authority | Daily testing is not required by DPH regulations. DPH agrees that sanitary surveys and bacteria source tracking are important tools for evaluating bathing water quality. |
| The proposed regulatory amendment appears to prioritize economic impacts over protection of public health/ leave the public more vulnerable to waterborne illness. | Surfrider Foundation | Beaches with known or potential pollution problems will still be required to post after any exceedance. For the beaches that will not be required to post until two consecutive days of exceedances, 97 percent of all samples (marine and freshwater combined) met water quality standards. Additional analyses demonstrated that a second day exceedance was observed in only 0.2% of samples. Most of those beaches were impacted by meteorologic conditions (e.g. rainfall). |
| DPH should consider changing the basis of the geometric mean calculation, as specified in 105 CMR 445.031, from the 5-most recent samples to all samples from the previous 30 days. The 30-day basis represents more of a long-term trend, especially for beaches sampled daily. | Massachusetts Water Resources Authority | DPH will consider modification of the calculation of the geometric mean in future regulatory amendments. |

This page is intentionally left blank.

This page is intentionally left blank.

Appendix

F. MDPH Beach Sampling Data Form

This page is intentionally left blank.

| Town/City of Collection: Date Collected: Collected By: |  |  |  |  | Time Delivered to Lab: Delivered By: Relinquished To: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instructions: Collect sample(s) in areas of greatest bather load and at locations subject to contamination at a uniform depth of 3 feet. Collect samples 12 inches below water surface. Do not collect samples within 6 inches of bottom. |  |  |  |  |  |  |  |  |  |  |  |
| Sample ID | Sample Location (Note beach and sampling location) | Marine or Fresh | Sample Time | Water Clarity | Water Temp (°F) | Days Since Rain (‘0’ if w/in 24 hrs.) | Bather Density (in water) (Circle appropriate # range) |  |  |  | Observations of bathing water |
|  |  |  |  | Clear Cloudy/Murky |  |  | 0-10 | 11-20 | 20-50 | >50 |  |
|  |  |  |  | Clear Cloudy/Murky |  |  | 0-10 | 11-20 | 20-50 | >50 |  |
|  |  |  |  | Clear Cloudy/Murky |  |  | 0-10 | 11-20 | 20-50 | >50 |  |
|  |  |  |  | Clear Cloudy/Murky |  |  | 0-10 | 11-20 | 20-50 | >50 |  |
|  |  |  |  | Clear Cloudy/Murky |  |  | 0-10 | 11-20 | 20-50 | >50 |  |
|  |  |  |  | Clear Cloudy/Murky |  |  | 0-10 | 11-20 | 20-50 | >50 |  |
|  |  |  |  | Clear Cloudy/Murky |  |  | 0-10 | 11-20 | 20-50 | >50 |  |
|  |  |  |  | Clear Cloudy/Murky |  |  | 0-10 | 11-20 | 20-50 | >50 |  |
|  |  |  |  | Clear Cloudy/Murky |  |  | 0-10 | 11-20 | 20-50 | >50 |  |
|  |  |  |  | Clear Cloudy/Murky |  |  | 0-10 | 11-20 | 20-50 | >50 |  |
| Observations: T=Trash WS=Waste Solids SD=Sludge Deposit O=Oils A=Algae F=Fish die-offs J=Jellyfish B=Birds D=Dogs N=None Current Weather Condition: Cloudy/Overcast Sunny Rainy Foggy Windy Air Temp: °F Wind Direction: |  |  |  |  |  |  |  |  |  |  |  |
| Comments: Please Note: This form MUST be utilized upon collection of samples and filled out in its entirety. For reporting purposes, a copy must be submitted to MDPH with any lab results. |  |  |  |  |  |  |  |  |  |  |  |

Cabelli, 1983; USEPA, 1986; Cabelli, 1989; Haile, 1996; Pruss, 1998.

MGL Chapter (C) 111, § Section (S) 5. See Appendix B.

105 CMR 445.000: Minimum Standards for Bathing Beaches (State Sanitary Code, Chapter VII). See Appendix A.

Cabelli et al., 1982; Cabelli, 1983; USEPA, 1986; Cabelli, 1989; Coye and Goldoft,1989; CDC, 1990-2004; Corbett et al., 1993; Haile, 1996; Pruss, 1998.

Pruss, 1998.

Wade et al., 2003; Wade et al., 2006

California, 1997; Gerba, 2000

The Beach Water Quality Locator can be accessed through the main MDPH/BEH website (see cover) and clicking on “Bathing Beaches.” It can also be accessed directly at http://mass.digitalhealthdepartment.com/public_21/index.cfm.