521 CMR 46.00:   GROUP 1 BEDROOMS

46.1	BEDROOMS IN GROUP 1 UNITS

Group 1 units shall provide or be capable of providing, wheelchair turning space as defined in 521 CMR 6.3, Wheelchair Turning Space, clear of the door swing, at one side of the bed in the primary bedroom (based on a full size bed).  Where more than one bedroom is provided, an additional bedroom shall also provide or be capable or providing wheelchair turning space (based on one twin size bed).

46.2	DOORS:

Doors to all bedrooms required to comply  under 521 CMR 46.1,  shall comply with 521 CMR 26.00:  DOORS AND DOORWAYS.

46.3	CLOSETS

All closets in bedrooms required to be accessible shall comply with 521 CMR 9.5.8, Closets.

46.4	ELECTRICAL OUTLETS AND CONTROLS

Shall comply with 521 CMR 39.00:  CONTROLS.

46.5	ALARMS

Shall comply with 521 CMR 40.00:  ALARMS.