# Purpose

This checklist is to provide Facility Managers and their agents (designers or contractors) with a tool to assess whether correction or modification of accessible elements or spaces in existing facilities is required because of planned work.

## Instructions

Complete the checklist with onsite observations. After completing the checklist, fill out and sign the Accessibility Findings document. Attach the Accessibility Scoping form, Accessibility Checklist, and the Accessibility Findings to the study. Incorporate any accessibility improvements needed into the scope of the project and cost estimate. Pictures of elements can be helpful in identifying issues and can be included in the study.

# Requirements

Any alteration, repair or renovation conducted in a public facility must comply with the accessibility requirements of Section 3.3.1 of 521CMR, The Rules and Regulations of the Massachusetts Architectural Access Board (AAB). When the cost of alteration (added to the value of any work conducted in the last three years) exceeds $100,000*, the AAB requires that the building be brought into minimal compliance with 521CMR, by providing at least the following:

- An accessible public entrance

- An accessible toilet room (male and female or a single-user toilet room)

- An accessible water fountain (if one exists)

- An accessible telephone (if one exists)

*NOTE: If the work being performed is limited to alterations of electrical, mechanical or plumbing systems; abatement of hazardous materials; retrofit of sprinkler systems; repair or replacement of roofs or windows; repointing and masonry repair work; alterations to site utilities; or landscaping, it is exempt from accessibility upgrades if the total construction investment (the current project and any work done over the previous three years) is less than $500,000. In addition, any space or element that is altered as part of the alteration must be brought into compliance with 521CMR, if it is not already compliant.

Disclaimer: This tool is intended for DCAMM use only. Not all required accessible elements or spaces are identified for review in this tool. Compliance of elements such as dispensers, mirrors, fire alarms, and other required elements or spaces are assumed to be part of general maintenance and operations review.  The tool is not intended to be a full measure of AAB or ADA compliance. It is not a formal interpretation of 521 CMR or legal advice.

# Project Information

Complete the following basic project information.

| ### Question | ### Answer |
| --- | --- |
| List the project number | Click or tap here to enter text. |
| Building name | Click or tap here to enter text. |
| Building CAMIS ID# | Click or tap here to enter text. |
| Name of person completing this checklist | Click or tap here to enter text. |
| Date of checklist completion | Click or tap here to enter text. |

# Site Arrival and Entryway

Consider the following when choosing an entrance to survey for accessibility:

- Is the entrance a primary, public entrance to the facility?

- If parking is provided, is there accessible parking located within 200’ from the entrance?

- Is there an accessible route from accessible parking to the entry?

- Is there signage directing people to the accessible entry if not all entrances are accessible?

- If there is a ramp along the route to the entry, is it compliant?

Use the box below to describe the entrance in relation to any of the above considerations:

Click or tap here to enter text.

# Accessible Entrance

Evaluate the selected entrance by answering the following questions. A yes answer indicates that the element is compliant. A no answer indicates that the element has deficiencies.

| ### Entrance Location | ### Answer |
| --- | --- |
| Give a name for the entrance to be surveyed. | Click or tap here to enter text. |
| Describe the location of the entrance. | Click or tap here to enter text. |

| ### Entry Door | ### Answer |
| --- | --- |
| Is the door threshold compliant with accessibility requirements? (52CMR, Section 26.10) | Choose an item. |
| Is there adequate maneuvering clearance on both sides of the entry door? (52CMR, Section 26.6) | Choose an item. |
| Is the maneuvering clearance level on both sides? (Slopes at door landings must not exceed 2.0% slope in all directions.) | Choose an item. |
| If the entryway includes a vestibule, is there adequate clearance between the entry doors? | Choose an item. |
| Is the door hardware compliant? | Choose an item. |
| Based on the above questions, does the entry appear to comply fully with accessibility requirements? If not, describe the issue in the text box provided below. | Choose an item. |
| Click or tap here to enter text. |  |

Note: The force required to open doors must comply with MAAB requirements. If this force limit cannot be met, an automatic door opener may be needed.

# Accessible Toilet Room(s)

Select the most accessible pair of public toilet rooms (or a public, gender neutral, single-user toilet room) in the facility to survey.

| ### Toilet Room Selection | ### Answer |
| --- | --- |
| List the toilet room location (floor and room number) | Click or tap here to enter text. |
| Identify the type of room(s) you are surveying: - single-user toilet room - multi-user toilet rooms | Choose an item. |

### Single-User Toilet Room Questions

Evaluate the selected toilet room by answering the following questions. A yes answer indicates that the element is compliant. A no answer indicates that the element has deficiencies. If surveying multi-user toilet rooms, skip this section.

| ### Sign and Room Size | ### Answer |
| --- | --- |
| Is a room designation sign provided on the wall with the International Symbol of Accessibility? | Choose an item. |
| Is the room size a minimum of 90” wide by 72” deep? If not, list the overall dimensions in the text box provided below. | Choose an item. |
| Click or tap here to enter text. |  |

| ### Entry Door | ### Answer |
| --- | --- |
| Is the door threshold compliant with accessibility requirements? (52CMR, Section 26.10) | Choose an item. |
| Is there adequate maneuvering clearance on both sides of the entry door? (52CMR, Section 26.6) | Choose an item. |
| Is the door hardware compliant? | Choose an item. |

| ### Toilet | ### Answer |
| --- | --- |
| Is the toilet 18” on center from the near wall? | Choose an item. |
| - If no, list the dimension. | Click or tap here to enter text. |
| Is the toilet seat height between 17” to 19” high from the floor? | Choose an item. |
| - If no, list the dimension. | Click or tap here to enter text. |
| If a manual flush is provided, is it located on the open side of the toilet? | Choose an item. |
| Are both side and rear grab bars installed at the toilet? | Choose an item. |

| ### Sink | ### Answer |
| --- | --- |
| Is adequate maneuvering clearance provided at the sink (52CMR, Section 30.9.1) | Choose an item. |
| Is adequate knee clearance provided under the sink? (521CMR, Section 30.9.2) | Choose an item. |
| Are insulation or other means of protection provided on sink pipes? | Choose an item. |

### Multi-User Toilet Room Questions (Men’s Toilet)

Evaluate the selected toilet room by answering the following questions. A yes answer indicates that the element is compliant. A no answer indicates that the element has accessibility deficiencies. If surveying a single-user toilet room, skip this section.

| ### Sign and Accessible Stall Size | ### Answer |
| --- | --- |
| Is a room designation sign provided on the wall with the International Symbol of Accessibility? | Choose an item. |
| Does the room have at least one accessible toilet stall with minimum dimensions of 60 inches wide by 72 inches deep? If not, list the dimension in the text box. | Choose an item. |
| - If no, list the dimension. | Click or tap here to enter text. |

| ### Entry Door | ### Answer |
| --- | --- |
| Is the door threshold compliant with accessibility requirements? (52CMR, Section 26.10) | Choose an item. |
| Is there adequate maneuvering clearance on both sides of the entry door? (52CMR, Section 26.9) | Choose an item. |
| Is the door hardware compliant? | Choose an item. |

| ### Stall Door | ### Answer |
| --- | --- |
| Is there adequate maneuvering clearance on both sides of the entry door? (52CMR, Section 26.9) | Choose an item. |
| Is the door hardware compliant? | Choose an item. |
| Is the door self-closing? | Choose an item. |

| ### Toilet | ### Answer |
| --- | --- |
| Is the toilet 18” on center from the near wall? If not, list the dimension in the text box. | Choose an item. |
| - If no, list the dimension. | Click or tap here to enter text. |
| Is the toilet seat height between 17” to 19” high from the floor? If not, list the dimension in the text box. | Choose an item. |
| - If no, list the dimension. | Click or tap here to enter text. |
| If a manual flush is provided, is it located on the open side of the toilet? | Choose an item. |
| Are both side and rear grab bars installed at the toilet? | Choose an item. |

| ### Urinal | ### Answer |
| --- | --- |
| If urinals are provided, is at least one lowered with the rim at 17 inches to 19 inches above the finished floor? | Choose an item. |

| ### Sink | ### Answer |
| --- | --- |
| Is adequate maneuvering clearance provided at the sink (52CMR, Section 30.9.1) | Choose an item. |
| Is adequate knee clearance provided under the sink? (521CMR, Section 30.9.2) | Choose an item. |
| Are insulation or other means of protection provided on sink pipes? | Choose an item. |

### Multi-User Toilet Room Questions (Women’s Toilet)

Evaluate the selected toilet room by answering the following questions. A yes answer indicates that the element is compliant. A no answer indicates that the element has accessibility deficiencies. If surveying a single-user toilet room, skip this section.

| ### Sign and Accessible Stall Size | ### Answer |
| --- | --- |
| Is a room designation sign provided on the wall with the International Symbol of Accessibility? | Choose an item. |
| Does the room have at least one accessible toilet stall with minimum dimensions of 60 inches wide by 72 inches deep? If not, list the dimension in the text box. | Choose an item. |
| - If no, list the dimension. | Click or tap here to enter text. |

| ### Entry Door | ### Answer |
| --- | --- |
| Is the door threshold compliant with accessibility requirements? (52CMR, Section 26.10) | Choose an item. |
| Is there adequate maneuvering clearance on both sides of the entry door? (52CMR, Section 26.9) | Choose an item. |
| Is the door hardware compliant? | Choose an item. |

| ### Stall Door | ### Answer |
| --- | --- |
| Is there adequate maneuvering clearance on both sides of the entry door? (52CMR, Section 26.9) | Choose an item. |
| Is the door hardware compliant? | Choose an item. |
| Is the door self-closing? | Choose an item. |

| ### Toilet | ### Answer |
| --- | --- |
| Is the toilet 18” on center from the near wall? If not, list the dimension in the text box. | Choose an item. |
| - If no, list the dimension. | Click or tap here to enter text. |
| Is the toilet seat height between 17” to 19” high from the floor? If not, list the dimension in the text box. | Choose an item. |
| - If no, list the dimension. | Click or tap here to enter text. |
| If a manual flush is provided, is it located on the open side of the toilet? | Choose an item. |
| Are both side and rear grab bars installed at the toilet? | Choose an item. |

| ### Sink | ### Answer |
| --- | --- |
| Is adequate maneuvering clearance provided at the sink (52CMR, Section 30.9.1) | Choose an item. |
| Is adequate knee clearance provided under the sink? (521CMR, Section 30.9.2) | Choose an item. |
| Are insulation or other means of protection provided on sink pipes? | Choose an item. |

# Accessible Drinking Fountains

If drinking fountains are provided in the building, then both an accessible high and low drinking fountain are required. Select the most accessible pair of drinking fountains in the facility to survey.

| ### Drinking Fountain Location | ### Answer |
| --- | --- |
| Describe the location of the drinking fountains to be surveyed. | Click or tap here to enter text. |

| ### High drinking fountain | ### Answer |
| --- | --- |
| Is adequate maneuvering clearance provided at the drinking fountain (521CMR, Section 36.2) | Choose an item. |
| Is the high drinking fountain protected from being a protruding object either by wing walls, an alcove, or a cane skirt? | Choose an item. |

| ### Low drinking fountain | ### Answer |
| --- | --- |
| Is adequate maneuvering clearance provided at the drinking fountain (521CMR, Section 36.2) | Choose an item. |
| Is the spout located no higher than 36 inches above the finish floor? | Choose an item. |
| Is the bottom edge of the fountain 27" minimum from the floor? (521CMR, Section 36.2.1) | Choose an item. |
| - If no, list the dimension. | Click or tap here to enter text. |

# Accessible Telephone

If public pay telephones are provided in the building, then an accessible telephone and/or a TTY machine may be required. Since pay telephones are no longer standard in most facilities, this tool does not require survey of this element. Instead, the Facility Manager should review and ensure that accessible telephone(s) are provided when necessary. The DCAMM Office of Planning can assist Facility Managers in understanding accessible telephone requirements.