AR 5-401

ADMINISTRATIVE REQUIREMENT MANUAL

EFFECTIVE DATE: June 1, 2026  AUTHORIZATION: Susan Lewis, Director

TITLE: Basic Life Support (BLS) Equipment List

SUPERSEDES: June 16, 2025

General Principles

- REGULATORY AUTHORITY: 105 CMR 170.455, 170.460, 170.470 - AUTHORIZED EQUIPMENT: Ambulance services must carry equipment and medications as required by Statewide Treatment Protocols. Ambulance services should not equip ambulances with equipment that is outside of scope of practice of its EMT employees, or outside of the service’s level of licensure. - PERFORMANCE STANDARDS: All equipment must be designed and constructed to meet medical performance objectives and must not endanger patients. - MAINTENANCE: All equipment and supplies must be maintained and used according to manufacturers’ specifications, including but not limited to, for storage, expiration date and replacement. Services must keep records of preventive maintenance and repairs for all equipment, including service-level and third-party inspection and maintenance records. - SPECIFICATIONS: All equipment must be latex free. Disposable is preferred unless otherwise specified.

| ITEM | # | DESCRIPTION | MIN. SIZE PER ITEM | TOTAL QUAN TITY | CLASS |  |  |  | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | I | II | IV | V |  |

| First-In Kit/Bag | 1.1 | First-In kit items may be incorporated into other units (i.e., portable oxygen unit) and vice versa. Each first-In kit must include the following: |  | 1 | I | II | IV | V | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.2 | Oropharyngeal airways (individually wrapped and sterile) | Adult Pediatric Infant | 2 each | I | II | IV | V | CC |
|  | 1.3 | Small dressings gauze (sterile) | 4" x 4" | 4 | I | II | IV | V | CC |
|  | 1.4 | Medium dressings gauze (sterile) | 5" x 9" | 4 | I | II | IV | V | CC |
|  | 1.5 | Large dressings gauze (sterile) | 10" x 30" | 2 | I | II | IV | V | CC |
|  | 1.6 | Hemostatic dressing approved by the Committee on Tactical Combat Casualty Care |  | 4 | I | II | IV | V | CC |
|  | 1.7 | Soft roller, self-adhering bandage | 4" x 5 yds | 2 | I | II | IV | V | CC |
|  | 1.8 | Bio-occlusive dressing with one-way release valve (individually packaged) |  | 1 | I | II | IV | V | CC |
|  | 1.9 | Cravats/triangular bandages | 40" wide | 4 | I | II | IV | V | CC |
|  | 1.10 | Tourniquets approved by the Committee on Tactical Combat Casualty Care for control of arterial bleeding |  | 2 | I | II | IV | V | CC |
|  | 1.11 | Adhesive tape (2") hypoallergenic | 5 yds | 1 | I | II | IV | V | CC |
|  | 1.12 | Bandage scissors (7") or equivalent |  | 1 | I | II | IV | V | CC |
|  | 1.13 | Sphygmomanometer | Adult Pediatric | 1 each | I | II | IV | V | CC |
|  | 1.14 | Stethoscope |  | 1 | I | II | IV | V | CC |
|  | 1.15 | Penlight-type flashlight |  | 1 | I | II | IV | V | CC |
|  | 1.16 | Sterile water or saline solution (in an unbreakable container) | 500 ml | 1 | I | II | IV | V | CC |

| ITEM | # | DESCRIPTION | MIN. SIZE PER ITEM | TOTAL QUAN TITY | CLASS |  |  |  | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | I | II | IV | V |  |

|  | 1.17 | Bulb syringe (sterile/wrapped) 3oz for irrigation purposes |  | 1 | I | II | IV | V | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.18 | Cold packs (instant) |  | 2 | I | II | IV | V | CC |
|  | 1.19 | Carbon Monoxide monitor (CO monitor) for measuring ambient air |  | 1 | I | II | IV | V | CC |
|  | 1.20 | Adhesive bandages (i.e. Band-Aids) | 3/4" | 6 | I | II | IV | V | CC |
|  | 1.21 | Exam type gloves (disposable only) |  | 2 pairs | I | II | IV | V | CC |
|  | 1.22 | Glucometer (kit with control solution, test strips, antiseptic wipes, small gauze, and Band-Aids) |  | 1 | I | II | IV | V | CC |
|  | 1.23 | Lancets (single use, self-retracting) |  | 8 | I | II | IV | V | CC |
|  | 1.24 | Sharps Container (OSHA approved) |  | 1 | I | II | IV | V | CC |

| ITEM | # | DESCRIPTION | MIN. SIZE PER ITEM | TOTAL QUANTITY | CLASS |  |  |  | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | I | II | IV | V |  |

| Portable | 2.1 | Oxygen cylinder | D or E | 1 | I | II | IV | V | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Oxygen Unit | 2.2 | Oxygen cylinder pressure gauge and regulator capable of delivering a range of 0-25 LPM |  | 1 | I | II | IV | V | CC |
|  | 2.3 | Bag-valve mask (BVM) device with oxygen supply reservoir (hand-operated, single use, with HEPA filter). Pediatric/infant BVMs must include PEEP. Each adult and Pediatric BVM/mask size must be SAME SIZE. For infant BVMs, both infant and neonate masks must be carried. | BVMs: Adult Pediatric Infant Masks for BVMs: Adult Pediatric Infant Neonate | 1 each | I | II | IV | V | CC |
|  | 2.4 | Pulse Oximetry |  | 1 | I | II | IV | V | CC |
|  | 2.5 | Non-Rebreather: High concentration oxygen masks with delivery tubes | Adult Pediatric | 2 each | I | II | IV | V | CC |
|  | 2.6 | Nasal cannula with delivery tube | Adult Pediatric | 2 1 | I | II | IV | V | CC |
|  | 2.7 | Cylinder wrench or wheel secured to unit |  | 1 | I | II | IV | V | CC |
|  | 2.8 | Spare oxygen cylinder (must be accessible from patient compartment, not part of a kit or stretcher. All spare cylinders must be stored in crash stable devices per AR 2-405 minimum specifications for ambulance vehicles.) | D or E | 1 | I | II | IV | V | CC |

| ITEM | # | DESCRIPTION | MIN. SIZE PER ITEM | TOTAL QUANTITY | CLASS |  |  |  | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | I | II | IV | V |  |

| Portable Suction Unit | 3.1 | Portable suction apparatus (battery powered), with charging cable, spare battery, or otherwise maintained in an operational state, capable of delivering minimum vacuum of 500 mm of mercury. A collection bottle (disposable preferred) of at least 250 ml shall be provided with (1) spare of the same size. |  | 1 | I | II | IV | V | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3.2 | Wide bore suction tubing (transparent/translucent, non-kinking) |  | 1 | I | II | IV | V | CC |
|  | 3.3 | Semi-rigid pharyngeal suction tip |  | 1 | I | II | IV | V | CC |
|  | 3.4 | Collection bottle or bag (non-breakable, disposable transparent) | 550 ml | 1 | I | II | IV | V | CC |
|  | 3.5 | Exam-type gloves (disposable only) |  | 1 pair | I | II | IV | V | CC |
|  | 3.6 | French suction catheter | 14 Fr | 1 each | I | II | IV | V | CC |

| Automatic External Defibrillator | 4.1 | AED appropriate to ambulance staffing configuration, with appropriate accessories to provide care in accordance with the Statewide Treatment Protocols and ILCOR/CoSTR standards |  | 1 | I | II | - | V | - |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4.2 | AED Pads (manufacturer recommended) | Adult Pediatric | 2 each | I | II | - | V | - |
|  | 4.3 | Razor (disposable only) |  | 1 | I | II | - | V | - |

| Diagnostic Equipment | 5.1 | Adult Sphygmomanometer | Large-Adult Adult | 1 each | I | II | IV | V | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5.2 | Pediatric Sphygmomanometer | Pediatric Infant | 1 each | I | II | IV | V | CC |
|  | 5.3 | Stethoscope |  | 1 | I | II | IV | V | CC |
|  | 5.4 | Penlight-type flashlight |  | 2 | I | II | IV | V | CC |

| Installed Oxygen System | 6.1 | An installed oxygen system conforming to AR 2-405 minimum specifications for ambulance vehicles. |  | 1 | I | II | IV | - | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 6.2 | Flow meters capable of delivering a range of 0-25 LPM (min.) |  | 2 | I | II | IV | - | CC |

| ITEM | # | DESCRIPTION | MIN. SIZE PER ITEM | TOTAL QUANTITY | CLASS |  |  |  | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | I | II | IV | V |  |

| Installed Suction | 7.1 | Electrically powered suction aspirator system furnished with an illuminated switch, and panel mounted (as required by AR 2-405 minimum specifications for ambulance vehicles) |  | 1 | I | II | IV | - | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 7.2 | Collection bottle or bag (non-breakable, disposable transparent) | 1000 ml capacity | 1 | I | II | IV | - | CC |
|  | 7.3 | Spare collection bags when bag-type system is furnished |  | 4 | I | II | IV | - | CC |
|  | 7.4 | Semi-rigid pharyngeal suction tip |  | 2 | I | II | IV | - | CC |
|  | 7.5 | Wide-bore suction tubing (transparent/translucent, non-kinking) | 1/4" diameter | 2 | I | II | IV | - | CC |
|  | 7.6 | French suction catheters | 6 Fr 8 Fr 14 Fr | 2 each | I | II | IV | - | CC |

| ITEM | # | DESCRIPTION | MIN. SIZE PER ITEM | TOTAL QUANTITY | CLASS |  |  |  | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | I | II | IV | V |  |

| Airway Management | 8.1 | Bag-valve mask (BVM) device with oxygen supply reservoir (hand-operated, single use, with HEPA filter). Pediatric/infant BVMs must include PEEP. Each BVM/mask size must be SAME SIZE; however, for infant BVMs, both infant and neonate masks must be carried. | BVMs: Adult Pediatric Infant Masks: Adult Pediatric Infant Neonate | 1 each | I | II | IV | - | CC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8.2 | Non-Rebreather: High concentration oxygen masks (transparent, disposable) with delivery tubes | Adult Pediatric Infant | 3 each | I | II | IV | - | CC |  |
|  | 8.3 | Nasal cannula with delivery tube | Adult Pediatric | 3 each | I | II | IV | - | CC |  |
|  | 8.4 | Oropharyngeal airways (individually wrapped) | Sizes covering all patients and weight ranges | 2 each | I | II | IV | - | CC |  |
|  | 8.5 | Nasopharyngeal airways (individually wrapped) | 20F 22F 24F 26F 28F 30F 32F 34F | 1 each | I | II | IV | V | CC |  |
|  | 8.6 | Pediatric nasopharyngeal airways (individually wrapped) | 12F 14F 16F 18F | 1 each | I | II | IV | V | CC |  |
|  | 8.7 | Water soluble lubricant (single use package) |  | 12 | I | II | IV | V | CC |  |

| ITEM | # | DESCRIPTION | MIN. SIZE PER ITEM | TOTAL QUANTITY | CLASS |  |  |  | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | I | II | IV | V |  |

| BLS Bronchodilator Kit | 9.1 | Nebulizers with mouthpiece |  | 3 | I | II | - | V | - |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 9.2 | Facemasks | Adult Pediatric | 3 each | I | II | - | V | - |
| CPAP Kit | 10.1 | CPAP device with equipment to do inline nebulizer treatment. High pressure outlets needed at oxygen sources, if required by device | Masks: Small Medium Large | 1 Kit 2 each Masks | I | II | - | V | - |

| Glucometer Kit | 11.1 | Glucometer (with control solution; antiseptic wipes; small gauze; bandages) |  | 1 | I | II | IV | V | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 11.2 | Glucometer test strips |  | 8 | I | II | IV | V | CC |
|  | 11.3 | Lancets (single use, self-retracting) |  | 8 | I | II | IV | V | CC |

| Wound Care | 12.1 | Small dressings gauze (sterile) | 4" x 4" | 4 | I | II | IV | - | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12.2 | Medium dressings gauze (sterile) | 5" x 9" | 4 | I | II | IV | - | CC |
|  | 12.3 | Large dressings gauze (sterile) | 10" x 30" | 2 | I | II | IV | - | CC |
|  | 12.4 | Soft roller, self-adhering bandage | 4" x 5 yds | 2 | I | II | IV | - | CC |
|  | 12.5 | Cravats / triangular bandages | 40" wide | 4 | I | II | IV | - | CC |
|  | 12.6 | Adhesive tape (1") hypoallergenic | 5 yds | 2 | I | II | IV | - | CC |
|  | 12.7 | Adhesive tape (2") hypoallergenic | 5 yds | 2 | I | II | IV | - | CC |
|  | 12.8 | Bandage scissors (7") or equivalent |  | 2 | I | II | IV | - | CC |
|  | 12.9 | Sterile water or saline solution (in an unbreakable container) | 3000cc total | 3 containers | I | II | IV | - | CC |
|  | 12.10 | Bioclusive dressing with 1-way release valve (individually packaged) |  | 1 | I | II | IV | - | CC |
|  | 12.11 | Cold packs (instant) |  | 4 | I | II | IV | - | CC |

| Burn Kit | 13.1 | Burn sheets (sterile) linen or disposable |  | 2 | I | II | IV | V | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| ITEM | # | DESCRIPTION | MIN. SIZE PER ITEM | TOTAL QUANTITY | CLASS |  |  |  | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | I | II | IV | V |  |

| Obstetrical Kit | 14.1 | Obstetrical kit (sterile) containing the following |  | 2 | I | II | IV | V | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 14.2 | Large towel |  | 1 | I | II | IV | V | CC |
|  | 14.3 | Receiving blanket, or equivalent |  | 1 | I | II | IV | V | CC |
|  | 14.4 | Sterile disposable plastic or latex-free gloves |  | 1 pair | I | II | IV | V | CC |
|  | 14.5 | Sterile gauze pads | 3" x 3" | 6 | I | II | IV | V | CC |
|  | 14.6 | Kelly clamps or sterile ties |  | 2 | I | II | IV | V | CC |
|  | 14.7 | Sanitary napkins |  | 6 | I | II | IV | V | CC |
|  | 14.8 | Infant bulb syringe |  | 1 | I | II | IV | V | CC |
|  | 14.9 | Bandage or surgical blade scissors |  | 1 | I | II | IV | V | CC |
|  | 14.10 | Container with lid for carrying placenta |  | 1 | I | II | IV | V | CC |
|  | 14.11 | Newborn swaddle system (i.e., space blanket, foil swaddle, or equivalent to retain body temperature) |  | 1 | I | II | IV | V | CC |
| (Commercial equivalent kit acceptable) |  |  |  |  |  |  |  |  |  |

| ITEM | # | DESCRIPTION | MIN. SIZE PER ITEM | TOTAL QUANTITY | CLASS |  |  |  | CC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | I | II | IV | V |  |  |
| Splints or equivalent immobilization devices | 15.1 | Traction splint (femur) device or equivalent; with ankle hitch and leg ties or equivalent. All accessory items to be stored with splints. | Adult Pediatric | 1 each | I | II | - | - | CC |  |
|  | 15.2 | Upper and Lower extremity immobilization devices (disposable recommended): Must be made of a rigid support material capable of immobilizing all four extremities, as well as the joint above and the joint below the fracture, in sizes appropriate for adults and pediatrics |  | 1 each: Adult arm Adult leg Pediatric arm Pediatric leg | I | II | - | V | CC |  |

| Spinal Immobilization | 16.1 | Full spine board, with easily cleanable 9' straps or functional equivalent and sufficient padding to maintain in-line head & cervical support/stabilization with head strap (2" tape or equivalent) |  | 1 board 4 straps | I | II | - | - | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16.2 | ADULT Cervical Collar (rigid) *Adjustable collar is equivalent to all 4 sizes* | No-Neck Small Medium Large | 2 each | I | II | IV | V | CC |
|  | 16.3 | PEDI Cervical Collar (rigid) *Adjustable collar is equivalent to all 3 sizes* | Pediatric Toddler Infant | 2 each | I | II | IV | V | CC |
| (Accessories must be stored together near boards; all must be antibacterial/impervious to contaminates, or disposable) |  |  |  |  |  |  |  |  |  |

| ITEM | # | DESCRIPTION | MIN. SIZE PER ITEM | TOTAL QUANTITY | CLASS |  |  |  | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | I | II | IV | V |  |

| Stretcher | 17.1 | 4-wheeled, multi-level ambulance cot |  | 1 | I | II | - | - | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 17.2 | Standard cot mattress with waterproof cover |  | 1 | I | II | - | - | CC |
|  | 17.3 | Patient restraining devices at chest (commercial shoulder harness or equal) hip, and knee to prevent lateral or longitudinal displacement of the patient during transport |  | 1 set each | I | II | - | - | CC |
|  | 17.4 | IV holder, capable of being cot mounted and holding 2 IV bags |  | 1 | I | II | - | - | CC |
|  | 17.5 | Padded wrist and ankle restraints |  | 1 set | I | II | - | - | CC |
|  | 17.6 | For power- type units, one extra battery, and charger (as recommended by manufacturer) |  | 1 | I | II | - | - | CC |

| Stair Chair | 18.1 | Stair chair with patient restraint straps |  | 1 | I | II | - | - | - |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 18.2 | For power-type units, one extra battery, and charger (as recommended by manufacturer) |  | 1 | I | II | - | - | - |

| Auxiliary Stretcher | 19.1 | Auxiliary stretcher with patient restraint straps, or equivalent (i.e., orthopedic "scoop" stretcher, "Reeves"- type stretcher, long spine board) |  | 1 | I | II | - | - | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Transfer Sheet | 20.1 | Transfer sheet with a minimum of six (6) handles, or equivalent |  | 1 | I | II | - | V | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Accessories | 21.1 | Adult bed pan urinal, large basin or equivalent |  | 1 each | I | II | IV | - | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 21.2 | Motion sickness bags, or equivalent, capable of being sealed |  | 2 | I | II | IV | V | CC |
|  | 21.3 | Sheets (disposable or linen) |  | 4 (I, II, CC) 1 (IV, V) | I | II | IV | V | CC |
|  | 21.4 | Blankets |  | 4 (I, II, CC) 1 (IV, V) | I | II | IV | V | CC |
|  | 21.5 | Towels |  | 4 | I | II | IV | V | CC |

| ITEM | # | DESCRIPTION | MIN. SIZE PER ITEM | TOTAL QUANTITY | CLASS |  |  |  | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | I | II | IV | V |  |

|  | 21.6 | Paper tissues (disposable) |  | 1 package | I | II | IV | - | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 21.7 | Drinking cups (disposable, individually wrapped) |  | 2 | I | II | IV | - | CC |
|  | 21.8 | Trash bin liners, plastic (with ties) |  | 2 | I | II | IV | - | CC |
|  | 21.9 | Biohazard bags (with ties) |  | 2 | I | II | IV | - | CC |
|  | 21.10 | Ring Cutter (with extra blade) |  | 1 | I | II | IV | - | CC |
|  | 21.11 | CPR board or functionally equivalent (i.e., short board or back plate of a mechanical CPR device): must have hard washable surface for patient torso and be accessible in patient compartment |  | 1 | I | II | IV | - | CC |
|  | 21.12 | Pediatric appropriate restraint device or combination of devices to provide safe transport of an infant/pediatric patient ˃2.3kg on stretcher |  | 1 | I | II | IV | - | CC |

| Personal | 22.1 | Exam-type gloves (disposable only) | Small | 1 box | I | II | IV | V | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Protection |  |  | Medium | each |  |  |  |  |  |
|  |  |  | Large |  |  |  |  |  |  |
|  |  |  | X-Large |  |  |  |  |  |  |
|  | 22.2 | NIOSH approved N95 Mask or better (disposable only) | All sizes per manufacturer | 3 each | I | II | IV | V | CC |
|  | 22.3 | Facemask and protective eye wear |  | 4 each | I | II | IV | V | CC |
|  |  | (combination face mask/eye shield |  |  |  |  |  |  |  |
|  |  | equivalent) |  |  |  |  |  |  |  |
|  | 22.4 | Antiseptic hand sanitizer dispenser or 25 |  | 1 | I | II | IV | V | CC |
|  |  | individually wrapped antiseptic hand wipes |  |  |  |  |  |  |  |
|  | 22.5 | Infection control kit, containing two (2) each |  | 1 kit | I | II | IV | V | CC |
|  |  | of disposable, fluid resistant gowns, masks, |  |  |  |  |  |  |  |
|  |  | caps, protective eye wear, and two (2) |  |  |  |  |  |  |  |
|  |  | different sizes of gloves |  |  |  |  |  |  |  |

| Safety/Access Equipment | 23.1 | ANSI vest or equivalent (Class II/III) |  | 2 | I | II | IV | V | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| ITEM | # | DESCRIPTION | MIN. SIZE PER ITEM | TOTAL QUANTITY | CLASS |  |  |  | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | I | II | IV | V |  |

| Vehicle Equipment | 24.1 | Warning Lights: Emergency warning beacon, visible 360 degrees, as permitted by M.G.L. c.90, s.7, or as required per AR 2-405 minimum specifications for ambulance vehicles |  |  | I | II | IV | V | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 24.2 | Audible Warning Device: A siren, per AR 2-405 minimum specifications for ambulance vehicles & AMD Standard 023 |  |  | I | II | - | V | CC |
|  | 24.3 | Global positioning system (GPS) navigation system, and street directories and road maps for primary and backup areas served |  |  | I | II | IV | V | CC |
|  | 24.4 | Fire Extinguishers: CO2 or dry pounder, UL approved. One fire extinguisher must be accessible from the patient compartment | 5 lbs. | 2 (I, II, CC) 1 (IV, V) | I | II | IV | V | CC |
|  | 24.5 | Hand lights: (6-volt/4.5-volt if rechargeable) bulb or LED or equivalent |  | 2 | I | II | IV | V | CC |
|  | 24.6 | Road Reflectors (US DOT approved triangles or equivalent) |  | 3 | I | II | - | V | CC |
|  | 24.7 | US DOT Emergency Response Guidebook (current edition) or electronic equivalent |  | 1 | I | II | IV | V | CC |
|  | 24.8 | National Institute of Occupational Health & Safety (NIOSH) pocket guide to Chemical Hazards (current edition) or electronic equivalent |  | 1 | I | II | IV | V | CC |
|  | 24.9 | Binoculars | 7 x 35mm | 1 | I | II | IV | V | CC |
|  | 24.10 | SMART triage tags |  | 25 | I | II | IV | V | CC |
|  | 24.11 | Additional protective equipment to adequately safeguard crew from anticipated exposures as defined by the licensee |  |  | I | II | IV | V | CC |

Optional Equipment: Required only for services that have implemented supraglottic airway (SGA) as an EMT-Basic intervention in cardiac arrest. Capnography must be immediately available once the SGA is placed.

| ITEM | # | DESCRIPTION | MIN. SIZE PER ITEM | TOTAL QUANTITY | CLASS |  |  |  | CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | I | II | IV | V |  |

| Supraglottic Airway Kit | 25.1 | Supraglottic Airway | Sizes covering adult and large adult patients per manufacturer | 1 each | I | II | - | V | - |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 25.2 | Syringe for inflation of cuff (if required) |  | 1 | I | II | - | V | - |
|  | 25.3 | Lubricant (water soluble) |  | 1 | I | II | - | V | - |
|  | 25.4 | Commercial securing device (or equivalent) |  | 1 | I | II | - | V | - |
|  | 25.5 | Capnography waveform device |  | 1 | I | II | - | V | - |