AR 5-402

ADMINISTRATIVE REQUIREMENT MANUAL

EFFECTIVE DATE: June 1, 2026  AUTHORIZATION: Susan Lewis, Director

TITLE: Advanced Life Support (ALS) Equipment List

SUPERSEDES: June 16, 2025

General Principles

- REGULATORY AUTHORITY: 105 CMR 170.455, 170.460, 170.470 - AUTHORIZED EQUIPMENT: Ambulance services must carry equipment and medications as required by Statewide Treatment Protocols. Ambulance services should not equip ambulances with equipment that is outside of scope of practice of its EMT employees, or outside of the service’s level of licensure. - PERFORMANCE STANDARDS: All equipment must be designed and constructed to meet medical performance objectives and must not endanger patients. - MAINTENANCE: All equipment and supplies must be maintained and used according to manufacturers’ specifications, including but not limited to, for storage, expiration date and replacement. Services must keep records of preventive maintenance and repairs for all equipment, including service-level and third-party inspection and maintenance records. - SPECIFICATIONS: All equipment must be latex free. Disposable is preferred unless otherwise specified.

| ITEM | # | DESCRIPTION | MIN. SIZE PER ITEM | TOTAL QUANTITY | LEVEL |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | AEMT | Paramedic |
| Advanced Airway Management Kit | 1.1 | Supraglottic Airway | Sizes covering all patients and weight ranges | 1 each | AEMT | Paramedic |
|  | 1.2 | Endotracheal Tubes (sterile, disposable only) | 2.0 2.5 3.0 3.5 3.0 3.5 4.0 4.5 5.0 5.5 6.0 6.5 7.0 7.5 8.0 8.5 9.0 | 2 each | - | Paramedic |
|  | 1.3 | Laryngoscope blades (must be appropriate for specific device and individually wrapped; must also be disposable, or reusable blades if carried must be reprocessed and maintained according to manufacturers’ and CDC guidelines) | Curve-2 Curve-3 Curve-4 Straight-1 Straight-2 Straight-3 Straight-4 | 1 each | - | Paramedic |
|  | 1.4 | Laryngoscope handle or high definition optical equivalent or video type (including, if applicable, spare batteries, bulbs) | Large Small | 1 each | - | Paramedic |
|  | 1.5 | Lubricant (water soluble, for airway insertion) |  | 2 | AEMT | Paramedic |

| ITEM | # | DESCRIPTION | MIN. SIZE PER ITEM | TOTAL QUANTITY | LEVEL |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | AEMT | Paramedic |

| Advanced Airway Management Kit (continued) | 1.6 | Waveform Capnography (EtCO2) Device capable of printing and/or recording EtCO2 tracings (may be incorporated into monitor) |  | 1 | AEMT | Paramedic |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.7 | Nasal Cannula for CO2 sampling | Adult Pediatric | 2 each | AEMT | Paramedic |
|  | 1.8 | Straight T-Connector (15mm) for CO2 sampling |  | 2 | AEMT | Paramedic |
|  | 1.9 | Stylet (sterile) | 14F 6F | 2 each | - | Paramedic |
|  | 1.10 | Gum bougie introducer device (sterile) |  | 2 | - | Paramedic |
|  | 1.11 | Syringe (sterile, larger may be packaged with supraglottic airways) | 10cc 30cc | 2 2 | AEMT | Paramedic |
|  | 1.12 | Commercial tube holder (or equivalent) | Adult Pediatric | 2 each | - | Paramedic |
|  | 1.13 | Nasogastric tubes (sterile) | 10F 14F | 2 each | - | Paramedic |
|  | 1.14 | Catheter-tip syringe (sterile) | 60mL | 1 | - | Paramedic |
|  | 1.15 | Hemostat |  | 1 | - | Paramedic |
|  | 1.16 | Magill Forceps | Adult Pediatric | 1 each | - | Paramedic |

| ITEM | # | DESCRIPTION | MIN. SIZE PER ITEM | TOTAL QUANTITY | LEVEL |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | AEMT | Paramedic |

| Intravenous Administration | 2.1 | Catheters (sterile, safety catheters) | 14ga 16ga 18ga 20ga 22ga 24ga | 5 each | AEMT | Paramedic |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2.2 | Macro administration sets (adjustable rate set acceptable) | 10gtt | 4 | AEMT | Paramedic |
|  | 2.3 | Micro administration sets | 60gtt | 4 | AEMT | Paramedic |
|  | 2.4 | Venous constricting bands |  | 4 | AEMT | Paramedic |
|  | 2.5 | Padded arm board | Pediatric | 1 | AEMT | Paramedic |
|  | 2.6 | Syringes (sterile) | 1cc, 3cc, 10cc, 30cc | 3 each | AEMT | Paramedic |
|  | 2.7 | Antiseptic wipes (sterile, individual) |  | 12 | AEMT | Paramedic |
|  | 2.8 | Bio-occlusive IV dressing |  | 6 | AEMT | Paramedic |
|  | 2.9 | 3-way stop-cock (for medication administration) |  | 2 | AEMT | Paramedic |
|  | 2.10 | Needles (sterile) | 18ga 21ga 23ga 25ga Filter needles for ampules | 5 each | AEMT | Paramedic |
|  | 2.11 | Intraosseous (automatic, drill-type) |  | 1 | AEMT | Paramedic |
|  | 2.12 | Intraosseous needles (sterile) | 15mm 25mm 45mm | 2 each | AEMT AEMT AEMT | Paramedic Paramedic Paramedic |
|  | 2.13 | Pressure infuser |  | 2 | AEMT | Paramedic |
|  | 2.14 | IV extension set |  | 2 | AEMT | Paramedic |

| ITEM | # | DESCRIPTION | MIN. SIZE PER ITEM | TOTAL QUANTITY | LEVEL |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | AEMT | Paramedic |

| Cardiac Monitor | 3.1 | Defibrillator | required technology |  | AEMT (AED) | Paramedic |
| --- | --- | --- | --- | --- | --- | --- |
|  | 3.2 | Cardiac Monitoring | required technology |  | - | Paramedic |
|  | 3.3 | Pacing (transcutaneous) | required technology |  | - | Paramedic |
|  | 3.4 | Cardioversion (synchronized and unsynchronized) | required technology |  | - | Paramedic |
|  | 3.5 | Adjustable joule setting | required technology |  | - | Paramedic |
|  | 3.6 | 12-Lead capability | required technology |  | - | Paramedic |
|  | 3.7 | Ability to print and save | required technology |  | - | Paramedic |
|  | 3.8 | Pads (hands-free, manufacturer recommended) | Adult Pediatric | 2 each | AEMT | Paramedic |
|  | 3.9 | Electrodes | Adult Pediatric | 36 each | - | Paramedic |
|  | 3.10 | Razor (disposable only) |  | 2 | AEMT | Paramedic |
|  | 3.11 | Spare ECG paper (roll or equivalent) |  | 2 | - | Paramedic |
|  | 3.12 | Spare batteries (3/4 - full charge) |  | 2 | - | Paramedic |
|  | 3.13 | Spare set of all necessary cables (including monitoring and therapy cables) |  | 1 | - | Paramedic |

| ITEM | # | DESCRIPTION | MIN. SIZE PER ITEM | TOTAL QUANTITY | LEVEL |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | AEMT | Paramedic |

| Other ALS Equipment | 4.1 | Nebulizer (mouthpiece, T-piece) | Adult Pediatric | 2 each | AEMT | Paramedic |
| --- | --- | --- | --- | --- | --- | --- |
|  | 4.2 | Pediatric age-, weight- or length-based medication and/or dosing reference (most current version) |  | 1 | AEMT | Paramedic |
|  | 4.3 | Needle decompression kit or equivalent (sterile) | 3 ¼" | 2 | - | Paramedic |
|  | 4.4 | Pulse-Oximetry device (may be incorporated into monitor) |  | 1 | AEMT | Paramedic |
|  | 4.5 | Pulse-Oximetry sensors | Adult Pediatric | 2 each | AEMT | Paramedic |
|  | 4.6 | Morgan Lenses eye shield |  | 2 | - | Paramedic |
|  | 4.7 | Nasal Atomizers (in addition to BLS requirement) |  | 2 | AEMT | Paramedic |
|  | 4.8 | Portable CMED radio or other portable two-way communications system to access medical direction |  | 1 | AEMT | Paramedic |

| ITEM | # | DESCRIPTION | MIN. SIZE PER ITEM | TOTAL QUANTITY | LEVEL |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | AEMT | Paramedic |

| IV Infusion Pump Required only if carrying Norepinephrine (must meet the following minimum requirements) | 5.1 | - FDA approved, and not excluded from transport use by manufacturer’s instructions. - Contain drug library with adult and pediatric dosing calculator - Manual programming capabilities for medications outside the device’s own library and limits. - Capable of infusing one medication (channel) at a time. - Must use needle-less administration sets - Capable of operating on AC/DC and battery power |  | 1 |  | Paramedic |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5.2 | Infusion sets for specific infusion pump. |  | 4 |  | Paramedic |
| Note: Device must be appropriately secured in ambulance during storage and use |  |  |  |  |  |  |

Optional Equipment: Required Only for Services That Have Implemented Medical Director Options Under the Statewide Treatment Protocols

| ITEM | # | DESCRIPTION | MIN. SIZE PER ITEM | TOTAL QUANTITY | LEVEL |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | AEMT | Paramedic |

| Needle Cricothyrotomy Kit | 6.1 | Betadine or alcohol prep pads |  | 4 |  | Paramedic |
| --- | --- | --- | --- | --- | --- | --- |
|  | 6.2 | 12-14 gauge angiocath, 8.5cm, over the needle catheter with 10ml syringe with 3ml of saline |  | 2 each |  | Paramedic |
|  | 6.3 | 3mm adaptor |  | 2 |  | Paramedic |
|  | 6.4 | High pressure jet ventilator device with oxygen tubing (50psi) |  | 1 |  | Paramedic |
|  | 6.5 | Securing device |  | 2 |  | Paramedic |
|  | 6.6 | As alternative to 6.1-6.5, equivalent commercially prepackaged kit |  | 2 |  | Paramedic |
| Surgical Cricothyrotomy Kit | 7.1 | #15 Scalpel Blade |  | 2 |  | Paramedic |
|  | 7.2 | 10ml syringe |  | 2 |  | Paramedic |
|  | 7.3 | #6.0 Cuffed Endotracheal Tube |  | 2 |  | Paramedic |
|  | 7.4 | Flexible Guidewire device |  | 2 |  | Paramedic |
|  | 7.5 | Securing device |  | 2 |  | Paramedic |
|  | 7.6 | Sterile gauze for bleeding control |  | 4 |  | Paramedic |
|  | 7.7 | Betadine or other skin cleanser |  | 2 |  | Paramedic |
|  | 7.8 | As alternative to 7.1-7.7, equivalent commercially prepackaged kit |  | 1 |  | Paramedic |
| Transport Ventilator | 8.1 | Ventilator machine set up with appropriate tubing designated oxygen tank and the ability to do in-line nebulizer treatments. Must include appropriately sized masks. |  | 1 |  | Paramedic |
| Ultrasound | 9.1 | Ultrasound machine set up with appropriate probes and conductive gel |  | 1 |  | Paramedic |

| ITEM | # | DESCRIPTION | MIN. SIZE PER ITEM | TOTAL QUANTITY | LEVEL |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | AEMT | Paramedic |

| Blood | 10.1 | All specific equipment required by the supplying blood bank, to administer blood in accordance with Statewide Treatment Protocols. |  |  |  | Paramedic |
| --- | --- | --- | --- | --- | --- | --- |
|  | 10.2 | Infusion sets for specific blood administration. |  | 4 |  | Paramedic |
| Note: All blood products and equipment must be packaged and stored separately from other vehicle equipment. |  |  |  |  |  |  |