- Drinking Water at Local Fairs

Massachusetts Department of Environmental Protection's Drinking Water Program (MassDEP) recommends the following procedures to help boards of health provide oversight of private water supply sources used for public drinking water at local, county, and state fairs. Currently, these sources are not regulated by MassDEP as a public water source but are regulated by the Massachusetts Department of Public Health and Local Boards of Health.

The recommendations listed below should be considered part of the local permitting process for these events. They are intended to help prevent the type of waterborne disease outbreak experienced at a New York county fair in the fall of 2000. That outbreak resulted in death and numerous illnesses.

- A buffer area of at least 100 feet should be established around wells. No animal shelters, animal feed lots, animal waste disposal, septic systems, portable sanitary facilities, chemical or fuel storage, or vehicle parking should occur within this buffer zone.

- Event operators should work with a certified drinking water professional at least 30 days prior to the opening of each fair to assure that water sources and all water lines are thoroughly flushed and disinfected.

- Total coliform bacteria samples should be collected and the samples must test "absent" of coliform bacteria. Coliform samples should be collected 30 days prior to the event and again at least 1 week prior to any use of the water system. Coliform bacteria samples should continue to be collected and analyzed each seven-day period the fair is in operation or as determined by the Board of Health as a result of a site-specific inspection. Samples for nitrate and nitrite should also be collected 30 days prior to the event and again at least 1 week prior to any use of the water system. If any contaminants are detected, or if site-specific criteria require it, additional sampling may be required.

- A Massachusetts certified laboratory must perform all testing. To locate a certified lab and certified drinking water professional in your area, visit the following Web sites or contact your regional MassDEP office:

Find a Certified Operator: Massachusetts Consumer Affairs Website
Board of Certified Operators: Board of Certification Website 
List of Massachusetts Certified Labs: Web page

Contact your MassDEP Drinking Water Program for more information on securing private sources of drinking water at Local Fairs or other information:

Program.director-dwp@state.ma.us   or 617-292-5770