Procedures Relative To The Detection of Contaminants

During The New Source Approval Process

Effective Date: 10-20-88				Policy, SOP or Guideline #87-27

Program Applicability:

Supersedes Policy, SOP or Guidance:

Approved by: Patricia L. Deese

This policy is adopted to expand and replace Scenario III of DWP Policy 87-04, Procedures for Use of EPA’s PMCLs.  It provides procedures to be followed by DWSP staff when chemicals with Drinking Water Standards or Guidelines contaminants of concern with benchmark levels are detected during the exploratory phase of the new source approval process. Benchmark levels include:  Proposed Maximum Contaminant levels (MCLs), Office of Research and Standards Guidelines (ORSG) and Health Advisories.

# Policy

If during the New Source Approval process (See MassDEP Drinking Water Program’s: “Guidelines for Public Water Systems Chapter 3: Surface Water Supply Development and Chapter 4: Groundwater Supply Development”Procedure for New Groundwater Source Approvals; DWS Policy 86-02), one or more of the contaminants described above is detected and verified by a confirmation sample, the following process will be followed:

- Are contaminants detected in test well during exploration program?

- NO – 	Proceed with New Source Approval Process.

- YES - 	a.  Request pumping and redevelopment of test well, and

b.  Resample using sanitary sampling protocols

- Do Analytical results again indicate contaminants?

1.	NO -	Proceed with New Source Approval Process

2.	YES -	Consider alternative sources

- Are alternative sources feasible?

1.           NO -	Contact ORS and request evaluation of health effects of detected chemicals to determine if treatment will be necessary.

2.           YES -     a.  Pursue new Source Approval Process for new site, and

b. Abandon old site and refer to DHW BWSC for investigation

- Are contaminants within treatable limits?

1.	NO -	a. Abandon source and resume search for alternatives

b. Refer to DHW BWSC for investigation

2.	YES -	b. Continue below

- Is Wellhead treatment necessary at this time?

1.	NO -	a. Proceed with New Source Approval Process, and

b. Design wellhead to easily accommodate treatment in the future, and

c. Design monitoring program and trend analysis for early warning of need for

treatment

2.	YES -	a. Design and conduct a pumping test, and

b. Conduct standard pump test water quality sampling, and

c. Request ORS review of pump test results, and

d. Design and construct wellhead treatment facility, and

e. Design and implement raw water monitoring program to assure early warning               of dramatic changes in raw water quality which may adversely impact treatment, and

f. Refer site to BWSC OSC for investigation of containment source.

Approved: 10-6-88

Effective: 10-20-88

_________________

Patricia L. Deese, P.E.

Director,

Drinking Water Program

Corrected September 5, 2018