# EOTSS Requirements

# Site-to-Site VPN Tunnel

Table of Contents

Overview	1

Technical Resource	1

VPN Preliminary	1

VPN Profile	1

Routing	2

Routing Protocol	3

Connectivity Checklist	3

Important Notes	3

Additional Details	4

Overview
The following form must be filled out in order to create a VPN Tunnel to the Commonwealth of Massachusetts network.  VPN Tunnels connecting to this network must now conform to the following requirements:

- Must use IKEv2

- Must use AES256 bit encryption or higher (we do not support GCM)

- Must use SHA256 or higher

- Must use Group 14 or higher

- Must be route based (no policy-based VPN will be supported)

- Must use either Static Routes or BGP as the routing protocol

- Must use for remote IKE Gateway IP Address 170.63.208.11/24 or 170.63.208.139/24

Technical Resource
Please provide technical contact information below. Partner to fill in BLUE color cells in the form.

| Partner Contact Name |  |
| --- | --- |
| Partner Contact Phone Number |  |
| Partner Contact E-Mail Address |  |

| COMA Contact |  |
| --- | --- |
| COMA Phone Number |  |
| COMA Email |  |

# VPN Preliminary

To be completed by Technical Contact. Please fill in BLUE color cells in the form.

| Tunnel Endpoint | Preliminary | Partner Tunnel Endpoint |
| --- | --- | --- |
|  | VPN Appliance Model |  |
|  | Tunnel Peer IP |  |

# VPN Profile

Partner to fill in BLUE color cells in the form and mark ‘X’ where appropriate.

| IKE Version: Only IKEv2 is supported |
| --- |
| Phase 1 (ISAKMP) Settings |  |
| Local Authentication (PSK) | Shared over secured media |
| Remote Authentication (IKEv2 only) | Shared over secured media |
| IKE Encryption Algorithm | Only AES-256 or Higher Supported |
| IKE Message Hash\PRF Algorithm (Hash changed to PRF in IKEv2) | SHA256; SHA384; SHA512. |
| IKE Diffie-Hellman Group | 14; 19; 20; 21; 24. |
| IKE Lifetime Seconds (default 86400) | 86400 |
| IKEv2 Integrity | SHA256; SHA384; SHA512. |
| Phase 2 (IPSEC) Settings |  |
| IPSec Encryption Algorithm Encapsulating Security Payload (ESP) | Only AES-256 or Higher Supported |
| IPSec Authentication / Integrity Algorithm / ESP (Integrity in IKEv2) | SHA-256; SHA384; SHA-512. |
| IPSec Lifetime Seconds (default 28800) | 28800 |
| IPSec Lifetime Kilobytes (default 4275000) | 4275000 |
| PFS (Perfect Forward Secrecy) Yes / No - Default | YES; NO. If YES Group Number:. 14; 19; 20; 21; 24. |
| Dead Peer Detection Yes – Default /No | YES; NO. |

# Routing

Specify Remote Routes that will access Commonwealth of Massachusetts, and what Commonwealth of Massachusetts routes you require access to.

|  | (IP Protocols & Ports Required) |  |  |  |
| --- | --- | --- | --- | --- |
| Partner Source | Source Protocol/Port | COMA DST | Destination Protocol/Port | Purpose |
| (examples) 172.16.0.0/16 172.16.0.1 | (example) ANY UDP/389 | (examples) 192.168.1.15/32 192.168.1.10/32 | (example) TCP/443 ANY | (example) Partner subnet access to Webserver VIP over 443 Partner LDAP server listening on 389 to connect with destination host |

# Routing Protocol

Specify which Routing Protocol you would like to use:

Static: Yes or No

BGP: Yes or No

If BGP, what AS# would you like to use on your end:

# Connectivity Checklist

| Task | Notes |
| --- | --- |
| VPN Phase 1 verification |  |
| VPN Phase 2 verification |  |
| Real traffic verification (i.e. Partner access to website) |  |
| Partner Specific* |  |

Full connectivity testing is expected to be performed during activation process. All partner resources are required to be present and ready. Checklist parameters to be filled prior activation.

# Important Notes

- Please make sure that your interesting traffic matches to the above.

- VPN filtering will be implemented on Commonwealth of Massachusetts side; therefore, Protocol and Port is required.

- Allow 1-2 business days for Change control approval process once the VPN form is submitted.

- Once Change control is approved activation activity can be scheduled.

# Additional Details

If you have any other important information or details, please include them below.