APPENDIX A

CALGRID MODEL PLATFORM EQUIVALENCY

DEMOSTRATION FOR NEW ENGLAND DOMAIN

CALGRID Platform Equivalency demonstration

for Modeling 1-Hour Ozone Concentrations

Prepared by:

Massachusetts Department of Environmental Protection

New Hampshire Department of Environmental Services

Introduction

In 1991 the U.S. Environmental Protection Agency (USEPA) designated all or portions of Connecticut, Maine, Massachusetts, New Hampshire, and Rhode Island as non-attainment for the 1-hour National Ambient Air Quality Standard (NAAQS) for ozone.  The designations of “moderate”, “serious”, and “severe” were made to each non-attainment area depending on the severity of NAAQS exceedance monitored in the previous three years.  The Clean Air Act Amendments (CAAA) of 1990 require that each state with non-attainment areas provide an ozone Attainment Demonstration documenting emission reduction plans to reduce ozone levels within each state by a designated date.  As part of the Attainment Demonstration, the states with “serious” and “severe” designations are to produce photochemical modeling results to evaluate the effectiveness of planned emission reduction schemes.  With photochemical modeling, states can evaluate the effectiveness of various nitrogen oxide (NOx) and volatile organic compound (VOC) emission reduction strategies.

Prior to 1997, New England states relied on the Urban Airshed Model (UAM-IV) to perform photochemical modeling.  Reproductions of meteorological and emissions data for several ozone episodes produced modeling results similar to monitored values at inland locations but failed to reproduce satisfactory results along the sea coasts of New England.  It was decided by the New England states and USEPA Region I, to explore the use of the photochemical model CALGRID to improve modeling accuracy along the air/sea interfaces of New England.

CALGRID was designed by Earth Tech (Concord, MA) as a modular photochemical model with significant improvements in atmospheric physics and chemistry beyond UAM-IV.  Improvements include: an ozone deposition removal scheme including stomatal resistances of the vegitative canopy, inclusion of cloud observations to determine spatial and temporal solar insolation, capacity for additional mixing layers at user defined levels for improved air/sea interface ozone predictions, and the inclusion of BEIS2 and corresponding chemistry corrections.

In early 1997 Earth Tech was retained by the Northeast States for Coorinated Air Use Management (NESCAUM) to prepare a CALGRID performance evaluation. The performance evaluation was completed in July of 1997. CALGRID was compared statistically to actual monitoring data, and to UAM-IV modeling results. The performance statistics demonstrated that CALGRID performed as well as, if not better than UAM-IV for these episodes.  Performance statistics are presented in Attachment 1 (Photochemical Grid Modeling of Four High Ozone Episodes in the New England Domain: CALGRID vs. UAM-IV. A technical discussion of CALGRID is contained in Attachment 2 (Technical Justification for CALGRID as an Alternative to the UAM-IV Model in the New England Domain).

Model Platform Equivalency Methodology

CALGRID was installed at the Massachusetts Department of Enviromental Protection and the New Hampshire Department of Environmental Services in early September of 1997. Test modeling runs were then made utilizing Earth Tech input control files, meteorological data, boundary conditions, and emission files for four ozone episodes; June 21-22, 1988, July 7-8, 1988, July 10-11, 1988, and August 14-17, 1987.

CALGRID input files from Earth Tech were updated with Massachusetts and New Hampshire platform specifics and files were renamed to reflect the new platform preferences.  Each episode day was then modeled individually and the correct linkages were updated to allow for modeling multiple day episodes without loosing the previous days results.  CALGRID produces a binary concentration file and a brief ASCII summary file as outputs for each episode day.  After all of the modeling was completed,Masachusetts and New Hampshire summary files were compared to Earth Tech’s summary files with the DOS File Compare (FC) command.  Then the ground level ozone modeling results were extracted from the CALGRID binary concentration files for both platforms.  The extracted data consisted of maximum ozone concentrations for each grid cell at the assigned vertical layer (ground level) for the episode day.  Again, FC was used to compare the Massachusetts, New Hampshire and Earth Tech extracted files.  As a final step, graphics comparing the modeling results for each episode day.

Platform Equivalency Results

Exact platform equivalency was demonstrated for July 7, 8, 10, and 11, 1988 and for August 14 and 15, 1987.  Each of these episode days produced identical graphical results (Figures 1-6), file comparisons (FC) of extracted concentrations, and summary files (with the exception of time stamps and title lines, this was expected).

The Massachusetts and New Hampshire platform modeling results demonstrated higher ozone concentrations than Earth Tech’s in some locations for August 16 and 17, 1987 and for June 21 and 22, 1988 .  In the case of August 16 and 17, 1987, the Earth Tech input control listed an older version of meteorological data for CALGRID input file, than was provided to New Hampshire.  The discrepancy was verified with Earth Tech; the results for August 16 and 17 would have been identical if identical data sets had been used. In the case of the June 21-22, 1988 episode, the exact cause of the output file discrepancy is uncertain.  Massachusetts DEP achieved identical results to New Hampshire’s, therefore it is most likely an issue with a mismatched data set from Earth Tech.

Conclusion

In conclusion, CALGRID is installed correctly and operating correctly on the Massachusetts and the New Hampshire platform and capable of producing reliable

one ozone modeling results in the New England Domain.

ATTACHMENT 1

PHOTOCHEMICAL GRID MODELING OF FOUR HIGH OZONE

EPISODES IN THE NEW ENGLAND DOMAIN: CALGRID VS UAM-IV

ATTACHMENT 2

TECHNICAL JUSTIFICATION FOR CALGRID AS AN ALTERNATIVE

TO THE UAM-IV MODEL IN THE NEW ENGLAND DOMAIN