APPENDIX A

CALGRID MODELING PROTOCOL

FOR THE NEW ENGLAND DOMAIN

CALGRID MODELING  PROTOCOL

FOR THE NEW ENGLAND DOMAIN

APRIL, 1998

COMMONWEALTH OF MASSACHUSETTS

EXECUTIVE OFFICE OF ENVIRONMENTAL AFFAIRS

DEPARTMENT OF ENVIRONMENTAL PROTECTION

ONE WINTER STREET, BOSTON, MA 02108

Table of Contents

Section                                                                                                                          Page

1.  Modeling Study Design								       1

a.  Background

b.  Objectives

c.  CALGRID Model

4

2.   Management Structure

Participating Organizations

New England Domain Photochemical Modeling Workgroup

New England Domain Photochemical Modeling Team

Relationship to Other Modeling Protocols

3.  Modeling Domain                                                                                                        5

a.  Description of New England Domain

b.  Horizontal Grid Size

c.  Number of Vertical Layers

4.   Episode Selection                                                                                                        7

a.  Episode Criteria

b.  Meteorological Regimes

c.  Ranked High Ozone Days

d.  Selected Episodes for the New England Domain

5.  Meteorological Databases                                                                                         12

a.  Surface Meteorological Data

b.  Upper Air meteorological Data

6.  Meteorological Input Files                                                                                        12

a.  CALMET Meteorological Model

b.  Wind Fields

c.  Surface Temperatures

d.  Mixing Depths

e.  Other Related Meteorological Data

7.   Initial Conditions and Boundary Conditions                                                             13

8.  Emission Databases                                                                                                    14

a.  1990 Emission Estimates

b.  Base Case Emission Files

c.  1999 Emission Files Reflecting CAA Controls

d.  2007 Emission Files Reflecting EPA Proposed SIP Call Reductions

9.   1999 Runs                                                                                                                 15

10. Emission Sensitivity Runs

11. 2007 Boundary Condition Runs                                                                               16

12. 2007 EPA SIP Call Runs                                                                                          16

13. Conclusions                                                                                                              16

14. References                                                                                                                16

Appendix A    CALGRID Model Platform Equivalency Demonstration

for the New England Domain

1.  Modeling study Design

a.  Background

In 1991, the Environmental Protection Agency (EPA) classified Massachusetts, Rhode Island, and portions of Connecticut and southern New Hampshire as "serious" non-attainment areas with respect to the ozone National Ambient Air Quality Standard (NAAQS). Parts of southern Maine were classified as "moderate" non-attainment areas. New England non-attainment areas are listed in Table 1. These areas are part of the densely populated Northeast Corridor in which the long-range transport of ozone and ozone precursors continue to significantly contribute to ozone exceedances.

The Clean Air Act Amendments of 1990  (CAA) require all states with ozone non-attainment areas to design control strategies and to demonstrate that such strategies will result in attainment of the ozone NAAQS by specified dates which depend on the area’s ozone severity. Attainment dates for "moderate" and "serious" non-attainment areas are 1996  and 1999, respectively. In addition, "serious" (or worse) non-attainment areas need to demonstrate attainment through application of a photochemical model (1).

In 1994, the Massachusetts DEP (MADEP) developed a modeling platform designed for simulating four ozone episodes using the EPA-approved Urban Airshed Model (2). Future year 1999 modeling showed that emission reductions reflecting minimum CAA mandated controls fell far short of bringing the domain into ozone attainment (2). The 1999 CAA control strategy simulation showed exceedances over western Massachusetts (caused by high upwind boundary concentrations) and in eastern Massachusetts over the metropolitan Boston area and over coastal New Hampshire and Maine.

Although the urban Airshed Model (UAM-IV) performance met EPA measures of acceptability, there were concerns regarding the model’s handling of plume transport along coastal Maine and New Hampshire, where ozone was under-predicted. Because of the  concerns with UAM-IV performance in coastal New Hampshire and Maine (elevated levels along the coast and could not be replicated), MADEP decided to investigate an alternative modeling platform for the New England Domain using the CALGRID model (3) developed by Earth Tech (Concord, MA). A performance evaluation with CALGRID and UAM-IV indicated that CALGRID performance was better that of UAM-IV (Appendix A).  One important feature was that CALGRID replicated plume penetration along the coast in New Hampshire and southern Maine. In August, 1997,  MADEP requested EPA to approve CALGRID for additional SIP ozone modeling analyses in the New England Domain. In November, 1997, following its review of technical justification comparing both models’ performance, EPA approved MADEP’s request to use CALGRID. The performance evaluation is contained in Appendix A of this document.

b.  Objectives

The MADEP, with assistance from the New England Domain Photochemical Modeling Workgroup, prepared ozone modeling analyses for the ozone attainment demonstrations that are required for several New England states.  A suite of model runs were performed with CALGRID to address the following key areas of interest:

- Geographic Areas

- VOC/NOx Matrix Runs

- Low Level and Elevated Source Impacts

- 1999 Emissions in New England with 2007 Boundary Conditions

- 2007 Emissions in New England with 2007 Boundary Conditions

These simulations, which are described in detail in sections 9-11, helped provide a means for evaluating the effectiveness of controls and guiding states in formulating emission reduction programs. The New England Domain ozone modeling analyses was designed to be used for attainment demonstrations required for "serious" non-attainment areas in Massachusetts, New Hampshire, and Rhode Island, and for "moderate" non-attainment areas in Maine. Due to Connecticut’s proximity to the New York City urban area,  EPA Region I recommended that New York Domain ozone modeling analyses be employed for non-attainment areas in  Connecticut, rather than the New England Domain analyses.

c.  CALGRID Model

The CALGRID model is a modular photochemical model with significant improvements in atmospheric physics and chemistry beyond UAM-IV (3).  Improvements include: ozone deposition removal scheme including stomatal resistances of the vegetative canopy; inclusion of cloud observations to determine spatial and temporal solar insolation; and, the capacity for additional mixing layers at user defined levels for improved air/sea interface ozone predictions. CALGRID uses version IV of the Carbon Bond Mechanism for solving chemical kinetics (4). Modifications to isoprene and formaldehyde chemistry have been made to more accurately simulate reactions associated with BEIS2 biogenic emissions (BEIS2 is the recommended EPA emission model for generating biogenic VOC and NOx emissions). A performance evaluation (Appendix A) indicated that CALGRID met EPA performance criteria and overall model performance was superior to that of UAM-IV in the New England Domain.

Table 1

Moderate, Serious and Severe Ozone

Non-attainment Areas in New England

State Non-Attainment Area Classification

Connecticut Bridgewater and New Milford Severe

in Litchfield County,

all of Fairfield County except

the city of Shelton

Connecticut All other cities and towns Serious

Maine Androscoggin, Cumberland, Moderate

Kennebec, Knox, Lincoln,

Sagadhoc, and York County

Massachusetts Entire state Serious

New Hampshire Strafford County Serious

The following cities and towns Serious

in Rockingham County - Atkinson,

Brentwood, Danville, Derry, Exeter,

Greenland,, Hampstead, Hampton,

Hampton Falls, North Hampton,

S.Hampton, Kensington, Kingston,

E. Kingston, Londonderry, New Castle,

Newfields, Newington, Newmarket,

Newton, Plaistow, Portsmouth, Rye,

Salem, Sandown, Seabrook, Stratham,

and Windham

The following cities and towns Serious

in Hillsborough County - Amherst,

Brookline, Hollis, Hudson, Litchfield,

Merrimack, Milford, Mont Vernon,

Nashua, Pelham, and Wilton

Rhode Island Entire state Serious

2.  Management Structure

a.  Participating organizations

The principal organizations participating in the New England Domain ozone attainment demonstration are listed below.

Connecticut Department of Environmental Protection (CTDEP)

Maine Department of Environmental Protection (ME DEP)

Massachusetts Department of Environmental Protection (MADEP)

New Hampshire Department of Environmental Services (NHDES)

Rhode Island Department of Environmental Management (RIDEM)

Vermont Department of Environmental Conservation (VTDEC)

U.S. Environmental Protection Agency - Region I (USEPA Region I)

In addition to these organizations, EPA's Office of Air Quality Planning and Standards will participate in the study by providing OTAG data bases needed for 2007 boundary conditions.

b.  New England Domain Photochemical Modeling Workgroup

In late summer of 1997, an updated New England Domain Photochemical Modeling Workgroup was established to provide policy and technical guidance for the New England Domain ozone modeling analyses. The members of this workgroup are listed below.

Connecticut              David Wackter

Kurt Kebschull

Maine                       Jeff Crawford

Cliff Michaelsen

Massachusetts           Steve Dennis

Richard Driscoll

Richard Fields

Nancy Seidman

Leah Weiss

Margaret Valis

New Hampshire        Kent Finemore

Jeff Underhill

Rhode Island             Barbara Morin

EPA Region I            David Conroy

Richard Burkhart

Ian Cohen

Robert McConnel

The New England Domain Photochemical Modeling Workgroup will meet as needed to address policy or technical issues which cannot be resolved by conference calls. MADEP will be responsible for preparing and circulating progress reports of the New England domaoin ozone modeling analyses to each member of the workgroup for review and comment.

c.  New England Domain Photochemical Modeling Team

The New England Domain Photochemical Modeling Team will be responsible for performing all CALGRID modeling runs, analyzing modeling results and preparing ozone modeling analyses needed for 1999 attainment demonstrations. Team members are listed below.

Steve Dennis

Richard Fields

Jeff Underhill

Margaret Valis

Each New England Domain state (CT, MA, ME, NY, NH, RI, and VT),  with EPA Region I  EPA coordination, will prepare emission file scaling factors needed to prepare 2007 emission files.

d.  Relationship to Other Modeling Protocols

In addition to New Engaln Domain, three other photochemical modeling analyses are anticipated for urban areas in the Northeast Corridor (New York-New Jersey-Connecticut domain, Washington-Baltimore-North Virginia domain, and the Philadelphia-Delaware-Southern New Jersey). The ROM model and the UAM-V model (OTAG regional modeling) will provide initial and boundary condition data for several of these urban scale modeling analyses.

3.  Modeling Domain

a.  Description of Domain

The boundaries of the New England Domain are as shown in Figure 1. To simplify

construction of model grids, Universal Tranverse Mercator (UTM) coordinates shown in Figure 2 are expressed in terms of a zone 19 coordinate system. The approximate area of the New England Domain is 115,600 square kilometers; almost one quarter of the domain extends out over the Atlantic Ocean. In general, SIP modeling domains should be large

enough to encompass downwind areas thought to be recipients of ozone created by the domain’s urban emissions on primary episode days. A previous modeling analysis indicated that the New England Domain satisfies these criteria (2).

b.  Horizontal Grid Size

The New England Domain employs a grid cell size of 5 km by 5 km.  This was a compromise between a finer grid mesh, which offered greater resolution but at higher computational costs, and a coarser mesh which might not have provide sufficient resolution for model performance evaluation. The New England Domain surface grids consist of 4624 five kilometer cells (68 across x 68 down).

c.  Vertical Structure

CALGRID was used with 14 fixed layers (20, 50, 100, 200, 400, 600, 800, 1100, 1400, 1700, 2000, 2400, 2800, 3300 meters).  The CALMET meteorological processor which was used to prepare meteorological input files for CALGRID employed the same 14 fixed layers.   This eliminated the need to do vertical averaging when CALGRID extracted gridded wind and temperature fields from CALMET model results. The modeling domain top was set at 3300 meters above ground for all episodes.

4. Episode selection

a.  Episode Criteria

Since it would be impractical to model every violation day, EPA  recommended targeting a select group of episode days for ozone attainment demonstrations (1). Such episode days should be (1) meteorologically representative of typical high ozone exceedance days in the domain, and (2)  severe enough that any control strategies predicted to attain the ozone NAAQS for that episode day would also result in attainment for all other exceedance days.

b.  Meteorological Regimes

MADEP analyzed National Weather Service (NWS) surface weather maps to determine the predominant synoptic regimes for all ozone exceedance days in the New England Domain over a ten year period (1982 - 1991). Several predominant regimes emerged, although many exceedance days exhibited characteristics of more than one synoptic regime. Predominant regimes are described below.

1. Anticyclonic flow with high pressure situated south of the region.  This

would normally be an extention of the "Bermuda" high pressure system

centered well east of the middle or south Atlantic states. The usual air

trajectory would be from southwest to northeast along the northeast

corridor into New England.

2.  Flat anticyclone diffusely centered over the eastern U.S. or western

Atlantic Ocean producing a weak pressure gradient across most of the

region, i.e.,  a "recirculation" pattern. Winds across New England would

typically be light and variable and feature coastal seabreezes. The airmass

would have lingered over the region for several days and would be of

high moisture content.

3.  A low pressure trough located across the region. This is more of a

secondary feature with the trough typically embedded within the existing

synoptic pattern.  The airflow would depend on the existing primary

synoptic regime and the position of the trough.

4.  A transitional pattern. High pressure, originally located over the midwest,

crosses the  middle Atlantic states before relocating over the western

Atlantic. During this transition, winds across New England would

generally be shifting from westerly to southwesterly with the air mass

moisture content still low.

5.  A slowly moving frontal boundary across portions of the region. This

provided the potential for elevated ozone in the warm, humid air mass

south of the front. There would normally be weak to moderate transport

along the northeast corridor south of the front.

6.  Low pressure located north and west of the region with high pressure to

the south and east. Isobars would exhibit cyclonic curvature or

"troughiness." Moderate to strong transport along the northeast corridor

into the region is typical.

c.  Ranked High Ozone Days

MADEP reviewed a five year set of ozone data (1987-1991) to identify Domain exceedance  days. Every exceedance day received three scores based on (1) peak value recorded on given day and (2) average of the peak concentration recorded on given day, and (3) percent of ozone stations recording ozone exceedances on given day. Each day’s three scores were then summed, forming the basis of the ranking scheme.  Table 2 lists the ranked episode days.  The prevailing meteorological regimes (one or more of the  six meteorological regimes described above)  for each day are also listed on Table 2. The term "none" in the table indicates that the prevailing regime was not one of the predominant six.

Table 2.  (Part 1 of 3)

Five Year Ranking (1987-1991) of  Ozone Episode Days

in the New England Domain

| Date | Ozone Sites | Met Regime | Total Rank | MaxPeak Value (ppb) |  |  | Rank | AvePeak Value (ppb) | Rank | %Sites >0.124 ppb | Rank |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 07/11/88 | 35 | 1,3,6 | 1 |  | 0.222 | 2 |  | 0.138 | 2 | 63 | 1 |
| 06/22/88 | 33 | 1,6 | 2 |  | 0.239 | 1 |  | 0.139 | 1 | 45 | 4 |
| 07/08/88 | 34 | 2,3 | 3 |  | 0.184 | 8 |  | 0.130 | 3 | 62 | 2 |
| 06/16/88 | 34 | 6 | 4 |  | 0.179 | 10 |  | 0.124 | 4 | 41 | 5 |
| 07/10/88 | 35 | 2,3 | 5 |  | 0.203 | 3 |  | 0.111 | 12 | 31 | 10 |
| 06/14/88 | 35 | 1 | 5 |  | 0.178 | 11 |  | 0.118 | 7 | 37 | 7 |
| 06/15/88 | 34 | 1,6 | 6 |  | 0.170 | 16 |  | 0.122 | 5 | 38 | 6 |
| 07/06/88 | 35 | 2 | 7 |  | 0.178 | 11 |  | 0.113 | 10 | 37 | 7 |
| 07/18/91 | 39 | 1,3 | 8 |  | 0.174 | 14 |  | 0.115 | 9 | 36 | 8 |
| 07/19/91 | 39 | 1,3 | 8 |  | 0.161 | 22 |  | 0.120 | 6 | 56 | 3 |
| 07/16/88 | 32 | 1,5 | 9 |  | 0.177 | 12 |  | 0.115 | 9 | 28 | 12 |
| 06/28/91 | 38 | 1,6 | 10 |  | 0.193 | 7 |  | 0.113 | 10 | 21 | 18 |
| 08/17/87 | 29 | 6 | 11 |  | 0.180 | 9 |  | 0.112 | 11 | 24 | 16 |
| 08/04/89 | 34 | 5,6 | 12 |  | 0.168 | 17 |  | 0.115 | 9 | 29 | 11 |
| 08/01/91 | 39 | 6 | 13 |  | 0.175 | 13 |  | 0.108 | 14 | 28 | 12 |
| 06/15/91 | 38 | 1,5 | 13 |  | 0.170 | 16 |  | 0.108 | 14 | 34 | 9 |
| 08/13/88 | 32 | 1 | 14 |  | 0.174 | 14 |  | 0.106 | 16 | 28 | 12 |
| 08/03/88 | 34 | 1,5 | 14 |  | 0.159 | 23 |  | 0.116 | 8 | 29 | 11 |
| 07/24/87 | 33 | 1 | 14 |  | 0.195 | 6 |  | 0.104 | 18 | 21 | 18 |
| 07/18/90 | 36 | 1 | 15 |  | 0.167 | 18 |  | 0.110 | 13 | 28 | 12 |
| 06/19/87 | 30 | 5 | 15 |  | 0.167 | 18 |  | 0.111 | 12 | 27 | 13 |
| 07/07/88 | 35 | 2,3 | 16 |  | 0.158 | 24 |  | 0.110 | 13 | 37 | 7 |
| 07/30/88 | 35 | 1,6 | 17 |  | 0.165 | 19 |  | 0.107 | 15 | 26 | 14 |
| 07/14/88 | 32 | 6 | 17 |  | 0.162 | 21 |  | 0.113 | 10 | 22 | 17 |
| 07/25/89 | 34 | 2,3 | 18 |  | 0.172 | 15 |  | 0.104 | 18 | 24 | 16 |
| 08/12/88 | 33 | 1,2,3 | 19 |  | 0.177 | 12 |  | 0.100 | 22 | 21 | 18 |
| 07/20/90 | 36 | 1 | 20 |  | 0.158 | 24 |  | 0.107 | 15 | 25 | 15 |
| 08/02/88 | 34 | 2 | 20 |  | 0.158 | 24 |  | 0.108 | 14 | 24 | 16 |
| 08/14/88 | 32 | 1,5 | 20 |  | 0.172 | 15 |  | 0.100 | 22 | 22 | 17 |
| 08/17/90 | 35 | 2 | 21 |  | 0.157 | 25 |  | 0.103 | 19 | 26 | 14 |
| 07/20/91 | 39 | 1,3 | 22 |  | 0.157 | 25 |  | 0.104 | 18 | 21 | 18 |
| 08/04/90 | 36 | 2 | 23 |  | 0.197 | 4 |  | 0.089 | 32 | 8 | 29 |
| 08/17/91 | 38 | 1,6 | 24 |  | 0.137 | 41 |  | 0.112 | 11 | 26 | 14 |
| 06/29/87 | 30 | 1 | 25 |  | 0.145 | 36 |  | 0.111 | 12 | 20 | 19 |
| 06/20/88 | 34 | 1,6 | 26 |  | 0.149 | 33 |  | 0.105 | 17 | 21 | 18 |
| 08/11/88 | 34 | 1,5 | 26 |  | 0.163 | 20 |  | 0.097 | 25 | 15 | 23 |
| 07/29/88 | 35 | 1,3 | 27 |  | 0.167 | 18 |  | 0.097 | 25 | 11 | 26 |
| 05/28/88 | 32 | 6 | 28 |  | 0.145 | 36 |  | 0.103 | 19 | 25 | 15 |
| 08/18/90 | 35 | 1,5 | 29 |  | 0.153 | 29 |  | 0.101 | 21 | 17 | 21 |
| 08/15/87 | 30 | 2 | 30 |  | 0.155 | 27 |  | 0.095 | 26 | 20 | 19 |

Table 2.  (Part 2 of 3)

Five Year Ranking (1987-1991) of  Ozone Episode Days

in the New England Domain

| Date | Ozone Sites | Met Regime | Total Rank | MaxPeak Value (ppb) | Rank | AvePeak Value (ppb) | Rank | %Sites >0.124 ppb | Rank |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 07/17/91 | 39 | 1,3 | 30 | 0.155 | 27 | 0.097 | 25 | 18 | 20 |
| 06/19/88 | 34 | 1 | 30 | 0.153 | 29 | 0.102 | 20 | 15 | 23 |
| 06/11/91 | 38 | 1,6 | 31 | 0.158 | 24 | 0.094 | 27 | 13 | 24 |
| 07/27/89 | 34 | 1,6 | 32 | 0.140 | 40 | 0.102 | 20 | 21 | 18 |
| 07/19/90 | 36 | 1,5 | 32 | 0.155 | 27 | 0.091 | 30 | 17 | 21 |
| 06/15/87 | 29 | 5 | 32 | 0.196 | 5 | 0.078 | 43 | 7 | 30 |
| 07/19/89 | 21 | 1,2 | 33 | 0.149 | 33 | 0.097 | 25 | 16 | 22 |
| 07/04/90 | 36 | 1,5,6 | 34 | 0.153 | 29 | 0.098 | 24 | 8 | 29 |
| 06/13/88 | 35 | 4 | 34 | 0.152 | 30 | 0.098 | 24 | 9 | 28 |
| 07/25/87 | 32 | 2,3 | 35 | 0.156 | 26 | 0.092 | 29 | 9 | 28 |
| 08/01/88 | 35 | 2,3 | 35 | 0.161 | 22 | 0.088 | 33 | 9 | 28 |
| 09/09/89 | 34 | 2 | 36 | 0.136 | 42 | 0.105 | 17 | 12 | 25 |
| 06/08/87 | 30 | 6 | 37 | 0.162 | 21 | 0.087 | 34 | 7 | 30 |
| 06/02/90 | 35 | 1 | 38 | 0.149 | 33 | 0.094 | 27 | 11 | 26 |
| 06/27/89 | 33 | 1,6 | 39 | 0.150 | 32 | 0.091 | 30 | 12 | 25 |
| 08/06/89 | 31 |  | 39 | 0.156 | 26 | 0.084 | 37 | 13 | 24 |
| 07/10/87 | 32 | 2,3 | 40 | 0.150 | 32 | 0.087 | 34 | 16 | 22 |
| 06/27/91 | 38 | 1 | 41 | 0.150 | 32 | 0.099 | 23 | 3 | 34 |
| 07/26/89 | 34 | 2,3 | 42 | 0.154 | 28 | 0.085 | 36 | 9 | 28 |
| 07/02/89 | 33 | 1,2 | 42 | 0.148 | 34 | 0.091 | 30 | 9 | 28 |
| 08/08/87 | 31 | 1 | 43 | 0.154 | 28 | 0.086 | 35 | 6 | 31 |
| 05/31/87 | 27 | 1,3 | 43 | 0.143 | 37 | 0.094 | 27 | 7 | 30 |
| 06/26/91 | 38 | 1 | 44 | 0.134 | 44 | 0.099 | 23 | 8 | 29 |
| 06/30/87 | 30 | 6 | 44 | 0.151 | 31 | 0.083 | 38 | 10 | 27 |
| 05/30/87 | 28 | 1,5,6 | 45 | 0.159 | 23 | 0.080 | 41 | 4 | 33 |
| 07/11/87 | 32 | 2,3 | 45 | 0.141 | 39 | 0.091 | 30 | 9 | 28 |
| 08/09/88 | 34 | 1 | 45 | 0.131 | 46 | 0.095 | 26 | 12 | 25 |
| 09/10/89 | 33 | 2 | 46 | 0.131 | 46 | 0.097 | 25 | 9 | 28 |
| 08/13/90 | 34 | 1,6 | 46 | 0.142 | 38 | 0.094 | 27 | 3 | 34 |
| 07/26/88 | 33 | 2 | 46 | 0.136 | 42 | 0.092 | 29 | 9 | 28 |
| 05/16/91 | 34 | 1 | 47 | 0.135 | 43 | 0.095 | 26 | 6 | 31 |
| 08/05/88 | 35 | 1 | 48 | 0.135 | 43 | 0.090 | 31 | 9 | 28 |
| 06/21/88 | 33 | 5 | 49 | 0.146 | 35 | 0.084 | 37 | 6 | 31 |
| 08/10/88 | 34 | 1,6 | 49 | 0.127 | 50 | 0.093 | 28 | 12 | 25 |
| 07/21/91 | 39 | none | 50 | 0.135 | 43 | 0.093 | 28 | 3 | 34 |
| 07/13/87 | 32 | 2,3,6 | 50 | 0.150 | 32 | 0.075 | 45 | 9 | 28 |
| 07/17/90 | 35 | 1 | 50 | 0.135 | 43 | 0.090 | 31 | 6 | 31 |

Table 2.  (Part 3 of 3)

Five Year Ranking (1987-1991) of  Ozone Episode Days

in the New England Domain

| Date | Ozone Sites | Met Regime | Total Rank | MaxPeak Value (ppb) | Rank | AvePeak Value (ppb) | Rank | %Sites >0.124 ppb | Rank |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 08/15/88 | 34 | 6 | 51 | 0.129 | 48 | 0.088 | 33 | 12 | 25 |
| 04/28/90 | 32 | 5 | 52 | 0.141 | 39 | 0.086 | 35 | 3 | 34 |
| 06/22/90 | 37 | 1 | 53 | 0.130 | 47 | 0.089 | 32 | 5 | 32 |
| 08/30/91 | 38 | 1,6 | 53 | 0.142 | 38 | 0.082 | 39 | 3 | 34 |
| 07/18/87 | 32 | 4 | 53 | 0.134 | 44 | 0.088 | 33 | 3 | 34 |
| 07/09/88 | 34 | 2,3 | 54 | 0.126 | 51 | 0.094 | 27 | 3 | 34 |
| 06/13/87 | 29 | 1,3,5 | 54 | 0.134 | 54 | 0.083 | 38 | 7 | 30 |
| 06/01/87 | 27 | 1,2,3 | 54 | 0.146 | 35 | 0.072 | 47 | 7 | 30 |
| 07/16/91 | 39 | 4 | 55 | 0.136 | 42 | 0.083 | 38 | 3 | 34 |
| 07/09/87 | 32 | 2 | 55 | 0.136 | 42 | 0.080 | 41 | 6 | 31 |
| 07/23/87 | 33 | 2,5 | 56 | 0.140 | 40 | 0.080 | 41 | 3 | 34 |
| 06/25/87 | 30 | 5 | 56 | 0.150 | 32 | 0.057 | 53 | 7 | 30 |
| 06/20/87 | 30 | 5 | 56 | 0.145 | 36 | 0.075 | 45 | 3 | 34 |
| 06/14/87 | 29 | 1,3 | 57 | 0.135 | 43 | 0.078 | 43 | 7 | 30 |
| 06/16/91 | 38 | 1,5 | 57 | 0.150 | 32 | 0.064 | 50 | 3 | 34 |
| 07/23/91 | 39 | 6 | 57 | 0.132 | 45 | 0.079 | 42 | 8 | 29 |
| 08/02/91 | 39 | 5,6 | 58 | 0.137 | 41 | 0.074 | 46 | 5 | 32 |
| 06/10/91 | 37 | 1 | 59 | 0.126 | 51 | 0.085 | 36 | 3 | 34 |
| 05/30/88 | 31 | 2,5 | 59 | 0.146 | 35 | 0.061 | 52 | 3 | 34 |
| 05/10/87 | 29 | 1,3,5 | 60 | 0.125 | 52 | 0.085 | 36 | 3 | 34 |
| 07/12/88 | 35 | 1,6 | 61 | 0.126 | 51 | 0.082 | 39 | 3 | 34 |
| 07/31/88 | 34 | 5 | 62 | 0.135 | 43 | 0.067 | 48 | 3 | 34 |
| 05/17/87 | 30 | 1,3,5 | 62 | 0.129 | 48 | 0.078 | 43 | 3 | 34 |
| 07/01/89 | 34 | 2,4 | 63 | 0.125 | 52 | 0.081 | 40 | 3 | 34 |
| 07/27/88 | 32 | 2 | 63 | 0.128 | 49 | 0.078 | 43 | 3 | 34 |
| 05/27/91 | 35 | 1,5 | 64 | 0.135 | 43 | 0.063 | 51 | 3 | 34 |
| 05/29/87 | 28 | 2,5 | 65 | 0.130 | 47 | 0.066 | 49 | 4 | 33 |
| 06/26/89 | 31 | none | 65 | 0.128 | 49 | 0.066 | 49 | 6 | 31 |
| 06/25/91 | 38 | 2,4 | 65 | 0.126 | 51 | 0.077 | 44 | 3 | 34 |
| 09/17/91 | 38 | none | 66 | 0.127 | 50 | 0.067 | 48 | 5 | 32 |
| 06/20/91 | 38 | none | 67 | 0.125 | 52 | 0.074 | 46 | 3 | 34 |
| 08/07/88 | 34 | 3 | 68 | 0.125 | 52 | 0.063 | 51 | 3 | 34 |

d.  Selected Episodes for the New England Domain

Three episodes in 1988 and one episode in 1987 were selected for the New England Domainozone attainment demonstration. The 1988 episodes - July 11, July 8, and June 22 - were the top three ranked high ozone days, and reflect most of the predominant meteorological regimes associated with ozone violation days in the domain. The 1987 episode day - August 17 - was selected because almost every exceedance was recorded north of Boston, indicating that southern New England emissions may have significantly contributed to elevated ozone levels Maine and New Hampshire on those days.

To minimize the effects of initial conditions, modeling simulations were started 24 hours before each high ozone day of concern. However, several ozone exceedances were also observed north of Boston on August 15, 1987. At the suggestion of EPA Region I, the August 17 modeling period was extended back to August 14 in order to include August 15 as an episode day. The complete set of simulation days were June 21-22,  1988,  July 7-8,  1988, July 10-11, 1988, and August 14-17, 1987. All four episodes were used for performance evaluations of UAM-IV and CALGRID.

5.  Meteorological Databases

a.  Surface Meteorological Data

Approximately 100 surface stations were used to construct hourly windfields for each episode. Surface data consisted of observations from NWS stations, Coast Guard stations, automated sites, offshore buoys, and state-run meteorological monitors in the New England Domain.  Additional surface sites outside the New England Domain (within 50 kilometers) were also employed after consulting with EPA Region I staff.

b.  Upper Air Meteorological Data

Upper air data consisted of radiosonde observations from three sites: Chatham,Massachusetts, Portland, Maine and Albany, New York.

6.  Meteorological Input files

a.  CALMET Meteorological Model

Key to obtaining acceptable and defensible model performance is using physically

realistic techniques to develop meteorological fields. The CALMET  meteorological

model (5) developed by Earth Tech  was selected for generating the hourly

meteorological fields (mixing heights, windfields and temperature fields) for each

episode day in the New England Domain. CALMET contains a micrometeorological module to develop gridded mixing height fields and a diagnostic wind module (including a topographic wind flow adjustment scheme) for deriving three-dimensional windfields.

CALMET calculates convective mixing depths for daytime hours and mechanical mixing depths for daytime and nighttime hours. The larger of the convective and mechanical depths is used for the daytime mixing height. CALMET's convective scheme uses opaque cloud cover, solar elevation angle, and land use characteristics to calculate an insolation-driven upward flux of sensible heat. This heat flux erodes the stable lower layer of the morning sounding, creating an adiabatic mixed layer that grows with further heating. CALMET also simulates the advection of upwind mixing heights. When winds blow onshore near land/sea boundaries, this advection  influences ozone buildup along the coast as lower over-water mixing heights are drawn inland.

b.  Windfields

The CALMET model was used to generate 14 layer windfields (20 minute time step) using the diagnostic wind model option in CALMET. The van Ulden-Holtslag6 scheme was used for interpolating winds vertically in the boundary layer.

c. Temperatures Fields

The CALMET model was used to generate 14 layer temperature fields (20 minute time step).

d.  Mixing Depths

The CALMET model was used to generate gridded mixing depths (20 minute time step) using a vertical heat flux method and an advection scheme to create realistic coastline gradients.

e.  Other Related Meteorological Data

Cloud cover information was obtained from sattelite data. All other meteorological variables and terrain and land use surface characteristics required as input for CALGRID were extracted from CALMET model output files.

7.  Initial Conditions and Boundary Conditions

Initial and boundary air quality concentrations for 1999 were extracted from ROM runs

supplied by EPA by means of EPA’s GMISS System for all four ozone episodes (June 21-22  1988,  July 7-8  1988, July 10-11, 1988, and August 14-17 1987).  The ROM-Interface System was then used to create input files for 1999 CAA modeling runs. Initial and boundary conditions for 2007 will be extracted from OTAG runs supplied by EPA for the July 7-8 1988 and the July 10-11 1988 episodes.

8. Emission databases

a.  1990 Emissions Estimates

In April 1994, EPA Region I prepared workfiles point, area,  and mobile source emissions for the six New England States (7). These data were extracted from the 1990 base year inventories which the States entered into the Aerometric Informational Retrieval System (AIRS) database in late 1993. New York provided EPA Region I with emission data for the seven New York counties in the New England Domain. EPA quality assured the New England AIRS data and all necessary modifications and corrections were made pursuant to EPA Region I instructions.  Since these workfiles were generated from AIRS, they were in a format suitable for processing with EPA's Emissions Preprocessor System (EPS) 2.0,  which was used to prepare emission files for each episode (8).

To simulate more accurately day-specific emissions for each episode day, day-specific on-road mobile emissions were estimated by applying MOBILE5a emission factors to 1990 annual vehicle miles traveled (VMT) data. The 1990 VMT data and day-specific MOBILE5a input files were provided by each state, and MOBILE5 was used to generate day-specific emission factors.

b.  Base Case Emission Files

The initial state-specific emissions estimates were for 1990. In order to model 1987 and 1988 historical episodes, it was necessary to backcast the 1990 emissions estimates to 1987 and 1988 levels, respectively. State-specific backcast factors were generated for each Standard Industrial Classification (SIC) and Area Source Code (ASC) using the EPA's EPS 2.0 BEAFAC program.  These backcast predictions were  based on data supplied by the Bureau of Economic Analysis' Regional Projections to 2040. The backcast factors were provided to each state for review and modification as needed. The factors were then applied to 1990 emissions to obtain 1987 and 1988 emission estimates (7). EPA provided 1987 and 1988 VMT backcast factors which were used to adjust the day-specific onroad mobile files to 1987 and 1988 VMT levels.

In addition, hourly NOx and VOC data for major utility sources in Connecticut, Massachusetts, New Hampshire and Rhode Island for the July 1988 episode days were supplied by Earth Tech (7).  Earth Tech obtained this data from the New England Power Pool (NEPOOL). The NOx and VOC data in the original AIRS file corresponding to these utilities were removed to avoid double counting. EPA's Emissions Preprocessor System (EPS) 2.0. was used to prepare emission files for each episode (8).

Day-specific utility data were not supplied for the August 1987 episode and the June 1988 episode. For these two episodes, EPA Region I recommended using emissions data from surrogate days in July, based on comparable load data (7).  Surrogate days are listed below.

Episode Day     Surrogate Day

Aug 14              July 10

Aug 15              July 10

Aug 16              July 10

Aug 17              July 11

June 21              July 8

June 22              July 8

EPA’s BEIS2 model was used to generate biogenic VOC and NOx emissions for the New England Domain.

c.  1999 Emission Files Reflecting CAA Controls

To model future episodes, the 1990 emissions were projected to 1999 levels to account for emission increases due to industrial growth, population growth, and growth in vehicle miles traveled. State-specific projection factors either were generated using the EPS 2.0 BEAFAC program, or were provided by the states. Once emissions were projected, control factors were applied to those categories in the inventory that have mandatory Clean Air Act control requirements (e.g., VOC RACT, NOx RACT, Stage II, enhanced I/M, new automobile standards, etc.). EPA worked with each state to produce state-specific growth factors and control strategies used to generate 1999 emission input files (7). NEPOOL provided estimated utility emission projections for weekday and weekend scenarios in 1999, which were incorporated into the future year emission files.

d.  Emission Files Reflecting 2007  EPA Proposed SIP Call Controls

Each state will estimate what their NOx and VOC emissions would be by the year 2007, reflecting EPA’s proposed SIP Call. This task will be accomplished by a two step process. The first step will be to project emissions using growth factors to account for increases or decreases in economic activity by industrial sector. The second step will be to apply control factors to source categories that would be regulated by the year 2007.

9.  1999 Runs

All four episodes (July 7-8 1988, July 10-11 1988, June 21-22 1988 and August 14-17 1987) will be run with emission files reflecting CAA controls for the New England Domain, and 1999 CAA ROM boundary conditions, except for the August episode (1999 ROM runs not available).  As recommended by EPA Region I, 1996 ROM runs will be used  as a surrogate for 1999  boundary conditions.

10. Emission Sensitivity Runs

A series of emission sensitivity runs will be performed with CALGRID in order to help design effective control strategies in the New England Domain.  The principal areas to be investigated are (1) the location of significant precursor sources, (2) the effectiveness of NOx emission reductions compared to VOC emission reductions, and (3) the effectiveness of low level emission reductions compared to elevated point source emission reductions.

11. 2007 Boundary Condition Runs

A series of runs will be performed using 1999 CAA emissions in the New England Doimain and boundary condtions derived from OTAG? regional modeling runs for the year 2007.  The purpose of these runs is to investigate how effective emission reductions in the OTAG region will be in reducing ozone levels in the New England domain

12. 2007 SIP Call Runs

A series of runs will be performed using 2007 SIP Call emissions in the New England Domain, and 2007 boundary condtions derived from OTAG? regional modeling runs for the year 2007. The purpose of these runs is to investigate how effective SIP Call emission reductions within the New England Domain will be in reducing ozone levels in the New England Domain.

13. Conclusions.

The modeling results will be analyzed and used to help determine the effectiveness of additional ozone controls within the New England domain, and upwind of the New England Domain.

14. References

1.   U.S. Environmental Protection Agency, 1991, Guideline for Regulatory Application

of the Urban Airshed Model,  EPA-450/4-91-013.

2.   Massachusetts Department of Environmental Protection, 1997,  Progress Report for

the New England Domain Ozone Attainment Demonstration.

3.   Yarmartino, R.J.., J.S, Scire, S.R. Hanna, G.R. Carmichael and Y.S. Chang, 1989:

CALGRID:  A Mesoscale Photochemical Grid Model: Volume 1: Model

Formulation Document. California ARB Report, Sacramento, CA.

4.   Morris R. et al., 1990, Users Guide for the Urban Airshed Model, Volume I: User's

Manual for UAM (CB-IV), EPA-450/4-90-007A.

5.  Scire,J.S., E.M. Insley, R.J. Martino, and M.E. Fernau, 1995: A Users’ Guide for the

CALMET Meteorological Model. Document No. 1406 prepared for the USDA

Forest Service, Cadilllac MI.

6.  VanUlden,A.P. and Holtslag, A.A.M., 1985, Estimation of Atmospheric Boundary

Layer Parameters Diffusion Application. Journal of Climate and Applied

Meteorology p. 1196 - 1207.7.

7.  U.S Environmental Protection Agency, 1995, Emission File Preparation for the Base

Case Performance Evaluation and Future Year Runs for New England Urban Airshed

Model Domain.

8.  Causley, M. et al., 1990, Users Guide for the Urban Airshed Model. Volume IV:

Manual for the Emissions Preprocessor System, EPA-450/4-90-007D.

APPENDIX A

CALGRID MODEL PLATFORM EQUIVALENCY

DEMOSTRATION FOR NEW ENGLAND DOMAIN

CALGRID Platform Equivalency demonstration

for Modeling 1-Hour Ozone Concentrations

Prepared by:

Massachusetts Department of Environmental Protection

New Hampshire Department of Environmental Services

Introduction

In 1991 the U.S. Environmental Protection Agency (USEPA) designated all or portions of Connecticut, Maine, Massachusetts, New Hampshire, and Rhode Island as non-attainment for the 1-hour National Ambient Air Quality Standard (NAAQS) for ozone.  The designations of “moderate”, “serious”, and “severe” were made to each non-attainment area depending on the severity of NAAQS exceedance monitored in the previous three years.  The Clean Air Act Amendments (CAAA) of 1990 require that each state with non-attainment areas provide an ozone Attainment Demonstration documenting emission reduction plans to reduce ozone levels within each state by a designated date.  As part of the Attainment Demonstration, the states with “serious” and “severe” designations are to produce photochemical modeling results to evaluate the effectiveness of planned emission reduction schemes.  With photochemical modeling, states can evaluate the effectiveness of various nitrogen oxide (NOx) and volatile organic compound (VOC) emission reduction strategies.

Prior to 1997, New England states relied on the Urban Airshed Model (UAM-IV) to perform photochemical modeling.  Reproductions of meteorological and emissions data for several ozone episodes produced modeling results similar to monitored values at inland locations but failed to reproduce satisfactory results along the sea coasts of New England.  It was decided by the New England states and USEPA Region I, to explore the use of the photochemical model CALGRID to improve modeling accuracy along the air/sea interfaces of New England.

CALGRID was designed by Earth Tech (Concord, MA) as a modular photochemical model with significant improvements in atmospheric physics and chemistry beyond UAM-IV.  Improvements include: an ozone deposition removal scheme including stomatal resistances of the vegitative canopy, inclusion of cloud observations to determine spatial and temporal solar insolation, capacity for additional mixing layers at user defined levels for improved air/sea interface ozone predictions, and the inclusion of BEIS2 and corresponding chemistry corrections.

In early 1997 Earth Tech was retained by the Northeast States for Coorinated Air Use Management (NESCAUM) to prepare a CALGRID performance evaluation. The performance evaluation was completed in July of 1997. CALGRID was compared statistically to actual monitoring data, and to UAM-IV modeling results. The performance statistics demonstrated that CALGRID performed as well as, if not better than UAM-IV for these episodes.  Performance statistics are presented in Attachment 1 (Photochemical Grid Modeling of Four High Ozone Episodes in the New England Domain: CALGRID vs. UAM-IV. A technical discussion of CALGRID is contained in Attachment 2 (Technical Justification for CALGRID as an Alternative to the UAM-IV Model in the New England Domain).

Model Platform Equivalency Methodology

CALGRID was installed at the Massachusetts Department of Enviromental Protection and the New Hampshire Department of Environmental Services in early September of 1997. Test modeling runs were then made utilizing Earth Tech input control files, meteorological data, boundary conditions, and emission files for four ozone episodes; June 21-22, 1988, July 7-8, 1988, July 10-11, 1988, and August 14-17, 1987.

CALGRID input files from Earth Tech were updated with Massachusetts and New Hampshire platform specifics and files were renamed to reflect the new platform preferences.  Each episode day was then modeled individually and the correct linkages were updated to allow for modeling multiple day episodes without loosing the previous days results.  CALGRID produces a binary concentration file and a brief ASCII summary file as outputs for each episode day.  After all of the modeling was completed,Masachusetts and New Hampshire summary files were compared to Earth Tech’s summary files with the DOS File Compare (FC) command.  Then the ground level ozone modeling results were extracted from the CALGRID binary concentration files for both platforms.  The extracted data consisted of maximum ozone concentrations for each grid cell at the assigned vertical layer (ground level) for the episode day.  Again, FC was used to compare the Massachusetts, New Hampshire and Earth Tech extracted files.  As a final step, graphics comparing the modeling results for each episode day.

Platform Equivalency Results

Exact platform equivalency was demonstrated for July 7, 8, 10, and 11, 1988 and for August 14 and 15, 1987.  Each of these episode days produced identical graphical results (Figures 1-6), file comparisons (FC) of extracted concentrations, and summary files (with the exception of time stamps and title lines, this was expected).

The Massachusetts and New Hampshire platform modeling results demonstrated higher ozone concentrations than Earth Tech’s in some locations for August 16 and 17, 1987 and for June 21 and 22, 1988 .  In the case of August 16 and 17, 1987, the Earth Tech input control listed an older version of meteorological data for CALGRID input file, than was provided to New Hampshire.  The discrepancy was verified with Earth Tech; the results for August 16 and 17 would have been identical if identical data sets had been used. In the case of the June 21-22, 1988 episode, the exact cause of the output file discrepancy is uncertain.  Massachusetts DEP achieved identical results to New Hampshire’s, therefore it is most likely an issue with a mismatched data set from Earth Tech.

Conclusion

In conclusion, CALGRID is installed correctly and operating correctly on the Massachusetts and the New Hampshire platform and capable of producing reliable

one ozone modeling results in the New England Domain.

ATTACHMENT 1

PHOTOCHEMICAL GRID MODELING OF FOUR HIGH OZONE

EPISODES IN THE NEW ENGLAND DOMAIN: CALGRID VS UAM-IV

ATTACHMENT 2

TECHNICAL JUSTIFICATION FOR CALGRID AS AN ALTERNATIVE

TO THE UAM-IV MODEL IN THE NEW ENGLAND DOMAIN