Appendix B

Preliminary Cost Estimates for 46 Different

On-Site Systems in the Barnstable, MA Community

Using COSMOTM: Costs of On-Site Management Options

83

COSMOTM:	Costs of On-Site Management Options facilitates estimation of costs for conventional septic systems including simple modifications such as chambers, styrene foam trenches, large diameter pipe trenches, panel block trenches, and biofabric treatment components that are used in the place of gravel in the drainfield trenches.  Modifications also include systems that use either gravity or pumped drainage around the drainfield as well as sand lined trenches in the drainfield.  The types of wastewater distribution techniques addressed in COSMOTM include conventional gravity distribution, pressure manifold distribution, low pressure pipe distribution, subsurface drip irrigation, and surface spray irrigation.  Pretreatment technologies addressed in COSMOTM include pressure-dosed single pass sand filters, pressure-dosed peat filters, pressure-dosed recirculating sand filters, constructed wetlands, aerobic treatment units, denitrification systems with blackwater and graywater separation, chlorination disinfection units, and ultraviolet light disinfection units.  Pressure-dosed at-grade systems, pressure-dosed sand mounds, and raised fill mounds that utilize gravity distribution are also included in COSMOTM .

Advanced on-site systems are made from combinations of technology components.  These components include pretreatment components, distribution technologies and other options.  During the system design stage system components are combined together like boxcars, flatcars, tankcars, engines and cabooses into a treatment train for a particular site.  COSMOTM includes cost information for a total of 46 different treatment trains that represent various combinations of on-site technology components (Table B-1).

COSMOTM consists of a series of 50 Excel worksheets on one diskette.  These include one input worksheet, 46 budget worksheets and three output worksheets.  Users enter information in just one of the 50 worksheets.  This worksheet, called LIST, contains over 170 pieces of information on technical system and site specifications as well as costs of equipment, materials, labor and economic parameters.

Table B-2 is a LIST worksheet for the Barnstable community and includes input data for COSMOTM.  This table is five pages long.  The first page of the LIST worksheet includes basic system specifications that are used to calculate the size of the systems.  The user must choose whether to use either the left side or right side of the page to determine system sizes.  In either case, these input numbers on the first page of the LIST worksheet can be changed by the user of COSMOTM in order to assess the effect of different loading rates, design flows etc. on system costs.  COSMOTM is a process to estimate costs, not a set of costs themselves.  We chose to use the right side of the page for this example and input system sizes directly into the LIST worksheet.  The left side of the page would be used if one desired to calculate system sizes from design flow, distribution system loading rates in gpd/ft2, pretreatment system loading rates in gpd/ft2, and drainfield size adjustment factors as a percentage.

We assumed a design flow of 440 gallons per day which represents a four bedroom home in Massachusetts.  We also assumed 0.74 gpd/ft2 as a representative soil loading rate for conventional septic systems in the Barnstable community, thereby resulting in the need for 594 square feet of effective absorption area.  Since Title 5 allows the use of sidewall absorption area for calculating the size of systems that utilize trenches, we assumed that one foot of sidewall absorption area was available on each side of each trench.  Therefore, a conventional trench

84

Table B-1.  Overview of systems evaluated by COSMOTM

Al	Septic tank & gravity distribution

A2	Septic tank & gravity distribution with chambers

A3	Septic tank & gravity distribution with styrene foam

A4	Septic tank & gravity distribution with large diameter pipes

A5	Septic tank & gravity distribution with panel blocks

A6	Septic tank & gravity distribution with biofabric treatment

B	Septic tank & gravity distribution w/ tile drainage

C	Septic tank & gravity distribution w/ pump tile drainage

D	Septic tank & gravity distribution w/sand lined trenches

E	Septic tank & gravity distribution w/sand lined trenches & tile drainage

F	Septic tank & gravity distribution w/sand lined trenches & pump drainage

GI	Septic tank & gravity distribution w/ pressure manifold

G2	Septic tank & gravity distribution w/ pressure manifold and chambers

G3	Septic W& & gravity distribution w/ pressure manifold and styrene foam

G4	Septic tank & gravity distribution w/ pressure manifold and large diameter pipes

G5	Septic tank & gravity distribution w/ pressure manifold and panel blocks

G6	Septic tank & gravity distribution w/ pressure manifold and biofabric treatment

HI	Septic tank & gravity distribution w/ sand filter pretreatment

H2	Septic tank & gravity distribution w/ peat filter pretreatment

I	Septic tank & gravity distribution w/ recirculating sand filter pretreatment

J	Septic tank & LPP distribution

K	Septic tank & LPP distribution w/ sand filter pretreatment

L	Septic tank & LPP distribution w/ recirculating sand filter pretreatment

M	Septic tank & drip distribution

N	Septic tank & drip distribution w/ sand filter pre-treatment

O	  Septic tank & drip distribution w/ recirculating sand filter pre-treatment

P        Septic tank & drip distribution w/ sand filter pre-treatment & chlorine disinfection

Q        Septic tank & drip distrib. w/ recirc. sand filter pre-treatment & chlorine disinfection

R        Septic tank & drip distribution w/ sand filter pre-treatment & UV disinfection

S        Septic tank & drip distribution w/ recirc. sand filter pretreatment & UV disinfection

T        Septic tank & spray irrigation w/ sand filter pre-treatment and chlorine disinfection

U       Septic tank & spray irrigation w/ recirc. sand filter pre-treatment and chlorination

V       Septic tank & spray irrigation w/ sand filter pre-treatment and UV

W      Septic tank & spray irrigation w/ recirculating sand filter pre-treatment and UV

X       Septic tank and gravity distribution with wetland cell

Y       Aerobic treatment unit and gravity distribution

Z1      Denitrification system w/ blackwater & graywater separation and gravity distribution

Z2      Denitrification system w/ blackwater & graywater separation and LPP distribution

ZAI    Septic tank & gravity distribution w/ 18" fill mound

ZA2    Septic tank & gravity distribution w/ 18" fill mound and chambers

ZA3    Septic tank & gravity distribution w/ 18" fill mound and styrene foam

ZA4    Septic tank & gravity distribution w/ 18" fill mound and large diameter pipes

ZA5    Septic tank & gravity distribution w/ 18" fill mound and panel blocks

ZA6    Septic tank & gravity distribution w/ 18" fill mound and biofabric treatment

ZB      Septic tank & LPP distribution in at-grade system

ZC      Septic tank & pressure-dosed sand mound system

85

system with 3-foot wide trenches required 120 linear feet of trench (594 sq.ft. / 5 ft. absorption area per linear foot of trench).  Similar assumptions were made for other types of system designs assuming varying trench widths and effective sidewall areas.  It is important to note here that COSMOTM assumes the use of generic designs that are not specific to the Massachusetts situation.  The cost estimates produced; therefore, do not reflect the true costs for systems that would be designed to meet the requirements of Title 5. Hence, these costs estimates should be only used to assess potential costs on an order of magnitude scale and the user is forewarned of the limitations of these data.

The second page of the LIST worksheet includes additional system design specifications and the frequencies of O&M monitoring inspections, as well as the anticipated frequencies of repair and maintenance activities.  For instance, we have assumed for this example that septic tanks for all systems will be pumped out every four years and that sand filter systems will be inspected twice per year.

The third page of the LIST worksheet includes estimates of the labor requirements for installation of all different types of system components on the left side of the page.  We have assumed that it takes a three person crew a combined total of 5 hours to install the septic tank itself, and that it takes 3 hours to install each 100 linear feet of drainfield trench for a conventional system.  These estimates are based on actual installation times provided by contractors in eastern North Carolina; however, the numbers can be changed by the user of COSMO if they are not accurate for a specific locality or contractor.  The right side of the page includes basic labor costs, heavy equipment costs, and general economic parameters including profit margin for the installer.  The data have been adjusted to be appropriate for the Barnstable community.

The fourth page of the LIST worksheet includes costs of all the pieces and parts of the 46 different systems included in COSMOTM.  These data should be adjusted so that they are accurate for the local cost center.  Most of the data currently included are for the Barnstable community; however, a few of the data were adapted from other communities when data from the Barnstable community were not available.

The final page of the LIST worksheet includes the computed trench lengths and treatment system sizes.  These numbers are calculated from the information on the first page and are transmitted to the upper left hand comer of each budget worksheet in the spreadsheet.  The materials, labor, and equipment needed to install each system are dependent upon the system size as determined in this fifth page of the LIST worksheet.  Since we chose option "I" at the top of the first page, the system sizes we input were transmitted directly to the fifth page of the LIST worksheet and used in the calculations in the budget spreadsheets.

Given this basic information in the LIST worksheet, COSMOTM automatically computes budgets for each of the 46 on-site systems included in the program.  Changes made to the information in the LIST worksheet are automatically transferred to the 46 specific budgets - one worksheet for each type of on-site system; that is, one worksheet for each treatment train.  These specific budget worksheets calculate the installation and long term management costs for each type of system.  They present a detailed breakdown of materials, labor and equipment costs, as

86

# Table B-2.  List Worksheet of COSMOTM  For Barnstable, Massachusetts.

# BASIC SYSTEM SPECIFICATIONS

System size can be input directly or can be computed from inputted design flow and loading rate.

Which method will be used? (0= compute system size, 1= input system size directly)............................................     1

DATA FOR COMPUTING SYSTEM SIZE	DIRECT INPUT OF SYSTEM SIZE

Design flow (gpd)	440	Distribution line length in drainfield (ft)

- Gravity distribution (9' centers)	120

Distribution system loading rates (gpd/ft2)		- Gravity distribution w/ pressure manifold	120

- Gravity distribution		- Gravity distribution w/ sand or peat filter	120

- Sand lined trench (SLT), buried gravity		- Gravity distribution w/ ATU pretreatment	120

- LPP distribution		- LPP distribution (5- centers)	200

- Drip distribution		- LPP distribution w/ sand filter pretreatment	200

- Spray irrigation		- Drip distribution (2'centm)	600

- Drip distribution w/ sand filter pretreatment	600

Pretreatment system loading rates (gpd/ft2)		- Sand lined trench (9- centers)	200

- Pressure-dosed single pass sand filter		- Tile drainage perimeter (100' lines)	272

- Pressure-dosed single pass peat filter		- Tile drainage perimeter w/ SLT (100' lines)	292

- Pressure-dosed recirculating sand filter		- Chamber system (9' centers)	120

- Denitrification system layered sand filter		- Styrene foam system (9' centers)	120

- Constructed wetland		- Large diameter pipes (9- centers)	300

- Sand mound (infiltrative area)		- Panel blocks (8- centers)	150

- Sand mound (basal area)		- Biofabric treatment system (9' centers)	100

Sand filter fine length (ft.)

Drainrield size adjustment		- Pressure-dosed single pass (2' centers)	110

Chamber system		- Pressure-dosed recirculating (2' centers)	45

Styrene foam		- Denitrification system layered sand filter	75

Large diameter pipes		Other systems' surface area (sq ft)

Panel blocks		- Pressure-dosed single pass sand filter	220

Biofabric treatment system		- Pressure-dosed single pass peat filter	120

Sand and peat filter systems		- Pressure-dosed recirculating sand filter	90

Aerobic treatment unit (ATU)		- Denitrification system layered sand filter	160

- Constructed wetland	733

- At-grade system (gravel bed)	594

- Sand mound infiltrative surface area	36			- Sand mound basal area	594

- Spray lrrigation system	9,800

LIST			                                                                                               Page 1

## ADDITIONAL SYSTEM SPECIFICATIONS AND MAINTENANCE SCHEDULE

Other system specifications	                                                   Maintenance frequencies (years)

Gravel bed width in at-grade system (ft)             12.0             	- Pumpout septic tank                                         4

- Desludge (aerobic treatment unit)	 	1

- Replace sewage effluent pump                         8

Sand depth in sand mound system (ft)                  2.0              	- Replace airrnotor puxnp (drip systeni)            8

- Replace 1/3 of gravity drainfield                   20

- Replace 1/3 of LPP drainfield                       20

Plant spacing in constructed wetand (ft)               2.0               	- Replace 1/3 of drip drainfield                       20

- Drip systern maintenance                               5

- Replace spray irrigation assemblies                5

Spray irrigation system                                                             	- Replace 1/2 of filter sand                              20

- Distance, filter to spray field (ft.)                   300	               - Replace 1/2 of recirc. filter sand                   20

- Sprinkler radius (ft.)                                       36                 	- Replace 1/2 of filter peat                               20

- Replace 1/3 of wetland cell contents            20

-Replace 1/3 of denitrification system filter  20

-Replace ATU unit                                         20

Additional system options (1=yes, 0=no)

Sand filter box? (I = box, 0 = liner)                         0

4" sleeves for LPP laterals?                                      0	              O&M inspection frequencies (times/year)

Septic tank outlet filter?                                           0	               - Septic tank & gravity distrib. Systern             1

Screened pump vault?                                              0	               - Tile drainage system                                       1

- Pump tile drainage systern                             1

- Pressure manifold system                               1

- Sand and peat filter systems                           2

- LPP system                                                     2

- Drip system                                                     2

- Spray irrigation system                                   2

- Wetland cell system                                        2

- Aerobic treatment unit                                    4

- Denitrification system                                    2

### LIST													Page 2

#### LABOR USE, LABOR AND EQUIPMENT COSTS, AND MISCELLANEOUS ECONOMIC PARAMETERS

INSTALLATION LABOR BY COMPONENT	LABOR COSTS

Components                                                             Hours	Item				                    Unit          Cost

Transport time to site (one-way)                                    1.0	Hand Labor (incl. Taxes, benefits, etc)	        hr            $22.00

Septic Tank setup (incl.  excavation and d-box)            5.0	Plumbing labor (denitrification system)                 hr            $50

Pump Tank setup (incl.  pump, floats alarm, control)    5.0                 			Electrical work (contracted)	                     unit           $100

O&M inspection				       unit	         $200

Distribution systems (hrs/100LF of line)					               Pumpout of septic tank or desludge ATU             unit          $325

-gravity distribution w/gravel			  3.0				Drip System Maintenance			       unit           $150

-gravity distribution without gravel		                 2.0

-gravity distribution with panel blocks                           3.0

-gravity distribution with bifabric treatment                   3.0				HEAVY EQUIPMENT COSTS

-gravity distribution w/ sand lined trenches                    4.0				Item                                                                       Unit        Cost

-fill mounds (hrs/100 sq ft)                                              1.0                                                  Backhoe					        day          $560

-pressure manifold (hrs/unit)			   5.0				 Excess transport cost			        unit              $0

-in-ground LPP					   5.0				 Trencher				         day          $400

-creation of gravel bed, sand mound, topsoil cover  	   2.0				 Vibrating plow (for drip line)		         day          $450

-drip irrigation					   1.0

-spray irrigation (hrs/10,000 sq ft spray area)		 12.0

ECONOMIC PARAMTERS

##### Perimeter drainage systems (hrs/100 LF)						   Item						        Value

-gravity tile drainage				  3.0				    Interest rate 					         9.0%

-pump tile drainage				  3.0				    Annual inflation				         3.5%

Length of loan payoff (years)			             30 	 Pretreatment units								    Installer’s margin on labor & equip. cost                           40%

-aerobic treatment units (hrs/unit)			12.0				    Administrative costs on total job cost		          15%

-single-pass sand filter (hrs/100 sq ft)		15.0

-recirculating sand filter (hrs/100 sq ft)		15.0

-wetland cell (hrs/100 sq ft)			 3.5

-chlorine disinfection (hrs/unit)			 2.0

-UV disinfection (hrs/unit)				 6.0

-dentrification system layered filter (hrs/100sq ft)	 28.0

-dentrification system plumbing(hrs/unit)		10.0

-peat filter (hrs/module)				  2.0

##### LIST																Page 3

MATERIALS PRICES                                                                                               MATERIALS PRICES

ITEM	UNIT	COST	ITEM                                                     Unit      Cost

Conventional systems			     Other systems (cont'd)

- septic tank	unit	$640	   - UV unit                                                         unit  $1,000

- riser	unit                  	$67             - UV unit, misc. materials (incl. container)    unit     $350

- graded gravel	ton	                    $11            - UV bulb                                                        unit       $75

- gravel delivery	ton	$2.75	- 30 mil PVC liner                                          sq ft    $0.44

- distribution box	unit	$53	- wetland cell discharge structure                  unit      $100

- septic tank outlet filter	unit	$74	- wetland plants                                              unit     $0.50

- filter fitbric                                                  sq ft     $0.06

Sand filter systems			- biofabric treatment units (4' x 3' x 7")        unit    $82.00

- filter sand (.35-.55 nun)	ton	$6.50	- peat treatment modules                           module  $1,000

- filter sand delivery	ton	$2.75	- peat treatment module delivery               module      $60

- filter box (concrete)	unit	$500

- filter cover (fiberglass w/ wood frame)	unit	$150	   Pumps and accessories

- pea gravel	ton	$12	- effluent purnp                                            unit        $285

- plywood (1/2" untreated)	sheet	$15	- airmotor pump (drip system)                     unit       $334

- pump tank                                                  unit        $540

Drip distribution systems			- control panel, floats, and alarm                 unit        $375

- control systern (good up to 720 gpd)	unit	$4,500	- screened pump vault                                  unit        $228

- control system delivery	unit	$120

- start-up inspection (drip system)	unit	$600	Pipe and drainline

- PE perforated drainfield pipe (4”)              foot      $0.50

Spray irrigation systems			- PVC, 1-1/4 in, sched. 40                            foot      $0.33

- sprinkler heads	unit	$20.00	- PVC, 1-1/4 in, sched. 160                          foot      $0.14

- state agency pennit	unit	$250	- PVC, 2", sched. 40                                    foot       $0.55

- engineering plans	unit	$1,000	- PVC9   4", sched. 40                                foot       $1.25

Other systems			- PVC, 4", sched. 160                                  foot       $1.40

- sandy loam topsoil	ton	$7	- drip irrigation tubing (1/2" WRAM)  1000’ roll $275.00

- sandy loarn topsoil delivery	ton	$2.75	- 4" slotted drainage tubing w/ sock     250'roll  $175.00

- fill sand delivery	ton	$2.75	- charnber end piece                                 unit        $9.00

- pressure manifold	unit	$172	- 12" styrene foaxn bundle (10')              unit       $44.00

- pressure manifold box	unit	$100	- 16" tall panel block, 4' length               unit       $54.00

- aerobic treatment unit	unit	$4,300	- 10”, PE tubing w/ filter wrap (20')       unit       $80.00

- chlorinator	unit	$250

- chlorinator, miscellaneous materials	unit	$50

- chlorine tablets	unit	$52

LIST			                                                              Page 4

COMPUTED LINE LENGTHS AND SURFACE AREAS (DO NOT ALTER!)

Distribution line length in drainfield (ft.)

- Gravity distribution (9' centers)	120

- Gravity distribution w/ pressure manifold	120

- Gravity distribution w/ sand or peat filter	120

- Gravity distribution w/ ATU pretreatment	120

- LPP distribution (5'centers)	200

- LPP distribution w/ sand filter pretreatment	200

- Drip distribution (2'centers)	600

- Drip distribution w/ sand filter pretreatment	600

- Sand lined trench (9' centers)	200

- Tile drainage perimeter (100' lines)	272

- Tile drainage perimeter w/ SLT (100'lines)	                            292

- Chamber system (9'centers)	                                                    120

- Styrene foam system (9' centers)	                                            120

- Large diameter pipes (9' centers)	                                            300

- Panel blocks (8' centers)	                                                         150

- Biofabric treatment system (9’ centers)	                                 100

Sand filter line leneth (ft.)

- Pressure-dosed single pass (2'ctr)	                                           110

- Pressure-dosed recirculating (2' ctr)	45

- Denitrification systein layered sand filter                              	75

Other system(sq ft)

- Pressure-dosed single pass sand filter	                                     220

- Pressure-dosed single pass peat filter	                                     120

- Pressure-dosed recirculating sand filter	90

- Denitrification system layered sand filter	                              160

- Constructed wetland	                                                              733

- At-grade system (gravel bed)	                                                594

- Sand mound basal area	594

- Sand mound infiltrative surface area	367

- Spray irrigation system	                                                       9,800

###### LIST                                                                                         							Page 5

well as operation and maintenance (O&M) costs over a 30 year period of system operation.  Table B-3 is an example of one of the budget worksheets for a relatively simple system a conventional septic system using gravity distribution.  Table B-4 is an example of a budget worksheet for a more complex type of on-site system that utilizes sand filter pretreatment and gravity distribution in the drainfield.  No reduction in the amount of drainfield area was assumed for systems that use pretreatment.

Finally, three summary worksheets tabulate the installation costs and O&M costs for all 46 systems.  A worksheet called COST SUMMARY (Table B-5) shows average installation and maintenance costs of all 46 systems in the original order presented in Table B-1.  A second sumnary worksheet called INSTALLATION COSTS displays only the installation costs.  Installation costs for the example LIST worksheet we used are shown in Table B-6.  This worksheet can be automatically sorted to arrange the systems from the least expensive to the most expensive system to install under the conditions specified in LIST.  A third worksheet called MONTHLY COSTS (Table B-7) shows the total monthly system costs (installation costs plus O&M costs) over the length of loan payoff period specified in the LIST worksheet.  This summary worksheet can also be automatically sorted to arrange the systems from the lowest average monthly costs to the highest average monthly costs.  This last summary worksheet basically represents what the costs would be if the installation and O&M costs were included in the mortgage for the home.

These summary worksheets should help people decide which system is the most economical among those that are technically appropriate for a particular site.  Also, COSMOTM can help community leaders preliminarily address some of the economic questions they may have regarding utilizing on-site technologies as the primary wastewater management infrastructure in their community.  Be aware however, that specific monitoring requirements to meet requirements for Title 5 systems can substantially increase monitoring costs to the point that on-site system monitoring, laboratory analyses, and certification of performance costs can exceed $2,000 per year.  These potential costs are not reflected in Tables B-5 and B-7.  COSMOTM can help community leaders evaluate the sensitivity of long term costs to such factors as changes in materials and labor costs and in the frequency of monitoring, maintenance, and repair of these advanced on-site technologies.

Finally, the individual 46 budget worksheets in COSMOTM can be used by engineers, contractors or homeowners who wish to evaluate the various materials and cost categories applicable to a particular technology.  For instance, a homeowner or developer might use the cost categories breakdown in the budget worksheet for a particular type of system to help them in their discussions with contractors as they negotiate the installation costs for an advanced on-site technology.  While the specific materials in these costs categories may vary substantially

depending upon local practices and regulations, it may prove beneficial for a homeowner or

developer to be able to understand the allocation of costs among these categories.  Likewise, a

contractor who is relatively unfamiliar with a particular technology, may be able to use a

particular budget worksheet to begin to understand what costs he/she is likely to incur during installation of that technology.

92

Table B-3.  Example COSMOTM budget sheet for a conventional system using gravity distribution.

A1

Septic tank & gravity distribution

cost

Design flow (gpd)                                                n/a                                                                                                                        unit                    #units       cost/unit      total cost     by category

Loading rate (gpd/ft2)		n/a		Materials

Drainfield line length (LF)		120		septic tank

tank	unit	1.00	$640.00	$640.00

river	unit	1.00	$67.00	$67.00

distribution box	unit	1.00	$53.00	$53.00

O&M Scbedule				outlet filter (optional)	unit	0.00	$74.00	$0.00

Inflation	    present						$760.00

year	cost	adjusted	value	gravity distribution

1	$200	$207	$196	             graded gravel		ton	                    11.00	              $11.25	$123.75

2	$200	$214	$192	        gravel delivery	ton	                    11.00	                $2.75	$30.25

3	$200	$222	$189	     perforated pipe	foot	                 120.00	                $0.50	$60.00

4	$525	$602	$486	     PVC line and fittings	unit	                     1.00		$45.00

5	$200	$238	$182							$259.00

6	$200	$246	$178

7	$200	$254	$175

8	$525	$691	$450

9	$200	$273	$168

10	$200	$282	$165

11	$200	$292	$162

12	$525	$793	$417

13	$200	$313	$156

14	$200	$324	$153	Labor

15	$200	$335	$150	septic tank		hour	5.00	$22.00	$110.00

16	$525	$910	$387	gravity distribution		hour	3.60	$22.00	$79.20

17	$200	$359	$144

18	$200	$371	$142	Additional for transport, set-up & inspection	hour	5.00	$22.00	$110.00

19	$200	$385	$139	TOTAL		hour	13.60	$22.00	$299.20	$299.20

20	$1,448	$2,881	$988

21	$200	$412	$134	Equipment: backhoe

22	$200	$426	$131	septic tank		hour	2.00

23	$200	$441	$129	gravity distribution		hour	2.00

24                 	$525	             $1,199	               $332	additional for transport, set-up,etc.	hour	2.00

25	$200	$473	$124	excess transportation charge (optional)                                 unit 		                 $0.00

26	$200	$489	$122	TOTAL	                                                                        day               1.00          $560.00	$560.00	$560.00

27	$200	$506	$119

28	$525	            $1,376	$307	TotaL Materials, Labor, & Equipment	$1,878.20

29	$200	               $542	$115

30	$200	               $561	$113	Overhead and Profit Margin

Administrative Costs	15%	$281.73

Total present value of O&M	                                $6,845		Margin on labor and equipment 	40%	$343.68

Total                                                                                                                                              	                  $625.41

Average monthly cost

Installation	          $20.14	TOTAL INSTALLATION				$2,503.61

O&M	          $19.01

TOTAL	          $39.16		Frequency	Frequency

O&M SCHEDULE	(no/year)	(years)	Cost

pumpout septic tank		4	$325.00

replace 1/3 of draintfield		20	$923.10

inspection	1		$200.00

93

Table B-4.            Example COSMOTM budget sheet for a pressure-dosed sand filter

Pretreatment system followed by a gravity distribution in the drainfield.

H1

Septic tank & gravity distribution w/sand filter pretreatment

Total           total by

Design flow (gpd)                       n/a                                                                                           units           #units    cost/unit      cost            category

Loading rate (gpd/ft 2)                 n/a            Materials

Drainfield size adjustment	           n/a	septic task

Drainfield line length (LF)     	 120	tank 	unit	1.00	$640.00    	$640.00

Filter Loading Rate  (gpd/ft2	 )	  n/a	riser	unit	1.00	$67.00	      $67.00

Filter surface area (sq. ft)           	220	distribution box	unit	1.00	$53.00	      $53.00

Filter line length (LF)            	110	outlet filter (optional)	unit	0.00	$74.00	        $0.00      $760.00

O&M Schedule			gravity distribution

Inflation    	present	graded gravel	ton	11.00	$11.25	     $123.75

year	cost	adjusted	value	gravel delivery		ton	              11.00	            $2.75       	$30.25

1	$400	$414	$392	perforated pipe		foot	           120.00	           $0.50	        $60.00

2	$400	$428	$385	PVC line and fiffiw		unit	               1.00		             $45.00

3	$400	$443	$378						                            $259.00

4	$725	$832	$672	sand filter

5	$400	$475	$363	pump tank		unit	              1.00	        $540.00	     $540.00

6	$400	$492	$357	riser		unit	               1.00	         $67.00	       $67.00

7	$400	$509	          $350                    filter box (9x 5'concrete tank=45ft2 )          unit	              0.00	        $500.00	        $0.00

8	$1,110	$1,462	          $952                    filter cover, fiber glass		unit	              0.00	        $150.00	        $0.00

9	$400	$545	 $337	 filter liner (30 mil PVC)		sq. ft	             589	            $0.44	       $259.16

10	$400	$564	$330	             plywood form (incl.misc hardware)	   4'x8' sheets	              8	          $14.50       	$127.60

11	$400	$584	$324	            effluent pump(inct. $50 for setup fittings)	unit	              1.00	         $335.00     	$335.00

12	$725	$ 1,096	$576	            screen pump vault (optional)	unit	              0.00	          $228.00	        $0.00

13	$400	$626	$312	            control panel, floats	unit	              1.00	          $375.00    	$375.00

14	$400	$647	$306	            electrical wiring (contracted)	unit	              1.00	         $100.00     	$100.00

15	$400	$670	$300	            filter sand (18” depth)	ton	              9.00	            $6.50	         $58.50

16	$1,110	$1,925	$817	            filter sand delivery	ton	              9.00	            $2.75	         $24.75

17	$400	$718	$299	            pea gravel (3' depth)	ton	2.00	$11.50	$23.00

15	$400	$743	$283	            graded gravel (10” depth)	ton	6.00	$11.25	$67.50

19	$400	$769	$278	            delivery of gravel	ton	8.00	$2.75	$22.00

20	$1,421	$2,828	$969	             PVC laterals	feet	121.00	$0.33	$39.93

21	$400	$824	$268	             PE sleeves	feet	110.00	$0.50	$55.00

22	$400	$853	$263	             PVC misc. (fittings, valves, supply line)	unit	1.00		$119.79

23	$400	$882	$258							$2,214.23

24	$1,110	$2,534	$701	Labor

25	$400	$945	$248	septic tank	hour	5.00	$22.00	$110.00

26	$400	$978	$243	gravity distribution	hour	4.00	$22.00	$88.00

27	$400	$1,013	$239	sand filter	hour	33.00	$22.00	$726.00

28	$725	$1,900	$424	pump tank setup	hour	5.00	$22.00	$110.00

29	$400	$1,085	$230	additional for inspection,transport time,	hour	7.00	$22.00	$154.00

setup, etc.

30	$400	$1,123	$225	TOTAL	hour	54.00	$22.00 	 $1,188.00	$1,188.00

Total present value of O&M	$12,069	Equipment: backhoe

septic tank		hour	2.00

gravity distribution	hour	2.00

Average monthly cost		sand filter		hour	11.00

Installation                   $58.70		pump tank setup		hour	2.00

$33.52		additional for transport, setup, etc	hour	2.00

TOTAL                        $92.23	               excess transportaion charge  (optional)	       unit			               $0.00

TOTAL                                                                 day              2.00       $560.00   	$1,120.00	$1,120.00

Total, Materials, Labor, & Equipment					$5,541.23

Overhead and Profit Margin

Administrative costs	15%			$831.18

Margin on labor and equipment	40%			$923.20

Total					$1,754.38

TOTAL INSTALLATION					$7,295.61

Frequency	Frequency

O&M SCHEDULE	(no/year)	(years)	Cost

pumpout septic tank		4	$325.00

replace 1/3 of drainfield		20	$454.58

filter pump replacement (incl. $100 labor)		8	$385.00

replace 1/2 of filter sand (incl $200 labor)		20	$241.63

inspection	2		$200.00

94

Table B-5.       Example of cost summary worksheet from COSMOTM showing installation

And long term O&M costs.

SUMMARY OF COSTS FOR ALTERNATIVE ON-SITE WASTEWATER MANAGEMENT SYSTEM                                                                                                                                                      .

Present value      TOTAL COST   Amortized Monthly   Avg.monthly       AVG.MONTHLY

Total materials&       of total 	             OVER LIFE      materials &             present value     COST OVER LIFE

ID	               System                                                                                                               Installation cost      O&M cost         OF SYSTEM      installation cost        of O&M cost           OF SYSTEMA

A1     Septic tank & gravity distribution                                                                                           $2,504                  $6,845                  $9,349	                     $20                        $19                         $39

A2     Septic tank & gravity distribution with chambers                                                                   $3,336                  $7,032                $10,368                     $27                        $20                         $46

A3	Septic tank & gravity distribution with styrene foam	$2,846	$6,920                $9,767		                $23                        $19                         $42

A4	Septic tank & gravity distribution with large diameter pipes	$3,816	$7,156               $10,971		               $31	                         $20 	$51

A5	Septic tank & gravity distribution with panel blocks	$4,855	$7,387               $12,242		                $39 	$21                         $60

A6	Septic tank & gravity distribution with biofabric treatment	$4,698	$7,341                $12,039		                $38                        $20	                           $58

B	Septic Tank & gravity distribution w/tile drainage	$3,241	$9,510	$12,751	$26	    $26	$52

C	Septic tank & gravity distribution w/pump tile drainage	$5,909	$10,367	$16,276	$48	$29	$76

D	Septic tank & gravity distribution w/sand lined trenches	$3,080	$2,477	$5,557	$25	$7	$32

E	Septic tank &gravity distribution w/sand lined trenches & tile drainage	$3,804	$10,200	$14,004	$31	$28	$59

F	Septic tank & gravity distribution w/sand lined trenches & pump drainage 	$6,540	$10,839	$17,379	$53	$30	$83

G1	Septic tank & gravity distribution w/pressure manifold	$4,774	$7,707	$12,482	$38	$21	$60

G2	Septic tank & gravity distribution  w/ pressure manifold and chambers	$5,593	$7,889	$13,482	$45	$22	$67

G3	Septic tank & gravity distribution w/pressure manifold and styrene foam	$5,103	$7,777	$12,881	$41	$22	$63

G4	Septic tank & gravity distribution w/pressure manifold and large diameter pipes	$6,073	$8,013	$14,085	$49	$22	$71

G5	Septic tank & gravity distribution w/pressure manifold and panel blocks	$7,112	$8,244	$15,356	$57	$23	$80

G6	Septic tank & gravity distribution w/pressure manifold and biofabric treatment 	$6,955	$8,198	$15,153	$56	$23	$79

H1	Septic tank & gravity distribution w/sand filter pretreatment	$7,296	$12,069	$19,364	$59	$34	$92

H2	Septic tank & gravity distribution w/ peat filter pretreatment	$11,808	$12,604	$24,412	$95	$35	$130

I	Septic tank &gravity distribution w/recirculating sand filter pretreatment	$6,226	$12,059	$18,285	$50	$33	$84

J	Septic tank & LPP distribution	$4,523	$12,319	$16,845	$36	$34	$71

K      Septic tank & LPP distribution w/sand filter pretreatment	$10,223	$13,338	$23,561	$82	$37	$119

L	Septic tank &LPP distribution w/ recirculating sand filter pretreatment	$8,232	$13,007	$21,239	$66	$36	$102

M     Septic tank & drip distribution	$11,163	$13,082	$24,245	$90	$36	$126

N      Septic tank & drip distribution w/sand filter pre-treatment	$15,994	$14,101	$30,095	$129	$39	$168

O	Septic tank & drip distribution w/recirculating sand filter pre-treatment	$14,872	$14,094                $28,966             	$120	$39	$159

P	Septic tank & drip distribution w/sand filter pre-treatment &chlorine disinfection	$16,408	$21,244                $37,652		$132	$59	$191

Q	Septic tank &drip distribution w/recirc sand filter pre-treatment &chlorine disinfection	$15,285	$21,237                $36,122		$123	$59	$182

R	Septic Tank & drip distribution w/ sand filter pre-treatment & UV disinfection	$17,867	$21,655                $39,522		$144	$60	$204

S	Septic tank &drip distribution w/recirc. Sand filter pre-treatment& UV disinfection	$16,744	$21,757                $38,501		$135	$60	$195

T	Septic tank & spray irrigation w/sand filter pre-treatment and chlorine disinfection	$11,890	$20,670	$32,560	$96	$57	$153

U	Septic tank &spray irrigation w/recirc. Sand filter pre-treatment and chlorination	$10,768	$20,663	$31,431	$87	$57	$144

V	Septic tank & spray irrigation w/sand filter pre-treatment and UV	$13,349	$21,190	$34,539	$107	$59	$166

W      Septic tank & spray irrigation w/recirculating sand filter pre-treatment and UV	$12,227	$21,183	$33,410	$98	$59                        $117

X	Septic tank and gravity distribution with wetland cell	$5,574	$23,231	$28,805	$45	$65	$109

Y	Aerobic treatment unit and gravity distribution	$8,037	$36,406	$44,443	$65	$101	$166

Z1     Denitrification system w/ blackwater & graywater separation and gravity distribution	$9,962	$13,508	$23,471	$80	$38	$118

Z2     Denitrification system w/blackwater &graywater separation and LPP distribution	$12,565	$15,070                $27,635	$101	$42	$143

ZA1   Septic tank & gravity distribution w/ 18” fill mound	$4,507	$6,850	$11,357	$36	$19	$55

ZA2   Septic tank & gravity distribution w/ 18 “ fill mound and chambers	$5,326	$7,032                $12,357 	$43	$20	$62

ZA3   Septic tank & gravity distribution w/ 18" fill mound and styrene foam	$4,836	$6,920	$11,756		$39	$19                          $58

ZA4   Septic tank & gravity distribution w/ 18" fill mound and large diameter pipes	$6,931	$7,156	$14,087		$56	$20	                          $76

ZA5   Septic tank & gravity distribution w/18” fill mound and panel blocks	$6,917	$7,062	$13,979		$56	$20	                          $75

ZA6   Septic tank & gravity distribution w/ 18" fill mound and biofabric treatment	$5,501	$7,004	$12,505		$44	$19                          $64

ZB     Septic tank & LPP distribution in at-grade system	$4,590	$12,345	$16,935		$37	$34                          $71

ZC     Septic tank & pressure-dosed sand mound system	$4,863	$12,407	$17,269	.	$39	$34	          $74

95

Table B-6.          Example of installation cost summary worksheet from COSMOTM.

INSTALLATION COSTS FOR ALTERNATIVE SYSTEMS (use ctrl-h to sort)

Total materials &

ID	System	installation cost

Al	Septic tank & gravity distribution	$2,504

A3	Septic tank & gravity distribution with styrene foam	$2,846

D	Septic tank & gravity distribution w/sand lined trenches	$3,080

B	Septic tank & gravity distribution w/ tile drainage	$3,241

A2	Septic tank & gravity distribution with chambers	$3,336

E	Septic tank & gravity distribution w/sand lined trenches & tile drainage	$3,804

A4	Septic tank & gravity distribution with large diameter pipes	$3,816

ZAI	Septic tank & gravity distribution w/ 18" fill mound	$4,507

J	Septic tank & LPP distribution	$4,523

ZB	Septic tank & LPP distribution in at-grade system	$4,590

A6	Septic tank & gravity distribution with biofabric treatment	$4,698

GI	Septic tank & gravity distribution w/ pressure manifold	$4,774

ZA3	Septic tank & gravity distribution w/ 18" fill mound and styrene foam	$4,836

A5	Septic tank & gravity distribution with panel blocks	$4,855

ZC	Septic tank & pressure-dosed sand mound system	$4,863

G3	Septic tank & gravity distribution w/ pressure manifold and styrene foam	$5,103

ZA2	Septic tank & gravity distribution w/ 18' fill mound and chambers	$5,326

ZA6	Septic tank & gravity distribution w/ 18" fill mound and biofabric  treatment                  	$5,501

X            Septic tank and gravity distribution with wetland cell	$5,574

G2	Septic tank & gravity distribution w/ pressure manifold and chambers	$5,593

C	Septic tank & gravity distribution w/ pump tile drainage	$5,909

G4	Septic tank & gravity distribution w/ pressure manifold and large diameter pipes	$6,073

I	Septic tank & gravity distribution w/ recirculating sand filter pretreatment	$6,226

F	Septic tank & gravity distribution w/sand lined trenches & pump drainage	$6,540

ZA5	Septic tank & gravity distribution w/ 18" fill mound and panel blocks	$6,917

ZA4	Septic tank & gravity distribution w/ 18" fill mound and large diameter pipes	$6,931

G6	Septic tank & gravity distribution w/ pressure manifold and biofabric treatment	$6,955

G5	Septic tank & gravity distribution w/ pressure manifold and panel blocks	$7,112

HI	Septic tank & gravity distribution w/ sand filter pretreatment	$7,296

Y	Aerobic treatment unit and gravity distribution	$8,037

L	Septic tank & LPP distribution w/ recirculating sand filter pretreatment	$8,232

ZI	Denitrification system w/ blackwater & graywater separation and gravity distribution	$9,962

K	Septic tank & LPP distribution w/ sand filter pretreatment	$10,223

U	Septic tank & spray irrigation w/ recirc. sand filter pre-treatment and chlorination	$10,768

M	Septic tank & drip distribution	$11,163

H2	Septic tank & gravity distribution w/ peat filter pretreatment	$11,808

T	Septic tank & spray irrigation w/ sand filter pre-treatment and chlorine disinfection	$11,890

W	Septic tank & spray irrigation w/ recirculating sand filter pre-treatment and LTV	$12,227

Z2	Denitrificafion system w/ blackwater & graywater separation and LPP distribution	$12,565

V	Septic tank & spray irrigation w/ sand filter pretreatment and UV	$13,349

0	Septic tank & drip distribution w/ recirculating sand filter pre-treatment	$14,872

Q	Septic tank & drip distrib. w/ recirc. sand filter pretreatment & chlorine disinfection	$15,285

N	Septic tank & drip distribution w/ sand filter pretreatment	$15,994

P	Septic tank & drip distribution w/ sand filter pre-treatrnent & chlorine disinfection	$16,408

S	Septic tank & drip distribution w/ recire. sand filter pretreatment & UV disinfection	$16,744

R	Septic tank & drip distribution w/ sand filter pretreatment & UV disinfection	$17,867

96

Table B-7.         Example of average monthly cost summary worksheet from COSMOTM.

AVERAGE MONTHLY COST OF ALTERNATIVE SYSTEMS (use ctrl-h to sort)

Average

ID	System	monthly cost

D	Septic tank & gravity distribution w/sand lined trenches	$32

Al	Septic tank & gravity distribution	$39

A3	Septic tank & gravity distribution with styrene foam	$42

A2	Septic tank & gravity distribution with chambers	$46

A4	Septic tank & gravity distribution with large diameter pipes	$51

B	Septic tank & gravity distribution w/ tile drainage	$52

ZA1	Septic tank & gravity distribution w/ 18' fill mound	$55

ZA3	Septic tank & gravity distribution w/ I 8" fill mound and styrene foam	$58

A6	Septic tank & gravity distribution with biofabric treatment	$58

E	Septic tank & gravity distribution w/sand lined trenches & tile drainage	$59

A5	Septic tank & gravity distribution with panel blocks	$60

G I	Septic tank & gravity distribution w/ pressure manifold	$60

ZA2	Septic tank & gravity distribution w/ 18" fill mound and chambers	$62

G3	Septic tank & gravity distribution w/ pressure manifold and styrene foam	$63

ZA6	Septic tank & gravity distribution w/ 18" fill mound and biofabric treatment	$64

G2            Septic tank & gravity distribution w/ pressure manifold and chambers	$67

J               Septic tank & LPP distribution	$71

G4             Septic tank & gravity distribution w/ pressure manifold and large diameter pipes	$71

ZB             Septic tank & LPP distribution in at-grade system	$71

ZC             Septic tank & pressure-dosed sand mound system	$74

ZA5           Septic tank & gravity distribution w/ 18" fill mound and panel blocks	$75

ZA4           Septic tank & gravity distribution w/ 18" fill mound and large diameter pipes	$76

C	Septic tank & gravity distribution w/ pump tile drainage	$76

G6	Septic tank & gravity distribution w/ pressure manifold and biofabric treatment	$79

G5	Septic tank & gravity distribution w/ pressure manifold and panel blocks	$80

F	Septic tank & gravity distribution w/sand lined trenches & pump drainage	$83

1	Septic tank & gravity distribution w/ recirculating sand filter pretreatment	$84

HI	Septic tank & gravity distribution w/ sand filter pretreatment	$92

L	Septic tank & LPP distribution w/ recirculating sand filter pretreatment	$102

X	Septic tank and gravity distribution with wetland cell	$109

Z1	Denitrification system w/ blackwater & graywater separation and gravity distribution	$118

K	Septic tank & LPP distribution w/ sand filter pretreatment	$119

M	Septic tank & drip distribution	$126

H2	Septic tank & gravity distribution w/ peat filter pretreatment	$130

Z2	Denitrification system w/ blackwater & graywater separation and LPP distribution	$143

U	Septic tank & spray irrigation w/ recire. sand filter pre-treatment and chlorination	$144

T	Septic tank & spray irrigation w/ sand filter pre-treatment and chlorine disinfection	$153

W	Septic tank & spray irrigation w/ recirculating sand filter pre-treatment and UV	$157

O	Septic tank & drip distribution w/ recirculating sand filter pre-treatment	$159

Y	Aerobic treatment unit and gravity distribution	$166

V	Septic tank & spray irrigation w/ sand filter pre-treatment and UV	$166

N	Septic tank & drip distribution wl sand filter pre-treatment	$168

Q	Septic tank & drip distrib. w/ recirc. sand filter pretreatment & chlorine disinfection	$182

P	Septic tank & drip distribution w/ sand filter pretreatment & chlorine disinfection	$191

S	              Septic tank & drip distribution w/ recirc. sand filter pre-treatment & UV disinfection	$195

R	Septic tank & drip distribution w/ sand filter pretreatment & UV disinfection	$204

97

To our knowledge, COSMOTM is the first attempt by anyone is the United States to provide a cost estimation tool for such a broad range of advanced on-site technologies.  Therefore, the user is warned that COSMTM has very substantial and severe limitations.  Many of these limitations are inherent in the fact that this type of cost estimation approach has not been attempted before for on-site technologies and in the fact that COSMOTM is new and relatively untested.  It is presented here as one of a variety of tools people can use and should not take the place of other accepted cost estimation techniques (e.g. placing a construction project out to public bid to obtain the lowest cost estimates, etc).  Other limitations of COSMOTM include:

·	Quality of data input equals quality of data output.  That is, the quality of information derived from COSMOTM Summary worksheets is directly related to how accurate the information is that is put into the LIST worksheet by the user.  The default values for the over 170 pieces of information in LIST are appropriate only to the one locality, a few contractors, and one time period.  The system specifications and cost data in the LIST worksheet must be checked for local accuracy and corrected for the rules, design approaches, and costs used locally.

·	Due to limitations of space, only 46 combinations of system components are addressed by COSMOTM.  All possible combinations of system modifications, pretreatment methods, wastewater distribution strategies, and types of systems are not included.

·	Use of advanced technologies may be limited in many areas.  The types of systems included in COSMOTM are biased towards what has been used or proposed for use in North Carolina.  Whether these systems can be used in Massachusetts will be determined by the state.

·	Similarly, system designs assumed in COSMOTM are specific to the North Carolina situation.  Generally, COSMOTM provides costs estimation for only one generic design of each treatment train.  The design typically used for a particular treatment train in local areas may be substantially different from that assumed in COSMOTM.  Even within a local area, the system designs used may vary substantially from engineer to engineer.  The design directly determines the materials needed to install the system.  Therefore, the list of materials and quantities specified in any COSMOTM budget worksheet may not be appropriate to your situation.  Basic design assumptions and technical specifications are described in a COSMOTM technical bulletin (Hoover and Renkow, 1997).

·	Costs may vary depending upon how extensively a particular type of system is used in a locality.  System costs are generally lower where a technology is used extensively than where a technology is just being introduced.  This is a result not only of competition, but also of installer and inspector familiarity with the technology.  It also is influenced by whether critical system components are readily available to installers at local supply houses.

·	Costs may vary substantially throughout the year depending upon market forces.  For instance, the same system may cost more to install during May or June when contractors have plenty of work than in January or February when construction activity has slowed down and contractors are hunting for work to keep their crews busy.  Of course, COSMOTM provides a

98

way to account for these effects since each budget worksheet contains a separate line item for administrative costs and profit margin.  These two pieces of information are input by the user in the LIST worksheet.

Costs associated with monitoring inspections are accounted for in COSMOTM through the O&M frequencies specified on page two of the LIST worksheet and the O&M inspection visit cost specified in the labor cost section of page three of the LIST worksheet.  The default value for a monitoring visit is $200 in the example given.  Yearly monitoring costs are determined by multiplying the inspection frequency by the cost per inspection.  If laboratory analyses is required for certain types of pretreatment units then estimates of the sampling and analyses costs can be included as a part of the O&M inspection costs input in LIST and the program rerun for those systems that would require effluent sampling.

Finally, please remember that COSMOTM provides order of magnitude cost estimates.  It is not intended to take the place of a public bidding process or other more specific costing methods to determine the true costs for installation and long term operation and maintenance of these on-site technologies.

99