LITERATURE CITED

Ackerman, M.T.  1989.  Compilation of Lakes, Ponds, Reservoirs and Impoundments Relative to the Massachusetts Lake Classification Program. Technical Services Branch, Massachusetts Division of Water Pollution Control, Department of Environmental Quality Engineering, Westborough, MA.

BCRPC. 1987. The Hoosic River Action Plan. Berkshire County Regional Planning Commission.  Pittsfield, MA.

Blasland, Bouck and Lee, Inc. 1998. Hoosic River Biota Monitoring Summary Report. Blasland, Bouck and Lee, Inc.  Syracuse, NY.

BRPC. 1998. Draft Assessment of Land Use Activities and Nonpoint Source Pollution in the Hoosic River Watershed. Nonpoint source project number 96-01/604. Berkshire Regional Planning Commission. Pittsfield MA.

Dallaire, T.D. 1999a.  Water Body Identification. Clean Water Act Section 305(b), Water Body System Database – (WBS).  Foxpro Application.  1999.  Massachusetts Department of Environmental Protection, Division of Watershed Management, Watershed Planning and Permitting Program, Database Manager.  Worcester, MA.

Dallaire, T.D. 1999b.  “NPDES Toxicity testing discharge monitoring report summary”. Toxicity Testing Database -- TOXTD.  Dbase Application.  1999.  Massachusetts Department of Environmental Protection, Division of Watershed Management, Watershed Planning and Permitting Program, Database Manager.  Worcester, MA.

Environment Canada.  04 November 1999.  Canadian Environmental Quality Guidelines.  [Online]. Environment Canada. http://www.ec.gc.ca/ceqg-rcqe/tistbl_e.doc [28 September 1998].

Environmental Law Reporter. 1988. Clean Water Deskbook. Environmental Law Institute. Washington, DC.

EOEA. 1997. MassGIS statewide 1:25,000 land use datalayer. Environmental Data Center. Executive Office of Environmental Affairs. Boston, MA.

EOEA. 1998. MassGIS Major Basins. Environmental Data Center. Executive Office of Environmental Affairs. Boston, MA.

EOEA. MassGIS Scanned USGS Quadrangle Images. Environmental Data Center. Executive Office of Environmental Affairs. Boston, MA.

EPA. 1997. Guidelines for Preparation of the Comprehensive State Water Quality Assessments (305(b) Reports) and Electronic Updates: Report Contents.  U. S. Environmental Protection Agency.  Assessment and Watershed Protection Division (4503F). Office of Wetlands, Oceans and Watersheds. Office of Water. Washington D.C.

Halliwell, D.B., W.A. Kimball and A.J. Screpetis.  1982.  Massachusetts Stream Classification Program Part I:  Inventory of Rivers and Streams.  Massachusetts Division of Fisheries and Wildlife, Department of Fisheries, Wildlife, and Recreational Vehicles and Massachusetts Division of Water Pollution Control, Department of Environmental Quality Engineering. Westborough, MA.

Hoosic River Watershed Association.  November 1998. Shoreline Survey:  South Branch Stream Team.  Hoosic River Watershed Association. Williamstown, MA.

- MA DEM. 1989a. Hudson River Basin Volume I: Inventory and Analysis of Current and Projected Water Use. Commonwealth of Massachusetts, Executive Office of Environmental Affairs, Department of Environmental Management. Boston, MA.

- MA DEM. 1989b. Hudson River Basin Volume II: Analysis of Water Resources and Water Uses. Commonwealth of Massachusetts, Executive Office of Environmental Affairs, Department of Environmental Management. Boston, MA.

- MA DEM. 1989c. Hudson River Basin Plan Volume III. Commonwealth of Massachusetts, Executive Office of Environmental Affairs, Department of Environmental Management. Boston, MA.

- MA DEM.  1993.  Areas of Critical Environmental Concern (ACEC) Program Guide June 1993.  Commonwealth of Massachusetts, Executive Office of Environmental Affairs, Department of Environmental Management, ACEC Program.  Boston, MA.

MA DEP. 1990a. Hoosic River Basin, 1985 Surveys Report. Department of Environmental Protection, Division of Water Pollution Control. Westborough, MA.

MA DEP. 1990b. 1986 Hoosic River Basin Survey, Water Quality Data, Wastewater Discharge Data.  Department of Environmental Protection, Division of Water Pollution Control. Westborough, MA.

MA DEP. 1991. Hoosic River Basin Survey Water Quality Data, Wastewater Discharge Data 1991. Department of Environmental Protection, Division of Water Pollution Control. Westborough, MA.

MA DEP.  1996. (Revision of 1995 report).  Massachusetts Surface Water Quality Standards.  Massachusetts Department of Environmental Protection, Division of Water Pollution Control, Technical Services Branch.  Westborough, MA  (Revision of 314 CMR 4.00, effective June 23, 1996).

MA DEP.  1997a.  Draft Quality Assurance Project Plan for 1997 Water, Fish Tissue, Habitat and Biological Studies in the Hoosic Basin, Worcester, MA. Attachments 1-14.

- Job Performance Report Project No. F-26-R-8. Re: 1976 Cold Water Fisheries Investigations, Survey and Inventory of Ponds and Streams in the Hoosic River. Department of Fisheries, Wildlife and Environmental Law Enforcement. MA.

- Johnson, A. S. 1982.  Memorandum to R. Isaac.  Re: Hoosic River Macroinvertebrate Study.  Massachusetts Division of Water Pollution Control, Technical Services Branch, Westborough, MA.

- Bode, R.  1986.  Memorandum to C. Callinan.  Re: 1985 Hoosic River Biomonitoring Results.  New York State Department of Environmental Conservation, Division of Water, Stream Biomonitoring Unit, NY.

- Maietta, R. J. 1987.  Memorandum to A.S. Johnson.  Re: 1986 Hoosic River Fish Toxics Monitoring.  Massachusetts Division of Water Pollution Control, Technical Services Branch.  Westborough, MA.

- Maietta, R. J. 1989.  Memorandum to A.S. Johnson.  Re: 1988 Hoosic River Fish Toxics Monitoring (BIO88-6).  Massachusetts Division of Water Pollution Control, Technical Services Branch. Westborough, MA.

- Maietta, R. J. 1990.  Memorandum to A.S. Johnson.  Re: Hoosic River Tributaries Fish Toxics Monitoring (BIO89-8).  Massachusetts Division of Water Pollution Control, Technical Services Branch. Westborough, MA.

- MA DPH. 1989.  Public Health Advisory:  Hoosic River Fish.  The Commonwealth of Massachusetts, Executive Office of Human Services.  Boston, MA.

- MA DPH. 1994.  Public Health Fish Consumption Advisory:  Hoosic River.  The Commonwealth of Massachusetts, Executive Office of Human Services.  Boston, MA.

- Weinberg, A.  (Alan.Weinberg@state.ma.us) 28 April 1992.  Request for biological monitoring of fish tissue samples in the Hoosic River Basin.  Email to R. Nuzzo (Robert.Nuzzo@state.ma.us).

- Nuzzo, R. M.  1993.  Technical Memorandum 91-3 to A. Weinberg  Re: WWSC-36: Hoosic River Bioassays--1991.  Massachusetts Division of Water Pollution Control.  Grafton, MA.

- Nuzzo, R. M.  1993 Hoosic River Bioassay PCB Data/Chart/Table. Unpublished data. Grafton, MA.

- Nuss, J.M. 1996. Letter to C.G. Wanat.  Re: American Annuity Group, Inc. Brown Street Facility, North Adams, Massachusetts – Results of Biota Sampling and Analysis.  Project #: 082.28#2.  Blasland, Bouck and Lee, Inc.  Syracuse, NY.

- DEP DWM.  1997 WMA Database Printout Hudson River Basin (Database manager: Levangie, current).  Boston, MA.

- DEP DWM.  1997 NPDES permit file review of Hoosic River Basin discharges.  Unpublished data. Worcester, MA.

MA DEP. 1997b.  1997 Synoptic Lake Surveys in the Hoosic River Basin. Unpublished data.  Massachusetts Department of Environmental Protection, Division of Watershed Management. Worcester, MA.

MA DEP.  1998. Commonwealth of Massachusetts Summary of Water Quality 1998.  Massachusetts Department of Environmental Protection, Division of Watershed Management.  Worcester, MA.

MA DEP.  1999a.  Open NPDES permit files.  Massachusetts Department of Environmental Protection, Division of Watershed Management. Worcester, MA.

MA DEP.  1999b.  Open NPDES and WMA permit files.  Massachusetts Department of Environmental Protection, Bureau of Resource Protection. Springfield, MA.

Nolan, P.M. 1999. Personal communication. Hoosic River PCB’s and other contaminants in caddis larvae tissue and sediments.  Hoosic River Watershed Association presentation at the State of the River Conference. October 1, 1999.

North Branch Stream Team.  1997.  Shoreline Survey Report North Branch of the Hoosic River.  North Branch Stream Team.

Persaud, D., R. Jaagumagi, and A. Hayton.  1993.  Guidelines for the protection and management of aquatic sediment quality in Ontario.  Water Resources Branch, Ontario Ministry of the Environment.  Queen’s Printer for Ontario. Canada.

Plotkin, S.P., and P.T. Kostecki. 1988. Environmental Risk Assessment for the Hoosic River: Point Source Discharge Data.  Water Resources Research Center. University of Massachusetts, Amherst, MA.

Prendergast, W.  1999. Personal Communication.  Status of surface drinking water supplies in the Hoosic River Basin.  Department of Environmental Protection, Bureau of Resource Protection, Drinking Water Program. Springfield,  MA.

Riggs, Dale. 1998. 1998 Green River Fecal Coliform Monitoring project.  Hoosic River Watershed Association presentation at the State of the River Conference. October 30, 1999.

Rojko, A. M., W. A. Kimball, and A. J. Screpetis.  1995.  Designated Outstanding Resource Waters of Massachusetts 1995.  Massachusetts Executive Office of Environmental Affairs, Department of Environmental Protection, Bureau of Resource Protection, Office of Watershed Management.  Grafton, MA.

- Schlesinger, R.C. 1999a. Personal Communication.  Temperature monitoring in the Hoosic River.  Hoosic River Watershed Association, South Branch Coordinator.  Adams, MA

Schlesinger, R.C.  (rschles@bcn.net) August and September 1999b.  Optic StowAway temperature datasets and site locations.  Emails to L. Kennedy (Laurie.Kennedy@state.ma.us).

- Schlesinger, R.C. 1999c. Unpublished Report. Preliminary Report on Temperature Monitoring of the Hoosic River in Adams, Mass.  Hoosic River Watershed Association, South Branch Coordinator.  Adams, MA.

Schropp, S.J. and H.L. Windon (editors).  1988.  A guide to the interpretation of metal concentrations in estuarine sediments.  Coastal Zone Management Section, Florida Department of Environmental Regulation, 2600 Blair Stone Rod. Tallahassee, FL.

Socolow, R.S., C.R. Leighton, J.L. Zanca, and L.R. Ramsbey.  1998.  Water Resources Data for Massachusetts and Rhode Island, Water Year 1997.  U.S. Geological Survey Report MA-RI-97-1.  Water Resources Division.  Marlborough, MA.

Socolow, R.S., L.Y. Comeau, J.L. Zanca, and L.R. Ramsbey.  1999.  Water Resources Data for Massachusetts and Rhode Island, Water Year 1998.  U.S. Geological Survey Report MA-RI-98-1. Water Resources Division.  Marlborough, MA.

USGS.  13 June 1997.  Hydrograph and station description for 01332500.  [Online]. USGS. http://mass1.er.usgs.gov/cgi-bin/rts_gen_station_pg.py?station=01332500 [1997, June 13, 7, 9 and July 10].

USGS.  1998.  Unpublished Data. Provisional low-flow frequency statistics for gaging stations (3.5” floppy disc).  United States Geological Survey, Water Resources Division. Marlborough, MA.

Wandle Jr., S.W.  1984.  Gazetteer of Hydrological Characteristics of Streams in Massachusetts— Hudson River Basin.  U.S. Geological Survey Water Resources Investigation Report 83-4250. Boston, MA.

# APPENDIX A - DEP DWM QA/QC

Introduction

Quality Assurance/Quality Control (QA/QC) activities were conducted as part of the DEP DWM Hudson River Basin Monitoring Survey in 1997.  This QA/QC review was conducted to ensure that the collection and analysis of the monitoring data was of high quality.  The 1997 monitoring data subjected to this QA/QC review includes the following: discrete water samples, fish tissue samples and in-situ water quality measurements.  All discrete water sample and fish tissue monitoring data were reviewed independently by the Wall Experiment Station’s (WES) Quality Assurance Program and the Division of Watershed Management’s  (DWM) Quality Assurance Officer and Assessment Coordinator. All in-situ water quality measurements were reviewed independently by DWM’s Hydrolab® Instrument Coordinator and Database Manager.  Data that fell outside established QA/QC acceptance criteria were investigated and may have been subject to censoring. This Quality Assurance/Quality Control appendix is divided into three sections: A.1 field and laboratory data objectives; A.2 QA/QC data; A.3 analytical methods.

### A.1 Field and Laboratory QA/QC Objectives

Data collected by DWM in the 1997 Hudson River Basin survey was subject to field and laboratory data quality objectives.  Section A.1.1 outlines the field collection objectives and laboratory quality control for discrete water samples.  Section A.1.2 includes fish tissue laboratory quality control methods and Section A.1.3 includes Hydrolab QA/QC procedures.

A.1.1	Discrete Water Sample Data

FIELD

The collection of discrete water sample analytes followed DWM Standard Operating Procedures (1,2).  Four field collection quality control criteria were applied to the Hudson River Basin 1997 discrete water sample data:

1.0	Sampling/Analysis Holding Time: Each analyte has a standard holding time that has been established to ensure sample/analysis integrity.  Refer to DWM Standard Operating Procedure Table 1.0 CN# 1.0 (2) for a complete listing.  If the standard holding time was exceeded, this objective is violated.

2.0	Quality Control Sample Frequency: At a minimum, one field blank and one replicate must be collected for every ten samples by any given sampling crew on any given date. If less than one quality control sample per 10 field samples was collected, this objective is violated.

- Field Blank: Field blanks were prepared at the DWM Worcester Office.  Reagent grade water was transported into the field where it was transferred into a sample container and fixed using the same method as its corresponding field sample.   All blanks were submitted to WES laboratory “blind”.  If the field blanks were significantly different (>2 standard deviations (9)) from the detection limit, this data quality objective is violated.

4.0	Field Replicate: Two independent samples were collected from the same location and as close as possible to the same time in the field.  Both samples were submitted to WES laboratory “blind”. In order for this data quality objective to be met, the results must be:

<20% Relative Percent Difference (RPD) for method detection limits >1mg/L

<30% RPD for method detection limits <1mg/L

A detailed QA/QC summary of the four data quality objectives and additional DWM quality assurance observations for the 1997 Hudson River Basin data can be found in the 1997 Watershed QA/QC Assessment Report (8).

### Laboratory

Discrete water sample analysis followed EPA-approved laboratory QA/QC methodologies in accordance with WES Standard Operating Procedures (3).  The quality of data generated at WES was determined by analyzing the results of a variety of quality control procedures including but not limited to:

Low Calibration Standards – Checks the stability of the instrument’s calibration curve. Analyzes the accuracy of an instrument’s calibration within a 5% range.

Reference Standards  – Generally, a second source standard (a standard different from the calibration stock standard) that analyzes the accuracy of an instrument’s calibration within a 5% range.

Laboratory Reagent Blank/Method Blank (LRB) – Reagent grade water (de-ionized) extracted with every sample set to ensure that the system is free of target analytes (< MDL).

Duplicate Sample – Measures the precision (% Relative Percent Difference) of the extraction and analytical process.  The acceptable laboratory %RPD range is typically  25%.

Spike Sample (Laboratory Fortified Blank - LFB and Laboratory Fortified Matrix - LFB)– Measures the accuracy (% Recovery) of an analytical method.  The acceptable laboratory % recovery range is typically between 80 – 120% for LFB samples and 70 –130% for LFM discrete water samples.

The WES Laboratory is solely responsible for the administration of its Quality Assurance Program and Standard Operating Procedures.  The frequency of the laboratory’s quality control procedure was at times inconsistent with their Quality Assurance Plan (3).  In these circumstances additional quality assurance procedures were used.  Refer to WES’s Quality Assurance Plan (3) for specific laboratory analytical QA/QC criteria.  WES laboratory releases discrete water sample data when their established QA/QC criteria are met or the data are labeled as outside of these criteria.

A.1.2	Fish Tissue Data

Fish were collected and processed according to DWM’s Quality Assurance Project Plan (4). Tissue preparation and analysis strictly adhered to EPA-approved laboratory QA/QC methodologies in accordance with WES Standard Operating Procedures (6,7).  The quality of tissue data generated at WES was determined by incorporating a variety of quality control samples:

Laboratory Reagent Blank/Method Blank (LRB) – Clean clam tissue matrix extracted with every sample set to ensure that the system is free of target analytes (< MDL).

Laboratory Fortified Blank (LFB) – Clean clam tissue matrix spiked with a low concentration of target compounds.  LFB results are used to establish accuracy of system’s performance.  The acceptable laboratory % recovery range is typically 80 – 120%.

Laboratory Fortified Matrix (LFM) – Tissue matrix spiked with a low concentration of a target compound.  LFM results are used to establish accuracy of the extraction and analytical process.  The acceptable laboratory % recovery range is typically between 70 – 130% for metal analysis and 60 –140% for PCB/Organochlorine Pesticide analysis

Quality Control Standard (QCS) – A pre-spiked secondary tissue sample.  QCS results are used to establish accuracy in the extraction and test methods.  The acceptable laboratory  % recovery range is typically between 80–120%.

The WES Laboratory is solely responsible for the administration of its Quality Assurance Program and Standard Operating Procedures.  The frequency of the laboratory’s quality control procedure was at times inconsistent with their Quality Assurance Plan (3).  In these circumstances additional quality assurance procedures were used.  Refer to WES’s Quality Assurance Plan (3) for specific laboratory analytical QA/QC criteria.  WES laboratory releases tissue data when their established QA/QC criteria are met or the data are labeled as outside of these criteria.

A.1.3	In-situ Water Quality Analysis

Trained DWM staff members conducted in-situ measurements using a Hydrolab® Multiprobe Series 3 analyzer.  The Hydrolab® Multiprobe Series 3 analyzer measures dissolved oxygen, temperature, pH, conductivity, depth and turbidity and calculates total dissolved solids and % saturation of dissolved oxygen.  To ensure the quality of the in-situ data, the following QA/QC steps were taken:

1.0	Pre-Calibration: After each analytical probe on the Hydrolab® analyzer was calibrated, a pre-calibration check was conducted.  A low ionic standard was first analyzed to check the accuracy of the instrument.  Then an instrument check consisting of de-ionized water was analyzed to check the instrument for contamination.  The instrument check criteria is based on de-ionized water that that had been stored and vented to the air for at least three days.  If the pre-calibration check achieved the criteria in Table A.1-1 then the instrument was ready for field analysis but if the pre-calibration check failed to achieve the low ionic standard criteria than the instrument was re-calibrated and a second low ionic and instrument check was analyzed.  If the instrument failed to meet the established low ionic standard criteria a second time the Hydrolab® instrument could not be used to collect data and maintenance was scheduled. Refer to the DWM Hydrolab® Standard Operating Procedure (5).

- Post Survey Check: Once the Hydrolab® was returned from field sampling, a post survey check was performed to ensure that no malfunction or damage had occurred to any of the Hydrolab® probes.  The low ionic standard and the instrument check were re-analyzed.  If the post survey check achieved the established criteria in Table A.1-1, the data was deemed acceptable and was ready for the data reduction QA/QC step.  If, however, the post calibration failed to meet the criteria, the Hydrolab® Coordinator investigated the cause and recommended censoring of affected data to the Database Manager.

3.0	Data Reduction: The Hydrolab® Coordinator and Database Manager reviewed the Hydrolab® data for instability, instrument malfunction, operator technique and aberrant trends.  If any of these conditions were detected, the data was investigated and may have been recommended for censoring.  The Database Manager electronically tagged all data recommended for censoring in the database.

Table A.1-1 Hydrolab® Multiprobe Series 3 analyzer pre and post calibration specifications.

| Hydrolab® Analyte | Low-Ionic Standard | Instrument Check * |
| --- | --- | --- |
| Dissolved Oxygen | Saturation Chart (dependant on temperature & barometric pressure ) |  |
| pH | 6.90 ±1% | 5.6 ±0.2 units |
| Specific Conductance | 74 ±1% | 1.0 ±1% |
| Turbidity | 0.0 ±5% | 0.0 ±5% |
| Temperature | Ambient ±0.15°C** | Ambient ±0.15°C** |
| Depth | Field Calibrated ±0.45m | Field Calibrated ±0.45m |
| Salinity | Not Applicable | 0.0 ±0.2ppt |
| Redox | Not Applicable | 0.0±20mV |

* Based on Division of Watershed Management’s filtered de-ionized water

** Compared to the DWM laboratory’s wall thermometer

### References

(1)	MA DEP.  1999. CN 1.0 Grab Collection Techniques for DWM Water Quality Sampling 1999.  Massachusetts Department of Environmental Protection, Division of Watershed Management.  Worcester, MA.

(2)	MA DEP.  1999. CN 1.0 Grab Collection Techniques for DWM Water Quality Sampling, 1999. Massachusetts Department of Environmental Protection, Division of Watershed Management.  Worcester, MA.

(3)	MA DEP.  1995.  Laboratory Quality Assurance Plan and Standard Operating Procedures, Appendix B and C.  January 1995. Massachusetts Department of Environmental Protection, Division of Environmental Analysis, Senator William X. Wall Experiment Station.  Lawrence MA.

(4)	MA DEP.  1999.  CN 13.0 Fish Contaminant Monitoring Program Quality Assurance Project Plan, 1999. Massachusetts Department of Environmental Protection, Division of Watershed Management.  Worcester, MA.

(5)	MA DEP.  1999.  CN 4.0 Hydrolab® Multiprobe Series 3 and Appendixes CN 4.1 – 4.5, 1999. Massachusetts Department of Environmental Protection, Division of Watershed Management.  Worcester, MA.

(6)	MA DEP.  1995.  Laboratory Quality Assurance and Standard Operating Procedures, “Wet Tissue Digestion for Metals Analysis by Atomic Absorption Spectroscopy and/or ICP Emission Spectroscopy (Fish, Clams, Mussels, Etc.)”, January 1995. Massachusetts Department of Environmental Protection, Division of Environmental Analysis, Senator William X. Wall Experiment Station.  Lawrence MA.

(7)	MA DEP.  1995.  Laboratory Quality Assurance and Standard Operating Procedures, AOAC Method 983.21 “PCBs and Organochlorine Pesticides in Biological Tissue”, January 1995. Massachusetts Department of Environmental Protection, Division of Environmental Analysis, Senator William X. Wall Experiment Station.  Lawrence MA.

(8)	MA DEP.  1999.  CN 9.0 1997 QA/QC Assessment Report , 1999. Massachusetts Department of Environmental Protection, Division of Watershed Management.  Worcester, MA.

(9)	Clesceri, L.S., A.E. Greenberg, and A.D. Eaton (editors). 1998. Standard Methods for the Examination of Water and Wastewater, 20th Edition, American Public Health Association, Washington D.C.

A.2 QA/QC Data

Field blank and replicate sampling results for the discrete water quality sampling (physico/chemical and bacteriological) are provided in Tables A.2-1 through A.2-4.  Tables A.2-5 and A.2-6 contain laboratory QA/QC data for organics in tissue analyses and metals in tissue analyses, respectively.

Hudson River Basin 1997 Water Quality Assessment Report Appendix A- A 5 -

11wqar.docDWM CN 15.0

Table A.2-1. 1997 DEP DWM Hudson River Basin instream physico/chemical QA/QC field blank data.  (All units expressed in mg/L unless otherwise specified.)

Time	Alkalinity	Hardness	Specific 	Chloride	Suspended	Total 	Turbidity	Total 	Ammonia	Nitrate	Total

(24hr)	Conductivity 	 Solids	Solids	 (NTU)	Kjeldahl 	Phosphorus

(umhos)	Nitrogen

______________________________________________________________________________________________________

Field Blank Sample

11-0002	BLANK	7/8/97	9:55	2.0	0.8	--	<1.0	<2.5	--	**  	--	<0.02	<0.02	<0.01

11-0009	BLANK	7/8/97	10:40	4.0	<0.66 	--	<1.0	<2.5	--	**  	--	<0.02	<0.02	<0.01

11-0017	BLANK	8/12/97	**	3.0	<0.66 	--	1.0	<2.5	--	<0.1  	--	<0.02	<0.02	<0.01

11-0025	BLANK	8/12/97	**	3.0	<0.66 	--	1.0	<2.5	--	<0.1  	--	<0.02	<0.02	<0.01

11-0031	BLANK	9/16/97	10:50	3.0	<0.66 	--	<1.0	<2.5	--	0.10	--	<0.02	<0.02	<0.01

11-0036	BLANK	9/16/97	10:57	2.0	<0.66 	--	<1.0	<2.5	--	0.10	--	<0.02	<0.02	<0.01

____________________________________________________________________________________________________________________________

______________________________________________________________________________________________________

Hudson River Basin 1997 Water Quality Assessment Report Appendix A- A 6 -

11wqar.docDWM CN 15.0

Table A.2-2. 1997 DEP DWM Hudson River Basin instream physico/chemical QA/QC field replicate data.  (All units expressed in mg/L unless otherwise specified.)

Time	Alkalinity	Hardness	Specific 	Chloride	Suspended	Total 	Turbidity	Total 	Ammonia	Nitrate	Total

(24hr)	Conductivity 	 Solids	Solids	 (NTU)	Kjeldahl 	Phosphorus

(umhos)	Nitrogen

________________________________________________________________________________________________________________________

HOOSIC RIVER,  Station: HR08A

11-0014	11-0015	8/12/97	9:49	78  	77  	--	13  	<2.5	--	0.80	--	0.03	0.03	0.03

11-0015	11-0014	8/12/97	9:49	78  	76  	--	12  	<2.5	--	0.80	--	0.04	0.03	0.02

Relative Percent Difference (RPD):	0.0%	1.3%	8.0%	0.0%	0.0%	28.6%	0.0%	40.0%

___________________________________________________________________________________________________________________________________________________________________________________________________________

HOOSIC RIVER,  Station: HR07A

11-0028	11-0029	9/16/97	9:57	123  	134  	--	22  	<2.5	--	0.80	--	<0.02	0.45	0.01

11-0029	11-0028	9/16/97	9:57	127  	140  	--	22  	<2.5	--	0.80	--	<0.02	0.44	0.01

Relative Percent Difference (RPD):	3.2%	4.4%	0.0%	0.0%	0.0%	0.0%	2.2%	0.0%

___________________________________________________________________________________________________________________________________________________________________________________________________________

HOOSIC RIVER,  Station: HR07

11-0004	11-0005	7/8/97	11:50	101  	90  	--	18  	11  	--	**  	--	0.19	0.36	0.04

11-0005	11-0004	7/8/97	11:50	101  	105  	--	18  	12  	--	**  	--	0.24	0.37	0.03

Relative Percent Difference (RPD):	0.0%	15.4%	0.0%	8.7%	23.3%	2.7%	28.6%

___________________________________________________________________________________________________________________________________________________________________________________________________________

HOOSIC RIVER,  Station: HR03D

11-0010	11-0011	7/8/97	11:30	117  	126  	--	23  	5.2	--	**  	--	0.12	0.54	0.05

11-0011	11-0010	7/8/97	11:30	117  	125  	--	23  	7.0	--	**  	--	0.13	0.55	0.03

Relative Percent Difference (RPD):	0.0%	0.8%	0.0%	29.5%	8.0%	1.8%	50.0%

___________________________________________________________________________________________________________________________________________________________________________________________________________

HOOSIC RIVER,  Station: HR02

11-0037	11-0038	9/16/97	11:25	126  	141  	--	25  	4.4	--	1.1  	--	0.37	0.44	0.07

11-0038	11-0037	9/16/97	11:38	126  	142  	--	25  	4.4	--	1.1  	--	0.37	0.44	0.07

Relative Percent Difference (RPD):	0.0%	0.7%	0.0%	0.0%	0.0%	0.0%	0.0%	0.0%

GREEN RIVER,  Station: GN01

11-0021	11-0022	8/12/97	10:20	102  	103  	--	9.0	<2.5	--	0.90	--	<0.02	0.35	0.01

11-0022	11-0021	8/12/97	10:20	102  	104  	--	9.0	<2.5	--	0.80	--	<0.02	0.33	0.01

Relative Percent Difference (RPD):	0.0%	1.0%	0.0%	0.0%	11.8%	0.0%	5.9%	0.0%

___________________________________________________________________________________________________________________________________________________________________________________________________________

** = missing/censored data          -- = no data

Table A.2-3. 1997 DEP DWM Hudson River Basin instream bacteriological QA/QC field blank data.  (Units expressed in colonies/100ml.)

Sample	Time	FECAL	E-COLI	ENTEROCOCCUS	AEROMONAS

Id	(24hr)

_________________________________________________________________________________________________________

Field Blank Sample

11-0002	BLANK	7/8/97	9:55	<20	<20	--  	--

11-0009	BLANK	7/8/97	10:40	<20	<20	--  	--

11-0017	BLANK	8/12/97	**	**  	--  	--  	--

11-0025	BLANK	8/12/97	**	**  	--  	--  	--

11-0031	BLANK	9/16/97	10:50	<20	<20	--  	--

11-0036	BLANK	9/16/97	10:57	<20	<20	--  	--

** = missing/censored data          -- = no data

________________________________________________________________________________________

Sample	Sample	Time	FECAL	E-COLI	ENTEROCOCCUS	AEROMONAS

Id	Id	(24hr)

_________________________________________________________________________________________________________

HOOSIC RIVER,  Station: HR08A

11-0014	11-0015	8/12/97	9:49	**  	--  	--  	--

11-0015	11-0014	8/12/97	9:49	**  	--  	--  	--

Relative Percent Difference (RPD):

_______________________________________________________________________________________________________________________________________________

HOOSIC RIVER,  Station: HR07A

11-0028	11-0029	9/16/97	9:57	**  	**  	--  	--

11-0029	11-0028	9/16/97	9:57	**  	**  	--  	--

Relative Percent Difference (RPD):

_______________________________________________________________________________________________________________________________________________

HOOSIC RIVER,  Station: HR07

11-0004	11-0005	7/8/97	11:50	3.000	2.505	--  	--

11-0005	11-0004	7/8/97	11:50	3.079	2.380	--  	--

Relative Percent Difference (RPD):	2.6%	5.1%

_______________________________________________________________________________________________________________________________________________

HOOSIC RIVER,  Station: HR03D

11-0010	11-0011	7/8/97	11:30	3.041	2.301	--  	--

11-0011	11-0010	7/8/97	11:30	2.778	2.415	--  	--

Relative Percent Difference (RPD):	9.0%	4.8%

_______________________________________________________________________________________________________________________________________________

HOOSIC RIVER,  Station: HR02

11-0037	11-0038	9/16/97	11:25	2.301	<1.301	--  	--

11-0038	11-0037	9/16/97	11:38	2.477	1.301	--  	--

Relative Percent Difference (RPD):	7.4%	0.0%

_______________________________________________________________________________________________________________________________________________

_____________________________________________________________________________________

GREEN RIVER,  Station: GN01

11-0021	11-0022	8/12/97	10:20	**  	--  	--  	--

11-0022	11-0021	8/12/97	10:20	**  	--  	--  	--

Relative Percent Difference (RPD):

_______________________________________________________________________________________________________________________________________________

** = missing/censored data          -- = no data _________________________________________________________________________________________________

Table A.2-4.  1997 DEP DWM Hudson River Basin instream bacteriological QA/QC field replicate data (units in colonies/100 ml, data log10 transformed).

Hudson River Basin 1997 Water Quality Assessment Report Appendix A- A 8 -

11wqar.docDWM CN 15.0

Hudson River Basin 1997 Water Quality Assessment Report Appendix A- A 9 -

11wqar.docDWM CN 15.0

A.3 Analytical Methods

Discrete Water Sample Analytes			EPA Method*	SM Methods**	Other Methods

Fecal Coliform							SM 9222D

E. Coli, MTEC							SM 9213D

Alkalinity (titrimetric)				EPA 310.1	SM 2320B

Chloride (titrimetric)						SM 4500CL-B

Hardness (EDTA)					EPA 130.2	SM 2340B

Turbidity						EPA 180.1	SM 2130B

Ammonia-N (Automated – phenate)			EPA 350.1	SM 4500-NH3-H

Nitrate/Nitrite-N (automated – hydrazine)		EPA 353.1	SM 4500 –NO3 -H

Total Phosphorus					EPA 365.2	SM 4500P-E

Suspended Solids						SM 2540D

Fish Tissue Analytes

PCB Arochlor 1242								AOAC 983.21***

PCB Arochlor 1254									“

PCB Arochlor 1260									“

Chlordane										“

Toxaphene										“

a-BHC											“

b-BHC											“

Lindane											“

d-BHC											“

Hexachlorocyclopentadiene								“

Trifluralin										“

Hexachlorobenzene									“

Heptachlor										“

Heptachlor Epoxide									“

Methoxychlor										“

DDD											“

DDE											“

DDT											“

Aldrin											“

Arsenic	 (STGFAA)				EPA 200.9	SM 3113

Lead (ICP)					EPA 200.7	SM 3120B

Selenium (STGFAA)				EPA 200.9	SM 3113

Cadmium (ICP)					EPA 200.7	SM 3120B

Mercury (cold vapor)				EPA 245.1	SM 3112B

* =  “Methods for Chemical Analysis of Water and Wastes”, Environmental Protection Agency, Environmental Monitoring Systems Laboratory – Cincinnati (EMSL-CI), EPA-600/4-79-020, Revised March 1983 and 1979 where applicable.

** = Standard Methods, Examination of Water and Wastewater, 20th edition

***= PCBs and Organochlorine Pesticides in Biological Tissue, AOAC Official Methods of Analysis, 1990.

# APPENDIX B  - 1997 DEP HUDSON RIVER BASIN SURVEY DATA

MATERIALS AND METHODS

The DWM began sampling in July 1997 and continued through September 1997.  The DWM sampling plan matrix is summarized in Table B1.  Sampling components at river stations included: stream discharge measurements, in-situ Hydrolab measurements, physico-chemical and nutrient sampling, fecal coliform bacteria sampling, benthic macroinvertebrate and fish population sampling, and toxics in fish flesh.  Synoptic surveys of lakes were conducted during August 1997 to coincide with the maximum extent of macrophyte growth.  Each sampling component is described in the sections that follow.

Table B1.  1997 Hudson River Basin Surveys DEP-DWM sampling matrix.

| STREAM NAMES | STATION1 | 1997 JULY | 1997 AUG | 1997 SEPT |
| --- | --- | --- | --- | --- |
| HOOSIC RIVER SUBBASIN |  |  |  |  |
| Hoosic River | HR08A (Q0023) | B, H, N, C, Q | B, H, N, C | B, H, N, C |
|  | HR08B (Q0024) |  | Q | Q |
|  | HR07A (HR07U) | B, H, N, C | B, H, N, C, M | B, H, N, C |
|  | MA0100315 | C | C | C |
|  | HR07D |  | M |  |
|  | HR07 (Q0025) | B, H, N, C, Q | B, H, N, C, Q | B, H, N, C, Q |
|  | F0052 |  | T |  |
|  | HR03 | B, H, N, C | B, H, N, C | B, H, N, C |
|  | HR03D | B, H, N, C | B, H, N, C | B, H, N, C |
|  | HR03 |  | M |  |
|  | MA0100510 | C | C | C |
|  | HR02 |  | M |  |
|  | HR02 (Q0028) | B, H, N, C, Q | B, H, N, C, Q | B, H, N, C, Q |
| Bassett Brook | BB00 |  | F | M |
| Pecks Brook | PB00 |  | F | M |
| North Branch Hoosic River | F0051 |  | T |  |
|  | HR09A (Q0026) | B, H, N, C, Q | B, H, N, C, Q | B, H, N, C, Q |
|  | F0050 |  | T |  |
| Green River | GN04 |  | M, F |  |
|  | GN03 |  | M, |  |
|  | upstream of Blair Road, Williamstown |  | F |  |
|  | GN02 |  | M |  |
|  | GN01 | B, H, N, C | B, H, N, C, M, F | B, H, N, C |
| East Branch Green River | GE01 |  | M |  |
| West Branch Green River | GW01 |  | M |  |
| KINDERHOOK SUB BASIN |  |  |  |  |
| Kinderhook Creek | KC01 |  | M, F |  |
|  | KC02 |  | M, F |  |

1 Sampling did not necessarily occur at the same exact location although that which occurred in the general vicinity of the sampling station is listed together.

B=Bacteria (fecal coliform, E. coli); H=Hydrolab multiprobe meter (pH, dissolved oxygen, conductivity, temperature, total dissolved solids); N=Nutrients (total phosphorus, ammonia, nitrate-nitrogen); C=Chemistry (alkalinity, hardness, chlorides, total suspended solids, turbidity); M=Macroinvertebrate kick sampling, habitat assessment, and algae/periphyton sampling; F = Fish population sampling, Q  = Stream discharge measurements. T =  Toxics in fish tissue (Cd, Pb, Hg, As, Se, % lipids, PCBs, organochlorine pesticides).

##### Survey Conditions

Conditions prior to each survey were characterized by analyzing precipitation and streamflow data.  The closest weather station precipitation gage, Crane & Company Dalton Station #104, was used to determine precipitation and weather conditions in the five days prior to and on the sampling dates.  Data from this station were provided by the DEM Office of Water Resources (MA DEM 1998).  Discharge (hereinafter referred to as streamflow) and duration data were obtained from three continuous USGS stream gages in the basin (Figure B1), Hoosic River at Adams (01331500) and at Williamstown (01332500) and the Green River at Williamstown (01333000).  Streamflow statistics for the period-of-records for these gages are available from USGS.  These data can be found in their Water Resources Data Massachusetts and Rhode Island, Water Year 1997 report (Socolow et al. 1998) and the Gazetteer of Hydrologic Characteristics of Streams in Massachusetts—Hudson River Basin (Wandle 1984).  The period of record (POR) for the Hoosic River gage at Adams is from October 1931 to present, the Hoosic

Figure B1.  Location of 1997 water quality, stream discharge, and USGS gaging stations in the Hoosic River Subbasin.

River at Williamstown is from July 1940 to present, and the Green River at Williamstown is from September 1949 to present.  Stream discharge was also measured by DEP at selected locations to supplement the USGS gaging station data (Figure B1).

##### Stream Water Quality Monitoring

Water quality sampling was conducted at the stations identified in Figure B1.  Synoptic water quality sampling at these locations included the following: in-situ measurements using a Scout 3 Hydrolab multiparameter meter (water temperature, dissolved oxygen, conductivity, total dissolved solids, and pH), bacteria sampling (fecal coliform), physico-chemical sampling (alkalinity, hardness, specific conductivity, chloride, total and suspended solids, and turbidity), and nutrient sampling (ammonia and nitrate nitrogen and total phosphorus).

Procedures used for water sampling and sample handling are described in the Basin Program Standard Operating Procedures (MA DEP 1989).  The Wall Experiment Station (WES), the Department’s analytical laboratory, supplied all sample bottles and field preservatives, which were prepared according to the WES Laboratory Quality Assurance Plan and Standard Operating Procedures (MA DEP 1994). Samples were preserved in the field as necessary, transported on ice to WES, and analyzed according to the WES SOP. The quality control protocol that was followed for field and equipment blank samples is described in Appendix A of this report (Hudson River Basin 1997 Water Quality Assessment Report).  Both quality control samples (field blanks, trip blanks, and split samples) and raw water quality samples were transported on ice to WES on each sampling date; they were analyzed subsequently according to the WES SOP (MA DEP 1994).

Macroinvertebrates

A technical memorandum by Robert Nuzzo of DEP DWM entitled Hudson River Watershed (Kinderhook Creek and Hoosic River Subwatersheds) 1997 Biological Assessments presents the aquatic macroinvertebrate analysis of samples collected from selected sites (Figure B2) in the Hoosic River Subbasin.  This memorandum is provided in Appendix C of this report (Hudson River Basin 1997 Water Quality Assessment Report).

Fish Population

The DWM conducted fish population surveys in the Hoosic River Subbasin during the summer of 1997. In addition, two stations were sampled in the Kinderhook Subbasin on Kinderhook Creek.  The Kinderhook Creek stations were located above and below a permitted surface water withdrawal for Jiminy Peak Ski Area.  Hoosic River Subbasin surveys were conducted on the Green River (5 stations), Peck’s Brook (1 station), and Bassett Brook (1 station).  Surveys were conducted using techniques similar to Rapid Bioassement Protocols V (fish) as described by Plafkin (1989).  Surveys also included a habitat assessment component.

Fish populations were sampled by electroshocking using a Smith Root Model 12 battery powered backpack electrofisher. A reach of approximately 100m was sampled by passing a pole mounted anode ring, side to side through the stream channel and in and around likely fish holding cover.  All fish shocked were netted and held in buckets. Sampling proceeded from an obstruction or constriction, upstream to an endpoint at another obstruction or constriction such as a waterfall or shallow riffle.  Following completion of a sampling run, all fish were identified to species, counted, and released.

#### Fish Toxics

Uniform protocols, designed to assure accuracy and prevent cross-contamination of samples, were followed for collecting, processing and shipping fish collected via electroshocking.  Fish were collected from the North Branch Hoosic River and the Hoosic River on 11 August and 26 August 1997 (Figure B2).  Lengths and weights were measured and fish were visually inspected for tumors, lesions, or other anomalies.  Fish included in the samples were placed in ice filled coolers and were processed in the field.  Scale samples were obtained from each sample to determine the age of the fish.  Fish were filleted (skin off) on glass cutting boards and prepared for freezing.  All equipment used in the filleting process was rinsed in cold water to remove slime, scales, and other fluids such as blood, then re-rinsed twice in deionized water before (and/or after) each sample.  Individual fillets were wrapped in aluminum foil or stored in the single sample container, whereas two to three fillets from like-sized individuals of the same species (composite sample) were wrapped together in aluminum foil or stored in the single sample container.  Fillets targeted for metals analysis were placed in VWR 32-ounce high-density polyethylene (HDPE) cups with covers.  The opposite fillets were wrapped in aluminum foil for % lipids, PCBs and organochlorine pesticide analysis.  Samples were tagged and frozen for subsequent delivery to WES.

Figure B2. Location of 1997 benthic macroinvertebrate and fish toxics monitoring stations in the Hoosic River Subbasin.

Methods used at WES for analyzing metals include the cold vapor method using a VGA hydride generator for mercury and Varian 1475 flame atomic absorption for all remaining metals (MA DEP 1994).  PCB/organochlorine pesticides analyses were performed on a gas chromatograph equipped with an electron capture detector.

Lakes

Synoptic lake surveys were conducted during August 1997.  Synoptic surveys consisted of taking observations from at least one access point on each lake (multiple access points on larger lakes).  At each lake, an attempt was made to observe the entire surface area to determine the extent of areal macrophyte cover.

At each observation site the general water quality was noted and all aquatic and wetland macrophyte species were recorded along with their general abundance and an estimate of the total percent areal coverage of all species.  Qualitative macrophyte observations were aided by conducting several hauls with a plant "rake," which was constructed by bolting two garden rakes back-to-back, the handles cut to about half length, and then attached to about a 50' length of rope.  Each time the rake was thrown to its maximum extension and then retrieved along the lake bottom.  The rake was thrown several times in different directions from the observation site to provide more thorough coverage.

Where possible, transparency was measured using a standard 20-centimeter diameter Secchi disc attached to a rope with metric calibrations.  When Secchi disc measurements were not feasible, transparency was estimated as being above or below 1.2 meters (based on the 4 foot Secchi disc bathing beach standard).

All observations were recorded on standardized field sheets.  Assessments of trophic status and use impairment were made on site.  Later, the assessments and supporting information were entered into the US EPA Water Body System database.  Data on the presence of non-native plants were entered into a separate database intended for linking to the Massachusetts Geographic Information System (MassGIS).

RESULTS

Survey conditions

To fulfill the assessment guidance, information on precipitation (MA DEM 1998) and stream discharge (Socolow et al. 1998) and stream discharge measurements (Table B2) were analyzed to determine hydrologic conditions during the water quality sampling events.  This review was conducted to determine the streamflow condition in relation to the 7-day, 10-year (7Q10) low flow.  Additionally, this review was used to determine whether the fecal coliform bacteria data were representative of “wet” or “dry weather” sampling conditions.  Survey conditions are described below for each DWM sampling event reviewed for the assessment (MA DEP 1997a).

8 July 1997: Thunder showers/high winds preceded the 8 July 1997 sampling event on the Hoosic River apparently between 3 and 4 July 1997 (Figure B3).  Some precipitation was measured on 8 July (MA DEM 1998), however field sheet notes (MA DEP  1997) indicated the day of sampling was sunny and clear.  The daily mean stream discharge of the Hoosic River USGS gage at Adams (01331500) (Socolow et al. 1998) reflected precipitation (increase from 37 to 70 cfs between 7 and 8 July) however, the storm occurred after the sampling.  Interpretation of the 8 July 1997 data for the assessment will be “ dry weather” conditions.  Streamflow of the Hoosic River was between 2 and 3 times higher than 7Q10 conditions (calculated using the mean discharge reported in Socolow, et al. 1998 on 7 July -- prior to the storm event which occurred after the sampling).

12 August 1997: Much lower flow conditions occurred during the 12 August 1997 sampling event. No antecedent rainfall was recorded at the Dalton station until 12 August (0.42 inches of precipitation) (MA DEM 1998).  The daily mean discharge of the Hoosic River on 12 August was 23 cfs at the Adams gage and 56 cfs at the Williamstown gage (Socolow et al. 1998).  The flow was below the monthly mean at both gage locations over the period of record and was approximately 1.5 to 1.8 times higher than the 7Q10 condition.  Data collected during the 12 August 1997 survey will be interpreted as being representative of “dry” conditions.

16 September 1997: Mean daily discharge of the Hoosic River at the Adams gage was 17 cfs and at the Williamstown gage was 55 cfs (Socolow et al., 1998).  The flow was well below the average September flows over the period of record.  The river was also slightly lower than the 12 August survey but still exceeded 7Q10 conditions (1.3-1.4 times higher).  Although 0.39 inches of rain were recorded at the Dalton station (MA DEM 1998), no discernable affects on streamflow were observed.  The 16 September data will be interpreted as “dry weather” conditions.

Table B2. 1997 DEP DWM Hudson River Basin, Hoosic River Subbasin stream discharge measurements.

Time	Collecting	          Sampling	Velocity 	Discharge

(24hr)        Agency	                Technique                              (fps)	                               (cfs)

________________________________________________________________________________

HOOSIC RIVER

Station: HR08A,

Description: outlet Cheshire Reservoir (near Lakeside Restaurant and Route 8), Cheshire.

7/8/97	9:10	    DEP	     Swoffer 2100	0.51	52.5e

_________________________________________________________________________________

HOOSIC RIVER

Station: HR08B

Description: downstream/northeast of outlet of Cheshire Reservoir at first bridge crossing at Church Street, Cheshire.

8/12/97	9:50	    DEP	     Swoffer 2100	1.1  	11.2e

9/16/97	9:30	    DEP	     Swoffer 2100	0.98	10.2

_________________________________________________________________________________

HOOSIC RIVER

Station: HR07

Description: just upstream/south of Hodges Cross Road, North Adams.  (downstream/north of Adams WWTP)

7/8/97	10:30	   DEP	     Swoffer 2100	1.3  	84.5

8/12/97	11:25	   DEP	     Swoffer 2100	0.75	37.4e

9/16/97	10:45	   DEP	     Swoffer 2100	0.68	31.8e

_________________________________________________________________________________

HOOSIC RIVER

Station: HR02

Description: west of Route 7, Williamstown.  (downstream/north of Hoosac Valley WWTP)

7/8/97	12:00	    DEP	     Unknown	1.0  	130

8/12/97	13:15	    DEP	     Swoffer 2100	0.79	79.6

9/16/97	12:50	    DEP	     Swoffer 2100	0.46	66.3

_________________________________________________________________________________

_________________________________________________________________________________

NORTH BRANCH HOOSIC RIVER

Station: HR09A

Description: approximately 150 yards upstream/east of first bridge crossing of Route 8 (Beaver Street), North Adams.

7/8/97	**	    DEP	     Swoffer 2100	0.58	6.86e

8/12/97	15:35	    DEP	     Swoffer 2100	0.51	5.27e

9/16/97	14:45	    DEP	     Swoffer 2100	0.58	8.28e

_________________________________________________________________________________

_________________________________________________________________________________

** = missing/censored data       e = flow estimated see field sheet for details

Stream Water Quality Monitoring

All DEP water quality data is managed and maintained in an Access Database (Dallaire 1999).  The Hydrolab in-situ results are provided in Table B3.  Discrete water sampling data includes physico-chemical (Table B4) and bacterial data (Table B5).

Table B3.  1997 DEP DWM Hudson River Basin, Hoosic River Subbasin Survey, in-situ Hydrolab data.

Time	  Measurement 	Temp	pH 	Cond 	TDS 	DO 	SAT 	Turb

(24h)	  Depth (m)	(°C)	(SU)	(uS/cm)	(g/l)	(mg/l)	(%)	(NTU)

_______________________________________________________________________________________

HOOSIC RIVER

Station: HR08A,  Mile Point: 22.1

Description: downstream/east at Route 8 near outlet Cheshire Reservoir, Cheshire.

11-0001	7/8/97	10:06	<0.3  	22.7  	9.3  	187	0.1	6.8 	77	--

11-0014	8/12/97	09:54	0.4  	22.8  	8.5  	197	0.1	** 	**	--

11-0027	9/16/97	08:50	<0.3  	19.3  	8.3  	203	0.1	6.1 	65	7

_________________________________________________________________________________

HOOSIC RIVER

Station: HR07A,  Mile Point: 14

Description: approximately 50 feet upstream/south of Lime Street bridge, Adams.

11-0003	7/8/97	11:04	<0.3  	20.5  	8.3  	233	0.1	9.8 	107	--

11-0016	8/12/97	10:48	<0.3  	19.9  	8.7  	303	0.2	10.8 	116	--

11-0028	9/16/97	10:00	<0.3  	16.4  	8.5  	313	0.2	11.5 	115	1

_________________________________________________________________________________

HOOSIC RIVER

Station: HR07,  Mile Point: 11.5

Description: upstream/south at Hodges Cross Road bridge, North Adams.

11-0004	7/8/97	11:52	<0.3  	20.0  	8.1  	274	0.2	9.2 	99	--

11-0018	8/12/97	11:31	0.3  	20.1  	8.2  	375	0.2	9.1 	98	--

11-0030	9/16/97	10:44	<0.3  	18.4  	8.1  	397	0.3	9.3 	97	**

_________________________________________________________________________________

HOOSIC RIVER

Station: HR03,  Mile Point: 2

Description: approximately 1000 feet upstream/east of Route 7 bridge, Williamstown.

11-0023	8/12/97	11:08	<0.3  	22.6  	8.2  	360	0.2	8.2 	92	8

11-0035	9/16/97	10:59	0.3  	19.3  	8.3  	347	0.2	9.4 	100	--

_________________________________________________________________________________

HOOSIC RIVER

Station: HR03D,  Mile Point: 1.9

Description: approximately 10 feet downstream/west of Route 7 bridge, Williamstown.

11-0011	7/8/97	11:33	<0.3  	19.9  	8.2  	325	0.2	9.8 	105	**

* = outside calibrated range, ** = censored data,  -- = no data

Table B3.  1997 DEP DWM Hudson River Basin, Hoosic River Subbasin Survey, in-situ Hydrolab data. (Continued)

Time	  Measurement 	Temp	pH 	Cond 	TDS 	DO 	SAT 	Turb

(24h)	  Depth (m)	(°C)	(SU)	(uS/cm)	(g/l)	(mg/l)	(%)	(NTU)

_______________________________________________________________________________________

HOOSIC RIVER

Station: HR02,  Mile Point: 0.3

Description: west off Route 7 onto dirt road, cross railroad tracks to sample upstream of small unnamed tributary,

Williamstown.

11-0012	7/8/97	12:23	<0.3  	20.6  	8.2  	322	0.2	10.0 	109	**

11-0024	8/12/97	12:04	<0.3  	22.9  	8.5  	360	0.2	10.4 	118	7

11-0037	9/16/97	11:44	0.7  	19.6  	8.3  	355	0.2	9.9 	106	--

_________________________________________________________________________________

____________________________________________________________________________________

GREEN RIVER

Station: GN01,  Mile Point: 0.6

Description: approximately 20 feet upstream/southwest of Route 2 bridge, Williamstown.

11-0008	7/8/97	10:48	<0.3  	18.2  	8.5  	209	0.1	10.1 	105	**

11-0021	8/12/97	10:23	<0.3  	19.8  	8.3  	239	0.2	9.4 	100	17

11-0034	9/16/97	10:34	<0.3  	17.4  	8.5  	242	0.2	10.2 	104	--

_________________________________________________________________________________

____________________________________________________________________________________

NORTH BRANCH HOOSIC RIVER

Station: HR09A,  Mile Point: 2

Description: approximately 20 feet upstream/north of Hudson Brook confluence with North Branch Hoosic River, North

Adams.

11-0006	7/8/97	13:12	<0.3  	21.5  	8.8  	151	0.10	8.8 	98	--

11-0019	8/12/97	12:34	<0.3  	22.8  	9.3  	165	0.1	10.1 	114	--

11-0032	9/16/97	11:45	<0.3  	17.5  	9.2  	143	0.09	10.9 	111	**

_________________________________________________________________________________

____________________________________________________________________________________

* = outside calibrated range, ** = censored data,  -- = no data

Table B4. 1997 DEP DWM Hudson River Basin, Hoosic River Subbasin Survey, instream physico/chemical data.  All units in mg/L unless otherwise noted.

Hudson River Basin 1997 Water Quality Assessment ReportAppendix B- B10 -

11wqar.docDWM CN 15.0

Time	Alkalinity	Hardness	Specific 	Chloride	Suspended	Total 	Turbidity	Total 	Ammonia	Nitrate	Total

(24h)	Conductivity 	 Solids	Solids	 (NTU)	Kjeldahl 	Phosphorus

(umhos)	Nitrogen

_________________________________________________________________________________________________________________HOOSIC RIVER

Station: HR08A,  Mile Point: 22.1

Description: downstream/east at Route 8 near outlet Cheshire Reservoir, Cheshire.

11-0001	7/8/97	10:08	76  	73  	--	12  	<2.5	--	**  	--	<0.02	0.05	0.03

11-0014	11-0015	8/12/97	9:49	78  	77  	--	13  	<2.5	--	0.80	--	0.03	0.03	0.03

11-0015	11-0014	8/12/97	9:49	78  	76  	--	12  	<2.5	--	0.80	--	0.04	0.03	0.02

11-0027	9/16/97	8:55	83  	85  	--	14  	7.4	--	1.5  	--	0.02	0.06	0.02

____________________________________________________________________________________________________________________________

HOOSIC RIVER

Station: HR07A,  Mile Point: 14

Description: approximately 50 feet upstream/south of Lime Street bridge, Adams.

11-0003	7/8/97	11:00	88  	<0.66 	--	15  	11  	--	**  	--	<0.02	0.19	0.02

11-0016	8/12/97	10:46	120  	127  	--	19  	2.8	--	0.70	--	<0.02	0.41	0.02

11-0028	11-0029	9/16/97	9:57	123  	134  	--	22  	<2.5	--	0.80	--	<0.02	0.45	0.01

11-0029	11-0028	9/16/97	9:57	127  	140  	--	22  	<2.5	--	0.80	--	<0.02	0.44	0.01

____________________________________________________________________________________________________________________________

Pipe/Discharge to HOOSIC RIVER

Station: MA0100315,  Mile Point: 13.7

Description: Adams WWTP outfall approximately 2000 feet downstream/north of Lime Street bridge, Adams.

11-0007	7/8/97	12:25	180  	201  	--	52  	--  	--	**  	--	--  	--  	--

11-0020	8/12/97	**	--  	--  	--	**  	<2.5	--	--  	--	--  	--  	--

11-0033	9/16/97	11:15	187  	227  	--	61  	--  	--	5.1  	--	--  	--  	--

____________________________________________________________________________________________________________________________

** = missing/censored data          -- = no data

Table B4. 1997 DEP DWM Hudson River Basin, Hoosic River Subbasin Survey, instream physico/chemical data. All units in mg/L unless otherwise noted. (Continued)

Hudson River Basin 1997 Water Quality Assessment ReportAppendix B- B 11-

11wqar.docDWM CN 15.0

Time	Alkalinity	Hardness	Specific 	Chloride	Suspended	Total 	Turbidity	Total 	Ammonia	Nitrate	Total

(24h)	Conductivity 	 Solids	Solids	 (NTU)	Kjeldahl 	Phosphorus

(umhos)	Nitrogen

_________________________________________________________________________________________________________________HOOSIC RIVER

Station: HR07,  Mile Point: 11.5

Description: upstream/south at Hodges Cross Road bridge, North Adams.

11-0004	11-0005	7/8/97	11:50	101  	90  	--	18  	11  	--	**  	--	0.19	0.36	0.04

11-0005	11-0004	7/8/97	11:50	101  	105  	--	18  	12  	--	**  	--	0.24	0.37	0.03

11-0018	8/12/97	**	134  	150  	--	23  	<2.5	--	0.90	--	0.08	0.60	0.02

11-0030	9/16/97	10:41	144  	164  	--	26  	<2.5	--	1.1  	--	<0.02	0.58	0.02

____________________________________________________________________________________________________________________________

HOOSIC RIVER

Station: HR03,  Mile Point: 2

Description: approximately 1000 feet upstream/east of Route 7 bridge, Williamstown.

11-0023	8/12/97	11:06	124  	137  	--	28  	4.0	--	1.3  	--	<0.02	0.31	0.02

11-0035	9/16/97	10:53	126  	144  	--	25  	3.1	--	1.4  	--	<0.02	0.37	0.02

____________________________________________________________________________________________________________________________

HOOSIC RIVER

Station: HR03D,  Mile Point: 1.9

Description: approximately 10 feet downstream/west of Route 7 bridge, Williamstown.

11-0010	11-0011	7/8/97	11:30	117  	126  	--	23  	5.2	--	**  	--	0.12	0.54	0.05

11-0011	11-0010	7/8/97	11:30	117  	125  	--	23  	7.0	--	**  	--	0.13	0.55	0.03

** = missing/censored data          -- = no data

Table B4. 1997 DEP DWM Hudson River Basin, Hoosic River Subbasin Survey, instream physico/chemical data. All units in mg/L unless otherwise noted. (Continued)

Hudson River Basin 1997 Water Quality Assessment ReportAppendix B- B12 -

11wqar.docDWM CN 15.0

Time	Alkalinity	Hardness	Specific 	Chloride	Suspended	Total 	Turbidity	Total 	Ammonia	Nitrate	Total

(24h)	Conductivity 	 Solids	Solids	 (NTU)	Kjeldahl 	Phosphorus

(umhos)	Nitrogen

_________________________________________________________________________________________________________________Pipe/Discharge to HOOSIC RIVER

Station: MA0100510,  Mile Point: 1.5

Description: Hoosac WPCF discharge approximately 2000 feet downstream/west of Route 7 bridge, Williamstown.

11-0013	7/8/97	11:50	111  	120  	--	43  	--  	--	**  	--	--  	--  	--

11-0026	8/12/97	11:30	121  	122  	--	39  	--  	--	0.80	--	--  	--  	--

11-0039	9/16/97	**	157  	130  	--	43  	--  	--	1.4  	--	--  	--  	--

____________________________________________________________________________________________________________________________

HOOSIC RIVER

Station: HR02,  Mile Point: 0.3

Description: west off Route 7 onto dirt road, cross railroad tracks to sample upstream of small unnamed tributary, Williamstown.

11-0012	7/8/97	12:20	113  	120  	--	23  	6.0	--	**  	--	0.06	0.68	0.03

11-0024	8/12/97	12:04	121  	133  	--	28  	3.4	--	0.80	--	0.06	0.35	0.02

11-0037	11-0038	9/16/97	11:25	126  	141  	--	25  	4.4	--	1.1  	--	0.37	0.44	0.07

11-0038	11-0037	9/16/97	11:38	126  	142  	--	25  	4.4	--	1.1  	--	0.37	0.44	0.07

____________________________________________________________________________________________________________________________

______________________________________________________________________________________________________________________

GREEN RIVER

Station: GN01,  Mile Point: 0.6

Description: approximately 20 feet upstream/southwest of Route 2 bridge, Williamstown.

11-0008	7/8/97	10:50	89  	88  	--	8.0	<2.5	--	**  	--	<0.02	0.33	0.01

11-0021	11-0022	8/12/97	10:20	102  	103  	--	9.0	<2.5	--	0.90	--	<0.02	0.35	0.01

11-0022	11-0021	8/12/97	10:20	102  	104  	--	9.0	<2.5	--	0.80	--	<0.02	0.33	0.01

11-0034	9/16/97	10:30	103  	110  	--	10  	<2.5	--	0.80	--	<0.02	0.21	0.01

** = missing/censored data          -- = no data

Table B4. 1997 DEP DWM Hudson River Basin, Hoosic River Subbasin Survey, instream physico/chemical data. All units in mg/L unless otherwise noted. (Continued)

Hudson River Basin 1997 Water Quality Assessment ReportAppendix B- B 13-

11wqar.docDWM CN 15.0

Time	Alkalinity	Hardness	Specific 	Chloride	Suspended	Total 	Turbidity	Total 	Ammonia	Nitrate	Total

(24h)	Conductivity 	 Solids	Solids	 (NTU)	Kjeldahl 	Phosphorus

(umhos)	Nitrogen

_________________________________________________________________________________________________________________

NORTH BRANCH HOOSIC RIVER

Station: HR09A,  Mile Point: 2

Description: approximately 20 feet upstream/north of Hudson Brook confluence with North Branch Hoosic River, North Adams.

11-0006	7/8/97	13:12	46  	105  	--	15  	10  	--	**  	--	0.02	0.15	0.06

11-0019	8/12/97	**	52  	54  	--	15  	13  	--	8.3  	--	<0.02	<0.02	0.02

11-0032	9/16/97	11:42	44  	46  	--	14  	8.2	--	9.3  	--	<0.02	0.05	0.02

____________________________________________________________________________________________________________________________

______________________________________________________________________________________________________________________ ** = missing/censored data          -- = no data

Table B5. 1997 DEP DWM Hudson River Basin, Hoosic River Subbasin Survey, bacteria data.  Units in colonies/100 mls.

TIME	FECAL	E-COLI	ENTEROCOCCUS	AEROMONAS

(24h)

____________________________________________________________________________________

HOOSIC RIVER

Station: HR08A,  Mile Point: 22.1

Description: downstream/east at Route 8 near outlet Cheshire Reservoir, Cheshire.

11-0001	7/8/97	10:08	600	<20	--  	--

11-0014	11-0015	8/12/97	9:49	**  	--  	--  	--

11-0015	11-0014	8/12/97	9:49	**  	--  	--  	--

11-0027	9/16/97	8:55	1,100	300	--  	--

_________________________________________________________________________________

HOOSIC RIVER

Station: HR07A,  Mile Point: 14

Description: approximately 50 feet upstream/south of Lime Street bridge, Adams.

11-0003	7/8/97	11:00	1,400	180	--  	--

11-0016	8/12/97	10:46	**  	--  	--  	--

11-0028	11-0029	9/16/97	9:57	**  	**  	--  	--

11-0029	11-0028	9/16/97	9:57	**  	**  	--  	--

_________________________________________________________________________________

Pipe/Discharge to HOOSIC RIVER

Station: MA0100315,  Mile Point: 13.7

Description: Adams WWTP outfall approximately 2000 feet downstream/north of Lime Street bridge, Adams.

11-0007	7/8/97	12:25	--  	--  	--  	--

11-0020	8/12/97	**	--  	--  	--  	--

11-0033	9/16/97	11:15	--  	--  	--  	--

_________________________________________________________________________________

HOOSIC RIVER

Station: HR07,  Mile Point: 11.5

Description: upstream/south at Hodges Cross Road bridge, North Adams.

11-0004	11-0005	7/8/97	11:50	1,000	320	--  	--

11-0005	11-0004	7/8/97	11:50	1,200	240	--  	--

11-0018	8/12/97	**	**  	--  	--  	--

11-0030	9/16/97	10:41	760	140	--  	--

** = missing/censored data          -- = no data

Table B5. 1997 DEP DWM Hudson River Basin, Hoosic River Subbasin Survey, bacteria data.  Units in colonies/100 mls. (Continued)

TIME	FECAL	E-COLI	ENTEROCOCCUS	AEROMONAS

(24h)

____________________________________________________________________________________

HOOSIC RIVER

Station: HR03,  Mile Point: 2

Description: approximately 1000 feet upstream/east of Route 7 bridge, Williamstown.

11-0023	8/12/97	11:06	**  	--  	--  	--

11-0035	9/16/97	10:53	240	60	--  	--

_________________________________________________________________________________

HOOSIC RIVER

Station: HR03D,  Mile Point: 1.9

Description: approximately 10 feet downstream/west of Route 7 bridge, Williamstown.

11-0010	11-0011	7/8/97	11:30	1,100	200	--  	--

11-0011	11-0010	7/8/97	11:30	600	260	--  	--

_________________________________________________________________________________

Pipe/Discharge to HOOSIC RIVER

Station: MA0100510,  Mile Point: 1.5

Description: Hoosac WPCF discharge approximately 2000 feet downstream/west of Route 7 bridge, Williamstown.

11-0013	7/8/97	11:50	--  	--  	--  	--

11-0026	8/12/97	11:30	**  	--  	--  	--

11-0039	9/16/97	**	--  	--  	--  	--

_________________________________________________________________________________

HOOSIC RIVER

Station: HR02,  Mile Point: 0.3

Description: west off Route 7 onto dirt road, cross railroad tracks to sample upstream of small unnamed tributary,

Williamstown.

11-0012	7/8/97	12:20	800	100	--  	--

11-0024	8/12/97	12:04	**  	--  	--  	--

11-0037	11-0038	9/16/97	11:25	200	<20	--  	--

11-0038	11-0037	9/16/97	11:38	300	20	--  	--

** = missing/censored data          -- = no data

Table B5. 1997 DEP DWM Hudson River Basin, Hoosic River Subbasin Survey, bacteria data.  Units in colonies/100 mls. (Continued)

TIME	FECAL	E-COLI	ENTEROCOCCUS	AEROMONAS

(24h)

____________________________________________________________________________________

GREEN RIVER

Station: GN01,  Mile Point: 0.6

Description: approximately 20 feet upstream/southwest of Route 2 bridge, Williamstown.

11-0008	7/8/97	10:50	940	200	--  	--

11-0021	11-0022	8/12/97	10:20	**  	--  	--  	--

11-0022	11-0021	8/12/97	10:20	**  	--  	--  	--

11-0034	9/16/97	10:30	140	60	--  	--

_________________________________________________________________________________

____________________________________________________________________________________

NORTH BRANCH HOOSIC RIVER

Station: HR09A,  Mile Point: 2

Description: approximately 20 feet upstream/north of Hudson Brook confluence with North Branch Hoosic River, North

Adams.

11-0006	7/8/97	13:12	100	<20	--  	--

11-0019	8/12/97	**	**  	--  	--  	--

11-0032	9/16/97	11:42	60	<20	--  	--

** = missing/censored data          -- = no data

Macroinvertebrates

Results from DEP’s 1997 benthic macroinvertebrate study are presented in Appendix C of this report (Hudson River Basin 1997 Water Quality Assessment Report).

Fish Population

Results from the 1997 fish population survey (MA DEP 1997b) are presented in Table B6.

Fish Toxics

Brown trout, brook trout, rainbow trout Oncorhynchus mykiss,  longnose sucker Catostomus catostomus, and white sucker Catostomus commersoni, composites and individuals were analyzed for PCBs, organochlorine pesticides, and selected metals.  These data (MA DEP 1997c) are provided in Table B7.

PCBs in brown trout and brook trout samples (n=4) from below SEC ranged from 1.1 to 1.4 mg/Kg. PCBs were below detection in brown trout, brook trout, rainbow trout and sucker samples (n=8) from the North Branch of the Hoosic River upstream of SEC. While it is likely that the brook trout and rainbow trout collected from the upstream locations were probably fish stocked by the Massachusetts Division of Fisheries and Wildlife and Environmental Law Enforcement (MDFWELE) during the spring of 1997, there is no record of the stocking of brown trout during 1997.  The brown trout were either part of a self- sustaining population or stocked at an earlier date.  The absence of PCBs in fish tissue samples from the North Branch Hoosic River seems to indicate that PCB contamination is not posing a threat to aquatic biota at the two locations sampled.

Table B6.  1997 DEP DWM Hudson River Basin Survey.  Fish population data in the Hoosic River and Kinderhook Subbasins.

| Stations |  |  | Species1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | EBT | BT |  | LND | BND | SS |  | LNS | WS |
| Hoosic River Subbasin |  |  |  |  |  |  |  |  |  |  |  |
| East Branch Green River approximately 0.1 mile upstream of confluence with Green River, New Ashford. |  |  | 7(11)2 | (16) |  | 1 | 4 | 55 |  |  |  |
| Green River downstream of Mill on the Floss Restaurant, New Ashford. |  |  | 3(14) | 9(4) |  |  |  | 29 |  | 1 |  |
| Green River off Greylock Road upstream of Route 7 crossing, New Ashford. |  |  | 7(45) | 7(26) |  |  | 12 | 150 estimated |  |  |  |
| Green River upstream of Blair Road, Williamstown. |  |  |  | 15(3) |  | 89 | 75(TNTC3) | 86 |  |  | 3(3) |
| Green River adjacent to/northwest of East Lawn Cemetery, Williamstown. |  |  |  | 13(21) |  | 175 | 91 | 2* |  | 10 | 12 |
| Bassett Brook upstream/northwest of Fred Mason Road, Cheshire. |  |  | 10(15) |  |  |  |  |  |  |  |  |
| Pecks Brook off West Mountain Road, Adams. (upstream of powerline crossing to base of gorge) |  |  | 9(16) | 1(17) |  |  | 1 |  |  |  |  |
| Kinderhook Subbasin |  |  |  |  |  |  |  |  |  |  |  |
| Kinderhook Creek downstream/south of Brodie Mountain Road, Hancock. (downstream/south of Bentley Brook confluence and ‘Jiminy Peak water withdrawal’) |  |  | 4(2) | 2(62) |  |  |  | 103(TNTC) |  |  |  |
| Kinderhook Creek downstream/south of Brodie Mountain Road, Hancock. (between Whitman Brook and Bentley Brook upstream/north of ‘Jiminy Peak water withdrawal’) |  |  | 1(1) | 4(39) |  |  |  | 116(TNTC, TSTN4) |  |  |  |
| 1Species Code | Common Name | Scientific Name |  |  | 2 (number of young-of-the-year counted) |  |  |  |  |  |  |
| BND | blacknose dace | Rhinichthys atratulus |  |  | 3 (TNTC) too numerous to count |  |  |  |  |  |  |
| BT | brown trout | Salmo trutta |  |  | 4 (TSTN) too small to net |  |  |  |  |  |  |
| EBT | brook trout | Salvelinus fontinalis |  |  | *pickup estimated at 30% observed |  |  |  |  |  |  |
| LND | longnose dace | Rhinichthys cataractae |  |  |  |  |  |  |  |  |  |
| LNS | longnose sucker | Catostomus catostomus |  |  |  |  |  |  |  |  |  |
| SS | slimy sculpin | Cottus cognatus |  |  |  |  |  |  |  |  |  |
| WS | white sucker | Catostomus commersoni |  |  |  |  |  |  |  |  |  |

| Hudson River Basin 1997 Water Quality Assessment ReportAppendix B- B18- 11wqar.docDWM CN 15.0 Table B7. 1997 DEP DWM Hudson River Basin, Hoosic River Subbasin Survey. Fish toxics monitoring data for the Hoosic River, North Adams and North Branch Hoosic River, Clarksburg and North Adams. Data expressed in mg/kg unless otherwise noted. All concentrations are in wet weight. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Analysis # | Sample ID | Collection Date | Species Code1 | Sample Type2 | Length (cm) | Weight (gm) |  | Cd | Pb | Hg | As | Se | % Lipids | PCB3 (g/g) | Pesticides3 (g/g) |
| Hoosic River |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Station F0052: upstream of Route 2 and railroad bridge, North Adams. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 97005 | HRF97-20 | 08/11/97 | BT | I | 29.3 | *** | <0.020 |  | <0.140 | <0.020 | <0.040 | 0.387 | 2.1 | 1.1* | ND |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.27** |  |
| 97006 | HRF97-21 | 08/11/97 | BT | C | 26.1 | *** | <0.020 |  | <0.140 | <0.020 | <0.040 | 0.308 | 3.2 | 1.4* | ND |
|  | HRF97-22 | 08/11/97 | BT | C | 23.5 | *** |  |  |  |  |  |  |  | 0.24** |  |
|  | HRF97-23 | 08/11/97 | BT | C | 24.1 | *** |  |  |  |  |  |  |  |  |  |
| 97007 | HRF97-24 | 08/11/97 | BT | C | 22.3 | *** | <0.020 |  | <0.140 | <0.020 | <0.040 | 0.316 | 1.8 | 1.4* | ND |
|  | HRF97-25 | 08/11/97 | BT | C | 23.4 | *** |  |  |  |  |  |  |  | 0.26** |  |
|  | HRF97-26 | 08/11/97 | BT | C | 20.3 | *** |  |  |  |  |  |  |  |  |  |
| 97008 | HRF97-27 | 08/11/97 | EBT | C | 21.6 | *** | <0.020 |  | <0.140 | <0.020 | <0.040 | 0.419 | 2.5 | 4.1* | ND |
|  | HRF97-28 | 08/11/97 | EBT | C | 17.6 | *** |  |  |  |  |  |  |  | 1.4** |  |
| North Branch Hoosic River |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Station F0051: upstream of Henderson Road, Clarksburg. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 97001 | HRF97-01 | 08/11/97 | WS | C | 21.0 | *** | <0.020 |  | <0.140 | 0.200 | <0.040 | 0.241 | 0.36 | ND | ND |
|  | HRF97-02 | 08/11/97 | WS | C | 21.4 | *** |  |  |  |  |  |  |  |  |  |
|  | HRF97-03 | 08/11/97 | WS | C | 23.7 | *** |  |  |  |  |  |  |  |  |  |
| 97002 | HRF97-04 | 08/11/97 | RT | C | 37.0 | *** | <0.020 |  | <0.140 | <0.020 | 0.259 | 0.250 | 0.73 | ND | ND |
|  | HRF97-05 | 08/11/97 | RT | C | 32.3 | *** |  |  |  |  |  |  |  |  |  |
| 97003 | HRF97-06 | 08/11/97 | EBT | C | 28.5 | *** | <0.020 |  | <0.140 | <0.020 | 0.138 | 0.236 | 0.30 | ND | ND |
|  | HRF97-07 | 08/11/97 | EBT | C | 23.8 | *** |  |  |  |  |  |  |  |  |  |
| 97004 | HRF97-08 | 08/11/97 | BT | I | 26.6 | *** | <0.020 |  | <0.140 | 0.580 | <0.040 | 0.320 | 0.44 | ND | ND |
| Station F0050: upstream of Route 8, North Adams. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 97009 | HRF97-30 | 08/26/97 | WS | C | 22.0 | *** | <0.020 |  | <0.140 | 0.320 | <0.040 | 0.184 | 0.30 | ND | ND |
|  | HRF97-31 | 08/26/97 | LNS | C | 21.0 | *** |  |  |  |  |  |  |  |  |  |
|  | HRF97-32 | 08/26/97 | LNS | C | 22.6 | *** |  |  |  |  |  |  |  |  |  |
| 97010 | HRF97-33 | 08/26/97 | BT | I | 32.0 | *** | <0.020 |  | <0.140 | 0.420 | <0.040 | 0.302 | 0.90 | ND | ND |
| 97011 | HRF97-34 | 08/26/97 | BT | I | 30.5 | *** | <0.020 |  | <0.140 | 0.430 | <0.040 | 0.294 | 0.44 | ND | ND |
| 97012 | HRF97-35 | 08/26/97 | RT | I | 28.1 | *** | <0.020 |  | <0.140 | <0.020 | <0.040 | 0.215 | 0.34 | ND | ND |
| 1Species | brown trout (BT) Salmo trutta |  |  |  | 2Sample Type (All samples were fillets with skin off.) |  |  |  |  |  |  | 3Analyzed just beyond the EPA recommended holding time although extraction was within holding time. |  |  |  |
|  | eastern brook trout (EBT) Salvelinus fontinalis |  |  |  |  | Composite (C) |  |  |  |  |  |  |  |  |  |
|  | longnose sucker (LNS) Catostomus catostomus |  |  |  |  | Individual (I) |  |  |  |  |  | * Arochlor 1242 |  |  |  |
|  | rainbow trout (RT) Oncorhynchus mykiss |  |  |  |  |  |  |  |  |  |  | ** Arochlor 1254 |  |  |  |
|  | white sucker (WS) Catostomus commersoni |  |  |  | ND - not detected |  |  |  |  |  |  | *** not weighed |  |  |  |

- Lakes

Lake synoptic survey results (MA DEP 1997d) are presented in Table B8.

- Table B8. 1997 DEP DWM Hudson River Basin, Hoosic River Subbasin lake survey data.

| LAKE | SEGMENT NUMBER | SIZE (Acres) | TROPHIC STATE | OBSERVATIONS, Objectionable Conditions |
| --- | --- | --- | --- | --- |
| Berkshire Pond, Lanesborough | MA11001 | 22 | E | Non-native (Ms) Noxious plants |
| Cheshire Reservoir (North Basin), Cheshire/ Lanesborough | MA11002 | 218 | H | Non-native plants (Ms) Noxious plants Turbidity |
| Cheshire Reservoir (Middle Basin), Cheshire/ Lanesborough | MA11018 | 132 | H | Non-native plants (Ms) Noxious plants Turbidity |
| Cheshire Reservoir (South Basin), Cheshire/ Lanesborough | MA11019 | 67 | E | Non-native plants (Ms, Pc) Noxious plants |
| Mt. Williams Reservoir, North Adams ** | MA11010 | 43 | U | None noted |
| Notch Reservoir, North Adams ** | MA11011 | 25 | U | None noted |
| Windsor Lake, North Adams | MA11016 | 17 | U | None noted |

**  Indicates Class A (water supply) waterbody;  all others are Class B.

Trophic State-- E= Eutrophic, H= Hypereutrophic, U= Undetermined.

Non-native Plants-- Ms= Myriophyllum spicatum. , Pc= Potamogeton crispus.

### References

Dallaire, T.D. 1999.   “Water Quality Data ”. Water Quality Data Database -- WQD.  Access Application.  1999.  Massachusetts Department of Environmental Protection, Division of Watershed Management, Watershed Planning and Permitting Program, Database Manager.  Worcester, MA.

MA DEM. 1998. Open File.  Monthly Precipitation Report. Commonwealth of Massachusetts, Executive Office of Environmental Affairs, Department of Environmental Management. Boston, MA.

MA DEP.  1989.  Basin planning section standard operating procedures.  Massachusetts Department of Environmental Protection, Division of Water Pollution Control, Technical Services Branch.  Westborough, MA.

MA DEP.  1994.  Laboratory Quality Assurance Plan and Standard Operating Procedures.  Massachusetts Department of Environmental Protection, Division of Environmental Analysis.  Wall Experiment Station, Lawrence, MA.

MA DEP. 1997a. Open File.  1997 Stream Discharge Measurements in the Hoosic River Subbasin. Massachusetts Department of Environmental Protection, Division of Watershed Management, Watershed Planning and Permitting Program, Worcester, MA.

MA DEP. 1997b. Open File.  1997 Fish Population Survey Data in the Hudson River Basin.  Massachusetts Department of Environmental Protection, Division of Watershed Management, Watershed Planning and Permitting Program, Worcester, MA.

MA DEP. 1997c. Open File.  1997 Fish Toxics Monitoring Data in the Hoosic River Subbasin.  Massachusetts Department of Environmental Protection, Division of Watershed Management, Watershed Planning and Permitting Program, Worcester, MA.

MA DEP. 1997d. Open File.  1997 Synoptic Lake Surveys Coding Sheets.  Massachusetts Department of Environmental Protection, Division of Watershed Management, Watershed Planning and Permitting Program, Worcester, MA.

Plafkin, J.L., M.T. Barbour, K.D. Porter, S.K. Gross, R.M. Hughes.  1989.  Rapid bioassessment protocols for use in streams and small rivers: benthic macroinvertebrates and fish.  U.S. Environmental Protection Agency.  Washington, D.C.  EPA/444/4-89-001.

Socolow, R.S., C.R. Leighton, J.L. Zanca, and L.R. Ramsbey.  1998.  Water Resources Data for Massachusetts and Rhode Island, Water Year 1997.  U.S. Geological Survey Report MA-RI-97-1.  Water Resources Division.  Marlborough, MA.

Wandle Jr., S.W.  1984.  Gazetteer of Hydrological Characteristics of Streams in Massachusetts— Hudson River Basin.  U.S. Geological Survey Water Resources Investigation Report 83-4250. Boston, MA.

# APPENDIX C - DEP BIOMONITORING TECHNICAL MEMORANDUM

To:	Hudson River Watershed Team; Tom O’Brien, EOEA; Bill Prendergast, DEP, Western Regional Office

From:	Robert M. Nuzzo, DEP/DWM, Worcester

Date:	30 September 1999

HUDSON RIVER WATERSHED (KINDERHOOK CREEK AND HOOSIC RIVER SUBWATERSHEDS) 1997 BIOLOGICAL ASSESSMENTS

As part of the Division of Watershed Management’s 1997 Hudson River watershed assessments, benthic macroinvertebrates were collected to gauge the biological health of the Hoosic River and Kinderhook Creek drainages.  Based on priorities expressed by the Hudson watershed team, the monitoring effort in the Hoosic drainage was concentrated along the Green River but also included paired sites on the Hoosic River mainstem, as well as one site each on Peck’s and Bassett Brooks.  Two stations were sampled on Kinderhook Creek. The Green River is a major tributary to the Hoosic River, the two becoming confluent in Williamstown near Route 2 at Williams College.  The Green River monitoring data were intended to provide an update on the status of water quality and to see if the analysis detected any evidence of impacts from nonpoint source pollution or the minor discharges listing this river as the receiving water.  The Hoosic River monitoring sites were selected as upstream/downstream pairs, bracketing the Hoosac Valley Water Quality District wastewater treatment plant (WWTP) and the Adams Wastewater Treatment Plant.  Bassett Brook (tributary to a public water supply) and Pecks Brook drain an area on the eastern flanks of Saddle Ball Mountain and Mount Greylock that is slated for development.  These sites were intended to provide baseline data.  The two monitoring sites on Kinderhook Creek bracketed a water withdrawal structure used by Jiminy Peak ski area.  Sampling location descriptions and dates are given in Table 1 and station locations are shown on the watershed map in Figure 1.

Table 1.	Sampling locations and sampling dates for Hudson River watershed benthic biomonitoring.

| Station | Stream | Location | Date Sampled |
| --- | --- | --- | --- |
| GN04 | Green River | downstream from Mill-on-the-Floss Restaurant, New Ashford, MA | 11 August 1997 |
| GE01 | E. Br. Green River | upstream from Green River confluence, New Ashford, MA | 11 August 1997 |
| GW01 | W. Br. Green River | upstream from Old Mill Road, Williamstown, MA | 12 August 1997 |
| GN03 | Green River | upstream from lower Rte. 43, Williamstown, MA | 12 August 1997 |
| GN02 | Green River | downstream from Blair Rd., Williamstown, MA | 12 August 1997 |
| GN01 | Green River | upstream from Rte. 2, Williamstown, MA | 12 August 1997 |
| BB00 | Bassett Brook | upstream from Mason Rd. and gas pipeline, Cheshire, MA | 3 September 1997 |
| PB00 | Pecks Brook | upstream from West Rd. high-tension power lines, Adams, MA | 3 September 1997 |
| HR07U | Hoosic River | upstream from WWTP, Adams, MA | 13 August 1997 |
| HR07D | Hoosic River | downstream from WWTP, Adams, MA | 13 August 1997 |
| HR03 | Hoosic River | upstream from Hoosac WQD WWTP Williamstown, MA | 12 August 1997 |
| HR02 | Hoosic River | downstream from Hoosac WQD WWTP, Williamstown, MA | 12 August 1997 |
| KC01 | Kinderhook Creek | downstream from Brodie Mountain Rd. & upstream from Bentley Brook, Hancock, MA | 11 August 1997 |
| KC02 | Kinderhook Creek | downstream from Bentley Brook, Hancock, MA | 11 August 1997 |

Figure 1.	Location of benthic macroinvertebrate sampling stations within the Hoosic and Kinderhook drainages of the Hudson River watershed.

### METHODS

Sampling and processing procedures are described in detail in the benthos monitoring SOP (Nuzzo 1999) but a brief description is given here.  A 100 m reach of stream at each location was sampled by kicking bottom substrates in riffle habitats to dislodge resident invertebrates and capture them in a 500 μm mesh kick-net.  Ten kicks in squares approximately 0.46 m x 0.46 m were composted for a total sample area of about 2 m2.  Samples were preserved in the field with denatured 95% ethanol, then brought to the DWM lab for processing. Before leaving the sample reach, habitat qualities were scored using a modification of the evaluation procedure in Plafkin, et al. (1989).  A copy of the habitat evaluation form appears in Appendix A (Table A1).

Processing entailed distributing a sample in pans, randomly selecting grids within the pans, and sorting specimens from the other materials in the sample until approximately 100 organisms (±10%) were extracted.  Specimens were identified to genus or species as allowed by available keys, specimen condition, and specimen maturity.  Taxonomic data were analyzed using a modification of Rapid Bioassessment Protocol III (RBP III) metrics and scores (Plafkin, et al. 1989).  The modifications were: substitution of percent similarity for the Community Loss Index, elimination of the shredder/total ratio (no separate leaf-pack material was collected), and addition of the NYSPMA (New York State Percent Model Affinity) score as a metric.  The NYSPMA score is based on a sample assemblage’s affinity to a model of a “healthy” benthic community (Novak and Bode 1992).  Since this metric was developed for New York State’s streams, and this watershed is common to New York and Massachusetts, it seemed both useful and appropriate to include it in the suite of metrics for the assessment.

### RESULTS

### HOOSIC/HUDSON WATERSHED

The Hoosic River begins at the outlet of Cheshire Reservoir in Cheshire, MA.  It flows north-northeasterly through a valley carved out between the Hoosac Range to the east and the Mount Greylock Reservation to the west, passing through Adams and meeting the North Branch in downtown North Adams.  From there it flows in a mostly westerly direction to the confluence with the Green River in Williamstown, then mostly northerly into Vermont.  Bassett Brook, which flows off the southern end of Saddle Ball Mountain’s eastern flank, enters the Hoosic at Cheshire Harbor, just south of Adams.  Pecks Brook drains a portion of Mount Greylock’s eastern flank and is intercepted by the Hoosic in the heart of downtown Adams.

The Green River is the Hoosic’s largest tributary.  Its west branch starts in the Taconic mountain range, at the eastern edge of Stephentown, NY, draining into Hancock, MA through Gardner Hollow, then flowing north-northeasterly through Hancock into Williamstown.  The East Branch Green River flows north-northwesterly from the northern end of New Ashford, east of U.S. Rte. 7 and west of the Mount Greylock Reservation. The mainstem Green River begins in a wetland area east of U.S. Rte. 7 and south of Ingraham Road in New Ashford.  It flows in a generally northerly direction, becoming confluent with the East Branch just before crossing into Williamstown.  The mainstem meets the West Branch in south-central Williamstown, then flows in a generally northeasterly direction, until it crosses under Rte. 2 and flows into the Hoosic River in the vicinity of Williams College.

The list of taxa and counts at each of the sampling locations in the Hoosic drainage can be found in Appendix B, Tables B1 and B2.

### GE01—East Branch Green River, upstream from Green River confluence, New Ashford, MA

#### Habitat

The East Branch Green River begins in New Ashford, MA at the confluence of several small tributaries coming from the western slopes of Jones Nose (at the southwestern end of Saddle Ball Mountain) and a notch between Sugarloaf and Rounds Rock.  Most of the watershed is forested but the lower portion has three or four houses and mowed fields adjacent to the wooded riparian corridor.  Greylock Road (presumed to be lightly traveled) crosses through the upper portion of this basin.  Because the East Branch has relatively little human activity taking place within its drainage GE01 was chosen to serve as the reference for all the Hudson Watershed biomonitoring sites.

Located approximately 200 m upstream from the confluence with the Green River, GE01 provided good substrates for benthic macroinvertebrates but was limited by low water.  At higher volumes the sample reach would have provided excellent fast-flowing, coarse substrate habitat for the benthos, and good cover for fish.  At the time of sampling, however, much of the substrate materials, and what could serve as fish cover, were exposed.  This was likely the typical seasonal low flow condition for a stream of this type and drainage area (ca. 10.1 km2; 3.9 mi2).  The riparian buffer zone was very good on the western edge of the stream, but there was only about six meters of wooded buffer between the eastern edge and a mowed field.  The overall habitat score was 143/200 (Appendix A, Table A2).

#### Benthos

There were 32 distinct taxa counted in this sample (Table 2), the most abundant taxon accounting for 11% of the total.  The high richness and even distribution of this assemblage coupled with the relatively low biotic index indicate a very healthy aquatic community.  This is further supported by the NYSPMA score (74) indicating close agreement with the model.  These data tend to support the conclusion that the waters of the East Branch are of high quality, and thus serve as a good reference.

Also captured in the net but not retained with the sample were slimy sculpins and crayfish.  No algal mats or filaments were seen.

### GN04—Green River downstream from Mill-On-The-Floss Restaurant, New Ashford, MA

#### Habitat

Just east of U.S. Route 7 in New Ashford, MA, a wetland on the south side of Ingraham Road drains under the road to a small stream flowing northward.  From the USGS topographic maps it would appear that this is the beginning of the mainstem Green River.  GN04 was about 800 m downstream from Ingraham Road.  The upper end of the sample reach was at a pool at the base of a waterfall; the bottom of the reach was at the upper end of what had been an impoundment.  In between were scattered, shallow riffles with substrates ranging from bedrock slabs to gravel.  This reach ran through woods dominated by hemlocks, providing a riparian buffer zone of at least 18 m on both banks.  This site had an upstream drainage area of 11.0 km2 (4.3 mi2) and like GE01 also had habitat limitations attributable to seasonal low flow.  The overall habitat score was 160/200 (Appendix A, Table A2).

#### Benthos

Richness was high in this sample (30) but there was slight hyperdominance by the elmid beetle, Optioservus sp.  The abundance of the Optioservus sp. larvae helped generate a very high scraper/filtering collector ratio.  All of the other metrics performed poorly enough against the reference to result in point deductions, and ultimately in a score (24/48) that fell into the Slightly Impaired category (Table 2).

Hudson River Basin 1997 Water Quality Assessment ReportAppendix C- C 10- 11wqar.docDWM CN 15.0

Table 2.	RBP data summary for Green River biomonitoring sites sampled 11-12 August 1997.

| STATION # | GN01 | GN02 | GN03 | GN04 | GW01 | GE01 |
| --- | --- | --- | --- | --- | --- | --- |
| STREAM | Green River upst. fr. Route 2, Williamstown | Green River dnst. fr. Blair Rd., Williamstown | Green River upst. fr. “lower” Route 43 Williamstown | Green River dnst. fr. Mill-on-the-Floss Rest., New Ashford | W. Br. Gr. R. upst. fr. Old Mill Rd., Williamstown | E. Br. Gr. R. upst. fr. confluence, New Ashford |
| HABITAT SCORE | 160; >100% | 169; >100% | 163; >100% | 160; >100% | 164; >100% | 143; R |
| TAXA RICHNESS | 30 score: 94% 6 | 32 score: 100% 6 | 35 score: >100% 6 | 33 score: >100% 6 | 27 score: 84% 6 | 32 score: R 6 |
| BIOTIC INDEX | 4.67 76% 4 | 4.92 72% 4 | 4.84 73% 4 | 3.86 92% 6 | 4.64 77% 4 | 3.55 R 6 |
| EPT INDEX | 9 53% 0 | 10 59% 0 | 11 65% 0 | 15 88% 4 | 9 53% 0 | 17 R 6 |
| EPT/CHIRONOMIDAE | 0.69 6% 0 | 0.47 4% 0 | 0.82 7% 0 | 1.6 15% 0 | 0.94 9% 0 | 11 R 6 |
| RIFFLE COMMUNITY: SCRAPERS/FILT. COLL. | 2.0 >100% 6 | 0.50 >100% 6 | 0.53 >100% 6 | 0.41 >1.5 6 | 1.8 >100% 6 | 0.27 R 6 |
| [FC/Total] | (0.21) | (0.35) | (0.33) | (0.19) | (0.18) | (0.16) |
| % CONTRIBUTION (DOM. TAXON) | 25% 4 | 11% 6 | 10% 6 | 10% 6 | 24% 4 | 11% 6 |
| %SIMILARITY | 12% 0 | 13% 0 | 14% 0 | 32% 2 | 16% 0 | 100% 6 |
| NYSPMA | 63 4 | 63 4 | 68 6 | 67 6 | 66 6 | 74 6 |
| Reference | GE01 | GE01 | GE01 | GE01 | GE01 | R |
| Score | 24/48 24 | 26/48 26 | 28/48 28 | 36/48 36 | 26/48 26 | R 48 |
| Category | 50% MI | 54% SL | 58% SL | 75% SL | 54% SL | R |

R = reference

SL = slightly impaired

MI = moderately impaired

Also caught in the net were slimy sculpin and a young-of-year brook trout.  Filamentous algae scraped from bedrock in some of the riffles were probably Vauceria sp.; there was also an abundant presence of the diatom Melosira sp.

### GW01—West Branch Green River, upstream from Old Mill Road, Williamstown, MA

#### Habitat

From GN04 it is approximately 2.8 km to the confluence with the East Branch Green River, and another 4.5 km to the confluence with the West Branch.  The West Branch sampling station (GW01) was located approximately 1.9 km upstream from the confluence and about 100 m upstream from Old Mill Road in Williamstown.  By this point the West Branch has accumulated a drainage area of 32.4 km2 (12.5 mi2), about three times that of the GE01 or GN04.  The substrates at this site were dominated by cobble but some sand and fine organic deposits were noted.  In spite of the low flow status (channel only about 50% covered) the sample reach included a good variety of substrates, flow velocities, depth, and even useful fish habitat/cover.  The eastern bank in the reach was steep and moderately unstable, with areas of erosion along about 30% of its length.  The overall habitat score was 164/200 (Appendix A, Table A2).

#### Benthos

There was good richness of macroinvertebrate taxa at this site but there was slight hyperdominance by the elmid larva Optioservus sp., a relatively high HBI, and a suspiciously low EPT index.  As a result, the total score (26/48) was only 54% of GE01’s, ranking GW01 as Slightly Impaired (Table 2).

Because of the mostly closed canopy there was very little algal coverage, but a clump of filaments were collected from cobble in a riffle.  The algae in the sample were mostly diatoms.  Fragilaria sp. and naviculoid diatoms were most abundant, with Melosira sp., and the green, Microthamnion sp. common.  The diatom Synedra sp. was also present, but sparse.

### GN03—Green River  upstream from lower Route 43 crossing, Williamstown, MA

#### Habitat

The West Branch and mainstem Green River come together east of the junction of Route 43 (Green River Road) and US Route 7, in a barn yard of a dairy farm where cows could be observed standing in the river.  Most of the west bank downstream from the barnyard to the first Route 43 crossing has little or no riparian buffer: the adjacent land was mowed up to the river bank.  As the river crosses Route 43 about 900 m of the river is segmented to the west of the road before crossing back to the east side of the road.  In the upper part of this segment cows were seen grazing on the grassy banks, and appear to have complete access to the river.  GN03 was in the lower part of the segment, about 1.8 km downstream from the West Branch and Green River confluence and about 100 m upstream from the lower (more downstream, northern) Route 43 crossing.

Within the sample reach itself the epifaunal substrates were very good and the variety of water depths and velocities were also very good.  Though water did not fill the channel there was a lot of very good habitat for both fish and aquatic macroinvertebrates.  The overall habitat score was 163/200 (Appendix A, Table A2).

#### Benthos

Richness of taxa at GN03 (35) was the highest of any of the sites sampled in the Hudson watershed.  The HBI was inflated compared to the reference, however, and the EPT index and the EPT/Chironomidae ratios were lower.  These resulted in an overall score (28/48) low enough to place this site in the Slightly Impaired category (Table 2).

### GN02—Green River downstream from Blair Road, Williamstown, MA

#### Habitat

Approximately 3.6 km downstream from GN03, and 650 m downstream from Blair Road, was sampling reach GN02.  The substrates were dominated by boulders and cobbles but ranged from bedrock to sand and silt/clay, including some deposits of very fine organic matter.  Overall the substrates, water velocity, and depth characteristics were judged to be excellent for macroinvertebrate colonization.  These attributes along with the abundance of instream cover also made this reach excellent habitat for fishes.  The only apparent habitat deficiencies in this reach were instability and narrow riparian vegetative zone width on the east bank.  The habitat score for GN02 was 169/200 (Appendix A, Table A2).

#### Benthos

Taxa richness was very good at this site but the HBI was the highest of any of the Hudson watershed sites, giving the strongest indication of nutrient/organic loading problems.  The lowered EPT index, EPT/Chironomidae abundance ratio, and NYSPMA score all indicate a decline in water quality.  The final score for this site (26/48) was in the Slightly Impaired category (Table 2).

### GN01—Green River upstream from Route 2, Williamstown, MA

#### Habitat

The most downstream Green River sampling station was approximately 3.1 km downstream from GN02 and 1.3 km upstream from the confluence with the Hoosic River.  As the Green River enters the downtown Williamstown area it flows through a series of sharp curves, wrapping around Green River Linear Park-Mount Pleasant and alongside Eastlawn Cemetery.  The sample reach encompassed the last big bend, which spans the boundary of the park and cemetery.  In spite of the fact that less than 75% of the channel was filled with water there was excellent availability of usable epifaunal substrate, as well as fish cover, and there were excellent combinations of water depths and velocities.  The most serious habitat degradation was sediment deposition and this most certainly is due to inadequate stormwater management.  A culvert that enters the river opposite the Williamstown Municipal Garage, and has a steady flow of water, is probably conducting a small unnamed brook underneath the Garage and Rte. 43.  The sediment deposition becomes conspicuous just downstream from the culvert and is evident throughout the sample reach.  The overall habitat score was 160/200 (Appendix A, Table A2).

#### Benthos

Taxonomic richness remained high at this station but several of the metrics (HBI, EPT index, EPT/Chironomidae abundance ratio, NYSPMA) showed evidence of continued degradation in water quality in the Green River.  As at GW01, the elmid larva, Optioservus sp. was somewhat hyperdominant,  adding to the indicators of stress on the aquatic communities.  The overall score on these metrics (24/48) placed this site in the Moderately Impaired category (Table 2).

### BB00—Bassett Brook upstream from Mason Road, Cheshire, MA

#### Habitat

The Hoosic River begins at the outlet of Cheshire Reservoir in Cheshire, MA.  It flows for approximately 7.1 km before meeting Bassett Brook at Harbor Road in Cheshire Harbor.  BB00 was located on Bassett Brook about 730 m upstream from Harbor Road (about 150 m upstream from Mason Road). The drainage area upstream from the sample reach was smaller than that of the reference site: 7.5 km2 (2.9 mi2) compared to GE01’s 10.1 km2 (3.9 mi2) The streambed substrates at BB00 were nearly all cobble and boulder.  Though less than half the channel carried water there was an abundance of riffle habitat, a variety of water depths and velocities, and excellent cover for fish.  The overall habitat score was 181/200 (Appendix A, Table A3).

#### Benthos

In spite of a reasonably good EPT, this metric and the ratio of EPT individuals to midges resulted in large point losses.  Hyperdominance by the midge, Polypedilum aviceps also caused a loss of points for percent contribution.  Ironically, P. aviceps is considered a clean water indicator, seldom occurring in large populations in waters with any kind of impairment.  Based only on the outcome of the metric score (26/48) BB00 ranked as Slightly Impaired (Table 3).

Table 3.	RBP data summary for Bassett Brook (BB00) and Pecks Brook (PB00) biomonitoring sites sampled on 3 September 1997.

| STATION # | GE01 | BB00 | PB00 |
| --- | --- | --- | --- |
| STREAM | E. Br. Gr. R. upst. fr. confluence, New Ashford | Bassett Brook upst. fr. Gas pipeline, Cheshire | Peck’s Brook upst. fr. Power lines, Adams |
| HABITAT SCORE | 143 R | 181 >100% | 154 >100% |
| TAXA RICHNESS | 32 score: R 6 | 27 score: 0.84 6 | 23 score: 0.72 4 |
| BIOTIC INDEX | 3.55 R 6 | 3.81 0.93 6 | 3.47 >1 6 |
| EPT INDEX | 17 R 6 | 13 0.76 2 | 11 0.65 0 |
| EPT/CHIRONOMIDAE | 11 R 6 | 0.54 0.05 0 | 7.0 0.64 4 |
| RIFFLE COMMUNITY: SCRAPERS/FILT. COLL. | 0.27 R 6 | 0.31 >1 6 | 4.2 >1 6 |
| [FC/Total] | (0.16) | (0.16) | (0.05) |
| % CONTRIBUTION (DOM. TAXON) | 11% 6 | 33% 2 | 16% 6 |
| %SIMILARITY | R 6 | 26% 0 | 38% 2 |
| NYSPMA | 74 6 | 52 4 | 84 6 |
| Reference | R | GE01 | GE01 |
| Score | R 48 | 26/48 26 | 34/48 34 |
| Category | R | 54% SL | 71% SL |

R = reference  SL = slightly impaired

### PB00—Pecks Brook upstream from West Road, Adams, MA

#### Habitat

Approximately 3.5 km downstream from the confluence of Bassett Brook with the Hoosic River is where Pecks Brook enters.  PB00 was about 1.5 km upstream from the mouth of Pecks Brook and nearly 275 m upstream from West Road.  The sample reach was bounded on the downstream end by the right-of-way for the high tension power lines, and at the upstream end by the pool at the base of a steep waterfall.  The upstream drainage area (5.4 km2—2.1 mi2) was only little more than half that of the reference site, GE01.  PB00 offered excellent rocky substrates (mostly cobble and boulder) for macroinvertebrates and adequate, though not optimal, fish cover. Only about 50% of the stream channel was filled and water depth tended to be shallow throughout. Along at least half of the reach both banks appeared to be moderately unstable and subject to erosion during high water.  The overall habitat score was 154/200 (Appendix A, Table A3).

#### Benthos

Low relative richness and low EPT accounted for most of the point losses in scoring the metrics for this station.  The HBI (3.47) was the lowest among the Hudson watershed sites sampled, and PB00 had the highest affinity for the New York State model of any of these sites.  Based on the total score of the metrics alone this site would be ranked as Slightly Impaired (Table 3).

### HR07U and HR07D—Hoosic River at Adams WWTP, Adams, MA

#### Habitat

From the confluence with Pecks Brook the Hoosic flows through Adams appoximately 3.5 km to the sample reach, HR07U, the upstream half of an upstream/downstream pair of sites bracketing the Adams WWTP.  This was about 50 m upstream from the discharge, and adjacent to a small island.  The majority of water flowed along the west side of the island.  Fast current predominated and the water depth varied, up to a meter deep in places.  The channel along the eastern side of the island carried much less water, uniformly shallow but fast-flowing.  The bottom substrates were mostly cobble, with lesser amounts of boulder and gravel.  Stable fish cover was very limited.  The undisturbed riparian vegetative zone provided no more than six meters of buffer from cultivated fields on either bank.  The overall habitat score was 153/200 (Appendix A, Table A3).

The other half of the bracket, HR07D, was approximately 450 m downstream from the discharge.  Most of this distance was an extremely straight channel, rip-rapped along the eastern bank.  The HR07D sample reach itself appeared to have a normal, sinuous pattern, although the banks in this reach were rip-rapped at the bends.  Epifaunal substrates were excellent, primarily cobble; fish cover was good, but not optimal.  About 5 - 10% of the bottom appeared to be affected by sediment deposition and bar formation.  The velocity and depth combinations in this stretch were very good and more than 75% of the channel was covered with water.  Some erosion potential was detected along the west bank and the riparian zone buffer between the east bank and a cultivated field was no more than 6 m.  The overall habitat score was 158/200 (Appendix A, Table A3).

#### Benthos

The total richness of taxa at these sites was not low enough to lose points, but the richness of EPT taxa was very low at both sites, even the upstream  one.  Both sites exhibited slight hyperdominance.  Upstream the dominant was the elmid beetle Optioservus sp. while downstream it was a filter-feeding caddisfly in the Hydropsyche morosa group.  Both the upstream and downstream sites scored (28/48 for HR07U and 26/48 for HR07D) in the Slightly Impaired category when compared to the watershed reference, GE01.  When the downstream site, HR07D was compared against its upstream reference (HR07U) there was little difference in the individual metrics and the total score ranked HR07D as Nonimpaired (Table 4).

### HR03 and HR02—Hoosic River at Hoosac Water Quality District, Williamstown, MA

#### Habitat

For approximately the next 8.3 km downstream from HR07D, the Hoosic continues its northward path into downtown North Adams where it meets the North Branch Hoosic River amid high-sided concrete channels.  From there the river flows westward through North Adams before crossing into Williamstown and beginning a gentle northward arc into Vermont.  About 7.4 km from the North Branch confluence the Hoosic receives the waters of the Green River, and after another 3.0 km reaches HR03.  HR03 was the upstream half of a pair of sites bracketing the Hoosac Water Quality District (HWQD) discharge.  Lying about 230 m upstream from the discharge, HR03 was wide, with excellent substrates for benthos and well developed riffles and runs.  Most of the channel was covered with water and there was an excellent range of velocity/depth patterns.  Cover for fish was also excellent.  There were some problems with sediment deposition

Hudson River Basin 1997 Water Quality Assessment ReportAppendix C- C 10-  11wqar.docDWM CN 15.0

Table 4.	RBP data summary for Hoosic River biomonitoring sites sampled 11-13 August 1997.

| STATION # | GE01 | HR02 | HR03 | HR07D | HR07U | HR02 | HR03 | HR07D | HR07U |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| STREAM | E. Br. Gr. R. upst. fr. confluence, New Ashford | Hoosic River dnst. fr. HWQD, Williamstown | Hoosic River upst. fr. HWQD, Williamstown | Hoosic River dnst. fr. Adams WWTP | Hoosic River upst. fr. Adams WWTP | Hoosic River dnst. fr. HWQD, Williamstown | Hoosic River upst. fr. HWQD, Williamstown | Hoosic River dnst. fr. Adams WWTP | Hoosic River upst. fr. Adams WWTP |
| HABITAT SCORE | 143; R | 172; >100% | 162; >100% | 158; >100% | 153; >100% | 172; >100% | 162; R | 158; >100% | 153; R |
| TAXA RICHNESS | 32 score: R 6 | 28 score: 0.88 6 | 26 score: 0.81 6 | 28 score: 0.88 6 | 26 score: 0.81 6 | 28 score: >1 6 | 26 score: R 6 | 28 score: >1 6 | 26 score: R 6 |
| BIOTIC INDEX | 3.55 R 6 | 4.60 0.77 4 | 4.71 0.75 4 | 4.57 0.78 4 | 4.14 0.86 6 | 4.60 >1 6 | 4.71 R 6 | 4.57 0.91 6 | 4.14 R 6 |
| EPT INDEX | 17 R 6 | 9 0.53 0 | 11 0.65 0 | 5 0.29 0 | 7 0.41 0 | 9 0.82 4 | 11 R 6 | 5 0.71 2 | 7 R 6 |
| EPT/CHIRONOMIDAE | 11 R 6 | 1.68 0.15 0 | 1.64 0.15 0 | 2.41 0.22 0 | 2.71 0.25 2 | 1.68 >1 6 | 1.64 R 6 | 2.41 0.89 6 | 2.71 R 6 |
| SCRAPERS/FILT. COLL. | 0.27 R 6 | 0.77 >1 6 | 1.5 >1 6 | 1.0 >1 6 | 1.8 >1 6 | 0.77 0.51 6 | 1.5 R 6 | 1.0 0.56 6 | 1.8 R 6 |
| [FILT. COLL./TOTAL] | (0.16) | (0.38) | (0.19) | (0.31) | (0.21) | (0.38) | (0.19) | (0.31) | (0.21) |
| % CONTRIBUTION (DOM. TAXON) | 11% 6 | 24% 4 | 19% 6 | 27% 4 | 20% 4 | 24% 4 | 19% 6 | 27% 4 | 20% 4 |
| % SIMILARITY | R 6 | 10% 0 | 15% 0 | 7% 0 | 9% 0 | 57% 4 | R 6 | 52% 4 | R 6 |
| [NYSPMA] | 74 6 | 58 4 | 68 6 | 75 6 | 64 4 | 58 4 | 68 6 | 75 6 | 64 4 |
| Reference | Reference | GE01 | GE01 | GE01 | GE01 | HR03 | Reference | HR07U | Reference |
| Score | R 48 | 24/48 24 | 28/48 28 | 26/48 26 | 28/48 28 | 40/48 40 | R 48 | 40/44 40 | R 44 |
| Category | Reference | 50% MI | 58% SL | 54% SL | 58% SL | 83% NI | Reference | 91% NI | Reference |

NI = nonimpaired

SL = slightly impaired

MI = moderately impaired

and embeddedness of the benthic substrates.  The northerly bank had erosion evident along about 30% of the length of the reach, indicating a high susceptibility during flood events.  The vegetative buffer zone between the river and the roadway along the northerly bank was no more than 12 m.  The overall habitat score was 162/200 (Appendix A, Table A3).

HR02 was approximately 550 m downstream from the HWQD discharge.  This reach also had excellent benthic substrates and fish cover, with lesser evidence of sedimentation and embeddedness problems.  More of the streambed substrates were exposed here, though, than at HR03 and the flow pattern tended to be more monotonous (one long riffle/run).  Riparian zone vegetative cover and buffer distances were excellent.  The overall habitat score was 172/200 (Appendix A, Table A3).

#### Benthos

Relative to GE01, the HBI was higher at both the upstream (HR03) and downstream (HR02) sites.  These sites also had much lower EPT values resulting in scores of zero for that metric.  HR02 had slight hyperdominance by the filter-feeding caddisfly, Hydropsyche morosa group.  This site also had a reduced score for the NYSPMA.  When compared against GE01, the total score for HR03 was 28/48 and the score for HR02 was 24/48 resulting in determinations of Slightly Impaired and Moderately Impaired, respectively.  Using HR03 as the upstream reference, however, gave HR02 a total score of 40/48, placing it in the Nonimpaired category (Table 4).

### KINDERHOOK/HUDSON

Kinderhook Creek originates in a fairly steep-sided valley in Hancock, Massachusetts between the Brodie Mountain ridge to the east and the Taconic Range ridge running along the New York State line between Rounds Mountain and Misery Mountain.  This drainage abuts that of the West Branch Green River to the north.  The Kinderhook flows southward, then southwesterly, covering approximately nine kilometers (5.6 mi.) before crossing the state line into New York.

The list of taxa and counts at the sampling locations in the Kinderhook Creek drainage can be found in Appendix B, Table B3.

### KC01 and KC02—upstream and downstream from Bentley Brook confluence, Hancock, MA

#### Habitat

Stations KC01 and KC02 were upstream and downstream (respectively) of the Bentley Brook confluence and the intake structure for Jiminy Peak’s water withdrawal.  The bottom of the KC01 sample reach was approximately 180 m downstream from Brodie Mountain Road and about 45 m upstream from Bentley Brook.  KC02 was approximately 460 m downstream from Brodie Mountain Road, about 240 m downstream from Bentley Brook.  The drainage area upstream from KC01 was 18 km2 (6.8 mi2) whereas the total upstream drainage area for KC02 was 25 km2 (9.7 mi2 ).

Cobble and boulder were the dominant substrate materials at KC01.  Low water was very limiting at this site.  The mostly shallow water filled no more than 25% of the channel and left little useful habitat with instream cover for fish.  The riparian vegetative buffer was about 15 m to a mowed residential lawn on the east bank and 18 m or more on the west bank.  The overall habitat score was 147/200 (Appendix A, Table A4).

The benthic substrate character at KC02 was very similar to that at KC01.  Here, however, water filled more of the channel and provided some deeper water areas, resulting in more available habitat for both macroinvertebrates and fish.  The western bank was a bit unstable in places but had a deep vegetated buffer zone; the eastern bank appeared to be quite stable but with a vegetated buffer zone of somewhat less than 18 m.  The overall habitat score was 167/200 (Appendix A, Table A4).

#### Benthos

Though the richness of macroinvertebrates appeared to be very good at both KC01 and KC02, in both cases it was less than 80% of the watershed reference (GE01).  Somewhat surprising was the relatively high HBI and the low EPT, given the position of these sites within the watershed.  Hyperdominance also led to point losses at both sites.  The total score relative to GE01 was 14/48 for KC01 and 18/48 for KC02, placing both sites in the Moderately Impaired category (Table 5).

Table 5.  RBP data summary for Kinderhook Creek biomonitoring sites sampled 11 August 1997.

| STATION # | GE01 | KC01 | KC02 | KC01 | KC02 |
| --- | --- | --- | --- | --- | --- |
| Stream | E. Br. Gr. R. upst. fr. confluence, New Ashford | Kinderhook Creek upst. fr. Bentley Brook, Hancock | Kinderhook Creek dnst. fr. Bentley Brook, Hancock | Kinderhook Creek upst. fr. Bentley Brook, Hancock | Kinderhook Creek dnst. fr. Bentley Brook, Hancock |
| Habitat Score | 143 R | 147 >100% | 167 >100% | 147 >100% | 167 >100% |
| Taxa Richness | 32 score: R 6 | 25 score: 0.78 4 | 23 score: 0.72 4 | 25 score: R 6 | 23 score: 0.92 6 |
| Biotic Index | 3.55 R 6 | 4.68 0.76 4 | 4.31 0.82 4 | 4.68 R 6 | 4.31 >1 6 |
| Ept Index | 17 R 6 | 9 0.53 0 | 9 0.53 0 | 9 R 6 | 9 1.0 6 |
| Ept/Chironomidae | 11 R 6 | 0.80 0.07 0 | 2.68 0.24 0 | 0.80 R 6 | 2.68 >1 6 |
| Riffle Community: Scrapers/Filt. Coll. | 0.27 R 6 | 0.04 0.15 0 | 0 0 | 0.04 R 6 | 0 0 |
| [Fc/Total] | (0.16) | (0.23) | (0.33) | (0.23) | (0.33) |
| % Contribution (Dom. Taxon) | 11% 6 | 31% 2 | 20% 4 | 31% 2 | 20% 4 |
| %Similarity | R 6 | 21% 0 | 24% 0 | R 6 | 53% 4 |
| Nyspma | 74 6 | 64 4 | 79 6 | 64 4 | 79 6 |
| Reference | R | GE01 | GE01 | R | KC01 |
| Score | R 48 | 14/48 14 | 18/48 18 | R 42 | 34/42 38 |
| Category | R | 29% MI | 38% MI | R | 81% NI |

R = reference  NI = nonimpaired  MI = moderately impaired

When the metrics were recalculated using KC01 as the upstream reference site, the downstream site, KC02, lost points for its lack of scrapers, slight hyperdominance, and for only mediocre similarity to the assemblage at KC01.  Even so, against this reference KC02 scored 38/42 for a rating of Nonimpaired (Table 5).

DISCUSSION AND CONCLUSIONS

Overall the benthic data suggest healthy, robust communities that in most cases would receive high scores in other watersheds around Massachusetts, particularly in the eastern half of the state.  When compared to the extraordinary community attributes of the reference site (GE01—East Branch Green River), however, the scores indicate some degree of impairment at nearly every site assessed.  In all cases it appears that the cause of the “impairment” is related to nonpoint source (NPS) pollution.  While the results do not represent cause for alarm they nevertheless serve as indicators of developing problems and/or identify areas that could benefit from application of best management practices (BMPs) to control NPS pollution.

### THE GREEN RIVER

All of the Green River sites offered excellent aquatic habitat.  Interestingly, the reference station (GE01) earned the lowest overall score for habitat quality.  The total richness of taxa was very good at all sites,  each with 30 or more taxa represented, except GW04, which had 27.  The EPT index also appeared good at all stations, but when compared to the reference only GN04 had a sufficient number of EPT taxa to score any points.  The EPT/Chironomidae abundance ratio was so high (chironomid abundances were extremely low) at GE01 that none of the other Green River sites scored any points for that metric.  Primarily because of these two EPT-related metrics, all of the Green River sites scored either in the slightly impaired or moderately impaired category of the assessment.   In another watershed the metric values that resulted in a determination of slightly impaired for these Green River sites probably would have resulted in a determination of nonimpaired.

With no major NPDES-permitted direct discharges—and only one NPDES cooling water discharge—to any of the Green River branches, any detected impairment must be from NPS contamination.  A look at surrounding land use can offer clues as to the nature of the source.  The only readily apparent influences acting on GN04 were runoff from Rte. 7 and the restaurant/motel property, and possible interaction of septic systems from the few upstream houses and the restaurant/motel property.  For GN03, GN02, and GW01, agricultural practices are the likely cause of the detected impairment.   Though the level of impairment is slight at these locations the elevated HBI values suggest that oxygen demand pressures (probably from nutrient loadings) may be stressing populations and that if not mitigated might be expected to intensify.  The moderately impaired determination for GN01 is surely the result of the cumulative effects from upstream loadings plus ineffective stormwater management in the vicinity of the Williamstown Municipal Garage.  If BMPs were implemented in the upstream reaches, and a stormwater mitigation plan were implemented at the Municipal Garage the assessment at this site should improve in the future.

### THE HOOSIC RIVER

The Hoosic River receives wastewater from two major dischargers: the North Adams Wastewater Treatment Plant and the Hoosac Valley Water Quality District WWTP.  Each discharge was bracketed by an upstream/downstream pair of stations.  All four stations offered excellent substrate and flow conditions for the benthos.  The score for HR02, downstream from the HWQD, put it in the Moderately Impaired category, while the other three stations scored in the Slightly Impaired category relative to the watershed reference—GE01.  When compared against its respective upstream reference, however, each of the downstream stations (HR02 and HR07D) had scores that placed them in the Nonimpaired category.  It would appear, then, that these discharges place little additional stress on aquatic life in this system; but the system could clearly benefit from an examination and evaluation of urban runoff and other potential NPS pollution sources.

### PECKS BROOK AND BASSETT BROOK

The benthic habitat in Pecks Brook had excellent substrate characteristics and well developed riffles, though the water was shallow throughout and filled only about half of the channel.  Bassett Brook had superb habitat for benthos, though it too had roughly half of its channel substrates exposed.  Since both watersheds have very little development within them it is somewhat of a surprise to see their scores place them in the Slightly Impaired category.  Though there was some evidence that there may have been recent logging activities in the vicinity of Bassett Brook, there really didn’t seem to be any reason these watersheds would not be gauged unimpaired.  These are very small watersheds, however; the area of the Pecks Brook watershed above the sample location is only 74% of the GE01 upstream watershed area, and Bassett Brook’s is only 53%.  It may be that these watersheds are simply too small to have developed the nutrient and energy flow necessary to support the kind of robust community found at GE01.  Whatever the reason for these results they underscore the importance of establishing baseline data for these watersheds before any further development takes place.  Since DWM’s next scheduled monitoring isn’t until 2002 it would be desirable for other parties—perhaps the watershed association, perhaps the developer of the proposed resort—to conduct aquatic life assessments in these watersheds.

### KINDERHOOK CREEK

Both Kinderhook Creek sites provided very good substrates and well developed riffle habitat, though the site upstream from Bentley Brook (KC01) suffered from the low volume of water present (restricting the depth and availability of riffle habitat).  Against the watershed reference (GE01) both Kinderhook Creek sites were rated moderately impaired.  As compared to KC01, the site downstream from Bentley Brook (KC02) was Nonimpaired.  Clearly, then, the impairment is occurring upstream from the Bentley Brook confluence. Dilution from Bentley Brook may be mitigating effects somewhat, judging from the improved HBI, NYSPMA score, and percent dominance.  The only apparent sources of pollution in the upstream watershed are agricultural and road runoff.

LITERATURE CITED

Novak, M.A., and R.W. Bode.  1992.  Percent model affinity: a new measure of macroinvertebrate community composition.  Journal of the North American Benthological Society.  11:80-85.

Nuzzo, R.M.  1999.  Standard operating procedures: water quality monitoring in streams using aquatic macroinvertebrates.  Massachusetts DEP, Division of Watershed Management.  Worcester, MA.  8 p.

Plafkin, J.L., M.T. Barbour, K.D. Porter, S.K. Gross, R.M. Hughes.  1989.  Rapid bioassessment protocols for use in streams and small rivers: benthic macroinvertebrates and fish.  U.S. Environmental Protection Agency.  Washington, D.C.  EPA/444/4-89-001.

APPENDIX A Habitat Assessments

Table A1.  Massachusetts DEP/DWM Habitat Assessment Field Scoring Sheet   (page 1 of 2).

| Investigator(s) |  |  |  | Reference Site |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| River Basin |  |  |  | Stream Name |  |  |  | Saris # |  |  |  |
| Describe Site Location: |  |  |  |  |  |  |  |  |  |  |  |

Protocols for Wadable Riffle/Run Prevalent Streams: those in moderate to high-gradient landscapes that sustain water velocities of approximately 30 cm/sec or greater.  Natural streams have substrates primarily composed of coarse sediment particles (i.e., gravel or larger) or frequent coarse particulate aggregations along stream reaches.

| Habitat Parameter | CATEGORY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Optimal |  |  |  |  |  | Suboptimal |  |  |  |  | Marginal |  |  |  |  |  |  |  | Poor |  |  |  |  |  |  |  |  |  |
| 1. Instream Cover (Fish) | A mix of snags, submerged logs, undercut banks, rubble, or other stable habitat in greater than 50% of the sample area |  |  |  |  |  | 30-50% of area with a mix of stable habitat; adequate habitat for maintenance of populations |  |  |  |  | 10-30% of area with a mix of stable habitat; habitat availability less than desirable; substrate frequently disturbed or removed. |  |  |  |  |  |  |  | Less than 10% of area with a mix of stable habitat; lack of habitat is obvious; substrate unstable or lacking. |  |  |  |  |  |  |  |  |  |
| SCORE | 20 | 19 |  | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 |  | 7 |  | 6 |  | 5 | 4 |  | 3 | 2 |  | 1 |  | 0 |  |
| 2. Epifaunal Substrate (in sampled area only) | Well-developed riffle and run; riffle is as wide as stream and length extends two times the width of stream; abundance of cobble. (Boulders prevalent in headwater streams). |  |  |  |  |  | Riffle is as wide as stream but length is less than two times width; abundance of cobble; boulders and gravel common. |  |  |  |  | Run area may be lacking; riffle not as wide as stream and its length is less than 2 times the stream width; gravel or bedrock prevalent; some cobble present. |  |  |  |  |  |  |  | Riffles or runs virtually nonexistent; bedrock prevalent; cobble lacking. |  |  |  |  |  |  |  |  |  |
| SCORE | 20 |  | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 |  | 7 |  | 6 |  | 5 | 4 |  | 3 | 2 |  | 1 |  | 0 |  |
| 3. Embeddedness (riffles/runs) | Gravel, cobble, and boulder particles are 0-25% surrounded by fine sediment. |  |  |  |  |  | Gravel, cobble, and boulder particles are 25-50% surrounded by fine sediment. |  |  |  |  | Gravel, cobble, and boulder particles are 50-75% surrounded by fine sediment. |  |  |  |  |  |  |  | Gravel, cobble, and boulder particles are more than 75% surrounded by fine sediment. |  |  |  |  |  |  |  |  |  |
| SCORE | 20 |  | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 |  |  | 7 |  | 6 | 5 |  | 4 | 3 |  | 2 |  | 1 |  | 0 |
| 4. Channel Alteration | Channelization or dredging absent or minimal; stream with normal pattern. |  |  |  |  |  | Some channelization present, usually in areas of bridge abutments; evidence of past channelization, i.e., dredging, (greater than past 20 y) may be present, but recent channelization is not present. |  |  |  |  | New embankments present on both banks; and 40 to 80% of stream reach channelized and disrupted. |  |  |  |  |  |  |  | Banks shored with gabion or cement; over 80% of the stream reach channelized and disrupted. |  |  |  |  |  |  |  |  |  |
| SCORE | 20 |  | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 |  |  | 7 |  | 6 | 5 |  | 4 | 3 |  | 2 |  | 1 |  | 0 |
| 5. Sediment Deposition | Little or no enlargement of islands or point bars and less than 5% of the bottom affected by sediment deposition. |  |  |  |  |  | Some new increase in bar formation, mostly from gravel, sand or fine sediment; 5-30% of the bottom affected; slight deposition in pools. |  |  |  |  | Moderate deposition of new gravel, sand or fine sediment on old and new bars; 30-50% of the bottom affected; sediment deposits at obstructions, constrictions, and bends; moderate deposition of pools prevalent. |  |  |  |  |  |  |  | Heavy deposits of fine material, increased bar development; more than 50% of the bottom changing frequently; pools almost absent due to substantial sediment deposition. |  |  |  |  |  |  |  |  |  |
| SCORE | 20 |  | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 |  | 8 |  | 7 |  | 6 | 5 |  | 4 | 3 |  | 2 |  | 1 |  | 0 |

Date:                                                                Station:

Table A1.  Massachusetts DEP/DWM Habitat Assessment Field Scoring Sheet   (page 2 of 2)

| HABITAT PARAMETER | CATEGORY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Optimal |  |  |  |  |  | Suboptimal |  |  |  |  |  |  | Marginal |  |  |  |  | Poor |  |  |  |  |  |
| 6. Velocity-Depth Combinations slow deep fast deep slow shallow fast shallow (frequency of riffles or bends) | All 4 velocity/depth patterns present. Occurrence of riffles relatively frequent; ratio of distance between riffles divided by width of the stream <7:1 (generally 5 to 7); variety of habitat is key. In streams where riffles are continuous, placement of boulders or other large, natural obstructions is important. |  |  |  |  |  | Only 3 of 4 velocity/depth patterns present (i.e., slow [<0.3 m/s]-deep [>0.5 m]; slow-shallow; fast-deep; fast-shallow). Occurrence of riffles infrequent; distance between riffles divided by the width of the stream is between 7 to 15. |  |  |  |  |  |  | Only 2 velocity/depth patterns present; usually lacking deep areas. Occasional riffle or bend; bottom contours provide some habitat; distance between riffles divided by the width of the stream is between 15 to 25. |  |  |  |  | Dominated by one velocity/depth pattern. Generally all flat water or shallow riffles; poor habitat; distance between riffles divided by the width of the stream is a ratio of >25. |  |  |  |  |  |
| SCORE | 20 | 19 | 18 | 17 | 16 |  | 15 | 14 |  | 13 | 12 |  | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| 7. Channel Flow Status | Water reaches base of both lower banks, and minimal amount of channel substrate is exposed. |  |  |  |  |  | Water fills >75% of the available channel; or <25% of channel substrate is exposed. |  |  |  |  |  |  | Water fills 25-75% of the available channel, and/or riffle substrates are mostly exposed. |  |  |  |  | Very little water in channel and mostly present as standing pools. |  |  |  |  |  |
| SCORE | 20 | 19 | 18 | 17 | 16 |  | 15 | 14 |  | 13 | 12 |  | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| 8. Bank Vegetative Protection (score each bank) Note: determine left or right side by facing downstream. | More than 90% of the streambank surfaces covered by naturally occurring vegetation, including trees, understory shrubs, or nonwoody macrophytes; vegetative disruption through grazing or mowing minimal or not evident; almost all plants allowed to grow naturally. |  |  |  |  |  | 70-90% of the streambank surfaces covered by naturally occurring vegetation, but one class of plants is not well-represented; disruption evident but not affecting full plant growth potential to any great extent; more than one-half of the potential plant stubble height remaining. |  |  |  |  |  |  | 50-70% of the streambank surfaces covered by vegetation; disruption obvious; patches of bare soil or closely cropped vegetation common; less than one-half of the potential plant stubble height remaining. |  |  |  |  | Less than 50% of the streambank surfaces covered by vegetation; disruption of streambank vegetation is very high; vegetation has been removed to 5 centimeters or less in average stubble height. |  |  |  |  |  |
| SCORE (LB) | Left Bank |  |  | 10 |  | 9 | 8 |  | 7 |  |  | 6 |  | 5 | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| SCORE (RB) | Right Bank |  |  | 10 |  | 9 | 8 |  | 7 |  |  | 6 |  | 5 | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| 9. Bank Stability (score each bank) | Banks stable; evidence of erosion or bank failure absent or minimal; little potential for future problems. <5% of bank affected. |  |  |  |  |  | Moderately stable; infrequent, small areas of erosion mostly healed over. 5-30% of bank in reach has areas of erosion. |  |  |  |  |  |  | Moderately unstable; 30-60% of bank in reach has areas of erosion; high erosion potential during floods. |  |  |  |  | Unstable; many eroded areas; "raw" areas frequent along straight sections and bends; obvious bank sloughing; 60-100% of bank has erosional scars. |  |  |  |  |  |
| SCORE (LB) | Left Bank |  |  | 10 |  | 9 | 8 |  | 7 |  |  | 6 |  | 5 | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| SCORE (RB) | Right Bank |  |  | 10 |  | 9 | 8 |  | 7 |  |  | 6 |  | 5 | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| 10. Riparian Vegetative Zone Width (score each bank riparian zone) | Width of riparian zone >18 meters; human activities (i.e., parking lots, roadbeds, clear-cuts, lawns, or crops) have not impacted zone. |  |  |  |  |  | Width of riparian zone 12-18 meters; human activities have impacted zone only minimally. |  |  |  |  |  |  | Width of riparian zone 6-12 meters; human activities have impacted zone a great deal. |  |  |  |  | Width of riparian zone <6 meters: little or no riparian vegetation due to human activities. |  |  |  |  |  |
| SCORE (LB) | Left Bank |  |  | 10 |  | 9 | 8 |  | 7 |  |  | 6 |  | 5 | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| SCORE (RB) | Right Bank |  |  | 10 |  | 9 | 8 |  | 7 |  |  | 6 |  | 5 | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| TOTAL SCORE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Comments:

Date:                                                                Station:

Table A2.	Habitat scores for Green River Stations and the reference station, GE01.

| Habitat Category | GE01 | GW01 | GN04 | GN03 | GN02 | GN01 |
| --- | --- | --- | --- | --- | --- | --- |
| Fish cover | 8 | 19 | 15 | 17 | 20 | 20 |
| Epifaunal Substrates | 15 | 19 | 10 | 16 | 20 | 19 |
| Embeddedness | 19 | 16 | 19 | 13 | 13 | 13 |
| Channel Alteration | 20 | 20 | 19 | 19 | 20 | 20 |
| Sediment Deposition | 14 | 11 | 18 | 16 | 10 | 10 |
| Velocity-Depth Combinations | 8 | 17 | 13 | 16 | 19 | 19 |
| Channel Flow Status | 7 | 8 | 7 | 14 | 15 | 9 |
| Bank Vegetative Protection | 20 | 20 | 20 | 19 | 20 | 18 |
| Bank Stability | 19 | 16 | 20 | 17 | 18 | 15 |
| Riparian Vegetated Zone Width | 13 | 18 | 19 | 16 | 14 | 17 |
| Total Points (out of 200 possible) | 143 | 164 | 160 | 163 | 169 | 160 |

Table A3.	Habitat scores for Bassett Brook, Pecks Brook, Hoosic River mainstem sites and the reference station, GE01.

| Habitat Category | GE01 | BB00 | PB00 | HR07U | HR07D | HR03 | HR02 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Fish cover | 8 | 19 | 15 | 9 | 15 | 20 | 18 |
| Epifaunal Substrates | 15 | 20 | 20 | 20 | 20 | 20 | 20 |
| Embeddedness | 19 | 20 | 16 | 17 | 17 | 11 | 13 |
| Channel Alteration | 20 | 20 | 20 | 15 | 13 | 18 | 20 |
| Sediment Deposition | 14 | 20 | 18 | 19 | 14 | 10 | 17 |
| Velocity-Depth Combinations | 8 | 17 | 10 | 13 | 16 | 20 | 14 |
| Channel Flow Status | 7 | 7 | 8 | 18 | 15 | 16 | 11 |
| Bank Vegetative Protection | 20 | 20 | 20 | 20 | 19 | 18 | 20 |
| Bank Stability | 19 | 18 | 8 | 18 | 16 | 15 | 19 |
| Riparian Vegetated Zone Width | 13 | 20 | 19 | 4 | 13 | 14 | 20 |
| Total Points (out of 200 possible) | 143 | 181 | 154 | 153 | 158 | 162 | 172 |

Table A4.	Habitat scores for Kinderhook Creek Stations KC01 and KC02 and the reference station, GE01.

| Habitat Category | GE01 | KC01 | KC02 |
| --- | --- | --- | --- |
| Fish cover | 8 | 6 | 13 |
| Epifaunal Substrates | 15 | 16 | 19 |
| Embeddedness | 19 | 19 | 17 |
| Channel Alteration | 20 | 20 | 19 |
| Sediment Deposition | 14 | 18 | 18 |
| Velocity-Depth Combinations | 8 | 8 | 15 |
| Channel Flow Status | 7 | 6 | 11 |
| Bank Vegetative Protection | 20 | 20 | 20 |
| Bank Stability | 19 | 18 | 17 |
| Riparian Vegetated Zone Width | 13 | 16 | 18 |
| Total Points (out of 200 possible) | 143 | 147 | 167 |

APPENDIX B - Taxa Lists for 1997 Hudson River Watershed Benthic Macroinvertebrate Sampling Stations.

Table B1.	These results are from the 1997 benthos sampling in the Green River (Hoosic/Hudson watershed).  There were four stations on the mainstem Green River: GN01, upstream from Route 2 in Williamstown; GN02, downstream from Blair Road, Williamstown; GN03,  upstream from the lower (more downstream) Route 43 crossing (B.M. 252.5 m), Williamstown; GN04, downstream from the falls at Mill on the Floss restaurant (east/southeast of New Ashford Cemetery), New Ashford.  There was also one site each on the West Branch Green River (GW01, upstream from Old Mill Road, Williamstown) and the East Branch Green River (GE01, about 200 m upstream from its confluence with the mainstem Green River, New Ashford).  GE01 was selected to serve as a watershed reference due to its relatively undisturbed drainage area.  All sites were in Massachusetts.  The columns labeled FFG and TOLVAL show the functional group and tolerance value, respectively, designated for each taxon, as used in the analyses.

| TAXON/METRIC | FFG | TOLVAL | GN01 | GN02 | GN03 | GN04 | GW01 | GE01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Physidae | GC | 8 |  | 1 |  |  |  |  |
| Tubificidae immature with capilliform chaetae | GC | 10 |  |  |  |  | 3 |  |
| Nais alpina | GC | 8 |  |  |  |  |  | 1 |
| Nais behningi | GC | 6 | 2 |  | 2 |  |  |  |
| Nais bretscheri | GC | 6 |  |  | 1 |  |  |  |
| Nais communis | GC | 8 |  |  |  |  |  | 1 |
| Lumbriculus sp. | GC | 5 | 1 | 6 | 4 | 2 | 1 |  |
| Eclipidrilus sp. | GC | 5 |  |  |  | 1 |  |  |
| Hydracarina | PR | 6 | 2 | 1 | 10 | 2 | 1 | 2 |
| Baetidae | GC | 6 | 2 | 1 |  | 1 | 2 | 4 |
| Baetidae undet. 1 (cerci only) | GC | 6 | 1 | 2 | 1 |  | 2 | 4 |
| Baetidae undet. 2 (short term.) | GC | 6 |  | 1 |  | 2 |  |  |
| Baetidae undet. 3 (subeq. term.) | GC | 6 | 1 | 1 | 3 | 6 | 4 |  |
| Acentrella sp. | GC | 4 |  | 5 |  |  |  |  |
| Baetis sp. 2 (short term.) | GC | 6 |  |  |  |  |  | 9 |
| Baetis sp. 3 (subeq. term.) | GC | 6 |  |  |  |  |  | 11 |
| Isonychia sp. | GC | 2 |  |  | 1 |  | 1 |  |
| Heptageniidae | SC | 3 | 6 |  |  |  |  |  |
| Epeorus (Iron) sp. | SC | 0 |  |  |  | 1 |  | 3 |
| Stenonema sp. | SC | 3 |  | 6 | 4 |  | 2 |  |
| Ephemerellidae | GC | 2 |  |  |  | 2 |  |  |
| Drunella cornutella | GC | 0 |  |  |  |  |  | 7 |
| Ephemerella catawba | GC | 1 |  |  | 1 |  |  |  |
| Serratella sp. | GC | 2 | 1 | 1 |  |  | 4 |  |
| Tricorythodes sp. | GC | 4 |  |  | 2 |  |  |  |
| Caenis sp. | GC | 6 | 1 |  |  |  |  |  |
| Leptophlebiidae | GC | 4 |  |  |  |  |  | 2 |
| Pteronarcys sp. | SH | 0 |  |  |  |  |  | 3 |
| Tallaperla sp. | SH | 0 |  |  |  |  |  | 2 |
| Nemouridae | SH | 2 |  |  |  |  |  | 2 |
| Leuctra sp. | SH | 0 |  |  |  | 2 | 1 | 3 |
| Agnetina sp. | PR | 2 |  |  |  | 9 | 6 |  |
| Perlodidae | PR | 2 |  |  | 1 |  |  |  |
| Diura sp. | PR | 2 |  |  |  | 3 |  | 1 |
| Sweltsa sp. | PR | 0 | 1 |  |  | 9 |  | 7 |
| Sialis sp. | PR | 4 |  |  |  |  |  | 1 |
| Dolophilodes sp. | FC | 0 |  | 1 |  |  |  | 3 |
| Psychomyia sp. | GC | 2 | 1 |  |  |  |  |  |
| Polycentropodidae | FC | 6 |  |  |  |  |  | 1 |
| Polycentropus sp. | PR | 6 |  |  |  | 1 |  |  |
| Cheumatopsyche sp. | FC | 5 |  | 3 | 6 | 1 |  | 3 |
| Hydropsyche sp. | FC | 4 |  |  | 4 | 1 | 5 |  |
| Hydropsyche dicantha? | FC | 2 |  |  |  | 1 |  |  |
| Hydropsyche morosa gr. | FC | 6 | 7 | 2 | 5 | 1 | 1 | 1 |
| Rhyacophila sp. | PR | 1 | 1 |  |  | 4 |  |  |
| Agapetus sp. | SC | 0 |  |  |  | 1 |  |  |
| Leucotrichia sp. | SC | 6 |  |  | 1 |  |  |  |
| Brachycentrus sp. | FC | 1 |  |  |  | 1 |  |  |
| Lepidostoma sp. | SH | 1 |  |  |  |  |  | 1 |
| Limnephilidae | SH | 4 |  | 1 |  |  |  |  |
| Neophylax sp. | SC | 3 |  |  |  |  | 1 |  |
| Helicopsyche borealis | SC | 3 |  | 3 | 2 |  |  |  |
| Ectopria sp. | SC | 5 |  |  |  |  |  | 1 |
| Psephenus herricki | SC | 4 | 1 |  | 1 |  |  |  |
| Optioservus sp. | SC | 4 | 24 | 9 | 10 | 1 | 23 |  |
| Optioservus ampliatus(A) | SC | 4 |  |  |  | 1 | 3 |  |
| Optioservus fastiditus (A) | SC | 4 | 1 |  |  |  |  |  |
| Optioservus trivittatus (A) | SC | 4 | 2 |  |  |  |  |  |
| Oulimnius latiusculus | SC | 2 |  |  |  | 1 | 1 |  |
| Promoresia sp. | SC | 2 |  |  |  | 2 |  |  |
| Stenelmis sp. | SC | 5 | 5 |  |  |  |  |  |
| Antocha sp. | GC | 3 | 2 |  | 5 | 1 |  |  |
| Dicranota sp. | PR | 3 |  |  |  |  | 1 | 2 |
| Hexatoma sp. | PR | 2 |  |  |  | 2 |  | 1 |
| Bezzia/Palpomyia sp. | PR | 6 |  |  |  | 1 |  | 2 |
| Simulium tuberosum complex | FC | 4 |  |  |  | 1 |  | 6 |
| Chironomidae | GC | 6 |  | 1 |  |  |  |  |
| Tanypodinae | PR | 7 |  |  |  |  | 1 |  |
| Conchapelopia sp. | PR | 6 | 6 | 3 | 1 | 1 |  |  |
| Helopelopia sp. | PR | 6 |  |  | 1 |  |  |  |
| Nilotanypus sp. | PR | 6 |  | 2 |  |  |  |  |
| Trissopelopia sp. | PR | 4 |  |  |  |  | 1 |  |
| Pagastia sp. | GC | 1 | 1 | 1 |  |  |  |  |
| Potthastia gaedii gr. | GC | 2 |  |  | 1 |  |  |  |
| Cardiocladius sp. | PR | 5 |  | 2 | 3 |  |  |  |
| Corynoneura sp. | GC | 4 |  | 1 |  |  |  |  |
| Cricotopus/Orthocladius sp. | GC | 7 |  | 1 | 2 |  |  |  |
| Cricotopus bicinctus | GC | 7 |  |  | 1 |  |  |  |
| Cricotopus vierriensis | SH | 7 | 1 |  |  |  |  |  |
| Eukiefferiella sp. | GC | 6 |  |  |  | 2 | 1 |  |
| Eukiefferiella pseudomontana gr. | GC | 8 |  |  | 1 |  |  |  |
| Helleniella sp. | GC | 5 |  |  |  |  | 1 |  |
| Orthocladius sp. | GC | 6 | 2 |  |  |  |  |  |
| Parametriocnemus sp. | GC | 5 | 1 |  |  | 3 | 1 | 3 |
| Synorthocladius sp. | GC | 6 |  | 1 |  |  |  |  |
| Tvetenia vitracies gr. | GC | 5 | 1 | 1 |  |  |  |  |
| Microtendipes sp. | FC | 5 |  | 2 |  |  |  |  |
| Microtendipes pedellus gr. | FC | 5 |  | 3 | 2 |  |  |  |
| Polypedilum aviceps | SH | 4 | 3 | 5 | 4 |  | 4 | 1 |
| Polypedilum convictum | SH | 6 |  | 7 | 3 |  | 1 |  |
| Tribelos/Phaenopsectra sp. | GC | 7 | 1 |  |  |  |  |  |
| Tanytarsini | FC | 6 |  |  |  | 1 |  |  |
| Cladotanytarsus sp. | FC | 5 |  | 1 | 1 |  |  |  |
| Micropsectra/Tanytarsus sp. | FC | 7 | 4 | 3 | 3 | 3 | 6 |  |
| Micropsectra sp. | GC | 7 | 1 |  |  | 7 | 6 |  |
| Micropsectra dives gr. | GC | 7 |  | 2 |  | 3 |  |  |
| Rheotanytarsus distinctissimus gr. | FC | 5 |  | 1 | 1 | 5 | 2 |  |
| Rheotanytarsus exiguus gr. | FC | 6 | 2 | 2 | 1 |  |  |  |
| Sublettea coffmani | FC | 4 | 7 | 7 | 7 |  | 1 |  |
| Tanytarsus sp. | FC | 6 |  | 11 | 4 | 2 | 2 |  |
| Zavrelia/Stempellinella sp. | GC | 4 | 2 | 1 | 2 | 1 | 4 | 2 |
| Dixa sp. | FC | 1 |  |  |  |  |  | 1 |
| Atherix sp. | PR | 4 |  |  |  |  |  | 4 |
| Chelifera sp. | PR | 6 |  | 1 | 1 | 2 |  | 1 |
| Hemerodromia sp. | PR | 6 | 3 |  |  |  |  |  |
| Scathophagidae ? | SH | 6 |  |  |  |  | 1 |  |
| TOTALS |  |  | 97 | 103 | 103 | 91 | 94 | 96 |
| HBI |  |  | 4.67 | 4.92 | 4.84 | 3.86 | 4.64 | 3.55 |
| RICHNESS |  |  | 30 | 32 | 35 | 33 | 27 | 32 |
| EPT |  |  | 9 | 10 | 11 | 15 | 9 | 17 |
| EPT/CHIRONOMIDAE |  |  | 0.69 | 0.47 | 0.82 | 1.64 | 0.94 | 11.17 |
| SC/FC |  |  | 1.95 | 0.50 | 0.53 | 0.41 | 1.76 | 0.27 |
| FC/TOTAL |  |  | 0.21 | 0.35 | 0.33 | 0.19 | 0.18 | 0.16 |
| %DOM |  |  | 25% | 11% | 10% | 10% | 24% | 11% |
| NYSPMA |  |  | 63 | 63 | 68 | 67 | 66 | 74 |
| % similarity |  |  | 12 | 13 | 14 | 32 | 16 | R |

Table B2.	These results are from DWM’s 1997 benthos sampling  in the Hoosic River and two of its tributaries, all within Massachusetts.  Station HR02 and HR03 bracketed (downstream and upstream, respectively) the Hoosac Valley Water Quality District effluent discharge, Williamstown; and HR07D and HR07U bracketed (downstream and upstream, respectively) the Adams Wastewater Treatment plant effluent discharge.  Station PB00 was located on Peck’s Brook, Adams, upstream from the power line crossing; and BB00 was located on Bassett Brook, Cheshire, upstream from the gas pipeline crossing. The results from the East Branch Green River (GE01) are provided for reference.  The columns labeled FFG and TOLVAL show the functional group and tolerance value, respectively, designated for each taxon, as used in the analyses.

| TAXON/METRIC | FFG | TOLVAL | GE01 | HR02 | HR03 | HR07D | HR07U | PB00 | BB00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lumbricina | GC | 8 |  |  |  |  | 1 |  |  |
| Enchytraeidae | GC | 10 |  |  |  |  |  | 1 |  |
| Tubificidae immature w/o capilliform chaetae | GC | 10 |  | 1 |  |  |  |  |  |
| Nais alpina | GC | 8 | 1 |  |  |  |  |  |  |
| Nais communis | GC | 8 | 1 |  |  | 1 |  |  |  |
| Lumbriculus sp. | GC | 5 |  | 5 | 5 |  |  | 2 |  |
| Eclipidrilus sp. | GC | 5 |  |  |  | 1 |  |  |  |
| Hydracarina | PR | 6 | 2 | 2 | 2 | 4 | 6 |  | 1 |
| Baetidae | GC | 6 | 4 |  |  |  | 1 |  |  |
| Baetidae undet. 1 (cerci only) | GC | 6 | 4 |  |  |  | 3 |  |  |
| Baetidae undet. 3 (subeq. term.) | GC | 6 |  |  |  | 1 | 5 |  |  |
| Acentrella sp. | GC | 4 |  |  | 1 | 1 |  |  |  |
| Baetis sp. | GC | 6 |  | 4 |  |  |  |  | 6 |
| Baetis sp. 2 (short term.) | GC | 6 | 9 |  |  |  |  | 16 |  |
| Baetis sp. 3 (subeq. term.) | GC | 6 | 11 |  | 9 |  |  | 11 |  |
| Isonychia sp. | GC | 2 |  | 1 |  |  |  |  |  |
| Heptageniidae | SC | 3 |  |  | 1 |  |  |  | 1 |
| Epeorus (Iron) sp. | SC | 0 | 3 |  |  |  |  |  |  |
| Stenonema sp. | SC | 3 |  | 3 |  | 21 | 9 | 4 |  |
| Attenella sp. | GC | 1 |  |  |  |  |  | 7 |  |
| Drunella cornutella | GC | 0 | 7 |  |  |  |  |  |  |
| Serratella sp. | GC | 2 |  |  | 1 |  |  |  |  |
| Leptophlebiidae | GC | 4 | 2 |  |  |  |  |  | 1 |
| Pteronarcys sp. | SH | 0 | 3 |  |  |  |  | 1 | 1 |
| Tallaperla sp. | SH | 0 | 2 |  |  |  |  | 2 |  |
| Nemouridae | SH | 2 | 2 |  |  |  |  |  |  |
| Leuctridae/Capniidae | SH | 2 |  |  |  |  |  |  | 1 |
| Leuctra sp. | SH | 0 | 3 |  |  |  |  | 4 |  |
| Paragnetina sp. | PR | 1 |  |  | 1 |  |  |  |  |
| Diura sp. | PR | 2 | 1 |  |  |  |  |  |  |
| Isogenoides sp. | PR | 0 |  |  |  |  |  |  | 2 |
| Sweltsa sp. | PR | 0 | 7 |  |  |  |  | 9 | 5 |
| Sialis sp. | PR | 4 | 1 |  |  |  |  |  |  |
| Chimarra sp. | FC | 4 |  | 2 | 3 |  |  |  |  |
| Dolophilodes sp. | FC | 0 | 3 |  |  |  |  |  | 4 |
| Psychomyia sp. | GC | 2 |  | 2 | 4 | 4 | 7 |  |  |
| Polycentropodidae | FC | 6 | 1 |  |  |  |  |  |  |
| Polycentropus sp. | PR | 6 |  |  |  |  |  |  | 1 |
| Hydropsychidae | FC | 5 |  |  | 1 |  |  |  |  |
| Cheumatopsyche sp. | FC | 5 | 3 | 4 | 5 |  |  |  |  |
| Hydropsyche morosa gr. | FC | 6 | 1 | 24 | 8 | 26 | 18 |  | 5 |
| Rhyacophila sp. | PR | 1 |  |  |  |  | 1 | 7 |  |
| Rhyacophila cardina gr. | PR | 1 |  |  |  |  |  |  | 1 |
| Rhyacophila fuscula gr. | PR | 1 |  |  |  |  |  |  | 1 |
| Glossosomatidae | SC | 1 |  | 1 |  |  |  |  |  |
| Glossosoma sp. | SC | 0 |  |  |  |  |  | 1 |  |
| Protoptila sp. | SC | 1 |  |  | 1 |  | 2 |  |  |
| Leucotrichia sp. | SC | 6 |  | 1 | 1 |  |  |  |  |
| Lepidostoma sp. | SH | 1 | 1 |  |  |  |  |  | 3 |
| Goera sp. | SC | 3 |  |  |  |  |  | 1 |  |
| Ectopria sp. | SC | 5 | 1 |  |  |  |  |  |  |
| Psephenus herricki | SC | 4 |  |  |  | 1 | 5 |  |  |
| Optioservus sp. | SC | 4 |  | 17 | 19 | 3 | 21 | 6 | 4 |
| Optioservus ovalis (A) | SC | 4 |  |  |  | 2 | 1 |  |  |
| Optioservus trivittatus (A) | SC | 4 |  |  | 6 | 1 | 1 |  |  |
| Oulimnius latiusculus | SC | 2 |  |  |  | 2 |  | 8 |  |
| Promoresia sp. | SC | 2 |  |  |  |  |  | 1 |  |
| Stenelmis sp. | SC | 5 |  | 6 |  |  | 2 |  |  |
| Stenelmis bicarinata (A) | SC | 5 |  | 1 |  |  |  |  |  |
| Antocha sp. | GC | 3 |  | 2 | 8 | 2 | 2 |  |  |
| Dicranota sp. | PR | 3 | 2 |  |  |  |  |  |  |
| Hexatoma sp. | PR | 2 | 1 |  |  |  |  | 1 | 1 |
| Bezzia/Palpomyia sp. | PR | 6 | 2 |  |  |  |  | 1 |  |
| Prosimulium sp. | FC | 2 |  |  |  | 1 |  |  |  |
| Simulium sp. | FC | 5 |  |  |  |  |  | 5 | 3 |
| Simulium tuberosum complex | FC | 4 | 6 |  |  |  |  |  |  |
| Chironomidae | GC | 6 |  |  |  |  |  | 1 |  |
| Conchapelopia sp. | PR | 6 |  |  | 1 | 2 | 1 |  |  |
| Nilotanypus sp. | PR | 6 |  |  |  |  | 1 |  |  |
| Thienemannimyia sp. | PR | 6 |  | 1 |  |  |  |  |  |
| Diamesa sp. | GC | 5 |  | 1 |  |  |  |  |  |
| Pagastia sp. | GC | 1 |  |  |  |  | 4 |  | 1 |
| Potthastia gaedii gr. | GC | 2 |  | 1 | 1 | 3 |  |  |  |
| Orthocladiinae | GC | 5 |  |  |  |  |  | 1 | 1 |
| Brillia sp. | SH | 5 |  |  |  |  |  |  | 1 |
| Cardiocladius sp. | PR | 5 |  | 1 | 1 | 1 |  | 1 |  |
| Cardiocladius albiplumus | PR | 5 |  |  | 1 |  |  |  |  |
| Corynoneura sp. | GC | 4 |  |  |  |  |  |  | 1 |
| Cricotopus/Orthocladius sp. | GC | 7 |  | 1 | 1 |  |  |  |  |
| Cricotopus sp. | GC | 7 |  | 1 | 4 | 1 |  |  |  |
| Cricotopus bicinctus | GC | 7 |  |  |  | 1 |  |  |  |
| Cricotopus tremulus gr. | SH | 7 |  | 3 | 2 | 1 |  |  |  |
| Cricotopus trifascia gr. | SH | 6 |  |  | 1 | 1 | 1 |  |  |
| Eukiefferiella sp. | GC | 6 |  |  |  |  | 1 |  | 1 |
| Heterotrissocladius sp. | GC | 4 |  |  |  |  |  |  | 1 |
| Nanocladius parvulus gr. | GC | 7 |  |  | 1 |  |  |  |  |
| Orthocladius sp. | GC | 6 |  | 3 | 2 |  |  |  |  |
| Parametriocnemus sp. | GC | 5 | 3 |  |  |  |  | 5 | 11 |
| Parorthocladius sp. | GC | 5 |  |  |  |  |  | 1 |  |
| Rheocricotopus sp. | GC | 6 |  |  |  |  |  |  | 3 |
| Tvetenia sp. (P) | GC | 5 |  |  |  | 1 |  |  |  |
| Tvetenia vitracies gr. | GC | 5 |  | 2 | 3 | 1 |  |  |  |
| Microtendipes sp. | FC | 5 |  |  |  | 1 |  |  |  |
| Phaenopsectra sp. | SC | 7 |  | 1 |  |  |  |  |  |
| Polypedilum aviceps | SH | 4 | 1 |  |  | 6 | 2 |  | 33 |
| Polypedilum convictum | SH | 6 |  |  |  | 1 |  |  | 2 |
| Cladotanytarsus sp. | FC | 5 |  |  |  |  | 1 |  |  |
| Micropsectra/Tanytarsus sp. | FC | 7 |  |  |  | 1 |  |  |  |
| Micropsectra sp. | GC | 7 |  | 1 | 2 |  | 2 |  |  |
| Rheotanytarsus exiguus gr. | FC | 6 |  |  |  |  | 2 |  |  |
| Sublettea coffmani | FC | 4 |  | 8 | 2 | 1 | 2 |  |  |
| Tanytarsus sp. | FC | 6 |  | 1 |  |  |  |  | 4 |
| Zavrelia/Stempellinella sp. | GC | 4 | 2 |  |  |  |  |  |  |
| Dixa sp. | FC | 1 | 1 |  |  |  |  |  |  |
| Atherix sp. | PR | 4 | 4 |  |  | 1 | 1 |  |  |
| Empididae | PR | 6 |  |  |  |  | 1 |  |  |
| Chelifera sp. | PR | 6 | 1 |  |  |  |  |  |  |
| Hemerodromia sp. | PR | 6 |  | 1 | 2 | 4 | 3 |  |  |
| Oreogeton ? sp. | PR | 6 |  |  |  |  |  | 2 |  |
| TOTALS |  |  | 96 | 102 | 100 | 98 | 107 | 99 | 100 |
| HBI |  |  | 3.55 | 4.60 | 4.71 | 4.57 | 4.14 | 3.47 | 3.81 |
| RICHNESS |  |  | 32 | 28 | 26 | 28 | 26 | 23 | 27 |
| EPT |  |  | 17 | 9 | 11 | 5 | 7 | 11 | 13 |
| EPT/CHIRONOMIDAE |  |  | 11.17 | 1.68 | 1.64 | 2.41 | 2.71 | 7.00 | 0.54 |
| SC/FC |  |  | 0.27 | 0.77 | 1.47 | 1.00 | 1.78 | 4.20 | 0.31 |
| FC/TOTAL |  |  | 0.16 | 0.38 | 0.19 | 0.31 | 0.21 | 0.05 | 0.16 |
| %DOM |  |  | 11% | 24% | 19% | 27% | 20% | 16% | 33% |
| NYSPMA |  |  | 74 | 58 | 68 | 75 | 64 | 84 | 52 |
| % similarity |  |  | R | 10 | 15 | 7 | 9 | 38 | 26 |
| % similarity (paired brackets) |  |  |  | 57 | R-HR02-3 | 52 | R-HR07 |  |  |

Table B3.	These results are from 1997 Kinderhook Creek benthos sampling.  Two reaches were sampled in Hancock, MA bracketing the point on the creek where Jiminy Peak withdraws water.  KC01 was upstream from the withdrawal point, between Brodie Mountain Road and Bentley Brook.  KC02 was about 250 m downstream from Bentley Brook and the water withdrawal site.  The results from the East Branch Green River (GE01)—in the Hoosic River drainage—are provided for reference.  The columns labeled FFG and TOLVAL show the functional group and tolerance value, respectively, designated for each taxon, as used in the analyses.

| TAXON/METRIC | FFG | TOLVAL | GE01 | KC01 | KC02 |
| --- | --- | --- | --- | --- | --- |
| Nais alpina | GC | 8 | 1 |  | 1 |
| Nais communis | GC | 8 | 1 |  |  |
| Hydracarina | PR | 6 | 2 |  | 3 |
| Baetidae | GC | 6 | 4 | 3 | 3 |
| Baetidae undet. 1 (cerci only) | GC | 6 | 4 | 5 | 5 |
| Baetidae undet. 2 (short term.) | GC | 6 |  | 4 | 9 |
| Baetidae undet. 3 (subeq. term.) | GC | 6 |  | 6 | 16 |
| Baetis sp. 2 (short term.) | GC | 6 | 9 |  |  |
| Baetis sp. 3 (subeq. term.) | GC | 6 | 11 |  |  |
| Epeorus (Iron) sp. | SC | 0 | 3 |  |  |
| Drunella cornutella | GC | 0 | 7 |  |  |
| Leptophlebiidae | GC | 4 | 2 |  |  |
| Pteronarcys sp. | SH | 0 | 3 |  |  |
| Tallaperla sp. | SH | 0 | 2 |  |  |
| Nemouridae | SH | 2 | 2 |  |  |
| Leuctra sp. | SH | 0 | 3 | 3 | 2 |
| Diura sp. | PR | 2 | 1 | 1 |  |
| Haploperla brevis | PR | 1 |  | 1 | 1 |
| Sweltsa sp. | PR | 0 | 7 |  |  |
| Sialis sp. | PR | 4 | 1 |  |  |
| Dolophilodes sp. | FC | 0 | 3 | 1 |  |
| Polycentropodidae | FC | 6 | 1 |  |  |
| Cheumatopsyche sp. | FC | 5 | 3 |  | 1 |
| Hydropsyche morosa gr. | FC | 6 | 1 |  | 2 |
| Rhyacophila sp. | PR | 1 |  | 1 |  |
| Rhyacophila fuscula gr. | PR | 1 |  |  | 1 |
| Brachycentrus sp. | FC | 1 |  | 14 | 19 |
| Lepidostoma sp. | SH | 1 | 1 |  |  |
| Ectopria sp. | SC | 5 | 1 |  |  |
| Elmidae | SC | 5 |  | 1 |  |
| Antocha sp. | GC | 3 |  | 2 | 1 |
| Dicranota sp. | PR | 3 | 2 |  |  |
| Hexatoma sp. | PR | 2 | 1 | 1 | 3 |
| Bezzia/Palpomyia sp. | PR | 6 | 2 |  |  |
| Simulium tuberosum complex | FC | 4 | 6 | 6 | 6 |
| Tanypodinae | PR | 7 |  | 1 |  |
| Diamesa sp. | GC | 5 |  | 1 |  |
| Pagastia sp. | GC | 1 |  | 2 | 3 |
| Orthocladiinae | GC | 5 |  |  | 1 |
| Corynoneura sp. | GC | 4 |  | 1 |  |
| Cricotopus sp. | GC | 7 |  |  | 2 |
| Cricotopus bicinctus | GC | 7 |  |  | 1 |
| Cricotopus vierriensis | SH | 7 |  |  | 1 |
| Eukiefferiella brevicalcar gr. | GC | 4 |  | 1 |  |
| Eukiefferiella devonica gr. | GC | 4 |  | 1 |  |
| Eukiefferiella pseudomontana gr. | GC | 8 |  | 1 | 1 |
| Parametriocnemus sp. | GC | 5 | 3 | 1 | 2 |
| Tvetenia bavarica gr. | GC | 4 |  | 5 |  |
| Tvetenia vitracies gr. | GC | 5 |  |  |  |
| Chironominae | GC | 6 |  | 1 |  |
| Polypedilum sp. | SH | 5 |  | 1 |  |
| Polypedilum aviceps | SH | 4 | 1 | 1 | 3 |
| Tanytarsini | FC | 6 |  | 1 |  |
| Micropsectra sp. | GC | 7 |  | 30 | 5 |
| Rheotanytarsus distinctissimus gr. | FC | 5 |  |  | 1 |
| Sublettea coffmani | FC | 4 |  |  | 2 |
| Tanytarsus sp. | FC | 6 |  | 1 |  |
| Zavrelia/Stempellinella sp. | GC | 4 | 2 |  |  |
| Dixa sp. | FC | 1 | 1 |  |  |
| Atherix sp. | PR | 4 | 4 |  |  |
| Chelifera sp. | PR | 6 | 1 |  |  |
| TOTALS |  |  | 96 | 98 | 95 |
| HBI |  |  | 3.55 | 4.68 | 4.31 |
| RICHNESS |  |  | 32 | 25 | 23 |
| EPT |  |  | 17 | 9 | 9 |
| EPT/CHIRONOMIDAE |  |  | 11.17 | 0.80 | 2.68 |
| SC/FC |  |  | 0.27 | 0.04 | 0.00 |
| FC/TOTAL |  |  | 0.16 | 0.23 | 0.33 |
| % DOM |  |  | 11% | 31% | 20% |
| NYSPMA |  |  | 74 | 64 | 79 |
| % SIMILARITY (PAIRED BRACKETS) |  |  | -- | R-KC | 53 |

Intentionally left blank

# APPENDIX D - HOOSIC RIVER SUBBASIN FISH TOXICS MONITORING

### Background information

Hoosic River fish (white suckers Catostomus commersoni) were first sampled for PCBs as part of an intensive water quality/biological survey of the Hoosic River conducted during the summer of 1985. A total of six locations were sampled within the river. In addition, golden shiners Notemigonus crysoleucas, and brown bullhead Ictalurus nebulosus were sampled from Cheshire Reservoir.

Results of the 1985 fish work indicated that although PCBs were detected in seven of the twelve samples analyzed, concentrations were very low in all but two samples. PCB concentrations of 0.84 and 1.28 mg/Kg (ppm) were detected in two composite samples from the Hoosic River mainstem in North Adams downstream from the Sprague Electric Company (SEC). Although a concern, the concentrations were still well below the USFDA’s Action Level for PCBs (2.0 ppm).

In an effort to sample additional stations downstream of SEC as well as to sample species, which were more desirable by sportsfishermen and women, DEP re-sampled the Hoosic River during the summer of 1986. Seven stations were sampled, five of which were located downstream of SEC. Analytical variables included selected metals in addition to PCBs. Although the goal of sampling downstream of SEC was met, we collected only one trout from these stations.

PCBs were detected in most samples analyzed, however, were highest in brown trout. Concentrations in brown trout Salmo trutta ranged from 0.35 to 30.6 mg/Kg. The only sample, which exceeded the USFDA Action Level, was the one brown trout captured downstream of SEC.

In 1988, DEP with assistance from the United States Department of Fish, Wildlife and Environmental Law Enforcement, Massachusetts Division (DFWELE) again attempted to catch brown trout from the mainstem of the Hoosic River below North Adams. DEP and DFWELE were successful in capturing four brown trout from two locations. In addition, Al Les and members of Trout Unlimited collected additional brown trout using rod and reel fishing techniques. All fish came from the mainstem Hoosic River downstream of SEC. A total of 13 brown trout were collected and analyzed. PCB concentrations ranged from 1.16 – 19.4 mg/Kg and averaged 7.8 mg/Kg.  In light of the extremely elevated levels of PCBs the Massachusetts Department of Public Health (MDPH) issued a fish consumption advisory which recommended:

“1. People should refrain from eating brown trout from the Hoosic River caught below the channelized section in North Adams. 2. Consumption of all other fish species from the Hoosic River and its tributaries should be limited to two meals per month per person. 3. Pregnant women and nursing mothers should not eat fish from the Hoosic River and its tributaries in order to prevent exposure of developing fetuses and infants to PCBs.”

In light of the fish consumption advisory issued in 1989, the Green River, Hemlock Brook, Broad Brook and Tophet Brook (all tributaries too the Hoosic River) were sampled for metals and PCBs during late summer of 1989.  Brook trout Salvelinus fontinalis, and/or brown trout were collected from each of these streams and analyzed individually.  PCB concentrations ranged from below detection to 2.1 mg/Kg. Only one of twenty-four fish exceeded the USFDA Action Level of 2.0 mg/Kg. In fact, only one fish exceeded 1.0 mg/Kg.

In 1994, as a result of changes in the MDPH PCB advisory issuing criteria and “trigger level” (MDPH established a new trigger level of 1.0 mg/Kg), as well as the consideration of data from 4 major Hoosic River tributaries, the fish consumption advisory was updated and now recommends that  “People should refrain from eating all fish from the Hoosic River caught below the channelized section in North Adams”.

In an attempt to re-confirm the presence of high PCBs in the Hoosic River downstream of SEC, in 1997 DEP sampled three stations (two upstream of SEC on the North Branch Hoosic River and one downstream of the SEC on the Hoosic River mainstem.  Brown trout, brook trout, rainbow trout Oncorhynchus mykiss, and longnose suckers Catostomus catostomus, composites and individuals were analyzed for PCBs, organochlorine pesticides, and selected metals.  PCBs in fish samples (n=4) from below SEC ranged from 1.1 to 1.4 mg/Kg. PCBs were below detection in all samples (n=8) from the North Branch of the Hoosic River upstream of SEC.

### REFERENCES

MA DEP.  1997a.  Draft Quality Assurance Project Plan for 1997 Water, Fish Tissue, Habitat and Biological Studies in the Hoosic Basin, Worcester, MA. Attachments 1-14.

- Job Performance Report Project No. F-26-R-8. Re: 1976 Cold Water Fisheries Investigations, Survey and Inventory of Ponds and Streams in the Hoosic River. Department of Fisheries, Wildlife and Environmental Law Enforcement. MA.

- Johnson, A. S. 1982.  Memorandum to R. Isaac.  Re: Hoosic River Macroinvertebrate Study.  Massachusetts Division of Water Pollution Control, Technical Services Branch, Westborough, MA.

- Bode, R.  1986.  Memorandum to C. Callinan.  Re: 1985 Hoosic River Biomonitoring Results.  New York State Department of Environmental Conservation, Division of Water, Stream Biomonitoring Unit, NY.

- Maietta, R. J. 1987.  Memorandum to A.S. Johnson.  Re: 1986 Hoosic River Fish Toxics Monitoring.  Massachusetts Division of Water Pollution Control, Technical Services Branch.  Westborough, MA.

- Maietta, R. J. 1989.  Memorandum to A.S. Johnson.  Re: 1988 Hoosic River Fish Toxics Monitoring (BIO88-6).  Massachusetts Division of Water Pollution Control, Technical Services Branch. Westborough, MA.

- Maietta, R. J. 1990.  Memorandum to A.S. Johnson.  Re: Hoosic River Tributaries Fish Toxics Monitoring (BIO89-8).  Massachusetts Division of Water Pollution Control, Technical Services Branch. Westborough, MA.

- MA DPH. 1989.  Public Health Advisory:  Hoosic River Fish.  The Commonwealth of Massachusetts, Executive Office of Human Services.  Boston, MA.

- MA DPH. 1994.  Public Health Fish Consumption Advisory:  Hoosic River.  The Commonwealth of Massachusetts, Executive Office of Human Services.  Boston, MA.

- Weinberg, A.  (Alan.Weinberg@state.ma.us) 28 April 1992.  Request for biological monitoring of fish tissue samples in the Hoosic River Basin.  Email to R. Nuzzo (Robert.Nuzzo@state.ma.us).

- Nuzzo, R. M.  1993.  Technical Memorandum 91-3 to A. Weinberg  Re: WWSC-36: Hoosic River Bioassays--1991.  Massachusetts Division of Water Pollution Control.  Grafton, MA.

- Nuzzo, R. M.  1993 Hoosic River Bioassay PCB Data/Chart/Table. Unpublished data. Grafton, MA.

- Nuss, J.M. 1996. Letter to C.G. Wanat.  Re: American Annuity Group, Inc. Brown Street Facility, North Adams, Massachusetts – Results of Biota Sampling and Analysis.  Project #: 082.28#2.  Blasland, Bouck and Lee, Inc.  Syracuse, NY.

- DEP DWM.  1997 WMA Database Printout Hudson River Basin (Database manager: Levangie, current).  Boston, MA.

- DEP DWM.  1997 NPDES permit file review of Hoosic River Basin discharges.  Unpublished data. Worcester, MA.