# MASSACHUSETTS COASTAL CONDITION ASSESSMENT

# FIELD OPERATIONS MANUAL

Field Operations Manual for the Massachusetts Probabilistic Monitoring and Assessment Program (MAP2), Coastal Waters

Document prepared by Massachusetts Bays National Estuary Partnership and Massachusetts Department of Environmental Protection

CN 528.0

April 2020

| Prepared by: |  | Date: |  |
| --- | --- | --- | --- |
| Prassede Vella James Meek | MassBays National Estuary Partnership, MA Office of Coastal Zone Management Massachusetts Department of Environmental Protection |  | April 2020 |
| Approved by: |  | Date: |  |
| Approved by: |  | Date: |  |
| Approved by: |  | Date: |  |

* see pdf version for valid signatures

LIST OF REVISIONS

| Rev. # | Date | Description of Revision(s) | Page #s |
| --- | --- | --- | --- |
| 0 | April 2020 | Original document CN 528.0 |  |

The Massachusetts Probabilistic Monitoring & Assessment Program (MAP2) is a component of the Massachusetts Department of Environmental Protection (MassDEP) water monitoring strategy and was initialized in 2011 to assist in fulfilling the requirements of the Clean Water Act (CWA) Section 305(b). The goal of MAP2 is to provide a comprehensive assessment of the condition of “waters” in Massachusetts through the implementation of probabilistic sampling designs.  With completion of the wadeable rivers and streams (2010-2015) and lakes (2016-2018) probabilistic surveys, MAP2 will focus on coastal waters from 2020 to 2023.

The Massachusetts Bays National Estuary Partnership (MassBays) is dedicated to protecting, restoring, and enhancing the estuarine ecosystems of Ipswich Bay, Massachusetts Bay and Cape Cod Bay.  As part of its mission, and as mandated by Section 320 of the CWA, MassBays monitors the status and trends of water quality conditions in estuarine waters of those Bays.  Considering the shared needs and responsibilities between the two programs, MassDEP and MassBays formed a partnership to conduct a probabilistic Massachusetts Coastal Condition Assessment (MCCA) that meets the needs and responsibilities of both programs.

This document contains an overview of the process involved in evaluating the candidate coastal water sites and selecting appropriate alternate sites when necessary.  It is adapted from the guidelines developed and followed in the 2015 National Coastal Condition Assessment conducted by USEPA (USEPA, 2014).  Methods described in this document are to be used specifically in work relating to probabilistic coastal waters.

Table of Contents

Roles and Contact Information	6

1. INTRODUCTION	7

2. BACKGROUND	7

2.1 Survey Design	7

2.2 Site Evaluation	7

2.3 Describing the MCCA Indicators	7

2.3.1 In Situ Water Column Measurements	7

2.3.2 Water Chemistry and Associated Measurements	8

2.3.3 Sediment Assessment	8

2.3.4 Benthic Macroinvertebrates Assemblage	8

2.3.5 Submerged Aquatic Vegetation: Eelgrass	8

2.4 Recording Data and Other Information	8

2.5 Data Management	9

2.6 Health and Safety	9

2.6.1 Safety Equipment	9

2.6.2 Safety Guidelines for Field Operations	10

3. PREPARING FOR SAMPLING	10

3.1 INSTRUMENT CHECKS AND CALIBRATION	11

3.2 Equipment and Supply Preparation	11

3.3 Safety and Spill Prevention	12

4. INITIAL SITE PROCEDURES	12

4.1 Site Verification	12

4.1.1 Site Verification Procedures	12

4.1.2 Site Relocation	13

4.1.3 Site Characteristics	13

4.2 Sample Collection	13

4.2.1 Sediment Samples	14

5. WATER QUALITY MEASUREMENTS	14

5.1 Sampling Procedure – Water Column Transparency (Secchi Depth)	15

5.2 Sampling Procedure – Multi-Parameter Sonde	15

5.2.1 Calibration	15

5.2.2 Dissolved Oxygen	16

5.2.3 pH (6.3.3)	17

5.2.4 Salinity (6.3.4)	17

5.2. Turbidity	17

5.2.6 Temperature (6.3.5)	17

5.3 Sampling Procedure – Dissolved Oxygen, pH, Temperature, Turbidity & Salinity	17

5.4 Photosynthetically Active Radiation (PAR) Meter	18

5.4.1 Sampling Procedure—Light Attenuation	18

6. WATER CHEMISTRY, NUTRIENTS, CHLOROPHYLL-A, and PHYTOPLANKTON SAMPLE COLLECTION AND PRESERVATION	18

6.1 Sampling Procedure for Nutrients and Chlorophyll-a	19

6.2	Sampling Procedure for Phytoplankton	20

7. SEDIMENT COLLECTION	20

7.1 Sampling Procedure	21

7.2 Processing Procedure – Benthic Macroinvertebrate [Bent]Composition and Abundance (12.4)	22

7.3 Processing Procedure – Sediment Composition, Chemistry, and Toxicity	23

8. SUBMERGED AQUATIC VEGETAION SAMPLING: EELGRASS	25

9. FINAL SITE ACTIVITIES (14.0)	27

9.1 General Site Assessment	27

9.1.1 Shoreline Activities and Disturbances	27

9.1.2 Site Characteristics	27

9.2 Processing the Chlorophyll-a & Dissolved Nutrients Indicators	27

9.2.1 Processing Procedure	28

9.3 Post-Measurement Calibration Check of Multi-Parameter Sonde	29

9.4 Post-Measurement Check of Eelgrass Sampling Equipment	30

9.5 Field Data & Tracking Form Review	30

9.5 Sample Packaging and Label Review	30

9.6 Sample Shipment and Tracking Form Submittal	31

9.7 Data Submittal	31

10. FIELD QUALITY CONTROL	31

10.1 Repeat Sampling	31

10.2 Infauna QA/QC Field Evaluation and Audit Visits	31

10.3 Field Evaluation and Audit Visits	31

11. LITERATURE CITED	33

APPENDIX A: EQUIPMENT AND SUPPLIES	34

APPENDIX B: FIELD FORMS, LABELS AND TRACKING FORMS	36

List of Figures

Figure 7.1. 	22

Figure 8.1. 	28

Figure 8.2. 	28

Figure 9.1. 	32

List of Tables

Table 3.1. Stock solutions, uses and methods for preparation	12

Table 5.1 Equipment and Supplies: Transparency, DO, pH, salinity, temperature, turbidity, and light attenuation	14

Table 6.1 Equipment and Supplies: Water chemistry and clholophyll-a collection	19

Table 7.1 Equipment and Supplies: Sediment collection	20

Table 8.1 Equipment and Supplies: Eelgrass monitoring ……………….………………………………………………………24

Table 9.1 Equipment and Supplies: Chlorophyll a and dissolved nutrients processing	27

# Roles and Contact Information

| Name/Role | Contact | Organization |
| --- | --- | --- |
| Prassede Vella MCCA Project Co-Lead | Prassede.Vella@mass.gov 617-626-1217 | Massachusetts Office of Coastal Zone Management Massachusetts Bays National Estuary Partnership Boston, MA |
| James Meek MCCA Project Co-Lead | James.Meek@mass.gov 508-767-2863 | Massachusetts Department of Environmental Protection Watershed Planning Program Worcester, MA |

# 1. Introduction

The goal of the Massachusetts Coastal Condition Assessment (MCCA) is to monitor and assess all coastal waters of the Commonwealth of Massachusetts.  The primary objectives of the MCCA include:

- Determine the percent of coastal waters that are supporting aquatic life use;

- Determine the key stressors impairing aquatic life use in coastal waters.

The MCCA is designed to be completed during the period of June through the end of August between 2020 and 2023. Field crews collect a variety of measurements and samples from 90 preselected sampling sites that are located at predetermined coordinates. 15 sites will be visited monthly during 2020 (pilot year) while 25 sites will be sampled monthly during each of the succeeding years – 2021, 2022, and 2023.

From each site, in situ water quality conditions will be measured. Water samples will be collected for analyses of chlorophyll-a, nutrients, and phytoplankton. Sediment samples will be collected to describe sediment characteristics and measure sediment chemistry and toxicity. Benthic macroinvertebrates will be collected to assess the composition and health of benthic communities in estuaries and coastal areas.

# 2. Background

## 2.1 Survey Design

The MCCA utilizes the Generalized Random Tessellation Stratified (GRTS) design strategy developed principally by EPA, National Health and Environmental Effects Research Laboratory, Western Ecology Division (EPA-NHEERL-WED) in Corvallis.  The basic survey design specifics for the MCCA are:

- The target population is as all coastal waters within MassDEP and MassBays estuarine assessment units and a nearshore boundary defined by a maximum distance from the Massachusetts shoreline of 3 miles and a maximum depth of 10 meters.

- Stratified into three geographic regions or strata to improve sampling logistics.  One region will be targeted and sampled each year from 2021 to 2023.  A pilot year of sampling will be conducted in 2020 in multiple strata using a small subset of the selected primary sites to test sampling methodology.

- Unequal probability categories were created based on the area of each polygon/estuary segment to ensure the selection of sites in smaller polygon/estuary segments.

- The target sample size is 90 sites with 30 sites in each region or stratum (primary sites).  In addition, 60 oversample sites were selected in each stratum to replace rejected primary sites.

Additional details of the survey design, selection of target population and sample frame are described in Appendix C.

## 2.2 Site Evaluation

Site evaluation is the process by which the points or sites selected using the survey design detailed in the previous section are evaluated to determine if they meet the criteria for inclusion into the survey.  Each point selected as a sample site is designated the “X-site” and represents the point at which sample collection should take place.  Here are the major steps in site evaluations.

- Review the Target Population Definition and Survey Design.

- Interpret the Site Evaluation Spreadsheet.

- Conduct a Desktop Evaluation to locate and verify that the X-site is Part of the Target Population and whether it is sampleable.

- Seek permission to sample, if necessary.

- Conduct Final Site Verification at the Location.

- Complete Site Evaluation/Verification Forms.

Details of the site evaluation process are described in CN527.0 Site Evaluation Guidelines for the Massachusetts Probabilistic Monitoring and Assessment Program (MAP2), Coastal Waters (MassDEP 2019)

## 2.3 Describing the MCCA Indicators

### 2.3.1 In Situ Water Column Measurements

Hydrographic Profile: Measurements for dissolved oxygen (DO), pH, salinity, turbidity and temperature will be taken with a calibrated water quality meter or multi-parameter sonde at each site. Measurements will be taken at specific depth intervals within 37 meters of the X-site. The specific location of the profile (and subsequently the area where several samples are collected) is referred to as the Y-location. This information will be used to detect extremes in condition that might indicate impairment.

Light Attenuation: A Photosynthetically Active Radiation (PAR) meter will be used to obtain a vertical profile of light in order to calculate the light attenuation coefficient at each station. PAR measurements are taken at the same depths as other water column indicators.

Secchi Disk Transparency: A Secchi disk is a commonly used black and white patterned disk used to measure the clarity of water within a visible distance.

### 2.3.2 Water Chemistry and Associated Measurements

Water chemistry measurements will be used to determine nutrient enrichment, as well as classification of trophic status. Parameters measured include total and dissolved nitrogen and phosphorus.

Chlorophyll-a: Measurement of Chlorophyll-a is used to determine algal biomass in the water.

Dissolved Nutrients: A portion of the filtrate produced from the processing of the chlorophyll-a sample will be collected in the field and processed in the laboratory for dissolved nutrients.

Phytoplankton Assemblage: Phytoplankton are plant microorganisms and the primary source of energy. Phytoplankton are highly sensitive to environmental changes in ecosystems (e.g., turbidity and nutrient enrichment).

### 2.3.3 Sediment Assessment

Sediment grab samples will be obtained to measure sediment composition (e.g., grain size and percent moisture, organic content), toxicity, and contaminant chemistry in order to determine sediment condition.

### 2.3.4 Benthic Macroinvertebrates Assemblage

Benthic macroinvertebrates include a variety of organisms such as aquatic worms, mollusks, and crustaceans. Populations in the benthic assemblage respond to a wide array of stressors in different ways so that it is often possible to determine the type of stress that has affected a macroinvertebrate assemblage (Klemm et al., 1990). Because many macroinvertebrates have relatively long life-cycles of a year or more and are relatively immobile, the structure of the macroinvertebrate assemblage is a response to exposure of present and/or past conditions. The benthic macroinvertebrate data will serve as the basis for assessing aquatic community health.

### 2.3.5 Submerged Aquatic Vegetation: Eelgrass

Where conditions are suitable for eelgrass (Zostera marina) to grow, its presence/absence serves as indicator of water quality conditions. Presence/absence of eelgrass as well as percent cover data will provide additional information for assessing the status and health of aquatic communities.

## 2.4 Recording Data and Other Information

Field data and sample information must be recorded completely, legibly, accurately, and consistently. Field crews will record field data in field forms made available for this project. Paper chain of custody forms must be included in every cooler/box which contains samples being shipped to the labs and must also be submitted to MassDEP and MassBays electronically. All samples need to be identified and tracked and associated information for each sample must be recorded.

To assist with sample tracking, sample ID numbers will be provided by MassDEP/MassBays for use on all field sheets, bottle labels and contract lab chain of custody forms.  A MCCA sample ID is created by taking the last three digits of the site ID (e.g. MAP2E-023) and adding a unique 4-digit number on the end (e.g. 023-0001).  It is recommended using sequential numbers for the 4-digit suffix to ensure the sample ID is unique.  This sample ID will be recorded on the field sheets, bottle labels and chain of custody forms.

All bottle label must include site ID, sample ID, indicator code (e.g. CHEM, SEDC) and date of collection.  Additional information the field crew consider useful can be added to the bottle labels to assist with sample management during collection.

## 2.5 Data Management

Field crew will be given access to a shared Google Drive. This site will be a resource for field crews to access important MCCA documentation, submit data forms and chain of custody forms as well as for facilitating document transfer. Lab forms will also be provided to the Project Coordinator.

## 2.6 Health and Safety

Sample collection and analysis can pose significant risks to personal safety and health. This section describes recommended training, communications, safety considerations, safety equipment and facilities, and safety guidelines for field operations.

All field personnel should successfully complete the necessary safety courses and follow all safety policies and procedures. Crew members should be familiar with the hazards involved with sampling equipment and establish appropriate safety practices prior to their use. All equipment should be in safe working condition. Personnel must consider and prepare for hazards associated with the operation of motor vehicles, boats, winches, tools, and other incidental equipment. Boat operators must meet Massachusetts requirements for boat operation and be familiar with U.S. Coast Guard rules and regulations for safe boating. While on the water, all crew members must wear Personal Flotation Devices (PFD). All boats with motors must be equipped with fire extinguishers, boat horns, PFDs, and flares or other U.S. Coast Guard approved signaling devices.

Crews should develop an Emergency Communications Plan. This plan will include contacts for police, fire departments, emergency medical services, hospitals and search and rescue personnel.

All field personnel must be fully aware of all lines of communication and able to initiate emergency communications if needed. Field personnel must be able to swim. A PFD and suitable footwear must always be worn while on board a boat.

### 2.6.1 Safety Equipment

Crews may face many hazards when working in coastal areas. Broken glass or other sharp objects may be embedded in the substrate. Infectious agents and toxic substances may be present in the water or sediment. Dangerous weather may approach with little warning. Vessels can lose power and navigation.

Crews must stock appropriate safety apparel such as gloves, foul weather gear, safety glasses, etc., and use them when necessary. All vessels must have first aid kits, fire extinguishers and blankets available in the field, and crew members must be trained in how to use them. All crews must carry cellular or satellite telephones and all crew members must be proficient in how to use them. Crews must carry supplies such as clean water, anti-bacterial soap, and ethyl alcohol for cleaning exposed body parts that may have been contaminated by pollutants in the water.

### 2.6.2 Safety Guidelines for Field Operations

Field crew members must become familiar with the health hazards associated with collecting, preserving, and storing field samples. All surface waters and sediments are considered potential health hazards due to the potential presence of toxic substances or pathogens, and chemical fixing and/or preserving agents are often comprised of hazardous materials. In addition, chemical wastes can be flammable, explosive, toxic, caustic, or chemically reactive. Therefore, all chemical wastes must be discarded according to standardized health and hazards procedures (e.g., National Institute for Occupational Safety and Health [1981]; U.S. EPA [1986]).

- At least two crew members must be present during all sample collection activities, and no one should be left alone while out on the water.

- Use caution and wear a suitable PFD.

- Exposure to water and sediments should be minimized as much as possible. Use gloves if necessary, and clean exposed body parts as soon as possible after contact.

- All electrical equipment must bear the approval seal of Underwriters Laboratories and must be properly grounded to protect against electric shock.

- Use appropriate protective equipment (e.g. gloves, safety glasses) when handling and using hazardous chemicals.

- Field personnel should be familiar with the symptoms of hypothermia and know what to do in case symptoms occur. Hypothermia can kill a person at temperatures much above freezing (up to 10oC or 50oF) if he or she is exposed to wind or becomes wet. Immersion in the cool waters experienced during the summer along most of the marine coasts can also rapidly result in hypothermia.

- Field personnel should be familiar with the symptoms of heat/sun stroke and be prepared to move a suffering individual into cooler surroundings and hydrate immediately.

- Handle and dispose of chemical wastes properly. Do not dispose of any chemicals in the field.

# 3. Preparing for Sampling

This section presents a general overview of the field activities and guidelines for field operations, recording data and labeling samples. MCCA field methods are designed to be completed in no more than one field day per site.  Depending on the time needed for sampling and travel, crews may require an additional day to complete pre-departure and post-sampling activities (e.g., cleaning equipment, repairing gear, shipping or delivering samples, and traveling to the next site). Conversely, some sites may be in relatively close proximity to each other, allowing multiple sites to be sampled in a single day.

A field crew typically consists of three to four people. However, a minimum of two people may be able to properly execute sampling activities. To ensure safety, at least two people are always required in a boat when conducting field work for the MCCA. In order to organize field activities efficiently, each field crew should define roles and responsibilities for each crew member prior to beginning field activities. One crew member is primarily responsible for boat operation and navigation. Additional crew members assist with sample collection/processing and/or provide logistical support.

Crew members responsible for collecting water chemistry and sediment grabs should not apply sunscreen or other chemical contaminants until after each of these samples is collected to avoid compromising the integrity of the sample (or implement measures to reduce contamination by such chemicals if applied such as washing, wearing long gloves, etc.).

## 3.1 Instrument Checks and Calibration

Instruments must be tested and calibrated prior to sampling. Site locations will be verified using a global positioning system (GPS) receiver. Measurements will be collected using a Photosynthetically Active Radiation (PAR) meter and a multi-parameter unit (sonde) for measuring DO, pH, temperature, salinity and turbidity. Access to backup instruments is recommended if any instruments fail the manufacturer performance tests or calibrations. Prior to departure, the following checks and calibrations must be performed:

- If using a hand-held GPS unit, turn on the GPS receiver and check the batteries. Replace batteries immediately if a battery warning is displayed. Boat-mounted GPS units run off the boat electrical system.

- Test and calibrate the multi-parameter meter following the manufacturer's calibration and maintenance procedures. Once each week, verify that the meter is functioning properly by performing manufacturer recommended internal diagnostic readouts (e.g. pH millivolts, cell constants, and/or other diagnostic readings). Records of these checks should be saved in a logbook or other documentation.

- Ensure that the PAR meter’s handheld display unit has fresh batteries, that the unit is functioning properly, and that the correct calibration factors are entered for each probe.

## 3.2 Equipment and Supply Preparation

Appendix A provides a list of equipment and supplies needed for field work. Crews should check the inventory of forms, supplies, and equipment prior to departure. Other preparatory tasks include:

- Obtain sufficient wet and dry ice for sample preservation and storage.

- Pack meters, probes, and sampling gear, taking care to do so in a way that minimizes physical shock and vibration during transport.

- Pack stock solutions as described in Table 3.1 below. Follow the regulations of the Occupational Safety and Health Administration (OSHA).

- Paper field forms, chain of custody forms and labels sufficient to cover the day’s sampling must be packed.

Table 3.1 Stock solutions, uses & methods for preparation

| Solution | Use | Preparation |
| --- | --- | --- |
| Bleach (1-10%) | Clean gear and inside of boat | Add 10 - 100 mL bleach to 1 L distilled water. |
| Quality Check Solution for multi-parameter sonde | Weekly check of meter calibration In place of weekly internal meter checks | No preparation needed (if purchased as ready-to-use solution) |
| Buffered Formalin | Preserve benthic samples | Add 8 tablespoons Borax to 2 gallons 100% Formalin (37% formaldehyde) solution. FOR USE AT ALL SITES: Add ¼ teaspoon Rose Bengal crystals to above solution. |
| Utermöhl's solution | Preserve phytoplankton samples | 100 g potassium iodide, 50 g iodine, and 50 g sodium acetate each are dissolved incrementally in distilled water to a final volume of 1 L. (Guillard 1973) |

## 3.3 Safety and Spill Prevention

- All preservatives (e.g., formalin solution, isopropyl alcohol) should be mixed in a safe place ideally in the lab or prior to leaving the dock. When using on the boat, they should be stored in a safe place and handled where there is adequate ventilation.

- Preservatives should only be handled when wearing gloves and eye protection.

- Preservatives will be poured into sample jars held above a bucket, pan, or bowl so that if some preservative is spilled, it can be captured in the container below.

- Any spills on the deck, table, or other surface will be wiped up immediately with a rag or paper towel and will be placed in the designated covered bucket.

- Preservative should never be poured down any drain or into the ocean.

# 4. Initial Site Procedures

Upon arriving at the site, the field crew must confirm that it is the correct site and determine if the site meets the criteria for sampling and data collection activities. The crew verifies site access, safety, and general conditions to determine if the site can be sampled within the swing of the anchored boat.

## 4.1 Site Verification

### 4.1.1 Site Verification Procedures

- Navigate the sampling vessel as close as possible to the target X-site using GPS (you must be no more than 0.02 nautical miles (NM) or 37 meters from the target X-site). Sampling may start when the sampling vessel is within 37 meters of the X-site. This distance provides the desired level of precision which is approximately equal to that of the GPS receiver without differential fix correction. This allowable deviation distance accounts for typical “anchor swing” of the sampling vessel.

- Once the anchor has been set and the vessel is essentially stationary, verify that the X-site is still within 0.02 NM or 37 meters. This location (where sampling will begin) is referred to as the Y-location. Determine if the site is sampleable.

- If not sampleable, proceed to Section 4.1.2

- If sampleable, record the time of arrival to the Y-location on the Site Verification Form.

- Record the coordinates of the Y-location on the Site Verification Form in decimal degrees in the NAD83 datum.

- Measure and record the water depth at the Y-location on the Site Verification Form.

### 4.1.2 Site Relocation

Every attempt should be made to sample within a 0.02 NM (~37m) radius of the X-site. If the proposed initial sampling location is not sampleable, then relocate using the following guidelines:

- Choose a specific compass heading (e.g., north, south, east, west) and look for a sampleable area within 37 m from the X-site.

- If no suitable sampling location is found along the first chosen heading, return to the X-site and follow a new heading until an acceptable sampling location is found.

- If after a sufficient amount of effort is expended and no suitable sampling location is found, then the determination may be made that the site is non-sampleable. Indicate on the Site Verification Form. No further sampling activities are conducted.

- Replace the rejected site with the next oversample site on the stratum site list. Exception: In the pilot year (2020) replace the rejected site with another primary site in the same stratum.  Notify the Project Co-Leads.

### 4.1.3 Site Characteristics

- Record the general habitat type and the dominant bottom type present at the sampling site. At many sites, it may not be possible to record the bottom type until after the sediment collections are performed.

- Record the presence and type of debris (if any), submerged aquatic vegetation (SAV) present, and/or macroalgae present in the area.

- Make any general comments about the site that may be important during the data review portion of the assessment or any unusual characteristics about the site, including weather conditions.

- On the back side of the Site Verification Form, draw a simple sketch of the area.

- Include the relative locations of the shoreline, launch point, X-site, Y-location, and, if different from the Y-location and sediment collection locations.  Include any other specific attributes of the site that may be important during data analysis.

- A printed or copied section of a map with the pertinent information may be submitted in place of the scene sketch. 7. Record the name of all crew members and indicate their primary duties.

- Record the name of all crew members and indicate their primary duties.

## 4.2 Sample Collection

In circumstances when site conditions limit full completion of the standard sampling protocol, crews should prioritize sample collection and follow a “checklist” for determining the order of sample completion:

- Measure in situ water parameters and collect all water samples.

- Collect benthic grab samples. Any size sediment grab is acceptable as long as it meets the definition of a “successful benthic grab” (see Section 7.3).

- Collect sediment composite material of sand-sized sediment grain or smaller (preferred size). If an acceptable sediment grab cannot be obtained at the Y-location or within a 37 m radius around the X-site, move to a secondary sediment collection area following the procedures in Section 4.2.1 below. Flag and note the reason for limited/missing sediment samples.

- Observe presence/absence of eelgrass (Zostera marina) at all sites. This involves deploying a specially designed equipment that incorporates a quadrat and camera to observe presence/absence of eelgrass in the area.

All water, benthos, and sediment samples are expected to be collected at the same location (the Y-location), which is as close to the X-site as possible (within the 37-meter radius around the X-site). However, circumstances may require the field crew to relocate to a secondary location to collect an acceptable sediment grab. In this case, in situ measurements and water collections do not need to be resampled. Guidelines for relocating to a secondary sample collection zone are covered in the sections below.

### 4.2.1 Sediment Samples

- If an acceptable sediment grab cannot be obtained at the Y-location where water samples were collected, move the vessel within the 37 m radius margin (of the X-site) and try to obtain the sediment sample. Use the site relocation method described previously.

- In cases where sediment sampling cannot be successfully conducted within 37m of the X-site, grabs may be taken in a secondary sediment collection zone (e.g., > 37 m radius but within a 100 m radius (~0.05 NM) of the X-site) without re-collecting the water samples.

- If a suitable location to collect sediment samples has not been found after a minimum of three collection attempts inside each of the acceptable relocation radii, sediment sampling is considered “complete” for the site. All appropriate field form notations and explanatory comments on the Sediment Sample Collection Form must be completed.

# 5. Water Quality Measurements

This section describes the procedures and methods for the field collection and analysis of the water quality indicators (in situ measurements, water column transparency, and light attenuation).

Crews obtain a hydrographic profile at each site (at the Y-location) by measuring DO, pH, salinity (marine sites), and temperature using a multi-parameter water quality meter (or sonde). They also assess water column transparency using a Secchi disk and light attenuation using a PAR meter. The protocol requires measurements at the prescribed depths as the probe/sensor is both lowered and retrieved, starting just below the surface, progressing down to 0.5 m from the bottom, and returning to just below the surface. Crews record in situ measurements on the Water Sample Collection Form.

Table 5.1 Equipment & supplies: transparency, DO, pH, salinity, temperature, turbidity & light attenuation

| For taking measurements and calibrating the water quality meter | multi-parameter water quality meter with DO, pH, salinity, temperature, and turbidity probes. extra batteries de-ionized water calibration cups and standards barometer to use for calibration thermometer Secchi disk (20 cm diameter, weighted) & 100' line with clip (marked in 0.5 m intervals) PAR meter |
| --- | --- |
| For recording measurements | Field Measurement form pencils |

## 5.1 Sampling Procedure – Water Column Transparency (Secchi Depth)

A Secchi disk is a 20-cm black and white disk suspended from a non-stretch line that is marked in 0.5 m intervals. Field crews use a Secchi disk to measure water column to nearest 0.1m transparency at every site (at the Y-location). Below are step-by-step procedures for measuring water column transparency.

- Slowly lower the Secchi disk until it is no longer visible. In the “DISAPPEARS” column of the Water Sample Collection Form, record the depth where the marking on the line meets the water level. Interpolate between the 0.5m markings on the rope to the nearest 0.1m.

- If the disk hits the bottom before disappearing, indicate on the data sheet under “Clear to Bottom?”

- Slowly raise the Secchi disk until it just becomes visible and record the depth in the “REAPPEARS” column. Interpolate between the 0.5m markings on the rope to the nearest 0.1m.

- Repeat steps 1-3 two more times, recording both disappearance and reappearance depths each time.

- Repeat the entire process if any one disappearance or reappearance measurement differs from the others by more than 0.5 m.

## 5.2 Sampling Procedure – Multi-Parameter Sonde

### 5.2.1 Calibration

Crews calibrate the DO, pH, turbidity, and salinity meter functions of the multi-parameter water quality meter (or sonde) each sampling day. Crews record the manufacturer and model number of the instruments in the “Calibration Information” section of the Calibration Log form.

- The pH probe should be calibrated according to the manufacturer’s instructions and laboratory policies, using at least a 2-point calibration method (typically pH of 7 and 10 for 2-point calibration and pH of 4, 7, and 10 for 3-point calibration). Any pH standards used must reference NIST Standard Reference Material (SRM) certifications to be used in the calibration of the pH probe. This applies for calibrations done both pre-sampling and post-sampling.

  - The calibration buffers must be accurate to 0.02 pH units or better.

  - The calibration buffers should be replaced with fresh solutions every 3-4 days or sooner if contamination happens.

- The salinity probe should be calibrated according to the manufacturer’s specifications and laboratory policies, using a commercially supplied, traceable conductivity standard.

- pH and salinity calibration should be checked after daily measurements are complete to document potential meter drift throughout the day.

- Calibrate the turbidity sensor prior to each sampling event in accordance with the manufacturer’s instructions.

- For instruments that are factory calibrated and checked (e.g. Sea-Bird Electronics meters, etc.), crews must ensure that factory-certified diagnostics have been completed according to manufacturer specifications (preferably conducted immediately prior to the sampling season) and provide documentation copies during assistance visits. Meters such as these do not require the daily calibration steps or the weekly diagnostic/Quality Check Solution (QCS) checks.

- Once each week, crews must verify that the multi-parameter meter is functioning properly by performing manufacturer recommended internal diagnostic checks. Results of these checks must be recorded in a logbook or other documentation and saved for potential review.

- Crews using a commercially purchased pH QCS for the weekly quality checks should follow the guidelines below:

  - The pH QCS containers should be labeled with expected values and preparation dates.

  - The pH of the QCS should approximate the pH expected at sampling sites.

  - Crews should have centrally located bulk solutions to replenish allotments needed for quality checks every 3-4 days or sooner if the crew suspects it has become contaminated.

- A commercially purchased primary salinity standard is used as the QCS for weekly quality checks of salinity.

  - A secondary salinity standard that is referenced against a certified standard may also be used. If a secondary standard is used, then preparation and certification test procedures and results must be logged in a QA notebook. The standard should be representative of the conditions expected in the field (~0.5-35 ppt for marine waters). The salinity calibration standard and QCS containers must be labeled with expected values and preparation dates.

  - The standards should be replaced with fresh solutions every 3-4 days or sooner if contamination may have happened.

- At least once per sampling season the temperature sensor should be calibrated NIST-traceable thermometer.

Once a week, the multi-parameter sonde must be checked using manufacturer recommended internal diagnostic checks (cell constants, millivolt output, or other readings) or against the QCS that they provide. In addition to recording the expected values and results, record the QCS date prepared in the Calibration Log form. Report any calibration or QC irregularities as described above.

Specific information about calibrating each probe function is presented below.

### 5.2.2 Dissolved Oxygen

The DO probe is calibrated in the field against an atmospheric standard (i.e. ambient air saturated with water or water saturated with air) according to manufacturer’s specifications prior to launching the boat. In addition, follow any of the manufacturer’s recommendations for periodic comparisons with internal quality checks (cell constants, millivolt output, or other readings), or a DO chemical analysis (e.g., Winkler titration) to check accuracy and linearity. Record results and report irregularities as described above.

### 5.2.3 pH

The pH probe is calibrated in accordance with the manufacturer’s instructions and with the field crew organization’s existing Standard Operating Procedure (SOP). After all in situ measurements have been completed for the sampling day, crews perform a post-measurement calibration check of the pH meter. Crews will record the Calibration Standard Value pH (expected) and the post-sampling measurement in the Calibration Log form. If significant drift (outside of manufacturer’s specification) is detected, it may indicate that the meter needs service.

### 5.2.4 Salinity

Prior to sampling the salinity probe is calibrated in accordance with the manufacturer’s instructions. After the sampling day is complete, measure the salinity of the calibration standard (expected) used earlier in the day to calibrate the instrument to conduct a post-measurement QC check... Record the expected and post-measurement values as described above.  Once a week, the salinity function should be checked using manufacturer recommended internal diagnostic checks (cell constants, millivolt output, or other readings) or against the QCS that they provide. Record results and report irregularities as described above.

### 5.2. Turbidity

The turbidity probe is calibrated in accordance with the manufacturer’s instructions prior to sampling. .  After the sampling day is complete, measure the turbidity of the calibration standard (expected) used earlier in the day to calibrate the instrument to conduct a post-measurement QC check. Record the expected and post-measurement values as described above.

### 5.2.6 Temperature

When performing the once-a-season temperature sensor check, the entire temperature range for the Northeast should be recorded. Typical range for the Northeast is: 6.8 °C ≤ T ≤ 32.3 °C

See below methods for measuring DO, pH, salinity, and temperature.

## 5.3 Sampling Procedure – Dissolved Oxygen, pH, Temperature, Turbidity & Salinity

- Measure the total water depth at the Y-location to the nearest 0.1 m and record on the Field Measurement Form. If the sonde is attached to a data recorder, crews may submit the hydrographic profile data via an electronic file.

- Lower the sonde into the water and record DO, pH, salinity, turbidity, and temperature measurements at the following depths: 0.1 m below the surface, 0.5 m below the surface, every 1 m from depths of 1.0 to 10.0 m, and if the site is greater than 10 m, every 5 m thereafter. Take the last set of measurements at 0.5 m from the bottom, making sure to not let the sonde touch the bottom. Record these results in the downcast section of the Field Measurement Form.

- Repeat the full sets of measurements at each of the same depth intervals as the probe is retrieved (upcast).

- After the in situ measurements have been completed for the sampling day, perform a “Post-Measurement Calibration Check” of the pH, salinity and turbidity probes. Record these values on the Calibration Log form.

## 5.4 Photosynthetically Active Radiation (PAR) Meter

Photosynthetically active radiation is typically measured using a PAR meter attached to a LI-COR® data logger. The PAR meter measures a vertical profile of light attenuation at each station. Measured light values are entered a regression equation and used to determine the coefficient of attenuation in the water column. PAR sensors require no field calibration; however, they should be returned to the manufacturer at least every 2 years for calibration. PAR is measured at the same depths as other water column indicators. See procedures below for measuring light attenuation.

### 5.4.1 Sampling Procedure—Light Attenuation

- Connect a deck sensor and an underwater sensor to the PAR meter following the manufacturer’s instructions. Enter the calibration factors (supplied by the manufacturer) for each probe.

- Place the deck sensor in an unshaded location on the boat to record the available ambient light. Turn on and follow manufacturer’s instructions to operate.

- Lower the underwater sensor, making sure that the sensor is facing up, on the SUNNY (or unshaded) side of the boat to a depth of 10 cm (represents “surface”). Allow the readings to stabilize and record the ambient and underwater light readings in the datalogger.

- Continue to lower the underwater sensor to each of the required depths: 0.5 m, every 1 m from 1.0 m to 10.0 m, every 5 m thereafter for sites greater than 10 m, and 0.5 m from the bottom. Allow the readings to stabilize at each depth before recording the values.

- Repeat the procedure at the same depths, but in reverse order on the upcast.

- Record the values from both sensors (μE/m2/s), at the appropriate water depths of the underwater sensor, on the datasheet. Record the deck sensor reading in the ambient column, and the underwater sensor reading in the underwater column.

- If the sensor hits bottom, allow 2-3 minutes for the disturbance to settle before taking the reading.

- If the light measurements become negative before reaching the bottom measurement, terminate the profile at that depth.

# 6. Total Nutrients [CHEM], Dissolved Nutrients [NUTS], and Chlorophyll-a [CHLA] Sample Collection and Preservation

This section describes the procedures and methods for the field collection and preservation of water samples that will be analyzed for chlorophyll-a [CHL-A], total nutrients including nitrogen and phosphorus [CHEM], and dissolved ammonia, nitrites, nitrates, and ortho-phosphate [NUTS].

Collect the water samples at the Y-location, 0.5 meters below the surface (or mid-depth if station depth is less than 1.0 meter), with a water sampling device such as a Niskin, Van Dorn, or Kemmerer bottle and transfer to a rinsed 250 mL amber HDPE bottle for total nutrients. Water for the chlorophyll-a sample will be collected and transferred to a separate 2 L amber HDPE bottle. Store all samples in darkness on ice in a closed cooler. After filtering the chlorophyll-a sample, the filter must be kept frozen until ready to ship.

If processing the samples on board the vessel or before shipping, the chlorophyll-a sample will be filtered, and the filter kept frozen until ready to ship. A portion of the filtrate from the chlorophyll-a processing will be collected for the dissolved nutrient sample.

Table 6.1 Equipment & supplies: water chemistry & chlorophyll-a sample collection

| For collecting samples | water sampling device or water pumping system nitrile gloves measuring cylinder HDPE bottle (250 mL, amber) [CHEM] HDPE bottle (2 L, amber) [CHLA] HDPE bottle (1 L, amber) [PHYTO] Utermöhl’s solution (see table 3.1) cooler with wet ice |
| --- | --- |
| For recording measurements | Sample Collection form water chemistry sample label pencils (for data forms) fine-tipped indelible markers (for labels) clear tape strips |

## 6.1 Sampling Procedure for Nutrients and Chlorophyll-a

The following describes the sampling procedures for collecting water chemistry samples.

- Collect the water chemistry samples at the Y-location, which is no more than 37 meters from the X-site.

- Complete the sample labels with site ID, sample ID, indicator code (e.g. CHEM) and date collected. Attach the completed label to the 250 mL amber HDPE sample bottle and the 2L amber HDPE bottles and cover with clear tape.

- Put on nitrile gloves.

- Using a water sampling device, collect a water sample at 0.5 m below the surface (or mid-depth if station depth is less than 1.0 meter).

- Rinse the sampling device and the sample containers three times with water from the site. Cap the bottles and rotate them so that the water contacts all the surfaces. Discard the water away from the sampling location if additional water is to be collected.

- Fill the 250 mL amber HDPE bottle (for total nutrients) and the 2 L amber HDPE bottle (for chlorophyll-a and dissolved nutrients) with sample water.

- Replace the lids and seal the lid of the 250 mL bottle tightly with electrical tape.

- Place both samples in a cooler on ice at 4°C.

- Record the collection data on the Water Sample Collection Form.

- Note anything that could influence sample chemistry (heavy rain, potential contaminants, etc.) in the Comments section.

- Proceed to Section 8.2 for instructions on processing chlorophyll-a and nutrients water sample to obtain a chlorophyll-a filter and the nutrients filtrate.

## Sampling Procedure for Phytoplankton

The following describes the sampling procedures for collecting phytoplankton samples.

- Collect the water chemistry samples at the Y-location, which is no more than 37 meters from the X-site.

- Complete the sample labels with Site ID and date collected. Attach the completed label to the 1-L amber HDPE sample bottle and cover with clear tape.

- Put on nitrile gloves.

- Using a water sampling device, collect a water sample 1-2 m below the surface (or mid-depth if station depth is less than 1.0 meter).

- Rinse the sampling device, the 1-L HDPE sample container and measuring cylinder three times with water from the site. Cap the bottles and rotate them so that the water contacts all the surfaces. Discard the water away from the sampling location if additional water is to be collected.

- Measure a portion of the water from the water sampling device (800 mls) into a clean graduated cylinder.

- Mix the sample with Utermöhl’s solution (1 ml Utermöhl’s per 100 ml water sampled) in the 1-L HDPE bottle and store in a cooler with ice.

# 7. Sediment Collection

Sediments will be collected for a variety of analyses. Field crews will sieve one or two sediment grabs and submit the resulting benthic infauna collection to the lab to be analyzed for species composition and abundance. Additional sediment grabs will be analyzed for chemical contaminants (organics/metals and TOC), grain size determination, and acute whole sediment toxicity. In order to provide the minimum volume of sediment for all analyses, different numbers of grabs at different sites, based on sediment characteristics, may need to be collected. While the biology (benthic assemblage) grab is being processed (sieved) by one crew member, other personnel collect the necessary grabs for chemistry, grain size, and toxicity tests. The grabs are composited, mixed and then split into four separate sample containers. A minimum of 3L of sediment must be collected at marine sites for chemistry (contaminants), toxicity, and grain size analyses.

A 0.04 m2, stainless steel, Young-modified Van Veen Grab sampler is appropriate for collecting sediment samples for both biological and chemical analyses. The top of the sampler is either hinged or otherwise removable so the top layer of sediment can be easily removed for chemical and toxicity sample collection. Record the dimensions and sample area of the grab used is recorded on the Sediment Sample Collection Form. The area of sediment the grab collects is important for data analysis.

Table 7.1 Equipment & supplies: sediment collection

| For collecting samples | Young-modified Van Veen weights and pads for grab nitrile gloves plastic tub or bucket 0.5 mm stainless steel sieve sieve box or bucket electrical tape forceps (fine-tipped) funnel (wide-mouth) Alconox Formalin (100% buffered) with stain Graduated cylinder for measuring formalin Rose Bengal Stain (for staining formalin solution) Borax ruler (cm) squirt bottle (for ambient water) stainless steel mixing pot or bowl with lid stainless steel or Teflon spoons (15”)/scoops/spatula HDPE bottle(s) (1 L, wide-mouth) [BENT] glass jar (60 mL, amber) [SEDC] glass jar (120 mL, amber) [SEDO] plastic bags (2, 1 quart) [SEDG] screw-top bucket (0.6 gallon) [SEDX] scrub brush cooler with wet ice |
| --- | --- |
| For recording measurements | Sample Collection form pencils fine-tipped indelible markers (for labels) clear tape strips |

## 7.1 Sampling Procedure

The following describes the sampling procedure to obtain sediment samples.

- Attach the sampler to the end of the winch cable with a shackle and tighten the pin.

- Set the grab according to the manufacturer’s instructions and disengage any safety device designed to lock the sampler open.

- Lower the grab sampler through the water column such that travel through the last 5 meters is no faster than about 1 m/sec. This minimizes the effects of bow wave disturbance to surficial sediments.

- Allow a moment for the sampler to settle into the substrate and then allow slack on the cable. Letting the cable go slack serves to release the jaws of the sampler so they will close as the sampler is retrieved.

- Retrieve the sampler and lower it into its cradle or a plastic tub on-board. Open the top and determine whether the sampling is successful or not.

- A successful grab is one having relatively level, intact sediment over the entire area of the grab, and a sediment depth at the center of at least 7 centimeters for the benthic macroinvertebrate grab (see Figure 7.1).

- Grabs containing no sediment, partially filled grabs, or grabs with shelly/rocky substrates or grossly slumped surfaces are unacceptable.

- Grabs filled to the top, where the sediment is in direct contact with the hinged top, are also unacceptable.

- It may take several attempts using different amounts of weight to obtain the first acceptable sample. More weight will result in a deeper bite of the grab. In very soft mud, pads may be needed to prevent the sampler from sinking into the mud. If pads are used, the rate of descent near the bottom should be slowed even further to reduce the bow wave.

- If, after several attempts, only grabs less than 7 centimeters deep can be obtained, use the next successful grab regardless of the depth of sediment at the center of the grab.

- Carefully drain overlying water from the grab. If the grab is used for benthic community analysis, the water must be drained into the container that will receive the sediment to ensure no organisms are lost.

- Record the condition of the sample (smell, substrate, presence of organisms on the surface, etc.) in Sediment Sample Collection Form.

- If the grab sampler size is less than 0.03 m2, take two grabs for the benthic macroinvertebrate collections and composite the sediment into the sieve.

- Process the grab sample for either benthic community analysis or chemistry/toxicity testing as described below.

- Repeat steps 4-8 until all samples are successfully collected.

Figure 7.1 Illustration of acceptable & unacceptable grabs for benthic community analysis. An acceptable grab is at least 7 cm in depth (using a 0.04m2 Van Veen sampler), but not oozing out of the top of the grab and has a relatively level surface.

## 7.2 Processing Procedure – Benthic Macroinvertebrate [BENt]Composition and Abundance

Grab samples obtained to assess the benthic macroinvertebrate community are processed as outlined below.

- Measure the depth of the sediment at the middle of the sampler and record the value on the Sediment Sample Collection Form. The depth should be >7 cm. Record descriptive information about the grab, such as the presence or absence of a surface floc, color and smell of surface sediments, and visible fauna.

- Dump the sediment into a clean plastic tub or bucket and then into a 0.5 mm mesh sieve. Place the sieve into a tray containing water from the sampling station, a larger bucket, or place the sieve over the side of the boat. Gently agitate the sieve to wash away sediments and leave organisms, detritus, sand particles, and pebbles larger than 0.5 mm. This method minimizes mechanical damage to fauna that is common when forceful jets of water are used to break up sediments. A gentle flow of water over the sample is acceptable. Extreme care must be taken to assure that no sample is lost over the side of the sieve. Only use salt water for rinsing sample.

- Let the water drain from the sieve. Using a squirt bottle with its tip snipped and filled with seawater, gently rinse the contents of the sieve to one edge. Remove large non-living items such as rocks and sticks after inspecting them and ensure that all benthic organisms are included in the collection.

- Using either a spoon, GENTLY scoop up the bulk of the sample and place it in a 1 L HDPE bottle. Complete the BENT sample label with site ID, sample ID, indicator code (i.e. BENT) date collected, and jar number. Attach the completed label to the 1 L wide mouth sample bottle and cover with clear plastic tape.

- Rinse the outside of the sample jar into the sieve, then, using a funnel, rinse the contents into the jar. The jar should be filled no more than one-half full. If the quantity of sample exceeds one half of the jar, place the remainder of the sample in a second container with a “2 of 2” label. For samples with a large amount of benthos, additional jars may be needed.

- Record sample collection location and the total number of jars on the Sediment Sample Collection Form.

- Carefully inspect the sieve to ensure that all organisms are removed. Use fine forceps (if necessary) to transfer fauna from the sieve to the bottle containing the proper sample number.

- A stained buffered formalin solution is used to fix and preserve benthic samples. The solution should be mixed according to the directions in Table 3.1. 100 mL of the formalin should be added to each sample jar (using a graduated cylinder) along with an additional teaspoon-full of borax to ensure saturation of the buffer. Rose Bengal stain is added to the stock formalin solution for use at all sites. Fill the jar to the rim with seawater to eliminate any air space. This eliminates the problem of organisms sticking to the cap because of sloshing during shipment.

- After preservation, tightly screw the cap on the jar and wrap the cap with electrical tape. Gently rotate the bottle to mix the contents and place in the dark. If the sample occupies more than one container, label all the sample bottles containing material from that grab together. All benthos jars from a single site will have the same sample ID number.

- Three replicate Van Veen grabs will be collected and sieved at each site. Samples from two of the grabs will be analyzed later in the lab. The third sample will be stored.

- Prior to sieving the sample at the next site, use copious amounts of forceful water and a stiff brush to clean the sieve, thereby minimizing cross-contamination of samples. Be sure to rinse the brush between each sieve cleaning.

## 7.3 Processing Procedure – Sediment Composition, Chemistry, and Toxicity

In addition to grab samples collected for benthic community analysis, additional grabs are collected for chemical analyses (organics/metals and TOC), grain size determination, and for use in acute toxicity tests. The top two centimeters of these grabs are removed, homogenized, and split into these four sample types.

The samples are removed and processed in the order described below.

- As each grab is retrieved, carefully examine it to determine acceptability. The grab is considered acceptable if the surface layer is intact. The grab need not be greater than 7 cm in depth for chemistry samples, but the other criteria outlined above apply (see Section 7.1 and Figure 7.1 above). Carefully drain off, or siphon, any overlying water, and remove and discard large, non-living surface items such as rocks or pieces of wood. Remove any submerged aquatic vegetation (SAV) after recording its presence on the Sediment Sample Collection Form.

- A clean stainless steel or Teflon spoon that has been washed with Alconox and rinsed with ambient site water is used to remove sediments from grab samples for these analyses.

- Remove the top 2 cm of sediment using the stainless steel or Teflon spoon. Sediment which is in direct contact with the sides of the sampler should be excluded as they may be contaminated from the device. Place the sediment into a pre-cleaned (washed with Alconox and rinsed with ambient site water) stainless steel pot or bowl and place the pot in a cooler on wet ice and stored at 4°C ..

- Repeat obtaining sediment samples from the grab and compositing the sediment in the same stainless pot/bowl until a sufficient  quantity of sediment has been collected for all samples (approximately 3L). Stir sediment homogenate after every addition to the composite to ensure adequate mixing. Keep the container covered and in the cooler between grabs.

- Record the location (zone) of the sediment collection on the Sediment Sample Collection Form. If sediment was collected from more than one zone, check the box for the zone where most of the sediment was collected and describe the proportions of sediment collected from each zone in the comments section.

- Homogenize the sediment by stirring with a Teflon paddle or stainless steel spoon for 10 minutes. Divide the composite into the four sample types listed below.

A) ORGANICS and METALS [SEDO]:

- Complete the SEDO sample label with site ID, sample ID, indicator code (SEDO) and date collected.

- Attach the completed label to the 120 mL glass sample jar and cover with clear tape.

- Using a clean stainless steel spoon, carefully place approximately 100 mL of sediment into the jar. Leave ½ inch headspace to avoid breakage due to possible sample expansion from freezing.

- Replace the lid and seal tightly with electrical tape and place the sample in a cooler with dry ice.

- Record sample ID along with any comments on the Sediment Sample Collection Form.

B) SEDIMENT TOXICITY [SEDX]:

- Complete the SEDX sample label with site ID, sample ID, indicator code (SEDX) and date collected.

- Attach the completed label to the 0.6 gallon plastic sample bucket and cover with clear tape.

- Using the stainless steel spoon, fill the bucket with sediment to within about 1 inch from the rim (Preferred volume is 1800 mL; minimum volume required is 900 mL).

- Replace the lid and close tightly.

- Record sample ID along with any comments on the Sediment Sample Collection Form.

- Place the sample on wet ice and stored at 4°C.

C) TOTAL ORGANIC CARBON [SEDC]:

- Complete the SEDC sample label with site ID, sample ID, indicator code (SEDC) and date collected.

- Attach the completed label to the 60 mL glass sample jar and cover with clear tape.

- Using a clean stainless steel spoon, place approximately 50 mL of sediment into the jar. Leave ½ inch headspace to avoid breakage due to possible sample expansion from freezing.

- Replace the lid and seal tightly with electrical tape and place the sample in a cooler with dry ice.

- Record sample ID along with any comments on the Sediment Sample Collection Form.

D) SEDIMENT GRAIN SIZE [SEDG]:

- Complete the SEDG sample label with site ID, sample ID, indicator code (SEDG)and date collected.

- Attach the completed label to a quart sized plastic sample bag and cover with clear tape.

- Using a clean stainless steel spoon, place approximately 100 mL of sediment into the pre-labeled bag. Double bag the sample into a second quart sized plastic bag, ensuring that the tops of both bags are sealed tightly.

- Record sample ID along with any comments on the Sediment Sample Collection Form.

- Place the sample on wet ice and stored at 4°C.

# 8. Submerged Aquatic Vegetation Sampling: Eelgrass

Eelgrass (Zostera marina) presence/absence is observed at every site during the July sampling. July and early August represent the peak growing season for eelgrass and the best time to observe the presence or absence of eelgrass beds. In addition, a survey device specifically developed for observing eelgrass from a boat will be deployed to take photographs as well as allow assessment of percent cover of eelgrass at a particular site. The method is described in detail below.

Table 8.1 Equipment and supplies: Eelgrass monitoring

| For collecting samples | Underwater digital camera, reel, and case 0.25 m2 PVC quadrat drop-frame, line SD card Charged battery for camera Measuring tape View Scope bucket |
| --- | --- |
| For taking measurements | Sample Collection form pencils |

The following describes the procedure to observe eelgrass presence/absence and estimate percent cover.

- Set up the camera follow the laminated camera guide included in the camera case.

- Four samples will be collected off the four corners of the boat.

- Write the last three digits of the site ID and sample ID on the white plastic tablet attached to the frame (use sample ID as xxx_1, xxx_2, xxx_3, xxx_4 to represent the four corners of the boat)

- Beginning on the windward and up-current side of the boat, with the camera turned on, gently lower the drop-frame over the side of the boat.

- Once it touches the bottom let it settle for approximately 10 seconds to allow any disturbed sediment to settle. View the monitor (camera screen) to ensure the quadrat is lying flat.

- Look at the monitor and ensure that the image shows the whole quadrat and that the bottom (and/or eelgrass) is clearly visible.

- On the DVR unit, press the center “OK” button to take a picture. If you are unsure if a picture was taken, press the “Preview” button on the DVR unit to view the last image captured.

- Record the timestamp from the picture.

- Record sediment type as mud, clay, sand, gravel and/or cobble and note other benthic characteristics (mussels, debris, algae or other observation) on the Eelgrass Sampling Form.

- Estimate the percent cover of eelgrass using the following bins (0%, 1-10%, 10-30%, 30-75%, 75-100%) and the provided coverage guide (Figure 8.1).

- Repeat at the remaining three (3) corners of the boat. Ensure that the sample ID is updated for each sample or photo at each site (see Step 3 above).

Figure 8.1 Estimating percent cover

75-100%

11-30%

1-10%

30-75%

Figure 8.2: Photos showing estimated percent cover of eelgrass

# 9. Final Site Activities

After sampling, crews complete a visual site assessment and, upon return to the launching location, the field crew must perform a post-measurement calibration check of the multi-parameter sonde, review all data forms and labels, inspect samples, complete tracking forms, ship, store or deliver samples, submit tracking forms, submit data forms, clean sampling equipment, and inventory supplies.

## 9.1 General Site Assessment

After sediment sampling, complete the site assessment with pertinent field observations. Record all observations from the site that were noted during the course of the visit.

### 9.1.1 Shoreline Activities and Disturbances

Rank shoreline activities and disturbances at the site. Consider only the shoreline that is ecologically significant to, adjacent to, and visible from the X-site. Do not consider the shoreline that is not in the same estuary, waterbody and/or embayment as the X-site. If the shore cannot be seen from the X-site (due to weather conditions or distance), note in the comments section the reason that the shoreline assessment was not possible. If an activity or disturbance is present, fill in the appropriate bubble: “L” for low, “M” for medium or “H” for high indicating the level of each.

### 9.1.2 Site Characteristics

Record the general characteristics of the site. When assessing site characteristics, look at a 200 m radius around the X-site. Rank the site on a scale of 1 to 5, with 1 indicating “pristine” or “appealing” and 5 indicating “highly disturbed” or “unappealing.” Observations may be used by analysts to help explain data and results. If any items of trash listed are visible in the water from the X-site, check the box estimating the amount of each trash type. If none are visible, check None. If possible, list “Other plastic items”, types of “Fishing gear” and “Other” items not accounted for above. Additional information on aquatic trash may be written in the General Assessment area at the crew’s discretion.  Document dominant land use. Document the weather conditions on the day of sampling, as well as any extreme weather conditions just prior to sampling.

Record any additional information and observations. Include observations on biotic integrity, presence of SAV, presence and abundance of endangered and/or exotic species, local anecdotal information, or any other pertinent information about the site or its adjacent areas. Record any observations that may be useful for future data interpretation (attach any additional notes to the field sheet).

## 9.2 Processing the Chlorophyll-a & Dissolved Nutrients Indicators

Water samples for chlorophyll-a and dissolved nutrients must be filtered prior to analysis. Ideally this step is conducted at the lab. However, if the samples cannot be delivered to the lab within the acceptable holding time, in order to preserve the integrity of the sample collected, water samples for chlorophyll-a and dissolved nutrient analyses may need to be filtered and processed on site. The chlorophyll-a sample is submitted to the lab as residue on a Whatman GF/F filter. Upon receipt of the filters, the lab extracts the pigment from the filter and quantifies it using fluorometry. A portion of the filtrate produced from collecting the chlorophyll-a sample is submitted to the lab and processed for dissolved nutrients. In order to avoid cross-contamination, a new filter funnel will be used at each site.

Table 9.1 Equipment & supplies: chlorophyll-a & dissolved nutrients processing

| For filtering chlorophyll-a sample | Whatman GF/F 47mm 0.7-micron filter 500 mL side-arm filter flask Filtration unit including filter funnel, rubber stopper and large filter funnel adapter vacuum pump DI water nitrile gloves forceps graduated cylinder (250 mL) |
| --- | --- |
| For recording measurements | Sample Collection form chlorophyll-a & dissolved nutrients sample labels pencils (for data forms) fine-tipped indelible markers (for labels) clear tape strips |
| For sample collection and preservation | centrifuge tube (50 mL, screw-top) aluminum foil square HDPE bottle (250 mL, white) cooler with dry ice electrical tape plastic bag (sandwich size) |

### 9.2.1 Processing Procedure

The section below describes the field procedures for processing chlorophyll-a and dissolved nutrient samples. The steps below describe using the nutrients filtering chamber setup. Crews have the option of using a side-arm filtering flask or other filtrate collection device in place of the nutrients chamber. If a flask or other filtrate collection device is used to collect the filtered nutrients sample (as opposed to collecting the sample directly into the nutrients bottle with a chamber), the collection device must be rinsed three times with filtered sample water before allowing any sample to enter the bottle.

- Complete the NUTS sample label with site ID, sample ID, indicator code (NUTS), and date collected. Attach the completed label to the 250 mL clear HDPE sample bottle and cover with clear plastic tape.

- Set up the nutrients filtering system (this may be a side-arm filtering flask or other filtrate collection device) on a flat surface and attach the vacuum pump (Figure 9.1).

- Put on nitrile gloves.

- Use a 250 mL filter funnel, rubber stopper, and adapter that are specifically designated for chlorophyll filtering. The stopper and adapter are to be cleaned between sampling events. Prior to filtration of the sample, rinse the filter funnel adapter three times with DI water. Rinse graduated cylinders with DI water.

- Use clean forceps to place a Whatman GF/F 47 mm 0.7 micron filter on the support pad with the gridded/pressed side of the filter facing down, making sure both the support pad and filter are centered on the base.

- Reattach the funnel portion of the filter funnel to the base by pressing it straight down firmly until it snaps into place. This will firmly hold the filter in place.

- Remove the 2 L amber chlorophyll-a collection bottle from cooler and shake to mix the sample. Using the graduated cylinder, measure and pour 250 mL of water into the filter holder, replace the cover, and use the vacuum pump to draw a small portion of the sample through the filter. Do not exceed 7 inches of Hg of vacuum ~3.4 psi or a filtration duration of more than 5 minutes for a single sample volume, to avoid cell damage or loss of contents during filtering.

- Use the first 10-20 mL of filtrate to rinse the 250 mL sample bottle and discard the rinsate. Be sure to cap the bottle and rotate it so that the filtered water contacts all the surfaces. Replace the bottle and chamber cap and continue filtering. Repeat the rinse of the sample bottle with an additional two rinses of filtered site water then discard the rinsate.

- If the filter clogs before 250 mL of site water will pass through the filter, discard the filter and water remaining in the filter funnel, rinse the filter funnel with DI water, install a new filter, and repeat the procedures using 100 mL of site water.

- Observe the filter for readily visible color. If there is visible color, proceed to the next step; if not, filter additional aliquots until color is visible on the filter or until a maximum of 2,000 mL have been filtered.

- After collecting 250 mL of filtered site water in the dissolved nutrients sample bottle, remove the 250 mL HDPE bottle. Replace the lid and seal tightly with electrical tape. Submit this filtrate for dissolved nutrient analyses.

- Move the filter funnel and adapter to a side-arm filter flask to complete the filtering process. Additional filtrate will be discarded.

- Record the dissolved nutrients sample information on the Water Sample Collection Form. Place the sample on wet ice.

- After achieving a readily visible stain on the filter and collecting the filtrate for dissolved nutrient analyses, record the actual sample volume filtered in the Chlorophyll-a section on the Water Sample Collection Form and on the sample label.

- Attach the completed label with site ID, sample ID, indicator code (NUTS), volume filtered, and date collected to the 50 mL centrifuge tube and cover with clear plastic tape.

- Rinse the graduated cylinder and upper portion of the filter funnel thoroughly with DI water to include any remaining cells adhering to the sides and pump through the filter. Monitor the level of water in the lower chamber to ensure that it does not contact the filter or flow into the pump.

- Remove the filter from the holder with clean forceps. Avoid touching the colored portion of the filter. Fold the filter in half, with the colored side folded in on itself. Place the folded filter into the 50 mL screw-top centrifuge tube and replace the cap.

- Wrap the 50 mL tube in a foil square and place in the provided self-sealing plastic bag.

- Close the plastic bag and place it on dry ice.

Figure 9.1 Filtering set-up for chlorophyll-a and nutrients filtering

## 9.3 Post-Measurement Calibration Check of Multi-Parameter Sonde

After all in situ measurements have been completed for the sampling day, a post-measurement calibration check of the multi-parameter sonde is performed. To do this, measure the pH, salinity and turbidity of one of each of the respective calibration standards (expected) that were used earlier in the day to calibrate the instrument. Record these values in the Post-Measurement Calibration Check section on the Calibration Log form. If significant drift is detected as defined the manufacturer, the meter may need service and data collected since the last successful calibration and post-measurement calibration check should be flagged.

## 9.4 Post-Measurement Check of Eelgrass Sampling Equipment

At the end of each field day all gear that came in contact with saltwater must be rinsed thoroughly, taking particular care with the camera and lowering frame. The camera is then soaked in a tub of warm water. Cables, connections, or electronic equipment from the waterproof box should not be allowed to come into contact with water. The two plugs attached to the camera cable reel must also remain clean and dry at all times.

The camera case must be inspected to make sure it is clean and dry after each use. If necessary, the monitor screen is cleaned carefully with a paper towel. Any water or moisture in the box should be removed with a dry paper towel and all electronic equipment inspected to ensure no damage occurred. Equipment should be store in a cool, dry place. Batteries should be recharged and photos downloaded after each sampling day from the SD card.

## 9.5 Field Data & Chain of Custody Form Review

All data forms must be reviewed for completeness, legibility, accuracy, and consistency by the crew lead. The following are some checks to perform on the data forms:

- Ensure that all required data forms for the site have been completed.

- Confirm that the site ID, sample IDs, analysis, and date of visit are correct on all forms.

- Verify the accuracy and legibility of all recorded information.

- Ensure that issues are explained in the respective comments sections.

- Ensure that written comments are clear, with no “shorthand” or abbreviations.

- After reviewing each form, initial the upper right corner of each page of the form.

If information is missing from the forms, the Field Crew Leader must complete the missing sections. Upon completing the review, the Field Crew Leader must initial the field forms, indicating that they are complete, legible, accurate, consistent, and ready to be sent to the MassBays and MassDEP Project Leads.

## 9.5 Sample Packaging and Label Review

All samples must be appropriately preserved and packaged for shipping or delivery. The following are some checks to perform on the labels:

- All samples are collected.

- All samples are labeled.

- All labels are complete, legible, accurate, and consistent.

- Review all the data forms, chain of custody forms, and labels to ensure consistent sample ID information was utilized throughout all forms and labels.

- Each label is covered with clear plastic tape.

- Inspect the integrity of each sample container; be sure there are no leaks. Make sure that all sample containers are properly sealed.

- Verify that all sample containers are properly preserved for storage or immediate delivery or shipment to the lab.

## 9.6 Sample Shipment and Chain of Custody Form Submittal

The field crew must ship or deliver time-sensitive samples (i.e., water chemistry (CHEM), chlorophyll-a (CHLA), and dissolved nutrients (NUTS)) to the appropriate analytical laboratory so that the samples will arrive within 48 hours of collection. The Field Crew Leader will place the samples and lab chain of custody form(s) (in a waterproof bag or plastic sleeve) in a cooler.

## 9.7 Data Submittal

The field data forms will be submitted to the Project Lead. After checking the field data forms for completeness, legibility, accuracy, and consistency forms will be scanned or copied. All field forms must be turned in within 2 weeks of completing sampling.

# 10. Field Quality Control

The MCCA program requires that all cooperators and field crews follow strict quality assurance and quality control guidelines. Standardized training and data forms set the foundation to help ensure that data quality standards for field sampling are met. In addition, quality control samples, and field evaluation and assistance visits address specific aspects of the data quality standards for the MCCA.

## 10.1 QA/QC Sample Collection

Water Quality Samples:

To estimate the overall precision or repeatability of results, a subset of MCCA field samples will be replicated by taking co-located and sequential duplicate samples during the same site visit. At a minimum 10% of the total number of samples per analyte group will be collected as a duplicate.  In addition, ambient field blanks are taken at 10% of total samples to evaluate if any sample contamination may have occurred due to improper sample collection, atmospheric fallout or other causes.

Sediment Samples:

To estimate the overall precision or repeatability of results, a subset of MCCA field samples

are replicated by taking co-located (a new location within 37m radius around the X-site) and sequential duplicate samples. At a minimum 10% of the total number of samples per analyte group is typically collected as a duplicate.

During the field season, each site is sampled monthly between June and August. In order to ensure that sampling procedures are as comparable as possible during all visits, the same field crew should ideally collect samples from the same site each month.

## 10.2 Infauna QA/QC Field Evaluation and Audit Visits

Crews must ensure that all grabs processed are acceptable according to the criteria described above. Caution must be taken to ensure that no organisms are lost during any step, including transferring the sample to the sieve, and during sieving. Samples must be properly labeled and preserved to assure they are received by the processing lab in acceptable condition.

## 10.3 Field Evaluation and Audit Visits

A rigorous program of field and laboratory evaluation and audit visits supports the quality assurance and control for the MCCA. The following sections focus only on the field evaluation and assistance visits.

Field evaluation and assistance visits provide an opportunity to ensure that crews follow field procedures and meet minimum quality control requirements. In addition, field audits allow for uniform evaluation of the standard MCCA data collection methods. During a scheduled field audit, MassBays and MassDEP staff (Evaluators) observe and review the information listed on the Field Evaluation and Assistance Visit Checklist.

It is anticipated that a field audit will be conducted with the crew early in the sampling and data collection process, and that corrective actions will be conducted in real time. The purpose of the field audit is to provide additional training and guidance so that the procedures are being performed in a manner consistent with the Field Operations Manual, all data are recorded correctly, and paperwork is properly completed at the site.

Upon completion of the sampling operations, the Evaluators will review the results of the evaluation with the crew before leaving the site (if practicable). The Evaluators ensures that all crew members understand the findings and can perform the procedures properly in the future. The Evaluators will record field crew responses or concerns, if any, on the Field Evaluation and Assistance Visit Checklist. After the Evaluators complete the Field Evaluation and Assistance Visit Checklist, including a brief summary of findings, all field crew members must read and sign off on the evaluation.

# 11. Literature Cited

Costa A, Larson E, Stamieszkin K. 2014. Quality Assurance Project Plan (QAPP) for Water Quality Monitoring in Cape Cod Bay. Boston: Massachusetts Water Resources Authority. Report 2014-07. 94 p.

DMF. 2018. Citizen Scientist Eelgrass Monitoring. Standard Operating Procedures. Massachusetts Division of Marine Fisheries.

Klemm, D. J., P. A. Lewis, F. Fulk, and J. M. Lazorchak. 1990. Macroinvertebrate Field and

Laboratory Methods for Evaluating the Biological Integrity of Surface Waters. EPA

600/4-90/030. U.S. Environmental Protection Agency, Cincinnati, Ohio.

USEPA. 2014. National Coastal Condition Assessment: Field Operations Manual. EPA-841- R-14-007. U.S. Environmental Protection Agency, Washington, DC.

# APPENDIX A: EQUIPMENT AND SUPPLIES

| Item | Protocol |
| --- | --- |
| multi-parameter water quality meter with DO, pH, salinity, temperature, and turbidity probes. extra batteries de-ionized water calibration cups and standards barometer to use for calibration thermometer Secchi disk (20 cm diameter, weighted) & 100' line with clip (marked in 0.5 m intervals) PAR meter | In situ water column sampling |
| water sampling device or water pumping system nitrile gloves measuring cylinder HDPE bottle (250 mL, amber) [CHEM] HDPE bottle (2 L, amber) [CHLA] HDPE bottle (1 L, amber) [PHYTO] Utermöhl’s solution (see table 3.1) cooler with wet ice | Water chemistry (total N and total P) & chlorophyll-a |
| Young-modified Van Veen weights and pads for grab nitrile gloves plastic tub or bucket 0.5 mm stainless steel sieve sieve box or bucket electrical tape forceps (fine-tipped) funnel (wide-mouth) Alconox Formalin (100% buffered) with stain Graduated cylinder for measuring formalin Rose Bengal Stain (for staining formalin solution) Borax ruler (cm) squirt bottle (for ambient water) stainless steel mixing pot or bowl with lid stainless steel or Teflon spoons (15”)/scoops/spatula HDPE bottle(s) (1 L, wide-mouth) [BENT] glass jar (60 mL, amber) [SEDC] glass jar (120 mL, amber) [SEDO] plastic bags (2, 1 quart) [SEDG] screw-top bucket (0.6 gallon) [SEDX] scrub brush cooler with wet ice | Sediment Collection |
| underwater digital camera, reel, and case 0.25 m2 PVC quadrat drop-frame, line SD card charged battery for camera measuring tape view Scope bucket | Eelgrass Monitoring (Note: all equipment for eelgrass monitoring will be provided by MassBays) |
| Whatman GF/F 47mm 0.7 micron filter 500 mL side-arm filter flask filtration unit including filter funnel, rubber stopper and large filter funnel adapter vacuum pump De-ionized water nitrile gloves forceps graduated cylinder (250 mL) | Chlorophyll-a and dissolved nutrient sample processing |
| Forms (provided by MassBays) pencils fine-tipped indelible markers (for labels) clear tape strips electric tape coolers wet ice dry ice | General Supplies |

# APPENDIX B: FIELD FORMS, LABELS AND TRACKING FORMS

# APPENDIX C: MCCA Survey Design

Massachusetts Department of Environmental Protection

Massachusetts Bays National Estuary Partnership

Massachusetts Coastal Condition Assessment (MCCA)

Survey Design 2020 – 2023

Target Population

The target population for this survey is a combination of all coastal waters within:

- Massachusetts Department of Environmental Protection (MassDEP) estuarine assessment units

- Massachusetts Bays National Estuary Program (MassBays) estuarine assessment units

- A near-shore seaward boundary defined by a maximum distance from Massachusetts shoreline of 3 miles and a maximum depth of 10 meters

Excluded from the target population are any tidal rivers or streams that are only represented in GIS by polylines versus polygons (i.e. small tidal streams and ditches) and any areas classified as intertidal estuarine or marine wetland in the National Wetland Inventory (NWI).  The term “coastal waters” in the remainder of this document refers to all waters within the target population.

Sample Frame

The sample frame was derived from GIS coverages of the 2016 MassDEP estuarine assessment units, 2017 MassBays estuarine assessment units, 1999 MassGIS Bathymetry of the Gulf of Maine, MassGIS Massachusetts shoreline, and a 3 mile buffer of the shoreline.  The bathymetric map and 3-mile buffer coverages were used to identify areas seaward of the shoreline that are less than 10 meters in depth and less than 3 miles from shore.  Any polygons not contiguous with the shoreline (i.e. isolated shallow locations within the 3-mile buffer) where eliminated from the sample frame.  The sample frame contains some areas (approx. 130 sq. km) classified as intertidal estuarine or marine wetland in the National Wetland Inventory (NWI) that were not removed due to uncertainty regarding the delineated boundaries.  Target population determinations for sites selected in these areas will be determined on a case-by-case during site evaluations.

Survey Design

A Generalized Random Tessellation Stratified (GRTS) survey design for an area resource is used with regional stratification and unequal probability of selection based on polygon classification.   The details are given below.

Stratification

The survey design is stratified by three geographic regions within Massachusetts to improve sampling logistics (Figure 1).  One region will be targeted and sampled each year from 2021 to 2023, starting with the Region A in 2021 and concluding with Region C in 2023.  A pilot project will be conducted in 2020 in multiple strata using a small subset of the selected primary sites to test sampling methodology.

Table 1. Regional strata descriptions

| Stratum | Major Basin Description |
| --- | --- |
| Region A | Merrimack, Parker, Ipswich, North Coastal, Boston Harbor |
| Region B | South Coastal, North Cape Cod (Cape Cod Bay) |
| Region C | South Cape Cod (Nantucket Sound), Islands, Buzzards Bay, Narragansett Bay, Mount Hope Bay |

Figure 1. Regional strata

Unequal Probability Categories

The MAP2 estuaries design is an unequal probability design within each regional stratum. Unequal probability categories were created based on the area of each polygon/estuary segment to ensure the selection of sites in smaller polygon/estuary segments.  Region B and C were divided into 4 size categories while Region A was divided into just 3 size categories (Figure 2).  Unequal probability category targets categories were set to allow sufficient sites for analysis with and without the XL size category.

Panels

This survey design has a single panel.

Figure 2. Unequal Probability Categories

Expected Sample Size

The designed sample size is a total of 90 sites for the state with 30 sites in each stratum.  In addition 60 oversample sites were selected in each stratum.   It is expected that 10-15 of the sites (approx. 3-5 in each strata) will be sampled during the pilot project in 2020.  The remaining 75 – 80 sites will be sampled from 2021 – 2023 (approx. 25 sites per year), starting with the Region A in 2021 and concluding with Region C in 2023.

Site Use and Replacement

Each site selected to be sampled is given unique site identification (siteID).  Site numbers consist of the project abbreviation (MAP2E) and a number between 001 and 270.  Within each regional stratum, the total list of sites evaluated for potential sampling must have all site IDs from the largest to the lowest number evaluated (i.e. none can be skipped). For example, if MAP2E-178 is the largest site ID evaluated within the Region C stratum, then all site IDs that are lower than 178 within the Region C stratum must be evaluated. Even more critical is that if MAP2E-178 is the largest site ID that is actually sampled in the field, then all lower site IDs within the Region C stratum that are evaluated to be within the target population and are accessible must also be sampled in the field.

Sample Frame Summary

|  | Categories (sq. km) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Stratum | Small (0-0.4 km2) | Medium (0.4-2.9 km2) | Large (2.9-100 km2) | X – Large (>100 km2) | Total |
| Region A | 5 | 55 | 294 | 0 | 354 |
| Region B | 4 | 40 | 278 | 101 | 423 |
| Region C | 12 | 74 | 180 | 998 | 1264 |
| Total | 5 | 55 | 294 | 0 | 354 |

Site Selection Summary

|  |  | Categories |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Stratum | Small (0-0.4 km2) | Medium (0.4-2.9 km2) | Large (2.9-100 km2) | X – Large (>100 km2) | Total |
| Primary | Region A | 5 | 10 | 15 | NA | 30 |
|  | Region B | 4 | 16 | 8 | 2 | 30 |
|  | Region C | 5 | 8 | 9 | 8 | 30 |
|  | Total | 14 | 34 | 32 | 10 | 90 |
| Oversample | Region A | 10 | 25 | 25 | NA | 60 |
|  | Region B | 11 | 22 | 22 | 5 | 60 |
|  | Region C | 13 | 21 | 18 | 8 | 60 |
|  | Total | 34 | 68 | 65 | 13 | 180 |

Description of Sample Design Output

| Variable Name | Description |
| --- | --- |
| siteID | Unique identification label for each site in the sample |
| Longitude | Site location longitude in decimal degrees coordinates (see projection below for datum). |
| Latitude | Site location latitude in decimal degrees coordinates (see projection information below). |
| xcoord | X-coordinate of the site (see projection information below) |
| ycoord | Y-coordinate of the site (see Albers projection information below) |
| mdcaty | Multi-density categories used for unequal probability selection |
| weight | Weight, inverse of inclusion probability, to be used in statistical analyses |
| stratum | Strata used in the survey design |
| panel | Identifies and Oversample |
| EvalStatus | Site evaluation decision for site: TS: target and sampled, LD: landowner denied access, etc (see below) |
| EvalReason | Site evaluation text commment |
| auxiliary variables | Remaining columns are from the sample frame provided |

Inability to collect samples for sediment or benthic indicators does not disqualify a site from meeting sample criteria. See Site Evaluation Guidelines  to determine site sampleability (MassDEP 2019).

Acceptable means: a) A sediment grab that meets the criteria for benthic samples; or b) Enough sediment can be collected that will allow the crew to obtain the surficial sub-sample required for the sediment composite to send to the laboratory for abiotic indicator analysis (e.g., organics/metals, TOC, grain size, toxicity).

For Secchi depth, no sunglasses or hats may be used while measuring depths. The Secchi depth is assessed from the shady side of the boat. One crew member must make all Secchi measurements at a site, and it is desirable to have the same crew member complete Secchi depth readings throughout the entire field season whenever possible.

Do not apply sunscreen or other chemical contaminants until after the sample is collected (or implement measures to reduce contamination by such chemicals if applied such as washing, wearing long gloves, etc.).

Adapted from Costa A, Larson E, Stamieszkin K. 2014. Quality Assurance Project Plan (QAPP) for Water Quality Monitoring in Cape Cod Bay. Boston: Massachusetts Water Resources Authority. Report 2014-07. 94 p.

The sampler, spoons and mixing bowl or bucket must be thoroughly rinsed with ambient water after sampling at each site to ensure no sediments remain, and then at the next station washed with Alconox and rinsed with ambient water prior to use. This practice reduces the risk of the equipment carrying contaminants from site to site.

Do not apply sunscreen or other chemical contaminants until after the sample is collected (or implement measures to reduce contamination by such chemicals if applied such as washing, wearing long gloves, etc.). Be sure to use new clean nitrile gloves or wash gloves between stations if they are reused from one station to the next.

Crews must make every attempt to process chlorophyll-a samples in subdued light, out of direct sunlight.