Massachusetts Department of Environmental Protection (MassDEP)

Division of Watershed Management (DWM)

Watershed Planning Program (WPP)

8 New Bond Street

Worcester, MA

CN 540.5

- Overview

Background:

Sodium chloride (NaCl) is applied to roadways in winter as a deicer. Mattson and Godfrey (1994) found that road salt is the major source of salt loading to Massachusetts streams. Chloride (Cl-) levels have also been correlated with impervious surfaces and urbanization (Wallace and Biastoch, 2016). Chloride is a recognized contaminant with potential to impair waterbodies and impact biological communities. It has also been shown to contribute to year-round elevated Cl- levels (Todd and Kaltenecker, 2012).

Purpose:

This plan involves collection of continuous conductivity data in order to estimate chloride levels using DEP’s conductivity-chloride regression at 10 stream locations in the Nashua Watershed. This collection will take place starting in November 2020 through October 2021.  Estimated chloride data will help assess how seasonal road salt (NaCl) applications may affect surface water quality. This study will collect continuous conductivity data for 1) application of the MA correlation between specific conductivity and chloride; and (2) analysis of data for comparison to EPA ambient criteria for acute and chronic toxicity.

Sampling & Analysis:

Continuous conductivity data will be collected using HOBO U24 freshwater data loggers.  Attended, discrete quality control (QC) readings will be taken at each visit using separate YSI/Hydrolab multiprobe instruments.  On three of the survey rounds, water samples will be collected for chloride (only) and analyzed by the WES lab.

Data Analysis:

Conductivity data will be analyzed using WPP’s chloride regression tool, which was previously verified, to estimate ambient chloride levels.

Locations:

This study will occur at locations in the Nashua Watershed, in close vicinity to Worcester, MA, choosing from the following list of water bodies:

| Nissitissit River (Nas01) | Willow Branch (Nas05) | Fall Brook Southeast (Nas09) |
| --- | --- | --- |
| Reedy Meadow Brook (Nas02) | Catacoonamug Brook (Nas06) | Goodridge Brook (Nas10) |
| Cady Pond Brook (Nas03) | North Nashua River (Nas07) |  |
| James Brook (Nas04) | Unnamed Tributary (Nas08) |  |

The study will focus on waterbodies based on their proximity to major roadways, routes, intersections, and/or high salt deposit areas, as well as proximity to drinking water wells.

- Project Definition and Background

2.1 Project Goals and Objectives

The data collected in this study will help the Massachusetts Department of Environmental Protection (MassDEP) - Division of Watershed Management (DWM), Watershed Planning Program, (WPP) identify potential impacts of road salt on freshwater bodies. From this study, WPP can better understand how road salt, urbanization, and/or other factors affect chloride concentrations of adjacent waterbodies, aquatic biotic health, and drinking water supplies. The results of this study have the potential to influence DEP water quality alert levels for chloride and provide baseline data for potential future development of Total Maximum Daily Loads (TMDLs). The sampling objectives for this study are:

- Estimate chloride levels in-stream by collecting continuous conductivity data at approx. 10 stations using HOBO U24 conductivity loggers during the period of November 2020 through October 2021. The data will then be used by applying the MA correlation between specific conductivity and chloride.

- Analyze resulting data to determine if estimated concentrations exceed MA state surface water quality standards (EPA ambient criteria for chloride).

- Surface Water Quality Monitoring

3.1 Standard Operating Procedures

This SAP will be implemented consistent with DWM’s EPA-approved programmatic Quality Assurance Program Plan (QAPP) for surface water monitoring in 2020/2024 (CN 520.1), and the project-specific QAPP for Chloride Monitoring & Assessment (CN 540.0).

HOBO U24 Conductivity Loggers will be deployed at the selected water quality monitoring stations in the Nashua Watershed in Central MA. These deployments will be performed in order to collect continuous conductivity at fixed, 30-minute, recording intervals. The HOBO units will be used in accordance with the Watershed Planning Program’s Standard Operating Procedure for Continuous Conductivity Monitoring (CN 349.0) and the manufacturer’s instructions. The loggers’ sensor faces will be cleaned before each intermittent data collection shuttle-technology download.

YSI/Hydrolab multiprobes will be used for QC purposes approximately every 2nd month between ~November 2020 and October 2021 at each of the stations. This quality control will be conducted primarily to collect data on temperature, depth, and specific conductivity.  HOBO conductivity data will be transformed to specific conductance and then reviewed to compare to Hydrolab specific conductance data.

Water samples collected on three of the surveys for chloride will be collected in HDPE bottles using standard WPP sampling protocols as defined in the program QAPP.  Samples will be delivered to the WES lab for analysis using method SM 4500-Cl E, within 14 days of receipt.

3.2 Non-Direct Measurements

To better interpret data from the study, the following information may also be collected:

- Road salt use recorded by Massachusetts Department of Transportation and area towns to estimate

chloride loading

- Land use area (agriculture, developed, natural, wetland, and impervious cover) for each station

- Weather statistics

3.3 Data Analyses

Conductivity data will be corrected to specific conductivity and used to estimate ambient chloride levels using the DWP’s chloride assessment tool, previously verified for this study and in accordance with Standard Operating Procedure CN 349.0.  Once available, final data will be summarized in a Technical Memorandum (project-specific or bundled into a larger report).

3.4 Design Rationale and Sampling Locations

Specific sampling locations are shown below.  In general, sites were selected to be representative of typical urban and suburban conditions. In certain cases, a site may have been selected based on indications of historically high levels of chloride or due to site proximity to a suspected high salt loading area or salt storage area, or a drinking water well or withdrawal point.  Of the sites proposed, none lie in any AUs that are currently impaired for aquatic life use due to chloride.

Site: Nas01 (Nissitissit River; Unique ID: W0486; AU ID: MA81-21)

The Nissitissit River is an approximately 9.2 miles long river with its headwaters originating from Potanipo Pond in Brookline, New Hampshire. It flows south across the state border into Pepperell, Massachusetts where it discharges into the Nashua River. The Nissitissit River is fed by Beaver, Gulf, Rocky Pond and Sucker Brooks. This river basin has a total area of imperviousness of approximately 7.7% and is designated as an Outstanding Water Resource (OWR) with high aesthetic quality and great wildlife habitat. A good location for sample collection and the deployment of the conductivity logger is immediately downstream the Mill Street bridge (42.671763, -71.576780). This is the last bridge over the Nissitissit before in merges with the Nashua River and should therefore give a good indication of the amount of chloride influx from high road application areas along the river.

Site: Nas02 (Reedy Meadow Brook; Unique ID: W2867; AU ID: MA81-64??)

Reedy Meadow Brook is an approximately 1.92 miles long watercourse in East Pepperell MA, that flows out of Wattles Pond and discharges in the Nashua River. It flows through conservation land, farmland and residential areas, picking up runoff from high salt application areas such as Route 113/Lowell Rd, Leighton St., Shawnee Rd. and parking lots of large business centers, before its confluence with the Nashua River. Immediately downstream the Lowell Rd bridge (42.670669 -71.562360) is a good location for sample collection and the deployment of the conductivity logger.

Site: Nas03 (Cady Pond Brook Unique ID: W2868, AU ID: TBD)

Cady Pond Brook flows out of Cady Pond and is a tributary of James Brook. Approximately 120 meters after leaving Cady Pond, the brook crosses Route 119/225/Boston Rd and is bordered by two businesses and the Groton Community School, all of which have large parking lots. Route 119/225 and these parking lots are the main source of chloride contamination for Cady Pond Brook. A good sample point is the area just downstream the Groton Community School’s parking lot (42.595536, -71.560633). That area of the brook can easily be accessed via the southwest end of the Pediatrics West PC parking lot.

Site Nas04: (James Brook, Unique ID: W1000, AU ID: MA81-20)

James Brook is a 4.4 miles long watercourse that originates in the Groton town center and flows south through agricultural lands and old fields. It runs parallel to Old Ayer Road for about a mile before flowing west through a residential area before it discharges into the Nashua River at the Ayer State Game Farm. Significant sources of Chloride runoff impacting James Brook would come from Old Ayer Road, Groton School Rd (Route 111) and the impervious surfaces in Groton town center near the head of the brook. The Chloride concentration from a potable water sample collected at the Groton Regional High School was 308 ppm. This water came from Well #1 (after treatment and storage). A good location for sample collection and deployment of the conductivity logger is at the Groton School Rd bridge (42.579376, -71.588472). Parking at the Groton School pump house next to the brook.

Site: Nas05 (Willow Branch, Unique ID: W2869, AU ID: TBD)

Willow Branch is a watercourse located in Devens MA and is a tributary of Nonacoicus Brook. The major sources of salt runoff that would impact the stream include Barnum Road, Jackson Road, West Main Street and the parking lots of large business centers (e.g.: Francis W. Parker Charter Essential School & Shriver Job Corps Center). An ideal location for the collection of Chloride data on Willow Branch, is immediately downstream the W. Main St. bridge (42.553156 -71.607977).

Site: Nas06 (Catacoonamug Brook, Unique ID: W2794, AU ID: MA81-16)

Catacoonamug Brook is located primarily in Lunenburg and Shirley municipalities in Massachusetts. It is fed by Bow Brook, Spruce Swamp Brook and five other unnamed tributaries. It originates in Lununburg and flows through Lake Shirley and Phoenix Pond, before discharging into the Nashua River in Shirley. Runoff from high salt areas that are likely to impact Catacoonamug Brook and its tributaries include Lancaster Road, Leominster Shirley, Catacunemaug Road and the parking lots and other impervious surfaces that border those waterways. A good location for the collection of samples and deployment of conductivity logger is just downstream the Lancaster Road bridge (42.543846 -71.656484).

Site: Nas07 (North Nashua River, Unique ID: W2795, AU ID: MA81-02)

The Northern Nashua River forms from the confluence of Whitman River and Phillips Brook and flows 19.9 miles, generally south eastward through Fitchburg and Leominster to Lancaster where it merges with the South Nashua River, to form the Nashua River. The river receives runoff from high salt areas such as I190, Route 2, Route 2A, Route 12, Route 13, Route 70, Route 117 and the many impervious surfaces in the municipalities that it flows through. A good sampling location is just off the corner of 1st Street and Railroad Street in Fitchburg (42.579167 -71.791111). River access and parking at this point of the river is easy

Site: Nas08 (Unnamed Tributary, Unique ID: W2981, AU ID: TBD)

This unnamed tributary originates from the Wyman Pond in Westminster MA, and discharges into the Sawmill Pond in Fitchburg, MA. On leaving Wyman Pond, the stream crosses East Road, then flows along Narrow Road (for approx. 1 mile). It then flows under Route 2, and then between Authority Drive and Route 2 (for approx. 1 mile) before being bordered by an industrial complex and terminating in Sawmill Pond. Route 2 is a heavily traffic roadway, and runoff from that section of the roadway will enter the stream. This unnamed tributary will also be impacted by runoff from the industrial park, as well as the other roads that borders it. A good location for the deployment of the conductivity logger is point at which the stream flows behind Mar-Lee Packaging and Consumer (42.550762 -71.852794). If permission is not granted to access the property, then an alternate site would be the point at which the stream flows closest to Authority Drive (near the intersection with Development Road: 42.549943 -71.861158).

Site: Nas09 (Fall Brook, Unique ID: W3014, AU ID: TBD)

Fall Brook, located primarily in Leominster MA, is a 4.7-mile-long river, that flows through Fall Brook Reservoir and Lake Samoset, before discharging into the Nashua River. It is fed by 5 tributaries. Fall Brook and its tributaries are very likely to receive runoff from high salt application areas like Central Street (Route 12), Meadow Pond Drive, Fallbrook Condos, Meadowbrook Acres and other similar high traffic areas. A suitable location for sampling would be ~350 meters downstream the Litchfield Street bridge, immediately past the former dam wall (42.509747, -71.741931). Nearest parking at Union Square Sports Hub parking lot, ~100 meters downstream of site.

Site: Nas10 (Goodridge Brook, Unique ID: W3015, AU ID: TBD)

Goodridge Brook is approximately 5.5 miles long and is a tributary of the South Nashua River in South Lancaster MA. Runoff from high road salt application areas such as Route 62, Route 70/Main Street and parking lots of businesses that border the brook. A good sampling location is just downstream Route 70/Main Street bridge, to the east side of Next Door Market (42.435468 -71.684807).

Table 1. 2020-2021 Chloride Sampling and Survey Sites: Nashua Watershed

| Site # | Waterbody | Site Name | Unique ID | GPS Coordinates (Decimal Degree) | AU ID | Site Description/Location |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Nissitissit River | Nas01 | W0486 | 42.671763, -71.576780 | MA81-21 | Site Nas04: D/S Mill Street bridge, Pepperell |
| 2 | Reedy Meadow Brook | Nas02 | W2867 | 42.670669, -71.562360 | MA81-64 | Site Nas01 D/S Lowell Road bridge, Pepperell |
| 3 | Cady Pond Brook | Nas03 | W2868 | 42.595536, -71.560633 | TBD | Site Nas02: D/S Groton Community School’s parking lot |
| 4 | James Brook | Nas04 | W1000 | 42.579376, -71.588472 | MA81-20 | Site Nas03: D/S Groton School Road/Route 111 bridge, Groton |
| 5 | Willow Branch | Nas05 | W2869 | 42.553156, -71.607977 | TBD | Site Nas05: D/S West Main Street bridge, Devens |
| 6 | Catacoonamug Brook | Nas06 | W2794 | 42.543846, -71.656484 | MA81-16 | Site Nas06: D/S Lancaster Road bridge, Shirley |
| 7 | North Nashua River | Nas07 | W2795 | 42.579167, -71.791111 | MA81-02 | Site Nas07: Point in river at Corner of 1st Street and Railroad Street, Fitchburg |
| 8 | Unnamed Tributary | Nas08 | W2981 | 42.550762, -71.852794 | TBD | Site Nas08: Stream at back of Mar-Lee Packaging and Consumer on Authority Drive, Fitchburg |
| 9 | Fall Brook | Nas09 | W3014 | 42.509747, -71.741931 | TBD | Site Nas09: D/S the Litchfield Street bridge, Leominster |
| 10 | Goodridge Brook | Nas10 | W3015 | 42.435468, -71.684807 | TBD | Site Nas10: D/S Route 70/Main Street bridge, South Lancaster |

References

Hach Company. Hydrolab DS5X, DS5, AND MS5: Water Quality Multiprobes User Manual, Edition 3. 2006, Catalog Number 003078HY.

Map data ©2018 Google. Google Earth Imagery. Accessed October – November 2020.

MassDEP. Quality Assurance Program Plan: Surface Water Monitoring & Assessment. 2020, CN 520.1. Massachusetts Department of Environmental Protection, Division of Watershed Management-Watershed Planning Program 2020-2024.

MassDEP. Quality Assurance Program Plan: Chloride Monitoring and Assessment. 2020, CN 540.0. Massachusetts Department of Environmental Protection, Division of Watershed Management-Watershed Planning Program.

MassDEP. Standard Operating Procedure: Continuous Conductivity Monitoring. 2015, CN 349.0. Massachusetts Department of Environmental Protection, Division of Watershed Management.

Mattson M.D., and Godfrey P.J., Identification of road salt contamination using multiple regression and GIS: Environmental Management [ENVIRON. MANAGE.], vol. 18, no. 5, pp. 767-773, 1994.

Nashua River Watershed 5 Year Action Plan 2003 - 2007. https://www.nashuariverwatershed.org/5yr_plan/. Accessed October 2020.

Onset Computer Corporation. HOBOware User’s Guide: Version 3.1. 2010, Doc#: 12730-C, Part#: MAN-BHW-UG

Todd, A.K., and Kaltenecker, M.G., Warm season chloride concentrations in stream habitats of freshwater mussel species at risk. Environmental Pollution, Vol. 171, December 2012, pp.199-206.

Wallace, A.M., Biastoch, R.G. Detecting Changes in the Benthic Invertebrate Community in Response to Increasing Chloride in Streams in Toronto, Canada. Freshwater Science 35, no.1, March 2016: 353-363.