Massachusetts Department of Environmental Protection (MassDEP)

Division of Watershed Management (DWM)

Watershed Planning Program (WPP)

8 New Bond Street

Worcester, MA

2023-2024 Chloride Project SAP

CN 603.0

- Overview

1.1 Background:

Sodium chloride (NaCl) is applied to roadways in winter as a deicer. Mattson and Godfrey (1994) found that road salt is the major source of salt loading to Massachusetts streams. Chloride (Cl-) levels have also been correlated with impervious surfaces and urbanization (Wallace and Biastoch, 2016). Chloride is a recognized contaminant with potential to impair waterbodies and impact biological communities (Delaune, Nesich, Goos, and Relyea, 2021). It has also been shown to contribute to year-round elevated Cl- levels (Todd and Kaltenecker, 2012).

1.2 Purpose:

This plan involves collection of continuous conductivity data and discrete surface water samples to estimate chloride levels using DEP’s conductivity-chloride regression at the selected stream locations in the watershed. This collection will take place starting in October 2023 through November 2024.  Estimated chloride data will help assess how seasonal road salt (NaCl) applications may affect surface water quality. This study will collect continuous conductivity data for (1) application of the MA correlation between specific conductivity and chloride; and (2) analysis of data for comparison to EPA ambient criteria for acute and chronic toxicity.

1.3 Sampling & Analysis:

Continuous conductivity data will be collected using HOBO U24 freshwater data loggers. Attended, discrete quality control (QC) readings will be taken at each visit using an Orion Portable Conductivity Meter.  On five of the survey rounds, water samples will be collected for chloride (only) and analyzed by the Wall Experiment Station (WES) laboratory.

1.4 Data Analysis:

Conductivity data will be analyzed using WPP’s chloride regression tool, which was previously verified, to estimate ambient chloride levels. Chloride concentrations from the discrete water samples shall be used to strengthen the chloride regression formula.

1.5 Locations:

This study will occur at locations in the Connecticut, Deerfield, Millers, and Westfield Watersheds. The waterbodies and sampling locations in each watershed are identified in Table 2 and the figures in Appendix A.

The study will focus on waterbodies based on their proximity to major roadways, routes, intersections, and/or estimated high salt deposit areas, as well as proximity to drinking water wells. See Appendix B for individual land use characteristics.

- Project Definition and Background

2.1 Project Goals and Objectives

The data collected in this study will help the Massachusetts Department of Environmental Protection (MassDEP) - Division of Watershed Management (DWM), Watershed Planning Program, (WPP) identify potential impacts of road salt on freshwater bodies. From this study, WPP can better understand how road salt, urbanization, and/or other factors affect chloride concentrations of adjacent waterbodies, aquatic biotic health, and drinking water supplies. The results of this study have the potential to influence MassDEP water quality alert levels for chloride and provide baseline data for potential future development of Total Maximum Daily Loads (TMDLs). The sampling objectives for this study are:

- Estimate chloride levels in-stream by collecting continuous conductivity data at multiple stations using HOBO U24 conductivity loggers during the period of October 2023 through November 2024. The data will then be used by applying the MA correlation between specific conductivity and chloride.

- Analyze resulting data to determine if estimated concentrations exceed MA state surface water quality standards (EPA ambient criteria for chloride).

- Collect water samples for chloride measurement. Measured amounts of chloride in water samples will be compared to simultaneously specific conductance measures to continue to evaluate and improve the MA correlation between specific conductance and chloride.

- Project Personnel and Responsibilities

3.1 Project Personnel

Specific descriptions of WPP staff roles and responsibilities for this monitoring project are detailed in Table 1. The WPP fulltime Chloride Project staff will be augmented by the seasonal employees from May through September to ensure that enough personnel are available to carry out field surveys throughout the height of the 2023 monitoring season.

| Table 1. Project Roles and Responsibilities related to monitoring and data use |  |
| --- | --- |
| Project Personnel | Responsibility |
| Project Oversight - Matthew Reardon - Richard Chase - Shervon De Leon | Provide project guidance, review and approval of SAP. |
| Project Coordinators - Shervon De Leon (Group 1) - Mason Saleeba (Group 2) - Peter Mitchell (Group 3) | Responsible for site reconnaissance, obtaining landowner access permission, defining logistics for efficient monitoring and generation of useable data at assigned sites using the procedures contained in WPP SOPs. |
| Water quality survey crews - Peter Mitchell (lead) - Mason Saleeba (lead) - Shervon De Leon (lead) - WPP staff and seasonal employees | Responsible for the collection of samples and data at assigned sites using the sample collection techniques and probe use procedures contained in WPP SOPs. |

For each field monitoring survey event, the staff member serving as the survey crew leader (at a minimum) will have met the following qualifications:

- Familiarity with this SAP and all applicable SOPs for that survey

- Completion of a multiprobe sampling/grab sampling/QC training segment

- Prior field experience with survey equipment and with similar monitoring surveys

- Be physically able to access the sites, carry equipment and samples, and perform the sampling.

Survey crew leaders will be accompanied by one or more additional crew members for each survey. All field survey crew personnel and WES/WPP lab personnel will be trained in the proper application of standard operating procedures (SOPs).

Dr. Oscar Pancorbo, Director of MassDEP’s Wall Experiment Station (WES), and/or his designees, will coordinate with the WPP regarding sample delivery, analyses, and reporting.  WES has been selected to perform chloride analysis.

- Surface Water Quality Monitoring

4.1 Standard Operating Procedures

This SAP will be implemented consistent with DWM’s EPA-approved programmatic Quality Assurance Program Plan (QAPP) for surface water monitoring in 2020/2024 (CN 520.1), and the project-specific QAPP for Chloride Monitoring & Assessment (CN 540.0).

Onset HOBO U24 Conductivity (and Temperature) Loggers will be deployed at the selected water quality monitoring stations in the Massachusetts western watersheds from October 2023 to October 2024.. These deployments will be performed to collect continuous conductivity and temperature readings at fixed, 30-minute, recording intervals. The HOBO units will be used in accordance with the Watershed Planning Program’s Standard Operating Procedure for Continuous Conductivity Monitoring (CN 349.0) and the manufacturer’s instructions. The loggers’ sensor faces will be cleaned before each intermittent data collection shuttle-technology download. After retrieval of deployed multiprobes, post-deployment calibration and QC checks on the data will be performed.  At deployment and prior to retrieval of multiprobes, as well as at various times during the deployment, QC readings will be taken using a separate meter as specified in WPP’s unattended probe SOPs.

A Thermo Scientific Orion Star A322 Conductivity Portable Meter (Thermo Scientific) will be used for QC purposes approximately every 3rd month between October 2023 and October 2024 at each of the stations. This quality control will be conducted primarily to collect data on temperature and specific conductivity.  HOBO conductivity data will be transformed to specific conductance and then reviewed to compare to Thermo Scientific specific conductance data.

Water samples will be collected at each sampling location on the five separate surveys for each watershed group in 250ml HDPE bottles, using standard WPP sampling protocols as defined in the program QAPP.  Samples will be preserved and delivered to the WES lab for analysis using method SM 4500-Cl E, within 14 days of receipt.

  - Non-Direct Measurements

To better interpret data from the study, the following information will also be collated:

- Road salt use recorded by Massachusetts Department of Transportation and Municipalities

- Forest cover, impervious cover, urban cover, and road miles per watershed area) within each watershed.

- Location of drinking water wells/intakes and surface water supply zones

- Daily temperature and precipitation observations

4.3 Data Analyses

Conductivity data will be corrected to specific conductivity and used to estimate ambient chloride levels using the WPP’s chloride assessment tool, previously verified for this study and in accordance with Standard Operating Procedure CN 349.0.  Once available, final data will be summarized in a Technical Memorandum (project-specific or bundled into a larger report).

4.4 Design Rationale and Sampling Locations

Specific sampling locations are shown below (Table 2) and in Appendix A. In general, sites were selected to be representative of typical urban and suburban conditions including percentage impervious cover, impact from roadways and parking lots. In certain cases, a site may have been selected based on indications of historically high levels of chloride or due to site proximity to a suspected high salt loading area or salt storage area, or a drinking water well or withdrawal point.  On occasion, recent or concurrent WPP monitoring projects may yield non-continuous conductivity measures resulting in sites of interest to the Chloride Project. These sites may then be included into the Chloride Project for further investigation. Concurrent and contemporary WPP projects may also include biological and chemical measures that may enhance the Chloride Project. Year round safe and easy access to sites were taken into consideration during site selection. Of the sites proposed, none lie in any AUs that are currently impaired for aquatic life use due to chloride. One site is typically chosen per AU, but if the WPP Assessment requests data for multiple sites in an AU, then that request will be fulfilled.

Table 2A - 2023-2024 Chloride Project Site details for Group 1 - Millers & Deerfield

| Basin | Order | Town/City | Waterbody | AUID | Unique ID | Site ID | Coordinates | Site Description | Site Selection Rationale |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Millers | 1 | Gardner | Pond Brook | NO AUID | W3319 | POND | 42.5598, -72.0011 | 100m SE from end of Kinzer Dr., Gardner | Receives surface runoff from Gardner and Route 2 |
| Millers | 2 | Gardner | Otter River | MA35-06 | W0051 | OTTER | 42.5646, -72.0117 | Upstream from Route 2A, Gardner | Most of this watershed is forested, but near this site is west Gardner |
| Millers | 3 | Templeton | Otter River | MA35-07 | W0691 | OTTGAGE | 42.5883, -72.0412 | Turner St., Gardner | There is a gage here, and flow data is vital for modelling chloride loading. |
| Millers | 4 | Athol | Mill Brook | NO AUID | W3320 | MILBKM | 42.5916, -72.2368 | Canal St., Athol | Large parking lots upstream |
| Millers | 5 | Athol | Millers River | MA35-04 | W3321 | MATHOL | 42.5891, -72.2406 | Near Jones St., Athol | Incorporates all runoff from Athol |
| Millers | 6 | Erving | Millers River | MA35-05 | W0690 | MILGAGE | 42.5978, -72.4378 | Bridge St., Erving | USGS gage here for loading determination |
| Connecticut | 7 | Greenfield | Fall River | MA34-33 | W3322 | FALLS | 42.617, -72.5498 | Factory Hollow road, Greenfield | Historic location |
| Deerfield | 8 | Greenfield | Cherry Rum Brook | MA33-97 | W3323 | CHERRY | 42.6086, -72.6007 | Country Club Road, Greenfield | Receives runoff from Route 2, Route 91, and Greenfield |
| Deerfield | 9 | Greenfield | Mill Brook | MA33-97 | W3320 | MILBKD | 42.6065, -72.6135 | Near Nash’s Mill Rd., Greenfield | Receives runoff from Route 91, and flow from Cherry Rum Brook. |
| Deerfield | 10 | Greenfield | Green River | MA33-30 | W0005 | GREEN | 42.575, -72.5985 | Near East Greenfield Dog Park. Petty Plain Rd. | Receives run off from Town of Greenfield |

Table 2B - 2023-2024 Chloride Project Site details for Group 2 - Connecticut River Central

| Basin | Order | Town/City | Waterbody | AUID | Unique ID | Site ID | Coordinates | Site Description | Site Selection Rationale |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Connecticut |  | 1 |  | South Hadley |  | Buttery Brook |  | MA34-42 |  | W2852 |  | BUTT |  | 42.2141, -72.5954 |  | Near Main St. South Hadley |  | Receives run off from South Hadley |  |  |
| Connecticut |  | 2 |  | South Hadley |  | Stony Brook |  | MA34-19 |  | W1792 |  | STONY |  | 42.2465, -72.5805 |  | Route 116. South Hadley |  | Receives run off from South Hadley and Mt. Holyoke. |  |  |
| Connecticut |  | 3 |  | Easthampton |  | Broad Brook |  | NO AUID |  | W2851 |  | BROAD |  | 42.2762, -72.6545 |  | Near Manhan Rail Trail. Easthampton |  | Receives run off from Easthampton |  |  |
| Connecticut |  | 4 |  | Easthampton |  | Manhan River |  | MA34-11 |  | W3193 |  | MANLOVE |  | 42.2808, -72.6502 |  | Lovefield Rd., Easthampton |  | Receives run off from Easthampton and Broad Brook. |  |  |
| Connecticut |  | 5 |  | Northampton |  | Mill River Diversion |  | MA34-42 |  | W3325 |  | MILHAMP |  | 42.3049, -72.6475 |  | Pynchon Meadow Rd., Northampton |  | Receives runoff from Northampton |  |  |
| Connecticut |  | 6 |  | Hatfield |  | Mill River |  | MA34-24 |  | W3326 |  | MCHEST |  | 42.3802, -72.625 |  | Chestnut Plain Rd., Hatfield |  | Receives runoff from Rte. 91 |  |  |
| Connecticut |  | 7 |  | Hatfield |  | Mill River |  | MA34-24 |  | W1061 |  | MILHAT |  | 42.3664, -72.6049 |  | Maple Street, Hatfield |  | Receives runoff from Rte. 91 and Hatfield |  |  |
| Connecticut |  | 8 |  | Hadley |  | Fort River |  | MA34-27 |  | W1051 |  | FORT |  | 42.3327, -72.5788 |  | Route 47, Hadley |  | Receives run off from South Amherst & Hadley fields. |  |  |
| Connecticut |  | 9 |  | Hadley |  | Unnamed Tributary |  | NO AUID |  | W3327 |  | MILTHAD |  | 42.3846, -72.5465 |  | North Hadley Rd., Hadley |  | Receives runoff from Route 9 malls. |  |  |
| Connecticut |  | 10 |  | Hadley |  | Mill River |  | MA34-25 |  | W3325 |  | MILAMH |  | 42.3898, -72.5393 |  | Near Mullins Arena, UMass / Amherst |  | Receives runoff from Route 116, UMass, & Amherst and flow from Tan Brook. |  |  |
| Connecticut |  | 11 |  | Hadley |  | Unnamed Tributary |  | NO AUID |  | W3329 |  | TAN |  | 42.3912, -72.5367 |  | Foot path near Forestry Way, Amherst |  | Receives runoff from UMass and Amherst |  |  |

Table 2C - 2023-2024 Chloride Project Site details for Group 3 - Westfield & Connecticut River South

| Basin | Order | Town/City | Waterbody | AUID | Unique ID | Site ID | Coordinates | Site Description | Site Selection Rationale |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Connecticut | 1 | Springfield | Mill River | MA34-29 | W3330 | MILSPFLD | 42.0906, -72.5762 | Near Mill St., Springfield | Receives urban runoff from Springfield |
| Connecticut | 2 | Springfield | Unnamed Tributary | NO AUID | W2853 | PEC | 42.0753, -72.5809 | Laurel Hill Rd., Springfield | Receives runoff from Forest Park and Springfield |
| Connecticut | 3 | Agawam | Threemile Brook | NO AUID | W3201 | THREE | 42.0442, -72.6187 | Veteran’s Memorial Cemetery, Agawam | Receives runoff from Agawam and Rte. 159. |
| Westfield | 4 | West Springfield | Westfield River | MA32-07 | W3331 | WMOUTH | 42.0906, -72.628 | Near River St., Westfield | Receives almost all inputs from Westfield watershed and Westfield. |
| Westfield | 5 | Southwick | Great Brook | MA32-25 | W0232 | GREAT | 42.0664, -72.7356 | Route 57, Southwick | Receives runoff from forested area |
| Westfield | 6 | Westfield | Little River | MA32-08 | W0808 | LITL | 42.1166, -72.7339 | Main St., Westfield | Receives run off from Westfield and agriculture. |
| Westfield | 7 | Russell | Potash Brook | MA32-22 | W0244 | POTASH | 42.166, -72.8306 | Woronoco Rd., Russell | Receives runoff from Rte. 90 and forested area. |
| Westfield | 8 | Westfield | Pond Brook | MA32-24 | W3332 | PBWEST | 42.1257, -72.7211 | Near Papermill Rd., Westfield | Receives runoff from Barnes AFB and Hampton Ponds. |
| Connecticut | 9 | Westfield | Powdermill Brook | MA32-09 | W3333 | POWDER | 42.1358, -72.736 | Near Greenwood St., Westfield | Receives runoff from Rte. 90 |
| Connecticut | 10 | West Springfield | Schoolhouse Brook | MA34-43 | W3199 | SCHOOL | 42.1497, -72.6316 | Near Riverdale St., West Springfield | Receives runoff from Rte.s 90 and 5. |
| Connecticut | 11 | Chicopee | Unnamed Tributary | MA34-60 | W3334 | UNTSPFLD | 42.1917, -72.5954 | Pendelton Ave., Springfield | Receives run off from Westover AFB and Springfield |

D/S =  Downstream, U/S = Upstream, PWS = Public Water Supply

References

Delaune K.D, Nesich D., Goos J.M., and Relyea R. A. Impacts of salinization on aquatic communities: Abrupt vs. gradual exposures. Environmental Pollution Volume 285 117636. 2021.

Map data ©2018 Google. Google Earth Imagery. Accessed October – November 2020.

MassDEP. MassDEP Online Map Viewer 2016 Integrated List of Waters Map. Massachusetts Department of Environmental Protection, Division of Watershed Management-Watershed Planning Program

MassDEP. Quality Assurance Program Plan: Surface Water Monitoring & Assessment. 2020, CN 520.1. Massachusetts Department of Environmental Protection, Division of Watershed Management-Watershed Planning Program 2020-2024.

MassDEP. Quality Assurance Program Plan: Chloride Monitoring and Assessment. 2020, CN 540.0. Massachusetts Department of Environmental Protection, Division of Watershed Management-Watershed Planning Program.

MassDEP. Standard Operating Procedure: Continuous Conductivity Monitoring. 2015, CN 349.0. Massachusetts Department of Environmental Protection, 2021.

MassGIS. MassDEP Online Map Viewer Water Supply Protection Map. Massachusetts Department of Environmental Protection, Division of Watershed Management-Watershed Planning Program

Mattson M.D., and Godfrey P.J., Identification of road salt contamination using multiple regression and GIS: Environmental Management [ENVIRON. MANAGE.], vol. 18, no. 5, pp. 767-773, 1994.

Onset Computer Corporation. HOBOware User’s Guide: Version 3.1. 2010, Doc#: 12730-C, Part#: MAN-BHW-UG

Thermo Scientific. Thermo Scientific Orion Star A320 Series User Manual. June 2015.

Todd, A.K., and Kaltenecker, M.G., Warm season chloride concentrations in stream habitats of freshwater mussel species at risk. Environmental Pollution, Vol. 171, December 2012, pp.199-206.

Wallace, A.M., Biastoch, R.G. Detecting Changes in the Benthic Invertebrate Community in Response to Increasing Chloride in Streams in Toronto, Canada. Freshwater Science 35, no.1, March 2016: 353-363.

Appendix A – Site Maps

Figure 1 – Overview of Chloride 2023-2024 Sites in the Millers, Deerfield, Connecticut, and Westfield watersheds

Figure 2 – Chloride 2023-2024 Sites in the Millers & Deerfield Watersheds (Group 1)

Figure 3 – Chloride 2023-2024 Sites in the Central Connecticut Watershed (Group 2)

Figure 4 – Chloride 2023-2024 Sites in the Southern Connecticut & Westfield Watersheds (Group 3)

Appendix B - Site Watershed Characteristics

| Basin Group | Site Order | Site ID | Watershed Area (mi2) | Percent Forest Cover | Percent Impervious Cover | Percent Developed Land | Road miles in Watershed Area (mi) | Road Miles per Watershed Area (mi/mi2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Group 1 - Millers & Deerfield | 1 | POND | 2.12 | 8% | 37% | 71% | 31.09 | 14.67 |
|  | 2 | OTTER | 20.2 | 50% | 10% | 24% | 121.6 | 6.02 |
|  | 3 | OTTGAGE | 34.1 | 52% | 10% | 23% | 195.4 | 5.73 |
|  | 4 | MILBKM | 5.1 | 60% | 8% | 26% | 31.01 | 6.08 |
|  | 5 | MATHOL | 281 | 71% | 3% | 10% | 996.4 | 3.55 |
|  | 6 | MILGAGE | 373 | 72% | 3% | 10% | 1282.9 | 3.44 |
|  | 7 | FALLS | 33.2 | 75% | 2% | 10% | 106.07 | 3.19 |
|  | 8 | CHERRY | 1.3 | 24% | 22% | 59% | 15.91 | 12.24 |
|  | 9 | MILBKD | 10.9 | 55% | 8% | 24% | 57.44 | 5.27 |
|  | 10 | GREEN | 89.2 | 75% | 3% | 11% | 300.89 | 3.37 |
| Group 2 - Connecticut River Central | 1 | BUTT | 3.12 | 15% | 24% | 70% | 40.63 | 13.02 |
|  | 2 | STONY | 19.6 | 29% | 6% | 25% | 85.44 | 4.36 |
|  | 3 | BROAD | 11.8 | 41% | 10% | 32% | 83.91 | 7.11 |
|  | 4 | MANLOVE | 82 | 64% | 3% | 12% | 278.33 | 3.39 |
|  | 5 | MILHAMP | 54.9 | 73% | 2% | 10% | 175.9 | 3.2 |
|  | 6 | MCHEST | 38 | 66% | 3% | 11% | 127.25 | 3.35 |
|  | 7 | MILHAT | 48.5 | 65% | 3% | 11% | 166.87 | 3.44 |
|  | 8 | FORT | 54.6 | 60% | 3% | 13% | 205 | 3.75 |
|  | 9 | MILTHAD | 2.91 | 16% | 18% | 56% | 29.76 | 10.23 |
|  | 10 | MILAMH | 24.2 | 69% | 4% | 12% | 85.81 | 3.55 |
|  | 11 | TAN | 0.35 | 10% | 52% | 90% | 4.96 | 14.17 |
| Group 3 - Westfield & Connecticut River South | 1 | MILSPFLD | 33.6 | 23% | 19% | 57% | 383.27 | 11.41 |
|  | 2 | PEC | 7.44 | 16% | 27% | 75% | 122.58 | 16.48 |
|  | 3 | THREE | 5.6 | 16% | 24% | 66% | 63.28 | 11.3 |
|  | 4 | WMOUTH | 515 | 77% | 2% | 9% | 1528.8 | 2.97 |
|  | 5 | GREAT | 16.8 | 40% | 5% | 20% | 76.66 | 4.56 |
|  | 6 | LITL | 85.4 | 75% | 2% | 8% | 224.88 | 2.63 |
|  | 7 | POTASH | 6.6 | 78% | 2% | 12% | 28.69 | 4.35 |
|  | 8 | PBWEST | 8.76 | 43% | 8% | 34% | 51.61 | 5.89 |
|  | 9 | POWDER | 9.57 | 43% | 15% | 40% | 61.19 | 6.39 |
|  | 10 | SCHOOL | 1.43 | 35% | 20% | 58% | 18.4 | 12.87 |
|  | 11 | UNTSPFLD | 2.67 | 4% | 39% | 89% | 43.89 | 16.44 |