Bureau of Resource Protection				Revised, May 1999

DEP NITROGEN LOADING

### COMPUTER MODEL GUIDANCE

INTRODUCTION

The DEP Nitrogen Loading Computer Model is a computerized mass balance nitrate-nitrogen (NO3) loading model for hydrogeologically defined wellhead protection areas (DEP approved Zone IIs) for public supply wells.

The Commonwealth of Massachusetts, Department of Environmental Protection, funded the computerization of this nitrogen loading model in response to the public health threat caused by the increasing number of public supply wells that are approaching the federally mandated drinking water standard for nitrate of 10 milligrams per liter (mg/l).  At the time of this writing there are over sixty public water supply wells that exceed the stated 5 mg/l nitrate planning goal.  A planning goal that is one-half of the drinking water standard has been adopted by Massachusetts because a consistent background concentration of 5 mg/l has been shown to correlate to isolated exceedances of the drinking water standard.

Ingestion of water exceeding the nitrate standard has been shown to cause varying levels of methemoglobinemia and sometimes death in infants younger than six months.  It has also been linked to birth defects and the formation of carcinogenic compounds within the body and frequently is an indicator of the presence of other toxins in the drinking water.  Increased attention nationwide on nitrate as a contaminant has been prompted by health studies and an increasing number of groundwater  sources experiencing contamination.

It is prudent to react early to gradually increasing nitrate concentrations in public supply wells because nitrate is one of the most expensive contaminants to remove from drinking water.  Using this model, the concentration that will be realized in the future can be estimated.  Proper land use planning, zoning, and nitrogen management controls, if employed early on, will save the community from investing millions of dollars in treatment systems to remove nitrate or to locate and develop new sources of drinking water.

DEP recommends that all communities experiencing rising levels of nitrate in their drinking water wells utilize this model to begin planning an approach to avoid the public health and fiscal consequences of nitrate contaminated drinking water.

## MODEL USES

Planning Tool:	The model is primarily a planning tool that allows communities to quickly determine how land use development decisions will impact water quality in their public drinking water wells.  The model, when loaded with the appropriate existing and potential land use development data, will demonstrate how differing development scenarios translate to water quality changes.

Source Approval	   Due to concerns over growing numbers of public supply wells experiencing increasing nitrate

for New Wells: 	concentrations, use of the model may be required when developing new public supply wells   designed to pump over 100,000 gallons per day or expanding an existing source by that same amount.Changes to Drinking Water Regulations 310 CMR 22.00 will reflect this requirement.

Water Management	   Communities that are required through the Water Management Act's permitting process to

Withdrawal Permits:	delineate Zone IIs for their wells, may, in certain cases, also be required to conduct a nitrate   loading analysis utilizing the model.

Title 5:	        The revised Title 5, 310 CMR 15.216(1), Aggregate Determinations of Flows and Nitrogen Loadings, may direct communities to use the DEP nitrogen loading model to predict the nitrate concentration they can expect in their public wells.  Flexibility from the septic system density requirement of 440 gallons per day per acre may be granted to communities that demonstrate that they will remain below the 5 mg/l nitrate planning goal assuming a zoning buildout scenario.  Accompanying the modeled results must be a nitrogen loading plan approved by the community which demonstrates that, despite varying the density restriction, the community has a development plan with appropriate regulatory support that will not result in exceedances of 5 mg/l in the drinking water well.

## User Friendly

The model was developed to be a user-friendly tool.  It was designed for use by people with elementary computer skills.  The most time consuming part of the nitrogen loading assessment is the compilation of land use development data within the Zone II.  Once the land use development data has been obtained and entered, it takes only seconds to query the model about how different development scenarios will affect water quality.

## NITROGEN LOADING CRITERIA AND MODEL ASSUMPTIONS

To quantify model inputs and standardize the nitrogen analysis, DEP has established default values based on literature review and our best professional judgement.  These default values have been programmed into the model and will be used automatically unless, after careful consideration and supportive documentation, the user changes these defaults.

Domestic wastewater flows are calculated based on the assumption that there are 3 occupants per residence.  This assumption applies to existing residences and those to be built and occupied in the future.  These assumptions are based on extensive census and demographic research about communities in Massachusetts.  This in no way contradicts or negates the need to use Title 5 design flows when designing Title 5 systems.  The census figures are averages appropriate for use on a regional (Zone II) scale with a broad timeframe.  Since households and family sizes change frequently, septic system designs must be done in accordance with Title 5 criteria.  For non-residential land uses, nitrogen inputs should be based on metered flows, if available, or flows from similar facilities.  Extrapolations from existing flows should be used for predicting future comparable land uses.  Title 5 design flows should be used in both cases if no metered or water bill data are available.

Examples of model assumption default values are:

- 35 mg/l nitrate concentration for on-site septic systems

- 3 lbs/1000 s.f. fertilizer application per 5000 s.f. lawn per lot at 25% leachability

- 3 persons per household

- 0.3 mg/l nitrate concentration in precipitation and runoff  (incorporating NO3 in runoff and precipitation)

## MODEL ADJUSTMENTS

In order to address specific circumstances, the model has been designed to let a user change default values to better represent site specific conditions that may apply.  However, changing default values will require supportive documentation subject to DEP approval.  For example, a town may have specific census data that justifies using a different number of persons per household.

MODEL APPLICATIONS / Hydrogeologic Conditions

The model has been designed to predict the nitrate concentration at the well based on land use development in the Zone II.  The model should not be applied to Interim Wellhead Protection Areas (IWPAs) which are based on a policy definition of circles of specified radius.  Therefore, a DEP approved Zone II delineation is a prerequisite for the model application.  Additionally, the nitrogen model may not be applicable in certain Zone IIs for wells in confined aquifers.

Induced Infiltration: The amount of water pumped by a well through induced recharge from nearby surface waters should be addressed in the Zone II report.  In cases where the percent of induced infiltration is not addressed, DEP recommends a simple mass balance calculation to estimate the quantity of water pumped by the well from induced infiltration.  For this estimate, assume a recharge rate of 20 inches per year.  The volume of recharge for the Zone II divided by the annual approved pumping volume of the well equals the percent of ground water withdrawal.  Therefore, the percent of induced infiltration from surface water will equal the annual pumping volume (100%) minus the percent of ground water withdrawal.

Multiple Well Zone II: In cases where there is only one well in a Zone II, use of the model is very straightforward.  In more complex situations involving multiple wells in one Zone II or overlapping Zone IIs, require hydrogeologic skills.   In these situations, the preparation of a Zone II Flow Path Analysis (Appendix A) may be necessary in order to segregate individual single well Zone IIs.  A model run would be required for each segregated Zone II along with its associated land uses.  Guidance on the Flow Path Analysis procedure and how to determine this area of recharge for a specific well in a multiple well Zone II is provided in Appendix A in and in the Division of Water Supply's 1996 Guidelines and Policies For Public Water Systems.

MODEL OUTPUTS

At least two model runs resulting in a modeled nitrate concentration at the wellhead will be required for the DEP nitrate analysis study: 1) Existing Condition, and 2) Zoning Buildout.

1. Existing Condition

This model run would account for nitrogen input derived from all existing land uses.  All existing development such as residences, municipal buildings, commercial stores, treatment plants, etc. as well as other land uses such as golf courses and agricultural uses would be incorporated into this model run along with Zone II hydrogeologic inputs.  The resulting modeled NO3 concentration, which is based on the assumption that all sources of NO3 have had time to flow with the groundwater to the well, should be compared with the observed nitrate monitoring data for the well. The modeled concentration should be higher than the observed NO3 concentration because equilibrated conditions may take many years to be realized.

2. Zoning Buildout

A Buildout model run for the Zone II based on current zoning must also be submitted.  Based on the dimensional lot requirements of frontage and area for each zoning district, the Buildout Analysis Table (Appendix B) must be completed as necessary input for this model run and is also required as part of the nitrate model submittal.  The Buildout Table offers guidance for the buildout analysis which accounts for existing and all possible future development on a parcel by parcel basis for existing and vacant lots as well as for large tracts of land within the Zone II that may be subdivided based on current zoning.

**  Nitrate Planning Goal:  5 mg/l:

The modeled Zoning Buildout result will be compared to the planning goal of 5 mg/l to determine if development impacts would pose a threat to water quality based on current zoning.  If the modeled outcome is 5 mg/l or greater at the well, nitrogen management options should be pursued.  Since the nitrogen model is based on buildout land uses dictated by the town's zoning, the model should be rerun whenever the zoning bylaw is revised.

OTHER MODEL APPLICATIONS

1. Site-Specific Project:  For planning purposes, it may be useful for a town to know the nitrogen loading potentially contributed by a specific development project.  To answer this "What if" question, the estimated nitrogen contribution from this site-specific development can be added to the current existing development model input and the model rerun.  The nitrate difference between the two model runs would be the nitrogen contribution from the specific project. This is a very simple exercise.

2. Alternative Development Scenarios:  If the Buildout modeled result is greater than the 5 mg/l planning threshold, alternative development strategies are recommended to ensure safe drinking water.  Such management options could involve land acquisitions, land use regulation changes to guide future development, and nitrogen reduction strategies such as sewering or enhanced nitrogen removal treatment for on-site septic systems for existing development to lessen NO3 source inputs.  Examples of management strategies are:

Land Use Zoning could modify commercial uses in Zone IIs to require limiting nitrate-nitrogen sources on an acreage basis.

Health Regulation could further limit nitrogen loading beyond Title 5 by restricting the number of bedrooms per dwelling or by establishing nitrogen reduction standards that would require enhanced nitrogen removal for on-site septic systems.

Site Review Process / Subdivision Rules and Regulations could require that a nitrogen loading analysis be conducted for the proposed site development project or subdivision and that nitrogen reducing systems be employed on an individual basis or for community systems.

To ensure that a proposed project within a Zone II would not impact water quality at the well, a developer may elect to go the extra mile and conduct a nitrogen loading analysis for the Zone II.  The developer gains favor for project approval by comprehensively predicting nitrogen impacts beyond the proposed site and the town benefits by acquiring a Zone II nitrogen assessment.

NITROGEN REDUCTIONS IN ZONE IIs

In situations where modeled results indicate that current development or zoning buildout will exceed the

5 mg/liter planning threshold and thereby present a threat to water quality, the model may be used to demonstrate how wastewater retrofit improvements will affect drinking water quality.

For example, the nitrogen reductions from sewering a small part of the Zone II and treating the wastewater to a higher standard, or employing several small advanced wastewater treatment plants, or retrofitting several homes with innovative or alternative enhanced nitrogen removal systems are easily calculated by the model and compared to the 5 mg/l planning goal.

Changing existing local zoning bylaws and regulations in nitrogen stressed Zone IIs may be necessary to avoid costly and dangerous water quality problems.  The model can be used to determine the effectiveness of various zoning and other nitrogen management options.

SUBMITTAL REQUIREMENTS

The nitrogen analysis, at a minimum, must include:

- Zoning map and any relevant nitrogen-related local regulations;

- Flow Path Analysis for the Zone II, if applicable;

3.  Buildout Analysis Table;

4.  Model run for Existing Condition;

5.  Model run for Zoning Buildout;

6.  Model run for Alternative Development Scenario, if applicable;

7.  Management plan consistent with achieving the 5 mg/l planning goal.

Two copies of the Nitrogen Loading Computer Model report should be sent to the appropriate DEP Regional Office.

The DEP Nitrogen Loading Computer Model may be downloaded at no charge from the DEP website homepage at www.state.ma.us/dep.

If you require further information on the DEP Nitrogen Loading Computer Model, please contact

Tom Lamonte, BRP/Boston, at 617 292-5532.

APPENDIX A

PREPARATION OF FLOW PATHS IN ZONE II

For DEP Nitrogen Loading Computer Model Application

The following procedure provides guidance to users of the DEP Nitrogen Loading Computer Model to determine the area of contribution for a particular well in a multiple well Zone II.  This procedure enables the Zone II for two or more wells to be subdivided into subzone areas which contribute recharge to each well by delineating groundwater flow paths across the horizontal plane of the Zone II.

A flow path analysis is not necessary where a Zone II has been delineated for an individual well, or where multiple wells have a long and narrow Zone II configuration.  However, the Drinking Water Program may require additional information in determining the flow path for a particular well in a Zone II.

This procedure requires specific information and entails certain assumptions and limitations.

1.   DEP approved Zone II boundary is already delineated.

2.   Groundwater contours can be drawn for the Zone II, and the groundwater elevation data used to determine the groundwater contours can be obtained or estimated for various points within the Zone II from observation well data, topographic elevations of streams and surface water bodies, or a combination of both.

Basic Assumptions / Limitations:

1.   The aquifer is unconfined.

2.   The aquifer is homogeneous and isotropic.  This means the hydraulic conductivity of the aquifer material, or the ability of the aquifer to transmit water, does not vary significantly from one place in the aquifer to another or in any direction.

3.   Use of surface water elevations to prepare the groundwater contours assumes the elevation of the surface water represents the intersection of the groundwater with the surface water at that point on the topographic map.

Steps for Flow Path Analysis:   (See illustrations on following page)

1. 	Obtain a copy of the map showing the approved Zone II boundary and the location of all wells for which the Zone II was determined.  This map will be at the scale of the USGS topographic map (1:25,000) or larger, but typically the Zone II is superimposed over the relevant portion of a USGS topographic map.

2. 	Locate the known and/or estimated groundwater elevations on the map.  The groundwater elevations may be obtained from observation well data in the Zone II report and elevations of streams (as groundwater intersections) as shown on the USGS topographic map.  From these elevations, plot the estimated groundwater contours across the entire Zone II.  Usually the Zone II Report filed with DEP/Drinking Water Program includes a groundwater contour map under pumping conditions.

3. 	If necessary, extrapolate to draw the groundwater contours at more frequent intervals and to draw a sufficient number of groundwater flow lines so the "groundwater flow divides" between wells can be estimated.  Groundwater flows perpendicular to groundwater contours.  These flow lines illustrate the path of water as its drawn to a specific well.  The entire Zone II must be covered by these subzones.  Generally, there will be one subzone for each well, although wells in close proximity may share a subzone.

4.	Boundaries of each subzone are located where the groundwater flow lines diverge between the wells.

The associated land use nitrogen contributions at buildout for each well's subzone recharge delineation are then calculated in a model run to determine the nitrate loading delivered to each well.  A model run will be required for each well's individual Zone II.

Appendix A (cont.)

Preparation of Flow Paths for Individual Subzones in Multiple Well Zone II

Zone II Delineation

80

70

Stream

60

50

Well

40

Well

Well

30

Step 1.  Obtain Zone II map	       	        Step 2.  Determine groundwater contours

# Step 3.  Construct groundwater flow lines	                    Step 4. Determine subzone for each well.

# to each well.

|  |  |  |  |  |  | Appendix B |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Zone II Buildout Table |  |  |  |  |  |  |  |  |  |  |  | Report Date: __________ |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Page: __________ |  |  |  |  |
| Town: _______________________ |  |  |  |  |  |  |  | Well Name: ______________________ |  |  |  |  |  |  |  |  |  |  |  |
| Zoning Bylaw Date: _____________ |  |  |  |  |  |  |  |  | PWS ID # : ______________________ |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | A | B |  |  |  | C | D | E |  |  |
|  |  |  |  | Existing | Grand- | Actual | Required | Minimum | Potential | Potential | Total | Sewer | Water Use |
| Assessor |  |  | Zoning |  | Dev'able | fathered | Frontage | Frontage | Lot Size | ANR Lots | Subdiv Lots | Dev'able Lots | Connect | Com / Indus |
| Map # | Lot # | Use | Acres | Units | Vacant Lots | (ft) | (ft) | (acres) |  |  | A+B+C+D | Lots | only |
| Totals: |  |  |  |  |  |  |  |  |  |  |  |  |  |