93-AC-1

NORTH COASTAL WATERSHED

1997/1998 WATER QUALITY

ASSESSMENT REPORT

COMMONWEALTH OF MASSACHUSETTS

EXECUTIVE OFFICE OF ENVIRONMENTAL AFFAIRS

ROBERT DURAND, SECRETARY

MASSACHUSETTS DEPARTMENT OF ENVIRONMENTAL PROTECTION

LAUREN A. LISS, COMMISSIONER

BUREAU OF RESOURCE PROTECTION

ARLEEN O’DONNELL, ASSISTANT COMMISSIONER

DIVISION OF WATERSHED MANAGEMENT

GLENN HAAS, DIRECTOR

NOTICE OF AVAILABILITY

LIMITED COPIES OF THIS REPORT ARE AVAILABLE AT NO COST BY WRITTEN REQUEST TO:

MASSACHUSETTS DEPARTMENT OF ENVIRONMENTAL PROTECTION

DIVISION OF WATERSHED MANAGEMENT

627 MAIN STREET

WORCESTER, MA  01608

This report is also available from the Department of Environmental Protection, Division of Watershed Management’s home page on the World Wide Web at:

http://www.state.ma.us/dep/brp/wm/wmpubs.htm

NOTE: For web publication, corrections to the printed version of this assessment report are highlighted in purple.

Furthermore, at the time of first printing, eight (8) copies of each report published by this office are submitted to the State House in Boston; these copies are subsequently distributed as follows:

On shelf; retained at the State Library (two copies);

Microfilmed; retained at the State Library;

Delivered to the Boston Public Library at Copley Square;

Delivered to the Worcester Public Library;

Delivered to the Springfield Public Library;

Delivered to the University Library at UMass Amherst;

Delivered to the Library of Congress in Washington, D.C.

Moreover, this wide circulation is augmented by inter-library loans from the above-listed libraries.  For example, a resident of Winchendon can apply at their local library for a loan of any DEP/Division of Watershed Management report from the Worcester Public Library.

A complete listing of reports published since 1963 is updated annually and printed in July.  This report, entitled, “Publications of the Massachusetts Division of Watershed Management – Watershed Planning Program, 1963 – (current year)” is also available by writing to the DWM in Worcester.

DISCLAIMER

Reference to trade names, commercial products, manufacturers, or distributors in this report constitutes neither endorsement nor recommendation by the Division of Watershed Management for use.

NORTH COASTAL WATERSHED

1997/1998 WATER QUALITY ASSESSMENT REPORT

Prepared by:

Gregory J. DeCesare, Laurie E. Kennedy, and Mollie J. Weinstein

Department of Environmental Protection

Division of Watershed Management

DWM Control Number:

17.0

Report Number:

93-AC-1

MASSACHUSETTS DEPARTMENT OF ENVIRONMENTAL PROTECTION

DIVISION OF WATERSHED MANAGEMENT

WORCESTER, MASSACHUSETTS

May 2000

ACKNOWLEDGEMENTS

Coordination of local, state and federal agencies and private organizations is fundamental to the success of the Massachusetts Watershed Initiative.  We would like to thank Larry Gil, EOEA North Coastal Watershed Team Leader and the North Coastal Watershed Team.  Special recognition is also given to Brad Chase of the Division of Marine Fisheries who provided much information on the aquatic resources of the North Coastal Watershed and answered many questions that improved the quality of this report.

Much appreciation is also extended to several DEP employees for their contributions:  Rick McVoy, Jane Colonna-Romano, Juliet Mathers, Mark Guilmain, Tom Dallaire, and Arthur Screpetis.

It is impossible to thank everyone who contributed to the assessment report process:  field, laboratory, data management, writing, editing, and graphics, as well as meetings, phone calls, and many e-mails.  All of these contributions are very much appreciated.

# TABLE OF CONTENTS

TABLE OF CONTENTS	i

LIST OF ACRONYMS	iii

EXECUTIVE SUMMARY	iv

NORTH COASTAL WATERSHED 1997/1998 WATER QUALITY	iv

ASSESSMENT REPORT	iv

RIVERS, ESTUARIES, COASTAL EMBAYMENTS, and MARINE WATERS	iv

LAKES/PONDS	viii

INTRODUCTION	1

ASSESSMENT METHODOLOGY	1

NORTH COASTAL WATERSHED DESCRIPTION AND CLASSIFICATION	11

REVIEW OF EXISTING CONDITIONS AND PERCEIVED PROBLEMS	14

SOURCES OF INFORMATION	15

DEP GRANT AND LOAN PROGRAMS	17

DEP REGULATORY PROGRAMS	17

OBJECTIVES	18

SEGMENT REPORT FORMAT	19

RIVERS AND COASTAL SEGMENTS	20

IPSWICH BAY	21

THE ESSEX BAY SYSTEM	22

ESSEX RIVER (Segment MA93-11)	23

ESSEX BAY (Segment MA93-16)	25

THE ANNISQUAM RIVER SYSTEM	26

ALEWIFE BROOK (Segment MA93-26)	27

UNNAMED TRIBUTARY (Segment MA93-27)	28

MILL RIVER (SEGMENT MA93-28)	29

ANNISQUAM RIVER (Segment MA93-12)	31

ROCKPORT HARBOR (Segment MA93-17)	33

GLOUCESTER HARBOR (Segment MA93-18)	35

SALEM SOUND SYSTEM	37

MANCHESTER HARBOR SYSTEM	38

CAT BROOK (Segment MA93-29)	39

MANCHESTER HARBOR (Segment MA93-19)	42

BEVERLY HARBOR SYSTEM	44

BASS RIVER SYSTEM	45

BASS RIVER (Segment MA93-07)	46

BASS RIVER (Segment MA93-08)	48

FROST FISH BROOK (Segment MA93-36)	50

PORTER RIVER (Segment MA93-04)	52

CRANE RIVER SYSTEM	54

BEAVER BROOK (Segment MA93-37)	55

CRANE BROOK (Segment MA93-02)	57

CRANE RIVER (Segment MA93-38)	59

CRANE RIVER (Segment MA93-41)	62

WATERS RIVER (Segment MA93-01)	63

NORTH RIVER SYSTEM	66

GOLDTHWAIT BROOK (Segment MA93-05)	67

PROCTOR BROOK (Segment MA93-39)	71

PROCTOR BROOK (Segment MA93-40)	74

NORTH RIVER (Segment MA93-42)	75

DANVERS RIVER (Segment MA93-09)	76

BEVERLY HARBOR (Segment MA93-20)	78

SALEM HARBOR SYSTEM	80

FOREST RIVER (Segment MA93-10)	81

SALEM HARBOR (Segment MA93-21)	84

MARBLEHEAD SYSTEM	87

MARBLEHEAD HARBOR (Segment MA93-22)	88

SALEM SOUND (Segment MA93-25)	89

NAHANT BAY (Segment MA93-24)	93

SAUGUS RIVER SYSTEM	96

SAUGUS RIVER (Segment MA93-34)	97

BEAVERDAM BROOK (Segment MA93-30)	100

SAUGUS RIVER (Segment MA93-35)	103

MILL RIVER (Segment MA93-31)	106

HAWKES BROOK (Segment MA93-32)	109

HAWKES BROOK (Segment MA93-33)	111

SAUGUS RIVER (Segment MA93-14)	113

PINES RIVER (Segment MA93-15)	120

LYNN HARBOR (SEGMENT MA93-23)	122

LAKE/POND ASSESSMENTS	125

LAKE USE ASSESSMENTS	125

SUMMARY	127

RECOMMENDATIONS	130

REFERENCES	131

APPENDIX A - DEP DWM QA/QC	A1

APPENDIX B - 1997/1998 DEP DWM NORTH COASTAL WATERSHED SURVEY DATA	B1

APPENDIX C - DEP BIOMONITORING TECHNICAL MEMORANDUM	C1

INTRODUCTION	C1

METHODS	C1

RESULTS AND DISCUSSION	C1

CONCLUSIONS	C9

LITERATURE CITED	C9

APPENDIX D - SEWAGE POLLUTION PROBLEMS CITY OF SALEM	D1

APPENDIX E - DEP 1999 GRANT AND LOAN PROGRAMS	E1

APPENDIX F - DMF SHELLFISH DATA, NORTH COASTAL WATERSHED	F1

LIST OF TABLES AND FIGURES

Table 1. North Coastal Watershed Lakes Use Support Summary Summer, 1997 	viii

Table 2. North Coastal Watershed Lakes Trophic Status Summary Summer, 1997	ix

Table 3. Summary of Massachusetts Surface Water Quality Standards	4

Table 4. Metropolitan District Commission (MDC) fecal coliform bacteria data for Kings and Nahant

beaches, summer 1997.	94

Table 5. Metropolitan District Commission (MDC) fecal coliform bacteria data for Revere Beach,

summer 1997	123

Table 6. 1997 North Coastal Watershed Lake Assessments summer 1997	128

Figure 1.1997/1998 DEP DWM water quality, macroinvertebrate and fish consumption station data    summary in the North Coastal Watershed	xi

Figure 2. Clean Water Act Implementation Cycle	1

Figure 3. Location of the North Coastal Watershed	11

Figure 4. Location of lakes assessed in the North Coastal Watershed	125

# LIST OF ACRONYMS

| 7Q10 | Seven Day Ten Year Low Flow |  |
| --- | --- | --- |
| ACEC | Areas of Critical Environmental Concern |  |
| BPJ | Best Professional Judgement |  |
| CFS | Cubic Feet per Second |  |
| CMR | Code of Massachusetts Regulations |  |
| CNOEC | Chronic No Observed Effect Concentration |  |
| CWA | Clean Water Act |  |
| DDT | Dichlordiphenyltrichloroethane |  |
| DEM | Department of Environmental Management |  |
| DFWELE | Department of Fisheries, Wildlife, and Environmental Law Enforcement |  |
| DMR | Discharge Monitoring Report |  |
| DO | Dissolved Oxygen |  |
| MA DPH | Massachusetts Department of Public Health |  |
| DWM | Department of Watershed Management |  |
| EPA | United States Environmental Protection Agency |  |
| GIS (MassGIS) | Geographic Information System |  |
| GP | Gravel Packed |  |
| GPM (D) | Gallons per Minute (Day) |  |
| JTU | Jackson Turbidity Unit |  |
| LC50 | Lethal Concentration to 50% of the Test Organisms |  |
| MDC | Metropolitan District Commission |  |
| DEP | Massachusetts Department of Environmental Protection |  |
| MEPA | Massachusetts Environmental Policy Act Unit |  |
| MGD | Million Gallons per Day |  |
| mg/L | Milligram per Liter |  |
| NCCW | Non-Contact Cooling Water |  |
| NH3-N | Ammonia-Nitrogen |  |
| NPDES | National Pollutant Discharge Elimination System |  |
| NPS | Non Point Source |  |
| NTU | Nephelometric Turbidity Units |  |
| ORW | Outstanding Resource Waters |  |
| PAH | Polyaromatic Hydrocarbons |  |
| PALIS | Pond and Lake Information System |  |
| PCB | Polychlorinated Biphenols |  |
| PPM | Parts Per Million |  |
| PWS | Public Water Supply |  |
| QAPP | Quality Assurance Project Plan |  |
| QA/QC | Quality Assurance/ Quality Control |  |
| RBP | Rapid Bioassessment Protocol |  |
| SARIS | Stream and River Inventory System |  |
| SS | Suspended Solids |  |
| SU | Standard Units |  |
| SWQS | Massachusetts Surface Water Quality Standards |  |
| TMDL | Total Maximum Daily Load |  |
| TOC | Total Organic Carbon |  |
| TRC | Total Residual Chlorine |  |
| USFDA | United States Food and Drug Administration |  |
| USGS | United States Geological Survey |  |
| VOC | Volatile Organic Compound |  |
| WMA | Water Management Act |  |
| WWTP | Waste Water Treatment Plant |  |

# EXECUTIVE SUMMARY

# NORTH COASTAL WATERSHED 1997/1998 WATER QUALITY

# ASSESSMENT REPORT

The assessment of current water quality conditions is a key step in the successful implementation of the Watershed Approach.  This critical phase provides basic information needed to focus resource protection and remediation activities later in the watershed management planning process.  Detailed information regarding individual river segments, estuaries, coastal embayments, marine waters and lakes in the North Coastal Watershed is presented in this assessment report.  The Massachusetts Surface Water Quality Standards designate the most sensitive uses (Aquatic Life, Fish Consumption, Primary and Secondary Contact Recreation, Shellfishing and Aesthetics) for which surface waters shall be protected.  The following is a brief summary of water quality conditions in the North Coastal Watershed in relation to their support of those designated uses.

## RIVERS, ESTUARIES, COASTAL EMBAYMENTS, and MARINE WATERS

A total of 37.3 river miles and 34.12 square miles of coastal/marine waters in the North Coastal Watershed are evaluated in this report.   The status of each designated use is summarized below.

###### Aquatic Life Use

The Aquatic Life Use is supported when suitable habitat (including water quality) is available for sustaining a native, naturally diverse, community of aquatic flora and fauna.  Impairment of the Aquatic Life Use (non-support or partial support) may result from anthropogenic stressors that include point and/or nonpoint source(s) of pollution and hydrologic modification.  The status of the Aquatic Life Use in the North Coastal Watershed is as follows:

| Aquatic Life Use Summary - Inland | Aquatic Life Use Summary - Coastal and Marine |
| --- | --- |
| - 1.9 river miles support | - 10.74 square miles support |
| - 19.5 river miles partial support | - 0.8 square miles non-support |
| - 3.9 river miles non-support | - 22.58 square miles not assessed |
| - 12.0 river miles not assessed |  |

The Aquatic Life Use is supported for the entire length/area of six waterbodies in the North Coastal Watershed: Frost Fish Brook, Porter River, Waters River, Danvers River, Forest River, and Salem Sound. Additionally portions of the Bass and Crane rivers also support this use.

Multiple waterbodies in the North Coastal Watershed are not assessed for the Aquatic Life Use at this time including:

- Essex Bay System (Essex River and Essex Bay),

- Annisquam River System (Alewife Brook, Mill and Annisquam rivers, as well as Rockport and Gloucester harbors),

- Manchester Harbor,

- Bass River System (portions of the Bass River)

- Crane River System (portions of the Crane River),

- North River System (portions of Proctor Brook and the North River),

- Beverly Harbor,

- Salem Harbor,

- Marblehead Harbor,

- Nahant Bay,

- Saugus River System (portions of the Saugus River, Hawkes Brook, Pines River and Lynn Harbor).

The Aquatic Life Use is partially supported in three small streams (Cat Brook in the Manchester Harbor System, and Beaver and Crane brooks in the Crane River System) due low levels of dissolved oxygen (DO).  The low pH in Cat Brook also impaired this use.  Whether or not these conditions are naturally occurring merits further investigation.

The Aquatic Life Use in the North River System is not supported in portions of two brooks (Goldthwait and Proctor).  In the upper portion of Goldthwait Brook, hydromodification and habitat degradation impair (partial support) this use.  The combined effects of the Water Management Act (WMA) withdrawals (Eastman Gelatine Company, Peabody Water Department, Salem Country Club, Kearnwood Country Club) likely diminish instream flow thereby impacting aquatic communities.  Downstream from the Eastman Gelatine Company’s discharge, this use is not supported due to severe instream toxicity during stormwater events. The Aquatic Life Use in Proctor Brook is also assessed as non-support downstream from its confluence with Goldthwait Brook based on habitat quality limitations, nutrient enrichment, and a severely impaired benthic community.

Where assessed, the Aquatic Life Use in the Saugus River System is either partial or non-support.  In the freshwaters of the Saugus River System, low DO, slightly elevated nutrients, and low flow (habitat limitations) were documented. The Saugus River and Beaverdam Brook are impaired by streamflow and/or habitat limitations that are likely related, in part, to water withdrawals.  The Saugus River estuary is heavily influenced by several National Pollution Discharge Elimination System (NPDES) discharges: General Electric Lynn Company (GE Lynn), Refuse Energy Systems Company (RESCO) and the Lynn Water and Sewer Commission.  While the cumulative impacts of the GE Lynn and RESCO facility salt water intakes and thermal discharges on the biota of the Saugus River estuary are unknown, the Aquatic Life Use is evaluated as non-support for the following reasons:

- the facilities are located on opposite shores of the estuary,

- impingement reduction mechanisms for at least one of the GE salt water intakes did not return the organisms to the river,

- the potential for thermal barriers to passage/migration resulting from the cumulative effects of intakes and thermal discharges, and

- biocide residuals in the discharges (concentrations unknown).

The impacts of water withdrawals and wastewater discharges (both municipal and industrial) as well as stormwater runoff to aquatic life in the North Coastal Watershed need further investigation. Investigations into the impacts of water withdrawals on streamflow should be conducted prior to WMA permit review and/or reissuance. For facilities which generate power via once-through cooling water systems, a comprehensive review of the present NPDES permit requirements should be performed to identify areas within the permits that may need modification, and/or specific issues that are not currently being addressed.  In the North Coastal Watershed these facilities include the Eastman Gelatine Corporation, GE Company in Lynn, RESCOE, and USGenNE (formerly New England Power Salem Station).  Since the quality of some aquatic resources (fish, shellfish, etc.) within receiving waters is declining rapidly, the impact of once-through cooling needs to be re-examined during the next NPDES permit review/reissuance.  Analysis of the information gathered should include, at a minimum, review by the Environmental Protection Agency, Department of Environmental Protection, Division of Marine Fisheries, National Oceanographic and Atmospheric Administration and Coastal Zone Management.   Local watershed groups, because of their intimate relationship with the receiving waters in question, should actively participate in the public review process.

###### Fish Consumption Use

The Fish Consumption Use is supported when there are no pollutants present that result in unacceptable concentrations in edible portions of marketable fish or shellfish or for the recreational use of fish, shellfish, other aquatic life or wildlife for human consumption.  This use is assessed using the Department of Public Health’s (MA DPH) Fish Consumption Advisory List.  It should be noted, however, that in September 1994, the MA DPH issued a statewide Interim Freshwater Fish Advisory because of risks associated with elevated levels of mercury in certain species of freshwater fish.  The interim advisory recommends that “Pregnant women should be advised of the possible health risk from eating fish taken from Massachusetts fresh-waterbodies in order to prevent exposure of developing fetuses to mercury.”  The interim advisory does not include stocked trout or farm-raised fish sold commercially.  Because of the interim advisory, however, the assessment of the Fish Consumption Use is limited to those waterbodies specifically listed on the MA DPH Fish Consumption Advisory List where the use is non-support.  The status of the Fish Consumption Use is the North Coastal Watershed is as follows:

| Fish Consumption Use Summary - Inland | Fish Consumption Summary - Coastal and Marine |
| --- | --- |
| - 37.3 river miles not assessed | - 34.12 square miles not assessed |

With the exception of Chebacco Lake (see Lakes/Ponds) there are no site-specific MA DPH fish consumption advisories in place in the North Coastal Watershed.

###### Recreational Uses

The Primary Contact Recreational Use is supported when conditions are suitable for any recreation or other water activity during which there is prolonged and intimate contact with the water with a significant risk of ingestion.  Activities include, but are not limited to, wading, swimming, diving, surfing and water skiing. The status of the Primary Contact Recreational Use in the North Coastal Watershed is as follows:

| Primary Contact Use Summary - Inland | Primary Contact Use Summary - Coastal and Marine |
| --- | --- |
| - 3.8 river miles support - 20.9 river miles non-support | - 0.05 square miles partial support - 12.92 square miles non-support |
| - 12.6 river miles not assessed | - 21.15 square miles not assessed |

While 34% of the river miles and 62% of the coastal areas are not assessed for the Primary Contact Recreational Use, portions of only two rivers in the North Coastal Watershed were fully supported for this use.  These two rivers are:

- the lower 0.3 miles of the Bass River between the outlet of upper Shoe Pond to the outlet of lower Shoe Pond, and

- the upper 3.5 miles of the Saugus River between the Lynn Water & Sewer Commission diversion canal and the confluence with Hawkes Brook  (DEP/DWM data summary, Figure 1).

Multiple Combined Sewer Overflows (CSOs) have recently been removed from the North Coastal Watershed and water quality conditions are improving.  However, CSOs and failing septic systems still impair the Primary Contact Recreational Use in Gloucester, Beverly and Lynn harbors, Nahant Bay, and the Saugus River.  Two additional streams (Danvers River and Hawkes Brook) are suspected to receive flow from CSOs.

Fecal coliform bacteria levels were extremely elevated in the Saugus River Subwatershed.  Current remediation activities such as the Lynn Water and Sewer Commission CSO Consolidation Conduit and Storage/Treatment – Stacey Brook State Revolving Fund (SRF) project continue to reduce bacteria levels. Post implementation monitoring of such projects is recommended to document whether or not water quality improvements are being realized.  Organizations such as Salem Sound 2000 and the Saugus River Watershed Council have been instrumental in the identification of areas of concern.

Additionally, failing septic systems are known contributors of fecal coliform bacteria in the North Coastal Watershed.  The City of Gloucester is currently conducting town-wide Title V repairs and the Department of Public works has been correcting sewage problems in the City of Salem since 1994 (Appendix D).  Upon completion of these updates/upgrades additional fecal coliform bacteria sampling during wet and dry weather conditions is recommended.

The Secondary Contact Recreational Use is supported when conditions are suitable for any recreation or other water use during which contact with the water is either incidental or accidental.  These include, but are not limited to, fishing, boating and limited contact incident to shoreline activities. The status of the Secondary Contact Recreational Use in the North Coastal Watershed is as follows:

| Secondary Contact Use Summary - Inland | Secondary Contact Use Summary - Coastal and Marine |
| --- | --- |
| - 14.3 river miles support | - 11.99 square miles support |
| - 9.1 river miles partial support | - 0.98 square miles non-support |
| - 4.6 river miles non-support | - 21.15 square miles not assessed |
| - 9.3 river miles not assessed |  |

Thirty-eight percent of the river miles and 35% of the coastal areas assessed in the North Coastal Watershed fully support the Secondary Contact Recreational Use.  In addition to those areas that supported the Primary Contact Recreational Use, the Secondary Contact Recreational Use is supported in four streams and two coastal waterbodies:

- Beaver Brook and the majority of Crane Brook in the Crane River System,

- Beaverdam and Hawkes brooks in the Saugus River System,

- Nahant Bay, and

- Lynn Harbor.

In addition to elevated fecal coliform bacteria levels related to CSOs and failing septic systems, causes of impairment to the Secondary Contact Recreational Use include oil sheens, turbidity, and algae (mats blooms and filaments).

###### Aesthetics Use

The Aesthetics Use is supported when surface waters are free from pollutants in concentrations or combinations that settle to form objectionable deposits; float as debris, scum or other matter to form nuisances; produce objectionable odor, color, taste or turbidity; or produce undesirable or nuisance species of aquatic life. The status of the Aesthetics Use in the North Coastal Watershed is as follows:

| Aesthetics Use Summary - Inland | Aesthetics Use Summary - Coastal and Marine |
| --- | --- |
| - 10.3 river miles support | - 10.01 square miles support |
| - 7.8 river miles partial support | - 0.9 square miles non-support |
| - 4.8 river miles non-support | - 23.21 square miles not assessed |
| - 14.4 river miles not assessed |  |

The Aesthetics Use is supported in four freshwater streams including Frost Fish Brook in the Bass River System, portions of Crane Brook in the Crane River System, as well as Beaverdam Brook and the Saugus River (from the diversion canal to the Saugus Iron Works) in the Saugus River System.  The only coastal water assessed as supporting this use is Salem Sound.

The Aesthetics Use is impaired in a total of four freshwater streams and three coastal areas including:

- a portion of Crane Brook and the Crane River in the Crane River System,

- a portion of Proctor Brook in the North River System,

- the Saugus River from the outlet of Lake Quannapowitt to the diversion canal in the Saugus River System,

- a portion of the upper Porter River (from the Route 62 bridge to the Route 128 bridge) in the Bass River System,

- the Waters River in the Crane River System, and

- the Saugus River estuary (from the Iron Works to the Lynn Harbor) in the Saugus River System.

Common causes of impairment to the Aesthetics Use include sedimentation, turbidity/grey water, trash and debris and algal blooms (DEP/DWM data summary, Figure 1).  Surface oil sheens related to the GE Lynn stormwater outfalls impair the Aesthetics Use in the Saugus River estuary.

###### Shellfishing Use

The Shellfishing Use is supported when shellfish harvested from approved Open Shellfish Areas (Class SA) are suitable for consumption without depuration and shellfish harvested from approved Restricted Shellfish Areas (Class SB) are suitable for consumption with depuration.  The Division of Marine Fisheries (DMF) classifies shellfishing areas in the North Coastal Watershed. The Shellfishing Use for this report was assessed using the DMF shellfishing closure listing dated 1January 1999. The status of the 132,742.1 acres of shellfishing beds in the entire North Coastal Watershed (including areas that extend into open-water not specifically included in this assessment report) is as follows:

| DMF Classification Type | DEP Use Support Status | DMF Area (acres) | % of total acreage |
| --- | --- | --- | --- |
| Approved | Support | 18777.44 | 14% |
| Conditionally Approved | Partial Support | 2575.16 | 2% |
| Management Closure | Not assessed | 45720.53 | 34% |
| Prohibited | Non-support | 65668.92 | 49% |

Individual DMF management area classifications are provided in Appendix F of this report.  It should be noted that their areas are defined in acres of shellfishing habitat.

## LAKES/PONDS

Overall use support status and trophic status of the lakes surveyed in the North Coastal Watershed are presented in Tables 1 and 2, respectively.  It should be noted that lakes, or portions of lakes, are listed as undetermined when indicators were not readily observable.  With this approach, the assessment of lakes in the North Coastal Watershed is limited to a "best case" picture (i.e., only the most obvious impairments are reported).  Potentially more of the lake acreage would be listed as impaired or in a more enriched trophic status if more variables were measured and more criteria assessed.

Table 1.  North Coastal Watershed Lakes Use Support Summary Summer, 1997.  (In Acres)

| USE/DEGREE SUPPORTED | SUPPORT (THREATENED) | PARTIAL SUPPORT | NON-SUPPORT | NOT ASSESSED |
| --- | --- | --- | --- | --- |
| Aquatic Life | 0 | 545 | 5 | 1550 |
| Fish Consumption* | 0 | 0 | 204 | 1896 |
| Swimmable | 0 | 593 | 249 | 1258 |
| Secondary Contact | 1252 | 593 | 249 | 6 |
| Aesthetics | 1252 | 593 | 249 | 6 |

* Fish Consumption results do not include the statewide consumption advisory (i.e., they are site-specific advisories as reported by the Department of Public Health (1998).

N.B. - These results represent the most recent assessments of lakes/ponds in the North Coastal Watershed.  These data also represent about 48% (43 of 89) of the lakes/ponds in the North Coastal Watershed and about 87% (2,100 of 2,415) of the acreage.

Table 2.  North Coastal Watershed Lakes Trophic Status Summary Summer, 1997.

| TROPHIC STATUS | NUMBER OF LAKES | ACRES |
| --- | --- | --- |
| Oligotrophic | 0 | 0 |
| Mesotrophic | 4 | 177 |
| Eutrophic | 17 | 930 |
| Hypereutrophic | 4 | 44 |
| Dystrophic | 0 | 0 |
| Undetermined/ Not Attainable | 18 | 949 |
| Total | 43 | 2100 |

N.B. - These results represent the most recent assessments of lakes/ponds in the North Coastal Watershed.  These data also represent about 48% (43 of 89) of the lakes/ponds in the North Coastal Watershed and about 87% (2,100 of 2,415) of the acreage.

Despite the "best case" scenario that is favored by the North Coastal Watershed lake assessment approach, 49% (representing 46% of the surveyed area) of the lakes showed severe (eutrophic or hypereutrophic) symptoms of succession.  Presumably, additional testing of dissolved oxygen, chlorophyll, and/or nutrients would corroborate that trophic status conditions are this advanced.

Four non-native aquatic plant species were found infrequently in the lakes of the North Coastal Watershed. Neither South American waterweed nor water hyacinth are likely to be immediate threats to spread from their current locations.  The estuarine environment and northern climate should keep them in check.  Fanwort (Cabomba caroliniana) and curly leaf pondweed (Potamogeton crispus) , however, are particularly invasive species that can spread vegetatively via cuttings that may float downstream or be transported mechanically between lakes.

The most frequently occurring non-native wetland species was purple loosestrife (Lythrum salicaria).  Populations of this plant are pervasive throughout the entire watershed.  Its presence was recorded at over three-quarters (77%) of the lakes.  The other non-native wetland plant observed in the watershed is the reed grass (Phragmites australis.). The two non-native wetland species were co-located at three lakes (Cape Pond, Rockport, Floating Bridge Pond, Lynn, and Niles Pond, Gloucester).

Based on the data collected by DEP DWM’s fish contaminant survey, the MA DPH issued a fish consumption advisory for Chebacco Lake in Essex/Hamilton.  Therefore the Fish Consumption Use is not supported for the entire 204 acres of this lake.

Intentionally left blank

North Coastal Watershed 1997/1998 Water Quality Assessment Report xi

93exec.docDWM CN 17.0

Figure 1. 1997/1998 DEP DWM water quality, macroinvertebrate and fish consumption station data summary in the North Coastal Watershed

NORTH COASTAL WATERSHED 1997/1998 DEP DWM

Water Quality, Macroinvertebrate and Fish Consumption Station Data Summary