RADON SCREENING SURVEY Placentino Elementary School 235 Woodland Street Holliston, MA Prepared by: Massachusetts Department of Public Health Bureau of Environmental Health Indoor Air Quality Program Radon Assessment Unit April, 2019

# Background

| Dates of Assessment: | 3/25/2019 through 3/27/2019 |
| --- | --- |
| Building Name: | Placentino Elementary School |
| Address: | 235 Woodland Street, Holliston, MA |
| Assessment Requested by: | Holliston Public School Dept. |
| Massachusetts Department of Public Health Bureau of Environmental Health (MDPH/BEH) Staff Conducting Assessment: | Lisa A. Hébert, Senior Technical Radon Specialist, Indoor Air Quality (IAQ) Program, Radon Unit |

# Methods

Radon samples were collected for the purpose of this radon survey using the United States Environmental Protection Agency (US EPA) Method #402-R-92-004 Liquid Scintillation (LS). Sampling devices were obtained through AccuStar Labs, a certified radon laboratory. Ninety-four devices were deployed throughout the building for a minimum 48-hour time period. Devices were deployed on 3/25/2019 and collected on 3/27/2019.

# Results

Tests were taken during normal operations at Placentino Elementary School. Of the 94 areas where test devices were deployed, devices from 93 areas were collected and submitted to AccuStar Labs. The lab results from the survey appear in Appendix A.

# Discussion

Radon Unit staff reviewed the lab results from AccuStar Labs and found all quality assurance parameters within acceptable limits. Appendix A shows that radon levels in the building ranged from less than 0.4 pCi/L to 5.2 pCi/L. One area screened had a radon level above the US EPA action guideline of 4 picocuries per liter of air pCi/L (Storage room on Peppy’s Path). Radon levels were not elevated elsewhere. Please note that one vial could not be submitted due to being disturbed/ tampered. Some areas could not be tested due to factors such as elevated temperatures, lack of access. These areas were:

•	Room 1 - (Vial # 4012093)

•	Electrical Room on Peppy’s Path

•	Room 113 (inner)

Nurse sick bay (small inner room)

•	Electrical Room in Boiler Room

•	Transformer room near room 11

•	Electrical Room near server room

In addition, two rooms had issues with open windows:

Timeout area between rooms 10 and 12. Windows were found open on 3/25/2019. Windows were closed and test was initiated.

Room 216 – Windows were found open on day of collection, 3/27/2019.

Both of these rooms should be retested in the future.

## Radon Gas

According to the National Research Council (NRC, 2009), “low levels of radon are present in all the air we breathe.” Radon is a naturally occurring, radioactive gas that is produced by the natural decay of uranium in the soil. The average outdoor radon level is about 0.4 pCi/L (US EPA, 2009). Once radon is formed, it migrates through various pathways in the soil and can enter a building through cracks, holes, and joints in a building’s foundation (US EPA, 2009). The EPA recommends mitigation for indoor radon levels at or above 4 pCi/L.

According to the US EPA (2009), radon gas “decays into radioactive particles that can get trapped in your lungs when you breathe. As they break down further, these particles release small bursts of energy.” This activity can damage your lung tissue and increase a person’s risk of developing lung cancer (US EPA, 2009). Radon is the leading cause of lung cancer in non-smokers. In 2005, the U.S. Surgeon General issued a health advisory stating that indoor radon is the second leading cause of lung cancer (US EPA, 2013).

# Recommendations

Based on the measurements and observations made during the visit, the following recommendations are made.

It would be prudent to contact certified radon measurement and mitigation specialists to confirm the results found and to assess any occupied rooms above the room in question and to mitigate any elevated areas. Untested rooms above unoccupied or untested rooms should be assessed as well. The two organizations that certify radon measurement and mitigation specialists are:

  - National Radon Safety Board (NRSB) www.nrsb.org

  - American Association of Radon Scientists and Technologists (AARST) http://aarst-nrpp.com/wp/

It would be prudent for your radon measurement specialist to test areas with disturbed/ tampered sampling devices and untested areas during confirmation testing if possible. These areas include:

Room 1

Room 113 (inner)

Nurse sick bay (small inner room)

Timeout area between rooms 10 and 12

Room 216

Electrical Room in Boiler Room

Electrical Room on Peppy’s Path

Electrical Room near server room

Transformer room near room 11

Refer to resource manual and other related IAQ documents located on the MDPH’s website for further building-wide evaluations and advice on maintaining public buildings. These documents are available at: http://mass.gov/dph/iaq.

If you have any questions or concerns, please call the Massachusetts Department of Public Health Radon Assessment Unit at (413) 586-7525.

References

National Research Council (MRC). 1999. Risk Assessment of Radon in Drinking Water, National Academy Press.

United States Environmental Protection Agency (US EPA). 2009. “A Citizen’s Guide to Radon. The Guide To Protecting Yourself and Your Family From Radon”. US Environmental Protection Agency. EPA402/K-09/001. January 2009.

United States Environmental Protection Agency (US EPA). 2013. “Home Buyers and Seller’s Guide to Radon”. US Environmental Protection Agency. EPA402/K-13/002. September 2013.

# Appendix A

# Radon Survey Results

NELAC NY 11769

NRPP 103216 AL NRSB ARL0017

# Laboratory Report for:	Property Tested: Dr. Bradford L. Jackson, Superintendent

EPA Method #402-R-92-004

Liquid Scintillation NRPP Device Code 8088 NRSB Device Code 12193

# Holliston Public Schools 370 Hollis Street

Placentino Elementary School 235 Woodland Street

Holliston  MA	01746	Holliston MA  01746

| Log Number | Device Number | Result Test Exposure Duration: Area Tested pCi/L |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2460860 | 3228337 | 03/25/2019 | 3:40 pm | 03/27/2019 | 4:45 pm | Main Office | 0.7 |
| 2462061 | 3228338 | 03/25/2019 | 3:40 pm | 03/27/2019 | 4:50 pm | Main Office Copy Area | 0.8 |
| 2462062 | 3228339 | 03/25/2019 | 3:45 pm | 03/27/2019 | 4:50 pm | Principal | 0.5 |
| 2462063 | 3228340 | 03/25/2019 | 3:45 pm | 03/27/2019 | 4:50 pm | Assistant Principal | 0.5 |
| 2462064 | 3228341 | 03/25/2019 | 3:45 pm | 03/27/2019 | 4:55 pm | Conference Room | 0.7 |
| 2462065 | 3228342 | 03/25/2019 | 3:50 pm | 03/27/2019 | 4:55 pm | Nurse | 0.7 |
| 2462066 | 3228343 | 03/25/2019 | 3:50 pm | 03/27/2019 | 4:55 pm | Nurse Sick Bay | 0.5 |
| 2462067 | 3228345 | 03/25/2019 | 3:55 pm | 03/27/2019 | 5:00 pm | Counsling Lobby | 0.6 |
| 2462068 | 3228346 | 03/25/2019 | 3:55 pm | 03/27/2019 | 5:00 pm | Mrs. Dalton | 0.5 |
| 2462069 | 3228347 | 03/25/2019 | 4:00 pm | 03/27/2019 | 5:00 pm | Counselor Cohen | 0.6 |
| 2462070 | 3228348 | 03/25/2019 | 4:00 pm | 03/27/2019 | 5:00 pm | Chill Zone | 0.6 |

## Comment:

### A copy of this report was emailed to lisa.hebert@state.ma.us.

Test Performed By: Lisa Hebert

Distributed by: Massachusetts Dept of Public Health-Radon Program

Date Received: 03/29/2019

Date Logged:

03/29/2019

Date Analyzed: 03/30/2019

Date Reported:

04/01/2019

Disclaimer:

Report Reviewed By: _   ____   ___   ___

Report Approved By: ___   ___   ___   ____   __

Shawn Price, Director of Laboratory Operations, AccuStar Labs

The uncertainty of this radon measurement is ~+/- 10 %. Factors contributing to uncertainty include statistical variations, daily and seasonal variations in radon concentrations, sample collection techniques and operation of the dwelling. Interference with test conditions may influence the test results.

This report may only be transferred to a third party in its entirety. Analytical results relate to the samples AS RECEIVED BY THE LABORATORY. Results shown on  this report represent levels of radon gas measured between the dates shown in the room or area of the site identified above as "Property Tested". Incorrect information will affect results. The results may not be construed as either predictive or supportive of measurements conducted in any area of this structure at any other time. AccuStar Labs, its employees and agents are not responsible for the consequences of any action taken or not taken based upon the results reported or any verbal or written interpretation of the results.

NELAC NY 11769

NRPP 103216 AL NRSB ARL0017

# Laboratory Report for:	Property Tested: Dr. Bradford L. Jackson, Superintendent

EPA Method #402-R-92-004

Liquid Scintillation NRPP Device Code 8088 NRSB Device Code 12193

# Holliston Public Schools 370 Hollis Street

Placentino Elementary School 235 Woodland Street

Holliston  MA	01746	Holliston MA  01746

| Log Number | Device Number | Result Test Exposure Duration: Area Tested pCi/L |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2462071 | 3228349 | 03/25/2019 | 4:05 pm | 03/27/2019 | 5:05 pm | Conference Special Education | 0.6 |
| 2462072 | 3228350 | 03/25/2019 | 4:05 pm | 03/27/2019 | 5:05 pm | Room 102 | 1.5 |
| 2462073 | 3228351 | 03/25/2019 | 4:10 pm | 03/27/2019 | 5:05 pm | Room 102 Inner Office | 1.9 |
| 2462074 | 3228352 | 03/25/2019 | 4:15 pm | 03/27/2019 | 5:05 pm | Teacher's Lounge | 0.7 |
| 2462075 | 3228353 | 03/25/2019 | 4:15 pm | 03/27/2019 | 5:10 pm | Library Reading Room | 0.4 |
| 2462076 | 3228354 | 03/25/2019 | 4:20 pm | 03/27/2019 | 5:10 pm | Library | 0.5 |
| 2462077 | 3228355 | 03/25/2019 | 3:45 pm | 03/27/2019 | 4:50 pm | QA-1 | 0.7 |
| 2462078 | 3228356 | 03/25/2019 | 3:45 pm | 03/27/2019 | 4:50 pm | QA-2 | 0.5 |
| 2462079 | 3228357 | 03/25/2019 | 4:20 pm | 03/27/2019 | 5:10 pm | QA-3 | < 0.4 |
| 2462080 | 3228358 | 03/25/2019 | 4:25 pm | 03/27/2019 | 5:15 pm | Library Work Room | < 0.4 |
| 2462081 | 3228359 | 03/25/2019 | 4:25 pm | 03/27/2019 | 5:15 pm | Work Room Storage L | 0.5 |

## Comment:

### A copy of this report was emailed to lisa.hebert@state.ma.us.

Test Performed By: Lisa Hebert

Distributed by: Massachusetts Dept of Public Health-Radon Program

Date Received: 03/29/2019

Date Logged:

03/29/2019

Date Analyzed: 03/30/2019

Date Reported:

04/01/2019

Disclaimer:

Report Reviewed By: _   ____   ___   ___

Report Approved By: ___   ___   ___   ____   __

Shawn Price, Director of Laboratory Operations, AccuStar Labs

The uncertainty of this radon measurement is ~+/- 10 %. Factors contributing to uncertainty include statistical variations, daily and seasonal variations in radon concentrations, sample collection techniques and operation of the dwelling. Interference with test conditions may influence the test results.

This report may only be transferred to a third party in its entirety. Analytical results relate to the samples AS RECEIVED BY THE LABORATORY. Results shown on  this report represent levels of radon gas measured between the dates shown in the room or area of the site identified above as "Property Tested". Incorrect information will affect results. The results may not be construed as either predictive or supportive of measurements conducted in any area of this structure at any other time. AccuStar Labs, its employees and agents are not responsible for the consequences of any action taken or not taken based upon the results reported or any verbal or written interpretation of the results.

NELAC NY 11769

NRPP 103216 AL NRSB ARL0017

# Laboratory Report for:	Property Tested: Dr. Bradford L. Jackson, Superintendent

EPA Method #402-R-92-004

Liquid Scintillation NRPP Device Code 8088 NRSB Device Code 12193

# Holliston Public Schools 370 Hollis Street

Placentino Elementary School 235 Woodland Street

Holliston  MA	01746	Holliston MA  01746

### Log Number

Device Number

Test Exposure Duration:

Area Tested

Result

pCi/L

2462082  3228360

2462083  3228361

2462084  3228362

2462085  3228363

2462086  3228364

2462087  3228365

2462088  3228366

2462089  3228368

2462090  3228369

2462091  3228370

2462092  3228371

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

4:25 pm

4:30 pm

4:30 pm

4:30 pm

4:30 pm

4:35 pm

4:35 pm

4:40 pm

4:40 pm

4:45 pm

4:45 pm

03/27/2019  5:20 pm

03/27/2019  5:25 pm

03/27/2019  5:25 pm

03/27/2019  5:25 pm

03/27/2019  5:25 pm

03/27/2019  5:25 pm

03/27/2019  5:30 pm

03/27/2019  5:30 pm

03/27/2019  5:30 pm

03/27/2019  5:35 pm

03/27/2019  5:35 pm

Work Room Storage R Room 129 Storage Electrical Room

Room 127

Room 128

Room 126

Room 125

Room 2

Time Out Room Room 4

Room 3

0.6

< 0.4

0.4

0.4

0.6

< 0.4

< 0.4

< 0.4

0.4

< 0.4

< 0.4

Comment:

A copy of this report was emailed to lisa.hebert@state.ma.us.

Test Performed By: Lisa Hebert

Distributed by: Massachusetts Dept of Public Health-Radon Program

Date Received: 03/29/2019

Date Logged:

03/29/2019

Date Analyzed: 03/30/2019

Date Reported:

04/01/2019

Disclaimer:

Report Reviewed By: _   ____   ___   ___

Report Approved By: ___   ___   ___   ____   __

Shawn Price, Director of Laboratory Operations, AccuStar Labs

The uncertainty of this radon measurement is ~+/- 10 %. Factors contributing to uncertainty include statistical variations, daily and seasonal variations in radon concentrations, sample collection techniques and operation of the dwelling. Interference with test conditions may influence the test results.

This report may only be transferred to a third party in its entirety. Analytical results relate to the samples AS RECEIVED BY THE LABORATORY. Results shown on  this report represent levels of radon gas measured between the dates shown in the room or area of the site identified above as "Property Tested". Incorrect information will affect results. The results may not be construed as either predictive or supportive of measurements conducted in any area of this structure at any other time. AccuStar Labs, its employees and agents are not responsible for the consequences of any action taken or not taken based upon the results reported or any verbal or written interpretation of the results.

NELAC NY 11769

NRPP 103216 AL NRSB ARL0017

# Laboratory Report for:	Property Tested: Dr. Bradford L. Jackson, Superintendent

EPA Method #402-R-92-004

Liquid Scintillation NRPP Device Code 8088 NRSB Device Code 12193

# Holliston Public Schools 370 Hollis Street

Placentino Elementary School 235 Woodland Street

Holliston  MA	01746	Holliston MA  01746

### Log Number

Device Number

Test Exposure Duration:

Area Tested

Result

pCi/L

2462093  3228372

2462094  3228373

2462095  3228374

2462096  3228375

2462097  3228376

2462098  3228377

2462099  3228378

2462100  3228379

2462101  3228380

2462102  3228381

2462103  3228382

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

4:50 pm

4:50 pm

4:55 pm

4:55 pm

4:25 pm

4:45 pm

4:55 pm

4:55 pm

5:00 pm

5:00 pm

5:05 pm

03/27/2019  5:40 pm

03/27/2019  5:40 pm

03/27/2019  5:40 pm

03/27/2019  5:40 pm

03/27/2019  5:15 pm

03/27/2019  5:35 pm

03/27/2019  5:40 pm

03/27/2019  5:45 pm

03/27/2019  5:50 pm

03/27/2019  5:55 pm

03/27/2019  5:55 pm

Room 6

Time Out Room Room 8

Room 5 QA-4 QA-5 QA-6

Time Out Room Room 7

Room 124 Server Room

0.5

0.5

0.6

< 0.4

0.4

< 0.4

< 0.4

< 0.4

0.4

0.4

0.7

Comment:

A copy of this report was emailed to lisa.hebert@state.ma.us.

Test Performed By: Lisa Hebert

Distributed by: Massachusetts Dept of Public Health-Radon Program

Date Received: 03/29/2019

Date Logged:

03/29/2019

Date Analyzed: 03/30/2019

Date Reported:

04/01/2019

Disclaimer:

Report Reviewed By: _   ____   ___   ___

Report Approved By: ___   ___   ___   ____   __

Shawn Price, Director of Laboratory Operations, AccuStar Labs

The uncertainty of this radon measurement is ~+/- 10 %. Factors contributing to uncertainty include statistical variations, daily and seasonal variations in radon concentrations, sample collection techniques and operation of the dwelling. Interference with test conditions may influence the test results.

This report may only be transferred to a third party in its entirety. Analytical results relate to the samples AS RECEIVED BY THE LABORATORY. Results shown on  this report represent levels of radon gas measured between the dates shown in the room or area of the site identified above as "Property Tested". Incorrect information will affect results. The results may not be construed as either predictive or supportive of measurements conducted in any area of this structure at any other time. AccuStar Labs, its employees and agents are not responsible for the consequences of any action taken or not taken based upon the results reported or any verbal or written interpretation of the results.

NELAC NY 11769

NRPP 103216 AL NRSB ARL0017

# Laboratory Report for:	Property Tested: Dr. Bradford L. Jackson, Superintendent

EPA Method #402-R-92-004

Liquid Scintillation NRPP Device Code 8088 NRSB Device Code 12193

# Holliston Public Schools 370 Hollis Street

Placentino Elementary School 235 Woodland Street

Holliston  MA	01746	Holliston MA  01746

| Log Number | Device Number | Result Test Exposure Duration: Area Tested pCi/L |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2462104 | 3228383 | 03/25/2019 | 5:05 pm | 03/27/2019 | 5:55 pm | Room 123 | 1.2 |
| 2462105 | 3228384 | 03/25/2019 | 5:10 pm | 03/27/2019 | 6:00 pm | Room 10 | 0.6 |
| 2462106 | 3228385 | 03/25/2019 | 5:15 pm | 03/27/2019 | 6:00 pm | Time Out Room | 0.6 |
| 2462107 | 3228386 | 03/25/2019 | 5:15 pm | 03/27/2019 | 6:00 pm | Room 12 | 0.4 |
| 2462108 | 3228387 | 03/25/2019 | 5:15 pm | 03/27/2019 | 6:05 pm | Room 9 | 0.6 |
| 2462109 | 3228388 | 03/25/2019 | 5:20 pm | 03/27/2019 | 6:05 pm | Time Out Room | 1.7 |
| 2462110 | 3228389 | 03/25/2019 | 5:20 pm | 03/27/2019 | 6:05 pm | Room 11 | 0.8 |
| 2462111 | 3228390 | 03/25/2019 | 5:20 pm | 03/27/2019 | 6:05 pm | Storage Across From Room 121 | 0.8 |
| 2462112 | 3228391 | 03/25/2019 | 5:25 pm | 03/27/2019 | 6:10 pm | Room 122 | 0.7 |
| 2462113 | 3228392 | 03/25/2019 | 5:25 pm | 03/27/2019 | 6:10 pm | Room 121 | 0.6 |
| 2462114 | 3228393 | 03/25/2019 | 5:30 pm | 03/27/2019 | 6:10 pm | Room 14 | 0.9 |

## Comment:

### A copy of this report was emailed to lisa.hebert@state.ma.us.

Test Performed By: Lisa Hebert

Distributed by: Massachusetts Dept of Public Health-Radon Program

Date Received: 03/29/2019

Date Logged:

03/29/2019

Date Analyzed: 03/30/2019

Date Reported:

04/01/2019

Disclaimer:

Report Reviewed By: _   ____   ___   ___

Report Approved By: ___   ___   ___   ____   __

Shawn Price, Director of Laboratory Operations, AccuStar Labs

The uncertainty of this radon measurement is ~+/- 10 %. Factors contributing to uncertainty include statistical variations, daily and seasonal variations in radon concentrations, sample collection techniques and operation of the dwelling. Interference with test conditions may influence the test results.

This report may only be transferred to a third party in its entirety. Analytical results relate to the samples AS RECEIVED BY THE LABORATORY. Results shown on  this report represent levels of radon gas measured between the dates shown in the room or area of the site identified above as "Property Tested". Incorrect information will affect results. The results may not be construed as either predictive or supportive of measurements conducted in any area of this structure at any other time. AccuStar Labs, its employees and agents are not responsible for the consequences of any action taken or not taken based upon the results reported or any verbal or written interpretation of the results.

NELAC NY 11769

NRPP 103216 AL NRSB ARL0017

# Laboratory Report for:	Property Tested: Dr. Bradford L. Jackson, Superintendent

EPA Method #402-R-92-004

Liquid Scintillation NRPP Device Code 8088 NRSB Device Code 12193

# Holliston Public Schools 370 Hollis Street

Placentino Elementary School 235 Woodland Street

Holliston  MA	01746	Holliston MA  01746

### Log Number

Device Number

Test Exposure Duration:

Area Tested

Result

pCi/L

2462115  3228394

2462116  3228395

2462117  3228396

2462118  3228397

2462119  3228398

2462120  3228399

2462121  3228400

2462122  4012045

2462123  4012046

2462124  4012047

2462125  4012048

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

5:30 pm

5:30 pm

5:00 pm

5:05 pm

5:05 pm

5:35 pm

5:35 pm

5:40 pm

5:45 pm

5:45 pm

5:45 pm

03/27/2019  6:10 pm

03/27/2019  6:10 pm

03/27/2019  5:50 pm

03/27/2019  5:55 pm

03/27/2019  6:15 pm

03/27/2019  6:15 pm

03/27/2019  6:20 pm

03/27/2019  6:20 pm

03/27/2019  6:25 pm

03/27/2019  6:25 pm

03/27/2019  6:25 pm

Time Out Room Room 16

QA-7

QA-8 QA-9

Room 13

Time Out Room Room 15

Room 114

Room 115

Room 116

1.9

0.9

< 0.4

1.3

< 0.4

0.6

0.4

0.8

< 0.4

0.5

0.5

Comment:

A copy of this report was emailed to lisa.hebert@state.ma.us.

Test Performed By: Lisa Hebert

Distributed by: Massachusetts Dept of Public Health-Radon Program

Date Received: 03/29/2019

Date Logged:

03/29/2019

Date Analyzed: 03/30/2019

Date Reported:

04/01/2019

Disclaimer:

Report Reviewed By: _   ____   ___   ___

Report Approved By: ___   ___   ___   ____   __

Shawn Price, Director of Laboratory Operations, AccuStar Labs

The uncertainty of this radon measurement is ~+/- 10 %. Factors contributing to uncertainty include statistical variations, daily and seasonal variations in radon concentrations, sample collection techniques and operation of the dwelling. Interference with test conditions may influence the test results.

This report may only be transferred to a third party in its entirety. Analytical results relate to the samples AS RECEIVED BY THE LABORATORY. Results shown on  this report represent levels of radon gas measured between the dates shown in the room or area of the site identified above as "Property Tested". Incorrect information will affect results. The results may not be construed as either predictive or supportive of measurements conducted in any area of this structure at any other time. AccuStar Labs, its employees and agents are not responsible for the consequences of any action taken or not taken based upon the results reported or any verbal or written interpretation of the results.

NELAC NY 11769

NRPP 103216 AL NRSB ARL0017

# Laboratory Report for:	Property Tested: Dr. Bradford L. Jackson, Superintendent

EPA Method #402-R-92-004

Liquid Scintillation NRPP Device Code 8088 NRSB Device Code 12193

# Holliston Public Schools 370 Hollis Street

Placentino Elementary School 235 Woodland Street

Holliston  MA	01746	Holliston MA  01746

| Log Number | Device Number | Result Test Exposure Duration: Area Tested pCi/L |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2462126 | 4012049 | 03/25/2019 | 5:50 pm | 03/27/2019 | 6:25 pm | Room 118 | 1.7 |
| 2462127 | 4012050 | 03/25/2019 | 5:50 pm | 03/27/2019 | 6:30 pm | Room 120 | 1.1 |
| 2462128 | 4012051 | 03/25/2019 | 5:55 pm | 03/27/2019 | 6:30 pm | Storage Room Peppy's Path | 5.2 |
| 2462129 | 4012052 | 03/25/2019 | 6:00 pm | 03/27/2019 | 6:35 pm | Room 119 | 0.9 |
| 2462130 | 4012053 | 03/25/2019 | 6:00 pm | 03/27/2019 | 6:35 pm | Room 117 | 1.2 |
| 2462131 | 4012054 | 03/25/2019 | 6:05 pm | 03/27/2019 | 6:35 pm | Custodial | 3.8 |
| 2462132 | 4012055 | 03/25/2019 | 6:10 pm | 03/27/2019 | 6:35 pm | Room 111 | 0.7 |
| 2462133 | 4012056 | 03/25/2019 | 6:10 pm | 03/27/2019 | 6:40 pm | Room 109 | 0.8 |
| 2462134 | 4012058 | 03/25/2019 | 6:15 pm | 03/27/2019 | 6:40 pm | Room 112 | 0.6 |
| 2462135 | 4012059 | 03/25/2019 | 6:15 pm | 03/27/2019 | 6:40 pm | Room 110 | 1.0 |
| 2462136 | 4012060 | 03/25/2019 | 6:20 pm | 03/27/2019 | 6:40 pm | QA-12 | < 0.4 |

## Comment:

### A copy of this report was emailed to lisa.hebert@state.ma.us.

Test Performed By: Lisa Hebert

Distributed by: Massachusetts Dept of Public Health-Radon Program

Date Received: 03/29/2019

Date Logged:

03/29/2019

Date Analyzed: 03/30/2019

Date Reported:

04/01/2019

Disclaimer:

Report Reviewed By: _   ____   ___   ___

Report Approved By: ___   ___   ___   ____   __

Shawn Price, Director of Laboratory Operations, AccuStar Labs

The uncertainty of this radon measurement is ~+/- 10 %. Factors contributing to uncertainty include statistical variations, daily and seasonal variations in radon concentrations, sample collection techniques and operation of the dwelling. Interference with test conditions may influence the test results.

This report may only be transferred to a third party in its entirety. Analytical results relate to the samples AS RECEIVED BY THE LABORATORY. Results shown on  this report represent levels of radon gas measured between the dates shown in the room or area of the site identified above as "Property Tested". Incorrect information will affect results. The results may not be construed as either predictive or supportive of measurements conducted in any area of this structure at any other time. AccuStar Labs, its employees and agents are not responsible for the consequences of any action taken or not taken based upon the results reported or any verbal or written interpretation of the results.

NELAC NY 11769

NRPP 103216 AL NRSB ARL0017

# Laboratory Report for:	Property Tested: Dr. Bradford L. Jackson, Superintendent

EPA Method #402-R-92-004

Liquid Scintillation NRPP Device Code 8088 NRSB Device Code 12193

# Holliston Public Schools 370 Hollis Street

Placentino Elementary School 235 Woodland Street

Holliston  MA	01746	Holliston MA  01746

### Log Number

Device Number

Test Exposure Duration:

Area Tested

Result

pCi/L

2462137  4012061

2462138  4012062

2462139  4012063

2462140  4012064

2462141  4012065

2462142  4012066

2462143  4012067

2462144  4012068

2462145  4012069

2462146  4012070

2462147  4012071

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

5:55 pm

6:00 pm

6:05 pm

6:20 pm

6:20 pm

6:25 pm

6:25 pm

6:30 pm

6:35 pm

6:35 pm

6:40 pm

03/27/2019  6:30 pm

03/27/2019  6:35 pm

03/27/2019  6:45 pm

03/27/2019  6:50 pm

03/27/2019  6:50 pm

03/27/2019  6:55 pm

03/27/2019  6:55 pm

03/27/2019  6:55 pm

03/27/2019  7:00 pm

03/27/2019  7:00 pm

03/27/2019  7:05 pm

QA-10 QA-11

Room 113

Room 107

Room 105

Room 108

Room 106

Room 104

Room 204

Room 203

Room 215

4.1

0.9

1.0

0.5

0.5

0.7

0.5

< 0.4

< 0.4

< 0.4

0.4

Comment:

A copy of this report was emailed to lisa.hebert@state.ma.us.

Test Performed By: Lisa Hebert

Distributed by: Massachusetts Dept of Public Health-Radon Program

Date Received: 03/29/2019

Date Logged:

03/29/2019

Date Analyzed: 03/30/2019

Date Reported:

04/01/2019

Disclaimer:

Report Reviewed By: _   ____   ___   ___

Report Approved By: ___   ___   ___   ____   __

Shawn Price, Director of Laboratory Operations, AccuStar Labs

The uncertainty of this radon measurement is ~+/- 10 %. Factors contributing to uncertainty include statistical variations, daily and seasonal variations in radon concentrations, sample collection techniques and operation of the dwelling. Interference with test conditions may influence the test results.

This report may only be transferred to a third party in its entirety. Analytical results relate to the samples AS RECEIVED BY THE LABORATORY. Results shown on  this report represent levels of radon gas measured between the dates shown in the room or area of the site identified above as "Property Tested". Incorrect information will affect results. The results may not be construed as either predictive or supportive of measurements conducted in any area of this structure at any other time. AccuStar Labs, its employees and agents are not responsible for the consequences of any action taken or not taken based upon the results reported or any verbal or written interpretation of the results.

NELAC NY 11769

NRPP 103216 AL NRSB ARL0017

# Laboratory Report for:	Property Tested: Dr. Bradford L. Jackson, Superintendent

EPA Method #402-R-92-004

Liquid Scintillation NRPP Device Code 8088 NRSB Device Code 12193

# Holliston Public Schools 370 Hollis Street

Placentino Elementary School 235 Woodland Street

Holliston  MA	01746	Holliston MA  01746

### Log Number

Device Number

Test Exposure Duration:

Area Tested

Result

pCi/L

2462148  4012072

2462149  4012073

2462150  4012074

2462151  4012075

2462152  4012076

2462153  4012077

2462154  4012078

2462155  4012079

2462156  4012080

2462157  4012081

2462158  4012082

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

6:40 pm

6:45 pm

6:50 pm

6:50 pm

6:55 pm

7:00 pm

7:00 pm

7:05 pm

7:05 pm

6:35 pm

6:35 pm

03/27/2019  7:10 pm

03/27/2019  7:15 pm

03/27/2019  7:15 pm

03/27/2019  7:20 pm

03/27/2019  7:45 pm

03/27/2019  7:45 pm

03/27/2019  7:45 pm

03/27/2019  7:45 pm

03/27/2019  7:50 pm

03/27/2019  7:00 pm

03/27/2019  7:00 pm

Room 216

Room 102

Room 102 Storage Music Room

Music Room Storage

Stage R Stage L Cafeteria R Cafeteria L QA-13

QA-14

0.5

< 0.4

1.9

0.4

0.9

0.6

0.6

0.5

< 0.4

< 0.4

< 0.4

Comment:

A copy of this report was emailed to lisa.hebert@state.ma.us.

Test Performed By: Lisa Hebert

Distributed by: Massachusetts Dept of Public Health-Radon Program

Date Received: 03/29/2019

Date Logged:

03/29/2019

Date Analyzed: 03/30/2019

Date Reported:

04/01/2019

Disclaimer:

Report Reviewed By: _   ____   ___   ___

Report Approved By: ___   ___   ___   ____   __

Shawn Price, Director of Laboratory Operations, AccuStar Labs

The uncertainty of this radon measurement is ~+/- 10 %. Factors contributing to uncertainty include statistical variations, daily and seasonal variations in radon concentrations, sample collection techniques and operation of the dwelling. Interference with test conditions may influence the test results.

This report may only be transferred to a third party in its entirety. Analytical results relate to the samples AS RECEIVED BY THE LABORATORY. Results shown on  this report represent levels of radon gas measured between the dates shown in the room or area of the site identified above as "Property Tested". Incorrect information will affect results. The results may not be construed as either predictive or supportive of measurements conducted in any area of this structure at any other time. AccuStar Labs, its employees and agents are not responsible for the consequences of any action taken or not taken based upon the results reported or any verbal or written interpretation of the results.

NELAC NY 11769

NRPP 103216 AL NRSB ARL0017

# Laboratory Report for:	Property Tested: Dr. Bradford L. Jackson, Superintendent

EPA Method #402-R-92-004

Liquid Scintillation NRPP Device Code 8088 NRSB Device Code 12193

# Holliston Public Schools 370 Hollis Street

Placentino Elementary School 235 Woodland Street

Holliston  MA	01746	Holliston MA  01746

### Log Number

Device Number

Test Exposure Duration:

Area Tested

Result

pCi/L

2462159  4012083

2462160  4012084

2462161  4012085

2462162  4012086

2462163  4012087

2462164  4012088

2462165  4012089

2462166  4012091

2462167  4012092

2462168  4012103

2462169  4012104

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

03/25/2019

7:05 pm

7:10 pm

7:15 pm

7:15 pm

7:20 pm

7:20 pm

7:25 pm

7:25 pm

7:30 pm

7:10 pm

7:20 pm

03/27/2019  7:50 pm

03/27/2019  7:50 pm

03/27/2019  7:55 pm

03/27/2019  7:55 pm

03/27/2019  8:00 pm

03/27/2019  8:00 pm

03/27/2019  7:55 pm

03/27/2019  7:55 pm

03/27/2019  8:00 pm

03/27/2019  7:50 pm

03/27/2019  8:00 pm

QA-15

Receiving Boiler Room

Generator Room Gym Office  Gym Storage Gym South  Gym North

Prep Area Kitchen QA-16

QA-17

< 0.4

1.0

< 0.4

< 0.4

< 0.4

0.8

0.4

0.4

0.5

0.8

0.4

Comment:

A copy of this report was emailed to lisa.hebert@state.ma.us.

Test Performed By: Lisa Hebert

Distributed by: Massachusetts Dept of Public Health-Radon Program

Date Received: 03/29/2019

Date Logged:

03/29/2019

Date Analyzed: 03/30/2019

Date Reported:

04/01/2019

Disclaimer:

Report Reviewed By: _   ____   ___   ___

Report Approved By: ___   ___   ___   ____   __

Shawn Price, Director of Laboratory Operations, AccuStar Labs

The uncertainty of this radon measurement is ~+/- 10 %. Factors contributing to uncertainty include statistical variations, daily and seasonal variations in radon concentrations, sample collection techniques and operation of the dwelling. Interference with test conditions may influence the test results.

This report may only be transferred to a third party in its entirety. Analytical results relate to the samples AS RECEIVED BY THE LABORATORY. Results shown on  this report represent levels of radon gas measured between the dates shown in the room or area of the site identified above as "Property Tested". Incorrect information will affect results. The results may not be construed as either predictive or supportive of measurements conducted in any area of this structure at any other time. AccuStar Labs, its employees and agents are not responsible for the consequences of any action taken or not taken based upon the results reported or any verbal or written interpretation of the results.

NELAC NY 11769

NRPP 103216 AL NRSB ARL0017

# Laboratory Report for:	Property Tested: Dr. Bradford L. Jackson, Superintendent

EPA Method #402-R-92-004

Liquid Scintillation NRPP Device Code 8088 NRSB Device Code 12193

# Holliston Public Schools 370 Hollis Street

Placentino Elementary School 235 Woodland Street

Holliston  MA	01746	Holliston MA  01746

### Log Number

Device Number

Test Exposure Duration:

Area Tested

Result

pCi/L

2462170  4012105

03/25/2019

7:30 pm

03/27/2019  8:05 pm

QA-18

< 0.4

Comment:

A copy of this report was emailed to lisa.hebert@state.ma.us.

Test Performed By: Lisa Hebert

Distributed by: Massachusetts Dept of Public Health-Radon Program

Date Received: 03/29/2019

Date Logged:

03/29/2019

Date Analyzed: 03/30/2019

Date Reported:

04/01/2019

Disclaimer:

Report Reviewed By: _   ____   ___   ___

Report Approved By: ___   ___   ___   ____   __

Shawn Price, Director of Laboratory Operations, AccuStar Labs

The uncertainty of this radon measurement is ~+/- 10 %. Factors contributing to uncertainty include statistical variations, daily and seasonal variations in radon concentrations, sample collection techniques and operation of the dwelling. Interference with test conditions may influence the test results.

This report may only be transferred to a third party in its entirety. Analytical results relate to the samples AS RECEIVED BY THE LABORATORY. Results shown on  this report represent levels of radon gas measured between the dates shown in the room or area of the site identified above as "Property Tested". Incorrect information will affect results. The results may not be construed as either predictive or supportive of measurements conducted in any area of this structure at any other time. AccuStar Labs, its employees and agents are not responsible for the consequences of any action taken or not taken based upon the results reported or any verbal or written interpretation of the results.