105 CMR 410.000: MINIMUM STANDARDS OF FITNESS FOR HUMAN HABITATION (STATE SANITARY CODE, CHAPTER II)

Section

General Administration

410.001: Purpose

410.002: Authority

410.003: Citation

410.010002: Scope

410.003: General Provisions

Definitions

410.020 010: Definitions

Building and Plumbing

410.100: Kitchen Facilities

410.150 110: Washbasins Bathroom Facilities: Sinks, Toilets, Tubs, and Showers

410.151: Shared Facilities

410.152.120: Privies and Chemical Approved Toilets Prohibited; Exceptions

410.180 130: Potable Water/Sanitary Drainage

410.350 140: Plumbing Connections

410.190 150: Hot Water

410.200 160: Heating Facilities Required Systems

410. 170: Venting

410.201 180: Temperature Requirements

410.202: Venting

410. 200: Provision and Metering of Electricity or Gas

410. 210: Provision of Oil

410.250: Habitable Rooms Other than Kitchen -- Natural Light and Electrical Outlets

410.251: Kitchen Lighting and Electrical Outlets

410.252: Bathroom Lighting and Electrical Outlets

410.253: Light Fixtures Other than in Habitable Rooms or Kitchens

410.254: Light in Passageways, Hallways, and Stairways

410.255: Amperage

410.256: Temporary Wiring

410.257: Light Obstructions

410.258: Exemption of Dwellings More than 600 Feet from Electrical Service

410.280220: Natural and Mechanical Ventilation

410.281: Ventilation Shut-off

410.230: Owner’s Installation and Maintenance Responsibilities

410.235: Owner’s Laundering Responsibilities

410.240: Occupant’s Installation and Maintenance Responsibilities

410.250: Asbestos-Containing Material

410.260: Means of Egress

410.270: Locks

Electrical and Fire

410.300: Electricity Supply and Illumination

410.310: Auxiliary Emergency Lighting Systems and Exit Signs

410.320: Electrical Service

410.330: Smoke Detectors and Carbon Monoxide Alarms

410.300: Sanitary Drainage System Required

410.350: Plumbing Connections

410.351: Owner's Installation and Maintenance Responsibilities

410.352: Occupant's Installation and Maintenance Responsibilities

410.353: Asbestos Material

410.354: Metering of Electricity, Gas and Water

Health and Safety

410.400: Owner/Manager Contact Information

410.410: Building Identification

410.420: Habitability Requirements

410.430: Natural Light and Obstructions

410.440: Temporary Housing and Cabins

410.450: Alternative Housing

410.460: Homeless Shelters

410.4760:  Existence of Lead-based Paint Hazards in Residences

410.400: Minimum Square Footage

410.401: Ceiling Height

410.402: Grade Level

410.430: Temporary Housing Allowed Only with Board of Health Permission

410.431: Any Exceptions to Minimum Standards Must Be Specified

410.450: Means of Egress

410.451: Egress Obstructions

410.452: Safe Condition

410.480: Locks

410.481: Posting of Name of Owner

410.482: Smoke Detectors and Carbon Monoxide Alarms

410.483: Auxillary Emergency Lighting Systems and Exit Signs

410.484: Building Identification

410.500: Owner's Responsibility to Maintain Building and Structural Elements

410.501: Weathertight Elements

410.502: Use of Lead Paint Prohibited

410.503: Protective Railings and Walls

410.504: Non-absorbent Surfaces

410.505 510: Occupant's Responsibility Respecting Regarding Building and Structural Elements

410. 5230: Protective Railings and Walls

410.501 5430: Weathertight Elements

410.5540: Installation of Screens

410.550: Extermination of Insects, Rodents and Skunks Elimination of Pests

410.551: Screens for Windows

410.552: Screens for Doors

410.560: Refuse	
410.600: Storage of Garbage and Rubbish

410.5076601: Refuse Collection of Garbage and Rubbish

410.602 570: Maintenance of Areas Free from Garbage and Rubbish Refuse in a Sanitary and Safe Condition

410.620: Curtailment Prohibited

410.700: Inspectors Duty to Classify Violations

410.800: General Administration

410.810: Access for Repairs and Alterations

Enforcement Procedures

410.820 600: Inspection upon Request

410.821 610: Inspection Form

410.822 620: Conduct of Inspections

410. 630: Conditions Deemed to Endanger or Impair Health or Safety

410.830 640: Correction Orders Timeframes for Correction of Violations

410.831 650: Dwellings Unfit for Human Habitation; Hearing; Condemnation; Order to Vacate; Demolition

410.660: Reinspections

410.832 670: Content of Orders Order to Correct Violations

410.833 680: Service of Orders and Notices

410.840 700: Variances

410.710: Permitting Requirements For Alternative Housing

410.850 800: Right to Hearing

410.851 810: Hearing Notice

410.852 820: Time for Hearing

410.853 830: Hearing Procedures

410.854 840: Final Decision after Hearing; Failure to Comply with Final Order

410.855 850: Official Hearing Review Record

410.860: Appeal of Final Decisions

410.950 900: Condemnation, Placarding and Vacating Dwellings Residences

410.900 910: Penalties for Interference with Inspections

410.910 920: Penalty for Failure to Comply with Order

410.920 930: Penalty for Other Offenses

410.960 940: Correction of Violations by Board of Health; Expenses

410.950: Severability

Appendix

Appendix A:  Summary of Legal Remedies

410.990: Appendix: Forms

410.001: Purpose

The purposes of 105 CMR 410.000 are to:

(A) Establish minimum standards for housing to protect the health, safety, and well-being of the occupants of housing and of the general public; ,

(B) Provide enforcement procedures for local boards of health to ensure compliance with these minimum standards; and

(C) to Ffacilitate the use of legal remedies available to occupants of substandard housing. , to assist boards of health in their enforcement of this code and to provide a method of notifying interested parties of violations of conditions which require immediate attention.

410.002: Authority

105 CMR 410.000 is adopted under authority of M.G.L. c. 111, §§ 3 and 127A.

410.003: Citation

105 CMR 410.000 shall be known, and may be cited, as 105 CMR 410.000:  State Sanitary Code Chapter II:  Minimum Standards of Fitness for Human Habitation.

410.010 002: Scope

(A) No person shall occupy, as owner-occupant, or let to another for occupancy, or allow a person to occupy, offer to rent, or occupy as owner-occupant any dwelling, dwelling unit, residence mobile dwelling unit, or rooming unit for the purpose of living, sleeping, cooking or eating therein, which does not comply with the requirements of 105 CMR 410.000.

(A) The provisions of 105 CMR 410.000 apply to all residences as defined in 105 CMR 410.010, unless otherwise specified in 105 CMR 410.000.  Applicable requirements of Massachusetts General Law, 780 CMR: Massachusetts State Building Code and other specialized codes shall be adhered to in the design, construction, and maintenance of buildings, structures and equipment.  Should a conflict exist between 105 CMR 410.000 and another agency’s regulation, the standard that is more protective of public health or safety shall apply.

(B) The provisions of 105 CMR 410.000 shall not apply to any dwelling, residence which:

(1) is located on a campground that is being operated in compliance with 105 CMR 420.000, 105 CMR 430.000, or 310 CMR 14.00; or (2) Is oOtherwise required to conform with to minimum housing habitation standards of fitness for human habitation elsewhere existing in specified in other chapters of the State Sanitary Code, or otherwise exempt by statute or;

(2)(3) Is uUsed exclusively as a civil defense shelter or a temporary overnight shelter in the case of an emergency; or

(3)  Is in any residential facility that is licensed Owned by an agency of the Commonwealth

(4) In any hospital, convalescent, nursing home, rest home, or assisted living facility licensed by the Department of Public Health in accordance with the provisions of M.G.L. c. 111, § 51 or 71 unless regulations pertaining to such facilities require compliance with 105 CMR 410.000;

; or

(5)(4) Is federally owned housing. On a federal military base or where enforcement is otherwise pre-empted by federal law.

(C) It is the duty of the local health official to identify violations and order correction of such violations pursuant to 105 CMR 410.640 through 410.680 and the legal obligation of the person to whom the order is issued to comply with such order.

(C) (D) Nothing contained herein shall be construed to limit or otherwise restrict any person from seeking judicial relief in a court of competent jurisdiction notwithstanding any hearing, proceeding, or other administrative remedy set forth in 105 CMR 410.000.

(E) The provisions of 105 CMR 400.000: State Sanitary Code, Chapter I shall govern the administration and enforcement of these minimum standards 105 CMR 410.000 except as supplemented by 105 CMR 410.600 through 410.950.

410.003: General Provisions

(A) No person shall occupy, or let to another for occupancy, or allow a person to occupy, offer to rent, or occupy as owner-occupant any residence for the purpose of living, sleeping, cooking or eating therein, which does not comply with the requirements of 105 CMR 410.000.

(B) Unless otherwise specified in 105 CMR 410.000, the owner is responsible for providing all maintenance, repairs, and equipment necessary to achieve compliance with 105 CMR 410.000.

(C) No owner or occupant shall cause any service, facility, equipment, or utility which is required to be made available by 105 CMR 410.000, or any optional service, facility, equipment, or utility provided by the owner, to be removed from or shut off from any occupied residence except for such temporary period as may be necessary during actual repairs or alterations and where reasonable notice of curtailment of service is given to the occupant, or during temporary emergencies when curtailment of service is approved by the board of health.  If any such service, facility, equipment, or utility becomes curtailed, the responsible party shall take immediate steps to cause its restoration. (See M.G.L. c. 186, § 14.)

(D) An owner may only remove optional equipment when a unit has been vacated or a tenancy has been terminated.

(E) (D) Every occupant of a  residence, upon reasonable notice and if possible by appointment, shall give the owner thereof, or his the owner’s representative, access to the residence  for the purpose of effecting compliance with the provisions of 105 CMR 410.000.  Access shall be at a reasonable time and shall include, but not be limited to, any cooperation required for repairs, alterations, pest elimination, and service of utilities. An owner shall provide at least 48 hours’ notice to the occupant, except for emergency repairs for which no notice is required.

410.020 010: Definitions

Affected Parties means the occupants if the petitioner is the owner, and the owner if the petitioner is the occupant of any residence on which an order to correct has been served.

Alternative Housing means a single family, owner-occupied dwelling residence, whether mobile or permanent, that is approved by a local board of health pursuant to 105 CMR 410.710 to alter standards set forth in 105 CMR 410.000 for heating, plumbing, electrical, and sanitary facilities and minimum square footage requirements in order to reduce energy use or environmental impact. used for permanent occupancy and designed to minimize the environmental impact when compared to traditional housing. Alternative housing also includes permanent structures known as Tiny Houses.

Asbestos means:

(1) chrysotile Chrysotile, amosite, crocidolite; or

(2) in In fibrous form, tremolite-asbestos, anthophyllite-asbestos, or actinolite-asbestos.

Asbestos-Containing Material (ACM) means asbestos or any material containing 1% or more asbestos as determined by a laboratory using protocols set forth in the “Method for the Determination of Asbestos in Bulk Building Materials” found in EPA report EPA/600/R-93/116, or another method as directed by the Massachusetts Department of Environmental Protection. The term includes, but is not limited to, sprayed-on and troweled-on materials applied to ceilings, walls, and other surfaces; insulation on pipes, boilers, tanks, ducts, and other equipment, structural and non-structural members; tiles; asphalt roofing or siding materials; or asbestos-containing paper.

Bed and Breakfast Operation Establishment means a private owner-occupied house where four or more rooms are let and a breakfast is included in the rent, and all accommodations are reserved in advance.

Bed and Breakfast Home means a private owner-occupied house where three or fewer rooms are let and a breakfast is included in the rent, and all accommodations are reserved in advance.

Board of Health  means the appropriate and legally designated health authority of the city, town, county, or other legally constituted governmental unit within the Commonwealth having the usual powers and duties of the board of health of a city or town, or his or its authorized agent or representative. a municipal board of health, a municipal health department, other municipal entity when acting under local authority to enforce the State Sanitary Code, a regional health district, or any other legally constituted city, town, or county governmental unit within the Commonwealth serving as a public health agency, as established under M.G.L c. 41, § 1, or M.G.L. c. 111, §§ 26, 26A, 27A, or 27B, or its authorized agent or representative under M.G.L. c. 111, § 30. Board of health shall also mean an inspectional services department in a city or town where the inspectional services department is responsible for the enforcement of 105 CMR 410.000.

Building and Structural Elements means any component of a residence including, but not limited to, the foundation, floors, walls, doors, windows, ceilings, roofs, gutters, soffits, siding, staircases, porches, decks, and chimneys.

Bulk Items means waste items which because of size or weight cannot be collected as part of routine household refuse collection and have been designated by the local community through rules and regulations to require specific disposal procedures. Bulk items shall include, but not be limited to, large appliances, furniture, large auto parts, stumps, and trees, as well as large branches or brush that exceeds local size restrictions for yard waste collection.

Cabin means a structure that provides basic shelter and contains at least one habitable room for living, sleeping, eating, cooking or sanitation that is intended to be occupied by a single family or household.  A cabin is intended for vacation, recreation, or leisure use in wilderness areas for intermittent periods of time.

Chemical Toilet means a self-contained toilet where material is held in a sealed tank, containing chemicals and deodorizers, and which is emptied on a periodic basis.

Chronic Dampness means the regular and/or periodic appearance of moisture, or water, or mold or evidence of such moisture or water as indicated by the presence of mold or other fungi.

Compliance with Accepted Standards means meeting all the requirements of 105 CMR 410.000 It shall also mean correcting any violations of 105 CMR 410.000 in a work-personlike fashion and restoring all parts of the dwelling, or unit thereof, to the condition they were in before occurrence of any such violations.  Compliance shall also mean in those cases where licenses or permits are required to perform work necessary to correct the violations, such as, but not limited to building, plumbing and wiring that the appropriate official certifies that the work has been completed in accordance with applicable laws and regulations.   and correcting any violation of 105 CMR 410.000 in a work person-like manner and in accordance with accepted building, plumbing, heating, gas fitting, electrical wiring standards, or advisories issued by the Department, so that the residence or cited item or component is returned to its intended function or use.  Materials and equipment shall be appropriate for the use intended and affected areas shall be left in a properly cleaned condition. When licenses or permits are required to perform the work necessary to correct the violation, including, but not limited to building, plumbing, wiring/electrical, heating, gas fitting, asbestos removal, lead-based paint abatement, and integrated elimination of pests management, compliance shall also mean that such licenses and permits have been obtained and that any conditions or requirements imposed by such licenses and permits have been met.

Compostable Material means an organic material excluding waste water treatment residuals which has the potential to be composted and which is pre-sorted and is not contaminated by significant amounts of toxic substances, as those terms are or may be defined by 310 CMR 19.00: Solid Waste Management.

Composting means a process of accelerated biodegradation and stabilization of organic material under controlled conditions yielding a product which can safely be used, as those terms are or may be defined by 310 CMR 19.00: Solid Waste Management.

A Condition Making a Unit Unfit for Human Habitation is a condition meeting the standard set forth in the Massachusetts General Laws M.G.L. c 111, §§127A and 127B under which a board of health may justify closing down, securing, condemning, or demolishing a residence. It shall mean a violation which poses such immediate harm or threat of harm to an occupant or to the public that other legal remedies cannot be reasonably expected to bring about removal of the condition with sufficient speed to prevent the serious harm or injury to the occupants or to the public.

A Condition Which May Endanger or Materially Impair the Health or Safety and/or Wellbeing of an Occupant means the existence of a condition, listed in 105 CMR 410.750 630 or any other condition so certified by the board of health to be a violation, which may expose or subject a person or persons to harm, or may materially impair the health, or safety, and/or the well-being of an occupant or the public.

Condominium means the land or the lessee’s interest in any lease of such land which is submitted pursuant to M.G.L. c. 183A, the building or buildings, all other improvements and structures thereon, and all easements, rights and appurtenances belonging thereto, which have been submitted pursuant to M.G.L. c. 183A.

Condominium Unit means a part of the condominium including one or more rooms, with appurtenant areas such as balconies, terraces, and storage lockers if any stipulated in the master deed as being owned by the unit owner, occupying one or more floors or a part or parts thereof, including the enclosed space therein, intended for any type of use, and with a direct exit to a street or way to a common area leading to a street or way.

Department means the Massachusetts Department of Public Health.

Dwelling means every building or structure shelter  including but not limited to rooming houses and temporary housing used for or intended for human habitation and every other structure or condition feature located within the physical boundaries same  of the lot line whose existence may causes or is likely to effect noncompliance with the provisions of 105 CMR 410.000.

Dwelling Unit means the room or group of rooms within a dwelling residence used or intended for use by an individual, one family, or household for living, sleeping, cooking, and eating, and other area of which the occupant has exclusive use. Dwelling unit shall also mean a condominium unit.

Entry Door of a Dwelling Residence means any door of a dwelling residence which provides access to the common areas of the dwelling residence from the exterior of the dwelling residence except that when there are two doors which enclose an entryway between the common areas of the dwelling residence and the exterior of the dwelling residence it shall mean either of the doors.

Entry Door of a Dwelling Unit means any door of a dwelling unit which provides access to the common areas of the dwelling residence or access to the outside of the dwelling residence.

Exterior Openable Window means any window designed and installed to open which opens to the common interior areas of the dwelling or to the outside of the dwelling residence.

Exterminate means to eliminate insects and rodents pests.

Fuel means the electricity or any combustible fuel such as oil, gas, coal, wood or biofuel pellets, or wood used in the provision of heat and/or hot water.

Garbage means the animal, vegetable or other organic waste resulting from the handling, preparing, cooking, consumption or cultivation of food, and containers and cans which have contained food unless such containers and cans have been cleaned or prepared for recycling.

Guard means a building component or a system of building components, including but not limited to, newel posts, upper and lower rails, and balusters, or a solid construction .located at or near the open sides of elevated walking surfaces that minimize the possibility of a fall from the walking surface to a lower level.

Habitable Room means every room or enclosed floor space used or intended to be used for living, sleeping, cooking, or eating purposes, excluding rooms containing toilets, bathtubs or showers and excluding laundries, pantries, foyers, communicating corridors, closets and storage spaces.

Handrail means a horizontal or sloping rail intended for grasping by the hand for guidance or support.

Historic building means any building covered by 105 CMR 410.000 which meets the definition of historic building as defined in 780 CMR 780 CMR 3409.0.

Central Heating System means:

(1) a system in which air or water is heated at a central furnace and that heats air or water which is distributed throughout the residence through vents, ducts, pipes, or radiators;

(2) It shall also mean is a series of hard-wired electrical heaters or heating/air conditioning units permanently installed and secured to a structural element within the residence; or

(3) gas-fired kitchen stoves with factory installed heaters.

Homeless Shelter means a residence operated by a service agency which provides temporary, overnight sleeping accommodations and offers transitional assistance to homeless individuals and families in need of permanent housing.

Humus/Composting Toilet means a self-contained system, approved in accordance with 310 CMR 15.000: Title 5 of the State Environmental Code, consisting of a composter with a separate toilet fixture from which no liquid or solid waste materials are discharged to the surface or subsurface environment and from which a humus/compost-like end product is produced.

Incinerating toilet means a self-contained waterless system that does not require connection to a sewer system or in ground septic system, except to dispose of graywater, approved in accordance with 248 CMR 10: Uniform State Plumbing Code.

Infestation means the recurrent presence of insects and/or rodents pests that may present a hazard cause disease or health risk to humans or pets, or damage to property, or the environment, or create a nuisance.

Integrated Pest Management means a systematic strategy for managing pests which considers prevention, avoidance, monitoring, and suppression. Where chemical pesticides are necessary, a preference is given to materials and methods which maximize public safety and reduce environmental health risk.

Legal Remedy means any common law and other rights provided guaranteed by judicial decision, or the laws or regulations of the Commonwealth of Massachusetts which are intended to protect the rights and interests of the occupants affected by violations of 105 CMR 410.000 whether such provision authorizes an affirmative civil action, criminal penalties, a defense to an action, or claim by another.

Manufactured Home means a structure that is ready for occupancy upon leaving the factory. It is built on and remains on a permanent chassis and is designed to be placed on a permanent foundation. It is designed for single family occupancy.  means a structure, built in conformance to the National Manufactured Home Construction and Safety Standards which is transportable in one or more sections, which in the traveling mode, is eight body feet or more in width or 40 body feet or more in length, or, when erected on site, is 320 or more square feet, and which is built on a permanent chassis and designed to be used as a dwelling unit with or without a permanent foundation when connected to the required utilities, and includes the plumbing, heating, air conditioning, and electrical systems contained therein.

Manufactured Home Park means any lot or track of land upon which three or more manufactured homes occupied for dwelling purposes are located, including buildings, structures, fixtures and equipment used in connection with the manufactured home and licensed under M.G.L c. 140 § 32B.

Means of Egress means a continuous and unobstructed path providing a route of exit from the interior of any occupied portion of a residence to a public way. of travel from the any point in a dwelling residence to an abutting public way (780 CMR Massachusetts Building Code).

Mechanical Ventilation means a system designed to remove air from a room or space to the outdoors by using a fan or other mechanical means and to provide active or passive make up air. and which terminates to the outdoors.

Mobile Dwelling Unit means a dwelling unit built on a chassis and containing electrical, plumbing, and sanitary facilities and designed to be installed on a temporary or permanent foundation for permanent living quarters.

(1) An auto home, as defined in M.G.L. c. 90 § 1;

(2) A house trailer, as defined in M.G.L. c. 90 § 1;

(3) A small, usually single-room dwelling (often called a “Tiny House”) when built on a chassis with wheels; or

(4) A house boat.

Natural Ventilation means air exchange provided by a non-mechanical means.  Sources of natural ventilation include, but are not limited to, windows or doors in exterior walls that are intended to supply or remove air from any space.

Occupant means every any person living or sleeping in a dwelling residence.

Representative or Occupant's Representative means any adult person designated and duly authorized to act on the occupant's behalf, including, but not limited to, any person or group designee from a tenant's organization or other community group.

Owner means every person who alone or severally with others:

(1) has Has legal title to any residence, dwelling, dwelling unit, mobile dwelling unit, or parcel of land, vacant or otherwise, including a manufactured home park; or

(2) has Has care, charge or control of any residence, dwelling, dwelling unit, mobile dwelling unit, or parcel of land, vacant or otherwise, including a manufactured home park, in any capacity including but not limited to agent, executor, executrix, administrator, administratrix, trustee or guardian of the estate of the holder of legal title; or

(3) is Is a mortgagee in possession of any such property; or

(4) is Is an agent, trustee or other person appointed by the courts and vested with possession or control of any such property; or

(5) is Is an officer or trustee of the association of unit owners of a condominium.  Each such person is bound to comply with the provisions of these minimum standards as if he were the owner. Owner also means every person who operates a rooming house.

Owner’s Representative means any adult person designated and duly authorized to act on the owner’s behalf to effect compliance with the provisions of 105 CMR 410.000.

Person means every individual, partnership, corporation, firm, association, or group, including a city, town, county or other governmental unit, owning property or carrying on an activity regulated by 105 CMR 410.000.

Pest means insects, arachnids, rodents or other animal life which have characteristics regarded as injurious, unwanted, or a nuisance, and which may also have the ability to cause damage, disease or act as a disease vector. any of the following that may cause disease, injury or other health risk to humans or pets, or act as a disease vector, or cause an infestation:

1) Insects, including but not limited to, mosquitoes, bees, flies, ants, and bed bugs, but 	excluding lice and dust mites;

2) Arachnids, including but not limited to, spiders, daddy long legs, scorpions, and ticks;

3) Rodents; or

4) Other unwanted animal life.

Potable Water means water from any source that has been approved by the Department of Environmental Protection or local board of health for human consumption.

Privy means a small room or outhouse building containing a toilet, usually with an earth closet, rather than a water closet. In accordance with 310 CMR 15.000: Title 5 of the State Environmental Code, privies are deemed non-conforming systems.

Provide means to supply and pay for.

Recyclables Material means any type of refuse designated by the local community through rules, and regulations, or policies to be separated for recycling.

Refuse means discarded solid material resulting from household activities and shall include, but not be limited to, garbage, rubbish, recyclable materials, bulk items, or yard waste. Refuse shall not include items designated as household hazardous waste which require separate disposal.

Residence means every dwelling, dwelling unit, condominium, condominium unit, manufactured home, rooming house, rooming unit, and mobile dwelling unit building or structure including but not limited to single or multi-unit structures used for, or intended for, human habitation and every other structure or condition located within the physical boundaries of the same lot whose existence may cause noncompliance required to comply with the provisions of 105 CMR 410.000. Residences include, but are not limited to, rooming houses, manufactured homes, homeless shelters, temporary housing, alternative housing, and condominiums.

Rooming House means every dwelling residence or part thereof which contains one or more rooming units in which space is let or sublet for compensation by the owner, rental of which is based on a contractual agreement between owner and occupant(s) for rent of a specified room or rooms which are not shared with other occupants of the rooming house.  or operator to four or more persons not within the second degree of kindred to the person compensated. Rooming houses include, but are not limited to, boarding houses, hotels, motels, inns, lodging houses, bed and breakfast establishments, bed and breakfast homes operations, dormitories, fraternity and sorority houses, hostels, and other similar dwelling places residences are included, except to the extent that they are where governed by stricter standards in accordance with other state laws and regulations. elsewhere. created; provided that the provisions of 105 CMR 410.000 shall not apply to any hospital, sanitorium, convalescent or nursing home, infirmary or boarding home for the aged licensed by the Department of Public Health in accordance with the provisions of M.G.L. c. 111, § 51 or 71.

Rooming Unit means the room or group of rooms in a rooming house that are let to an individual or household for use as living and sleeping quarters but not for cooking, except as provided in M.G.L. c. 140, §22.  whether or not common A rooming unit is based on a contractual agreement between owner and occupant(s) for rent of a specified room or rooms which are not shared with other occupants of the rooming house. A rooming unit may by contractual agreement also include use of shared cooking facilities that are separate from any rooming unit and providing that the individual/household can access the cooking facilities without for cooking are made available; provided, that cooking facilities shall not be deemed common if they can be reached only by passing through any part of the any other dwelling unit or rooming unit of another.

Rubbish means combustible and noncombustible waste materials, except garbage, and includes including but not limited to, such material as paper, rags, cartons, boxes, wood, excelsior, rubber, leather, tree branches, yard trimmings, grass clippings, tin cans, metals, machine or machine parts, mineral matter, glass, plastic bags and containers, crockery, dust, and the residue from the burning of wood, coal, coke and other combustible materials. Rubbish does not include garbage, yard waste, recyclable material, or bulk items.

Shared Facilities means facilities shared by more than one residence and placed so as not to require the passing through any part of another dwelling unit or rooming unit for access.

Stairway means any group of stairs consisting of three or more risers.

Temporary Housing means any tent, mobile dwelling unit, or other structure used for human habitation shelter which is designed to be transportable and which is 1) a mobile structure used for habitation, including a tent, that is not attached to the ground, to another structure, or to any utility system, on the same premises for more than 30 calendar days or 2) a mobile or permanent structure that provides basic shelter and contains at least one habitable room for living, sleeping, eating, cooking or sanitation that is intended to be occupied by a single family or household for intermittent periods of time not to exceed 90 consecutive days. Temporary housing does not include a structure used for human habitation for a period of no more than twelve months while a dwelling destroyed by a fire or natural disaster is rebuilt. (M.G.L c. 40A, § 3)

Temporary Overnight Emergency Shelter means any building, facility, or space therein designed and used primarily as a church or house of worship for religious services or instruction or related activities which is owned or operated by a religious organization and qualified for exemption under 26 U.S.C. section 501(c)(3) of the Internal Revenue Code that may, on occasion, provide temporary overnight accommodation to a limited number of individuals for a limited period of time.

Use Group R-3 means all buildings arranged for occupancy as one or two family dwelling units, including not more than five lodgers per family and multiple single family dwellings where each unit has an independent means of egress and is separated by a two-hour fire separation assembly.

Exceptions:

(1)   In multiple single-family dwellings that are equipped throughout with an approved sprinkler system installed in accordance with 780 CMR 906.2.1 or 906.2.2, the fire resistance rating of the dwelling unit separation shall not be less than one hour. Dwelling unit separation wall shall be constructed as fire partitions.

(2)   In multiple single-family dwellings that are equipped throughout with an approved automatic sprinkler system installed in accordance with 780 CMR 906.2.3, a two hour fire separation assembly shall be provided between each pair

of dwelling units. The fire resistance rating between each dwelling unit shall not be less than one hour and shall be constructed as a fire partition (780 CMR 310.5).

Use Group R-4 means all detached one and two family dwellings not more than three stories in height and all accessory structures (780 CMR 310.6).

Violation means any condition in a residence, dwelling, dwelling unit, mobile dwelling unit, or rooming house, or upon a parcel of land which fails to meet any requirement of 105 CMR 410.000.

Water Conservation Device means for all showers, shower stalls, shower compartments or shower baths, a low-flow showerhead which shall have a maximum flow rate not exceeding 2 two and ½ gallons of water per minute, for all faucets a maximum flow rate not exceeding two and 2/10 gallons of water per minute and for all water closets, ultra-low-flush water closets not exceeding one and 6/10 gallons of water per flush, contained within a dwelling unit residence.

Water Submetering means the use of a meter by an owner who receives water from a water company, as defined in M.G.L. c. 186, § 22, which meter measures water supplied to a dwelling unit residence to enable the owner to charge the tenant of the dwelling unit residence separately for water usage, or which meter measures water supplied to a common area.

Wood or Pellet Stove means a stove that burns wood, compressed wood pellets, or biomass pellets and is designed to radiate heat from the stove wall to provide heat to a room.  A wood or pellet stove does not include the necessary vents, ducts, pipes, or radiators to distribute heat to the entire dwelling unit.

Work Person-Like Manner means the way work is completed in accordance with accepted trade practices and regulatory requirements.

Yard Waste means grass clippings, leaves, weeds, hedge clippings, garden waste, and twigs, as well as brush or branches that meet size restrictions specified by the local community through rules and regulations relating to yard waste collection.

410.100: Kitchen Facilities

(A) Every dwelling unit, and every rooming house where common cooking facilities are provided, shall contain suitable space The owner shall provide and maintain, in compliance with the manufacturer’s intended use, the following facilities suitable to store, prepare and serve foods in a sanitary manner: The owner shall provide within this space:

(1) Cabinets, pantry or shelving,

(2) A countertop;

(3) A kitchen sink of sufficient size and capacity for washing dishes and kitchen utensils;

(4) A conventional cook top /and oven combination unit with at least 1.7 cubic feet of oven area or a permanent cooktop with separate oven containing at least 1.7 cubic feet of oven area; and

(5) Space and connection for a A refrigerator with freezer containing a combined storage area of at least 11.0 cubic feet.

(1) A kitchen sink of sufficient size and capacity for washing dishes and kitchen utensils; and

(2) A stove and oven in good repair (see 105 CMR 410.351) except and to the extent the occupant is required to do so under a written letting agreement; and

(3) Space and proper facilities for the installation of a refrigerator.

(B) The facilities required in by 105 CMR 410.100(A) shall have smooth and impervious surfaces and be free from defects that which make them difficult to keep clean, or creates an accident hazard.

(C) The wall above the countertop containing a sink in a kitchen or pantry shall have a smooth, nonabsorbent, and easily cleanable surface, and form a watertight seal with the countertop. Gypsum board is an allowable substrate above the countertop provided it is coated with paint, or other appropriate material, applied in accordance with the manufacturer’s recommendations. The surface shall extend at least 24 inches above the countertop where practical.

(D) (E) The floor surfaces of every kitchen and pantry shall be smooth, noncorrosive, and nonabsorbent. Wood flooring is allowed provided the flooring has a water-resistant finish and is maintained so as to prevent the accumulation of dirt and food or the harborage of pests.

(E)(D) The owner of a rooming house may provide, by contractual agreement, use of shared cooking facilities that are separate from any rooming unit and provided that the individual/household can access the cooking facilities without passing through any part of the any other dwelling unit or rooming unit of another. Wwhere shared kitchen facilities are provided is responsible for ensuring that the owner shall ensure the facilities and all food contact surfaces required by 105 CMR 410.100(A) are maintained in a clean and sanitary condition and all food contact surfaces and are sanitized at least once every 24 hours.

410.150 110: Washbasins Bathroom Facilities: Sinks, Toilets, Tubs, and Showers

- The owner shall provide no less than the following: a bathroom,with a door capable of being closed for privacy which:

- is not used for living, sleeping, cooking or eating purposes, and

- is accessible from within the building without passing through any part of another dwelling unit or rooming unit that contains the following: and

- with has a door capable of being closed.  Bathrooms in homeless shelters shall not require a door capable of being closed provided that entry to the bathroom is designed to block the view from an adjacent room or common area.

(B)(A) For Eeach bathroom shall include:  dwelling unit residence other than rooming units:

(1) A toilet with a toilet seat. in a room which is not used for living, sleeping, cooking or eating

purposes and which affords privacy to a person within said room.(a) When Wheremore than one toilet is required by 105 CMR 410.110(F):i. eEach toilet shall be separated by walls or partitions and a door to afford privacy and (iv) Eeach urinal shall be separated by walls or partitions to afford privacy.;

(2) A wash basin sink in the same room as the toilet provided that;

(a) If the wash basin sink cannot be placed in the same room as the toilet, it shall be placed in close proximity adjacent to the door leading directly into the room in which the toilet is located.

(b) The kitchen sink may not be substituted for the wash basin sink required in by 105 CMR 410.150(A)110(AB)(2) .; and

(3) A bathtub or shower. in the same room as the toilet or in another room which is not used for living, sleeping, cooking or eating purposes and which affords privacy to a person within said room.

(4) Each room which contains a toilet, bathtub or shower shall be fitted with a door which is capable of being closed.
e

(D) (C) Facilities The fixtures as required by in 105 CMR 410. 150110(A) and 410.150 (B) shall have smooth and impervious surfaces and be free from defects which make them difficult to keep clean or create an accidental a risk of injury or accident hazard.

(E) In a rooming house in which the facilities required by 105 CMR 410.110(B)  are not provided for each rooming unit, tThe owner shall ensure be responsible for ensuring that  the facilities are maintained in a clean and sanitary condition and the fixtures are sanitized at least once every 24 hours.

(F)(B) For no more than each every eight occupants of rooming units and rooming houses where private facilities are not already provided each rooming house in which bathroom facilities are not provided in each room, with the exception of Homeless shelters, the owner shall provide: in a room not used for living, sleeping, cooking or eating purposes and which affords privacy to a person within said room:

(1)   One toilet with a toilet seat and wash basin sink in the same room provided, that where more than one toilet is required in any toilet room used exclusively by males, urinals may be substituted for up to ½ of the total number of toilets required, on the basis of one urinal substituted for one toilet;  (a) If the wash basin cannot be placed in the same room as the toilet, it shall be adjacent to the door leading directly into the room in which the toilet is located. A kitchen sink may not be substituted for the wash basin required in by 105 CMR 410.150(A)110(B)(2). (3) (b) Where In a room with more than one toilet is required i. each toilet shall be separated by walls or partitions and a door to which afford privacy.

ii. One uUrinals may be substituted for each toilet for up to ½ of the total number of toilets required in any toilet bathroom used exclusively by males.iii. Substitutions are based on one toilet to one urinal. iv.  each urinal shall be separated by walls or partitions to afford privacy.

(2) A bathtub or a shower. One shower or bathtub in the same room as the toilet and wash basin or in another room not used for living, sleeping, cooking or eating purposes and which affords privacy to a person within said room.

(3)   In a room with more than one toilet, each toilet shall be separated by walls or partitions which afford privacy.

(C)   Toilet, bathtub and shower facilities as required in 105 CMR 410.150(A) and 410.150(B) shall be accessible from within the building and shall be so placed as not to require passing through any part of another dwelling unit or rooming unit.

410.151: Shared Facilities

The owner of any dwelling in which any toilet, wash basin, shower or bathtub is to be shared by the occupants of more than one dwelling unit or one rooming unit shall maintain that toilet, wash basin, shower, bathtub, walls and floors in a clean and sanitary condition, which shall include the cleaning and sanitizing of  said fixtures at least once every 24 hours.

410.152:   410.120: Privies and Chemical Approved Toilets Prohibited; Exceptions

(A) No privy or chemical toilet shall be constructed, nor shall an existing privy be or continued in use;  without the written permission of the board of health.

(B) provided, that the board of health may approve in writing the construction or continued use of any privy or chemical toilet which it determines will not No chemical toilet shall be used without written the permission of the board of health.

(A) The following toilets are approved for residential use:

(1)Humus/Composting toilets provided they meet the requirements of 310 CMR 15.000:

Title 5 of the State Environmental Code

(2)Incinerating toilets provided they meet the requirements of 248 CMR 10.00: Uniform State Plumbing Code

(3) A privy or chemical toilet provided they are approved in writing by the board of health in accordance with 310 CMR 15.000: Title 5 of the State Environmental Code

(C)(B) A privy or chemical toilet shall not:

(1) (a)Endanger the health of any person.

or (b) (2) Cause objectionable odors or other undue annoyance.

(3) Be located within 30 feet of any building used for sleeping or eating, or of any lot line or street.

(D) When so approved, a A privy or chemical toilet may, subject to written authorization of the board of health in accordance with 310 CMR 15.000, qualify as a toilet notwithstanding the requirements of 105 CMR 410.110(A)(1). (see 105 CMR 410.840).  In no event may a privy be located within 30 feet of any building used for sleeping or eating, or of any lot line or street.

410. 180130: Potable Water/Sanitary Drainage

(A) The owner shall provide, for the occupant of every dwelling, dwelling unit, and rooming unit,  residence a supply of potable water sufficient in quantity and pressure to meet the ordinary needs of the occupant,. This supply shall be connected to the public water supply system,  or with any other source that the board of health has determined, in writing, does not endanger the health of any potential user. . (See 105 CMR 410.350 through 410.352).

(B) For each dwelling residence the owner shall provide and maintain in a sanitary condition a sanitary drainage system consisting of,:

(1) A lawful connection to the public sewerage system;

(2) A means of on-site sewage disposal in compliance with 310 CMR 15.000: Massachusetts Department of Environmental Protection Title 5 of the State Environmental Code; or

(3) A lawful connection to a privately owned wastewater treatment facility permitted in accordance with 314 CMR 3.00: Surface Water Discharge Permit Program or 314 CMR 5.00: Massachusetts Department of Environmental Protection Ground Water Discharge Permit Program.

(C) If the owner intends to separately bill the occupant for water or sewer costs, then the owner must be in compliance with all requirements of M.G.L. c. 186, § 22, including, but not limited to:

(1) Installing and maintaining, when necessary, a water submetering device that measures only water that is supplied for the exclusive use of the particular dwelling unit residence and only to an area within the exclusive possession and control of the occupant of such dwelling unit residence ;

(2) Installing, or causing to be installed, water conservation devices on all showers, faucets, and toilets in the dwelling unit residence ;

(3) Having a written letting agreement with the occupant that describes the details of the water submetering and water billing arrangements; and

(4) Filing a certificate, on a form provided by the Department, with the board of health or other appropriate municipal agency charged with enforcing the State Sanitary Code, and signed by the owner under the pains and penalties of perjury, that the dwelling unit residence is in compliance with M.G.L. c. 186, § 22. The owner shall have a licensed plumber sign the certificate certifying that the water submetering devices and ultra-low-flush toilets have been installed in accordance with accepted plumbing standards and the requirements of M.G.L. c. 186, § 22, and shall attach appropriate documentation to verify the services provided by the licensed plumber.  The owner shall also provide a copy of the certificate to the occupants of each residence with the written letting agreement that describes the details of the water submetering and water billing arrangements.

In dwellings that are in compliance with the requirements of M.G.L. c. 186, § 22, the owner may charge the occupants for actual water usage in accordance with M.G.L. c. 186, § 22.   (D) An owner may not shut off or refuse water service to an occupant on the basis that the occupant has not paid a separately assessed water/sewer usage charge . in dwelling units residences that are in compliance with the requirements of M.G.L. c. 186, § 22.

Examination of the water system shall include an examination of the plumbing system and its actual performance.  If possible, such examination shall occur at the times and under such conditions as the occupant has identified the system as being insufficient.

(E) The owner shall allow occupants to have access to any water submeters that affect their dwelling unit in order to ensure that such submeters are functioning properly.

410. 140: Plumbing Connections

(A) Every required kitchen sink, wash basin bathroom sink, and shower or bathtub shall be connected to the hot and cold water lines of the water distribution system and to a sanitary drainage systemin compliance with accepted plumbing standards as defined in 105 CMR 410.010. 248 CMR Massachusetts Plumbing Code.

(B) Every provided toilet shall be connected to the water distribution system and to a sanitary drainage system in accordance compliance with accepted plumbing standards as defined in 105 CMR 410.010. 248 CMR Massachusetts Plumbing Code.

410. 190150: Hot Water

The owner shall provide and maintain in good operating condition the facilities capable of heating and delivering hot water. The owner shall also provide the hot water for use at a temperature of not less than 110°F (43° C) and in a quantity and pressure sufficient to satisfy the ordinary use of all plumbing fixtures which normally need hot water for their proper use and function, unless and to the extent the occupant is required to provide fuel for the operation of the facilities under a written letting agreement. The hot water shall not exceed 130°F (54° C).:

(A) In a quantity and pressure sufficient to satisfy the ordinary use of all plumbing fixtures which normally need hot water for their proper use and function.

(B) In a temperature range of not less than 1100F (430C) and not greater than 1300F (540C) for fixtures other than a bathtub or shower.

(C) In a temperature range of not less than 1100 F (430C)  1000 F (380C) and not greater than 1120F (440C) 1200F (480C) for a bathtub and or shower.

Inspection of the hot water system shall include an examination of the hot water system and its actual performance.  If possible, such examination shall occur at the times and under such conditions as the occupant has identified the system to be insufficient.

410. 200: 160: Heating Facilities Required Systems

(A) The owner shall provide and maintain in good operating condition a central heating system that includes a distribution system capable of for heating every habitable room and every room containing a toilet, shower, or bathtub to such temperature required by 105 CMR 410. 201.180.

(B) The following shall not meet the requirements of a central heating system:

(1) Fireplace;

(2) Woodstove (3) or Ppellet stove; or

(34) Portable electric space heater;. or

(5) Unvented propane or natural gas-fired space heaters (527 CMR Massachusetts Fire Code).

(B) Portable space heaters, parlor heaters, cabinet heaters, room heaters and any similar heaters having a barometric fed fuel control and its fuel supply tank located less than 42 inches from the center of the burner as well as the type of heating appliance adapted for burning kerosene, range oil or number one fuel oil and any portable wick type space heaters shall not be used and shall not meet the requirements of 105 CMR 410.200.  (See M.G.L. c. 148, §§ 5A and 25B.)

(C) The following are prohibited from use in any residence:

(1) Any portable space heater that has a barometric fed fuel control and a fuel supply tank located less than 42 inches from the center of the burner, included but not limited to, parlor heaters, cabinet heaters, or room heaters.

(2) Heating appliances adapted for burning propane, kerosene, range oil or number one fuel oil.

(3) Portable wick type space heaters.

(4) Unvented propane or natural gas-fired space heaters not approved in accordance with 527 CMR 1.00:  Massachusetts Comprehensive Fire Safety Code.

410.170: Venting

(A) All devices that combust fuel must be properly vented. Such devices, including, but are not limited to, furnaces, boilers, water heaters and space heaters permitted under 410.160, and, shall be properly vented to the outdoors in accordance compliance with accepted standards as defined in 105 CMR 410.010. 527 CMR Massachusetts Fire Code, 248 Massachusetts Plumbing Code, and 780 CMR Massachusetts Building Code. This section shall not apply to unvented propane or natural gas-fired space heaters approved in accordance with 527 CMR 1.00:  Massachusetts Comprehensive Fire Safety Code.

(B) All electric and gas clothes drying equipment shall be vented to the outdoors except electric dryers listed and labeled as ventless by the manufacturer.

(C) All electric hoods for ranges shall be vented to the outdoors,. except unless electric hoods listed and labeled as ductless or recirculating by the manufacturer.

410.201 180: Temperature Requirements

(A) The owner shall provide heat in every habitable room and every room containing a toilet, shower, or bathtub from September 15th through May 31st inclusive so that it shall be:

(1) shall be a At least 680F (200C) between 7:00 A.M. and 11:00 P.M.; and

(2) shall be a At least 640F (170C) between 11:01 P.M. and 6:59 A.M. every day other than during the period from June 15th to September 15th, both inclusive, in each year except and to the extent the occupant is required to provide the fuel under a written letting agreement

(3) Shall at no time exceed 780F (250C) during the heating season.

(B) At no time shall the central heating system, required by 105 CMR 410.160(A), used during the heating season cause the temperature to exceed 780F (250C) in any room.

(B) (C) The temperature may be read and the requirement shall be met at a height of five three feet above floor level on a wall any point more than five two feet from the exterior wall.

(C) The number of days per year during which heat must be provided in accordance with 105 CMR 410.000 may be increased or decreased through a variance granted in accordance with the provisions of 105 CMR 410.840 notwithstanding the prohibitions of the first clause of the first sentence of 105 CMR 410.840(A).

(D) A board of health may alter the heating season, as defined in 105 CMR 410.180(A), by ending it earlier than May 31st or delaying the start date later than September 15th in a particular year for all residences within its jurisdiction,  the number of heating days as required by 105 CMR 410.180(A), in any year by posting notice of such alteration on the municipality’s website. The board of health’s decision to alter the number of heating days shall not be required to conform to the provisions for the granting of a variance required by 105 CMR 410.700.

410.202:   Venting

Space heaters and water heaters, except electrical ones, shall be properly vented to a chimney or vent leading to the outdoors.

410. 200: Provision and Metering of Electricity, or Gas

(A) The owner shall provide the electricity or gas used in each dwelling unit residence unless:

(1) Such electricity or gas is metered through a meter which serves only the dwelling unit residence or other area under the exclusive use of an occupant of that dwelling unit residence, except as allowed by 105 CMR 410.254(B)300(F); and

(2) A written letting agreement provides for payment by the occupant.

(B) If the owner is required by 105 CMR 410.000 or by a written letting agreement consistent with 105 CMR 410.000, to pay for the electricity or gas used in a dwelling unit residence or other area under the exclusive use of an occupant of that residence, then such electricity or gas may be metered through meters which serve more than one dwelling unit residence.

(C) If the owner is not required to pay for the electricity or gas used in a dwelling unit residence or other area under the exclusive use of an occupant of that residence, then the owner shall install and maintain wiring and piping so that any such electricity or gas used in the dwelling unit residence is metered through meters which serve only such dwelling unit residence or other area under the exclusive use of an occupant of that residence, except as allowed by 105 CMR 410.300(F).

(D) Owners shall provide occupants with access to their dwelling unit’s electrical distribution panel when:

(1) tThe occupants are required to pay for electrical service in accordance with 105 CMR 410.200(A);

(2) The owner, as defined by 105 CMR 410.010

does not reside within the

residence: or

(3) A facility manager is not available within one hour’s notice.

(E) If it is determined upon inspection that electricity or gas may not meet the requirements of 105 CMR 410.200(A), the board of health shall immediately notify the local plumbing or electrical inspector of any possible violations observed.

410. 210: Provision of Oil

The owner shall provide for the oil used for heating and/or hot water in each dwelling unit residence unless:

(A) Oil is provided through a separate oil tank which serves only that dwelling unit residence; and

(B) The occupant is required to provide the oil under a written letting agreement

410.250:   Habitable Rooms Other than Kitchen – Natural Light and Electrical Outlets

The owner shall provide for each habitable room other than a kitchen:

(A)   transparent or translucent glass which admits light from the outdoors and which is equal in area to no less than 8% of the entire floor area of that room.

(B)   two separate wall-type convenience outlets, or one such outlet and one electric light fixture.  The outlets shall be placed in practical locations and shall insofar as practicable, be on different walls and at least ten feet apart.  (See 105 CMR 410.351.)

410.251:   Kitchen Lighting and Electrical Outlets

The owner shall provide for each kitchen:

(A)   one electric light fixture;

(B)   two wall-type convenience outlets located in convenient locations; and

(C)   For each kitchen over 70 square feet, transparent or translucent glass which admits light from the outdoors and which is equal in area to no less than 8% of the entire floor area of that kitchen.

410.252:   Bathroom Lighting and Electrical Outlets

The owner shall provide in each room containing a toilet, bathtub, or shower one electric light fixture.  (See 105 CMR 410.150(A)(1) and 410.150(B).)

410.253:   Light Fixtures Other than in Habitable Rooms or Kitchens

(A)   The owner shall provide and so locate electric light switches and fixtures in good working order so that illumination may be provided for the safe and reasonable use of every laundry, pantry, foyer, hallway, stairway, closet, storage place, cellar, porch, exterior stairway and passageway.

(B)   The owner shall provide working incandescent light bulbs or fluorescent tubes in all required light fixtures in all common areas of any dwelling.

410.254:   Light in Passageways, Hallways, and Stairways

(A)   Except as allowed in 105 CMR 410.254(B), the owner shall provide light 24 hours per day so that illumination alone or in conjunction with natural lighting shall be at least one foot candle as measured at floor level, in every part of all interior passageways, hallways, foyers and stairways used or intended for use by the occupants of more than one dwelling unit or rooming unit:

(B)   In a dwelling containing three or fewer dwelling units, the light fixtures used to illuminate a common hallway, passageway, foyer and/or stairway may be wired to the electric service serving an adjacent dwelling unit provided that if the occupant of such dwelling unit is responsible for paying for the electric service to such dwelling unit:

(1)   a written agreement shall state that the occupant is responsible for paying for light in the common hallway, passageway, foyer and/or stairway; and

(2)   the owner shall notify the occupants of the other dwelling units.

410.255:   Amperage

The electrical service supplying each dwelling, dwelling unit, rooming house and/or rooming unit shall supply sufficient amperage to meet the reasonable needs of the occupants. Should the amperage be determined to be inadequate it shall be corrected so that it meets the amperage requirements of 527 CMR 12.00:  The Massachusetts Electrical Code.

410.256:   Temporary Wiring

No wiring shall lie under a rug or other floor covering, nor shall any extend through a doorway or other opening in a structural element.  No temporary wiring shall be used or made available for use by any owner or occupant; provided, that extension cords which connect portable electric appliances or fixtures to convenience outlets shall not be considered temporary wiring.

410.257:   Light Obstructions

If any light obstructing structure is located less than three feet from the outside of and extends to a level above the lower level of the transparent or translucent glass required by 105 CMR 410.250(A) and 410.251(C), that portion so obstructed shall not be included as contributing to the required minimum total glass area.

410.258:   Exemption of Dwellings More than 600 Feet from Electrical Service

The provisions of 105 CMR 410.250 through 410.257 regarding the furnishing of electrical facilities shall apply only if a source of electricity is available from power lines within 600 feet of the dwelling.

410.280 220: Natural and Mechanical Ventilation

The owner shall provide for each habitable room, and room containing a toilet, bathtub, or shower, ventilation to the outdoors consisting of: as follows:

(A) Habitable rooms, and rooms with a toilet, bathtub or shower:

(1) Windows, skylights, or doors or transoms in through the exterior walls or roofs that can be easily opened to a minimum with a combined opening of at least 4% of the floor area of that habitable room or room containing a toilet, bathtub or shower, provided that a skylight shall not satisfy this requirement which if, when it is open, it exposes the interior of the dwelling residence to direct rainfall, shall not satisfy this requirement; or

(2) mechanical Mechanical ventilation capable of exhausting air to the outdoors at the following the ventilation rates specified in 105 CMR 410.220(C). rates

(B) Rooms containing a toilet, bathtub, or shower

On or before September 1, 2016,  mechanical ventilation capable of exhausting air to the outdoors at the ventilation rates specified in 105 CMR 410.220(C).

(B)  Except when necessary to prevent chronic dampness, as determined by the board of health. mechanical ventilation is not required when natural ventilation requirements in 105 CMR 410.220(A) are met.

(C) Ventilation rate requirements

The following minimum ventilation rates shall be provided in all rooms where mechanical ventilation is required or provided:in accordance with 780 Massachusetts State Building Code and 271 CMR Uniform Sheet Metal Installation Rules.Ventilation rates are measured in cubic feet per minute (cfm).

Occupancy Classification	 	Ventilation Rate (in cubic feet per minute or cfm)

Habitable rooms 				100 cfm

Bath, toilet or shower rooms 		  50 cfm

(D) 410.281:   Ventilation Shut-off

Each mechanical ventilation system specified in required by 105 CMR 410.280 220(A) and (B) shall be equipped with a readily accessible means for either shut-off., unless such system is designed to run constantly or is barometrically controlled. or volume reduction. and any other ventilation system shall be equipped with a readily accessible means for shut-off. (See 105 CMR 410.351.)

410.230: Owner’s Installation and Maintenance Responsibilities

The owner shall install or cause to be installed, in accordance compliance with  accepted standards as defined in 105 CMR 410.010 780 CMR Massachusetts Building Code, 527 CMR Massachusetts Fire Code, 527 CMR 12 Massachusetts Electrical Code, 248 CMR Massachusetts Plumbing Code, 271 CMR Uniform Sheet Metal Installation Rules, and accepted plumbing, gasfitting and electrical wiring standards, and shall maintain in operable condition free from leaks, obstructions or other defects, the following:

(A)   all facilities and equipment which the owner is required to provide including, but not limited tol:

(1) Sinks

(2) washbasins

(2) Bathtubs

(3) Showers

(4) Toilets

(5) Waterheating facilities

(6) Gas pipes

(7) Heating equipment

(8) Water pipes

(9) Cook tops and ovens;

(10) Catch basins, drains, vents and other similar supplied fixtures;

(11) Connections to water, sewer and gas lines;

(12) Subsurface sewage disposal system, if any;

(13) Electrical fixtures, outlets and wiring;

(14) Smoke detectors and carbon monoxide alarms;

(15) All heating and ventilating equipment and appurtenances thereto; and

(16) Refrigerator with freezer

(B) All owner-installed optional equipment, including, but not limited to:

(1) Refrigerators;

(1) Dishwashers,;

(2) Clothes washing machines and dryers;

(3) Garbage grinders,;

(4) Sub-metering devices designed to measure the usage of water,;

(5) Air conditioners;

(6) Microwave ovens; and

(7) Range ventilation hoods.

410.235: Owner’s Laundering Responsibilities: If a residence, including a rooming house and homeless shelter, provides bed linens, towels, pillows or mattresses to occupants:

(A)	Bed linens and towels shall be laundered between use by different individuals. If continuously used by one individual, linens shall be cleaned weekly and more often if needed.  Shared towels are prohibited.

(B)	Blankets shall be replaced when worn. Each blanket shall be laundered between use by different individuals. If used continuously by one individual, each blanket shall be cleaned at least every three months, and more often if needed.

(C) 	Pillow covers shall be laundered between use by different individuals, and more often if needed.

(D)	Mattresses shall be clean, fire-retardant, and in good condition. Each mattress shall be cleaned or replaced when soiled or when not in good condition. Each mattress pad shall be cleaned when soiled and at least every three months, and more often if needed.

410.240: Occupant’s Installation and Maintenance Responsibilities

- The occupant shall ensure proper installation install in accordance compliance with accepted standards as defined in 105 CMR 410.010, 780 CMR Massachusetts Building Code, 527 CMR Massachusetts Fire Code, 527 CMR 12 Massachusetts Electrical Code, 248 CMR Massachusetts Plumbing Code, 271 CMR Uniform Sheet Metal Installation Rules, and shall maintain free from leaks, obstructions and other defects, all occupant owned and installed equipment such as, but not limited to:

(1) Refrigerators;

(1) Clothes washing machines and dryers;

(2) Dishwashers; stoves

(3) Garbage grinders;

(4) Electrical fixtures; and

(5) Air conditioners.

(B) Every occupant of a residence shall keep all facilities and appliances in a clean and sanitary condition and exercise reasonable care in the proper use and operation thereof. Facilities and appliances include, but are not limited to:

(1) Toilets;

(2) Wash basins;

(2)(3) Sinks;

(3) (4) Showers;

(4) (5) Bathtubs;

(5) (6) Cook tops and ovens;

(6) (7) Refrigerators with freezer;

(7) (8) Dishwashers;

(8) (9) Garbage grinders;

(9) (10) Air conditioners; and

(10) Smoke detectors and carbon monoxide alarms.

410. 250: Asbestos-Containing Material

Every owner shall maintain all asbestos-containing material in good repair, and free from any defects including, but not limited to, holes, cracks, tears, or any looseness which may allow the release of asbestos dust, or any powdered, crumbled, or pulverized asbestos-containing material. Every owner shall correct any violation of 105 CMR 410. 250 in accordance compliance with accepted standards as defined in 105 CMR 410.010. 310 CMR 7.00: Massachusetts Department of Environmental Protection and 453 CMR 6.00: Massachusetts Department of Labor and Workforce Development.

410.260: Means of Egress

(A) Every residence shall have at a minimum two means of egress to allow for the safe passage of all people.

(B) No person shall obstruct any means of egress or passageway.

(C)(1) The owner is responsible for maintaining free from obstruction every means of egress used or intended for use by occupants of more than one residence dwelling unit, except as provided for in 105 CMR 410.570(C)(2).

(D)(2) The occupant of single unit residences shall be responsible for maintaining free from obstruction all means of egress of single unit residences.

(3) The occupant of a multi-unit residence shall maintain free from obstruction all means of egress within their dwelling or rooming unit

(C) The owner shall ensure each means of egress is maintained at all times in a safe, operable condition, including, but not limited to:

(1) Keeping all exterior stairways, fire escapes, egress balconies and bridges free of snow and ice

(2) Keeping all corrodible structural parts thereof shall be kept painted or otherwise protected against rust and corrosion.;

(3) Treating all wood structural members shall be treated to prevent rotting and decay.; and

(4) Sealing all joints where these structural elements tie directly into the building’s structural system all joints shall be sealed to prevent water from damaging or corroding the structural elements. to prevent water from damaging or corroding the structural elements.

(D) The occupant is responsible for maintaining free of snow and ice, the means of egress under their exclusive use and control, provided there is a written letting agreement that clearly identifies the occupant’s responsibility.

410. 270: Locks

(A) The owner shall provide, install and maintain locks in the following manner and shall comply with the requirements of 780 CMR: State Massachusetts Building Code and 521 CMR: Architectural Access Board, and M.G.L. c. 143, § 3R  so that:

(1) Every residence shall be capable of being secured against unlawful entry.

(2) Every entry door of a dwelling unit residence shall be capable of being secured against unlawful entry.

(3) The main entry door of a residence containing more than three dwelling units shall be so designed or equipped so as to close and lock automatically with a lock, including a lock with an electrically-operated striker mechanism, a self-closing door, and associated equipment. If two doors enclose an entryway between the common areas of the residence and the exterior of the residence, only one door need to meet this requirement.  This requirement shall not apply to:

(a) Lodging houses, as defined in M.G.L. c. 140, § 22;

(b) Dormitories of charitable, educational or philanthropic institutions;

(c) Housing Authorities, as defined in M.G.L. c. 121B; or

(d) Homeless shelters

(4) Every exterior door of the residence, except the main entry door listed in 105 CMR 410.270(A)(3), shall be equipped with an operating lock.

(5) All egress doors shall be capable of being opened from the side in which egress is to be made without use of any key, knowledge, or effort in order to avoid entrapment in the building.

(6) Every openable exterior window shall be capable of being secured.

(B) The owner, except in a homeless shelter, shall provide a key to the occupant for:

(1) The main entry door; and

(2) The occupant’s residence; and

(3) Areas common to their unit.

(C) If aAn occupant that replaces the lock on the any entry door to their in the  residence,  shall provide a key shall be provided to the owner.

410.300: Electricity Supply and Illumination

The owner of a residence shall provide no less than the following minimum requirements for electrical supply and illumination:

(A) Habitable rooms other than a kitchen:

(1) Two separate wall-type receptacle outlets in accessible locations, on different walls, and at least ten 10 feet apart where practical; or

(2) One outlet and one electric light fixture.

(B) Kitchen:

(1) One electric light fixture; and

(2) Two wall-type receptacle outlets, installed in convenient accessible locations in addition to the refrigerator and cook top and oven outlets.

(C) One electric light fixture in aA room containing a toilet, bathtub, or shower.:

(1) One electric light fixture.

(D) Spaces other than habitable rooms, kitchens, or rooms containing a toilet, bathtub, or shower:

(1) Electric light switches and light fixtures in good working order so that illumination may be available for the safe and reasonable use of every:

(a) Laundry;

(b) Pantry;

(c) Foyer;

(d) Hallway;

(e) Stairway;

(f) Closet;

(f) Storage place;

(g) Cellar;

(h) Porch;

(i) Deck;

(j) Passageway; and

(k) Exterior stairways; and passageway,

(l) Fire escapes.

(2) Lightbulbs in all required light fixtures in common areas.

(E) Passageways, hallways, and stairways:

(1) Light shall be available 24 hours per day in every part of all interior passageways, hallways, foyers and stairways used or intended for use by the occupants of more than one dwelling unit or rooming unit.

(2) Illumination alone or in conjunction with natural lighting shall be at least one foot candle as measured at floor level.

(F) Dwellings Residences containing three or fewer dwelling units:

Light fixtures used to illuminate a common hallway, passageway, foyer and/or stairway may be wired to the electric service serving an adjacent dwelling unit provided that if the occupant of such dwelling unit is responsible for paying for the electric service to such dwelling unit provided:

(1) A written letting agreement shall state that the occupant is responsible for paying for light in the common hallway, passageway, foyer, and/or stairway; and

(2) The Any requirement for an occupant to pay for lights in common areas  a written letting agreement shall begin only upon commencement of a new tenancy and shall not apply to an existing tenancy; and

(3) The owner shall provide written notification to the occupants of the other dwelling units.

410.310: Auxiliary Emergency Lighting Systems and Exit Signs

The owner of every dwelling residence of ten10 or more units shall provide and maintain in good working order the auxiliary emergency lighting systems and exit signs required by and in compliance with any applicable regulations promulgated by the Commissioner of Public Safety (780 CMR and M.G.L. c. 143, § 21D).

(A) An auxiliary emergency lighting system independent of the conventional lighting system in common areas.

- Lighted signs on each floor indicating both a primary and secondary means of egress, by a diagram or signal so as to assure recognition by all persons regardless of their English speaking ability.

410.320: Electrical Service

(A) Temporary Wiring:

(1) No temporary wiring shall be used or made available for use by any owner or occupant.

(2) No owner or occupant shall place wiring under a rug or other floor covering.

(3) No wiring shall extend through a doorway or other opening in a structural element.

(4) Extension cords which connect portable electric appliances or fixtures to outlets and which have an appropriate size rating for the electrical load shall not be considered temporary wiring.

(B) Amperage:

The electrical service supplying each residence shall supply be of sufficient amperage to meet the reasonable needs of the occupants. and shall meets the amperage requirements of 527 CMR 12.00:  The Massachusetts Electrical Code.

(C) Exemption for Dwellings residences more than 600 feet from electrical service exemption:

The provisions of 105 CMR 410.300, 410. 310, and 410.320 regarding the furnishing of electrical facilities shall apply only if a source of electricity is available from power lines within 600 feet of the residence.

410. 330: Smoke Detectors and Carbon Monoxide Alarms

(A) The owner shall provide, install, and maintain in operable condition smoke detectors and carbon monoxide alarms in every residence that is required to be equipped with smoke detectors and carbon monoxide alarms in accordance with any provision of the Massachusetts General Laws,  MGL c. 148, §§ 26 C-F½ and 780 CMR: Massachusetts State Building Code, 527 CMR: Massachusetts Fire Code, and 248 CMR 10.00: Uniform State Plumbing Code Massachusetts Plumbing Code and in compliance with accepted standards as defined by 410.010.

(B) The board of health shall immediately notify the chief of the local fire department of any violation of 105 CMR 410. 330 which is observed during an inspection of any residence.

(C) If any residence is found by the local fire department to be adequately equipped with smoke detectors and carbon monoxide alarms, the board of health shall not be authorized by 105 CMR 410. 330 to impose any additional or differing smoke detector or carbon monoxide alarm requirement beyond that which has been found sufficient by the local fire department.

410.300: Sanitary Drainage System Required

The owner shall provide, for each dwelling, a sanitary drainage system connected to the public sewerage

system, provided, that if, because of distance or ground conditions, connection to a public sewerage system is

not practicable, the owner shall provide, and shall maintain in a sanitary condition, a means of sewage disposal

which is in compliance with 310 CMR 15.00: Subsurface Disposal of Sanitary Sewage (Title V). (See 105

CMR 410.840.)

In dwellings that are in compliance with the requirements of M.G.L. c. 186, § 22, the owner may charge

the occupants for the cost of sewer service in accordance with M.G.L. c. 186, § 22.

410.350: Plumbing Connections

(A) Every required kitchen sink, wash basin and shower or bathtub shall be connected to the hot and cold water lines of the water distribution system (See 105 CMR 410.180) and to a sanitary drainage system (See 105 CMR 410.300) in accordance with accepted plumbing standards

(B) Every provided toilet shall be connected to the water distribution system (See 105 CMR 410.180) and to a sanitary drainage system (See 105 CMR 410.300) in accordance with accepted plumbing standards

410.351:   Owner's Installation and Maintenance Responsibilities

The owner shall install or cause to be installed, in accordance with accepted plumbing, gasfitting and electrical wiring standards, and shall maintain free from leaks, obstructions or other defects, the following:

(A)   all facilities and equipment which the owner is or may be required to provide including, but not limited to, all sinks, washbasins, bathtubs, showers, toilets, waterheating facilities, gas pipes, heating equipment, water pipes, owner installed stoves and ovens, catch basins, drains, vents and other similar supplied fixtures; the connections to water, sewer and gas lines; the subsurface sewage disposal system, if any; all electrical fixtures, outlets and wiring, smoke detectors and carbon monoxide alarms, and all heating and ventilating equipment and appurtenances thereto; and

(B)   all owner-installed optional equipment, including but not limited to, refrigerators, dishwashers, clothes washing machines and dryers, garbage grinders, and submetering devices designed to measure the usage of electricity, gas or water.

410.352:   Occupant's Installation and Maintenance Responsibilities

(A) The occupant shall install in accordance with accepted plumbing, heating, gas fitting, and electrical wiring standards, and shall maintain free from leaks, obstructions and other defects, all occupant owned and installed equipment such as, but not limited to, refrigerators, clothes washing machines and dryers, dishwashers, stoves, garbage grinders and electrical fixtures.

(B) Every occupant of a dwelling unit shall keep all toilets, wash basins, sinks, showers, bathtubs, stoves, refrigerators and dishwashers in a clean and sanitary condition and exercise reasonable care in the proper use and operation thereof.

410.353: Asbestos Material

Every owner shall maintain all asbestos material in good repair, and free from any defects including, but not limited to, holes, cracks, tears, or any looseness which may allow the release of asbestos dust, or any powdered, crumbled, or pulverized asbestos material. Every owner shall correct any violation of 105 CMR 410.353 in accordance with the regulations of the Department of Environmental Protection appearing at (310 CMR 7.00) and in accordance with the regulations of the Department of Labor and Workforce Development appearing at (453 CMR 6.00).

410.354: Metering of Electricity, Gas and Water

(A) The owner shall provide the electricity and gas used in each dwelling unit unless

(1)   Such gas or electricity is metered through a meter which serves only the dwelling unit or other area under the exclusive use of an occupant of that dwelling unit, except as allowed by 105 CMR 410.254(B); and

(2)   A written letting agreement provides for payment by the occupant.

(B) If the owner is required, by 105 CMR 410.000 or by a written letting agreement consistent with 105 CMR 410.000, to pay for the electricity or gas used in a dwelling unit, then such electricity or gas may be metered through meters which serve more than one dwelling unit.

(C) If the owner is not required to pay for the electricity or gas used in a dwelling unit, then the owner shall install and maintain wiring and piping so that any such electricity or gas used in the dwelling unit is metered through meters which serve only such dwelling unit, except as allowed by 105 CMR 410.254(B).

(D) If the owner intends to separately bill the occupant for water or sewer services in accordance with the provisions of M.G.L. c. 186, § 22, then the owner must be in compliance with all requirements of M.G.L. c. 186, § 22, including, but not limited to:

(1) Installing and maintaining, when necessary, a water submetering device that measures only water that is supplied for the exclusive use of the particular dwelling unit and only to an area within the exclusive possession and control of the occupant of such dwelling unit;

(2) Installing, or causing to be installed, water conservation devices on all showers, faucets, and toilets in the dwelling unit;

(3) Having a written letting agreement with the occupant that describes the details of the water submetering and water billing arrangements; and

(4) Filing a certificate, on a form provided by the Department of Public Health, with the Board of health or other appropriate municipal agency charged with enforcing the State Sanitary Code, and signed by the owner under the pains and penalties of perjury, that the dwelling unit is in compliance with M.G.L. c. 186, § 22. The owner shall have a licensed plumber sign the certificate certifying that the water submetering devices and ultra-low-flush toilets have been installed in accordance with accepted plumbing standards and the requirements of M.G.L. c. 186, § 22, and shall attach appropriate documentation to verify the services provided by the licensed plumber.  The owner shall also provide a copy of the certificate to the occupants of each dwelling unit with the written letting agreement that describes the details of the water submetering and water billing arrangements.

(E) The owner shall allow occupants to have access to any water submeters that affect their dwelling unit in order to ensure that such submeters are functioning properly.

410.355: Provision of Oil

The owner shall provide for the oil used for heating and/or hot water in each dwelling unit unless:

(A) Such oil is provided through a separate oil tank which serves only that dwelling unit; and

(B) provided however, that 105 CMR 410.000 shall only apply to tenancies created or renewed after July 1, 1994.

410. 400: Owner/Manager Contact Information and Notice of Occupants’ Legal Rights and Responsibilities

(A) Every owner of a residence who does not reside therein shall post signage, on durable material and not less than 20 square inches in size, which contains, at a minimum, the owner’s name, address and telephone number, and, if applicable:

(1) The name, address and telephone number of the president of the corporation. if a corporation;

(2) The name, address and a telephone number of the managing trustee or partner if the owner is a realty trust or partnership; or

(3) When employed Tthe name address and telephone number of the a property manager or agent who does not reside therein in the building.

(B) The telephone number of the owner or agent shall be regularly monitored, but no less than once every 12 hours, unless the owner has provided occupants with an alternative contact person and phone number for periods of temporary absence. one which provides for response within 24 hours.

(C) Every owner of a residence shall provide to each occupant or post in a location specified in 105 CMR 410.400(D) a copy of the Notice of Occupants’ Legal Rights and Responsibilities issued by the Department;

(D)(C) The pPostings required by this section shall be on durable material not less than 20 square inches in size and placed at the residence adjacent to the mailboxes or within the interior of the residence in a location visible to the occupants.

410. 410: Building Identification

The owner shall affix to every building covered by 105 CMR 410.000, a number representing the address of such building. The number shall be of a nature and size and shall be situated on the building so that, to the extent practicable, it is visible from the nearest street providing vehicular access to such building (M.G.L. c. 148, § 59).

410.420: Habitability Requirements

(A) No room or area in a residence may be used for habitation if it is subject to chronic dampness.

(B) No room shall be considered habitable if more than ¾ of its total floor area has a floor-to-ceiling height of less than seven feet.

(C) No area of a room where the ceiling height is less than five feet shall be considered habitable space required by 105 CMR 410.420(D).

(D) Minimum Square Footage

(1) Every residence shall contain at least 150 square feet of habitable floor space for its first occupant, and at least 100 square feet of habitable floor space for each additional occupant,

(2) A rooming unit may contain a minimum of 100 square feet of habitable floor living space when:

(a) The unit contains one single room for living and sleeping only; and

(b) Is occupied by no more than one person.

(3)    In every residence, each room occupied for sleeping purposes by one occupant shall contain at least 70 square feet of floor space;

(4) In every residence, each room occupied for sleeping purposes by more than one occupant shall contain at least 50 square feet of floor space for each occupant.

(5) Homeless shelters are exempt from 105 CMR 410.420(D)(1)-(3) but must provide a minimum of:

(a) 3 feet of separation between each single bed;

(b) 4.5 feet of separation between each double bunk; and

(c) An alternating sleeping configuration to allow for 6 feet of separation head to head.

410.430: Natural Light and Obstructions

(A) The owner shall provide transparent or translucent glass which admits unobstructed light from the outdoors equal in area to a minimum 8% of the entire floor area in:

(A1) Habitable rooms other than a kitchen.

(B2) Kitchens which have a floor area greater than 70 square feet.

(BC) If any light obstructing structure is located less than three feet from the outside of and extends to a level above the lower level of the transparent or translucent glass, that portion so obstructed shall not be included as contributing to the required minimum total glass area.

410.440: Temporary Housing and Cabins

(A) No person may allow temporary housing or cabin may to be used for occupancy occupied without having obtained the written permission of the board of health.

(B) All temporary housing and cabins shall be subject to the requirements of these minimum standards105 CMR 410.000, except as the board of health may otherwise provide in its written permission.

410.450: Alternative Housing

No owner shall occupy or permit occupancy of alternative housing without obtaining a permit from the board of health issued pursuant to 105 CMR 410.710.

410.460: Homeless Shelters

All homeless shelters shall comply with 105 CMR 410.000 with the following exceptions:

(A) Homeless shelters are not required to provide a bathroom door capable of being closed to afford privacy as required by 105 CMR 410.110(A), provided entry into the room is designed to block the view from an adjacent room or common area.

(B) Homeless shelters are not required to comply with the toilet and bathroom ratios established in 105 CMR 410.110(C).

(C) Homeless shelters are not required to have a lock with a striker mechanism or provide keys to all occupants, as required by105 CMR 410.270(A)(3) and (C).

(D) Homeless shelters are exempt from the minimum square footage requirements established in 105 CMR 410.420(D), provided there shall be a minimum of 3 feet separation between beds (4 ½ feet for double bunks) and the space should be configured to provide at least 6 feet head-to-head separation.

(E) Homeless shelters are not required to comply with the installation of screens on doorways required by 105 CMR 410.540(C).

(F) Homeless shelters are not required to comply with the inspection requirement in 105 CMR 550(F), provided they establish pest management policies that include periodic inspection for pest infestation

410. 460470:    Existence of Lead-based Paint Hazards in Residences

All residences must be in compliance with the requirements of 105 CMR 460.000: Lead Poisoning Prevention and Control.

410.400: Minimum Square Footage

(A) Every dwelling unit shall contain at least 150 square feet of floor space for its first occupant, and at least

100 square feet of floor space for each additional occupant, the floor space to be calculated on the basis of total

habitable room area.

(B) In a dwelling unit, every room occupied for sleeping purposes by one occupant shall contain at least 70

square feet of floor space; every room occupied for sleeping purposes by more than one occupant shall contain

at least 50 square feet of floor space for each occupant.

(C) In a rooming unit, every room occupied for sleeping purposes by one occupant shall contain at least 80

square feet of floor space; every room occupied for sleeping purposes by more than one occupant shall contain

at least 60 square feet for each occupant.

410.401: Ceiling Height

(A) No room shall be considered habitable if more than ¾ of its floor area has a floor-to-ceiling height of less

than seven feet.

(B) In computing total floor area for the purpose of determining maximum permissible occupancy, that part of

the floor area where the ceiling height is less than five feet shall not be considered.

410.402: Grade Level

No room or area in a dwelling may be used for habitation if more than ½ of its floor-to-ceiling height is

below the average grade of the adjoining ground and is subject to chronic dampness.

410.430: Temporary Housing Allowed Only with Board of Health Permission

No temporary housing may be used except with the written permission of the board of health.

410.431: Any Exceptions to Minimum Standards Must Be Specified

All temporary housing shall be subject to the requirements of these minimum standards, except as the

board of health may provide in its written permission. (See 105 CMR 410.840.)

410.450: Means of Egress

Every dwelling unit, and rooming unit shall have as many means of exit as will allow for the safe passage

of all people in accordance with 780 CMR 104.0, 105.1, and 805.0 of the Massachusetts State Building Code.

410.451: Egress Obstructions

No person shall obstruct any exit or passageway. The owner is responsible for maintaining free from

obstruction every exit used or intended for use by occupants of more than one dwelling unit or rooming unit.

The occupant shall be responsible for maintaining free from obstruction all means of exit leading from his unit

and not common to the exit of any other unit.

410.452: Safe Condition

The owner shall maintain all means of egress at all times in a safe, operable condition and shall keep all

exterior stairways, fire escapes, egress balconies and bridges free of snow and ice, provided, however, in those

instances where a dwelling has an independent means of egress, not shared with other occupants, and a written

letting agreement so states, the occupant is responsible for maintaining free of snow and ice, the means of

egress under his or her exclusive use and control. All corrodible structural parts thereof shall be kept painted

or otherwise protected against rust and corrosion. All wood structural members shall be treated to prevent

rotting and decay. Where these structural elements tie directly into the building structural system, all joints

shall be sealed to prevent water from damaging or corroding the structural elements.

410.480: Locks

The owner shall provide, install and maintain locks so that:

(A) Every dwelling unit shall be capable of being secured against unlawful entry.

(B) Every door of a dwelling unit shall be capable of being secured from unlawful entry.

(C) The main entry door of a dwelling containing more than three dwelling units shall be so designed or

equipped so as to close and lock automatically with a lock, including a lock with an electrically-operated

striker mechanism, a self-closing door and associated equipment. Every door of the main common entryway

and every exterior door into said dwelling, other than the door of such main common entryway which is

equipped as provided in the preceding sentence shall be equipped with an operating lock. (M.G.L. c. 143, §

3R.)

(D) Every entry door of a dwelling unit or rooming unit shall be capable of being secured from unlawful

entry.

(E) Every openable exterior window shall be capable of being secured.

(F) Locking devices shall comply with the requirements of 780 CMR 1017.4.1 to avoid entrapment in the

building.

410.481: Posting of Name of Owner

An owner of a dwelling which is rented for residential use, who does not reside therein and who does not

employ a manager or agent for such dwelling who resides therein, shall post and maintain or cause to be posted

and maintained on such dwelling adjacent to the mailboxes for such dwelling or elsewhere in the interior of

such dwelling in a location visible to the residents a notice constructed or durable material, not less than 20

square inches in size, bearing his name, address and telephone number. If the owner is a realty trust or

partnership, the name, address and telephone number of the managing trustee or partner shall be posted. If the

owner is a corporation, the name, address and telephone number of the president of the corporation shall be

posted. Where the owner employs a manager or agent who does not reside in such dwelling, such manager or

agent's name, address and telephone number shall also be included in the notice. (See M.G.L. c. 143, § 3S.)

410.482: Smoke Detectors and Carbon Monoxide Alarms

(A) Owners shall provide, install, and maintain in operable condition smoke detectors and carbon monoxide

alarms in every dwelling that is required to be equipped with smoke detectors and carbon monoxide alarms in

accordance with any provision of the Massachusetts General Laws and any applicable regulations of the State

Board of Fire Prevention (527 CMR), State Board of Building Regulations and Standards (780 CMR), or the

Board of Examiners of Plumbers and Gas Fitters (248 CMR).

(B) The board of health shall immediately notify the chief of the local fire department of any violation of 105

CMR 410.482 which is observed during an inspection of any dwelling.

(C) If any dwelling is found by the local fire department to be adequately equipped with smoke detectors and

carbon monoxide alarms, the board of health shall not be authorized by 105 CMR 410.482 to impose any

additional or differing smoke detector or carbon monoxide alarm requirement beyond that which has been

found sufficient by the local fire department.

410.483: Auxiliary Emergency Lighting Systems and Exit Signs

The owner of every multiple dwelling of ten or more units shall provide such dwelling with an auxiliary

emergency lighting system independent of the conventional lighting system, and with lighted signs indicating

both a primary and secondary means of egress, by a diagram or signal so as to assure recognition by all persons

regardless of their English speaking ability. Such lighting system signs shall be maintained in good working

order in compliance with any applicable regulations promulgated by the Commissioner of Public Safety (See

780 CMR 1023.0, 780 CMR 1024.0 and M.G.L. c.143, § 21D).

410.484: Building Identification

The owner shall affix to every building covered by 105 CMR 410.000, a number representing the address

of such building. The number shall be of a nature and size and shall be situated on the building so that, to the

extent practicable, it is visible from the nearest street providing vehicular access to such building (M.G.L. c.

148, § 59).

410.500:   Owner's Responsibility to Maintain Building and Structural Elements

Every owner of a residence shall maintain, in accordancecompliance with accepted building standards as defined in 105 CMR 410.010 or advisories issued by the Department, all building and structural elements so that they: the foundation, floors, walls, doors, windows, ceilings, roof, staircases, porches, chimneys, and other structural elements of his dwelling so that the dwelling

- Eexcludes wind, rain and snow, are pest-proof and is rodent-proof, watertight and free from chronic dampness, weathertight, in good repair,  and in every way fit for the use intended;

- Further, he shall maintain every structural element Are free from holes, cracks, loose plaster, or other defect where such holes, cracks, loose plaster or defect that renders the area difficult to keep clean, or constitutes creates an accident hazard,  or an insect or rodent harborage provides an entry or harborage for pests.; and

(C) Are free from mold and other fungi in amounts that may affect the health of the occupants

410.502: Use of Lead Paint Prohibited

No paint that contains lead shall be used in painting any surface of any dwelling. (See 105 CMR

460.000.)

410.503: Protective Railings and Walls

The owner of all dwellings shall provide:

(A) A safe handrail for every stairway that is used or intended for use by the occupant as required by 780

CMR: Massachusetts State Building Code.

(B) A wall or guardrail on the open side of all stairways no less than 30 inches in height. Any such guardrail

replaced or constructed after August 28, 1997 (effective date of Massachusetts State Building Code, Sixth

Edition) shall be not less than 34 inches in height (780 CMR 1022.2.2 and 3603.14.2.1).

(C) A wall or guardrail at least 36 inches in height, enclosing every porch, balcony, mezzanine, landing, roof

or similar place, which is 30 inches or more above the ground and that is used or intended for use by the

occupants. Any such wall or guardrail for other than Use Group R-4 and along opens sided floor areas,

mezzanines and landings in occupancies in Use Group R-3, replaced or constructed after August 28, 1997,

shall not be less than 42 inches in height (780 CMR 102 and 3603.14).

(D) Between all required guardrails and open handrails, balusters placed at intervals of no more than six

inches, or any other ornamental pattern between the guardrail or handrail and floor or stair such that a sphere

six inches in diameter can not pass through the opening. Any balusters or ornamental work constructed or

replaced after August 28, 1997 shall have no space greater than 4½ inches and in all use groups other than R-4,

shall not be constructed as to provide a ladder effect (780 CMR 1021 and 3603.14).

410.504: Non-absorbent Surfaces

The owner shall provide:

(A) On the floor surfaces of every room containing a toilet, shower or bathtub and every kitchen and pantry, a

smooth, noncorrosive, nonabsorbent and water proof covering. This shall not prohibit the use of carpeting in

kitchens and bathrooms, nor the use of wood in the kitchen, provided they meet the following qualifications:

(1) Carpeting must contain a solid, nonabsorbent, water repellent backing which will prevent the passage

of moisture through it to the floor below; and

(2) Wood flooring must have a water resistant finish and have no cracks to allow the accumulation of dirt

and food, or the harborage of insects.

(B) On the walls of every room containing a toilet, shower or bathtub up to a height of 48 inches, a smooth

noncorrosive, nonabsorbent and waterproof covering.

(C) On wall areas above built-in bathtubs having installed shower heads and in shower compartments up to

height not less than six feet above the floor level, with a smooth, noncorrosive, nonabsorbent waterproof

covering. Such wall shall form a watertight joint with each other and with either the tub, receptor or shower

floor.

410.505 510: Occupant's Responsibility Respecting Regarding Building and Structural Elements

The Every occupant of a residence shall exercise reasonable care in the use of the floors, walls, doors, windows, ceilings, roof, staircases, porches, chimneys, and other all building and structural elements of the dwelling their residence.

410. 5320: Protective Railings and Walls

(A) Every owner of a residence shall provide:

(A1) A safe handrail no less than 34 inches in height on each at least one side of an enclosed stairway for every stairway that is used or intended for use by the occupant.

(B2)   A wall or guardrail no less than 34 30 inches in height on the open side of all stairways. Any such guardrail replaced or constructed after August 28, 1997 (effective date of Massachusetts State Building Code, Sixth Edition) shall be not less than 34 inches in height. (780 CMR 1022.2.2 and 3603.14.2.1).

(C3) A wall or guard no less than 36 42 inches in height, enclosing every porch, balcony, mezzanine, landing, roof or similar place, which is 30 inches or more above the ground. and that is used or intended for use by the occupants. Any such wall or guardrail for other than Use Group R-4 and along opens sided floor areas, mezzanines and landings in occupancies in Use Group R-3, replaced or constructed after August 28, 1997, shall not be less than 42 inches in height.(780 CMR 102 and 3603.14).

(DB) In all guards systems, balusters must be placed at intervals of no more than four inches. , or any other oOrnamental patterns between the guardrail or handrail and floor or stair may not allow for such that a sphere four inches in diameter cannot to pass through the opening. 

(EC) Any bBalusters or ornamental work constructed or replaced after August 28, 1997 may not be constructed so as to create a horizontal climbing risk (a “ladder effect”) notwithstanding any other regulatory provision to the contrary.

410.501 5430: Weathertight Elements

(A)   An exterior openable window shall be considered weathertight only when:

(1)   Aall panes of glass are in place, unbroken and properly sealed;

(2)   Tthe window sash opens and closes fully without excessive effort; and

(3)   exterior cracks between the prime window frame and the exterior wall are caulked; and

(43)   Oone of the following conditions is met:

(b)   weatherstripping is applied such that the space between the window sash and the prime window frame is no larger than 1/16 inch at any point on the perimeter of the sash, in the case of double hung windows and 1/32 inch in the case of casement windows; or

(c)  (a)   Tthe window sash is sufficiently well-fitted such that it minimizes the infiltration of air or moisture and, without weatherstripping, the space between the window sash and the prime window frame is no larger than 1/816 inch at any point on the perimeter of the sash in the case of double hung windows and 1/1632 inch in the case of casement windows; or

(a)  (b) Aa storm window is affixed to the prime window frame in such a way as to not allow minimize the infiltration of air or moisture., with caulking installed so as to fill exterior cracks between the storm window frame and the prime window frame; or

(B)   An exterior door or a door leading from a residence or dwelling unit to a common passageway the exterior shall be considered to be weathertight only if:

(1)   Aall panes of glass are in place, unbroken and properly sealedcaulked; and

(2)   Tthe door opens and closes fully without excessive effort; and

(3)   exterior cracks between the prime door frame and the exterior wall are caulked; and

(4)   Oone of the following conditions is met:

(b)   weatherstripping is applied such that the space between the door and the prime door frame is no larger than 1/16 inch at any point on the perimeter of the door or

(c)  (a)  Tthe door is sufficiently well-fitted so that it minimizes the infiltration of air or moisture and such that, without weather-stripping, the space between the door and the prime door frame is no larger than 1/816 inch at any point on the sides of the door or _  inch at any point on the top or bottom of the door.; or

(a) (b) Aa storm door is affixed to the prime door frame in such a way as to not allow minimize the infiltration of air or moisture., with caulking installed so as to fill exterior cracks between the storm door frame and the prime door frame; or

(C)  A wall, floor, ceiling or other Building and structural elements shall be considered weather-tight only if when all cracks and spaces not part of heating, ventilating, or air conditioning systems are caulked or filled in sealed as to prevent infiltration of exterior air or moisture.

410.550540:   Installation of Screens

(A) The owner shall provide and install screens for exterior openable windows so that:

(1) They are in place during the period from April 1st to October 31st, both inclusive, in each year;

(2) They are no less than 16 mesh per square inch;

(3) They cover that part of the window designed to be opened, except in double hung windows where half screens shall meet this requirement; and

(4) They are tight fitting and prevent the entrance of pests.

(B) Expandable, temporary screens shall not satisfy the requirements of 105 CMR 410.540(A).

(C) The owner shall provide and install a screen doors for all doorways opening directly to the outside from any dwelling unit or rooming unit where the screen doors will be permitted to slide to the side or open in an outward direction, so that :

(1) They are in place during the period from April 1st to October 31st, both inclusive, in each year;

(2) They are 16 mesh per square inch

(3) They are equipped with a self-closing device except where the screen is designed to open to the side; and

(4) They are tight fitting and prevent the entrance of pests.

(D) 105 CMR 410.540(C) shall not apply to homeless shelters.

410.5560550: Extermination of Insects, Rodents and Skunks Elimination of Pests

Owners and occupants are responsible for the control and elimination of pests in accordance with the requirements of 333 CMR Pesticide Board and as follows:

(A) Residences containing one dwelling unit:

(1) The occupant of a dwelling containing one dwelling unit shall maintain the unit free from all rodents, skunks, cockroaches and insect pest infestation, and shall be responsible for exterminating them, pest elimination.

(2) provided, however, that tThe owner shall be responsible for pest elimination if an inspection by the board of health reveals the owner has not maintained maintain any screen, fence or other structural element or other building elements necessary to keep rodents and skunks pests from entering the dwelling.

(B)The owner of a dwelling residence containing two or more dwelling units, including a homeless shelter or a rooming house, shall maintain it and its premises free from all rodents, skunks, cockroaches and insect infestation and shall be responsible for exterminating them pest elimination.

(C) Extermination shall be accomplished by:

(1) Eliminating the harborage places of insects and rodents;

(2) Removing or making inaccessible materials that may serve as food or breeding ground for pests;

(3) Poisoning, spraying, fumigating, trapping pests; or

(4) Any other recognized and legal pest elimination method.

(D) All use of pesticides within the interior of a residence shall be in accordance with  applicable laws and regulations of the Department of Food and Agriculture's Pesticide Board, including those appearing at 333 CMR 13.00.

(1) Pesticide applicators or their employers must give at least 48 hours pre-notification to occupants of all residential units prior to any routine commercial application of pesticides for the control of indoor household or structural indoor pests.

(2) Pre-notification shall be made in writing and contain the following information:

(a) Name and phone number of the company making the application;

(b) Proposed date of application;

(c) Locations to be treated; and

(d) Names, EPA Registration Numbers, and active ingredients for the pesticide products that may be used.

(3) Owners and occupants shall:

(a) Provide access to common areas and interior units, upon receipt of notice required by 105 CMR 410.550(B)(2); and

(b) Adhere to all instructions provided by the pesticide applicator in preparing the common areas and units prior to the application of pesticides.

(C) The owner of a rooming house shall maintain it and its premises free from all rodents, skunks, cockroaches and insect infestation, and shall be responsible for exterminating them.

(C)(E) The occupant of a residence shall maintain the unit in a sanitary manner so as to prevent the attraction of pests.

(D)   Extermination shall be accomplished by eliminating the harborage places of insects and rodents, by removing or making inaccessible materials that may serve as their food or breeding ground, by poisoning, spraying, fumigating, trapping or by any other recognized and legal pest elimination method.  All use of pesticides within the interior of a dwelling, dwelling unit, rooming house, or mobile home shall be in accordance with applicable laws and regulations of the Department of Food and Agriculture's Pestici’e Board, including those appearing at 333 CMR 13.00, which provide, among other things, that pesticide applicators or their employers must give at least 48 hours pre-notification to occupants of all residential units prior to any routine commercial application of pesticides for the control of indoor household or structural indoor pests.

(D)(F) The owner of a residence, except for a homeless shelter, containing four or more dwelling units or rooming units shall implement and maintain an Integrated Pest Management (IPM) plan by conducting onsite inspections at least every four months. The IPM plan shall be designed and implemented so as to:

(1) Eliminate entry points for pests

(2) Ensure the residence is maintained in a clean and sanitary manner to minimize the availability of food and water for pests;

(3) Provide information to occupants regarding their role in controlling pests;

shall conduct an inspection of each unit prior to a new occupancy to (4)monitoridentify the presence of pests. Homeless shelters shall establish pest management policies that include periodic inspection for pest infestation.

(E)  An owner required by 105 CMR 410.560(D) to implement an IPM plan shall maintain a record documenting the following activities conducted within the residence:

(1) Inspection results;

(2) Complaints filed by occupants;

(3) The date, location, product name, and name of any person applying pesticides; and

(4) Modifications to the original IPM plan.

(G) Owners shall make the record documented inspection results and actions taken available upon request by the board of health. Documentation shall include:

(1) Monitoring the presence of pests;

(2) Eliminating entry points for pests;

(3) Documenting results of the inspection; and

(4) Documenting actions taken as a result of the inspection to include:

(a) Repairs made;

(b) Location of pesticides applied and by whom; and

(c) Date of a follow-up inspection.

410.551: Screens for Windows

The owner shall provide screens for all windows designed to be opened on the first four floors opening directly to the outside from any dwelling unit or room unit provided, that in an owner-occupied unit, the owner need provide screens for only those windows used for ventilation. All new or replacement screens shall be of not less than 16 mesh per square inch.

Said screens:

(1) shall cover that part of the window that is designed to be opened but in no case less than the area as required in 105 CMR 410.280(A); and

(2) shall be tight fitting as to prevent the entrance of insects and rodents around the perimeter.

(3) Expandable temporary screens shall not be deemed to satisfy the requirements of 105 CMR 410.551(1) or (2).

410.552: Screens for Doors

The owner shall provide a screen door for all doorways opening directly to the outside from any dwelling unit or rooming unit where the screen door will be permitted to slide to the side or open in an outward direction, provided, that in an owner-occupied unit, the owner need provide screens only for those doorways used for ventilation. All new or replacement screens in screen doors shall be of not less that 16 mesh per square inch.

Said screen door:

(1) shall be equipped with a self-closing device except where the screen is designed to slide to the side; and (2) shall be tight-fitting as to prevent the entrance of insects and rodents around the perimeter; and

410.553: Installation of Screens The owner shall provide and install screens as required in 105 CMR 410.551 and 410.552 so that they shall be in place during the period between April first to October 30th, both inclusive, in each year.

410.601:   410.5760: Refuse Collection of Garbage and Rubbish

(A) Except as provided in 105 CMR 410.560(C) and 105 CMR 410.560(E)(4), Tthe owner of any dwelling residence that contains three two or more dwelling units, the owner of any rooming house, homeless shelters, the owner of any manufactured home parks, and the occupant of any other dwelling unit place shall be responsible for and pay for the final collection or and ultimate disposal or incineration of refuse. garbage and rubbish by means of:

(A)   The regular municipal collection system; or

(B)   Any other collection system approved by the board of health; or

(C)   when otherwise lawful, a garbage grinder which grinds garbage into the kitchen sink drain finely enough to ensure its free passage, and which is otherwise maintained in a sanitary condition; or

(D)   when otherwise lawful, a garbage or rubbish incinerator located within the dwelling which is properly installed and which is maintained so as not to create a safety or health hazard; or

(E)   when otherwise lawful, by backyard composting of compostable material, provided that the composting operation does not attract rodents or other vectors and does not create a nuisance, and provided further that in the case of composting by an occupant, the occupant obtain the prior written permission of the owner.

(F)  any other method of disposal which does not endanger any person and which is approved in writing by the board of health. (See 105 CMR 410.840.)

(B)  Occupants of any residence shall follow the requirements established by their community for separation of recyclables, yard waste, food waste, and/or bulk-item materials from refuse.

(C)  Occupants shall be responsible for the disposal of their bulk-items in accordance with requirements established by their community.

410.600:   (D) Storage of Garbage and Rubbish

(A) (1) Refuse  Garbage or mixed garbage and rubbish shall be stored in watertight receptacles with tight-fitting covers. Said Receptacles and covers shall be of metal or other durable, pest-proof and deter access by wildlife rodent-proof material. Garbage and rubbish shall be put out for collection no earlier than the day of collection.

(B) (2) Plastic bags or biodegradable liners shall be used as a liner to store refuse garbage or mixed rubbish and garbage only if used as a liner in the required receptacles. watertight receptacles with tight-fitting covers as required in 105 CMR 410.600(A). provided that the plastic bags may be put out for collection except in those places where such practice is prohibited by local rule or ordinance or except in those cases where the Department of Public Health determines that such practice constitutes a health problem. For purposes of the preceding sentence, in making its determination the Department shall consider, among other things, evidence of strewn garbage, torn garbage bags, or evidence of rodents.

(3)(C) If a cart-based automated or semi-automated system is used for storage and collection of garbage or mixed rubbish and garbage, no liner shall be required,

(4)(D) Recyclables and garbage or mixed garbage and rubbish shall be stored in a manner that does not create a nuisance obstruct an egress, cause disease or health risk to humans or pets, or cause damage to property or the environment and is in accordance with the municipality’s requirements.

(C) (E) Collection:

(1) The owner of any residence dwelling that contains three or more dwelling units and the owner of any rooming house, and the occupant of any other dwelling place shall provide as many a sufficient number of receptacles required by 105 CMR 410.560(D)(1) for the storage of refuse and recyclables garbage and rubbish as are sufficient to contain the accumulation before final collection or ultimate disposal. and shall locate them so as to be convenient to the tenant and so that no objectionable odors enter any dwelling.

(2) An owner who provides a dumpster or other suitable container, shall locate them on the property so as to be convenient to the occupant, so that no objectionable odors enter any residence, and in accordance with 527 CMR: Massachusetts Fire Code.	Dumpsters shall be covered, protected from leakage, and placed on an impervious material.

(D) (23) The occupants of each dwelling unit and rooming unit residence shall be responsible for the proper placement of his their garbage and rubbish refuse in the receptacles provided by the owner, required in 105 CMR 410.600(C) or at the point of collection in any dumpster or suitable container if provided by the owner, or according to the municipality’s requirements.

(34) In municipalities with a Pay as You Throw Program in which a fee per bag of waste to be collected is charged, the occupant shall follow all municipal requirements and shall be responsible for all fees.

(45) Refuse shall be put out for collection in accordance with local municipal requirements, but no earlier than the day of before collection. unless Refuse put out prior to the day of collection must be placed in containers meeting the requirements of 105 CMR 410.560(D)(1).

(56) Plastic bags may be put out the day of collection, except in those places where such practice is prohibited by local rule or ordinance or in those cases where the board of health determines that such practice constitutes a health problem.

(F) Disposal may be by means of:

(1) the The regular municipal collection system; or

(2) any Any other collection system approved by the board of health; or

(3) when When otherwise lawful, a garbage grinder which grinds garbage into the kitchen sink drain finely enough to ensure its free passage, and which is otherwise maintained in a sanitary condition; or

(4) When otherwise lawful, a garbage or rubbish incinerator located within the dwelling which is properly installed and which is maintained so as not to create a safety or health hazard; or

(4) when When otherwise lawful, by backyard composting of compostable material, provided that the composting operation does not attract rodents or other vectors pests and does not create a nuisance, and provided further that in the case of composting by an occupant, the occupant obtains the prior written permission of the owner; or

(5) any Any other method of disposal which does not endanger any person and which is approved in writing by the board of health. (See 105 CMR 410.840.)

410.602 570: Maintenance of Areas  Free from Garbage and Rubbish Refuse in a Sanitary and Safe Condition

(A) Land:

The owner of any parcel of land, vacant or otherwise, shall be responsible for maintaining such parcel of land in a clean, safe, and sanitary condition and free from: garbage, rubbish or other refuse.

(1) Refuse;

(2) Pests;

(3) Potential injury hazards;

(4) Conditions contributing to the accumulation of standing water; or

(5) Any other condition which affects the health, safety, or well-being of the occupants of any residence or of the general public.

The owner of such parcel of land shall correct any condition caused by or on such parcel or its appurtenance which affects the health or safety, and well-being of the occupants of any dwelling or of the general public.

(B) Dwelling Units Residences:

The occupants of any dwelling unit residence shall be responsible for maintaining in a clean and sanitary condition and free of garbage, rubbish, refuse or other filth or causes of sickness that part of the dwelling residence which he or she they exclusively occupies occupy or controls.

(C)   Dwellings Containing Less than  Three Dwelling Units.  In a dwelling that contains less than three dwelling units, the occupant shall be responsible for maintaining in a clean and sanitary condition, free of garbage, rubbish, other filth or causes of sickness the stairs or stairways leading to his dwelling unit and the landing adjacent to his dwelling unit if the stairs, stairways or landing are not used by another occupant.

(C) Common Areas:

(1) In any dwelling residence, the owner shall be responsible for maintaining in a clean, and sanitary, and safe condition free of garbage, rubbish, refuse or other filth or causes of sickness, that part of the dwelling residence which is used in common by all the occupants of a residence and which is not occupied or controlled by one occupant exclusively.

(2) For common stairways, the occupants shall be responsible for maintaining the landing, porch, or deck adjacent to their residence dwelling unit in a clean and sanitary condition, free of obstructions, refuse, filth, causes of sickness, or potential injury hazard.

(D) Private Passage or Right of Way:

The owner of any dwelling residence abutting a private passageway or right-of-way owned or used in common with other dwellings residence(s) or which the owner or occupants under his control the occupants of the owner’s property have the right to use, or are in fact using, shall be responsible for maintaining in a clean and sanitary condition free of refuse garbage, rubbish, other filth or causes of sickness that part of the passageway or right-of-way which abuts his the owner’s property and which he the owner or the the occupants of the owner’s property under his control have the right to use, or are in fact using, or which he the owner owns.

410.620:   Curtailment Prohibited

No owner or occupant shall cause or allow any service, facility, equipment, or utility which is optional or required to be made available by 105 CMR 410.000 to be removed from or shut off from any occupied dwelling except for such temporary period as may be necessary during actual repairs, or alterations, or replacement and where reasonable notice of curtailment of service is given to the occupant, in writing where practical, or during temporary emergencies when curtailment of service is approved by the board of health.

If any such service or facility that a person is required to provide by 105 CMR 410.000 or has agreed to supply by a written letting agreement becomes curtailed, that person shall take immediate steps to cause its restoration.  (See M.G.L. c. 186, § 14.)

410.700: Inspectors Duty to Classify Violations

Any one or more of the conditions specified in 105 CMR 410.750, when found to exist in residential

premises, shall always be deemed to be conditions which may endanger or materially impair the health or

safety, and well-being of an occupant or the public. The conditions specified in 105 CMR 410.750 are

specifically not intended as an exhaustive enumeration of such conditions. In addition to the conditions

specified in 105 CMR 410.750, the inspector shall determine if any other violations of 105 CMR 410.100

through 410.620, or any other conditions, are conditions which may endanger or materially impair the health or

safety, and well-being of an occupant or the public.

410.800:   General Administration

The provisions of 105 CMR 400.000:   State Sanitary Code, Chapter I shall govern the administration and enforcement of these minimum standards except as supplemented by 105 CMR 410.810 through 410.960.

410.810:   Access for Repairs and Alterations

Every occupant of a dwelling, dwelling unit, or rooming unit shall give the owner thereof, or his agent or employees, upon reasonable notice, reasonable access, if possible by appointment, to the dwelling, dwelling unit, or rooming unit for the purpose of making such repairs or alterations as are necessary to effect compliance with the provisions of 105 CMR 410.000.

410.820 600: Inspection upon Request

(A) The board of health shall inspect a dwelling or dwelling unit residential unit and common areas upon receipt of a written, oral,  or telephonic, or electronic request for inspection from an occupant of that residential dwelling or rooming unit. The board of health shall conduct this inspection regardless of whether:

(1) The occupant the person requesting the inspection has previously notified the owner of the alleged condition;  dwelling of the condition(s) within the dwelling.  All interior inspections shall be done in the company of the occupant or the occupant's representative.

(2) There is an eviction, litigation, or other dispute pending between the owner and the occupant; or

(3) The occupant requesting the inspection is anonymous or requests the board of health keep his/her their name confidential.

(B) The board of health shall investigate complaints or requests for inspection concerning a residential unit received from a person who is not an occupant of that residential unit to determine if there is sufficient cause for an inspection. The investigation may consist of a visual assessment from a public way, record review, or contact with the occupants by telephone or in person.

Upon completion of the investigation, the board of health shall either conduct an inspection or, if it determines there is no justification for an inspection, record the decision in the record book as required by 105 CMR 410.600(D).

(A)   (C) The board of health shall use its best efforts to schedule and complete an inspection at a time mutually satisfactory to the occupant and the board of health.:

(1) The inspection shall be conducted within one business day after the receipt of a request if alleged conditions include violations enumerated in 105 CMR 410.630(A).

(2) The inspection shall be conducted within five business days after receipt of a request if alleged conditions do not include any of the violations enumerated in 105 CMR 410.630(A).

(1)   within 24 hours after a receipt of a request when the condition or conditions alleged to exist include one of the following:

(a)   failure to maintain a supply of water connected with a safe water supply as required in 105 CMR 410.180; or

(b)   failure to provide heat and to provide or maintain heating facilities in proper condition as required by 105 CMR 410.200 through 410.202; or

(c)   failure to provide light as required by 105 CMR 410.254 and 410.253; or

(d)   failure to provide and maintain a sanitary drainage system as required by 105 CMR 410.300; or

(e)   failure to maintain in safe operating condition any facilities fixtures and systems listed in 105 CMR 410.351; or

(f)   termination or failure to restore promptly, water, hot water, heat electricity or gas; and

(g)   failure to maintain exits unobstructed and in a safe condition as required by 105 CMR 410.451 and 410.452; or

(h)   failure to maintain every entry door of a dwelling or dwelling unit as required by 105 CMR 410.480(B) and 410.480(D); or

(i)   failure to maintain a dwelling unit free from leaks as required in 105 CMR 410.500; or

(j)   failure to maintain a porch, balcony, roof or exterior stairway in a safe condition as required in 105 CMR 410.500, 410.503(B), and 410.503(C); or

(k)   failure to maintain a dwelling or dwelling unit free from rodents, skunks, cockroaches and insect infestation as required by 105 CMR 410.550; or

(2) within five calendar days after receipt of a request when the condition or conditions alleged to exist does not include any of the violations enumerated in 105 CMR 410.820(A)(1).

(B)   (D) The board of health shall keep a record of all requests for inspections,  in a bound book with numbered pages.  The information to be recorded which shall include, but need not be limited to, the following:

(1) the  nName, if given,  of the person requesting the inspection if provided, which may be kept confidential by the board of health;

(2) the t Time and date of each such request;

(3) the LLocation of the dwelling residence;

(4) tThe nature of the alleged violation(s); and

(5) the dDate the inspection is conducted.; and

(6) Any decision not to conduct an inspection requested by a person who is not an occupant.

In lieu of the above, the required records may be maintained within a computer system.

410.821 610: Inspection Form

Each board of health shall adopt and use a printed an inspection report form available from the Department, or any other inspection report form provided it includes, at a minimum, which must include, but need not be limited to, the following:

(A)   Specifically labeled spaces for:

(1) (A) the The name of the inspector;

(2) (B) the The date and time of the inspection or investigation;

(3) (C) the The location of the dwelling or dwelling unit residence inspected;

(4) the date and time of any scheduled follow-up re-inspection;

(4) (D) The need for an additional inspection by a specialized inspector pursuant to 105 CMR 410.620(A) and the reason that such inspection is necessary;

(5) (E) a A description of the conditions constituting violations;

(6) (F) a A listing of the specific provisions of 105 CMR 410.000 or other applicable laws, ordinances, by-laws, rules or regulations that appear to be violated;

(7) (G) a A determination by the official inspecting the premises whether the violations noted are listed in 105 CMR 410.750 630(A), and whether the effect of any violation(s) or conditions not listed in 105 CMR 410.750 630(A) may endanger or materially impair the health, or safety, or well-being of any person(s) occupying the premises; and

(8) (H) the The signature of the inspector preceded by the following statement:  “This inspection report is signed and certified under the pains and penalties of perjury.”; and

(B)   A brief summary of the legal remedies available to the occupant of the affected premises, as described in Guidance pertaining to Tenants Legal Rights and Responsibilities, issued by the Department, Appendix A of this regulation, followed by this statement:

"The information presented above is only a summary of the law.  Before you decide to withhold your rent or take any other legal action, it is advisable that you consult an attorney.  If you cannot afford to consult an attorney, you should contact the nearest Legal Services Office which is (name of Legal Services Office), (address), (telephone number)."

410.822 620: Conduct of Inspections

(A) At the time of the inspection, the inspector shall record all violations if any of 105 CMR 410.000. and shall complete an inspection report form which conforms to the requirements of 105 CMR 410.821(A) and 410.821(B).

(1) If assistance of a specialized inspector, which who is not immediately available, is necessary to fully complete the inspection report, such report shall be completed to the fullest extent feasible at the time of the inspection, noting thereon the reason of possible violations for which a separate inspection by a specialized inspector appears to be necessary.

(2) The need for such separate inspection shall in no way delay the normal processing or issuing of orders pursuant to 105 CMR 410.830 640 through 410.833 680. The board of health shall use its best efforts to schedule the separate inspection promptly, at a mutually satisfactory time to for all individuals involved.

(3) A copy of 105 CMR 410.000:  Minimum Standards of Fitness for Human Habitation (State Sanitary Code, Chapter II) shall be made available upon request, free of charge or at a cost which is no greater than the board's own cost for each copy. A copy shall be made available for review at no cost.

(B)   Each inspection of a dwelling unit shall include at a minimum the condition alleged to be in violation and all those standards found in 105 CMR 410.750(A) through (O) except as otherwise provided in 105 CMR 410.822(B)(1) through 410.822(B)(4).  A violation found in a common area shall be considered as a violation which exists in each unit in the dwelling which may be affected by such violations.

(1)   An occupant shall be informed of his/her right to a comprehensive inspection at the start of said inspection.  A comprehensive inspection will be carried out if the occupant so requests.

(B)  At the beginning of an inspection, an occupant shall be informed of his or her the right to a comprehensive inspection, and a comprehensive inspection shall be conducted if the occupant so requests.

(1) Each inspection of a residence shall include at a minimum the condition alleged to be in violation and those standards listed in 105 CMR 410.630(A).

(2) A violation found in a common area shall be considered a violation which exists in each dwelling or rooming unit in which occupants have access to that area. and utilize that portion of the common area, where the violation exists.

(2)  The inspection as required in 105 CMR 410.822(B) shall not be required from September 15 to June 15, inclusive, if the complaint relates solely to the lack of heat pursuant to 105 CMR 410.200 or 410.201, however, a comprehensive inspection, will be carried out if the occupant so requests.

(3) From September 15 to May 31 inclusive, if the complaint relates solely to the lack of heat pursuant to 105 CMR 410.160 or 410.180 and a comprehensive inspection is requested, the comprehensive inspection will not be required during the lack of heat investigation but shall be conducted:

(a) Within one business day when violations observed or alleged are violations listed in 105 CMR 410.630(A); or

(b) Within five business days for all other violations listed in 105 CMR 410.000.

(3) (4) Where an inspection reveals a condition or conditions which present such an imminent threat to the life, health or safety of the occupants, and that immediate steps must be taken by the inspector to order compliance, an inspection as required in by 105 CMR 410.822 620(B) may be delayed until after such steps are taken, but such inspection shall be completed in a timely manner.

(4) Where a reinspection is made in order to determine compliance with a previously issued order, the inspection may be sufficient if it includes those items previously identified as violations unless additional violations have been identified in a subsequent complaint. ]

(C) A verbal or written summary of the conditions noted during the inspection shall be given to the occupant or the occupant’s representative at the conclusion of the inspection. A written summary shall be provided at the time of the inspection if requested by the occupant or the occupant’s representative.

Such report shall indicate the need for additional inspection by a specialized inspector, if necessary. If a written report is requested at the time of the inspection, it shall be left with the person making the request.

(D) Pest Inspections:

The board of health shall conduct a pest inspection in accordance with the provisions of 105 CMR 410.620 and include, at a minimum, examination of the areas relevant to the type of pest:

(1) Insects and Arachnids:

(a) Interior of residence;

(b) Common areas adjacent to the residence;

(c) All units adjacent to the residence where the presence of bed bugs have been confirmed;

(d) Exterior perimeter of the residence; and

(e) Areas previously treated, as documented in accordance with the requirements of 105 CMR 410.550(F)(4)(b).

(2) Rodents and Other Pests:

(a) Interior of residence;

(b) Basement or crawl space;

(c) Accessible attics;

(d) Exterior perimeter of the residence.

(e) Areas previously treated, as documented in accordance with the requirements of 105 CMR 410.550(F)(4)(b).

(E) Mold Chronic Dampness Inspections:

(1) When a board of health inspector discovers the existence of dampness or presence of mold during an inspection, he or she they shall investigate the potential sources of moisture, to the best of their ability, in order to identify whether chronic dampness exists. The following may be causes of chronic dampness which should be investigated:

(a) Plumbing leaks;

(b) Structural defects which allow the penetration of moisture;

(c) Improper or inoperable mechanical or natural ventilation required by 105 CMR 410.220;

(d) Improperly maintained accessible heating and ventilation ductwork;and

(e) Areas subject Other causes that could lead to chronic dampness or other moisture sources that may contribute to mold growth.

(2) The laboratory test Boards of health shall not be required to conduct environmental testing for mold.  Any environmental test results for mold shall not constitute a violation of 105 CMR 410.000. results from air sampling conducted in a residence shall not in and of themselves constitute a violation of 105 CMR 410.000.

410. 630: Conditions Deemed to Endanger or Materially Impair Health or Safety

(A) The following conditions, when found to exist in a residence  residential premises, shall always be deemed conditions which may endanger or materially impair the health, or safety and well-being of a person or persons occupying the premises:

(1) Failure to provide and maintain a supply of water sufficient in quantity, pressure and temperature, both hot and cold, to meet the ordinary needs of the occupant, required by 105 CMR 410. 130, 410.140, and 410. 150, for a period of 24 hours or longer.

(2) Failure to provide heat as required by 105 CMR 410. 180 or improper venting or use of a space heater or water heater prohibited by 105 CMR 410. 160(C) and 410. 170.

(3) Shutoff and/or failure to restore electricity, gas, or water.

(4) Failure to provide the electrical facilities required by 105 CMR 410. 300(A) through (E).

(5) Failure to provide a safe supply of water required by 105 CMR 130(A).

(6) Failure to provide a toilet and maintain a sewage disposal system in operable condition required by 105 CMR 410. 110(A) and (B), and 410. 130(B).

(7) Failure to provide and maintain adequate exits, or the obstruction of any exit, passageway or common area caused by any object, including refuse, which prevents egress in case of an emergency required by 105 CMR 410. 260.

(8) Failure to comply with the security requirements of 105 CMR 410. 270(A).

(9) Failure to comply with any provisions of 105 CMR 410. 560 and 410.570, which results in any accumulation of refuse, filth or other causes of sickness which may provide a food source or harborage for rodents, insects or other pests or otherwise contribute to accidents or to the creation or spread of disease.

(10) The presence of lead based paint on a dwelling or dwelling unit in violation of 105 CMR 460.000: Lead Poisoning Prevention and Control.  (See M.G.L c. 111, §§ 190 through 199.)

(11) Roof, foundation, or other structural defects in violation of 105 CMR 410.500 that may expose the occupant or anyone else to fire, burns, shock, accident or other dangers or impairment to health or safety required by 105 CMR 410.500.

(12) Failure to install or maintain electrical, plumbing, heating and gas-burning facilities in compliance with  accepted standards as defined in 105 CMR 410.010 for plumbing, heating, gasfitting and electrical wiring standards or failure to maintain such facilities required by 105 CMR 410. 230 and 410. 240, so as to expose the occupant or anyone else to fire, burns, shock, accident or other danger or impairment to health or safety.

(13) Any defect in asbestos material used as insulation or covering on a pipe, boiler or furnace which may result in the release of asbestos dust or which may result in the release of powdered, crumbled or pulverized asbestos material in violation of 105 CMR 410. 250.

(14) Failure to provide a smoke detector or carbon monoxide alarm required by 105 CMR 410. 330.

15) Failure to provide and maintain a safe handrail or protective railing or guard for every stairway, porch, balcony, roof or similar place with balusters spaced as required by 105 CMR 410.520.

(16) Failure to maintain the premises free from pests required by 105 CMR 410.550(A) and (B).

(17) Any other violation of 105 CMR 410.000 not enumerated in 105 CMR 410. 630(A) shall be deemed to be a condition which may endanger or materially impair the health or safety and well-being of an occupant upon the failure of the owner to remedy said condition within the time so ordered by the board of health.

(B) The health official, upon viewing the specific condition, may include determine that any condition that violates violation listed in 105 CMR 410.100 through 410.580 which in the health official’s opinion may is a condition that may endanger or impair the health, safety, or well-being of a person or persons occupying the premises. Conditions identified by the inspector as conditions deemed to endanger or impair the health, safety or well-being of a person shall be noted as such on the correction order and the inspection report.

410.830 640: Correction Orders Timeframes for Correction of Violations

(A) If an inspection or examination as provided for in 105 CMR 400.100 (State Sanitary Code I: General Administrative Procedures) and/or 105 CMR 410.820 600 (State Sanitary Code Chapter II:  Minimum Standards of Fitness for Human Habitation) reveals that a dwelling residence does not comply with the provisions of 105 CMR 410.000, the board of health or its designated agent shall:

(A) (1) Within 12 hours after the inspection, order the owner or occupant to make a good faith effort to correct within 24 hours of service any of the violations cited pursuant to in 105 CMR 410.630(A).

(1)   failure to maintain a supply of water connected to a safe water supply as required in 105 CMR 410.180; or

(2)   failure to provide heat and to provide or maintain heating facilities in proper condition as required by 105 CMR 410.200 or 410.201; or

(3)   failure to provide light as required by 105 CMR 410.254; or

(4)   failure to provide and maintain a sanitary drainage system as required by 105 CMR 410.300; or

(5)   failure to maintain in safe operating condition any facilities fixtures and systems listed in 105 CMR 410.351; or

(6)   termination or failure to restore promptly water, hot water, heat, electricity or gas; or

(7)   failure to maintain exits unobstructed as required by 105 CMR 410.451; or

(8)   failure to maintain every entry door of a dwelling unit as required by 105 CMR 410.480(D); or

(9)   failure to maintain a dwelling unit free from leaks as required by 105 CMR 410.500; or

(10)   failure to maintain a porch, balcony, roof or exterior stairway in a safe condition as required by 105 CMR 410.500; or

(11)   failure to maintain a dwelling or dwelling unit free from rodents, skunks, cockroaches and insect infestation as required by 105 CMR 410.550.

(B)  (2) Wwithin five business seven calendar days after the inspection, order the owner or occupant to make a good faith effort to correct, begin necessary repairs or contract in writing with a third party to correct, within  14 calendar days of service, any of the following violations:

five days for correction of all other violations or conditions listed in 105 CMR 410.750, 410.351 and 410.550 and to make a good faith effort to substantially correct all violations within a period determined by the board of health but not exceeding 30 days.

(a) Lack of Failure to provide as required by 105 CMR 410.100(A) a kitchen sink of sufficient size and capacity for washing dishes, or and kitchen utensils, or lack of a stove and oven, or a refrigerator, or any defect that renders either this equipment inoperable required by 105 CMR 410.100(A);

(b) Failure to provide as required in by 105 CMR 410.110(A)(2) and (3) a washbasin, sink and shower or bathtub required in 105 CMR 410.110(A)(2) and (3) or any defect which renders them  this equipment inoperable;

(c) Any defect in the electrical, plumbing, or heating system which makes such system or any part thereof in violation of relevant state codes generally accepted plumbing, heating, gasfitting, or electrical wiring standards that do not create an immediate hazard; or

(d) Any condition, which, in the opinion of the inspector, has the potential to become a condition which may pose a significant threat to the health, safety or well-being of the occupants if left uncorrected for more than 14 calendar days.

(c) within five days after the dates for compliance specified in an order issued pursuant to 105 CMR 410.830, the board of health shall make an onsite inspection to determine whether there has been compliance with said order; provided, that said inspection shall be made within 24 hours of the dates for compliance specified in an order if one or more of the violations or conditions are determined to be conditions which may endanger the health or safety, and well-being of the occupant(s) as defined in 105 CMR 410.750.  An inspection under 105 CMR 410.830 shall comply with the requirements of 105 CMR 410.822.

(3) Within five business seven calendar days after the inspection, order the owner or occupant to correct, within 30 calendar days of service, any violations not listed in 105 CMR 410.640(A)(1) or (2).

(B) No order shall exceed 30 calendar days for the correction of violations, nor shall the timeframes listed in the original order be extended beyond the original date, unless a hearing, in accordance with 105 CMR 410.800 through 410.850, has been conducted.

410.831 650: Dwellings Unfit for Human Habitation; Hearing; Condemnation; Order to Vacate; Demolition

(A) Finding that a dwelling residence or portion thereof is unfit for human habitation:

If an inspection conducted pursuant to 105 CMR 400.100 or 105 CMR 410.820 600 reveals that an occupied dwelling residence or portion thereof is unfit for human habitation, the board of health may shall , subject to(after complying with 105 CMR 410.831 650(B), (C), or (D), if the dwelling residence is occupied) issue a written finding that the dwelling residence or portion thereof is unfit for human habitation.  The finding shall include a statement of the material facts and conditions upon which the finding is based.

(B) Prior notification to occupant(s) and owner:

If the dwelling residence or portion thereof is occupied, the board of health shall, prior to issuing a finding under 105 CMR 410.831 650(A), provide written notice to the occupant(s) and owner which shall include:

(1) Identification of the dwelling residence (address and apartment number, if any);

(2) A copy of the inspection report;

(3) A statement that the board of health will consider issuing a finding that the dwelling residence or a specifically identified portion thereof is unfit for human habitation;

(4) A statement that this finding may result in an order of condemnation requiring the owner to secure the dwelling residence and requiring the occupant(s) to vacate the dwelling residence;

(5) A statement of the time and place of a public hearing which the board of health will conduct in order to determine whether the dwelling residence or portion thereof is unfit for human habitation, and whether an order to secure and vacate should be issued.

(C) Service of notice:

The notice shall be served on the occupant(s) and owner in accordance with 105 CMR 410.833 680.

(D) Hearing if dwelling residence or portion thereof is occupied:

If the dwelling residence or portion thereof is occupied, then the board shall, prior to issuing a finding under 105 CMR 410.831 650(A), and at least five calendar days after service of the notice required by 105 CMR 410.831 650(B), conduct a public hearing to determine whether the dwelling residence or portion thereof is unfit for human habitation and whether an order to secure and to vacate should be issued. At the hearing the occupant(s), owner, or any other affected party shall be given an opportunity to be heard, to present witnesses or documentary evidence and to show why the dwelling residence or portion thereof should or should not be found unfit for human habitation, and why an order to vacate and an order to close-up secure should or should not be issued.

(E) Exception to notification and hearing requirements:

If at any time the board of health determines in writing that the danger to the life or health of the occupant(s) is so immediate that no delay may be permitted, then the board of health may immediately issue a finding that an occupied dwelling residence or portion thereof is unfit for human habitation without providing the notification or hearing specified in 105 CMR 410.831 650(B) and (C) (D). This emergency determination must include a written explanation of the conditions presenting an immediate danger. A copy of the determination of immediate danger, and a copy of the finding of unfitness for human habitation shall be sent to each affected occupant, and to the owner. The determination of immediate danger shall include a statement advising the owner and occupant of their right to a hearing in accordance with 105 CMR 410.800(A).

(E) (F) Condemnation, order to vacate, order to secure:

(1)  At the same time, or at any time after the board of health issues a finding that a dwelling residence or portion thereof is unfit for human habitation, the board may issue an order condemning the dwelling residence or portion thereof and an order to vacate the dwelling residence or portion thereof, and an order requiring the owner to secure the dwelling residence or portion thereof.

(2)  If the dwelling residence or portion thereof which is ordered to be secured is unoccupied, and therefore no public hearing was conducted prior to the issuance of the order, then the owner or any other affected person shall have the right to request a hearing in accordance with 105 CMR 410.850 800 through 410.860.

(3) No dwelling residence or portion thereof which is ordered to be secured shall be occupied without the prior written permission of the board of health based upon the board's written finding that the dwelling residence or portion thereof to be occupied is fit for human habitation.

(F) (G) Demolition:

If, one year after the issuance of an order to secure, compliance with the minimum standards set forth in 105 CMR 410.000 has not been effected achieved, then the board of health may cause the dwelling residence or portion thereof to be demolished or removed provided the requirements of 105 CMR 410.800(A) have been met.

410.660: Reinspections

(A) The purpose of aA reinspection is to determine compliance with an existing order and shall be conducted within:

(1) 24 hours after the date for compliance of violations required by specified in 105 CMR 410.640(A); or

(2) Five business Seven calendar days after the date for compliance of violations required by specified in 105 CMR 410.640(B) and (C).

(B) New violations identified during the conduct of the reinspection to determine compliance with an existing order shall be addressed in a new complaint.

(C) The board of health shall identify, in writing, all violations corrected and those that remain uncorrected at the time of the reinspection.

(C) (D) Reinspection dates shall be calculated based on the date of service pursuant to 105 CMR 410.680(C).

410.832 670: Content of Orders Order to Correct Violations

(A) Every order authorized by 105 CMR 410.000 shall:

(1) Be in writing;

(2) Include a copy of the inspection report; and

(3) Include a copy of Appendix A: Summary of Legal Remedies.    the Guidance document pertaining to Tenants Legal Rights and Responsibilities issued by the Department.

(B) Subject to Except as otherwise specified under the emergency provisions of 105 CMR 400.200(B), any order issued under the provisions of 105 CMR 410.000 shall:

(1) shall include Include a statement of the violations, conditions, or defects identified in 105 CMR 410.630(A); and, in the case of occupied dwelling units residences, a determination whether any violation(s) or conditions, or the cumulative effect of more than one violation or condition may endanger or materially impair the health, or safety, and or well-being of an occupant; copy of all inspection reports

(2) shall contain Contain notice of the right to a hearing; of the deadline and proper procedure for requesting a hearing; the right to inspect and obtain copies of all relevant inspection or investigation reports, orders, notices, and other documentary information in the possession of the board of health; the right to be represented at the hearing; and that any affected party has a right to appear at said hearing;

(3) shall indicate Include the time limit for compliance pursuant to 105 CMR 410.830 640;

(4) Indicate the requirement for a reinspection(s) pursuant to 105 CMR 410.660(A) and (C);

(4) (5) shall include Include the following statement translated into any non-English language that is spoken as a primary language by greater than 1% of the population of that community.  "This is an important legal document. It may affect your rights. You should have it translated.”; and

(5) (6) and, shall, in In an order to an owner, advise the owner that the conditions which exist may permit the occupant of the residence dwelling to exercise one or more statutory remedies.

(C) If an inspection for all the standards in 105 CMR 410.000 reveals no violation of 105 CMR 410.000, the board of health shall forward a copy of the inspection report and a letter so stating to the owner and occupant within seven business calendar days of completion of the inspection.

410.833 680: Service of Orders and Notices

(A) All orders issued under 105 CMR 410.670 shall be served on the persons responsible for the violation.  Orders and/or notices issued under 105 CMR 410.831 shall be served on the owner or his agent and the affected occupants.

(B) All orders and/or notices shall be served:

(1) personally In person by an a person authorized to serve civil process; or

(2) by By a person authorized to serve civil process leaving a copy at his or her the person’s last and usual place of abode; or

(3) by By sending the person him or her a copy by registered or certified mail, return receipt requested if he or she is within the Commonwealth; or

(4)   If the person’s last and usual place of abode is unknown or outside the Commonwealth, by posting a copy in a conspicuous place on or about the dwelling or portion thereby affected; or

(5) By sending a copy where upon delivery service is established with an electronic copy of a signature.; or

(5) If his or her last and usual place of abode is unknown, by posting a copy in a conspicuous place on or about the residence or portion thereby affected.

(D) (C) A copy of every order or subsequent notice issued under the provisions of 105 CMR 410.830 640 or 410.650 to an owner shall also be personally delivered or sent by first class mail to the occupants of all affected premises, except that when a violation in a common area affects more than three dwelling units or rooming units residences, the notification required by 105 CMR 410.000680(C) may be satisfied by posting a copy of every order or subsequent notice in a conspicuous place in or on the building.

(D) A copy of every order or subsequent notice issued under the provisions of 105 CMR 410.831 650 to an occupant of the affected premises shall also be personally delivered or sent by first class mail to the owner of the affected premises, except that when a violation in a common area affects more than three residences, the notification required by 105 CMR 410. 000680(C)  may be satisfied by posting a copy of every order or subsequent notice in a conspicuous place in or on the building.

(E) All orders and/or notices issued under 105 CMR 410.650 shall be served on the owner or their agent.

(C) (F) A copy of every order issued under the provisions of 105 CMR 410.831 650 shall also be served upon every mortgagee and lien holder of record by sending it registered or certified mail, return receipt requested., or by sending a copy where upon delivery service is established with an electronic copy of a signature.

410.840 700: Variances

the board of health may vary the application of any provision of 105 CMR 410.000 when,

(A) The board of health shall not vary the following provisions of 105 CMR 410.000:

(1) Conditions Deemed to Endanger or Impair Health or Safety, required by 105 CMR 410.630(A);

(2) Alternative Housing, required by 105 CMR 410.710;

(3) Enforcement Procedures required by 105 CMR 410.600 through 410.930;

(4) Submetering of Water and Sewer, required by 105 CMR 410.130(C), (D), and (E);

(5) Any regulatory provision that is governed by the authority of another regulatory agency; or

(6) Any regulatory provision that is required by law.

(B) (A)  Except for those conditions enumerated under 105 CMR 410.750(A) through (O),t The board of health may vary the application of any provision of 105 CMR 410.000 with respect to any particular case, except those conditions listed in 105 CMR 410.700(A), when: in its opinion,

(1) Tthe enforcement thereof would do manifest injustice;

(2) The applicant has proved the same degree of protection required under 105 CMR 410.000 can be achieved without strict application of the particular provision(s);

(3) when insurance is utilized, the applicant has provided written confirmation from the

insurance carrier confirming the continuation of full coverage(s) if the minimum health and safety provision(s) are varied;

(24) provided that  tThe decision of the board of health shall does not conflict with the spirit of these minimum standards or any other applicable statute, code, or regulation and  provided further, such variances may be granted only after notice is given to all affected occupants and after a hearing is  held.;

(35) All affected parties have been notified in writing of the date, time, and place of a hearing to consider the variance;

(46) All affected parties have been given the opportunity to be heard to show why the variance should or should not be granted; and

(57) The petitioner is notified within three business calendar days of the board’s decision to grant the variance.

(C) Any variance granted by the board of health shall be in writing and shall include:

(1) The factors determining that the enforcement of this provision in 105 CMR 410.000 would be a manifest injustice; 
(2) The measures the petitioner will incorporate to ensure equal or greater protection of the health and safety of the occupants as provided by 105 CMR 410.000; and
(3) The date of expiration of the variance.

(D) The variance shall be:

(1) Filed by the board of health with Made available to the Director of the Community Sanitation Program at the Department by request within 14 business days of its decision to approve; 
(2) Available to the public at all reasonable hours in the office of the clerk of the city or town, or in the office of the board of health;
(3) Provided to all affected occupants by the owner and attached to the letting agreement for new tenancies created while the variance is in effect; and

(4) Filed by the owner in the registry of deeds for the county or appropriate district thereof, in which the residence is located.

(B) (E) Any variance or other modification authorized to be made by 105 CMR 410.000 may be subject to such qualification, revocation, suspension or expiration as the board of health expresses in its grant approval.

(F) A variance or other modification authorized to be made by 105 CMR 410.000 may otherwise be revoked, modified, or suspended in whole or in part, only after the owners and affected occupants have been notified in writing and have been given an opportunity to be heard, in conformity with the requirements of 105 CMR 410.80030 through 410.85055.

(G) (C)A variance from 105 CMR 410.480 270 may be granted only in writing by the Massachusetts State Building Code Commissioner when in its his or her their opinion, other security measures are in force which adequately protect the resident(s) of such residence dwelling. (M.G.L. c. 143, § 3R)

(H) A variance granted by the board of health shall be in effect only upon meeting the requirements established in 105 CMR 700(A) through (D).

(I) An owner may request that the variance be renewed by submitting to the board of health, at least 30 calendar days prior to expiration, a written request that contains a statement of any relevant changes to the conditions of the residence since the original variance request or most recent renewal. The board of health may conduct an inspection of the residence prior to issuing its decision. If the board of health finds that the health and safety of the occupants and general public will not be compromised, it may renew the variance in writing. The notification requirements of 105 CMR 410.700(D) shall be followed for each variance renewal.

(D) A variance from 105 CMR 410.150(A)(2) which prohibits a kitchen sink to substitute for the required washbasin may be granted by the board of health only if compliance would require extensive costly renovation.

(E) A variance from 105 CMR 410.503 (B), (C) and (D) may be granted by the board of health for historic buildings provided that the board of health finds that the public health will not be compromised.

410.710: Permitting Requirements For Alternative Housing

440(A) The board of health may approve alternative housing that does not meet one or more of the following requirements of 105 CMR 410.000 in single-family residences or owner-occupied dwelling units:

(1) Electrical service required by 105 CMR 410.300 and 410.320;

(2) Plumbed system for the delivery of hot and cold running water required by 105 CMR 410.130, 410.140, and 410.150;

(3) Centralized Heating system required by 105 CMR 410.160 that is capable of meeting the requirements of 105 CMR 410.180;

(4) Sanitary drainage system required by 105 CMR 410.130 provided the following approved systems are installed:

(a) A toilet capable of incinerating or composting waste; and

(b) A grey water system; or

(5) Minimum square footage required by 105 CMR 410.420(D).

(B) An owner shall apply to the board of health for a permit to occupy alternative housing. An application for alternative housing must include the following:

(1) A detailed plan of alternative energy and other provisions that will provide comparable measures to protect the health and safety of the occupants and that demonstrate that the same degree of protection required under 105 CMR 410.000 can be achieved without strict application of the particular provision;

(2) A draft deed restriction that the owner will record with the Rregistry of Ddeeds upon approval of a permit for alternative housing that indicates that the property has an alternative housing permit pursuant to 105 CMR 410.710(D) and that upon transfer of ownership or occupancy the property must comply with all requirements in 105 CMR 410.000 unless a new permit is issued by the board of health;

(3) A signed, written statement by the owner that the residence subject to the alternative housing permit will only be used as owner occupied property and that the owner understands that the permit will be nullified if the property changes occupancy or is offered or used as a rental;

(4) Documentation that any necessary variances have been obtained from other regulatory agencies when an exemption would also violate another state regulatory code; and

(5) Payment of any applicable fee set by the board of health.

(C) Upon receipt of an application for alternative housing, the board of health shall schedule a public hearing within 30 calendar days. The applicant shall provide to the board of health verification of notice to all abutters, as defined in MGL 40A s. 11,  of the date, time, and location of said hearing.

(D) Following the hearing, the board of health shall issue a written decision as to whether to grant or deny the alternative housing permit and a statement regarding the rationale for the decision.  A grant of approval shall be provisional pending a comprehensive inspection by the board of health, and may contain any conditions deemed necessary by the board of health to protect the health and safety of the occupants.

(E) Upon final approval of an alternative housing permit:

(1) The board of health shall submit a copy of the permit to the Director of the Community Sanitation Program within five business seven calendar days;

(2) The owner shall record at the Rregistry of Ddeeds for the city or town where the dwelling residence is located a deed restriction as approved by the board of health;

(3) The owner shall submit a copy of the record of the deed restriction to the board of health once available; and

(4) Said permit shall not go into effect until the deed restriction has been recorded at the Registry of Deeds.

(F) The alternative housing permit shall be valid until such time as modifications subject to the terms of the permit are made to the property or ownership of the property changes. Modifications subject to the terms of the permit require resubmission of the permit request. Failure to do so will result in the revocation of the permit. Prior to permit modification, the board of health may conduct an inspection of the residence.

(G) An alternative housing permit may be modified or revoked by the board of health for any alleged violation of the terms of the permit or any other reason necessary to protect the health and safety of the occupants only after notice to the owner and an opportunity to be heard in conformity with the requirements for an order and hearing as required by 105 CMR 410.800 through 410.850. The board of health shall submit a copy of the modification or revocation to the Director of the Community Sanitation Program within five business seven calendar days.

410.850 800: Right to Hearing

Unless otherwise specified in 105 CMR 410.000, the following persons may request a hearing before the board of health by filing a written petition:

(A) Any person or persons upon whom any order or notice has been served, pursuant to any regulation of 105 CMR 410.000 (except for an order issued after the requirements of 105 CMR 410.831 650 have been satisfied ) provided, such petition must be filed within seven calendar days after the day the order was served;

(B) Any person aggrieved by the failure of any inspector(s) or other personnel of the board of health:

(1) to To inspect upon request any premises as required under by 105 CMR 410.000; provided, such petition must be filed within 30 calendar days after such inspection was requested;

(2) to To issue a report on an inspection as required by 105 CMR 410.000; provided, such petition must be filed within 30 calendar days after the inspection;

(3) Upon an inspection Tto find after an inspection violations of 105 CMR 410.000 where when such violations are claimed to exist or to certify that a violation or combination of violations may endanger or materially impair the health or safety, and well-being of the occupants of the premises; provided, such petition must be filed within 30 calendar days after receipt of the inspection report;

(4) to To issue an order as required by 105 CMR 410.830 640; provided, that such petition must be filed within 30 calendar days after receipt of the inspection report;

(5) To enforce the provisions of 105 CMR 410.000 pursuant to M.G.L c. 111, §127A; provided such petition must be filed within 45 calendar days after receipt of the order; or

(6) To follow the provisions of 105 CMR 410.700 in the approval of a variance; provided such petition must be filed within 30 calendar days of the board of health’s grant of the variance.

410.851 810: Hearing Notice

(A) Upon receipt of a petition, the board of health shall in writing inform the petitioner and other affected parties in writing (affected parties shall include the occupants of all affected premises if the petitioner is an owner, and the owner if the petitioner is an occupant) of the date, time and place of the hearing and of their right to inspect and copy the board of health's file concerning the matter to be heard.

(B) If a written petition for a hearing is not filed with the board of health within the appropriate time provided for in 105 CMR 410.800, the right to a hearing is waived.

410.852 820: Time for Hearing

The hearing shall be commenced not no later than 30 14 calendar days after the date the order was served. from the date of receipt of the petition for a hearing. Failure to hold a hearing within the time period specified herein shall not affect the validity of any order. Provided, however, the hearing shall be commenced no later than:

(1)   14 days after an order was served pursuant to 105 CMR 410.830(A) and 410.830(B) and the petitioner refuses to begin remedial activity as required pending the outcome of the hearing; or

(2)   14 days after request for a hearing was received in instances where the petitioner alleges that an inspector or other personnel of the board of health has

(a)   failed to inspect upon request any premises; or

(b)   failed to issue an inspection report on an inspection as required by 105 CMR 410.000; or

(c)   failed to find violations of the law where such violations are claimed to exist or to certify that such violations may endanger or materially impair the health or safety, and well-being of the occupant(s); or

(d)   failed to issue an order as required by 105 CMR 410.830.

410.853 830: Hearing Procedures

At the hearing the petitioner and other affected parties shall be given an opportunity to be heard, to present witnesses or documentary evidence, and to show why an order should be modified or withdrawn, or why a dwelling residence should not be condemned, vacated, or demolished or why an action or failure to act by an inspector or other personnel of the board of health should be reconsidered, rescinded, or ordered. Failure to hold a hearing within the time period specified herein shall not affect the validity of any order.

410.854 840: Final Decision after Hearing; Failure to Comply with Final Order

(A)   The board of health shall sustain, modify, or withdraw the order and shall inform the petitioner in writing of its decision within not more than seven days after the conclusion of the hearing.  If the board of health sustains or modifies the order, it shall be carried out within the time period allotted in the original order or in the modification.

(B) If a written petition for a hearing is not filed with the board of health within the appropriate time provided for in 105 CMR 410.850,

or if after a hearing the order has been sustained in whole or part, each day's failure to comply with the order as issued or modified shall constitute an additional offense.

The board of health shall notify the petitioner and all affected parties in writing of its decision to sustain, modify, or withdraw the order within three business five calendar days after the conclusion of the hearing.

(A) If the board of health sustains or modifies the order, it shall be carried out within the time period allotted in the original order or in the modification.

(B) If after a hearing the order has been sustained in whole or part, each day's failure to comply with the order as issued or modified shall constitute an additional offense.

410.855 850: Official Hearing Record

Every notice, order, or other record prepared by the board of health in connection with the hearing shall be entered as a matter of public record in the office of the clerk of the city or town, or in the office of the board of health.

410.860: Appeal of Final Decisions

Any person aggrieved by the final decision of the board of health with respect to any order issued under the provisions of 105 CMR 410.000 may seek relief there from in any court of competent jurisdiction, as provided by the laws of this Commonwealth.

410.950 900: Condemnation, Placarding and Vacating Dwellings Residences

(A) A residence or portion thereof shall be placarded as unfit for human habitation by the board of health when:

(1) If a A written petition for a hearing is not filed in the office of the board of health within seven calendar days after an order of condemnation of any dwelling  residence or portion thereof has been issued; or

(2) if after written notice that the board of health is considering ordering a dwelling or portion thereof condemned and/or vacated and demolished, or if aAfter a hearing, the order of condemnation of a dwelling residence or portion thereof is issued. , the dwelling or portion thereof so affected by the order shall be placarded by the board of health.

(B) No dwelling residence or portion thereof which has been condemned and placarded as unfit for human habitation shall again be used for human habitation until written approval is secured from, and such placard is removed by, the board of health.

(C) No person shall deface or remove the placard, except that the board of health shall remove it whenever the defect or defects upon which the condemnation and placarding action was based have been eliminated.

(D) If any person refuses to leave a dwelling residence or portion thereof which has been ordered condemned and vacated and has been placarded in accordance with MGL 111 s. 127B and 105 CMR 410.830 640 through 410.950 930, such person may be forcibly removed by the board of health or by state or local police authorities on upon request of the board of health. (See also 150 CMR 410.830 through 410.920).

(E) If an occupant of a residence or portion thereof which has been condemned and placarded as unfit for human habitation is not the owner of the residence, the owner is responsible to provide comparable, suitable housing for the term of the lease or the remainder of rental period. Such comparable housing shall be provided until the residence is deemed suitable for habitation or until such time as the occupant finds alternative, suitable housing.

(E) The board of health may undertake to demolish any dwelling residence after an order for whose its destruction was has been properly served on the owner and every mortgagee of record in accordance with the requirements of notice and hearing in 105 CMR 410.831 650 and 410.800 through 410.860, and M.G.L. c. 111, § 127B. upon the completion of the work and the rendering of an account therefore to the owner of such structure, and shall be recoverable from such owner in an action of contract.  Said debt, together with interest thereon at the rate of 6% per annum from the date said debt becomes due, shall constitute a lien on the land upon which the structure was located if a statement of claim, signed by the board of health, setting forth the amount claimed without interest is filed, within ninety days after the debt becomes due, with the register of deeds for record or registration, as the case may be, in the county or in the district, if the county is divided into districts, where the land lies.  Such lien shall take effect upon the filing of the statement aforesaid and shall continue for two years from the first day of October next following the day of such filing.  Such lien may be dissolved by filing with the register of deeds for record or registration, as the case may be, in the county or in the district, if the county is divided into districts, where the land lies, a certificate from the collector of the city or town that the debt for which such lien attached, together with interest and costs thereon, has been paid or legally abated.  Such collector shall have the same powers and be subject to the same duties with respect to such claim as in the case of the annual taxes upon real estate; and the provisions of law relative to collection of such annual taxes, the sale or taking of land for the nonpayment thereof, and the redemption of land so sold or taken shall apply to such claim.

(F) A claim for the expense incurred by said board in demolishing a dwelling residence or any portion thereof so doing shall constitute a debt due the city or town in accordance with M.G.L. c. 111, § 127B, and M.G.L. c. 111, § 139.

410.900 910: Penalties for Interference with Inspections

Any owner, occupant, or other person who refuses, impeded, impedes, inhibits, interferes with, restricts, or obstructs entry and free access to every part of the structure, operation, or premises where inspection authorized by this code is sought after a search warrant has been obtained and presented in accordance with 105 CMR 400.100(C), shall be fined upon conviction not less than ten $10.00 nor more than $500.00.

410.910 920: Penalty for Failure to Comply with Order

Any person who shall fail to comply with any order issued pursuant to the provisions of 105 CMR 410.000 shall upon conviction be fined not less than $10.00 nor more than $500.00. Each day's failure to comply with an order shall constitute a separate violation.  See also 105 CMR 410.854(B).

.

410.920 930: Penalty for Other Offenses

Any person who shall violate any provision of 105 CMR 410.000 for which penalty is not otherwise provided in any of the General Laws or in any other provision of 105 CMR 410.000 shall upon conviction be fined not less than $10.00 nor more than $500.00.

410.960 940: Correction of Violations by Board of Health; Expenses

(A) If a failure to comply with an order requiring that any dwelling residence or its premises be properly cleaned or repaired results in a condition which endangers or materially impairs the health or well-being of the occupant or the public, the board of health may cause such proper cleaning or repair and charge the responsible person or persons as hereinbefore provided with any and all expenses incurred. Any such charges by the board of health shall not absolve the responsible person or persons from any penalty warranted by the failure to comply with the order.

(B) The board of health may also act in an emergency under the provisions of 105 CMR 400.200(B) to clean or repair any dwelling residence which so fails to comply with the provisions of 105 CMR 410.000 as to endanger or materially impair the health or safety, and well-being of the occupant or the public, and to charge the responsible person or persons with any and all expenses incurred.

410.950: Severability

If any provision of 105 CMR 410.000 shall be declared invalid for any reason whatsoever, that decision shall not affect any other portion of 105 CMR 410.000, which shall remain in full force and effect, and to this end the provisions of 105 CMR 410.000 are hereby declared severable.

REGULATORY AUTHORITY

105 CMR 410.000:  M.G.L. c. 111, §§ 3 and 127A.

Appendix A:  Summary of Legal Remedies Forms

THE FOLLOWING IS A BRIEF SUMMARY OF SOME OF THE LEGAL REMEDIES TENANTS MAY USE IN ORDER TO GET HOUSING CODE VIOLATIONS CORRECTED.

1.   Rent Withholding (General Laws Chapter 239 Section 8A).

If Code Violations Are Not Being Corrected you may be entitled to hold back your rent payment.  You can do this without being evicted if:

A. You can prove that your dwelling unit residence or common areas contain violations which are serious enough to endanger or materially impair your health or safety and that your landlord knew about the violations before you were behind in your rent.

B. You did not cause the violations and they can be repaired while you continue to live in the building.

C. You are prepared to pay any portion of the rent into court if a judge orders you to pay for it. (For this it is best to put the rent money aside in a safe place.)

2.   Repair and Deduct (General Laws Chapter 111 Section 127L).

This law sometimes allows you to use your rent money to make the repairs yourself. If your local code enforcement agency certifies that there are code violations which endanger or materially impair your health, safety or well-being and your landlord has received written notice of the violations, you may be able to use this remedy. If the owner fails to begin necessary repairs (or enter into a written contract to have them made) within five days after notice or to complete repairs within 14 days after notice you can use up to four months' rent in any year to make the repairs.

3.   Retaliatory Rent Increases or Eviction Prohibited (General Laws Chapter 186, Section 18 and Chapter 239 Section 2A).

The owner may not increase your rent or evict you in retaliation for making a complaint to your local code enforcement agency about code violations. If the owner raises your rent or tries to evict within six months after you have made the complaint he or she will have to show a good reason for the increase or eviction which is unrelated to your complaint. You may be able to sue the landlord for damages if he or she tries this.

4.   Rent Receivership (General Laws Chapter 111 Sections 127C-H).

The occupants and/or the board of health may petition the District or Superior Court to allow rent to be paid into court rather than to the owner. The court may then appoint a "receiver" who may spend as much of the rent money as is needed to correct the violation. The receiver is not subject to a spending limitation of four months' rent.

5.   Search of Warranty of Habitability.

You may be entitled to sue your landlord to have all or some of your rent returned if your dwelling unit residence does net meet minimum standards of habitability.

6.   Unfair and Deceptive Practices (General Laws Chapter 93A)

Renting an apartment with code violations is a violation of the consumer protection act and regulations for which you may sue an owner.

THE INFORMATION PRESENTED ABOVE IS ONLY A SUMMARY OF THE LAW, BEFORE YOU DECIDE TO WITHHOLD YOUR RENT OR TAKE ANY LEGAL ACTION. IT IS ADVISABLE THAT YOU CONSULT AN ATTORNEY, YOU SHOULD CONTACT HE NEAREST LEGAL SERVICES OFFICE WHICH IS:

___________________________________________________________________

(NAME)                                                       (TELEPHONE NUMBER)