Commonwealth of Massachusetts

Department of Environmental Protection

Bureau of Water Resources, Division of Watershed Management

Generic Quality Assurance Project Plan for Model Simulations

in the Total Maximum Daily Load (TMDL) Program

CN 388.0, rev. 2

RFA #10136

Prepared by

Massachusetts Department of Environmental Protection

Division of Watershed Management

Watershed Planning Program

8 New Bond St.

Worcester, MA, 01606

Prepared for

U.S. Environmental Protection Agency

New England Office

5 Post Office Square, Suite 100

Boston, MA 02109-3912

Approval Signatures and Printed Names (required prior to project start):

____________________________________________   Date:  ___________________

DWM Program Director (Kimberly Groff)

____________________________________________   Date:  ___________________

DWM TMDL Section Chief (Barbara Kickham)

____________________________________________   Date:  ___________________

DWM Data/Assessment/QA Section Chief (Richard Chase)

____________________________________________   Date:  ___________________

U.S. EPA Project Manager/Officer (Mary Garren)

____________________________________________   Date:  ___________________

U.S. EPA QA Manager (John Smaldone)

Table of Contents

Section												    									Page

- PROJECT MANAGEMENT

1.1  Title Page ………………………………………………………………………………………..	 1

1.2  Table of Contents ……………………………………………………………………………….	 2

1.3  Distribution List ………………………………………………………………………………… 3

1.4  Project Scope & Application ……………………………………………………………………	 4

1.5  Project Organization …………………………………………………………………………….	 4

1.6  Problem Definition/Background ………………………………………………………………..	 5

1.7  Project/Task Description and Schedule …………………………………………………………	 9

1.8  Quality Objectives and Criteria for Measurement Data and Models ……………………………10

1.9  Special Training Requirements ……………………………………………………………….	17

1.10  Documents and Records ………………………………………………………………………	18

- DATA GENERATION AND ACQUISITION

2.1  Data Acquisition Requirements (including Non-Direct Measurements) ……………………….	20

2.2  Data Management ………………………………………………………………………………	22

- ASSESSMENT AND OVERSIGHT

3.1  Assessments/Oversight and Response Actions ………………………………………………..	23

- MODEL APPLICATION

4.1  Model Parameterization (Calibration) …………………………………………………………	24

4.2  Model Corroboration (Validation and Simulation) …………………………………………….	26

4.3  Reconciliation with User Requirements ……………………………………………………….	28

4.4  Reports to Management ………………………………………………………………………..	29

5.0  MODELING REPORTS ……………………………………………………………………	30

6.0  REFERENCES ………………………………………………………………………………..	32

APPENDICES

Appendix A:  Example Scope of Work for Modeling (MassDEP) …….…………………………..	34

Appendix B:  Example Technical Memorandum for Model Calibration/Validation (MassDEP)….	39

Appendix C:  Example Technical Memorandum for Model Simulation (MassDEP) …...................	43

Appendix D:  Example Modeling Reports  ………………………………………………………..	47

Appendix E:  Example Tables for TMDL Modeling Reports …………………………………….	50

Appendix F:  EPA-NE Modeling Journal Checklist and Template ……………………………….	57

Acknowledgement:   This document was formulated using the EPA-Region 1 modeling QAPP template (http://www.epa.gov/region1/lab/qa/qamodeling.html) and the EPA-approved Maine DEP TMDL Program QAPP for model simulations (July, 2009) http://www.epa.gov/region1/lab/qa/pdfs/MEDEPFinalModelingQAPP.pdf.  Where necessary, the template was revised to reflect information relevant to the MassDEP TMDL Program.

1.0 PROJECT MANAGEMENT

1.1 Title and Approval Page - See page 1.

1.2 Table of Contents - See page 2.

1.3 Distribution List – The Massachusetts DEP TMDL modeling QAPP distribution list is as follows:

QAPP Recipient/Title 			Organization	 		       				Telephone Number

Vacant, QA Coordinator	 			DEP Commissioner's Office

Rebecca Weidman, Director			DEP, Bureau of Water Resources	 		617-654-6612

Division of Watershed Management

Kimberly Groff, Program Director		DWM-Watershed Planning Program (WPP)	508-767-2876

Kimberly.groff@state.ma.us

Barbara Kickham, Section Chief		DWM-WPP (TMDLs)			  		508-767-2724

Barbara.Kickham@state.ma.us

Richard Chase, Section Chief			DWM WPP (Data, Assessments, QA)		508-767-2859

Richard.f.chase@state.ma.us

TBD                        , Modeler(s)		TBD	(DWM or other)					TBD

John Smaldone, QA Manager			USEPA								617-918-8312

Smaldone.John@epa.gov

Mary Garren, Project Officer			USEPA								617-918-1322

Garren.Mary@epa.gov

If Required:

Jane Peirce, 319 Grant Coordinator		DWM Nonpoint Source Program 			508-767-2792

Jane.peirce@state.ma.us

1.4 Project Scope & Application

The purpose of this document is to present the QAPP for conducting modeling to support development of TMDLs.  The QAPP provides general descriptions of the work to be performed to support TMDLs, the procedures that will be used to ensure that the modeling results are scientifically valid and defensible, and the efforts undertaken to reduce uncertainty to a known and practical minimum. Since the majority of modeling is performed by expert contractors working for MassDEP, this QAPP is also applicable to TMDL modeling performed by contractors under DEP supervision.

A graded approach will be applied to projects in order to apply an appropriate QA level with the confidence needed in modeling results.  The fundamental requirements that define the QA level include:

• The intended use of the model – Higher standards are required for projects that

involve potentially large consequences.

• The scope and magnitude of the project – The more complex the project and model,

the more detailed the QA effort that will be necessary.

Although there are no explicit categorizations or guidelines for applying the graded approach, a generalized methodology has been identified in QA/G-5M – Guidance for QAPPs for Modeling (EPA 2002).  It allows QA activities to be adapted to meet the rigor needed for the project at hand.  If a project addresses regulatory compliance or TMDL implementation, significant QA planning is necessary.

1.5 Project Organization

The Department’s TMDL Project Leaders are Kimberly Groff (Program Director) and Barbara Kickham (TMDL Section Chief). Their responsibilities are listed in Element 1.5.1 and include notifying the Bureau QA Managers within the State and EPA Project Officer when new models will be created, justifying the inability to use existing models and if modifications to the model code will be necessary.

1.5.1 Key Individuals/Titles and Responsibilities

VACANT, DEP QA Coordinator:  Responsible for programmatic QA within DEP, including updates to the Quality Management Plan (QMP).  Has independence from all units generating data and modeling.  Oversees QA training for the Department.

Rebecca Weidman, DWM Director:  Responsible for overseeing programs within the Division of Watershed Management.

Kimberly Groff, WPP Director:  Responsible for overseeing TMDL, assessment and monitoring sections within the Watershed Planning Program.  Has the authority to issue stop work orders or require revisions to models.

Barbara Kickham, TMDL Section Chief:  Responsible for overseeing TMDL modeling efforts and has authority to issue stop work orders and/or require changes. Also oversees model development and application.

Richard Chase, Data/Assessment/QA Section Chief:  Responsible for overseeing quality control/quality assurance (QA/QC) procedures and activities, data management and 305(b) assessments within the Watershed Planning Program.  Is the primary record keeper for QAPPs and QA documentation for the program.

(varies), TMDL Project Manager(s):  Staff-level management activities, including coordination with contractors, review of draft reports, monitoring and model development as needed, etc.

Mary Garren, EPA Project Officer:  Responsible for overall project management at EPA, including reviewing the modeling report with respect to potential future TMDL development.  Also, approves QAPP and reports, and provides support.

John Smaldone, EPA QA Manager:  Responsible for overseeing EPA-Region 1 QA Office.  Assigns QAPP for review/approval.  Ensures that QA assistance and guidance is provided as necessary.

1.6 Problem Definition/Background

This document represents a generic modeling Quality Assurance Program Plan for MassDEP’s Division of Watershed Management, Watershed Planning Program, Total Maximum Daily Load (TMDL) Section.  It covers quality assurance elements for model applications related to TMDL development, and is intended to generally cover the most important factors that affect the credibility of model results.  Modification of this QAPP will be required when projects will involve new model development.

Excess pollutant loading can result in violations of water quality standards.  Historically, development of TMDLs was first mandated by the Clean Water Act in 1972 (Section 303d), and was applied primarily to point sources of water pollution. As a result of public pressure to further clean-up water bodies, lake, stream, and estuary TMDLs are now being prepared for both Point Sources (PS) and Non-Point Sources (NPS) of water pollution.

TMDL analyses are prepared to estimate the total daily maximum load (mass per time) that a water body can accept while maintaining target waterbody concentrations (40 CFR 130.2(I)).   A calculated TMDL establishes the allowable pollutant loading from all contributing sources at a level necessary to achieve the applicable water quality standards. The TMDLs must account for seasonal variability and include a margin of safety (MOS) to account for uncertainty of how pollutant loadings may impact the receiving water’s quality.  The TMDL may also be expressed in terms of other increments for the purposes of implementation.

TMDL reports (http://www.mass.gov/dep/water/resources/tmdls.htm) identify regulatory criteria for water bodies and are based on available water quality data, such as nitrogen, phosphorus, biotic indicators of eutrophication (e.g., chlorophyll-a), dissolved oxygen, etc..  The TMDL process includes a public participation component to allow for public review.  Model performance and model outcomes under this QAPP will address the available regulatory criteria.

The use of water quality models are often necessary to analyze hydrologic and water quality dynamics, in order to determine allowable pollutant loads.  MassDEP conducts TMDL modeling studies on lotic, lacustrine, and estuarine systems.  TMDL complexity can range from simple mass balance loading functions to multi-dimensional modeling of hydrodynamically complex waterbodies.

Model Specification and Selection

Model specification and selection begins with defining the problem and objectives of the modeling effort and investigating existing models in common use for their applicability or developing a new conceptual model approach. It involves an iterative, collaborative effort among decision makers to specify all aspects of the problem that will inform subsequent selection or development of a model framework. (USEPA 2009, p9)

According to the USEPA, model selection or development can be viewed as a process with three main steps: (a) specify the environmental problem (or set of issues) the model is intended to address and develop a conceptual model, (b) identify an existing model or develop the model framework (develop the mathematical model) that can achieve the desired project objectives, and (c) parameterize the model to develop the application tool. (USEPA 2009, p8)

Model development is a collaborative effort involving, model developers, intended users, and decision makers (the “project team”). The perspective and skills of each group are important to develop a model that will provide an appropriate, credible, and defensible basis for addressing the environmental issue of concern. (USEPA 2009, p8).

During problem specification, the project team must clearly define the regulatory or research objectives, determine the type and scope of the model best suited to meet those objectives (i.e. empirical vs. deterministic, static vs. dynamic, deterministic vs. stochastic),  the data criteria , the model’s domain of applicability, and identify any programmatic constraints. Problem specification and project objectives must be defined sufficiently so the project team can determine whether or not an existing model can be used to meet those needs (USEPA 2009, p9).

Model assessment and selection is usually completed at the initiation of modeling projects by MassDEP and/or its consultant in order to identify a successful approach for modeling.  As part of the review process, publicly available simulation models are evaluated in order to identify the most appropriate modeling tool for characterization of point and non-point sources.  A number of standardized modeling packages are reviewed.  These have the following advantages:

- Comprehensive documentation is distributed including a user's manual, conceptual representation of the model process, explanation of theory and numerical procedures, data needs, data input format, and description of model output.

- Technical support is typically provided in the form of training, use-support, and continual development from federal or academic research organization like EPA, USDA, and USGS.

- Standardized modeling software has a proven track record, providing validity and defensibility when faced with legal challenges.

- They are usually readily available to the general public (non-proprietary).

Selection criteria include length of model development history, applicability at the needed scale, and ability to predict the impact of land management practices on water quality and sediment. The degree of certainty needed in model outputs is defined on a project specific basis through model optimization techniques.  Certainty end-points in the form of qualitative and/or quantitative measures , when specified, are model performance goals as some amount of irreducible error is inherent in all modeling.  If other model selection tools are applied, their application will be documented in the modeling journals and reports.  Section 4.3.1 identifies a few assumptions for modeling.  Project-specific assumptions in the modeling process will be documented in the modeling journals and reports.

While most models used by MassDEP for TMDL purposes are free and available in the public domain, proprietary models may be used on a project-specific basis. Existing hydrologic and water quality models that may be utilized when developing TMDLs include, but are not limited to, the following.

- HSPF, HSPFParm (Hydrologic Simulation Program Fortran, and variations)

- QUAL2E, QUAL2EU, QUAL2K (river and stream water quality)

- Various land use export models (e.g., NPSLAKE (MassDEP); Generalized Watershed Loading Function (GWLF); ArcView Generalized Watershed Loading Function (AVGWLF)

- various mass balance models using water and nutrient/pollutant budgets (e.g., Vollenweider (1975), Dillon and Rigler (1974), Reckhow (1979))

- SWMM (Stormwater Management Model)

- CE-QUAL-W2/RIV1 (surface water quality)

- BASINS (Better Assessment Science Integrating point and Nonpoint Sources)

- SPARROW (SPAtially Referenced Regression On Watershed attributes)

- WAM (Watershed Assessment Model)

- Aquatox (pollutant fate and ecology)

- CORMIX (mixing zones)

- WQMAP™ (surface water quality)

- EPD-Riv1 (Riverine Hydrodynamic and Water Quality Model)

- EFDC (Environmental Fluid Dynamics Code)

- WASP (Water quality Analysis Simulation Program)

- LSPC (Loading Simulation Program in C++)

- STRMDEPL (streamflow depletion)

- MODFLOW (groundwater)

- RMA-2 (hydrodynamic/flushing of complex river and estuarine systems)

- RMA-4 (water quality of complex river and estuarine systems)

1.7 Project/Task Description and Schedule

Modeling will be conducted where necessary to support TMDL development.  TMDLs are important tools for maintaining and protecting acceptable water quality.  They are primarily designed to 'get a handle' on the magnitude of the pollution problem and to develop plans for implementing Best Management Practices (BMPs) and/or capital improvements to address the problem. Models should include the ability to quantify future spatial and temporal distribution of pollutants in the watersheds, flexibility to evaluate historical and relative contributions of various pollutant sources in the watersheds, and adequate resolution to identify the relative in-stream impacts of pollutant loading to the stream system from various urban and non-urban point and non-point sources.

To address the need for TMDL modeling data, targeted surface water monitoring is conducted by MassDEP-DWM on an annual basis or is contracted to qualified firms.  Sampling generally consists of physico-chemical variables for nutrients (N, P) and E. coli bacteria, and may also include other inorganic analytes, biological indicators, dissolved metals and streamflow estimates.  Most 303(d) listed TMDL water bodies in Massachusetts are monitored during the spring, summer and fall seasons.  Monitoring conducted directly by MassDEP is performed consistent with DWM’s five year programmatic QAPP (MassDEP 2015) and annual Sampling and Analysis Plans (SAPs) are produced as QAPP addendums. Monitoring conducted under contract to MassDEP is conducted in accordance with a detailed monitoring QAPP in support of project goals and needs.  The resulting data are used to support TMDL modeling.  Additional data sources are identified in Section 2.0.

Schedules for modeling work are project specific and will be shared with QAPP signatories for review and comment.  In general, modeling work can take one or more years to complete depending upon its complexity and available resources for the project.  However, with many regulating agencies involved, there may be additional technical evaluations requested that may require additional time that may impact the schedule.  Regulatory agencies may also require more time for review of model results and to reach consensus at key decision points.  More specific resource or time constraints cannot be foreseen at this time but, if significant, will be communicated from the MassDEP Project Manager to the EPA Project Officer.

An example Scope of Work for contract modeling pursuant to TMDL development is provided in Appendix A.

1.8 Quality Objectives and Criteria for Measurement Data and Models

Quality objectives and criteria for model inputs and outputs can be qualitative and/or quantitative statements that (1) clarify study objectives, (2) define the appropriate type and acceptance criteria of existing data, (3) establish acceptable model input and parameterization (calibration) criteria, (4) outline model performance evaluation obligations, and (5) specify tolerable levels of potential model accuracy, sensitivity, and uncertainty.  Each is discussed in the following sections.

Assessing whether the DQOs have been achieved for a modeling study is less straightforward than for a typical sampling and analysis program.  The usual data quality indicators (e.g., completeness, representativeness, comparability) are difficult to apply and in many cases do not adequately characterize model output.  The ultimate quality test for the model is whether the output sufficiently represents the natural system that is being simulated.  To a large extent, this is determined by the expertise of the modelers and the amount of available data.  Nonetheless, there are objective techniques that can be used to evaluate the quality of the model performance and output.  Where applicable, these techniques shall be applied by MassDEP for TMDL modeling projects and the results presented in a modeling report or technical memorandum. Justification shall be provided where these techniques are not applied.  The methods, and the proposed performance expectations, are discussed in Section 1.8.2 below.

1.8.1 Objectives and Project Decisions

This QAPP has been completed by MassDEP to ensure that (1) modeling input data are valid and defensible, (2) model setup and parameterization (calibration) protocols are followed and documented, (3) model applications and output data are reviewed and evaluated in a consistent manner and 4) that models are able to predict hydrologic or water quality conditions over time in support of TMDL development.

An example overarching purpose may be if modeling indicates that water quality standards are attainable, then discharge permits may be modified (or other pollution prevention measures taken) to improve water quality.  To this end, modelers will work with program managers to align model outputs with the types of decisions to be made.

1.8.2 New Data Measurement Performance Criteria/Existing Data Acceptance Criteria

The use of existing data of known quality in modeling efforts is extremely important and helps ensure that the modeling yields accurate predictions with an acceptable level of model uncertainty.  All model input or parameterization (calibration) data sources will be collected in accordance with a QAPP (and associated SOPs) prior to its use in the modeling effort.   Data with unknown quality (i.e. collected without a documented QAPP or using SOPs not approved by EPA or MassDEP) will be flagged and noted as either conditionally acceptable for limited use or not acceptable for use at all.  See also Section 2.1 for additional procedures for excluding data.

New and Existing Monitoring Data

Data Quality Objectives (DQOs) for model input and calibration are specified in individual QAPPs for each data source.  For DWM-generated data, for example, ambient field blank and duplicate water samples are typically obtained for approximately one out of every 10th water body sampled.  Blanks are expected to be analyte-free (<method detection limit), and duplicate results are expected to be generally within 20% of each other.  Laboratories are expected to provide their own internal approach to quality control in the SOPs for each test and in their comprehensive Quality Assurance Plans (QAPs).

When DQOs are not met, and best professional judgment indicates sampling error,

procedures are reviewed to determine which steps are critical for establishing consistency.

Further detail may be added or modifications made to the SOP.  When quality objectives are not met, and best professional judgment indicates analytical error, the lab will be contacted and some resolution to the problem will be sought.  As a result of DWM’s data validation process for DWM-generated data and its external data review procedures, resulting data may be accepted, accepted with qualification or censored.  For additional information on DWM’s data validation process and it’s procedures for review of non-direct (external) data, see the DWM programmatic monitoring QAPP (MassDEP 2010).

MassDEP has established general Data Quality Objectives (DQOs) for modeling projects in order to specify the acceptance criteria for model input, and parameterization (calibration) or corroboration (validation) data.  DQO’s identify the (1) type and quality of data that will be appropriate for use in the modeling project, (2) spatial and temporal input data coverage requirements, (3) data quality and currency, and (4) technical soundness of the collection methodology.

Generalized DQOs for MassDEP modeling efforts include:

- All input and parameterization (calibration) data for the model will be of a known and documented quality.

- Data will be collected from as many sources as available, and provide the maximum temporal and spatial coverage of the watershed drainage, if necessary and applicable.

- The data will be comparable with respect to previous and if possible future studies.

- Modeling data will be representative of the parameters being measured with respect to time, location, and the conditions from which the data are obtained.

DQO’s were further refined in order to define performance criteria that limit the probability of making decision-based errors.  They address the data validity and reliability of the modeling effort and each is briefly described below in the context of completeness, representativeness, and comparability.  The traditional context of precision and accuracy is not included due to the fact that, in most cases, the data has already been collected and analyzed through acceptable analytical procedures.

Completeness is a measure of the amount of valid input data obtained during a process.  The target completeness for models will be 100 percent – e.g. all available sources included.  The actual completeness may vary depending on the intrinsic availability of monitoring data. Deficiencies in water quality, climatic, or stream flow data are outside of the control of the modeling effort and will be addressed as part of the data compilation and assessment effort.  In order to provide surrogate data, the most current statistical or stochastic methods will be used to extend or fill in missing time-series data.  The normal-ratio will be used to fill precipitation gaps. Discharges will be linearly interpolated or estimated using other fitting methods such as regression analysis.  In some cases, additional data will be gathered to reach completeness objectives.  MassDEP will address any data completeness issues as they develop.

Representativeness is a measure of how closely the input or parameterization (calibration) data will reflect the physical characteristics of hydrology and water quality over time.  Standardized monitoring plan design and the use of Standard Operating Procedures (SOPs) for discharge measurement, soils identification, land cover mapping, sample collection and handling, and acquisition of weather data are crucial to ensuring representative data quality.  All model input or parameterization data sources deemed to be usable shall have had a QAPP in place prior to sample collection or be of known, documented and acceptable quality.

Comparability expresses the confidence with which one data set can be compared to another. Data comparability from external sources is very much tied to the individual project methodology and time at which it was collected.  For the purpose of the modeling effort, comparability will be maintained by using consistent units, appropriate temporal scales, and reproducible methods. Unit conversions (metric may be the required default), datum transformations, and grid re-projections will likely be required to make data for the modeling comparable.  Information that exists outside a reasonable temporal scale, has significantly changed or will potentially diminish the modeling results are not comparable.  MassDEP will make these determinations using best professional judgement, as necessary.  Comparability between other model indicators will be evaluated on a case-by-case basis.

Acceptance Criteria for Model Parameterization (calibration)

Some models are “calibrated” to a set of specific parameters. Calibration is defined as the process of adjusting model parameters within defensible ranges until the resulting predictions give the best possible fit to the observed data. The acceptance criteria for model parameterization (calibration) define the procedures whereby the difference between the predicted and observed values of the model are within an acceptable range, or are optimized. This can occur either qualitatively or quantitatively and documented accordingly (USEPA 2009, p65). Often parameterization is the only method to ensure that model predictions correlate with values observed in the field or within ranges documented in scientific studies.  Parameterization uses observed hydrometeorological data in a systematic search for parameters that yield an acceptable fit of computed results.  This search is performed to find a reasonable best estimate that will yield the minimum value of an objective function, or variable that is critical in application.

Parameterization has become increasingly important with the need for valid and defensible models for TMDL development.  Each time a model is calibrated, it is potentially altered. Therefore, it is important that all calibrations be properly documented including the approaches taken (e.g. qualitative vs quantitative) along with the acceptance criteria.

Acceptance criteria for the modeling projects should be established in order to provide a numerical ruler for determining whether the model is an appropriate tool for TMDL decision-making.  As an example, the model parameterization criteria may be based on  recommended error percentages for seasonal, annual, and storm-based water yields (Table 1).  Generalized information related to model parameterization criteria, and corroboration considerations, include the following references: Thomann, 1982; James and Burges, 1982; ASTM, 1984.

Table 1. Example of Acceptable Model Parameterization/Calibration Hydrology Criteria (example)

Errors (Simulated-Observed) 			Recommended Criteria

Error in Total Volume 					10%

Error in 50% Lowest Flows 				10%

Error in 10% Highest Flows				15%

Seasonal Volume Error – Summer			30%

Seasonal Volume Error – Fall 				30%

Seasonal Volume Error – Winter 			30%

Seasonal Volume Error – Spring 			30%

Error in Winter Storm Volumes 			20%

Error in Summer Storm Volumes 			50%

Graphical comparisons of model performance may also be used including time series plots of observed and simulated flows and state variables, and residual scatter plots (observed versus simulated values).  Time series plots are generally evaluated visually for agreement, or lack thereof, between the simulated and observed values.  When observed data are adequate, or uncertainty estimates are available, confidence intervals can then be calculated so they can be considered in the model performance evaluation.

A number of statistical tests are also available for watershed model evaluation and optimization. The Sum of the Squared Residuals and the Nash & Sutcliffe Coefficient of Efficiency are two that have been identified for the purpose of modeling projects. Each is described below.

Sum of Squared Residuals is a commonly used objective function for hydrologic model

parameterization (calibration).  It compares the difference between the modeled and observed ordinates, and uses the squared differences as the measure of fit.  Thus a difference of 10 feet3/second between the predicted and observed values is one hundred times worse than a difference of 1 feet3/second.  Squaring the differences also treats both overestimates and underestimates by the model as undesirable.  The function implicitly is a measure of the comparison of the magnitudes of the peaks, volumes, and times of peak of the hydrographs and water quality constituents.  The equation for calculation of the sum of least squares is shown below (Diskin and Simon, 1977).

Nash and Sutcliffe Efficiency is a goodness-of-fit test can be used as another statistical method for evaluating the hydrologic variability between measured and predicted model values.  The Nash and Sutcliffe Coefficient of Efficiency (COE) provides a normalized estimate of the relationship between the observed and predicted model values and is calculated as below (Nash and Sutcliffe 1970).

A COE value of one indicates a perfect fit between measured and predicted values for all events. COE values between zero and one suggest a positive relationship between observed and predicted values, thus allowing for the use of predicted values in lieu of observed data.  A zero value indicates that the fit is as good as using the average value of all the measured data.  See also Section 4.1 for additional parameterization considerations and stop criteria.

No matter what method is finally chosen the calibration results should be properly documented in a project modeling document, technical memorandum or other appropriate vehicle which describes how the calibration was conducted and tested for acceptance.

Model Corroboration (Validation)

Corroboration (validation) is defined as the comparison of modeled results with independently derived numerical observations from the simulated environment.  Model corroboration is an extension of the parameterization (calibration) process.  Its purpose is to assure that the calibrated model properly assesses the range of variables and conditions that are expected within the simulation.  Although there are several approaches to validating a model, perhaps the most effective procedure is to use only a portion of the available record of observed values for parameterization.  The rest is used for corroboration.  Once final parameterization parameters are developed, simulation is performed for the remaining period of observed values and the goodness-of- fit between recorded and simulated values is reassessed.  Wherever possible, this type of split-sample parameterization and corroboration procedure will be used for modeling projects.

The credibility of the model hinges on the deterministic ability to predict conditions over the entire range of observed data: in effect, validating the model. As with model calibration, model corroboration can be conducted either qualitatively or quantitatively. Qualitative corroboration involves expert judgment and test of intuitive behavior using “knowledge” of the behavior of the system in question but it is not formalized or statistically-based. Expert knowledge can establish model reliability through consensus and consistency. For example, an expert panel consisting of model developers could be convened to determine whether there is agreement that the methods and outputs of a model are consistent with processes, standards, and results of other models. Expert judgment can also establish model credibility by determining if model-predicted behavior of a system agrees with best-available understanding of internal processes and functions (USEPA 2009, p65).

Model corroboration can also be quantitative in nature. This would involve comparing model predictions to independent empirical observations to investigate how well a modeler’s description of the system fits the observational data. This procedure involves using statistical measures of goodness of fit and numerical procedures to facilitate calculations. This can be accomplished graphically or by calculating various statistical measures of fit between the modeler’s results and observed data. (USEPA 2009, p65)

Graphical methods can be used to compare the distribution of the model outputs to independent observations. The degree to which these two distributions overlap, and their respective shapes, provide an indication of model performance with respect to the data. (USEPA 2009, p65)

Quantitative measures include methods for calculating model bias or model bias and precision. For model bias a calculation can be made of mean error. The mean error calculates the average deviation between models and data by dividing the sum of errors by the total number of data points compared. For model bias and precision a calculation of Mean Square Error or Mean Absolute Error can be made (USEPA 2009, p65-66)

Regardless of the method chosen to evaluate corroboration the process used and the findings of the analysis must be documented in the final project modeling document, technical memorandum or other appropriate vehicle which describes how the corroboration was conducted and the outcome s of that effort .

Model Sensitivity

Sensitivity analysis determines the effect of a change in a model input parameter or variable on the model outcome.  The sensitivity of a model parameter is typically expressed as a normalized sensitivity coefficient (Brown and Barnwell, 1987).  One methodology for identifying the sensitivity of a model parameter is shown below.

MassDEP will qualitatively assess the sensitivity of model parameters during manual

parameterization (calibration) through parameter perturbation.  A summary of model sensitivity will be included in the final modeling report or technical memorandum.  Details will include the variables modified for model parameterization (calibration), the percent modification (e.g. ± 10%), percent change in the modeling results, and the normalized sensitivity coefficient (Table 2). An example format is shown below.

Table 2. Sensitivity Coefficients (Example)

Model Parameter					% Perturbation		% Change 		NSC

Curve Number 					-15% 				-29%	 		2.2

Soil Available Water Capacity 		20%				25% 			1.3

Channel Erodibility 				05% 				01% 			0.1

Alternative algorithmic techniques for sensitivity and uncertainty assessment are available through several water quality modeling programs (Monte Carlo Simulation, first-order error analysis, or automated objective function optimization).

Model Uncertainty

Uncertainty is broadly defined as the lack of knowledge regarding model input parameters and the processes the model attempts to describe.  Our ability to define model uncertainty is marginalized by our limited ability to accurately describe complex processes.  As a result, all engineering computations are attended to a degree of uncertainty due to the simplification of natural process and the limitations of input and parameterization (calibration) data.  Computed values differ from observed ones, and the magnitude and frequency of these differences characterize the uncertainty of the best model estimate.  Uncertainty analysis is the terminology associated with the examination of how the lack of knowledge in model parameters, variables, and processes propagates through the model structure as model output or forecast error.  Sources of model uncertainty are characterized by MassDEP during the initial stages of planning in order to better understand how the model input data and parameters would potentially influence model output and prediction.  Potential sources of model uncertainty include:

- Estimated model parameter values

- Observed model input data

- Model structure and forcing functions

- Numerical solution algorithms

MassDEP and its agents will be responsible for conducting documenting any areas of potentially significant uncertainty in the model documentation or via technical memorandum.

1.9 Special Training Requirements

The Commonwealth of Massachusetts job classification system has established minimum qualifications required for all levels of State employment.  The job classifications for individuals employed in the TMDL Section are primarily “Environmental Engineer” and “Environmental Analyst”.    Staff qualifications typically range from a Bachelor’s degree to a Ph.D. in one of the natural sciences or environmental engineering.  In addition, individuals typically have a number of years of field experience collecting environmental data.  Because the Massachusetts hiring process establishes training and experience levels required to be employed by these Sections, there is no need to include resumes for each individual in the Section.

The only requirement related to modeling QA is that project personnel are expected to read and observe this QAPP for all TMDL modeling projects.  Personnel responsible for implementation will be required to review this QAPP, and as new modeling staff or managers are hired by MassDEP, they will also be required to review this QAPP.

For the TMDL program, WPP’s Program Director, TMDL Section Chief and TMDL Project Manager shall ensure that any necessary TMDL/modeling training has been received.  The location for training records is in WPP offices for paper records (QA and/or TMDL section) and on the networked TMDL folder for electronic records.

In addition to modeling activities by MassDEP staff, work may also be performed by outside contractors hired by MassDEP.   Contract staff qualifications shall be documented as part of the hiring package.   In general, there is no special training needed for contract modeling personnel, since the firms and individuals hired are expected to have the required expertise to perform the work.  Contracted  TMDL project personnel are expected to read and observe this QAPP for all TMDL modeling projects.

For training requirements related to sampling and data collection, see DWM’s five-year programmatic QAPP (MassDEP 2015).

1.10 Documents and Records

Adequate documentation for modeling projects helps ensure transparency, informs reviewers and decision-makers, provides historical records and enhances the defensibility of model outcomes (USEPA 2009).

1.10.1  QAPP Distribution

This QAPP will be implemented as needed by MassDEP once USEPA has given approval.  This QAPP is to be considered a “working document”.  This QAPP will be periodically updated and revised, in accordance with Section 1.10 as technology, policy and protocol change.  As required by EPA-NE, an updated QAPP will be formally re-submitted for approval every five years as appropriate. All QAPP updates will be distributed by the MassDEP Project Manager according to the distribution list in Section 1.3 and with notification to EPA from the WPP.  Upon approval and implementation of this QAPP, the final copy shall be kept on the MassDEP network drive.  A copy will also be placed on DEP’s TMDL web site (http://www.mass.gov/dep/water/resources/tmdls.htm).

1.10.2  Modifications to the Modeling QAPP

This section addresses procedures to be followed when modifications are needed to a) this Program QAPP, including associated SOPs, or b) any SAP accepted under this Program QAPP that requires real-time modification to achieve project goals.  Examples of such modifications include changes in procedures, assessment and reporting. The formal DWM Control Number (CN) for this document (CN # 388.0, rev. 2) shall be modified if substantial revisions are made.

Discussions involving changes to the Program QAPP may be initiated at any level.  Contact should be made with the WPP Program Director to discern whether modification is warranted.  The scope of effect of the proposed change will determine the formality of the approval process.  A formal QAPP modification will include reference to the section(s) of text being modified or added to, the reason why the modification is necessary and the actual replacement/additional language.  It will be the responsibility of the WPP Program Director to seek review and approval from others within the agency and from EPA.  Signatories of the original Program QAPP will receive such updates for approval.  SOP modifications, additions and retirements follow the same procedure as modifications to the QAPP. Additionally, SOPs must be organized and formatted according to MassDEP-DWM procedures.  SOPs under development should be included as part of the QAPP as soon as practicable.

1.10.3  Modeling Journal, Report, or Technical Memorandum

A modeling journal, report, technical memorandum or other form of approved documentation will be kept by the project manager or his/her designee to identify the internal model parameters that were adjusted during the parameterization process to meet the criteria identified in Section 1.8.  This document is an important tool to ensure transparency in the modeling process.  It is intended to document all parameterization iterations made during the project along with the justification and professional reasoning behind the changes.  For example, each time that a separate model parameterization run is completed, changes should be documented. The level of detail in the modeling document (s) should be sufficient to allow another modeler to duplicate the parameterization method given the same data and model.  The modeling documentation should include complete recordkeeping of each step of the modeling process. In general, the documentation should include all of the items identified in Section 5 of this QAPP.

The modeling journal, report, or technical memorandum, all data files, source codes, and executable versions of the computer software used in modeling studies will be retained for ten (10) years for auditing or post-project reuse, if necessary.  These files will include:

• Version and source of the executable code used.

• Parameterization (calibration) input and output data.

• Corroboration (validation) input and output data.

• Model application input and output (i.e., for each scenario studied).

Documentation of any response action taken to correct model implementation is also described in Section 3.0.  Technical Memoranda may also be used as in the example in Appendix B/C, or EPA-NE’s Modeling Journal Template and Checklist (Appendix F) may be used as a guide for journal contents and/or format.

1.10.4  Project Reports

In addition to the final modeling journal or report, project documentation should also include the following (as applicable):

- Scope(s) of Work (for contract modeling)

- Progress Reports

- QAPP, SAP and/or SOPs (for additional data collection activity)

- Technical Memoranda

- QA/QC Reports

- Misc. Correspondence

- Modeling Report (draft and final; see also Section 5.0)

The timing and frequency of Progress Reports and required QA/QC reports shall be determined by the Project Manager for each modeling project depending on the complexity and expected duration of the project.

2.0 DATA GENERATION AND ACQUISITION

2.1 Data Acquisition Requirements (including Non-Direct Measurements)

Each project manager under this QAPP will be responsible for summarizing how well the data quality objectives (DQOs) were met.  The reports to management (Section 4.4) and modeling reports (Section 5.0) and will document this activity.

Potential Model Input Data

Water bodies listed on the 303(d) list (designated for TMDLs) have a historical dataset that has been evaluated using assessment criteria.  These data may have originated from MassDEP or from another data source (e.g., State or Federal agency, consultant report, etc.).  When needed and feasible, additional primary data may be collected by DWM or its agents to augment existing data sources, following the QA provisions outlined in Element 1.8 (e.g., QAPP, SAP, SOPs, etc.).

MassDEP Data

Data collected by MassDEP-DWM undergoes an extensive review and validation process to determine usability for waterbody assessment and/or TMDL determinations (including modeling).  This process is documented in Sections D1 and D2 of the five-year DWM programmatic QAPP (MassDEP 2010).

Data Originating from Others

A significant amount of watershed input data may be necessary for setup and parameterization (calibration) of models.  Usable data may be available from a variety of sources.  Web links to some of these data source providers are shown below.  Quality information can be viewed in subsequent links, along with information regarding development and disclaimers on use.  Much of the data originates from published agencies like USGS, EPA, USDA, NCDC and other MA. groups.  The rigor in which these organizations implement QA/QC fully meet the quality objectives identified in this QAPP.

- EPA STORET http://www.epa.gov/storet/dbtop.html

- National Climatic Data Center (NCDC) http://www.ncdc.noaa.gov/oa/ncdc.html

- National Elevation Dataset (NED) http://ned.usgs.gov/

- National Hydrography Dataset (NHD) http://nhd.usgs.gov/

- National Land Cover Dataset (NLCD) http://landcover.usgs.gov/index.asp

- National Water Information System (NWIS) http://waterdata.usgs.gov/nwis/

- Remote Automated Weather Stations (RAWS) http://www.fs.fed.us/raws/

- Snow Telemetry (SNOTEL) http://www.wcc.nrcs.usda.gov/snotel/

- STATSGO Soils http://www.soils.usda.gov/survey/geography/statsgo/

- Integrated Compliance Information System (ICIS) https://icis.epa.gov/icis/jsp/common/LoginBody.jsp

- Census data http://www.umass.edu/miser/

- MassGIS  http://www.mass.gov/mgis/

MassDEP TMDL staff  (and contractors) use the latest geographic data derived from USGS maps and MassGIS coverages.  Flow and water quality parameterization (calibration) data are also subject to the DQOs identified in Sections 1.8.   Sources of effluent, discharge, and in-stream water quality data will have an approved QAPP from MassDEP and/or the U.S. EPA or will be collected and analyzed in accordance with EPA approved techniques as part of required monitoring under existing permits (e.g., NPDES).  Data with unknown quality (i.e. collected without a documented QAPP or using unapproved SOPs) will be flagged and noted as either conditionally acceptable for limited use or not acceptable for use at all.  In addition to defined data review protocols for secondary data (further defined in DWM’s programmatic monitoring QAPP, Section B9.3), EPA’s secondary data guidance (EPA 2009b) and best professional judgment are applied to accept or exclude data from the dataset.  To ensure consistency in excluding data for use in TMDL models, justification for excluded data will be provided in the final modeling documentation.

The final responsibility rests with the individual using data and associated geographic information to become aware of the limitations inherent in any information source.  These data sources may be used within the section for decisions regarding trend evaluation, use attainment, and as historical background for TMDL projects.

2.2 Data Management

MassDEP Data for Model Input

Field data are entered on standard fieldsheet forms.  Several standard forms exist and are updated annually.  Both field and lab data are entered into databases manually and electronically (e.g., batch uploads).  All of the data are proofed for transcription errors by MassDEP staff.  Proofed sheets are initialed and dated by the proof reader. Extensive quality control review follows DWM data validation procedures, as described in DWM’s programmatic QAPP.  Both objective criteria and best professional judgment are used to validate the data and to exclude data from the modeling dataset.  Data are managed using the DWM’s internal data management systems.  Write access to these files is restricted. These data are stored and backed-up on the MassDEP computer network file server and are additionally backed up periodically onto CD.  Backup project files on CD are stored both on-site and off-site. Additional information regarding DWM data management, including fieldsheet forms, can be found in the program QAPP:  http://www.mass.gov/eea/agencies/massdep/water/watersheds/environmental-monitoring-quality-management-program.html

Model Application Data

Standard parameterization (calibration) and data management procedures will be implemented during modeling projects to ensure that modeling results are valid, reproducible, and comparable. The standard procedures include the following: (1) use of modeling techniques accepted within the professional industry, (2) parameterization (calibration) methods that can be performed repeatedly by a qualified person to obtain similar results, (3) documentation that is clear, concise, and thorough, and (4) the use of standard units for data management.

Data used during modeling projects will be maintained in hard copy and/or electronic format at DWM’s offices (TMDL program files).  Hard copy records will be kept for a minimum duration of 10 years.  Electronic data records will be kept indefinitely.  Duplicate set(s) of model data will be maintained (e.g., at DWM and at project engineer’s offices).

Database entry and manipulation within a model is identified as one of the major preventable error sources in the modeling effort.  Unlike the limitations of the model and model driver data itself, user induced error is correctable under an appropriate level of QA/QC.  Multiple steps will be taken to ensure errors are minimized.  Data formatting will be reviewed prior to the final version of the database being generated, including the data element type, format, allowable values and ranges, and other parameters.  All data used to populate the modeling database will be screened before upload to the model application.   Manually entered parameter values from paper sources will be evaluated by reviewing printouts of summaries and randomly selecting portions of the model application.  The review will include a comparison of the original data sources and paper documentation.  Any record identified as having problems will be reviewed to determine whether corrected data can be acquired or the record omitted in accordance with Section 2.1.  The model input files will be checked by MassDEP or its consultant for reasonability and correctness in order to detect errors that may occur during the data management or transfer process.

3.0 ASSESSMENT AND OVERSIGHT

3.1 Assessments/Oversight and Response Actions

The data generated as part of the modeling results will be evaluated during the corroboration (validation) process.  Model performance assessments will be made by MassDEP as described in the parameterization and corroboration processes in Sections 4.1 and 4.2.  Performance audits will consist of review of technical memoranda, comparison of model results with observed historical data, and general evaluation of model behavior for state variables and other output lacking historical data.  Auditors may include a MassDEP QA Scientist who is knowledgeable in the subject matter, the TMDL Section Chief, or the WPP Program Director.  All auditors are authorized to “stop work” for good cause, if necessary to initiate corrective action(s).

At the end of the parameterization and corroboration period, MassDEP will assess the ability of the model to predict hydrologic and water quality response over time.  Section 1.8 identifies criteria that will be part of the model performance assessment and also assessed are:

• Modeling input and output validity.

• Model parameterization and corroboration performance determinations.

• Sensitivity and uncertainty analysis assessments.

Parameter deviation and post-simulation corroboration of predictions are major issues in the quality assurance framework.  MassDEP or its consultant will document the model data entry, parameter estimation, and parameterization activities, and will provide this documentation as part of the modeling journal, model report and the project file and any technical memoranda.  A typical MassDEP internal assessment is described below.

Modeling data and project deliverables will be internally quality controlled by MassDEP in-house review.  Anticipated MassDEP review staff members responsible for this process include the Project Manager and TMDL program staff.  The Project Manager will maintain overall responsibility for examining the work to ensure that methodologies and processes are consistent with the procedures outlined in the QAPP.  He or she will provide advice to project staff and QA Manager regarding any deviations from the QAPP, so that appropriate actions may be taken either to correct the problem or to amend the QAPP as needed.  The QA Manager (or Project Manager) will monitor the extent to which this QAPP is being implemented and is supporting its intended use.  For TMDLs involving modeling, the draft and final TMDL reports will include references to model documentation, including the modeling report.

If the quality control audit results in detection of unacceptable conditions or data, the Project Manager will be responsible for developing and initiating corrective action.  Corrective response actions may include:

- Review or corroboration of modeling input and parameterization data.

- Re-definition of model extents or spatial distribution.

- Performing additional model runs.

- Editing and modifying report deliverables.

4.0 MODEL APPLICATION

4.1 Model Parameterization (Calibration)

All models, by definition, are a simplification of the environmental processes they intend to represent.  The optimization of empirical parameters that form the numerical basis of the model is referred to as parameterization (calibration).  Parameterization iteratively adjusts model coefficients or parameters until predicted values accurately reproduce those measured in the field. Some models have internal parameterization tools that aid the user in managing parameterization scenarios and refining model runs until acceptable parameterization criteria are met.  Once an acceptable parameterization is reached, the run can then be verified on an independent data set to judge the extent to which the model is able to predict hydrologic or water quality conditions over time.

A complete watershed model parameterization involves a successive examination of the characteristics of the watershed hydrology and water quality, including if applicable  (1) annual and seasonal water balance and streamflow, (2) sediment , (3) nutrients and (4) biotic indicators.  Simulated and observed values for reach characteristic are examined and critical parameters are adjusted to attain acceptable levels of agreement.  The refinement of parameters should reflect the scientific literature and not exceed reasonability.  The rationale for any model adjustments should be based on the parameterization procedures outlined in the QAPP and documented in the modeling journal and report.

Parameterization Considerations

Parameterization should consider the most important hydrologic and water quality response variables.  The sensitivity of these parameters has a significant influence on the uncertainty of the model and should be equally considered during the parameterization process.  Ideally, both high and low flow years, and the anticipated range of conditions and scenarios for which the TMDL will be developed will be used.  Factors for defaulting to non site-specific data include the following:

- Practical or technical limitations to site-specific data collection

- Availability of sufficient quality-controlled data from other sources

- Censoring of site-specific data during the data validation process

Parameterization should be completed in sequential order, using the most upstream point first and then moving downstream to the next point of parameterization (calibration).  It is important that parameters of files associated with the drainage area upstream of a calibrated point, are not changed during subsequent steps.

Parameterization Stop Criteria

Stop criteria can be useful to prohibit never-ending parameterization and allow the introduction of more subjective criteria, if necessary.  This can stop the “ever decreasing circles” that some optimization methods tend to follow in search of an exact minimum in the objective function (European Commission 2005).  In consultation with the Project Manager, a prioritized list of stop criteria includes the following:

- available resources

- a specified end date

- a specified maximum number of model runs

- a specified change in size of a function value (objective function, parameter values, etc.) towards an apparent minimum.

Any deficiencies during the parameterization process are worked through the Project Manager and documented in the modeling journal and report.  See also Section 4.4.

Following parameterization, verification on an independent data set is necessary to evaluate the effectiveness of the model to represent physical processes beyond those for which the model was calibrated.  Decisions made during model parameterization and verification will be documented in the modeling journal, report, or technical memoranda so that an experienced user could complete the parameterization process and obtain similar modeling results.  See also Section 1.8 Quality Objectives and Criteria.

4.2 Model Corroboration (Validation and Simulation)

Criteria used to review/validate data are listed in Sections 1.8 and here.  Best professional judgment will also be used in conjunction with these criteria.

Once an acceptable parameterization is reached, the model corroboration (validation) is run on an independent data set to judge the extent to which the model is able to predict hydrologic or water quality conditions over time.

The quality of the model fit is examined by: either qualitative or quantitative measures. Qualitative corroboration involves expert judgment and test of intuitive behavior using “knowledge” of the behavior of the system in question but it is not formalized or statistically-based. Expert knowledge can establish model reliability through consensus and consistency. For example, an expert panel consisting of model developers could be convened to determine whether there is agreement that the methods and outputs of a model are consistent with processes, standards, and results of other models. Expert judgment can also establish model credibility by determining if model-predicted behavior of a system agrees with best-available understanding of internal processes and functions (USEPA 2009, p65).

Model corroboration can also be quantitative in nature. This would involve comparing model predictions to independent empirical data (not used in model calibration) to investigate how well a modeler’s description of the system fits the observational data. This procedure involves using statistical measures of goodness of fit and numerical procedures to facilitate calculations. This can be accomplished graphically or by calculating various statistical measures of fit between the modeler’s results and observed data (USEPA 2009, p65).

Graphical methods can be used to compare the distribution of the model outputs to independent observations. The degree to which these two distributions overlap, and their respective shapes, provide an indication of model performance with respect to the data.

Quantitative measures include methods for calculating model bias or model bias and precision. For model bias a calculation can be made of mean error. The mean error calculates the average deviation between models and data by dividing the sum of errors by the total number of data points compared. For model bias and precision a calculation of Mean Square Error or Mean Absolute Error can be made. Additional quantitative measures that can be used include (USEPA 2009, p65-66):

1) the coefficient of determination (r2) of the linear regression between simulated and observed;

2) the coefficient of model-fit efficiency, which measures the proportion of variance in the observed as explained by the simulated (Nash and Sutcliffe, 1970).

The coefficient of determination (r2) and the coefficient of model-fit efficiency are similar because both provide a measure of the variation in the simulated value explained by the observed value.  The coefficient of model-fit efficiency, however, provides a more rigorous evaluation of the fit quality than does the (r2) because the model-fit efficiency measures the magnitude of the differences between simulated and observed values, whereas the (r2) measures the difference between the mean values (Duncker and Melching, 1998).  In cases were the observed values and model residuals are normally distributed, the value of the (r2) and the model-fit efficiency should be equal.  The difference between simulated and observed values may also be reported as the (1) standard error, in ft3/s; (2) root mean square error, in percent; (3) percent of time differences were within x percent, (4) median percent error, (5) minimum percent error, and (6) maximum percent error.

Models for Comparative Analyses:

Occasionally, comparative modeling is used to evaluate potential water flow and water quality benefits from combinations of storm water management practices and designs that have yet to be implemented.  A cost benefit analysis of varying designs and design combinations may be the basis for this type of modeling.  In these instances, the following will be addressed and included in the modeling report.

- Definition of the Base Line Conditions - the specific conditions, parameters and values that define the baseline condition.

- Criteria for Comparisons - the terms for comparing the model simulation results to the base line condition.  For example, the terms may be found in quantities or percentages of runoff, infiltration or storm water contaminant loads.

- Identify Significant Change from Baseline - the application of statistical tools and criteria used to determine if there are significant differences between the baseline condition and model simulation results.

- Identify Simulation Scenarios from Sensitivity Analysis - how the simulation scenarios take into account what is understood from the model sensitivity analysis.

- Corroboration of Model Outputs - use of literature searches, calculations and the growing number of storm water performance databases to “ground truth” the projected water flow and/or water quality benefits from storm water management designs.  Some examples include the following:

- EPA Urban Best Management Practices Performance Tool http://cfpub.epa.gov/npdes/stormwater/urbanbmp/bmpeffectiveness.cfm

- University of New Hampshire Stormwater Center http://www.unh.edu/erg/cstev/pubs_specs_info.htm

- University of Massachusetts Stormwater Technologies Clearinghouse http://www.mastep.net/

- International Stormwater Database http://www.bmpdatabase.org/

- Center for Watershed Protection http://www.cwp.org/

- Boston Metropolitan  Area Planning Council - Massachusetts Low Impact Development Tool Kit http://www.mapc.org/regional_planning/LID/LID_Links_References.html#national

- EPA Low Impact Development Literature Review http://www.epa.gov/owow/nps/lid/lid.pdf

- General stormwater reference sites: http://newmoa.org/prevention/webconferences/stormwaterweb/stormwaterresources.pdf

4.3 Reconciliation with User Requirements

MassDEP is committed to developing a representative modeling product and will ensure that: (1) complete documentation is maintained, (2) departures from corroboration (validation) criteria are addressed, (3) corroboration (validation) methods are properly documented, and (4) the modeling data is properly reviewed.  In this context, reconciliation with user requirements connotes establishing how model results will be tested and evaluated in order to ensure that the models are producing results of sufficient quality.

As part of the reconciliation process, the model deliverables (modeling reports, technical memoranda, etc.) will be reviewed by the MassDEP Project Manager to assess whether the quality requirements of the QAPP have been met.  A comprehensive review of the final model files and documentation will be completed and recommendations provided regarding the effectiveness of the model to be used in watershed planning and TMDL decision-making.  The determination will largely be based on the effectiveness of the model to predict hydrologic and water quality response.

Each project manager under this QAPP will be responsible for identifying whether there have been departures from the assumptions in the planning process.  Any significant departures from the QAPP and initial assumptions will be reported to the Section Manager.  See also Section 4.4.

4.3.1 Model Limitations and Final Evaluation Criteria

It should be noted that all models are a simplification of the environmental processes they intend to represent.  Although there is no consensus on model performance criteria in the literature, a number of basic statements are likely to be accepted by most professional modelers.

• Models are approximations of reality and cannot precisely represent natural systems.

• There is no single, accepted test that determines whether or not a model is validated.

• Models cannot be expected to be more accurate than the sampling and statistical error

(e.g., confidence intervals) in the input and observed data.

These considerations must be included in the development of appropriate procedures for quality assurance of the models.  Despite a lack of agreement on how models should be evaluated, the following principles provide a final set of evaluation criteria for the modeling projects.

- Exact duplication of observed data is not possible, nor is it a performance criterion for projects. The model corroboration (validation) process will measure the ability of the model to simulate measured values.

- No single procedure or statistic is widely accepted as measuring, nor capable of establishing, acceptable model performance.  Therefore graphical comparisons, statistical tests and professional judgment are proposed activities which can be conducted individually or in concert to provide sufficient evidence upon which to base decisions on acceptance or rejection of model results.

- All model and observed data comparisons must recognize, either qualitatively or quantitatively, the inherent error and uncertainty in both the model and the observations.  Model sensitivity and uncertainty will be documented, where possible, as part of each modeling study.

The Clean Water Act statute and regulations require that a TMDL include a margin of safety (MOS) to account for any lack of knowledge and limitations concerning the relationship between the load and wasteload allocations and water quality.  If conservative assumptions are made in the analyses and modeling that account for the MOS, this implicit MOS must be described for future use in the TMDL report.  Otherwise, the TMDL report must identify an explicit MOS (e.g. expressed in the TMDL as loadings set aside for the MOS).   Implicit or explicit margins of safety are included as part of MassDEP TMDLs (including those involving modeling processes).

4.4 Reports to Management

Existing Data

TMDL reports include: a water body description, watershed GIS assessment and estimation of pollutant sources, identification of a total target goal (acceptable amount), allocation of watershed/land-use loadings, and uncertainty concerns and seasonal variation.  These reports include a public participation component to allow for public review prior to submission to EPA.

An Integrated Water Quality Report is submitted to EPA on a biannual cycle. This satisfies

reporting requirements in Sections 314, 305(b) and 303(d).

Model Application Data

The MassDEP Project Manager will report the status of project activities to the Section Manager. Problems encountered during performance evaluations, system assessments and data quality evaluations will be identified and appropriate corrective actions will be determined and implemented, if necessary.  These problems and corrective actions will also be documented in the modeling document(s).

5.0 MODELING REPORTS/DOCUMENTATION

The Modeling Report or other agreed upon documentation is essential to documenting the entire modeling process for a specific project.  It provides a comprehensive, detailed and clear description of the modeling objectives, technical approach, parameter estimation, results, error analyses and conclusions.  Brief examples of modeling report documentation from past projects are provided in Appendix E.

Modeling documents in a form acceptable to MassDEP will be prepared for all MassDEP TMDL modeling projects and will contain the following information, as applicable.  Where not applicable, an explanation will be provided.  Although pre-dating the specifications in this QAPP, example modeling reports that contain much of the information below are included in Appendix D.

Introduction and Background

Water Quality Model

Purpose of Modeling/Modeling Objectives

Watershed map, including impaired segments and data stations

Land use map

Scope and Approach for Each Model Used (including):

Physical Setting (and Hydrology, if applicable)

Observational Data Used to Support Modeling

Quality of Acquired Data (and references to data quality reports)

Achievement in Meeting Acceptance Criteria

References to Monitoring Data

Discussion on Excluded Data and Basis for Exclusion

Description of Model(s) (including):

Type

Framework

History (e.g., recent use locally)

Documentation

Model Configuration (discusses how model was applied, including):

Spatial and Temporal Resolution

Nature of Grid, Network Design or Sub-watershed Delineation

Application of Sub-models

Model Inflows, Loads and Forcing Functions

Key Assumptions (associated limitations, if any)

Changes and Verification of Changes Made in Code, if any

Water Quality Model Transport and Chemical Parameterization (calibration)

Model Parameterization (calibration)) and Corroboration (validation)

Objectives, Activities and Methods

Parameter Values and Sources

Rational for Parameter Values Estimated in the Absence of Data

Parameterization Variables and Targets

Measures of Parameterization Performance

Model Load Inputs

Parameterization (calibration) Input, Output and Results Analysis

Model Corroboration Results

Model Prediction Runs

Model Use Scenario Analysis and Results (should relate to purpose)

Output of Model Runs and Interpretation

Summary of Assessments and Response Actions, if any

Soundness of the Parameterization, Corroboration and Simulations

Review of Initial Assumptions and Model Suitability Evaluation

Performance Against the Performance Criteria Including: Model Parameterization and Corroboration

Model Sensitivity Analysis and Components of Impact

Model Sensitivity and Uncertainty Analyses

Pre- and Post-Processing Software Development, if any

Deviations from the QAPP Including a List of Non-Applicable Reporting Elements with Explanations

Conclusions

Recommendations

Reference Information for Model User Manual

General References

Appendices (including: Maps, GIS Data, Photographs and Drawings (as appropriate))

6.0   USEFUL LINKS AND REFERENCES

ASTM. 1984. Standard Practice for Evaluating Environmental Fate Models of Chemicals. Designation E978-84. American Society of Testing and Materials. Philadelphia, PA. 8 p.

Barlow, P.M.  2000.   Documentation of computer program STRMDEPL-A program to calculate streamflow depletion by wells using analytical solutions, In Zarriello, P.J. and Ries, K.G. III, 2000, A precipitation-runoff model for the analysis of the effects of withdrawals on streamflow, Ipswich River Basin, Mass., U.S. Geological Survey Water-Resources Investigation Report 00-4029, 99 p.

Brown, C.L., and T.O. Barnwell, Jr. 1987. The Enhanced Stream Water Quality Models QUAL2E and QUAL2E-UNCAS – Documentation and User’s Manual. Environmental Research Laboratory. EPA/600/3-87/007. Athens, GA.

Chapra, S.C.  1997.  Surface Water Quality Modeling.  WCB/McGraw-Hill

Dillon, P.J. and F.H. Rigler. 1974. A test of a simple nutrient budget model predicting the phosphorus concentration in lake water. J. Fish. Res. Bd. Can. 31:1771-1778.

Diskin, M.H. and Simon, E. 1977. “A procedure for the selection of objective functions for hydrologic simulation models.” Journal of Hydrology, 34, 129-149.

Duncker, J.J. and Melching C.S., 1998, Regional rainfall runoff relations for simulations of streamflow for watersheds in DuPage county, Illinois: U.S. Geological Survey Water-Resources Investigations Report 98-4035, 80 p.

European Commission, 2005 – Harmonising Quality Assurance (HarmoniQuA) in model based catchment and river basin management. http://harmoniqua.wau.nl/

James, L.D. and S.J. Burges. 1982. Selection, Calibration, and Testing of Hydrologic Models. In: Hydrologic Modeling of Small Watersheds. ASAE Monograph No. 5. C.T. Haan, H.P. Johnson,

Kirchner, W.B. and P.J. Dillon. 1975. An empirical method of estimating the retention of phosphorus in lakes. Water Resour. Res. 11:182-183.

Maine DEP.  2008.  State of Maine Department of Environmental Protection QAPP for Model Simulations in the TMDL Program.  52p; approved by EPA in July, 2009.

MassDEP. 2015.   DWM-WPP Quality Assurance Program Plan for Surface Water Quality Monitoring and Assessment, 2015-2019; CN 460.0

Mattson, M.D. and R.A. Isaac. 1999. Calibration of Phosphorus Export coefficients for Total Maximum Daily Loads of Massachusetts Lakes. Lake and Reservoir Man. 15(3):209-219.

Montana DEQ. 2005.  Bitteroot Soil Water Assessment Tool (SWAT) Model Quality Assurance Project Plan.  Montana Department of Environmental Quality-Water Quality Planning Bureau, Data Management Section. March.

Nash, J.E. and Sutcliffe, J.V., 1970. River flow forecasting through conceptual models; Part-I, A discussion of principals: Journal of Hydrology, v. 10, p. 282–290.

NEIWPCC.  2005.   Calibration and Validation of the AVGWLF Model for New England and New York State; Prepared for USEPA by the New England Interstate Water Pollution Control Commission

Reckhow, K.H. 1979. Uncertainty Analysis Applied to Vollenweider's Phosphorus Loading Criteria. J. Water Poll. Control Fed. 51(8):2123-2128.

Reckhow, K.H., M.N. Beaulac, J.T. Simpson. 1980. Modeling Phosphorus Loading and Lake Response Under Uncertainty: A Manual and Compilation of Export Coefficients. U.S.E.P.A. Washington DC. EPA 440/5-80-011.

Thomann, R.V. 1982. Verification of Water Quality Models. Jour. Env. Engineering Div. (EED) Proc. ASCE, 108:EE5, October.

U.S. EPA.  2000.  Quality Assurance of Environmental Models (DRAFT); NRCSE Technical Report Series #042

U.S. EPA. 2002.  QA/G-5M - Guidance for Quality Assurance Project Plans for Modeling. EPA/240/R-02/007- December. http://www.epa.gov/quality/qs-docs/g5m-final.pdf

U.S. EPA Office of Research and Development.   2004.   Model Transparency at the U.S. EPA.  International Marine Environmental Modeling Seminar, October 19-21, 2004

U.S. EPA Region I. 2007.  Draft Generic Modeling Quality Assurance Project Plan Template, June. http://www.epa.gov/region1/lab/qa/qamodeling.html

U.S. EPA.  2009a.  Guidance on the Development, Evaluation and Application of Environmental Models, Council for Regulatory Environmental Modeling (EPA/100/k-09/003), March, 2009; www.epa.gov/crem

- USEPA 2009b.  QAPP Guidance for Environmental Projects Using Only Existing (Secondary) Data, Rev. #2, 10/13/09, EPA-Region 1, QA Unit http://www.epa.gov/region1/lab/qa/pdfs/EPANESecondaryDataGuidance.pdf

Vollenweider, R.A. 1975.   Input-Output Models with Special Reference to the Phosphorus Loading Concept in Limnology. Sch. Zeit. Hydrologic 37:53-84.

Appendix A - Example Scope of Services for TMDL Modeling (MassDEP)

#### SCOPE OF SERVICES

Assabet River Nutrient Total Maximum Daily Loading (TMDL) Project

I.	Purpose

The purpose of this Contract is to provide Assabet River nutrient Total Maximum Daily Loading (TMDL) modeling services to the Massachusetts Department of Environmental Protection (the Department).  Nutrient Total Maximum Daily Loading (TMDL) modeling in the Assabet River is necessary because of the presence of summertime eutrophic conditions in the river system.  As a result of water quality problems associated with these conditions, the Assabet River was placed on a list of impaired waterbodies requiring water quality improvement, known as a Section 303(d) list and implementation of a nutrient TMDL allocation plan is required.

II.	Scope of Services

The Scope of Services for this Contract shall consist of the following deliverables as outlined below.  In order for a deliverable to be complete under the Contract, the deliverable must be done in accordance with the Contract specifications (and the Contract schedule).  Deliverables must be approved by the Department and otherwise satisfy the Contract provisions, as determined by the Department.  The scope of services shall consist of the following tasks. As specified in the deliverables paragraph of each task below, the Contractor will develop technical memorandum that will address both the in-stream and watershed components of the Assabet River model.

Task 1:  Literature Review

The Contractor has performed a detailed literature review as part of another Department sponsored project and will review any additional literature that may become available as part of the model set-up and development task (Task 5).

Deliverables: Copy of previously completed literature review with any additional literature reviewed as part of this project.

Task 2:  Identification of Appropriate Models

Based on the modeling requirements for the Assabet River system and potentially critical modeling factors identified by the Contractor in their proposal, the Contractor has tentatively identified two models as most appropriate for the Assabet River nutrient TMDL modeling project.  The model to be applied will be selected with the concurrence of the Department.  The two models recommended for consideration are:

- HSPF11, the USGS/EPA combined watershed and instream water quality model Hydrological Simulation Program – Fortran, and

- WASP5, the EPA instream Water Quality Analysis Simulation Program.  The WASP modeling package would be coupled with a separate watershed land use based model.

Each of these models has the modeling capabilities necessary for this project and is well suited to successfully complete the Assabet River nutrient TMDL allocation modeling application.

Deliverables: A Technical Memorandum (TM#1) will be submitted to the Department describing the model selection process, including a detailed description of HSPF and WASP.  TM#1 will provide documentation of the model selection process including both the in-stream and watershed model.  TM#1 will also identify the selected model and provide the rationale for its selection.  The contractor will seek input and comment from the Consortium relative to the model selection and inputs.

Task 3:  Hydrologic and Nutrient Budgets

The Contractor shall develop water quality predictions using the selected model and apply these predictions to establish appropriate TMDL allocations for sources throughout the Assabet River system and to evaluate alternative loading allocation options.  Development of the model including model set-up, calibration, and validation shall follow the approach outlined in the Contractor’s proposal or as otherwise approved by the Department.

The hydrologic and nutrient budget models will accurately simulate the hydrology and nutrient budgets of the Assabet River system over time.  The model will provide a mass balance of nutrients at each time step and will simulate nutrient kinetic processes observed in the field.  The developed hydrologic and nutrient budget will be fully capable of providing the predictions required to perform the TMDL allocation.

Deliverables: A Technical Memorandum (TM#2) will be submitted to the Department describing the approach to be applied to the modeling process.  TM#2 will include a description of all key model settings and the rationale to be applied in assigning values for each.  TM#2 will also include a description of the approach to be applied in the model calibration and validation process. The TM

will address the modeling approaches to be used for both the water quality model as well as land use model. The Contractor will coordinate these efforts with the Consortium who will be collecting

data and mapping the stormwater systems for each town so that the same subwatersheds and physical attributes are used in both studies where feasible.

Task 4: Land Use Based Modeling to Determine Non-point Source Loadings

The Contractor shall develop a land use based model using a GIS platform that will be fully integrated with the hydrologic nutrient budget model as described in their proposal.  The process of developing an effective land use model for the Assabet River watershed will involve identification of subwatersheds, assignment of a set of physical attributes and functional coefficients to each subwatershed, and adjustment of these parameters to improve the correlation between model predictions and field measurements.  Land use based model set-up, calibration, and validation tasks will be provided to the Department in a Technical Memorandum.

The land use based model will provide dynamic predictions of nutrient loadings in each Assabet River subwatershed in support of TMDL modeling goals.  The land use based model will be applied to provide quantitative predictions of water quality improvements associated with hypothetical modifications to land use in each subwatershed.  For example, the model may be applied to predict reductions in nutrient loadings over time associated with implementation of specific BMPs at golf courses.

Deliverables:

A Technical Memorandum (TM#3) will be submitted to the Department providing the results of the model calibration and validation process for both the water quality and watershed model applications. TM#3 will include a discussion of all key model parameters adjusted during the calibration process and the technical basis for each calibration adjustment.

Task 5:  Develop the nutrient TMDL and evaluate alternative loading options.

The Assabet River TMDL model developed by the Contractor will include the hydrodynamic and nutrient budget model and land use based watershed model components described above.  The Contractor shall apply the developed Assabet River TMDL model to provide dynamic predictions of the hydrology, water quality, and biology of the Assabet River and impoundments along its course with emphasis on quantification of ambient nutrient concentrations and associated water quality (e.g., dissolved oxygen concentration) and biological (e.g., alga and macrophyte biomass) conditions.  The Contractor will apply the model to predict water quality, at a minimum, under the following scenarios:

Present summertime conditions during low flow (7Q10) and average flow conditions. The scenarios will account for time-varying nutrient loadings such as those associated with wet-

weather point and non-point source loadings. Nutrient TMDL allocation conditions including identification of all point source (WLA) and non-point source (LA) allocations necessary to

achieve the TMDL allocation.  The TMDL scenario will be specified by performing numerous model simulations with incrementally reduced loadings from present levels towards acceptable levels until the model predicts that both narrative and numerical water quality standards for the Assabet River will be met. Nutrient TMDL modeling will quantify the nutrient assimilation loading capacity of the Assabet River.

- Alternative loading options.  The Contractor will perform simulations of 20 alternative loading scenarios, selected collaboratively with the Department.  Each alternative loading option will provide an alternative means of achieving the TMDL allocation using a different set of WLAs and LAs.  Other methods of improving water quality, such as dam removal and land use modifications, may also be evaluated as alternative loading options. The contractor will seek the input of the Consortium during the selection of alternative scenarios.

Modeling results for each of the scenarios outlined above will include all necessary predictions including time-varying flows, nutrient concentrations, dissolved-oxygen concentration and algal and macrophyte biomass.  Since the proposed modeling package includes a river model integrated with a land use based watershed model, the in-stream water quality impacts of land use changes in specific subbasins may be quantified using the model.

The nutrient TMDL allocation modeling application will result in identification of a nutrient TMDL allocation for the Assabet River and 20 alternative loading scenarios to support the TMDL planning and implementation process.  TMDL allocations will include all required factors, such as allocation of both point and non-point source loadings, seasonal considerations, margin of safety and future growth allowances.

Deliverables:

1) Draft Set of Alternative Loadings Scenarios.  A Technical Memorandum (TM#4) will be submitted describing a set of draft model application scenarios.  Each scenario would likely feature an alternative set of nutrient loading reductions (e.g., different tradeoffs between point and non-point source reductions).  TM#4 will be written in a format suitable for delivery to interested parties in order to seek input and reach consensus on a reasonable set of model application scenarios.  Scenarios described in TM#4 will likely include a set of alternative approaches to reducing nutrient loadings to the river.  TM#4 will also include the rationale for each proposed model application scenario.

2) Final Set of Alternative Loadings Scenarios. A Technical Memorandum (TM#5) will be submitted to the Department describing the final set of model application scenarios.  TM#5 will provide a final

list of model application scenarios to be run and will provide a rationale for each.  TM#5 will also include a response to feedback provided by interested parties.  Scenarios added or modified based

on input from interested parties will be noted and the rationale for declining to add scenarios suggested by interested parties if deemed necessary, will also be included.  TM#5 will provide a list of modeling scenarios to be applied and will provide a technical basis for each.

Task 6: Develop a draft nutrient TMDL report for review.

The Contractor will prepare and submit a clear and concise draft TMDL report in accordance with EPA guidance and input provided by the Department.   The draft TMDL report will provide, at a minimum a narrative with accompanying tables and charts that address the 12 items listed in the RFR.

Task 7: Draft and Final Report

The Contractor will prepare and submit a clear and concise draft and final report in accordance with the RFR.  The draft report will be submitted at least two months prior to the scheduled termination date of the contract. The Contractor will receive comments from reviewers, perform necessary edits, and submit a final report, including a “Response to Comments” document for inclusion in the final report.

Task 8: Coordination and Timelines

The Contractor will work with MA DEP to ensure that the TMDL allocation process (WLA and LA) is closely coordinated with the comprehensive wastewater management planning process currently underway with the Assabet River Consortium. Coordination with Consortium will include regular communication throughout the project in order to work closely with the Consortium to develop environmentally acceptable  and implementable TMDL allocation strategies. The Contractor will attend and participate in five (5) meetings on a quarterly basis for the duration of the project to inform and receive comments from the consortium regarding the progress of the TMDL process.

Task 9: Public Meetings

During the TMDL development process The Contractor will hold six public meetings to inform the public of the TMDL process, schedule, and findings. The Contractor will coordinate each meeting with the Consortium Program Manager, EOEA SuAsCo Watershed Team Leader, the Organization for the Assabet River (OAR), the Department, and EPA Region I.  At the conclusion of the project, the Contractor shall hold up to two public meetings to summarize the findings of the TMDL allocation project.

Task 10: Quarterly and Final Reports

The Contractor will prepare and submit brief quarterly progress reports, as well as draft documents or models required for each task. The Contractor shall also provide a final camera-ready copy of the Final Report; electronic versions of all data, models, model runs and modeling results in formats approved by the Department (e.g. on CD disk); and 25 copies of the Final Report.

Appendix B – Example Technical Memorandum for Calibration & Validation (MassDEP)

Date: 10 April, 2001

To:   MADEP

From: Contractor

RE: Assabet River Nutrient TMDL Model:  Water Quality Model Calibration

This memo contains a summary of the water quality model calibration and validation components of the HSPF numerical modeling application performed in support of the Assabet River Nutrient TMDL project. The water quality model calibration includes water quality, sediment quality, and biological components and represents the final task prior to model simulation. The model calibration process involves using field measurements of hydrologic, water quality, sediment quality, and biology to support parameterization of the selection the model to accurately simulate present water quality conditions.

The objective of the water quality model was to develop a simulation tool to provide a reasonable numerical representation of the numerous interrelated physical and biological processes that occur in the Assabet River. The model could then be applied to simulate the present water quality conditions and a set of alternative conditions in the Assabet River system. Thus, the model may provide a technical basis for evaluating alternative river management scenarios for improving water quality.

Specific calibration goals were to predict:

• Concentrations of orthophosphorus, nitrate and ammonia along the river profile;

• Diurnally varying DO concentrations with focus on minimum daily DO values; and

• Quantities of biomass of free-floating and rooted aquatic vegetation within the impoundments.

These metrics were considered indicative of the Assabet River system water quality status.  The water quality model was calibrated to field data collected in August of 2000. This data set was chosen for calibration purposes because the data set is more extensive and the flow conditions were more readily simulated than those of the July 1999 data set. Field data collected during the August 2000 survey included nutrient concentrations (nitrate, ammonia, total and ortho-phosphorus) and DO concentration measurements at 21 river locations, continuous, in-situ DO concentration measurements in six reaches and biomass measurements in each of the impoundments. This was the primary data used  to calibrate the water quality model. The calibration model simulation was run from June 1997 to September 2000.

Boundary Conditions

Boundary conditions specific to the water quality segment of the model were applied to specify nutrient concentrations, dissolved oxygen concentrations and subsequent biological reactions at model boundaries. Boundary conditions are specified were based on measured or observed values. Data was analyzed to determine the appropriate resolution for accurate model predictions. External boundary conditions include point source loads, non-point source loads, rainfall, evapotranspiration and solar radiation.

Point source loads in the Assabet watershed are contributed by the four major wastewater treatment plants on the mainstem of the River. The treatment plants are Westboro (RM 30), Marlboro (RM 23.6), Hudson (RM 15.7) and Maynard (RM 6.7). Monthly data for each plant was available for January 2000 to October 2000. Missing data was interpolated among available values. Monthly average values were used at each plant to specify flow and nutrient loads. Daily discharge values were averaged over each month. Nutrient concentrations, including ammonia, nitrate, orthophosphorus, dissolved oxygen, BOD, TSS and alkalinity were measured periodically throughout the period of record, generally with two or three measurements each month. Monthly average nutrient values, in lbs/hr, were added to the appropriate model reach for each treatment plant. Table 1 presents the model input values of flow and nutrient concentrations at each treatment plant. Although flow and concentrations vary diurnally and daily, variations are not great, and monthly averaged values were assumed representative of the loadings.

Non-point source loads were determined for each watershed, based on land use distribution and export coefficients. One-half of the predicted nutrient load from each watershed was assumed to reach the river based in literature source recommendations. Each of the watersheds delineated in the model contributes flow and nutrient loadings, including particulate phosphorus, orthophosphorus, nitrate, ammonia and sediment, to the model reaches of the Assabet River. Nutrients are washed in with flow during rainfall events throughout the year.

Figures 1 through 3 present the model inputs of point and non-point source loadings for total phosphorus, orthophorus and nitrate. The point source dissolved phosphorus nutrient loadings are significantly higher than the non-point source loadings. Non-point source loadings of total phosphorus and dissolved nitrate are generally greater than or equal to the point source loads.

Meteorological boundary conditions include rainfall, evapotranspiration and solar radiation. The data sets for rainfall and evapotranspiration have been discussed in earlier communications. Hourly solar radiation values were computed from a data set for percent cloud cover recorded at Worcester, MA. The data set consisted of daily values ranging from 0 to 10, representing percent cloud cover. The daily solar radiation was computed from this data set, as a function of latitude. The daily solar radiation values were disaggregated into hourly values.

After initial model runs, the solar radiation data set was further modified to better represent the amount and timing of solar radiation that reaches the river. Initial model results revealed that predicted DO concentration began to rise much earlier each day than measured values. This was attributed to trees along the river bank that shade the river in the morning hours. The solar radiation data was modified to reflect this by setting the first few hours to zero, in effect having the sun rise later and more quickly.

This resulted in predicted diurnal DO variation the more closely matched observed values.

Calibration Procedure

To begin the model calibration, initial values were selected for each of the model parameters. Initial conditions were also established for each of the predicted parameters. However, the model was fairly insensitive to initial conditions, and equilibrium values were achieved within a few time steps (ΔT = 1 hour). When available, field data were used to establish initial parameter values and initial conditions. When field data were unavailable, average literature values were selected.

To complete the calibration, model parameters were adjusted to achieve reasonable agreement between measured and predicted values. In general, a single parameter was adjusted at a time to isolate its effect on the system. Parameters were adjusted within reasonable limits, based on allowable model ranges or literature values. Of the parameters calibrated, a few had a significant effect on the simulation results. Table 2 presents significant model calibration parameters and supporting information.

Calibration Results

The results of the model calibration are presented in Figures 4 through 6. Where model results are presented with rivermile in the x-axis and parameter value in the y-axis. Model predictions are

compared with measured values in Figures 4 through 6. Model predictions were averaged over the week of the field program, in order to eliminate diurnal variation in the data. Temporal comparisons were made at the actual time of collection.

Figure 4 presents the measured and week-averaged predicted nutrient concentrations. The predicted values are in the range of the measured values, and follow the spatial trend of nutrient concentrations with downstream distance. Spikes in observed nutrient concentrations occur at the locations of the WWTPs. Modeled treatment plant loadings were assigned based on data provided by the treatment plants, and monthly average values were established. It is possible that these data may not be representative of the actual plant discharge during the period of the field program, as indicated by the absence of a spike in predicted orthophosphorus concentration at the Hudson treatment plant.

Figure 5 presents the modeled and measured aquatic biology in each of the impoundments.  Phytoplankton, or floating algae, is presented in units of mg/L, and the benthic algae, or rooted plants, is presented in units of mg/m2. In general, model predictions match the observed concentrations and follow the observed spatial trends.

Figure 6 presents the modeled and measured dissolved oxygen in several reaches along the river. The diurnal DO variation is captured in the model predictions, with the exception of the most upstream sampled location, School Street. The maximum DO concentrations are slightly under or overpredicted, but the minimum concentrations are well predicted, as well as the duration of low DO and the rates of diurnal increase and decrease.

Validation Results

Figures 7 through 9 present the results of the model validation. Data from the July 1999 field programwas used for model validation. The figures follow the same presentation format described above. The predicted validation values do not provide as good a match to the field data as the calibration values.  However, general spatial trends in nutrients and biomass are predicted. In the downstream reaches, minimum DO concentrations are well predicted, but less so in the upstream reaches. Validation difficulty is likely attributable, at least in part, to the extremely low flows that complicated the hydrologic and hydraulic calibration.

Summary

We are generally pleased with the Assabet River model calibration and believe that the model has been successfully calibrated. We hope that you find this memorandum informative and thought provoking.  We look forward to meeting with you to discuss this important process and to moving forward with model simulations. Please do not hesitate to call with any questions or comments.

Appendix C – Example Technical Memorandum for Model Application (MassDEP)

To:  MA DEP

Date: 6 June, 2003

From: Contractor

File:

RE: Assabet River Nutrient TMDL: Water Quality Modeling Results for 4 Alternative Management Scenarios

This technical memorandum provides Assabet River water quality model predictions associated with a set of 4 alternative management scenarios. The alternative management scenarios are described briefly below, followed by a presentation of water quality modeling results.

Introduction

The 4 alternative management scenarios consist of reductions in WWTP effluent phosphorus and of reductions in benthal release of phosphorus at both permitted and projected WWTP flows, as described below and summarized in Table 1. Projected flows (Table 2) were provided by the Assabet River Consortium. The alternative scenarios presented in this memo are:

1. Permitted flows – In two scenarios, effluent dissolved phosphorus concentrations at the four treatment plants were reduced concurrently with reductions in the benthal release of phosphorus. These scenarios were run with the permitted effluent flows from the WWTPs (Table 2). The scenarios are:

- WWTP dissolved phosphorus at 0.2 mg/l and benthal release of phosphorus at 25% of existing flux rate; and b. WWTP dissolved phosphorus at 0.2 mg/l and benthal release of phosphorus at 10% of existing flux rate.

2. Projected flows – In two scenarios, effluent dissolved phosphorus concentrations at the four treatment plants were reduced concurrently with reductions in the benthal release of phosphorus. These scenarios were run with the projected effluent flow rates from the WWTPs (Table 2). The scenarios are:

- WWTP dissolved phosphorus at 0.2 mg/l and benthal release of phosphorus at 25% of existing flux rate; and

- WWTP dissolved phosphorus at 0.2 mg/l and benthal release of phosphorus at 10% of existing flux rate.

Assabet River Water Quality Modeling Results

Model results for each scenario are presented and discussed below. Model results are compared to the existing conditions scenario. Model results are reported for the low flow period observed during July 12 – 18, 1999. Model scenario results are presented in nine figures and two tables.  Three figures of instream nutrient concentration predictions (total and dissolved phosphorus and nitrate), three figures of DO predictions (maximum, minimum and range) and three figures of biomass predictions (total, floating and rooted). One table presents a tally of the number of hours that the model predicted DO concentration is below 5.0 mg/L and the number of hours that DO exceeds saturation. The second table presents the total predicted biomass (in kg) in each impoundment.

Nutrient Concentration Predictions

Figures 1 –3 present total phosphorus, orthophosphorus, and nitrate concentration predictions vs. river mile during low flow conditions. Model predictions indicate that in-stream phosphorus concentrations are greatly reduced for all alternative scenarios compared to existing conditions. In general, predicted phosphorus concentrations are similar for all alternative scenarios. Predicted phosphorus concentrations are slightly lower for the scenarios with a 90% reduction in benthal release of phosphorus. The projected flow scenarios result in the slightly lower predictions of phosphorus than the corresponding permitted flow scenarios. Predicted nitrate concentrations rise with reductions in WWTP dissolved phosphorus concentrations and/or benthal release of phosphorus due to decreased nitrate consumption and constant nitrate loading.

Dissolved Oxygen Concentration Predictions

Figures 4 - 6 present ambient DO max, min, and range predictions vs. river mile during low flow conditions. Model predictions of DO indicate that, for all alternative scenarios, the minimum DO concentration is generally increased, while the maximum DO concentration is generally reduced – reducing the overall DO range, as compared to existing conditions. Table 3 presents the number of hours over the one-week period with DO below 5.0 mg/L and with DO above saturation in all reaches. Model predictions indicate the minimum DO falls below 5.0 mg/L for less than 1% of the period of record for all alternative scenarios. The scenario with 90% reduction in benthal release of phosphorus and projected flows results in achievement of the 5.0 mg/L water quality standard for the entire period of record.

Biomass Predictions

Figures 7 – 9 present total biomass (floating and rooted), total floating biomass, and total rooted biomass predictions vs. river mile during low flow conditions. Model predictions indicate that, when phosphorus loads are reduced, predicted biomass for all alternative scenarios is reduced relative to existing conditions. Table 4 presents the total biomass (in kg) for existing and alternative scenarios in each of the Assabet River impoundments and provides a numerical representation of the model predictions provided in Figure 7. Predicted total biomass in the alternative scenarios is reduced 32% to 45% from existing conditions. Greater reductions in biomass are predicted under the projected flow scenarios than the corresponding permitted flow scenarios. It should be noted that biomass predictions are less certain than other model predictions due to several factors including the model’s simplifying assumptions and the complex and changing species composition characteristics of river systems.

Summary

The Assabet River water quality model predicts reductions in phosphorus concentrations, improvements in DO dynamics, and reductions in total biomass associated with implementation of the alternative management scenario evaluated (as shown in the attached figures and tables).

Increased water quality improvement is realized with a decrease in WWTP dissolved phosphorus concentrations and/or a decrease in the benthal release of phosphorus, indicated by predictions of lower phosphorus and biomass concentrations. Both of these reductions in phosphorus loads are predicted to contribute significantly to the overall improvement in water quality. Projected flow scenarios, with slightly higher effluent flows at two of the WWTPs, results in slightly greater improvements in water quality over the corresponding permitted flow scenarios.

Table 1 WWTP Phosphorus Reduction Series: Alternative Management Scenarios

Table 2 WWTP Permitted Flows

Table 3 WWTP Phosphorus Reduction Series: Prediction of Number of Hours (in all reaches) that DO falls below 5.0 mg/L or Exceeds Saturation Under Low Flow Conditions

Table 4 WWTP Phosphorus Reduction Series: Predicted Total Biomass (kg, Floating and Rooted) Under Low Flow Conditions

Appendix D – Example Modeling Reports:

1)  Development of an HSPF Model of the Upper and Middle Charles River

2)  A Hydrodynamic and Water Quality Model for the Lower Charles River Basin, Massachusetts

(Double-click on the title pages below to view the documents)

Appendix E – Example Tables and Figures

for Modeling TMDL Reports

Appendix F – Modeling Journal

Appendix F:  EPA-NE Modeling Journal Checklist and Template

(Double-click on the title page below to view the document)