Massachusetts Department of Environmental Protection

Division of Watershed Management

STANDARD OPERATING PROCEDURE

DRAFT: Protocols for Using the Global Flow Meter FP101-201 for Velocity Measurements in Wadeable Rivers and Streams

CN 68.7

May 2010

| Prepared by: | ###### Joan Beskenis, Biologist | Date: |  |
| --- | --- | --- | --- |
| Approved by: |  |  |  |
|  | ###### Richard Chase, Quality Control Analyst | Date: |  |
| Approved by: |  | Date: |  |
|  | ###### Arthur Johnson, Monitoring Program Coordinator |  |  |
| Approved by: |  | Date: |  |
|  | ###### Dennis Dunn, Program Supervisor |  |  |

Protocols for Using the Global Flow Meter FP101-201 in Wadeable Rivers and Streams

- Scope and Application

This is an SOP for the Global Flow Meter FP101-201 for use in determining velocity (cm/sec) from wadeable rivers and streams in Massachusetts, particularly at biomonitoring sampling stations. Velocity can influence the communities that are able to develop under the prevailing conditions, thus choosing sites with comparable velocities is essential.  For the algae, low velocities <10 cm/sec can promote growth that may reach nuisance levels while high velocities >60 cm/sec may cause sloughing or breaking off and scouring of filaments with the subsequent establishment of a different algal assemblage.   Velocities between these values are desired for sampling in the riffles and or runs. The velocity measurements are made in areas where periphyton samples have been collected to determine the velocities to which the algae were actually exposed.

Velocities should be measured within the wadeable areas of flow.  Stagnant water should not be included.  For safety purposes stream reaches with velocities that are judged to be excessive (difficult to stand or to hold sampling gear) will also not be sampled. If the stream appears to be stagnant, but earlier visits have indicated flowing water, the stream habitat will still be evaluated, but if first visit indicates stagnant water it will not be considered for sampling.

- Summary

Average velocities throughout the water column are determined using the Global Flow Meter.  Measurements are made at locations within the reach established for biological surveys in the vicinity of where samples were obtained.

- Safety considerations

The probe is used in wadeable streams, so care must be taken while wading since surfaces can be slippery and visibility through the water column can be limited.

- Sample collection

The Global Flow Meter FP101-201 will be used to collect average velocity data through the water column at 6-12 locations within a stream reach where transects have been established.  Stream edges should be avoided.  If tiles are deployed for algae or rock baskets for macroinvertebrates then readings should be obtained above these specific locations.  Since only velocity is being measured, and not discharge, the velocity at specific depths is not required.  Instead, place the probe perpendicular to the flow just above the substrata that were sampled.  The probe is then moved through the water column (of whatever depth) for ~ 20 seconds.  An effort should be made to stay at each point within the water column for approximately the same amount of time.  The probe should be taken out of the water between locations.  Once it is returned to the water it will resume averaging.  After taking the last velocity measurement the average velocity should be recorded on the PERIPHYTON BIOMASS FIELD DATA SHEET. (Appendix 2)

- Apparatus

Flow meter manufactured by Global Water Instrumentation.  It consists of a sensing device with a propeller held at the bottom of the probe and a meter attached to the end of the probe tube that displays average velocity and maximum velocity.

- Calibration

The flow meter comes fully calibrated, but the calibration must be checked annually against the results obtained from two other meters brought out to the field and used to measure velocity at the same exact location and height

- Procedure

Take the meter out of the case and attach the meter to the top of the probe.  It clicks into position.  Make sure that the display is set to m/sec.  The display should show II in the upper left hand corner.  If it is not set to m/sec then you need to use a pencil or a pin to press in the small “plug” on the back of the meter to toggle to II.  Also, press the bottom button on the meter to scroll until “avgspeed”  appears.  If the meter displays a measurement or after you have recorded average velocity then hold the top button for 5 seconds to rezero.

Put the probe at the starting point and point the propeller directly into the flow.  The arrow on the bottom of the probe should point downstream. For periphyton sampling collect average velocity from measurements taken in riffle/ run areas where sampling occurred.  At each location, keep the meter in one spot horizontally, but move it slowly through the water column for ~20 seconds. Make sure to remove the meter from the water when moving between locations.  After you take your last measurement, record the average velocity on the PERIPHYTON BIOMASS FIELD DATA SHEET.  Since it was recorded in m/sec you need to multiply by 100 to convert to cm/sec.

Remove the meter from the water before moving between stations.  The meter will hold the last reading until the button at the top of the computer is held for 5 seconds.

If the probe gets fouled by algae or debris, remove the probe, clean the propeller and start the readings over.

After all the readings have been taken write down the final average velocity on the field sheet (algae or habitat).  Remember to multiply the reading by 100 to get cm/sec.

The meter will time out automatically and shut down when removed from the water after 5 seconds.

The probe and computer should be dried with a clean rag when brought back to the laboratory.

The case and meter should be air-dried at the end of the day by propping the lid open.

- Quality control

At the beginning of each field season personnel who will use the meter will have a training/refresher session to allow them to become familiar again with the parts of the meter and with the SOP.

At the beginning of each field season (May/June) the precision of measurements will be checked against another probe measuring velocity.  The acceptable precision level is +/- 15 %.  If this level cannot be met then the Troubleshooting methods outlined in the Global Flow Meter manual should be consulted.  If the precision is still off then the flow meter will need to be returned for repair.

On one survey date over the summer, the precision will be re-checked by having someone hammer in a pole at a location within a sample reach with a flag marker a couple of inches above the bottom.  One person will measure velocity at this one location according to protocol and then a second person will follow the same procedure at this location using the flagged pole to determine at what depth to start the readings.  The precision level should still be +/- 15 %.  This information should be recorded on a QAQC record sheet

- Interferences

Measurements should be made at locations where there are no obvious obstructions that would alter the flow.  Take measurements from the top of stones, not in-between two stones since normal flow patterns can not be obtained in that type of area.  In areas with a lot of vegetation (moss, macrophytes or algae) care should be taken to keep the probe held just above the vegetation.

- Preventative maintenance

Replace old batteries with new ones at the start of the field season.  New batteries will be available in the probe case.

Remove old batteries at the end of the sampling season.

- Corrective actions

If the meter is not recording correctly, remove probe from water and check for an obstruction around the propeller.  Remove any algae, stones or debris. Change the batteries; the back of meter can be opened with a coin.

Report problems to Joan Beskenis or Richard Chase.

- References

The Instruction manual for the Global Flow Meter Model FP101-201 is stored in the flow meter case.

- Attachments

ATTACHMENT 1:  Global FP101-FP201 Probe Manual

ATTACHMENT 2:  Periphyton Biomass Field Data Sheet

ATTACHMENT 1:

Global FP101-FP201 Probe Manual (double-click cover page below to open)

| MASSACHUSETTS DEP-DWM PERIPHYTON BIOMASS FIELD DATA SHEET-2010 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Watershed Waterbody: Station #: |  |  |  |  |  |  |  |  |  |
| Date/Time: Investigators: |  |  |  |  |  |  |  |  |  |
| Photos: |  |  |  | GPS coordinates: |  |  |  |  |  |
|  | Natural Substrate |  |  |  |  |  |  | Tile Substrate |  |
| Method used:rock strap 1in' diameter other: |  |  |  |  |  | date deployed: |  | date taken out: |  |
|  |  |  |  |  |  | Crew: |  | Crew: |  |
| # of rocks : |  |  | Volume of sample (ml): |  |  | # of tiles |  |  | Volume of sample (ml) |
|  | dominant algal cover |  |  |  |  |  | dominant algal cover |  |  |
|  | biofilm | filamentous | color |  |  |  | biofilm | filamentous | color |
| Sample 1-1* |  |  |  |  |  | Sample 1-1* |  |  |  |
| Sample 1-2 |  |  |  |  |  | Sample 1-2 |  |  |  |
| Sample 1-3 |  |  |  |  |  | Sample 1-3 |  |  |  |
| Sample 2-1 |  |  |  |  |  | Sample 2-1 |  |  |  |
| Sample 2-2 |  |  |  |  |  | Sample 2-2 |  |  |  |
| Sample 2-3 |  |  |  |  |  | Sample 2-3 |  |  |  |
| Sample 3-1 |  |  |  |  |  | Sample 3-1 |  |  |  |
| Sample 3-2 |  |  |  |  |  | Sample 3-2 |  |  |  |
| Sample 3-3 |  |  |  |  |  | Sample 3-3 |  |  |  |
| Sample 4-1 |  |  |  |  |  | Sample 4-1 |  |  |  |
| Sample 4-2 |  |  |  |  |  | Sample 4-2 |  |  |  |
| Sample 4-3 |  |  |  |  |  | Sample 4-3 |  |  |  |
| *starts on right bank looking downstream |  |  |  | ** transects are minimum 5 m apart |  |  |  |  |  |
| surface velocity (cm/s) |  | average |  |  |  | width m (bank): |  |  |  |
|  |  |  |  |  |  | width m (wetted): |  |  |  |
| algal velocity (cm/s) |  | average |  |  |  | depth (m) |  |  |  |
| precision check average velocity: date: Initials participants: |  |  |  |  |  |  |  |  |  |