Massachusetts Department of Environmental Protection Division of Watershed Management

STANDARD OPERATING PROCEDURE

Water Quality Monitoring In Streams Using Aquatic Macroinvertebrates

CN 039.2

December, 2003

| Prepared by: |  | Date: |  |
| --- | --- | --- | --- |
|  | ###### Bob Nuzzo, Biologist |  |  |
| Approved by: |  | Date: |  |
|  | ###### Arthur Johnson, Monitoring Coordinator |  |  |
| Approved by: |  | Date: |  |
|  | ###### Richard Chase, Quality Assurance Analyst |  |  |
| Approved by: |  | Date: |  |
|  | ###### Rick McVoy, Assessment Coordinator |  |  |
| Approved by: |  | Date: |  |
|  | ###### Dennis Dunn, Program Supervisor |  |  |

# List of Revisions

| # Revision Date | # Revision | # Pages #s |
| --- | --- | --- |
| # November 20, 2007 | # Added quality control requirements and procedures for determining percent sorting efficiency | # 12 |

# PURPOSE

The purpose of this document is to describe the materials, methods, and quality assurance/quality control (QA/QC) measures used by the Massachusetts Department of Environmental Protection’s Division of Watershed Management in conducting standard investigations of water quality in streams using aquatic macroinvertebrates.  It is expected that all routine biological monitoring in streams based on macroinvertebrates will follow these procedures, or that whenever deviations become desirable/necessary they will be detailed (and a rationale explained) in the Quality Assurance Project Plan (QAPP) and the biomonitoring report.

# FIELD METHODS

### Kick Sampling

For routine biomonitoring in wadeable streams with coarse substrates (mixed gravel and cobble, or larger) and riffle habitats, benthic invertebrates are collected by kicking-over the substrate materials (kick sampling) and capturing the dislodged organisms in a kick-net (500 m mesh) pressed firmly against the stream bottom immediately downstream of the material being kicked.  Where desirable, organisms may be dislodged by rubbing the substrate item.  Generally, a stream sampling site is a 100 m reach representing the best available habitat in a portion of the watershed where data are desired.  Ten kick samples from a square area with dimensions equal to the width of the net opening (i.e., 0.46 m x 0.46 m) are composited for a total area sampled of approximately two square meters.  In streams where the riffles within the reach are inadequate to allow for a two-square-meter composite, other productive habitats may be sampled by jabs into snags, or rubbing substrates.  In such cases, notes are recorded on the field sheets indicating the number of kicks or jabs in each habitat category that contributed to the composite sample.

Any large debris items (rocks, sticks, etc.) caught in the net are rinsed in the net and returned to the stream, once any macroinvertebrates clinging to them are removed.  The residue in the net is placed in a 2 l wide-mouth leak-proof Nalgene bottle with enough denatured 100% reagent alcohol (5% methanol, 5% isopropanol, 90% ethanol) added to cover the residue (in a volume roughly equal to the volume of sample materials in the bottle).  Sample bottles should not be filled more than about halfway with collected materials before starting a new bottle.  Alternatively, the sample bottle may be filled as required and packed into a cooler of ice for transport back to the laboratory.  This alternative is not recommended if processing is not possible within 48 h of collection.

A label (Figure 1) indicating the sample identification code (consisting of the watershed code and station number separated by a dash; e.g., 12-KC01), date, water body name, sampling location, and the collector is placed inside the sample container.  The date, sample identification code, and the words, “preservative: denatured 100% reagent alcohol,” if used, are marked on the outside of the container, as well.  Upon arrival at the taxonomy laboratory the sample is recorded in the Laboratory Log-in notebook (see “Sample Log-in” section) and refrigerated/iced (if not preserved) until processed.

Figure 1.  Example of label to be placed in containers with benthos samples.

12-KC01				11 August 1997

Kinderhook Creek dnst. fr. Brodie Mountain Road, Hancock, MA

coll. R. Nuzzo

### Rock Baskets

For stream biomonitoring where kick sampling is deemed inappropriate, rock baskets may be used to collect benthic macroinvertebrates.  Situations making rock basket samplers desirable may include: the need for certain statistical treatments of benthos data; waters too deep for kick sampling; or benthic substrates too fine for kick sampling at one or more sites to be evaluated and compared.

Baskets will contain a like amount (by weight) of “roofing stone” (not crushed rock) in the size range of 2.5 to 7.5 cm (one to three inches).  The baskets will be placed on the stream bottom in a riffle or run with a current velocity of between 15 and 76 cm/s (0.5 and 2.5 ft/s).  Because of this, the configuration of baskets within a sample site may not be in regular arrays or even in similar patterns at each site. Baskets will be left in-place and undisturbed for a period of six to eight weeks.

At the completion of the exposure interval each basket is retrieved by pressing a kick-net tightly against the streambed along the basket’s downstream edge.  The basket is then moved carefully into the net before lifting it through the water column.  Where the water is deep enough to make this procedure difficult or impossible, a cover designed to be dropped over the basket may be employed (e.g., Courtemanch 1984).  Where this procedure is anticipated, a basket made with 500 m netting on its bottom surface is used to prevent the loss of organisms through the bottom of the sampler as it is raised to the surface.

Vegetation or debris snagged on the outside of the recovered sampler is removed and the basket is placed in a large bucket or tub of water. The sampler is then opened, emptied into the bucket, rinsed inside the bucket until free of any organisms or adhering material, and set aside.  Each rock is similarly rinsed and set aside.  The material remaining in the bucket is then sieved through a #30 mesh (600 m) soil screen, transferred to a sample container, and preserved in denatured 100% reagent alcohol.  Alternatively, sieved site water may be added to the sample container and the sample stored on ice for transport back to the laboratory (not recommended if sample processing is unlikely to take place within 48 h of collection).  A label made out as described under “Kick Sampling” above (see Figure 1) is placed inside the sample container.  The date, sample identification code, and the words, “preservative: denatured reagent alcohol,” (if used) are marked on the outside of the container.  Upon arrival at the taxonomy laboratory the sample is recorded in the Laboratory Log-in notebook (see “Sample Log-in” section) and refrigerated/iced if not preserved.

### Multiplates (Hester-Dendy)

In deep rivers or where kick sampling is inappropriate and rock baskets are impractical, multiplate (modified Hester-Dendy) samplers may be used.  Samplers with round plates and spacers assembled to EPA specifications (Klemm, et al. 1990)  are used.  In deep water the multiplate sampler is tethered to an anchored float so that the sampler is suspended one meter below the surface.  In shallow waters the sampler may be mounted on a patio block or “four inch” cinder block; all samplers to be compared should be placed in comparable current velocities, preferably in the range 15 and 76 cm/s (0.5 and 2.5 ft/s).  Samplers are left in place and undisturbed for a period of six to eight weeks.

At the end of the exposure period each sampler is harvested by enveloping it with a 500m mesh net, plastic bag, or 2 l wide-mouth jar.  The sampler is placed in a 2 l wide-mouth jar with enough water to cover it completely, tightly capped, placed on ice, and transported to the laboratory.  Labeling procedures are the same as for kick samples and rock baskets (see “Kick Sampling” section and Figure 1).  Upon arrival at the taxonomy laboratory the sample is recorded in the Laboratory Log-in notebook (see “Sample Log-in” section) and placed in a refrigerator or iced.  Samplers are processed (as described in the “SAMPLE PROCESSING” section) as soon as possible following receipt at the laboratory (preferably within 48 h of collection) so that the sample can be preserved with ethanol.

### Habitat Assessments

Recording site characteristics and rating habitat qualities is important to the interpretation of biomonitoring data.  The habitat data and assessments help distinguish between pollution impacts and habitat limitations.  These data can also help identify causes of habitat destruction and loss.  Examples of the field sheets and assessment forms are in Appendix A.

Habitat assessment is accomplished by a visual-based method (Barbour, et al. 1999) conducted at the time of sample collection.  Each of ten habitat categories is rated from 0 (lowest, “poor”) to 20 (highest, “optimal”).  The ten categories are: Instream cover (fish); Epifaunal substrate (in sampled portions of reach); Embeddedness; Channel alteration; Sediment deposition; Velocity-depth combinations; Channel flow status; Bank vegetative protection (each bank scored separately for a maximum of 10 points each); Bank stability (each bank scored separately for a maximum of 10 points each); Riparian vegetated zone width (each bank scored separately for a maximum of 10 points each).  Descriptions of the considerations for scoring each habitat category can be found in Barbour, et al.  (1999).

# SAMPLE LOG-IN

When aquatic macroinvertebrate samples are brought into the laboratory they are recorded in the Laboratory Log-in notebook.  Each sample is assigned a sample identification number (BenSampID#) at the time of sample log-in.  The associated habitat data (on field sheets and assessment forms) will be tagged with a matching identification number (HabSampID#).  The BenSampID and HabSampID are the same number except when benthic samples are collected as replicates from the same sample location.  In such cases the HabSampID number and the BenSampID numbers start with the same seven digits, but each replicate is distinguished by adding a decimal point and a sequential number beginning with “1” (e.g., the BenSampID for three replicates might be numbered 2002014.1, 2002014.2, 2002014.3).

The log-in notebook contains the following fields, which are filled in for each sample:

- Collection date

- Log-in date

- Watershed name

- Field ID# (if different from BenSampID#)

- BenSampID#

- HabSampID#

- Location (name of waterbody, georeference, town, state)

- Collector

- Comments (e.g., for describing sample type or purpose, number of sample containers).

After the sample has been logged-in it must be refrigerated or kept on ice if not preserved with alcohol.

# SAMPLE PROCESSING

In general terms, sample processing involves separating macroinvertebrates from other materials in the sample.  For the purposes of this document “macroinvertebrate” is defined to include:

- all aquatic Annelida;

- all aquatic Mollusca;

- aquatic macro Crustacea (except as noted below);

- all aquatic Arachnida; and

- the aquatic life stages of Insecta except Hemiptera and adult Coleoptera other than Elmidae.

Those macroinvertebrates excluded from the above list are not used for one of three reasons: either there is insufficient ecological information on them to make them useful for biomonitoring, they are surface film dwellers, or they are capable of escaping the aquatic environment at will to avoid temporarily unfavorable conditions.  One further exception is crayfish (Class Crustacea, Family Cambaridae), which often are seen evacuating the immediate area as kick-sampling begins, and even swimming out of the kick-net.  Crayfish species are noted when present in the sample but are not counted toward total numbers.

While each of the different sample types below require slightly different sorting procedures, the methodologies share certain basic components.  All sample sorting is done with the aid of a dissecting microscope set on low power.  Only specimens with a sufficiently complete head capsule are removed and counted (i.e., no headless bodies or fragments are counted, though heads alone are).  Once the specimens are separated from the sample residue they then are sorted by taxonomic group (usually family), stored in glass screw-cap vials (typically 2-4 dram—8-20 ml), and preserved in denatured 70% ethanol until further taxonomic determination can be made.  Each vial is to contain a label (Figure 2) with the following information (in this order): sample identifier (either the field id. number—e.g., 12-KC01—or the BenSampID# for samples prior to 2003; for 2003 and later samples, the BenSampID#, with the field id. optional); sample collection date; and name of taxon contained in vial.  For convenience in reading the labels once inside the vials, all labels are inserted left-edge-first.  All remaining sample residue is returned to a sieve over a container for collection of the final rinse water (see section, “Hazardous Materials Handling, Storage, and Record-Keeping”).

12-KC01				11 August 1997

Philopotamidae

Figure 2.  Example of label to be placed in specimen vials after sorting.

The specimen vials, grouped by station and date, are placed in 1l Nalgene PMP jars, tightly capped, and set aside for further taxonomic determination.  The enclosure in these jars drastically reduces, if not eliminates, ethanol loss from the specimen vials during long-term storage.  A stick-on label is placed on the outside of the jar indicating: sample identifier; date; denatured 70% ethanol used as preservative.

### Hazardous Materials Handling, Storage, and Record-Keeping

Alcohol greater than 24% v/v is a Class 1B flammable liquid requiring handling and storage in a manner consistent with local fire codes.  The alcohol used for preserving benthic macroinvertebrate samples is typically either 100% reagent alcohol (5% methanol, 5% isopropanol, and 90% ethanol) or a 70% v/v solution of the reagent alcohol, and must be stored in the lab inside flammables cabinets.  When processing samples containing alcohol preservative, the used alcohol (typically 1l per sample bottle) must be collected because of its hazardous waste status (310 CMR 30.122).  It can then be diluted to a concentration of less than 24%, as allowed by DWM’s Very Small Quantity Generator (VSQG) status.  This can be accomplished by adding enough water to bring each liter of collected alcohol up to a volume of four gallons.  This diluted waste then can be disposed of by pouring it down the drain.

This treatment of the waste is only permitted so long as the DWM laboratories continue to qualify as VSQG status.  This means that the total generation of hazardous wastes from the DWM laboratories (instrumentation, chlorophylls, microscopy) cannot exceed 27 gallons (102 liters) or 100 kg per month. Therefore, it is very important to keep on-going records of wastes that are generated.  A form (example in Appendix B) will be maintained in the microscopy lab (as well as the other two labs) to keep a daily record of each sample that is processed and the total amount of alcohol collected before dilution and disposal.  These records from all three labs will be monitored to make sure the total waste generation from DWM lab operations is not exceeding the monthly limits, or to evaluate the need to reassess DWM’s generator status.  The forms will be collected at the end of each month and kept in a file for at least three years.

Samples that have been collected from areas where environmental data or best professional judgement suggest possible contamination with hazardous materials will be processed the same as other samples with the following exceptions:

- Alcohol used to preserve the sample, as well as rinses of the sample, will be collected and disposed of as hazardous waste.

- Solid materials from the sample likewise will be retained and disposed of as hazardous waste.

The decision to handle sample wastes as hazardous will be based on the procedure shown in Figure 3.  Any resultant hazardous wastes will be stored in containers with the appropriate hazardous waste labeling, and the containers will be stored in approved flammable-liquid safety cabinets until transported to Wall Experiment Station (Lawrence, MA) for disposal.  Amounts accumulated before transport for disposal shall not exceed quantities allowed for self transport under regulations for VSQGs.

At or downstream from known source of hazardous contaminants?

Do sediment data exist?

NO

Do data support HW status?

Yes

NO

Yes

Dispose of as HW

Dispose of as HW

Dispose of as “regular” trash

Coarse grain

Fine grain

Yes

No

Figure 3.  Decision tree for disposal of benthic samples.

### Rapid Bioassessment Protocols

Routine benthic macroinvertebrate samples for biological monitoring are analyzed using Rapid Bioassessment Protocols based on Plafkin, et al. (1989).  Samples to be analyzed by protocol II (RBP II) or III (RBP III) may be collected by the kick technique or by rock basket samplers and are processed in the laboratory to obtain 100-organism subsamples.  Each sample is emptied onto a 600 m mesh sieve held over a waste collection vessel (WCV) for collection of the alcohol preservative (see “Hazardous Materials Handling, Storage, and Record-Keeping” section).  Three water rinses of the sample bottle are used to assure complete transfer of the sample onto the sieve.  The sample is then allowed to drain on the sieve for one minute before proceeding with the following steps:

- The sample is homogenized and spread evenly across pans with inside dimensions of 30.5 cm by 30.5 cm (12 in by 12 in) and with marks at the edges for grids 6 cm by 6 cm.  The sample is distributed (with enough water added to aid in distributing the material evenly) among a sufficient number of pans (usually four) to reduce the density of organisms per grid to a workable number.  Each pan used is assigned a letter at random such that pan “A” contains grids 1-25, “B” contains grids 26-50, etc.

- A set of random numbers is generated in the range dictated by the number of pans used.  For example, when four pans are used pan labels will be “A” through “D” and valid random numbers will be in the range of 1 to 100, inclusive; for seven trays pan labels will be “A” through “G,” and valid numbers will be 1 to 175, inclusive.

- Materials from the identified grids (in the order the grids were generated at random) are scooped into a petri dish divided into quarters.

- One of the petri dish quadrants is selected at random and all organisms within it are removed and become part of the subsample.

- The unused material from the other quadrants of the petri dish are emptied onto the sieve over the WCV with the spent alcohol preservative.

- Material from the next randomly generated grid is then transferred to the petri dish and all organisms within a randomly selected quadrant are again removed to add to the subsample.

- Steps 5 and 6 are repeated until the target subsample size of approximately 100 is reached.

Because the subsampling procedure cannot assure that subsample size will be exactly 100, the subsample will be considered acceptable if it contains no fewer than 90 organisms and the spread between the highest and lowest count among subsamples being compared is not greater than 10% of the average of these extremes.  The total number of grids used to reach the target is recorded in the bench notebook (see “Sample Processing and Taxonomy” in the “Quality Control” section).  Since samples will no longer be archived (2003 and later) an additional 100-200 organisms must be picked at random from the sample and kept separate from the “original” 100.  This allows for a randomized repicking of the sample should it be discovered that the final count in the subsample is less than 90, or that there is some other problem with the subsample.  These should be labeled as “subsample extras.”

The unpicked portion of the sample should be examined for specimens of taxa not included in the subsample.  Good-condition specimens of taxa that also appeared in the subsample may be retained, as well, to aid in taxonomic determinations.  These specimens may be kept unsorted in a vial and the words “large/rare/voucher specimens” included along with the usual label information.

All remaining sample residues are returned to the sieve, rinsing them with water from the sorting trays to the sieve, and collecting the rinse water in the WCV with the spent alcohol preservative.  The material collected on the sieve can then be discarded (after draining for a few minutes) in an appropriate manner (see “Hazardous Materials Handling, Storage, and Record-Keeping” section).  If the sample wastes are not deemed “hazardous” the collected liquids must be brought to a total volume of four gallons (per liter of alcohol preservative used in the sample) by adding water (see “Hazardous Materials Handling, Storage, and Record-Keeping” section) before being poured down the drain.  If the wastes are regarded as hazardous, the contents of the sieve and the WCV will both be retained (in separate containers) for proper hazardous waste disposal.

### Rock Baskets

Samples collected from rock baskets may be processed either as an RBP sample (as described above) or as a standardized quantitative unit.  If the sample is to be treated quantitatively then all qualifying macroinvertebrates (as defined at the beginning of the “SAMPLE PROCESSING” section) will be sorted from the sample.  Subsampling may be desirable if called for by the statistical design of the study or if densities of organisms are extreme.  Any waste alcohol will be collected and treated as described in the “Hazardous Materials Handling, Storage, and Record-Keeping” and “Rapid Bioassessment Protocols” sections).

### Multiplates (Hester-Dendy)

Within 48 h of field retrieval the refrigerated multiplate samples are processed.  Processing begins by emptying the sampler and surrounding water into a large pan.  The sample container is rinsed three times with small volumes of water—and added to the tray—to assure transfer of all material from the sample.  Processing continues as follows:

- disassemble sampler;

- rinse into water in the tray or pick organisms off of plates, spacers, and hardware;

- scrape off any material still adhering to plates, spacers, or hardware (set aside sampler parts when clean);

- pour water with sample from the tray through a #30 mesh sieve and rinse tray, sending the rinses through the sieve, also;

- transfer all of the material collected on the sieve to a jar and preserve with denatured 70% ethanol if sample will not be processed immediately;

- label jar in the same manner as the original sample container.

The sorting procedures applied to multiplate samples will be dictated by the study design and data requirements.  In general it is expected that all qualifying macroinvertebrates (as defined at the beginning of the “SAMPLE PROCESSING” section) will be sorted from the sample.  Extreme densities of organisms, however, may make subsampling desirable.

# TAXONOMY

### General

Most invertebrate taxonomic determinations are accomplished by using keys to genus or species and examining specimens under a dissecting microscope with high quality optics.  The most commonly used keys are:

- Annelida—Klemm 1985; Kathman and Brinkhurst 1998

- Mollusca—Jokinen 1983; Smith 1986; Smith 1987

- Crustacea—Smith 1988

- Insecta—Merritt and Cummins 1996; Peckarsky, et al. 1990

Proper determination of the identity of aquatic macroinvertebrates at the genus or species level is dependent on several factors:

- Maturity—the specimen must be mature enough to exhibit representative diagnostic characteristics;

- Condition—the specimen must be sufficiently undamaged to possess the critical diagnostic structures;

- Keys—taxonomic keys including relevant genera and species for the New England geographic region must be available.

If any of these three factors are not satisfactory then taxonomy may have to rest at the family or order level.

After being identified to genus/species (where possible), specimens are placed in glass vials and preserved with denatured 70% ethanol.  Each vial has a label inserted (Figure 4), which contains the following information (in this order): sample identifier (either the field id. number—e.g., 12-KC01—or the BenSampID# for samples collected prior to 2003; for 2003 and later samples, the BenSampID#, with the field id. optional); sample collection date; water body sampled; location descriptors (e.g., dnst. fr. Brodie Mountain Road); names of municipality and state; name of taxon contained in vial; and "det." plus the first initial and last name of the person who identified the specimen(s).  For convenience in reading the labels once inside the vials, the labels are inserted left-edge-first.  The specimen vials, grouped by station and date, are placed in 1l Nalgene PMP jars and tightly capped.  A stick-on label is placed on the outside of the jar indicating: sample identifier; date; denatured 70% ethanol used as preservative.

Figure 4.  Example of label to be placed in specimen vials after specimens have been identified to lowest practical taxonomic level.

12-KC01				11 August 1997

Kinderhook Creek dnst. fr. Brodie Mountain Road, Hancock, MA

Chimarra sp.

det. R. Nuzzo

### Oligochaeta (aquatic worms) and Chironomidae (midges)

Because taxonomic determination of specimens of the Oligochaeta and Chironomidae requires examination under a compound microscope, these specimens must be mounted on microscope slides in permanent or semi-permanent mounting medium.  The following mounting procedure is used:

- Place a 3” by 1” microscope slide on the work surface with the label end of the slide to the left;

- place three worms or three midges (do not mix midges and worms under the same cover slip, or even on the same slide) side by side with heads toward the top;

- place a sufficient amount of CMC-10 around the specimens to completely cover and surround them;

- for worms, uncoil specimen with forceps and leave resting on its side (if possible);

- for midges, separate head from body using forceps and an insect pin, orient the body on its side, and orient the head with its ventral surface up;

- place a #1 cover slip over the three specimens and apply gentle pressure to force out air bubbles;

- for midges apply enough gentle pressure to coax the mandibles to spread apart;

- repeat steps #2 through #7 for a second mount on the slide (if desired);

- turn slide around so that the label end is now to the right and fill in label information from the sample label;

- ring edge of cover slips with CMC-10 (or a non-aqueous sealant);

- allow mounts to dry in a horizontal position for at least one week before placing in slide storage boxes.

By the time the mounts have dried the specimens have usually cleared sufficiently for examination and taxonomic determination.  The most frequently used keys for these groups are:

- Oligochaeta—Kathman and Brinkhurst 1998

- Chironomidae--Bode 1990, Coffman and Ferrington 1996, Epler 2001, Wiederholm 1983, Wiederholm 1986

### Data Analysis

All data related to aquatic macroinvertebrate samples and the associated habitats are entered into a database called MABen.  The database and data management procedures are described in a separate document (Nuzzo 2002).

The data generated from the 100 organism subsamples are analyzed by a multimetric approach similar to the one described by Plafkin, et al. (1989), including calculation of taxa richness, Hilsenhoff Biotic Index (HBI; or Hilsenhoff’s family biotic index—FBI—when applied to RBP II work), EPT index (taxonomic richness among the orders Ephemeroptera, Plecoptera, and Trichoptera), ratio of EPT abundances to Chironomidae abundances, ratio of scrapers to filtering collectors, percent contribution of dominant family, and similarity.  This suite of metrics and scoring ranges is subject to review and may be modified as deemed appropriate.

Richness—Under circumstances when it becomes necessary to abandon determination beyond a particular taxonomic level (e.g., family) that level will not be counted toward the total species richness if there is at least one other determination that falls into the same category at that level.  For instance, on a species list that includes “undetermined Ephemeroptera,” “undetermined Baetidae,” and “Drunella sp.” the contribution of these toward the total richness would be two.  The undetermined Ephemeroptera would not be counted toward the richness because it is the mayfly order, which includes the family Baetidae and the genus Drunella.  The family Baetidae and the genus Drunella are both counted because the family Baetidae does not encompass the genus Drunella sp. (family Ephemerellidae).

Metrics and scoring—Tables 1 and 2 show the routine metrics used for the RBP III analyses and the scoring ranges, as well as the final scoring for impact determination.  Tables 3 and 4 show the scoring ranges for RBP II analysis.

Table 1.  RBP III metrics and scoring ranges.

| ##### METRIC | ##### SCORING RANGES |  |  |  |
| --- | --- | --- | --- | --- |
|  | 6 | 4 | 2 | 0 |
| Taxa Richness | >80% | 60-80% | 40-59% | <40% |
| EPT | >90% | 80-90% | 70-79% | <70% |
| EPT/Chironomidae (abundance ratio) | >75% | 50-75% | 25-49% | <25% |
| HBI (modified) | >85% | 70-85% | 50-69% | <50% |
| Scraper/Filtering collector Ratio | >50% | 35-50% | 20-34% | <20% |
| % Contribution of Dominant Taxon | <20% | 20-29% | 30-40% | >40% |
| Similarity Index: Community Loss % Similarity % Reference Affinity | <0.5 >70% >64% | 0.5-1.5 50-70% 50-64% | 1.6-4.0 30-49% 35-49% | >4.0 <30% <35% |

Table 2.  Scoring ranges for RBP III determination of level of biological impact.

| ##### % Comp to Reference | ##### Impact category |
| --- | --- |
| >83% | Not Impacted |
| 54-79% | Slight Impact |
| 21-50% | Moderate Impact |
| <17% | Severe Impact |

Table 3.  RBP II metrics and scoring ranges.

| ##### METRIC | ##### SCORING RANGES |  |  |
| --- | --- | --- | --- |
|  | 6 | 3 | 0 |
| Taxa Richness | >80% | 40-80% | <40% |
| EPT | >90% | 70-90% | <70% |
| EPT/Chironomidae (abundance ratio) | >75% | 25-75% | <25% |
| FBI (modified) | >85% | 50-85% | <50% |
| Scraper/Filtering collector Ratio | >50% | 25-50% | <25% |
| % Contribution of Dominant Taxon | <30% | 30-50% | >50% |
| Similarity Index: Community Loss % Similarity % Reference Affinity | <0.5 >70% >64% | 0.5-4.0 30-70% 35-64% | >4.0 <30% <35% |

Table 4.  Scoring ranges for RBP II determination of level of biological impact.

| ##### % Comp to Reference | ##### Impact category |
| --- | --- |
| >79% | Not Impacted |
| 29-72% | Moderate Impact |
| <21% | Severe Impact |

# QUALITY CONTROL

### Sample Collection

To assess the consistency of the sampling effort, collection of a duplicate sample is performed at 10% of the stations sampled for each watershed.  Two samples are collected “side by side” by the same person—a second kick sample (i.e., the duplicate) is taken adjacent to (where different assessment results are not expected due to the apparent absence of additional stressors) the original kick at each of the ten kicks conducted in a given 100 m sample reach. Duplicate samples are composited in a similar manner as the original sample; yet, they are preserved in a separate sample bottle marked “duplicate” and with all other information regarding station location remaining the same. Duplicate samples will be used in the evaluation of precision of the benthos data.

After sampling is completed at a given site, all nets, sieves, pans, etc. that have come in contact with the sample are rinsed thoroughly, examined carefully, and picked free of organisms or adhering debris. For good measure, these materials are again examined before use at the next sampling location.  These precautions help assure “cross-contamination” (specimen carry-over) between samples.

Habitat Assessments performed as part of the biosurvey entail multiple observers (at least both DWM benthic biologists, and ideally a third person—all observers shall be trained in DWM’s Habitat Assessment procedures) performing the Habitat Assessment at each biomonitoring station. A standardized Habitat Assessment Field Scoring Sheet will be completed at all biomonitoring stations. Disagreement in habitat parameter scoring will be discussed and resolved before the Habitat Assessment can be considered complete.

Information on samples received at the taxonomy laboratory are recorded in the  Laboratory Log-in notebook (see “Sample Log-in” section).  Progress of each sample is recorded on the “Sample Tracking Sheets.”  The tracking sheets are updated as each step is completed (sorting, mounting of midges and worms, taxonomy) for a particular sample.

### Sample Processing and Taxonomy

All sample information is recorded in a taxonomist’s bench notebook on notebook pages (usually two, facing) dedicated to that particular sample, with only one sample to a page.  The information recorded on the first page for the sample includes:

- all sample label information;

- the number of grids used to reach the target number for the subsample;

- the identity (usually at family level) and count of the organisms in the subsample;

- a list of macroinvertebrates observed in the sample but not in the final subsample.

Quality Control checks of sample sorting efficiency should be performed on 10% of all samples. For the designated QC sample, the “picker” places picked-over residue (pickate) in a separate dish for QC check on 1st 100 organisms (the primary subsample) and QC check on the 200 "extras."  When the sample is completely picked the QC reviewer sorts through the pickate looking for missed specimens.  Any that are discovered are counted and retained in a vial with complete label information and the words, "QC on 1st 100" and "QC on 200 extras," respectively.  Percent sorting efficiencies (PSE) are then calculated separately by the formula:

(n0 / (n0 + nr)) * 100

where n0 = original # of specimens found by sorter, and nr = # of recoveries (i.e., the number found by the reviewer)

The suggested management quality objective (MQO) is 90%.  A failure to meet the 90% SE on any of the 10% checked would require a repicking of the subsample with the addition of the “discovered” specimens. To repick a subsample, redistribute the original 100 or 200-organism subsample along with the “discovered” organisms in a quartered petri dish, then from randomly selected quadrants count organisms in each quadrant until the target sample (either 100 or 200 organisms, depending on which subsample did not meet MQO) is achieved again.

Once all of the samples are processed, identification of the sorted specimens to genus/species can take  place.  Determinations of unmounted specimens are usually recorded on the second page of the bench notebook dedicated to the corresponding sample.  The determinations of mounted specimens are recorded directly onto a bench sheet designed for associating specimen identifications with specific mounts (“Slide-mount Bench Sheet,” Appendix C).  When all taxonomic determinations for a sample are completed the taxa names and counts will be transcribed from the bench notebook and the “Slide-mount Bench Sheet” to the “Aquatic Macroinvertebrate Data Sheet” (Appendix C),.

When identifications for a sample or set of samples are completed and the information has been transcribed to the “Aquatic Macroinvertebrate Data Sheets,” a taxonomist not responsible for the original identifications (the reviewer):

- examines the sheets for errors in transcription from the bench notebook, count totals, and spelling of taxa names;

- brings any detected errors to the attention of the taxonomist for correction;

- spot checks the identity of taxa listed;

- works with the taxonomist toward determining the correct identity of a taxon when there is disagreement;

- seeks assistance from an authority on the taxonomic group if the identity of the organism(s) cannot be resolved;

- places his initials next to checked taxa when there is concurrence;

- initials and dates the data sheet when the quality control check is completed satisfactorily.

Once this check is completed the “Aquatic Macroinvertebrate Data Sheets” can be used for entry of data into the database and/or entry into a spreadsheet to create tables listing taxa present and counts at each station.  After the data are entered, a reviewer (not the data entry person) will check for transcription errors by comparing the entered data against the “Aquatic Macroinvertebrate Data Sheets.”  Once any errors are corrected data tables can be printed out for release to data users.

### Data Analysis

When the aquatic macroinvertebrate data are used in calculations a reviewer (not the analyst performing the calculations) checks the calculations for errors.  This may involve recalculating some statistics, indices, etc., and comparing results; and/or the reviewer may examine formulae used and judge their suitability to the desired analysis.  For RBP analyses, metrics are run from the MABen database.  The RBP score, and subsequent placement into an impact category are calculated by the analyst.  The reviewer spot-checks the metric outputs (particularly those based on richness) as well as checking the scoring, totaling of scores, and the determination of the impact category.

Precision, a measure of mutual agreement among individual measurements or enumerated values of the same property of a sample and usually expressed as a standard deviation in absolute or relative terms, will be compared using raw benthos data and metric values. If metric values and resulting scoring are significantly different (i.e., beyond an acceptable Relative Percent Difference) between the original and duplicate samples, the investigators will attempt to determine the cause of the discrepancy. Guidance regarding the calculation of Precision, including Relative Percent Difference (RPD) calculations and recommendations, can be found in US EPA (1995).

# QUALITY ASSURANCE

From time to time specimens may be sent to authorities for particular taxonomic groups.  Specimens sent out for taxonomy validations will be recorded in the “Taxonomy Validation Notebook” showing label information, and date sent out.  When the specimens are returned, the date received and the finding are also recorded in the notebook.  The label will be modified to show corrections to the identification, if any, and to add the abbreviation “val.” plus the first initial and last name of the authority who validated the identification. Such specimens will be placed in the reference collection.

# LITERATURE CITED

Barbour, M.T., J. Gerritsen, B.D. Snyder, and J.B. Stribling.  1999.  Rapid bioassessment protocols for use in wadeable streams and rivers: Periphyton, benthic macroinvertebrates, and fish, second edition.  U.S. Environmental Protection Agency.  Washington, DC.  EPA 841-B-99-002.

Bode, R.W.  1990.  Chironomidae.  In  B.L. Peckarsky, P.R. Fraissinet, M.A. Penton, and D.J. Conklin, Jr.  Freshwater macroinvertebrates of northeastern North America.  Comstock Publishing Assoc.  Ithaca, NY.  p. 225-267.

Coffman, W.P., and L.C. Ferrington.  1996.  Chironomidae.  In  R.W. Merritt and K.W. Cummins (eds.).  An introduction to the aquatic insects of North America.  Kendall/Hunt Publishing Co.  Dubuque, IA.  p. 635-754.

Courtemanch, D.L.  1984.  A closing artificial substrate device for sampling benthic macroinvertebrates in deep rivers.  Freshwater Invertebrate Biol. 3(3):143-146.

Epler, J.H.  2001.  Identification Manual for the larval Chironomidae (Diptera) of North and South Carolina.  A guide to the taxonomy of the midges of the southeastern United States, including Florida. Special Publication SJ2001-SP13.  North Carolina Department of Environment and Natural Resources, Raleigh, NC, and St. Johns River Water Management District, Palatka, FL. 526 p.

Jokinen, E.H.  1983.  The freshwater snails of Connecticut.  State Geological and Natural History Survey of Connecticut.  Bulletin 109.  83 p.

Kathman, R.D. and R.O. Brinkhurst.  1998.  Guide to the freshwater oligochaetes of North America.  Aquatic Resources Center.  College Grove, TN.  264 p.

Klemm, D.J. (ed.).  1985.  A guide to the freshwater Annelida (Polychaeta, naidid and tubificid Oligochaeta, and Hirudinea) of North America.  Kendall/Hunt Publishing Co.  Dubuque, IA.  198 p.

Klemm, D.J., P.A. Lewis, F. Fulk, and J.M. Lazorchak.  1990.  Macroinvertebrate field and laboratory methods for evaluating the biological integrity of surface waters.  U.S. Environmental Protection Agency, Cincinnati, OH.  EPA/600/4-90/030.

Merritt, R.W. and K.W. Cummins (eds.).  1996.  An introduction to the aquatic insects of North America.  Kendall/Hunt Publishing Co.  Dubuque, IA.  862 p.

Nuzzo, R.M. (ed.) 2002.  Benthic macroinvertebrate database descriptions and procedures (rev. 1.1).  Massachusetts DEP, Division of Watershed Management.  Worcester, MA.  20 p.

Peckarsky, P.R. Fraissinet, M.A. Penton, and D.J. Conklin, Jr.  1990.  Freshwater macroinvertebrates of northeastern North America.  Comstock Publishing Assoc.  Ithaca, NY.  442 p.

Plafkin, J.L., M.T. Barbour, K.D. Porter, S.K. Gross, and R.M. Hughes.  1989.  Rapid bioassessment protocols for use in streams and rivers: Benthic macroinvertebrates and fish.  U.S. Environmental Protection Agency, Washington, D.C.  EPA/444/4-89-001.

Smith, D.G.  1986.  Keys to the freshwater macroinvertebrates of Massachusetts (No. 1):  Mollusca Pelecypoda (clams, mussels).  Massachusetts Division of Water Pollution Control.  Westborough, MA.  52 p.

Smith, D.G.  1987.  Keys to the freshwater macroinvertebrates of Massachusetts (No. 2):  Mollusca Mesogastropoda (operculate snails).  Massachusetts Division of Water Pollution Control.  Westborough, MA.  34 p.

Smith, D.G.  1988.  Keys to the freshwater macroinvertebrates of Massachusetts (No. 3):  Crustacea Malacostraca (crayfish, isopods, amphipods).  Massachusetts Division of Water Pollution Control.  Westborough, MA.  58 p.

USEPA. 1995. Generic Quality Assurance Project Plan Guidance for Programs Using Community Level Biological Assessment in Wadeable Streams and Rivers. EPA 841-B-95-004, July 1995.

Wiederholm, T. (ed.).  1983.  Chironomidae of the Holarctic region: Keys and diagnoses (Part 1. larvae).  Ent. Scand. Suppl. 19.  457 p.

Wiederholm, T.  1986.  Chironomidae of the Holarctic region: Keys and diagnoses (Part 2. pupae). Ent. Scand. Suppl. 28.  482 p.

Appendix a

Habitat Field Sheets and Assessment Forms

| RECONNAISSANCE | HABITAT | INVERTEBRATE | FISH | ALGAE | WATER QUALITY | FLOW |
| --- | --- | --- | --- | --- | --- | --- |

STREAM CHARACTERIZATION

|  Subsystem Classification |  Stream Type |
| --- | --- |
|  Tidal |  Coldwater |
| Lower Perennial |  Warmwater |
|  Upper Perennial |  |
|  Intermittent |  |

RIPARIAN ZONE INSTREAM FEATURES

| Surrounding Land Use | Local Water Erosion | Est. Stream Width m |
| --- | --- | --- |
| % Forest |  None | Est. Stream Depth |
| % Field/Pasture |  Slight | Riffle m |
| % Agriculture |  Moderate | Run m |
| % Residential |  Heavy | Pool m |
| % Commercial | High Water Mark | Velocity |
| % Industrial | Dam present  Yes  No | m/s @ deployment |
| % Other | Channelized  Yes  No | m/s @ recovery |
| Canopy Cover % |  |  |
|  |  |  Est. Fish Reach Length m |
| Local Watershed NPS Pollution |  |  |
|  No evidence |  |  |
|  some potential sources: |  |  |
|  Obvious sources: |  |  |

| SEDIMENT/SUBSTRATE |  |  |  |
| --- | --- | --- | --- |
|  Odors |  Deposits |  Oils |  Are undersides of rocks |
|  None/normal |  None |  None | (not deeply embedded) black? |
|  Anaerobic |  Paper fiber |  Slight |  Yes |
|  Chemical |  Sand |  Moderate |  No |
|  Petroleum |  Sawdust |  Profuse |  |
|  Sewage |  Sludge |  Relict Shells |  |
|  Other |  Other |  Other |  |

| INORGANIC SUBSTRATE COMPONENTS |  | % Composition in Sampling: |  | WATER CHARACTER |  |
| --- | --- | --- | --- | --- | --- |
| Substrate | Size (Minshall 1984) | Area | Reach |  Water Odors |  Water Surface Oils |
| Bedrock |  | % | % |  Normal/None |  None |
| Boulder | > 256 mm (10 in) | % | % |  Chemical |  Flecks |
| Cobble | 64-256 mm (2.5-10 in) | % | % |  Fish |  Globs |
| Pebble | 16-64 mm (0.6-2.5 in) | % | % |  Petroleum |  Slick/Sheen |
| Gravel | 2-16 mm (0.1-0.6 in) | % | % |  Sewage |  |
| Sand | 0.06-2 mm (gritty) | % | % |  Other |  |
| Silt | 0.004-0.06 mm | % | % |  |  |
| Clay | < 0.004 mm (slick) | % | % |  Water Color |  Turbidity (if not measured) |
|  |  |  |  |  |  Clear |
|  |  |  |  |  |  Slight |
| Marl or travertine? |  present |  |  |  |  Moderate |
|  |  |  |  |  |  Severe (opaque) |

| ORGANIC SUBSTRATE COMPONENTS |  |  | HabSamp ID#: |  |
| --- | --- | --- | --- | --- |
| Substrate | Characteristic | % Comp. in sample reach |  |  |
| Detritus | Sticks, wood, coarse plant materiall (CPOM) | % | BenSamp ID#(s): |  |
| Muck-Mud | Black, very fine organics (FPOM) | % |  |  |

|  Weather Conditions: |  |  | How were samples collected? |
| --- | --- | --- | --- |
|  Now |  Rain/sleet/snow |  cloud cover % |  wading |
|  Antecedent Period Ppt. Amount (data from http://www.erh.noaa.gov/box/dailystns.shtml) |  |  |  from bank |
| 24 h— |  |  |  from boat |
| 7 d— |  |  |  |

|  Riparian vegetation (18 m buffer) Record dominant species present and % area covered |  Aquatic vegetation (coverage within reach: %) Record dominant spp. and % composition (should = 100%) |
| --- | --- |
| % trees | % rooted emergent |
|  | % rooted submergent |
| % shrubs& vines | % rooted floating |
|  | % free floating |
| % herbaceous | % mosses |

 Algae (coverage within reach: 	%)		 Number of algae samples taken:

| Forms | Color |  |  |  |  | Substrate |  |  |  |  |  | Microhabitat |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Green | Brown |  | Other |  | Rock |  | Wood | Plant |  | Other | Pool |  | Riffle | Other |  |
|  filamentous |  |  |  |  |  |  |  |  |  |  |  | % |  | % |  |  |
|  flock |  |  |  |  |  |  |  |  |  |  |  | % |  | % |  |  |
|  thin film |  |  |  |  |  |  |  |  |  |  |  | % |  | % |  |  |
|  other |  |  |  |  |  |  |  |  |  |  |  | % |  | % |  |  |
|  |  |  | Riffles |  | Snags |  | Stream Banks |  |  | Submerged Macrophytes |  |  | Other |  |  |  |
|  Number of jabs/kicks in ea. habitat type: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  habitat types by % of sample reach |  |  | % |  | % |  | % |  |  | % |  |  | % |  |  |  |

 Site sketch

Investigator(s): 						River Basin:

Stream Name: 						Saris#:

Describe site location:

Scoring for wadable riffle/run dominated streams (moderate to high gradient) with  velocities approx. 30 cm/s or greater.

| Habitat parameter | Category |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Optimal |  |  |  |  | Suboptimal |  |  |  |  | Marginal |  |  |  |  | Poor |  |  |  |  |  |
| 1. Instream cover (fish) | A mix of submerged logs, undercut banks, rubble, or other stable habitat in > 50% of the sample area. |  |  |  |  | 30-50% of area with a mix of stable habitat; adequate habitat for maintenance of populations. |  |  |  |  | 10-30% of area with a mix of stable habitat; habitat availability less than desirable; substrate frequently disturbed or removed. |  |  |  |  | < 10% of area with a mix of stable habitat; lack of habitat is obvious; substrate unstable of lacking. |  |  |  |  |  |
| SCORE | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| 2. Epifaunal Substrate (in sampled area only) | Well-developed riffle and run; riffle is as wide as stream and length extends two times the width of stream; abundance of cobble (Boulders prevalent in headwater streams). |  |  |  |  | Riffle is as wide as stream but length is < 2X width; abundance of cobble; boulders and gravel common. |  |  |  |  | Run area may be lacking; riffle not as wide as stream and its length < 2X the stream width; gravel or bedrock prevalent; some cobble present. |  |  |  |  | Riffles or runs virtually nonexistent; bedrock prevalent; cobble lacking. |  |  |  |  |  |
| SCORE | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| 3. Embeddedness (riffles/runs) | Gravel, cobble, and boulder particles are 0-25% surrounded by fine sediment. |  |  |  |  | Gravel, cobble, and boulder particles are 25-50% surrounded by fine sediment. |  |  |  |  | Gravel, cobble, and boulder particles are 50-75% surrounded by fine sediment. |  |  |  |  | Gravel, cobble, and boulder particles are > 75% surrounded by fine sediment. |  |  |  |  |  |
| SCORE | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| 4. Channel Alteration | Channelization or dredging absent or minimal; stream with normal pattern. |  |  |  |  | Some channelization present, usually in areas of bridge abutments; evidence of past channelization or dredging may be present but not recent (> 20 y). |  |  |  |  | New embankments present on both banks; and 40-80% of stream reach channelized and disrupted. |  |  |  |  | Banks shored with gabion or cement; over 80% of the stream reach channelized and disrupted. |  |  |  |  |  |
| SCORE | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| 5. Sediment Deposition | Little or no enlargement of islands or point bars and < 5% of the bottom affected by sediment deposition. |  |  |  |  | Some new increase in bar formation, mostly from gravel, sand, or fine sediment; 5-30% of the bottom affected; slight deposition in pools. |  |  |  |  | Moderate deposition of new gravel, sand, or fine sediment on old and new bars; 30-50% of the bottom affected; sediment deposits at obstructions, constrictions, and bends; moderate deposition in pools prevalent. |  |  |  |  | Heavy deposits of fine material, increased bar development; > 50% of the bottom changing frequently; pools almost absent due to substantial sediment deposition. |  |  |  |  |  |
| SCORE | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

| Habitat parameter | Category |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Optimal |  |  |  |  | Suboptimal |  |  |  |  | Marginal |  |  |  |  | Poor |  |  |  |  |  |
| 6. Velocity-Depth Combinations 1. slow deep 2. fast deep 3. slow shallow 4. fast shallow | All 4 velocity/depth patterns present. Occurrence of riffles relatively frequent; ratio of distance between riffles divided by width of the stream < 7:1 (generally 5-7); variety of habitat is key. In streams where riffles are continuous, location of boulders or other large, natural obstructions is important. |  |  |  |  | Only 3 of 4 velocity/depth patterns present. Occurrence of riffles infrequent; distance between riffles divided by the width of the stream is between 7 to 15. |  |  |  |  | Only 2 velocity/depth patterns present; usually lacking deep areas. Occasional riffle or bend; bottom contours provide some habitat; distance between riffles divided by the width of the stream is between 15 to 25. |  |  |  |  | Dominated by one velocity/depth pattern. Generally all flat water or shallow riffles; poor habitat; distance between riffles divided by the width of the stream is a ratio of > 25. |  |  |  |  |  |
| SCORE | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| 7. Channel Flow Status | Water reaches the base of both banks, and minimal amount of channel substrate is exposed. |  |  |  |  | Water fills > 75% of the available channel; or < 25% of channel substrate is exposed. |  |  |  |  | Water fills 25-75% of the available channel, and/or riffle substrates are mostly exposed.. |  |  |  |  | Very little water in channel and mostly present as standing pools. |  |  |  |  |  |
| SCORE | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| 8. Bank Vegetative Protection (score each bank) Note: Determine left or right side by facing downstream | More than 90% of the streambank surfaces covered by naturally occuring vegetation, including trees, understory shrubs, or nonwoody macrophytes; vegetative disruption through grazing or mowing minimal or not evident; almost all plants allowed to grow naturally. |  |  |  |  | 70-90% of the streambank surfaces covered by naturally occuring vegetation, but one class of plants is not well-represented; disruption evident but not affecting full plant growth potential to any great extent; more than one half of the potential plant stubble height remaining. |  |  |  |  | 50-70% of the streambank surfaces covered by vegetation; disruption obvious; patches of bare soil or closely cropped vegetation common; less than one-half of the potential plant stubble height remaining. |  |  |  |  | Less than 50% of the streambank surfaces covered by vegetation; disruption of streambank vegetation I very high; vegetation has been removed to 5 cm or less in average stubble height. |  |  |  |  |  |
| SCORE Left bank |  |  |  | 10 | 9 | 8 |  | 7 | 6 |  | 5 |  | 4 | 3 |  | 2 |  | 1 |  | 0 |  |
| SCORE Right bank |  |  |  | 10 | 9 | 8 |  | 7 | 6 |  | 5 |  | 4 | 3 |  | 2 |  | 1 |  | 0 |  |
| 9. Bank Stability (score each bank) | Banks stable; evidence of erosion of bank failure absent or minimal; little potential for future problems. < 5% of bank affected. |  |  |  |  | Moderately stable; infrequent. Small areas of erosion mostly healed over. 5-30% of bank in reach has areas of erosion. |  |  |  |  | Moderately unstable; 30-60% of bank in reach has areas of erosion; high erosion potential during floods. |  |  |  |  | Unstable; many eroded areas; “raw” areas frequent along straight sections and bends; obvious bank sloughing; 60-100% of bank has erosional scars. |  |  |  |  |  |
| SCORE Left bank |  |  |  | 10 | 9 | 8 |  | 7 | 6 |  | 5 |  | 4 | 3 |  | 2 |  | 1 |  | 0 |  |
| SCORE Right bank |  |  |  | 10 | 9 | 8 |  | 7 | 6 |  | 5 |  | 4 | 3 |  | 2 |  | 1 |  | 0 |  |
| 10.Riparian Vegetative Zone Width (score each side) | Width of riparian zone >18 m; human activities (e.g., parking lots, roadbeds, clear-cuts, lawns, crops, etc.) have not impacted zone. |  |  |  |  | Width of riparian zone 12-18 meters; human activities have impacted zone only minimally. |  |  |  |  | Width of riparian zone 6-12 m; human activities have impacted zone a great deal. |  |  |  |  | Width of riparian zone < 6 m; little or no riparian vegetation due to human activities. |  |  |  |  |  |
| SCORE Left zone |  |  |  | 10 | 9 | 8 |  | 7 | 6 |  | 5 |  | 4 | 3 |  | 2 |  | 1 |  | 0 |  |
| SCORE Right zone |  |  |  | 10 | 9 | 8 |  | 7 | 6 |  | 5 |  | 4 | 3 |  | 2 |  | 1 |  | 0 |  |

TOTAL

SCORE

### Comments:

appendix B

Hazardous Waste Generation Record Form

HAZARDOUS WASTE GENERATION RECORD

Lab #_________Year________

| Date | Initials | Description | Volume or weight |
| --- | --- | --- | --- |

appendix C

MACROINVERTEBRATE TAXA LIST

| Taxon | FFG* | TV* | TVr* |
| --- | --- | --- | --- |
| Mollusca |  | 99 |  |
| Gastropoda |  | 99 |  |
| Mesogastropoda |  | 99 |  |
| Bithyniidae | SC | 8 | B |
| Hydrobiidae | SC | 8 | B |
| Amnicola sp. | SC | 5 | L,B |
| Amnicola grana | SC | 5 | B |
| Amnicola limosa | SC | 5 | B |
| Valvata sp. | SC | 8 | B |
| Valvata piscinalis | SC | 8 | B |
| Viviparidae | SC | 6 | (B) |
| Campeloma sp. | SC | 6 | (B) |
| Campeloma decisum | SC | 6 | B |
| Cipangopaludina sp. |  | 99 |  |
| Cipangopaludina chinensis |  | 99 |  |
| Viviparus sp. | SC | 6 | (B) |
| Basommatophora |  | 99 |  |
| Ancylidae | SC | 6 | B |
| Ferrissia sp. | SC | 6 | B |
| Ferrissia fragilis | SC | 6 | (B) |
| Ferrissia rivularis | SC | 10 | B |
| Laevapex fuscus | SC | 7 |  |
| Lymnaeidae | GC | 6 | B |
| Fossaria sp. | GC | 6 | (B) |
| Lymnaea sp. | GC | 6 | (B) |
| Pseudosuccinea sp. | GC | 6 | (B) |
| Pseudosuccinea columella | GC | 6 | L |
| Stagnicola sp. | GC | 6 | (B) |
| Physidae | GC | 8 | (B) |
| Physa sp. | GC | 9 | L |
| Planorbidae | SC | 6 | B |
| Gyraulus sp. | SC | 8 | (B) |
| Gyraulus circumstriatus | SC | 8 | N |
| Gyraulus deflectus | SC | 8 | N |
| Gyraulus parvus | SC | 8 | B |
| Helisoma sp. | SC | 6 | (B) |
| Helisoma anceps | SC | 6 | B |
| Helisoma anterosa |  | 99 |  |
| Helisoma campanulatum | SC | 6 | B |
| Helisoma trivolvis | SC | 6 | B |
| Planorbula sp. | SC | 6 | (B) |
| Planorbula armigera | SC | 6 | (B) |
| Pelecypoda |  | 99 |  |
| Unionoida |  | 99 |  |
| Unionidae | FC | 5 | N |
| Anodonta sp. | FC | 6 | (B) |
| Anodonta cataracta | FC | 6 | B |
| Anodonta grandis grandis | FC | 6 | (B) |
| Anodonta implicata | FC | 6 | B |
| Elliptio sp. | FC | 8 | (B) |
| Elliptio complanata | FC | 8 | B |
| Lampsilis sp. | FC | 6 | (B) |
| Lampsilis radiata radiata | FC | 6 | B |
| Ligumia sp. |  | 99 |  |
| Veneroida |  | 99 |  |
| Pisidiidae | FC | 6 | B |
| Musculium sp. | FC | 6 | B |
| Musculium partumium | FC | 6 | B |
| Pisidium sp. | FC | 6 | B |
| Sphaerium sp. | FC | 6 | B |
| Sphaerium striatinum | FC | 6 | B |
| Annelida |  | 99 |  |
| Polychaeta | GC | 99 |  |
| Syllidae | GC | 99 |  |
| Oligochaeta | GC | 99 |  |
| Lumbricina | GC | 8 | B |
| Haplotaxida |  | 99 |  |
| Tubificida |  | 99 |  |
| Enchytraeidae | GC | 10 | L,B |
| Naididae | GC | 9 | (L,B) |
| Allonais sp. |  | 99 |  |
| Aulophorus sp. |  | 99 |  |
| Chaetogaster sp. | PR | 6 | B |
| Dero sp. | GC | 10 | B,L |
| Dero vaga | GC | 10 | (L,B) |
| Haemonais waldvogeli | GC | 8 | B |
| Nais sp. | GC | 8 | B |
| Nais alpina | GC | 8 |  |
| Nais behningi | GC | 6 | B |
| Nais bretscheri | GC | 6 | B |
| Nais communis | GC | 8 | B |
| Nais elinguis | GC | 10 | B |
| Nais pardalis | GC | 8 | B |
| Nais simplex | GC | 6 | B |
| Nais variabilis | GC | 10 | B |
| Ophidonais sp. | GC | 6 | (B) |
| Ophidonais serpentina | GC | 6 | B |
| Pristina sp. | GC | 8 | B |
| Pristina aequiseta | GC | 8 | B |
| Pristina breviseta | GC | 99 |  |
| Pristina leidyi | GC | 8 | B |
| Pristinella sp. | GC | 10 | B |
| Pristinella acuminata | GC | 10 | B |
| Pristinella jenkinae | GC | 10 | B |
| Pristinella osborni | GC | 10 | B |
| Pristinella sima | GC | 10 | (B,L) |
| Slavina sp. | GC | 6 | (B) |
| Slavina appendiculata | GC | 6 | B |
| Specaria sp. | GC | 6 | (B) |
| Specaria josinae | GC | 6 | B |
| Stylaria sp. | GC | 9 | (L) |
| Stylaria lacustris | GC | 8 | B |
| Tubificidae | GC | 10 | B |
| Aulodrilus pigueti | GC | 8 | B |
| Aulodrilus pluriseta | GC | 8 | B |
| Limnodrilus sp. | GC | 10 | L,B |
| Limnodrilus claparadianus | GC | 10 | B |
| Limnodrilus hoffmeisteri | GC | 10 | L,B |
| Limnodrilus profundicola | GC | 10 | B |
| Limnodrilus spiralis | GC | 99 |  |
| Limnodrilus udekemianus | GC | 10 | L,B |
| Tubifex sp. | GC | 10 | (L,B) |
| Tubifex ignotus | GC | 99 |  |
| Tubifex tubifex | GC | 10 | L,B |
| Tubificidae IWB | GC | 10 | B |
| Tubificidae IWH | GC | 10 | B |
| Lumbriculida |  | 99 |  |
| Lumbriculidae | GC | 7 | L |
| Eclipidrilus sp. | GC | 5 |  |
| Lumbriculus sp. | GC | 8 | B |
| Lumbriculus variegatus | GC | 5 | (B) |
| Stylodrilus sp. | GC | 8 | (B) |
| Stylodrilus heringianus | GC | 8 | B |
| Hirudinea |  | 7 | B |
| Rhynchobdellida | PR | 7 | (B) |
| Glossiphoniidae | PR | 7 | (B) |
| Actinobdella sp. |  | 99 |  |
| Actinobdella inequiannulata |  | 99 |  |
| Batrocobdella sp. |  | 99 |  |
| Batrocobdella paludosa |  | 99 |  |
| Batrocobdella phalera | PR | 7 | L,B |
| Batrocobdella picta |  | 99 |  |
| Glossiphonia sp. |  | 99 |  |
| Glossiphonia complanata |  | 99 |  |
| Glossiphonia heteroclita |  | 99 |  |
| Helobdella sp. |  | 7 | (B) |
| Helobdella elongata | PR | 7 | B |
| Helobdella fusca |  | 99 |  |
| Helobdella papillata |  | 99 |  |
| Helobdella stagnalis | PR | 7 | B,L |
| Helobdella triserialis | PR | 7 | B |
| Placobdella sp. |  | 99 |  |
| Placobdella ornata |  | 99 |  |
| Placobdella papillifera | PR | 7 | (B) |
| Placobdella parasitica | PR | 7 | L,(B) |
| Gnathobdellida |  | 99 |  |
| Hirudinidae |  | 99 |  |
| Haemopsis sp. |  | 99 |  |
| Haemopsis grandis |  | 99 |  |
| Nephelopsis sp. |  | 99 |  |
| Nephelopsis obscura |  | 99 |  |
| Pharyngobdellida |  | 99 |  |
| Erpobdellidae | PR | 8 | (L) |
| Erpobdella sp. | PR | 8 | (L) |
| Erpobdella punctata | PR | 8 | (L) |
| Mooreobdella sp. | PR | 8 | (L) |
| Mooreobdella fervida | PR | 8 | (L) |
| Mooreobdella melanostoma | PR | 8 | (L) |
| Arthropoda |  | 99 |  |
| Crustacea |  | 99 |  |
| Cladocera |  | 99 |  |
| Eucopopoda |  | 99 |  |
| Ostracoda | GC | 6 |  |
| Isopoda |  | 99 |  |
| Anthuridae |  | 99 |  |
| Asellidae | GC | 8 | H |
| Caecidotea sp. | GC | 8 | B |
| Caecidotea communis | GC | 8 | B |
| Amphipoda | GC | 6 |  |
| Corophiidae | GC | 5 | N |
| Corophium sp. | GC | 5 | N |
| Crangonyctidae | GC | 8 | (L) |
| Crangonyx sp. | GC | 6 | B |
| Crangonyx abberans | GC | 6 | (B) |
| Crangonyx pseudogracilis | GC | 8 | (L) |
| Crangonyx richmondensis richmondensis | GC | 8 | (L) |
| Gammaridae | GC | 6 | (B) |
| Gammarus sp. | GC | 6 | B |
| Gammarus deubeni | GC | 6 | (B) |
| Gammarus fasciatus | GC | 6 | B |
| Gammarus tigrinus | GC | 6 | B |
| Hyalellidae | GC | 8 | H |
| Hyalella sp. | GC | 8 | (L,B) |
| Hyalella azteca | GC | 8 | L,B |
| Decapoda |  | 99 |  |
| Cambaridae | GC | 6 | B |
| Orconectes sp. | GC | 6 | B |
| Orconectes limosus |  | 99 |  |
| Orconectes virilis |  | 99 |  |
| Xanthidae |  | 99 |  |
| Rhithropanopeus harrisii |  | 99 |  |
| Arachnida | PR | 6 | L,B |
| Halacaridae | PR | 6 |  |
| Hydrachnidia | PR | 6 | L,B |
| Limnocaridae | PR | 6 | B |
| Collembola | GC | 99 |  |
| Isotomidae | GC | 5 | (B) |
| Insecta |  | 99 |  |
| Ephemeroptera |  | 99 |  |
| Ameletidae | GC | 0 | (B) |
| Ameletus sp. | GC | 0 | B |
| Ameletus ludens |  | 0 | B |
| Baetidae | GC | 4 | H |
| Acentrella sp. | SC | 4 | B |
| Acentrella carolina | SC | 4 | (B) |
| Acerpenna sp. | GC | 5 | (B) |
| Baetis sp. | GC | 6 | B |
| Baetis flavistriga | GC | 4 | B |
| Baetis frondalis | GC | 8 | L |
| Baetis intercalaris | GC | 6 | L,B |
| Baetis levitans |  | 99 |  |
| Baetis phoebus |  | 99 |  |
| Baetis rusticans |  | 99 |  |
| Baetis tricaudatus | GC | 6 | B |
| Baetis vagans | GC | 6 | (B) |
| Baetis (cerci only) sp. | GC | 6 | (B) |
| Baetis (short term. fil.) sp. | GC | 6 | (B) |
| Baetis (subeq. term.) sp. | GC | 6 | (B) |
| Callibaetis sp. | GC | 9 | L,B |
| Centroptilum sp. | GC | 2 | B |
| Cloeon sp. | GC | 4 | B |
| Heterocloeon sp. | GC | 2 | B |
| Heterocloeon curiosum | GC | 2 | B |
| Pseudocloeon sp. |  |  |  |
| Baetidae (cerci only) | GC | 6 |  |
| Baetidae (short term. fil.) | GC | 6 |  |
| Baetidae (subeq. term.) | GC | 6 |  |
| Caenidae | GC | 7 | H |
| Brachycercus sp. | GC | 3 | B |
| Caenis sp. | GC | 6 | B |
| Caenis horarria gr. | GC | 6 | (B) |
| Caenis pseudorivulorum gr. | GC | 6 | (B) |
| Ephemerellidae | GC | 1 | H |
| Attenella sp. | GC | 1 | (B) |
| Attenella attenuata | GC | 1 | B |
| Dannella sp. | GC | 2 | B |
| Dannella simplex | GC | 2 | B |
| Drunella sp. | SC | 0 | (B) |
| Drunella cornuta | GC | 0 | B |
| Drunella cornutella | GC | 0 |  |
| Drunella lata | SC | 0 | L,B |
| Drunella walkeri | SC | 0 | B |
| Ephemerella sp. | GC | 1 | B |
| Ephemerella catawba | GC | 1 |  |
| Ephemerella dorothea | GC | 1 | B |
| Ephemerella invaria | GC | 1 | B |
| Ephemerella needhami | GC | 1 | B |
| Ephemerella rotunda | GC | 1 | B |
| Ephemerella subvaria | GC | 1 | B |
| Eurylophella sp. | GC | 2 | B |
| Eurylophella aestiva | GC | 99 |  |
| Eurylophella bicolor | GC | 5 | L |
| Eurylophella lutulenta | GC | 2 | (B) |
| Eurylophella temporalis | GC | 5 | L,B |
| Serratella sp. | GC | 2 | B |
| Serratella deficiens | GC | 2 | B |
| Serratella serrata | GC | 2 | B |
| Serratella serratoides | GC | 2 | L,B |
| Ephemeridae | GC | 4 | H |
| Ephemera sp. | GC | 2 | L,B |
| Ephemera simulans |  | 99 |  |
| Ephemera varia |  | 99 |  |
| Hexagenia sp. | GC | 6 | B |
| Hexagenia bilineata |  | 99 |  |
| Hexagenia limbata |  | 99 |  |
| Heptageniidae | SC | 4 | H |
| Epeorus sp. | SC | 0 | B |
| Epeorus (Iron) sp. | SC | 0 |  |
| Heptagenia sp. | SC | 4 | B |
| Heptagenia pulla | SC | 4 | B |
| Leucrocuta sp. | SC | 1 | B |
| Leucrocuta maculipennis |  | 99 |  |
| Rhithrogena sp. | GC | 0 | L,B |
| Stenacron sp. | SC | 7 |  |
| Stenacron interpunctatum | SC | 7 |  |
| Stenonema sp. | SC | 3 | B |
| Stenonema femoratum | SC | 7 | B |
| Stenonema ithaca | SC | 3 | B |
| Stenonema mediopunctatum | SC | 3 | B |
| Stenonema modestum | SC | 1 | B |
| Stenonema pudicum | SC | 2 | L |
| Stenonema pulchellum | SC | 3 | B |
| Stenonema vicarium |  | 99 |  |
| Isonychiidae | GC | 2 | (B) |
| Isonychia sp. | GC | 2 | (B) |
| Leptophlebiidae | GC | 2 | H |
| Choroterpes sp. | GC | 2 | B |
| Habrophlebia sp. | GC | 4 | B |
| Habrophlebia vibrans | GC | 4 | B |
| Habrophlebiodes sp. | SC | 6 | B |
| Habrophlebiodes americana | SC | 6 | (B) |
| Leptophlebia sp. | GC | 4 | B |
| Paraleptophlebia sp. | GC | 1 | L,B |
| Paraleptophlebia moerens |  | 99 |  |
| Metretopodidae | GC | 99 |  |
| Potamanthidae | GC | 4 | (B) |
| Potamanthus sp. | GC | 2 | L |
| Siphlonuridae |  | 99 |  |
| Siphlonurus sp. | GC | 7 | B |
| Siphlonurus alternatus |  | 99 |  |
| Siphlonurus quebescensis |  | 99 |  |
| Tricorythidae | GC | 5 | (L) |
| Tricorythodes sp. | GC | 4 | B |
| Odonata |  | 99 |  |
| Aeschnidae | PR | 3 | H |
| Aeschna sp. | PR | 99 |  |
| Aeschna eremita | PR | 99 |  |
| Aeschna umbrosa | PR | 99 |  |
| Anax sp. | PR | 99 |  |
| Anax junius | PR | 99 |  |
| Basiaeschna janata | PR | 6 | B |
| Boyeria sp. | PR | 2 | B |
| Boyeria vinosa | PR | 2 | (B) |
| Calopterygidae | PR | 5 | H |
| Calopteryx sp. | PR | 6 | B |
| Calopteryx maculata | PR | 99 |  |
| Hetaerina sp. | PR | 6 | B |
| Coenagrionidae | PR | 9 | H |
| Argia sp. | PR | 6 | B |
| Argia moesta | PR | 99 |  |
| Argia translata | PR | 99 |  |
| Argia violacea | PR | 99 |  |
| Chromagrion conditum | PR | 8 | (B) |
| Coenagrion sp. | PR | 8 | (B) |
| Enallagma sp. | PR | 8 | B |
| Enallagma civile | PR | 99 |  |
| Enallagma exsulans | PR | 99 |  |
| Enallagma geminatum | PR | 99 |  |
| Enallagma signatum | PR | 99 |  |
| Ischnura sp. | PR | 9 | L,B |
| Ischnura posita | PR | 99 |  |
| Ischnura verticalis | PR | 99 |  |
| Nehalennia sp. | PR | 99 |  |
| Nehalennia gracilis | PR | 99 |  |
| Cordulegastridae | PR | 3 | H |
| Cordulegaster sp. | PR | 3 | B |
| Cordulegaster maculata | PR | 99 |  |
| Cordulegaster obliqua | PR | 5 | (B) |
| Corduliidae | PR | 5 | H |
| Epitheca sp. | PR | 2 | (B) |
| Epitheca(Tetragoneuria) sp. | PR | 99 |  |
| Neurocordulia sp. |  | 99 |  |
| Neurocordulia obsoleta |  | 99 |  |
| Somatochlora sp. | PR | 9 | L |
| Gomphidae | PR | 5 | N |
| Dromogomphus sp. | PR | 6 | L |
| Dromogomphus spinosus | PR | 99 |  |
| Gomphus sp. | PR | 5 | B |
| Lanthus sp. | PR | 5 | B |
| Lanthus parvulus | PR | 99 |  |
| Ophiogomphus sp. | PR | 1 | B |
| Ophiogomphus carolus | PR | 1 | (B) |
| Stylogomphus sp. | PR | 1 | B |
| Stylogomphus albistylus | PR | 1 | (B) |
| Lestidae | PR | 9 | H |
| Lestes sp. | PR | 99 |  |
| Lestes congener | PR | 99 |  |
| Lestes disjunctus | PR | 99 |  |
| Lestes rectangularis | PR | 99 |  |
| Lestes unguiculatus | PR | 99 |  |
| Lestes vigilax | PR | 99 |  |
| Libellulidae | PR | 9 | H |
| Erythemis sp. | PR | 99 |  |
| Erythemis simplicicollis | PR | 8 | L |
| Erythrodiplax sp. | PR | 99 |  |
| Libellula sp. | PR | 10 | L |
| Libellula luctuosa | PR | 99 |  |
| Pachydiplax sp. | PR | 99 |  |
| Pachydiplax longipennis | PR | 10 | L |
| Perithemis sp. | PR | 10 | L |
| Perithemis tenera | PR | 99 |  |
| Plathemis sp. | PR | 99 |  |
| Plathemis lydia | PR | 10 | L |
| Sympetrum sp. | PR | 7 | L |
| Macromiidae | PR | 2 | (B) |
| Macromia sp. | PR | 2 | B |
| Plecoptera | GC | 3 | N |
| Capniidae | SH | 1 | H |
| Allocapnia sp. | SH | 3 | L,B |
| Chloroperlidae | PR | 1 | H |
| Alloperla sp. | GC | 0 | B |
| Haploperla sp. | PR | 1 | (L,B) |
| Haploperla brevis | PR | 1 |  |
| Sweltsa sp. | PR | 0 |  |
| Utaperla sp. | PR | 0 | (B) |
| Leuctridae | SH | 0 | H |
| Leuctra sp. | SH | 0 | B |
| Leuctra truncata | SH | 0 | (B) |
| Paraleuctra sp. | SH | 0 |  |
| Leuctridae/Capniidae | SH | 2 |  |
| Nemouridae | SH | 2 | H |
| Amphinemura sp. | SH | 1 |  |
| Nemoura sp. | SH | 1 | B |
| Peltoperlidae | SH | 0 | (B) |
| Peltoperla sp. | SH | 0 | (B) |
| Tallaperla sp. | SH | 0 |  |
| Perlidae | PR | 1 | H |
| Acroneuria sp. | PR | 0 | B |
| Acroneuria abnormis | PR | 0 | B |
| Acroneuria carolinensis | PR | 0 | L,B |
| Acroneuria lycorias | PR | 0 | B |
| Agnetina sp. | PR | 2 | B |
| Attaneuria sp. | PR | 3 |  |
| Beloneuria sp. | PR | 0 | L |
| Hansonoperla sp. | PR | 1 |  |
| Neoperla sp. | PR | 3 | B |
| Neoperla clymene | PR | 99 |  |
| Paragnetina sp. | PR | 1 | (B) |
| Paragnetina immarginata | PR | 1 | B |
| Paragnetina media | PR | 5 | B |
| Perlesta sp. | PR | 5 | (B) |
| Perlesta placida | PR | 5 | L,B |
| Phasganophora sp. | PR | 99 |  |
| Phasganophora capitata | PR | 99 |  |
| Perlodidae | PR | 2 | H |
| Diura sp. | PR | 2 |  |
| Isogenoides sp. | PR | 0 | L,B |
| Isoperla sp. | PR | 2 | B |
| Pteronarcyidae | SH | 0 | (B) |
| Pteronarcys sp. | SH | 0 | B |
| Taeniopterygidae | SH | 2 | H |
| Taeniopteryx sp. | SH | 2 | B |
| Hemiptera |  | 99 |  |
| Belostomatidae |  | 99 |  |
| Belostoma sp. |  | 99 |  |
| Corixidae |  | 99 |  |
| Corisella sp. |  | 99 |  |
| Hesperocorixa sp. |  | 99 |  |
| Palmacorixa sp. |  | 99 |  |
| Palmacorixa nana |  | 99 |  |
| Sigara sp. |  | 99 |  |
| Trichocorixa sp. |  | 99 |  |
| Gerridae |  | 99 |  |
| Gerris sp. |  | 99 |  |
| Metrobates sp. |  | 99 |  |
| Neogerris sp. |  | 99 |  |
| Neogerris hesione |  | 99 |  |
| Rheumatobates sp. |  | 99 |  |
| Trepobates sp. |  | 99 |  |
| Hydrometridae |  | 99 |  |
| Hydrometra sp. |  | 99 |  |
| Mesoveliidae |  | 99 |  |
| Mesovelia sp. |  | 99 |  |
| Mesovelia mulsanti |  | 99 |  |
| Naucoridae |  | 99 |  |
| Pelocoris sp. |  | 99 |  |
| Nepidae |  | 99 |  |
| Ranatra sp. |  | 99 |  |
| Notonectidae |  | 99 |  |
| Notonecta sp. |  | 99 |  |
| Notonecta irrorata |  | 99 |  |
| Pleidae |  | 99 |  |
| Neoplea sp. |  | 99 |  |
| Neoplea striola |  | 99 |  |
| Saldidae |  | 99 |  |
| Pentacora sp. |  | 99 |  |
| Saldula sp. |  | 99 |  |
| Veliidae |  | 99 |  |
| Microvelia sp. |  | 99 |  |
| Rhagovelia sp. |  | 99 |  |
| Megaloptera |  | 99 |  |
| Corydalidae | PR | 5 | (L,B) |
| Chauloides sp. | PR | 4 | B |
| Corydalus sp. | PR | 4 | (B) |
| Corydalus cornutus | PR | 4 | B |
| Nigronia sp. | PR | 0 | (B) |
| Nigronia serricornis | PR | 0 | B |
| Sialidae | PR | 4 | (B) |
| Sialis sp. | PR | 4 | B |
| Neuroptera |  | 99 |  |
| Sisyridae |  | 99 |  |
| Climacia sp. |  | 99 |  |
| Climacia areolaris | PR | 5 | (B) |
| Sisyra sp. |  | 99 |  |
| Trichoptera |  | 99 |  |
| Brachycentridae | FC | 1 | H |
| Adicrophleps hitchcocki | SH | 2 | B |
| Brachycentrus sp. | FC | 1 | N |
| Micrasema sp. | SH | 2 | B |
| Glossosomatidae | SC | 0 | H |
| Agapetus sp. | SC | 0 | L,B |
| Culoptila sp. | SC | 1 | (B) |
| Glossosoma sp. | SC | 0 | B |
| Glossosoma intermedium | SC | 0 | (B) |
| Protoptila sp. | SC | 1 | B |
| Protoptila/Culoptila sp. |  | 99 |  |
| Helicopsychidae | SC | 3 | H |
| Helicopsyche sp. | SC | 3 | B |
| Helicopsyche borealis | SC | 3 | B |
| Hydropsychidae | FC | 4 | H |
| Arctopsyche sp. | FC | 1 | B |
| Cheumatopsyche sp. | FC | 5 | B |
| Diplectrona sp. | FC | 0 | B |
| Diplectrona modesta | FC | 0 | (B) |
| Diplectrona modesta gr. | FC | 0 | (B) |
| Hydropsyche sp. | FC | 4 | B |
| Hydropsyche alhedra | FC | 5 | (B) |
| Hydropsyche betteni | FC | 6 | B |
| Hydropsyche betteni gr. | FC | 6 | B |
| Hydropsyche bronta | FC | 6 | B |
| Hydropsyche cuanis | FC | 99 |  |
| Hydropsyche dicantha | FC | 2 |  |
| Hydropsyche morosa | FC | 99 |  |
| Hydropsyche morosa gr. | FC | 6 | B |
| Hydropsyche phalerata | FC | 99 |  |
| Hydropsyche simulans | FC | 99 |  |
| Hydropsyche slossonae | FC | 99 |  |
| Hydropsyche sparna | FC | 6 | B |
| Hydropsyche valanis | FC | 6 | B |
| Hydropsyche venularis | FC | 4 | B |
| Hydropsyche walkeri | FC | 5 | (B) |
| Macrostemum sp. | FC | 3 | B |
| Macrostemum zebratum | FC | 3 | B |
| Hydroptilidae | GC | 4 | H |
| Agraylea sp. | GC | 8 | B |
| Dibusa angata | SC | 6 | (B) |
| Hydroptila sp. | GC | 6 | L,B |
| Leucotrichia sp. | SC | 6 | B |
| Leucotrichia pictipes | SC | 6 | (B) |
| Mayatrichia sp. | SC | 6 | B |
| Ochrotrichia sp. | GC | 4 | H |
| Oxyethira sp. | GC | 3 | B |
| Lepidostomatidae | SH | 1 | H |
| Lepidostoma sp. | SH | 1 | L,B |
| Leptoceridae | PR | 4 | H |
| Ceraclea sp. | GC | 3 | B |
| Ceraclea tarsipunctatus | GC | 99 |  |
| Leptocerus sp. | GC | 99 |  |
| Leptocerus americanus | GC | 99 |  |
| Mystacides sp. | GC | 4 | B |
| Nectopsyche sp. | SH | 3 | B |
| Oecetis sp. | PR | 5 | B |
| Oecetis avara | PR | 5 | B |
| Oecetis cinerascens | PR | 5 | B |
| Oecetis inconspicua | PR | 5 | B |
| Setodes sp. | GC | 2 | B |
| Triaenodes sp. | SH | 6 | B |
| Triaenodes abus |  | 99 |  |
| Triaenodes marginatus |  | 99 |  |
| Limnephilidae | SH | 4 | H |
| Apatania sp. | SC | 3 | B |
| Goera sp. | SC | 3 | B |
| Hydatophylax sp. | SH | 2 | B |
| Limnephilus sp. | SH | 4 | (B) |
| Limnephilus submonilifer |  | 99 |  |
| Pycnopsyche sp. | SH | 4 | B |
| Molannidae | SC | 6 | H |
| Molanna sp. | SC | 6 | B |
| Molanna uniophila |  | 99 |  |
| Odontoceridae | SH | 0 | H |
| Psilotreta sp. | SC | 0 | L,B |
| Philopotamidae | FC | 3 | H |
| Chimarra sp. | FC | 4 | B |
| Chimarra aterrima | FC | 4 | B |
| Chimarra obscura | FC | 4 | B |
| Dolophilodes sp. | FC | 0 | B |
| Dolophilodes distinctus | FC | 0 | (B) |
| Wormaldia sp. | FC | 0 | B |
| Phryganeidae | SH | 4 | H |
| Agrypnia sp. |  | 99 |  |
| Oligostomis sp. | PR | 2 | B |
| Phryganea sp. |  | 99 |  |
| Ptilostomis sp. | SH | 5 | B |
| Polycentropodidae | FC | 6 | H |
| Cernotina sp. | PR | 99 |  |
| Neureclipsis sp. | FC | 7 | B |
| Nyctiophylax sp. | PR | 5 | (B) |
| Phylocentropus sp. | FC | 5 | B |
| Phylocentropus placidus |  | 99 |  |
| Polycentropus sp. | PR | 6 | B |
| Polycentropus remotus | PR | 6 | B |
| Polycentropus sensu lato complex | PR | 6 | B |
| Psychomyiidae | GC | 2 | H |
| Lype sp. | SC | 3 |  |
| Lype diversa | GC | 2 |  |
| Psychomyia sp. | GC | 2 | (B) |
| Rhyacophilidae | PR | 0 | H |
| Rhyacophila sp. | PR | 1 | B |
| Rhyacophila carolina gr. | PR | 1 |  |
| Rhyacophila fuscula | PR | 0 | B |
| Rhyacophila fuscula gr. | PR | 1 |  |
| Sericostomatidae | SH | 3 | H |
| Agarodes sp. | GC | 3 | H |
| Uenoidae | SC | 3 | (B) |
| Neophylax sp. | SC | 3 | B |
| Lepidoptera |  | 99 |  |
| Pyralidae | SH | 5 | H |
| Nymphula sp. | SH | 7 | B |
| Parapoynx sp. | SH | 5 | B |
| Petrophila sp. | SC | 5 | B |
| Coleoptera |  | 99 |  |
| Chrysomelidae |  | 99 |  |
| Donacia sp. |  | 99 |  |
| Pyrrhalta sp. |  | 99 |  |
| Pyrrhalta (Galerucella) nymphaeae |  | 99 |  |
| Curculionidae |  | 99 |  |
| Dryopidae | SC | 5 | H |
| Helichus sp. | SC | 5 | L,B |
| Dytiscidae | PR | 5 | B |
| Acilius sp. | PR | 99 |  |
| Acilius mediatus | PR | 99 |  |
| Agabus sp. | PR | 5 | B |
| Carrhydrus sp. | PR | 99 |  |
| Carrhydrus crassipes | PR | 99 |  |
| Celina sp. | PR | 99 |  |
| Colymbetes sp. | PR | 99 |  |
| Colymbetes/Rhantus sp. | PR | 99 |  |
| Coptotomus sp. | PR | 9 | L |
| Deronectes sp. | PR | 99 |  |
| Dytiscus sp. | PR | 99 |  |
| Hydaticus sp. | PR | 99 |  |
| Hydroporus sp. | PR | 5 | B |
| Hydroporus consimilis | PR | 99 |  |
| Hygrotus sp. | PR | 99 |  |
| Hygrotus nubilus | PR | 99 |  |
| Hygrotus/Hydroporus sp. | PR | 99 |  |
| Ilybius sp. | PR | 5 | (B) |
| Laccophilus sp. | PR | 5 | B |
| Laccophilus maculosus | PR | 99 |  |
| Laccornis sp. | PR | 99 |  |
| Matus sp. | PR | 99 |  |
| Matus bicarinatus | PR | 99 |  |
| Oreodytes sp. | PR | 99 |  |
| Uvarus sp. | PR | 99 |  |
| Elmidae | SC | 4 | H |
| Ancyronyx variegata | GC | 5 | B |
| Dubiraphia sp. | GC | 6 | L,B |
| Dubiraphia bivittata | GC | 6 | B |
| Dubiraphia vittata | GC | 6 | B |
| Macronychus glabratus | SH | 5 | L,B |
| Microcylloepus sp. | GC | 3 |  |
| Microcylloepus pusillus | GC | 3 | B |
| Optioservus sp. | SC | 4 | B |
| Optioservus ampliatus | SC | 4 |  |
| Optioservus fastiditus | SC | 4 | B |
| Optioservus ovalis | SC | 4 |  |
| Optioservus trivittatus | SC | 4 |  |
| Oulimnius latiusculus | SC | 4 | B |
| Promoresia sp. | SC | 2 | B |
| Promoresia elegans | SC | 2 | L,B |
| Promoresia tardella | SC | 2 | B |
| Stenelmis sp. | SC | 5 | L,B |
| Stenelmis bicarinata | SC | 5 |  |
| Stenelmis concinna |  |  |  |
| Stenelmis crenata | SC | 5 | B |
| Stenelmis crenata gr. | SC | 5 | B |
| Stenelmis mirabilis | SC | 5 | (B) |
| Stenelmis sandersoni | SC | 5 | (B) |
| Gyrinidae | PR | 4 | (B) |
| Dineutus sp. | PR | 4 | B |
| Dineutus ciliatus | PR | 99 |  |
| Gyrinus sp. | PR | 4 | B |
| Gyrinus fraternus | PR | 4 | (B) |
| Gyrinus ventralis | PR | 99 |  |
| Halipildae | SH | 5 | (B) |
| Haliplus sp. | SH | 5 | B |
| Haliplus immaculicollis |  | 99 |  |
| Peltodytes sp. | SH | 5 | B |
| Peltodytes duodecimpunctatus |  | 99 |  |
| Peltodytes edentulus |  | 99 |  |
| Peltodytes pedunculatus |  | 99 |  |
| Hydraenidae | PR | 99 |  |
| Hydrophilidae | PR | 5 | (B) |
| Anacaena sp. |  | 99 |  |
| Anacaena limbata |  | 99 |  |
| Berosus sp. | PR | 5 | B |
| Berosus striatus |  | 99 |  |
| Cymbiodyta sp. |  | 99 |  |
| Cymbiodyta blanchardi |  | 99 |  |
| Enochrus sp. | GC | 5 | B |
| Helochares sp. | GC | 5 | B |
| Helocombus sp. |  | 99 |  |
| Helocombus bifidus |  | 99 |  |
| Helophorus sp. | SH | 5 | B |
| Hydrobius sp. | PR | 5 | B |
| Hydrochara sp. |  | 99 |  |
| Hydrochus sp. |  | 99 |  |
| Laccobius sp. | PR | 5 | B |
| Paracymus sp. |  | 99 |  |
| Tropisternus sp. |  | 99 |  |
| Psephenidae | SC | 4 | H |
| Ectopria sp. | SC | 5 | B |
| Ectopria nervosa | SC | 5 | B |
| Psephenus herricki | SC | 4 | B |
| Ptilodactylidae | SH | 99 |  |
| Anchytarsus sp. | SH | 4 | L |
| Anchytarsus bicolor | SH | 4 | L |
| Diptera |  | 99 |  |
| Athericidae | PR | 2 | H |
| Atherix sp. | PR | 4 | B |
| Atherix variegata |  | 99 |  |
| Blephariceridae | SC | 0 | H |
| Blepharicera sp. | SC | 0 | L,B |
| Blepharicera tenuipes |  | 99 |  |
| Ceratopogonidae | PR | 6 | H |
| Bezzia sp. | PR | 6 | B |
| Culicoides sp. | SC | 10 | B |
| Dashyhelea sp. | PR | 99 |  |
| Forcipomyia sp. | SC | 6 | B |
| Palpomyia sp. | PR | 6 | (B) |
| Palpomyia/Bezzia sp. | PR | 6 |  |
| Palpomyia/Probezzia sp. | PR | 6 | (B) |
| Probezzia sp. | PR | 6 | B |
| Stilobezzia sp. | PR | 6 | (B) |
| Chaoboridae | PR | 8 | (B) |
| Chaoborus sp. | PR | 8 | B |
| Chaoborus punctipennis | PR | 9 | L |
| Chironomidae | GC | 6 | H |
| Chironominae | GC | 6 | B |
| Chironomini | GC | 6 | B |
| Chironomus sp. | GC | 10 | L,B |
| Chironomus decorus/riparius | GC | 10 | (L,B) |
| Chironomus fumidus | GC | 10 | (L,B) |
| Chironomus reductus gr. | GC | 10 | (L,B) |
| Chironomus riparius | GC | 10 | (L,B) |
| Chironomus tentans | GC | 10 | (L,B) |
| Cladopelma sp. | GC | 9 | B |
| Cryptochironomus sp. | PR | 8 | (L,B) |
| Cryptochironomus fulvus gr. | PR | 8 | B |
| Cryptotendipes sp. | GC | 6 |  |
| Demicryptochironomus sp. | GC | 2 |  |
| Dicrotendipes sp. | GC | 8 | L |
| Dicrotendipes modestus | GC | 8 | B |
| Dicrotendipes neomodestus | GC | 8 | L,B |
| Dicrotendipes nervosus | GC | 8 | B |
| Dicrotendipes/Glyptotendipes sp. | GC | 9 | (L,B) |
| Einfeldia sp. | GC | 9 | B |
| Endochironomus sp. | SH | 10 | B |
| Endochironomus nigricans | SH | 10 | B |
| Glyptotendipes sp. | SH | 10 | B |
| Glyptotendipes lobiferus | SH | 10 | B |
| Glyptotendipes polytomus | SH | 99 |  |
| Glyptotendipes senilis | SH | 99 |  |
| Goeldichironomus sp. | GC | 8 |  |
| Harnischia sp. | GC | 8 | L |
| Kiefferulus sp. | GC | 10 | (L) |
| Lauterborniella sp. |  | 99 |  |
| Lauterborniella agrayloides | GC | 8 | B |
| Micropsectra polita gr. | GC | 7 | B |
| Microtendipes sp. | FC | 5 | (B) |
| Microtendipes caducus | FC | 99 |  |
| Microtendipes caelum | FC | 99 |  |
| Microtendipes pedellus |  | 99 |  |
| Microtendipes pedellus gr. | FC | 6 | B |
| Microtendipes rydalensis gr. | FC | 6 | B |
| Microtendipes tarsalis | FC | 99 |  |
| Nilothauma sp. | GC | 6 | B |
| Parachironomus sp. | PR | 10 | B |
| Parachironomus abortivus | PR | 10 | B |
| Paracladopelma sp. | GC | 7 | (B) |
| Paralauterborniella sp. | GC | 8 | B |
| Paratendipes sp. | GC | 6 | B |
| Paratendipes albimanus | GC | 6 | (B) |
| Phaenopsectra sp. | SC | 7 | L,B |
| Phaenopsectra dyari | SC | 7 | B |
| Phaenopsectra flavipes | SC | 7 | B |
| Phaenopsectra obediens | SC | 99 |  |
| Polypedilum sp. | SH | 6 | (B) |
| Polypedilum angulum | SH | 6 |  |
| Polypedilum aviceps | SH | 4 | L,B |
| Polypedilum fallax | SH | 6 |  |
| Polypedilum fallax gr. | SH | 6 | B |
| Polypedilum flavum | SH | 6 | B |
| Polypedilum halterale | SH | 6 | B |
| Polypedilum halterale gr. | SH | 6 | B |
| Polypedilum illinoense | SH | 6 | B |
| Polypedilum laetum | SH | 6 | B |
| Polypedilum obtusum | SH | 6 | B |
| Polypedilum ophiodes | SH | 99 |  |
| Polypedilum scalaenum | SH | 6 | B |
| Polypedilum scalaenum gr. | SH | 6 | B |
| Polypedilum simulans gr. | SH | 6 | B |
| Polypedilum simulans/digitifer | SH | 6 | (B) |
| Polypedilum trigonum | SH | 6 | H |
| Polypedilum tritum | SH | 6 | (B) |
| Pseudochironomus sp. | GC | 5 | B |
| Robackia sp. | GC | 4 | (L) |
| Stenochironomus sp. | GC | 5 | B |
| Stictochironomus sp. | GC | 9 | B |
| Stictochironomus divinctus | GC | 99 |  |
| Tribelos sp. | GC | 7 | L,B |
| Tribelos jucundus | GC | 5 | B |
| Tribelos/Phaenopsectra sp. | GC | 7 |  |
| Xenochironomus sp. | PR | 0 | (B) |
| Xenochironomus xenolabis | PR | 0 | B |
| Xenochironomus (Xenochironomus sp. | PR | 0 | (B) |
| Tanytarsini | FC | 6 | B |
| Cladotanytarsus sp. | FC | 5 | B |
| Cladotanytarsus mancus gr. | CF | 5 | (B) |
| Constempellina sp. | GC | 4 | B |
| Micropsectra sp. | GC | 7 | B |
| Micropsectra brunnipes | GC | 7 | B |
| Micropsectra dives gr. | GC | 7 |  |
| Micropsectra/Tanytarsus sp. | FC | 7 |  |
| Paratanytarsus sp. | FC | 6 | B |
| Paratanytarsus dissimilis |  | 99 |  |
| Rheotanytarsus sp. | FC | 6 | L,B |
| Rheotanytarsus distinctissimus gr. | FC | 6 | B |
| Rheotanytarsus exiguus gr. | FC | 6 | B |
| Rheotanytarsus pellucidus | FC | 5 | (B) |
| Stempellina sp. | GC | 2 | L,B |
| Stempellina bausei | GC | 2 | B |
| Stempellinella sp. | GC | 2 | B |
| Sublettea coffmani | FC | 4 | B |
| Tanytarsus sp. | FC | 6 | B |
| Tanytarsus glabrescens | FC | 6 | (B) |
| Tanytarsus glabrescens gr. | FC | 6 | B |
| Tanytarsus guerulus gr. | FC | 6 | B |
| Zavrelia sp. | FC | 4 | B |
| Zavrelia/Stempellinella sp. | GC | 4 |  |
| Diamesinae | GC | 2 | B |
| Diamesa sp. | GC | 5 | B |
| Pagastia sp. | GC | 1 | B |
| Potthastia sp. | GC | 2 | (B) |
| Potthastia gaedii gr. | GC | 2 | L,B |
| Potthastia longimana gr. | GC | 2 | B |
| Pseudodiamesa sp. | GC | 99 |  |
| Sympotthastia sp. | GC | 2 | B |
| Orthocladiinae | GC | 5 | B |
| Acricotopus sp. | GC | 10 | (B) |
| Brillia sp. | SH | 5 | L,B |
| Brillia flavifrons | SH | 5 | B |
| Brillia parva | SH | 5 | B |
| Brillia sera | SH | 5 | B |
| Cardiocladius sp. | PR | 5 | (B) |
| Cardiocladius albiplumus | PR | 5 | B |
| Cardiocladius obscurus | PR | 5 | B |
| Chaetocladius sp. | GC | 6 |  |
| Corynoneura sp. | GC | 4 | B |
| Corynoneura celeripes | GC | 4 | B |
| Corynoneura taris | GC | 4 | B |
| Cricotopus sp. | SH | 7 | (B) |
| Cricotopus albiforceps | GC | 7 | (B) |
| Cricotopus annulator | SH | 7 | (B) |
| Cricotopus bicinctus | GC | 7 | B |
| Cricotopus bicinctus gr. | GC | 7 | B |
| Cricotopus cylindraceus/festivellus gr. | GC | 7 | B |
| Cricotopus exilus |  | 99 |  |
| Cricotopus festivellus gr. | GC | 7 | B |
| Cricotopus flavocinctus | GC | 7 | (B) |
| Cricotopus fugax |  | 99 |  |
| Cricotopus fuscus gr. | SH | 7 | (B) |
| Cricotopus intersectus gr. | SH | 7 | B |
| Cricotopus junus |  | 99 |  |
| Cricotopus slossonae |  | 99 |  |
| Cricotopus sylvestris | SH | 7 | B |
| Cricotopus sylvestris gr. | SH | 7 | B |
| Cricotopus sylvestris/intersectus | SH | 7 | (B) |
| Cricotopus tibialis gr. | SH | 7 | (B) |
| Cricotopus tremulus | SH | 7 | (B) |
| Cricotopus tremulus gr. | SH | 7 | B |
| Cricotopus triannulatus | SH | 7 | B |
| Cricotopus trifaciatus | SH | 7 | (B) |
| Cricotopus trifascia | SH | 6 |  |
| Cricotopus trifascia gr. | SH | 6 | B |
| Cricotopus vierriensis | SH | 7 | B |
| Cricotopus/Orthocladius sp. | GC | 7 | (B) |
| Diplocladius sp. | GC | 8 | B |
| Eukiefferiella sp. | GC | 6 | N |
| Eukiefferiella brehmi gr. | GC | 4 | L,B |
| Eukiefferiella brevicalcar gr. | GC | 4 | B |
| Eukiefferiella brevinervis gr. | GC | 99 |  |
| Eukiefferiella claripennis gr. | GC | 8 | B |
| Eukiefferiella coerulescens gr. | GC | 4 | B |
| Eukiefferiella cyanea gr. | GC | 99 |  |
| Eukiefferiella devonica gr. | GC | 4 | B |
| Eukiefferiella gracei gr. | GC | 4 | B |
| Eukiefferiella pseudomontana gr. | GC | 8 | B |
| Eukiefferiella rectangularis gr. | GC | 99 |  |
| Heleniella sp. | GC | 5 |  |
| Heterotrissocladius sp. | GC | 4 | (B) |
| Hydrobaenus sp. | GC | 8 | (B) |
| Krenosmittia sp. | GC | 1 | B |
| Limnophyes sp. | GC | 8 | B |
| Lopescladius sp. | GC | 4 | B |
| Metriocnemus sp. | GC | 5 |  |
| Nanocladius sp. | GC | 7 | L,B |
| Nanocladius crassicornus | GC | 7 | B |
| Nanocladius minimus | GC | 3 | B |
| Nanocladius parvulus gr. | GC | 7 | (L,B) |
| Nanocladius rectinervis | GC | 7 | B |
| Nanocladius spiniplenus | GC | 6 | B |
| N. (Plecopteracoluthus) sp. | GC | 3 | B |
| N. (Plecopteracoluthus) branchicolus | GC | 3 | B |
| N. (Plecopteracoluthus) downesi | GC | 3 | B |
| Orthocladius sp. | GC | 6 | B |
| Orthocladius annectans | GC | 6 | B |
| Orthocladius carlatus | GC | 6 | B |
| Orthocladius obumbratus | GC | 6 | B |
| Orthocladius robacki | GC | 6 | B |
| O. (Pogonocladius) sp. |  | 99 |  |
| Parachaetocladius sp. | GC | 2 | B |
| Parakiefferiella sp. | GC | 4 | B |
| Paralimnophyes sp. | GC | 7 |  |
| Parametriocnemus sp. | GC | 5 | (B) |
| Paraphaenocladius sp. | GC | 4 | B |
| Parorthocladius sp. | GC | 5 |  |
| Psectrocladius sp. | GC | 8 | B |
| Psilometriocnemus sp. | GC | 4 | B |
| Rheocricotopus sp. | GC | 6 | (B) |
| Rheocricotopus robacki | GC | 5 | B |
| Symposiocladius lignicola | SH | 5 | L |
| Synorthocladius sp. | GC | 6 | (B) |
| Thienemanniella sp. | GC | 6 | L,B |
| Thienemanniella xena | GC | 6 | B |
| Tokunagaia sp. | GC | 5 |  |
| Trissocladius sp. | GC | 5 | B |
| Tvetenia sp. | GC | 5 | (B) |
| Tvetenia bavarica gr. | GC | 5 | B |
| Tvetenia vitracies gr. | GC | 5 | B |
| Xylotopus par | SH | 7 | L |
| Podonominae | GC | 1 | (B) |
| Prodiamesinae | GC | 6 | (H) |
| Prodiamesa sp. | GC | 3 | B |
| Prodiamesa olivacea | GC | 3 | B |
| Tanypodinae | PR | 7 | B |
| Ablabesmyia sp. | PR | 8 | B |
| Ablabesmyia mallochi | PR | 8 | L,B |
| Ablabesmyia peleensis | PR | 8 | B |
| Clinotanypus sp. | PR | 8 | (B) |
| Clinotanypus pinguis | PR | 8 | B |
| Coelotanypus sp. | PR | 4 | B |
| Conchapelopia sp. | PR | 6 | B |
| Conchapelopia cornuticandata | PR | 6 | (B) |
| Conchapelopia rurika | PR | 6 | B |
| Derotanypus sp. | PR | 7 | (B) |
| Djalmabatista sp. | PR | 99 |  |
| Guttipelopia sp. | PR | 5 | (B) |
| Helopelopia sp. | PR | 6 | (B) |
| Krenopelopia sp. | PR | 7 | (B) |
| Labrundinia sp. | PR | 6 | (L,B) |
| Labrundinia pilosella | PR | 7 | B |
| Larsia sp. | PR | 7 |  |
| Meropelopia sp. | PR | 6 | (B) |
| Meropelopia americana | PR | 6 | (B) |
| Meropelopia flavifrons | PR | 6 | (B) |
| Monopelopia sp. | PR | 7 | (B) |
| Natarsia sp. | PR | 8 | B |
| Nilotanypus sp. | PR | 6 | B |
| Paramerina sp. | PR | 6 | B |
| Pentaneura sp. | PR | 6 | B |
| Procladius sp. | PR | 9 | L,B |
| Procladius culiciformis | PR | 99 |  |
| Psectrotanypus sp. | PR | 10 | (L,B) |
| Psectrotanypus dyari | PR | 10 | L,B |
| Rheopelopia sp. | PR | 4 | B |
| Tanypus sp. | PR | 10 | L |
| Telopelopia sp. | PR | 99 |  |
| Thienemannimyia sp. | PR | 6 | B |
| Thienemannimyia gr. | PR | 6 | B |
| Thienemannimyia senata | PR | 6 | (B) |
| Trissopelopia sp. | PR | 4 |  |
| Zavrelimyia sp. | PR | 8 | B |
| Culicidae | FC | 99 |  |
| Anopheles sp. | FC | 9 | L |
| Culex sp. | FC | 10 | L |
| Culex pipiens | FC | 99 |  |
| Dixidae |  | 99 |  |
| Dixa sp. | FC | 1 |  |
| Dolichopodidae | PR | 4 | B |
| Empididae | PR | 6 | (B) |
| Chelifera sp. | PR | 6 | B |
| Clinocera sp. | PR | 6 | B |
| Dolichocephala sp. | PR | 6 | (B) |
| Hemerodromia sp. | PR | 6 | B |
| Oreogeton sp. | PR | 6 |  |
| Ephydridae | SH | 6 | (B) |
| Muscidae | PR | 6 | H |
| Limnophora sp. | PR | 7 | L |
| Phoridae |  | 99 |  |
| Psychodidae | GC | 10 | H |
| Ptychopteridae |  | 99 |  |
| Scathophagidae | SH | 6 |  |
| Sciomyzidae |  | 99 |  |
| Dictya sp. |  | 99 |  |
| Dictya pictipes |  | 99 |  |
| Sepedon sp. |  | 99 |  |
| Simuliidae | FC | 6 | H |
| Cnephia sp. | FC | 4 |  |
| Cnephia dacotensis | FC | 4 |  |
| Prosimulium sp. | FC | 2 |  |
| Simulium sp. | FC | 5 | B |
| Simulium fibrinflatum | FC | 6 | B |
| Simulium gouldingi | FC | 3 | B |
| Simulium jenningsi | FC | 4 | B |
| Simulium nyssa | FC | 5 |  |
| Simulium tuberosum | FC | 4 |  |
| Simulium tuberosum complex | FC | 4 |  |
| Simulium venustum | FC | 5 | B |
| Simulium venustum complex | FC | 5 | B |
| Simulium verecundum cplx. | FC | 5 | N |
| Simulium vittatum | FC | 7 | B |
| Simulium vittatum complex | FC | 9 |  |
| Stratiomyidae |  | 99 |  |
| Syrphidae | GC | 10 | H |
| Eristalis sp. | GC | 10 |  |
| Tabanidae | PR | 6 | H |
| Chrysops sp. | GC | 5 | B |
| Tanyderidae |  | 99 |  |
| Protoplasa fitchii | GC | 5 | L |
| Tipulidae | SH | 5 | N |
| Antocha sp. | GC | 3 | B |
| Dicranota sp. | PR | 3 | B |
| Eriocera sp. |  | 99 |  |
| Helius sp. | GC | 4 | B |
| Hexatoma sp. | PR | 2 | B |
| Hexatoma (Eriocera) spinosa | PR | 2 | (B) |
| Limonia sp. | SH | 6 | B |
| Molophilus sp. | SH | 3 |  |
| Ormosia sp. | GC | 4 | (B) |
| Paradelphomyia sp. | PR | 5 | (B) |
| Pedicia sp. |  | 99 |  |
| Pilaria sp. | PR | 7 | B |
| Pseudolimnophila sp. | SH | 3 |  |
| Tipula sp. | SH | 6 | B |

* See “Taxa List Explanations,” next page.

Taxa List Explanantions

FFG—functional feeding group.  This is a charcterization of the named organism’s predominant mode of feeding.  The codes stand for:

- GC—gathering collector

- FC—filtering collector

- SC—scraper

- SH—shredder

- PR—predator

TV—tolerance value.  Based on Hilsenhoff’s (1987) 10 point rating of tolerance to pollution, each taxon is assigned a value from 0 to 10.  The most sensitive invertebrates are assigned a “0” and the most tolerant ones receive a 10.

TVr—tolerance value reference.  These identify the sources for the tolerance values used for taxa in the list.

- B—Bode, R.W., M.A. Novak, and L.E. Abele.  1996. Quality assurance work plan for biological stream monitoring in New York State.  NYS Dept. of Environmental Conservation.  Albany, NY.  89 p.

- H—Hilsenhoff, W.L. 1987.  An improved biotic index of organic stream pollution. Great Lakes Entomologist 20:31-39.

- L—Lenat, D.R.  1993.  A biotic index for the southeastern United States: derivation and list of tolerance values, with criteria for assigning water quality ratings. J.N. Am. Benthol. Soc.  12(3):279-290.

- ( )—cited reference assigned the value to a different taxonomic level (e.g., genus) and is extrapolated to this taxon (e.g., family).

APPENDIX D

Bench Sheets

WATERSHED NAME:						WATERSHED CODE:

SLIDE BOX: ______ of ______					SURVEY DATE:

| SLOT/ STATION | COVER/TAXA | COMMENTS | SLOT/ STATION | COVER/TAXA | COMMENTS |
| --- | --- | --- | --- | --- | --- |
| ____/________ | A/______________________ A/______________________ A/______________________ B/______________________ B/______________________ B/______________________ |  | ____/________ | A/______________________ A/______________________ A/______________________ B/______________________ B/______________________ B/______________________ |  |
| ____/________ | A/______________________ A/______________________ A/______________________ B/______________________ B/______________________ B/______________________ |  | ____/________ | A/______________________ A/______________________ A/______________________ B/______________________ B/______________________ B/______________________ |  |
| ____/________ | A/______________________ A/______________________ A/______________________ B/______________________ B/______________________ B/______________________ |  | ____/________ | A/______________________ A/______________________ A/______________________ B/______________________ B/______________________ B/______________________ |  |
| ____/________ | A/______________________ A/______________________ A/______________________ B/______________________ B/______________________ B/______________________ |  | ____/________ | A/______________________ A/______________________ A/______________________ B/______________________ B/______________________ B/______________________ |  |
| ____/________ | A/______________________ A/______________________ A/______________________ B/______________________ B/______________________ B/______________________ |  | ____/________ | A/______________________ A/______________________ A/______________________ B/______________________ B/______________________ B/______________________ |  |

Mollusca

Plecoptera

Megaloptera

Trichoptera

Coleoptera

Diptera (Chironomidae spp. on back)

Other Insecta

Other Invertebrata

Gastropoda

Pelecypoda

Annelida

Oligochaeta

Hirudinea

Crustacea

Isopoda

Amphipoda

Decapoda

Hydrachnidia

Insecta

Ephemeroptera

Odonata

Chironominae

Chironomini

Tanytarsini

Diamesinae

Orthocladiinae

Tanypodinae

Total chironomid count: