| Monitoring Method Guidance CN 0.72 (January 2020) | data submittal guidelines & EXTERNAL Data review PROCESS For Clean water act 305(b)/303(d) reporting purposes Massachusetts Department of Environmental Protection Division of Watershed Management- Watershed Planning Program 8 New Bond St., Worcester, MA. 01606; 508-792-7470 |  |
| --- | --- | --- |

Purpose:  To provide guidance to non-DEP groups regarding the submittal of quality-assured environmental monitoring data and supporting information to the Massachusetts Department of Environmental Protection, Division of Watershed Management (DWM), Watershed Planning Program (WPP), and to provide information on DWM-WPP’s review procedures for such “external” data.

Background:  In addition to using data collected internally, DWM-WPP often uses quality-assured data from other groups to assess waterbody health and develop cleanup plans for impaired waterbodies.   In order to be considered usable by DWM-WPP for these purposes, these data must meet certain submittal requirements (as explained below) AND undergo detailed review to help evaluate the accuracy, precision and representativeness of the data.  Potential external data providers include, but are not limited to, grant recipients; local, state and/or Federal agencies; environmental consultants and volunteer monitoring organizations.

When to submit data:  For grant programs, please check the reporting requirements listed in the grant contract.

For MassDEP’s CWA 305(b) and 303(d) (Integrated List) assessment and reporting purposes, submittals of surface water quality or quantity data are welcome at any time. However, check the deadlines (listed here https://www.mass.gov/guides/external-data-submittals-to-the-watershed-planning-program) for submittals to be considered for inclusion in the current assessment cycle. Data submitted after the dates listed, including during the public comment period, will not be considered for the current assessment cycle, but will be reviewed and considered in the subsequent cycle. MassDEP’s goal is to submit Integrated Reports to EPA in the Spring of even-numbered years.

What to submit:  Guidance, templates and forms identified below (in #2, #3 and #5) can be found here: https://www.mass.gov/guides/external-data-submittals-to-the-watershed-planning-program.

- Cover letter or email.

- Statement of Data Integrity.  See WPP’s external data web page (link provided above).

- Data Files.   Quality-assured monitoring data must be sent electronically using DWM-WPP’s data submittal template, available via WPP’s external data web page (link provided above).  Data sent using other formats may not be reviewed for usability by DWM-WPP.  The Excel data file(s) can be sent via e-mail, acceptable file transfer protocol or CD.  Data file(s) should include quality control data. Graphic displays are optional.

- Copy of Approved QAPP.

- Summary Report (optional).   In addition to the final data submittal, a summary report may also be provided.  Suggested content for data reports can be found on the external data web page (link provided above).

- Supporting documentation (as appropriate).  Other information supporting the data may also be provided (or may be requested by WPP).

Where to submit data:   Electronic data files can be sent via e-mail to the following address:  WQData.Submit@mass.gov. For regular mail delivery, information can be sent to the following address:

External Data Coordinator

Massachusetts Department of Environmental Protection

Bureau of Water Resources, Division of Watershed Management, Watershed Planning Program

8 New Bond St., Worcester, MA. 01606

Pre-requisite guidelines for submittal of data for potential use in DWM-WPP’s waterbody assessments and TMDLs (Clean Water Act, Sections 305(b) and 303(d)):

- Monitoring data shall be generated through implementation of a DEP- or EPA-approved Quality Assurance Project Plan (QAPP) or Adoption of the Bacterial Monitoring General QAPP with approved Sampling and Analysis Plan (SAP).

The project QAPP shall follow applicable DEP and/or EPA guidance for monitoring QAPPs.  Example QAPP guidance is provided here: https://www.mass.gov/guides/environmental-monitoring-for-volunteers .Approved QAPPs shall include appropriate documentation from the analytical laboratory to be used, such as their current Quality Assurance Plan (QAP) and Standard Operating Procedures (SOPs), as well as project protocols for sample collection, training, quality control sampling and data management. Stated project objectives should be consistent with DWM’s use of data for waterbody assessment and/or TMDL development purposes.

- Analytical data shall be provided by an analytical laboratory certified by the Commonwealth of Massachusetts in the applicable analyses, or a laboratory with a documented and acceptable Quality Assurance Plan (QAP), as well as documented and acceptable Standard Operating Procedures (SOPs).

Use of a State-certified laboratory for all sample analyses is highly recommended, but is not always possible.  Lab data and metadata generated through the use of non-certified labs may receive a higher level of scrutiny than those from certified labs.   If not already provided in the QAPP, the data report should provide certified/non-certified laboratory quality assurance information (lab QAP, lab SOPs, contacts, etc.).  A list of State-certified labs is available at: https://www.mass.gov/certified-laboratories.

- Quality-assured data (and metadata) shall be provided in sufficient detail for DWM-WPP to evaluate the usability of the data.

QC data should be provided in electronic format using DWM-WPP’s standard template (https://www.mass.gov/guides/external-data-submittals-to-the-watershed-planning-program).  All data should include the appropriate metadata (i.e., sampling and analytical information related to the data).  Graphic and textual data analyses are optional.

DWM-WPP’s external data review process:   Quality-assured data from non-DEP groups are reviewed by DWM-WPP using the following general criteria and best professional judgment to evaluate their potential for use by DWM-WPP.  All data submittals should be citable.  Submittal of data does not guarantee its use by DWM-WPP.

- Overall clarity, organization and detail of the data submittal.

- Overall precision, accuracy, representativeness, comparability and completeness of the data, in comparison to QAPP data quality objectives and DWM-WPP data quality needs.

- Estimated accuracy of lab analyses, using Quality Control/Performance Evaluation (QC/PE) samples, spiked sample matrices, and positive/negative controls (for bacteria samples), as compared to project DQOs.

- Overall evaluation of QAPP implementation (i.e., documentation of actual QC measures to ensure data quality, such as the frequency of instrument calibration and maintenance, problem identification and response, and personnel training)

- Method consistency/variability among project participants and over time throughout the duration of the project.

- Evaluation of any field audit information, side-by-side sampling results and/or inter-laboratory QC audit information, if available, to assess inter-group and/or inter-lab precision.

- Availability of personal communication with project lead(s) and/or QC officer(s), if needed.

A standard external data review form is used for all DWM-WPP reviews. The data usability assessment begins with assembling all available information from the submittal, which may include data reports, data files, QC information, email, etc.  For information deemed missing, the contact for the external data group is contacted to see if the information is available and can be sent. The initial preliminary review determines if the recommended pre-requisites, as identified above, were met.

The subsequent detailed review involves reviewing the data in more detail, including the following, if available, and as appropriate:

  - Consistency and accuracy of all data elements (e.g. site descriptions, GPS locations, reported units, use of data qualifiers).

  - Likeliness of data (e.g. no pH readings of <0 or >14), or explanation of outliers.

  - Analytical holding time violations

  - Frequency of QC samples (blank and duplicates) taken for each survey, and compare to QAPP

  - Field blank sample results to verify lack of contamination

  - Field duplicate sample results to verify acceptable precision

  - Laboratory records (lab notebooks, lab bench sheets, if available) for potential effects on data quality, including multi-probe calibration books for potential effects on data quality

  - Quality control results contained in laboratory data reports for potential implications to data quality (based on lab accuracy and precision data), and lab analytical performance during survey period based on results of any QC/PE testing

  - Miscellaneous documentation (training records, e-mails, phone records, pers. comms., etc.) to highlight any potential problems affecting data quality

  - Overall quality of other data, as available (e.g., benthic macroinvertebrates, fish toxics, other “biological” data)

Communication with data providers regarding data completeness, missing information and other questions takes place as necessary.

Based on the review (and any follow-up), conclusions regarding the usability of the data, as a whole and/or by components, are documented on the data review form, and become the basis for DWM-WPP’s use or non-use of the submitted data.  Based on the review, data are categorized as Level 1, 2 or 3.  Some or all of the data deemed to be Level 3 (potentially suitable for use in waterbody assessments) by WPP can be accepted, accepted with caveat/qualification and/or not used, depending on the circumstances.