README for Site_Suitability_Criteria_Index_Data_Layers

This directory (Site_Suitability_Criteria_Index_Data_Layers) contains GIS raster data layers necessary to calculate Site Suitability scores for the Agricultural Resources, Biodiversity, and Carbon Sequestration and Storage criteria, as described in the Commonwealth of Massachusetts’ guidance for Site Suitability Assessments for Clean Energy Infrastructure. These are also data layers used in the Clean Energy Infrastructure Site Suitability Mapping Tool. The following describes the included subdirectories, raster data layers, and how they were derived.

Agricultural Resources Index

Subdirectory: AgResourcesIndex

GIS File: ARI.tif

Description: The Agricultural Resources Index (ARI) is an indicator of a location’s agricultural resources value based on soils that are particularly well-suited for agricultural production and land cover indicating active use as farmland. The ARI is used to calculated derived from the following data sources:

- National Resource Conservation Service’s Prime Farmland Soils categories for Massachusetts.

- The most recent year of land cover data from the U.S. Geological Survey’s Annual National Land Cover Database (NLCD). This version of ARI is based on 2024 NLCD land cover.

The ARI is determined for each 30-meter grid cell across Massachusetts based on the following table:

| Farmland Soils Category | Land Cover (Annual NLCD, 2024) | Agricultural Resources Index 0.0 = lowest agricultural value 5.0 = highest agricultural value |
| --- | --- | --- |
| Any | Developed | 0.0 |
| None | Other Undeveloped | 0.0 |
| Farmland of Unique Importance | Other Undeveloped | 1.0 |
| Farmland of Statewide Importance | Other Undeveloped | 2.0 |
| None | Agriculture (Cultivated Crops, Pasture/Hay) | 2.5 |
| Prime Farmland Soils | Other Undeveloped | 3.0 |
| Farmland of Unique Importance | Agriculture (Cultivated Crops, Pasture/Hay) | 3.0 |
| Farmland of Statewide Importance | Agriculture (Cultivated Crops, Pasture/Hay) | 4.0 |
| Prime Farmland Soils | Agriculture (Cultivated Crops, Pasture/Hay) | 5.0 |

Site Suitability scoring: Site Suitability scores for the Agricultural Resources criteria are calculated by:

- Identifying the ARI values of all grid cells with centers falling inside a Site Footprint; and

- Calculating the average of the top 50% of these ARI values.

Biodiversity Index

Subdirectory: BiodiversityIndex

GIS File: BI.tif

Description: The Biodiversity Index (BI) is an indicator of a location’s habitat and biodiversity conservation value based primarily on BioMap, the Commonwealth’s biodiversity conservation mapping initiative. BI is derived from the following data sources:

- BioMap Core Habitat, Critical Natural Landscape, Regional Connectivity, and Local Components designations.

- Priority Habitats designated by the MassWildlife Natural Heritage and Endangered Species Program (NHSEP).

- Index of Ecological Integrity values from the UMass Conservation Assessment Prioritization System (CAPS).

The BI is determined for each 30-meter grid cell across Massachusetts based on the following table:

| Biodiversity Criteria | Biodiversity Index 0.0 = lowest biodiversity value 5.0 = highest biodiversity value |
| --- | --- |
| Not in BioMap Core Habitat, Critical Natural Landscapes, Regional Connectivity, Local Components, or NHESP Priority Habitat | 0.0 – 2.0, proportional to CAPS Index of Ecological Integrity |
| In BioMap Regional Connectivity or Local component and not in Critical Natural Landscapes, Core Habitat or NHSEP Priority Habitat | 2.5 |
| In BioMap Critical Natural Landscape and not in Core Habitat or NHSEP Priority Habitat | 3.5 |
| In BioMap Core Habitat and not in NHSEP Priority Habitat | 4.5 |
| In NHESP Priority Habitat | 5.0 |

Site Suitability scoring: Site Suitability scores for the Biodiversity criteria are calculated by:

- Identifying the BI values of all grid cells with centers falling inside a Site Footprint; and

- Calculating the average of the highest 25% of these BI values.

Ecosystem Carbon Index

Subdirectory: EcosystemCarbonIndex

GIS File: ECI.tif

Description: The Ecosystem Carbon Index (ECI) is an indicator of a location’s carbon storage and sequestration value based on estimates of current forest carbon stocks, future forest carbon sequestration potential (50 year, no disturbance projections), and recent land cover indicating ecosystems with carbon storage potential. The ECI is derived from the following data sources:

- Total Ecosystem Carbon in 2070 data from the National Forest Carbon Monitoring System (NFCMS).

- The most recent year of land cover data from the U.S. Geological Survey’s Annual National Land Cover Database (NLCD). This version of ARI is based on 2024 NLCD land cover.

The ECI is determined for each 30-meter grid cell across Massachusetts as follows:

| Land Cover (Annual NLCD, 2024) | Total Ecosystem Carbon (NFCMS Total Ecosystem Carbon in 2070) | Ecosystem Carbon Index - = lowest carbon value 5.0 = highest carbon value |
| --- | --- | --- |
| Developed (excluding Developed Open Space) | Assumed to be minimal (regardless of NFMCS data) | 0.0 |
| Undeveloped (including Developed Open Space) | <200 MgCO2e/acre or “NoData” in NFMCS data | 1.0 |
| Undeveloped (including Developed Open Space) | ≥200 MgCO2e/acre | 2.0 to 5.0, rescaled from NFCMS Total Ecosystem Carbon in 2070 |

Site Suitability scoring: Site Suitability scores for the Carbon Storage and Sequestration criteria are calculated by:

- Identifying the ECI values of all grid cells with centers falling inside a Site Footprint; and

- Calculating the average of these ECI values.

This will use the Massachusetts state version of IEI, with values quantile rescaled to the 0.0 to 2.0 range after removing all areas overlapping BioMap and NHESP areas designated above.

In here means the grid cell center is inside a polygon in the designated data layer.

Values from the NFCMS Total Ecosystem Carbon in 2070 data layer have been rescaled to the 2.0 to 5.0 range after removing all areas with developed landcover or < 200 MgCO2e/acre.