52-AC-1

- TEN MILE RIVER BASIN

- 1997 WATER QUALITY ASSESSMENT REPORT

- COMMONWEALTH OF MASSACHUSETTS

- EXECUTIVE OFFICE OF ENVIRONMENTAL AFFAIRS

BOB DURAND, SECRETARY

MASSACHUSETTS DEPARTMENT OF ENVIRONMENTAL PROTECTION

LAUREN A. LISS, COMMISSIONER

BUREAU OF RESOURCE PROTECTION

ARLEEN O’DONNELL, ASSISTANT COMMISSIONER

DIVISION OF WATERSHED MANAGEMENT

GLENN HAAS, DIRECTOR

- NOTICE OF AVAILABILITY

- LIMITED COPIES OF THIS REPORT ARE AVAILABLE AT NO COST BY WRITTEN REQUEST TO:

MASSACHUSETTS DEPARTMENT OF ENVIRONMENTAL PROTECTION

DIVISION OF WATERSHED MANAGEMENT

627 MAIN STREET

WORCESTER, MA  01608

This report is also available from the Department of Environmental Protection, Division of Watershed Management’s home page on the World Wide Web at:

http://www.state.ma.us/dep/brp/wm/wmpubs.htm

NOTE: For web publication, corrections to the printed version of this assessment report are highlighted in purple.

Furthermore, at the time of first printing, eight copies of each report published by this office are submitted to the State Library at the State House in Boston; these copies are subsequently distributed as follows:

- On shelf; retained at the State Library (two copies);

- Microfilmed retained at the State Library;

- Delivered to the Boston Public Library at Copley Square;

- Delivered to the Worcester Public Library;

- Delivered to the Springfield Public Library;

- Delivered to the University Library at Umass, Amherst;

- Delivered to the Library of Congress in Washington, D.C.

Moreover, this wide circulation is augmented by inter-library loans from the above-listed libraries.  For example a resident in Winchendon can apply at their local library for loan of any DEP/DWM report from the Worcester Public Library.

A complete list of reports published since 1963 is updated annually and printed in July.  This report, entitled, “Publications of the Massachusetts Division of Watershed Management – Watershed Planning Program, 1963-(current year)”, is also available by writing to the DWM in Worcester.

- DISCLAIMER

References to trade names, commercial products, manufacturers, or distributors in this report constituted neither endorsement nor recommendations by the Division of Watershed Management for use.

- TEN MILE RIVER BASIN

- 1997 WATER QUALITY ASSESSMENT REPORT

- Prepared by:

- Laurie E. Kennedy, Robert J. Maietta, and John F. Fiorentino

- Department of Environmental Protection

Division of Watershed Management

- Report Number:

- 52-AC-1

- DWM Control Number:

- 18.0

- Massachusetts Department of Environmental Protection

Division of Watershed Management

Worcester, Massachusetts

March 2000

ACKNOWLEDGEMENTS

Coordination of local, state and federal agencies and private organizations is fundamental to the success of the Massachusetts Watershed Initiative.  The Environmental Protection Agency (EPA) New England Regional Laboratory, provided much support in the way of sediment quality sampling, toxicity testing and physico-chemical analysis.

Much appreciation is also extended to several DEP employees for their contributions:  Jane Colonna-Romano, Juliet Mathers, Mollie Weinstein, Mark Guilmain, Tom Dallaire, Rick McVoy, Arthur Screpetis and Dave Burns.

It is impossible to thank everyone who contributed to the assessment report process:  field, laboratory, data management, writing, editing, and graphics, as well as meetings, phone calls, and many emails.  These contributions are very much appreciated.

# TABLE OF CONTENTS

TABLE OF CONTENTS	i

LIST OF ACRONYMS	ii

EXECUTIVE SUMMARY	iv

TEN MILE RIVER BASIN 1997 WATER QUALITY ASSESSMENT REPORT	iv

RIVERS	iv

LAKES	vi

INTRODUCTION	1

ASSESSMENT METHODOLOGY	1

TEN MILE RIVER BASIN DESCRIPTION and CLASSIFICATION	10

OBJECTIVES	11

SEGMENT REPORT FORMAT	12

RIVER SEGMENT ASSESSMENTS	13

TEN MILE RIVER (Segment MA52-01)	14

TEN MILE RIVER  (Segment MA 52-02)	17

SCOTTS BROOK  (SEGMENT MA52-09)	21

TEN MILE RIVER (Segment MA 52-03)	23

BUNGAY RIVER  (Segment MA 52- 06)	29

SPEEDWAY BROOK (Segment MA52- 05)	33

SEVENMILE RIVER (Segment MA52-07)	36

FOURMILE BROOK (Segment MA52-10)	39

SEVENMILE RIVER (Segment MA52-08)	41

COLES BROOK (Segment MA52-11)	43

LAKE/POND ASSESSMENTS	45

LAKE USE ASSESSMENTS	46

SUMMARY	47

RECOMMENDATIONS	47

REFERENCES	50

APPENDIX A – DEP DWM QA/QC	A1

APPENDIX B– WATER RESOURCE MONITORING	B1

LIST OF TABLES	B1

LIST OF FIGURES	B1

MATERIALS AND METHODS	B2

Survey Conditions	B2

Stream Water Quality Monitoring	B4

Benthic Macroinvertebrates and Habitat Assessments	B4

Periphyton	B4

Fish Population	B5

Fish Toxics	B5

Lakes/Ponds	B7

RESULTS	B7

Survey Conditions	B7

Stream Water Quality Monitoring	B8

Benthic Macroinvertebrates and Habitat Assessments	B14

Periphyton	B21

Fish Population	B21

Fish Toxics	B21

Lakes/Ponds	B24

REFERENCES	B25

APPENDIX C – DEP BIOMONITORING TECHNICAL MEMORANDUM	C1

# LIST OF ACRONYMS

| 7Q10 | seven day ten year low flow |  |
| --- | --- | --- |
| ACEC | Areas of Critical Environmental Concern |  |
| BPJ | Best Professional Judgement |  |
| CFS | cubic feet per second |  |
| CMR | Code of Massachusetts Regulations |  |
| CNOEC | chronic no observed effect concentration |  |
| CWA | Clean Water Act |  |
| DEM | Department of Environmental Management |  |
| DEP | Department of Environmental Protection |  |
| DFWELE | Department of Fisheries, Wildlife, and Environmental Law Enforcement |  |
| DMR | Discharge Monitoring Report |  |
| DO | Dissolved oxygen |  |
| DWM | Department of Watershed Management |  |
| EPA | United States Environmental Protection Agency |  |
| GIS (MASSGIS) | Geographic Information System |  |
| GP(M) (D) | Gallons per (minute) (day) |  |
| LC50 | lethal concentration to 50% of the test organisms |  |
| MDC | Metropolitan District Commission |  |
| MMDPH | Massachusetts Department of Public Health |  |
| MEPA | Massachusetts Environmental Policy Act Unit |  |
| NCCW | non-contact cooling water |  |
| NH3-N | ammonia-nitrogen |  |
| NPDES | National Pollutant Discharge Elimination System |  |
| NPS | non point source |  |
| NTU | nephelometric turbidity units |  |
| ORW | Outstanding Resource Waters |  |
| PAH | polyaromatic hydrocarbons |  |
| PALIS | Pond And Lake Information System |  |
| PCB | polychlorinated biphenols |  |
| PPM | parts per million |  |
| PWS | Public Water Supply |  |
| QA/QC | Quality Assurance/ Quality Control |  |
| RBP | Rapid Bioassessment Protocol |  |
| SARIS | Stream and River Inventory System |  |
| SEC | Sprague Electric Company |  |
| SS | suspended solids |  |
| SU | standard units |  |
| SWQS | Massachusetts Surface Water Quality Standards |  |
| TMDL | total maximum daily load |  |
| TOC | total organic carbon |  |
| TRC | total residual chlorine |  |
| USFDA | United States Food and Drug Administration |  |
| USGS | United States Geological Survey |  |
| VOC | volatile organic compound |  |
| WMA | Water Management Act |  |
| WPCF | Water Pollution Control Facility |  |
| WWTP | Waste Water Treatment Plant |  |

Figure 1.  1997 DEP DWM water quality, macroinvertebrate, and fish toxics monitoring station data summary in the Ten Mile River Basin.

# EXECUTIVE SUMMARY

# TEN MILE RIVER BASIN 1997 WATER QUALITY ASSESSMENT REPORT

The assessment of current water quality conditions is a key step in the successful implementation of the watershed approach.  This critical phase provides basic information for focusing resource protection and remediation activities to be executed later in the watershed management planning process.  Detailed information regarding individual river segments and lakes in the Ten Mile River Basin is presented in this assessment report.  The following summary provides an overview of the status of water quality conditions in the Ten Mile River Basin based on information available to the Department of Environmental Protection’s Division of Watershed Management (DEP DWM) as of February 2000.

## Ten mile RIVER basin  -  RIVERS

This report presents the status of 32.8 river miles in the Ten Mile River Basin representing the Ten Mile River and all major tributaries.  These include Scotts Brook; the Bungay River; Speedway Brook; the Sevenmile River and its tributary, Fourmile Brook; and Coles Brook.  Smaller unnamed tributaries to these streams remain unassessed.

###### AQUATIC LIFE USE

The Aquatic Life Use is met when suitable habitat (including water quality) is available for sustaining a native, naturally diverse, community of aquatic flora and fauna. The assessment status of this use is derived primarily from benthic community data (Figure 1) and to a lesser extent fisheries and  sediment data. The designation of the Aquatic Life Use as impaired (non or partial support) results from a combination of stressors including both point and nonpoint sources of pollution.  The overall Aquatic Life Use status of the 32.8 river miles in the Ten Mile River Basin (mainstem and tributaries) discussed in this report is as follows:

Aquatic Life Use Summary

- 3.0 	 miles Full Support

- 6.1 	 miles Partial Support

- 18.8 miles Non Support

- 4.9 	 miles Not Assessed

None of the 14.5 river miles of the mainstem Ten Mile River supports the Aquatic Life Use.  The river partially supports the Aquatic Life Use upstream of West Bacon Street in Plainville and downstream of Dodgeville Pond in Attleboro. The Aquatic Life Use is not supported in the mainstem between these locations.  Causes and sources of impairment upstream of West Bacon Street are unknown.   Siltation and other forms of habitat alteration, as well as sediment contamination (particularly metals) impair the Aquatic Life Use between West Bacon Street and the North Attlborough WWTP discharge.  Downstream from the discharge, instream impacts to the benthic macroinvertebrate community attributable to the discharge were also documented.  Organic enrichment/low DO was evident in the mainstem Ten Mile River between the North Attleborough WWTP and Dodgeville Pond.  Downstream of Dodgeville Pond, instream dissolved oxygen and saturation levels were higher but dropped again as the river flowed into Seekonk.  Slight impairment of the benthic community was documented below the Attleboro WPCF discharge as the Ten Mile River entered Pawtucket, Rhode Island.  Downstream from Dodgeville Pond, the mainstem Ten Mile River was assessed as partially supporting the Aquatic Life Use.  Other causes of impairment to the biota in the Ten Mile River include unknown toxicity and sediment toxicity (Wetherells and Dodgeville Ponds) from municipal discharges (North Attleborough WWTP and Attleboro WPCF) and contaminated sediments.  Other unknown causes and sources of impairment may also exist.

Tributaries represent 18.3 of the 32.8 river miles in the Ten Mile River Basin.  Of these the Aquatic Life Use was impaired along 10.4 tributary miles.   Additional data would be required to assess the Aquatic Life Use in both the Bungay River and Fourmile Brook.

The major cause of impairment to the Aquatic Life Use in the tributaries of the Ten Mile River was attributed to low flow conditions.  Three stream segments - the upper Sevenmile River (Class A) segment, Scotts Brook, and Coles Brook - totaling 9.5 river miles, were adversely impacted presumably from either water withdrawals or regulation of impoundments (hydromodification).  Other unknown causes and sources of impairment may also exist.

Habitat degradation was observed in the form of sediment deposition and substrate embeddedness directly attributed to road runoff, construction activities, riparian zone disturbance, and other forms of NPS pollution in Speedway Brook impairs the Aquatic Life Use.  Although the Aquatic Life Use was not assessed in the Fourmile Brook, habitat degradation in the form of riparian zone disturbances was observed.

FISH CONSUMPTION USE

The Fish Consumption Use is met when pollutants do not result in unacceptable concentrations in edible portions of marketable fish or shellfish or for the use of recreationally caught fish, shellfish, other aquatic life or wildlife for human consumption.  This use is assessed using the Department of Public Health’s Fish Consumption Advisory List.

In June 1986, the Massachusetts Department of Public Health (MDPH) issued a Fish Consumption Advisory for the Ten Mile and Sevenmile Rivers due to elevated concentrations of lead in edible fillets.  Based on subsequent sampling data generated in 1987 and 1990, the MDPH rescinded their 1986 advisory in July 1994.  However, it should be noted that because of risks associated with elevated levels of mercury in certain species of freshwater fish, the MDPH has issued a statewide Interim Freshwater Fish Advisory as of September 1994.  The interim advisory recommends that “Pregnant women should be advised of the possible health risk from eating fish taken from Massachusetts freshwater bodies in order to prevent exposure of developing fetuses to mercury.”   The advisory does not include stocked trout or farm-raised fish sold commercially.   The Fish Consumption Use for the entire Ten Mile River Basin (32.8 river miles) included in this report is summarized as follows:

Fish Consumption Use Summary

32.8 river miles Not Assessed.   (There are no site specific MDPH fish consumption advisories in place in the Ten Mile River Basin.)

RECREATIONAL USES

The Primary Contact Recreational Use is defined as any recreation or other water activity during which there is prolonged and intimate contact with the water with a significant risk of ingestion. These include, but are not limited to, wading, swimming, diving, surfing and water skiing.  The Secondary Contact Recreational Use is defined as any recreation or other water use during which contact with the water is either incidental or accidental.  These include, but are not limited to, fishing, boating and limited contact incident to shoreline activities.  The overall support status (data summaries in Figure 1) for the entire Ten Mile River Basin  (32.8 river miles) included in this report are summarized as follows:

| Primary Contact Recreational Use Summary | Secondary Contact Recreational Use Summary |
| --- | --- |
| - 0.0 miles Full Support | - 0.0 miles Full Support |
| - 4.3 miles Partial Support | - 4.3 miles Partial Support |
| - 3.9 miles Non Support | - 0.9 miles Non Support |
| - 24.6 miles Not Assessed | - 27.6 miles Not Assessed |

Too little current data exists to assess any stream in the Ten Mile River Basin as full support for either the Primary or Secondary Contact Recreational Use.  Elevated fecal coliform densities were initially documented in Coles Brook during the DEP DWM Ten Mile River Basin survey.  Follow-up investigations, funded through the Massachusetts Watershed Initiative (MWI) Grant Program in a project entitled An Assessment of Non-point Source Pollution in the Coles Brook Subwatershed of the Ten Mile River Basin (Fennessey 1999), confirmed the presence of elevated fecal coliform bacteria in Coles Brook under wet weather conditions.  As a result, the Primary and Secondary Contact Recreational Uses are assessed as partial support in Coles Brook.  Additional monitoring will be required to isolate sources of bacteria.  Two other tributaries of the Ten Mile River, the upper segment of the Sevenmile River (Class A waterbody) and Speedway Brook, both had elevated fecal coliform densities that impaired both recreational uses.  Both the Primary and Secondary Contact Recreational Uses were not supported in Speedway Brook, while only the Primary Contact Recreational Use was not supported in the upper Sevenmile River (Class A) segment.  The source(s) of fecal coliform bacteria inputs to these streams are unknown at this time.

### AESTHETICS USE

The Aesthetics Use is met when surface waters are free from pollutants in concentrations or combinations that settle to form objectionable deposits; float as debris, scum or other matter to form nuisances; produce objectionable odor, color, taste or turbidity; or produce undesirable or nuisance species of aquatic life. The overall Aesthetics Use status (data summary, Figure 1) of the 32.8 river miles in the Ten Mile River Basin discussed in this report follows:

Aesthetics Use Summary

- 8.5 miles Full Support

- 12.9 miles Partial Support

- 1.2 miles Non Support

- 10.2 miles Not Assessed

The Aesthetics Use is partially supported in the mainstem Ten Mile River downstream from West Bacon Street in Plainville (12.9 river miles).  This use is impaired primarily from turbidity and noxious aquatic plants, particularly in the impounded (lakes) reaches of the river.  Sediment deposition and anthropogenic “trash” deposits also degrade the aesthetic quality in isolated areas.  Two tributaries, Speedway Brook and the lower 0.3 miles of the Bungay River, do not support the Aesthetics Use.  The cause of the impairment is siltation and turbidity in Speedway Brook, and turbidity from a waterfowl staging area (duck feeding pond) in the Bungay River near its confluence with the Ten Mile River.

## Ten mile RIVER basin  -   LAKES

Overall use support status and trophic status of the lakes surveyed in the Ten Mile River Basin are presented in Tables 1 and 2, respectively.  It should be noted that lakes or portions of lakes were listed as undetermined when obvious impairments were not readily observable.  With this approach, the assessment of lakes in the Ten Mile River Basin is limited to a "best case" picture (i.e., only the most obvious impairments are reported).  Potentially more of the lake acreage would be listed as impaired or in a more enriched trophic status if more variables were measured and more criteria assessed.  Surveys also focused on the presence or absence of non-native macrophytes.  Three non-native aquatic species and two non-native wetland species were observed.

Table 1.  Ten Mile River Basin lakes use support summary (in acres).

| USE/DEGREE SUPPORTED | SUPPORT | PARTIAL SUPPORT | NON SUPPORT | NOT ASSESSED | NOT ATTAINABLE |
| --- | --- | --- | --- | --- | --- |
| AQUATIC LIFE | 0 | 128 | 0 | 881 | 40 |
| FISH CONSUMPTION* | 0 | 0 | 0 | 1009 | 40 |
| SWIMMABLE | 0 | 121 | 112 | 776 | 40 |
| SECONDARY CONTACT | 776 | 121 | 112 | 0 | 40 |
| AESTHETICS | 776 | 121 | 112 | 0 | 40 |

*Fish Consumption results do not include the statewide consumption advisory.

N.B. - These results represent the most recent assessments of lakes/ponds in the Ten Mile River Basin.  These data also represent about 52% (22 of 42) of the lakes/ponds in the Ten Mile River Basin and about 80% (1,049 of 1,315) of the acreage.

Table 2.  Ten Mile River Basin lakes trophic status summary.

| TROPHIC STATUS | NUMBER OF LAKES | ACRES |
| --- | --- | --- |
| Oligotrophic | 0 | 0 |
| Mesotrophic | 3 | 234 |
| Eutrophic | 6 | 115 |
| Hypereutrophic | 8 | 364 |
| Dystrophic | 0 | 0 |
| Undetermined/ Not Attainable | 5 | 336 |
| Total | 22 | 1049 |

N.B. - These results represent the most recent assessments of lakes/ponds in the Ten Mile River Basin.  These data also represent about 52% (22 of 42) of the lakes/ponds in the Ten Mile River Basin and about 80% (1,049 of 1,315) of the acreage.

.

Despite the "best case" scenario that is favored by the Ten Mile River Basin lake assessment approach, 64 % (representing 46 % of the surveyed area) of the lakes showed symptoms of advanced succession (eutrophic or hypereutrophic).  Presumably additional testing of dissolved oxygen, chlorophyll, and/or nutrients would corroborate these trophic status conditions.

Three non-native aquatic plant species, fanwort, variable water milfoil, and Eurasian water milfoil were found infrequently in the lakes of the Ten Mile River Basin.  They are, however, particularly invasive species that spread vegetatively via cuttings that may float downstream or be transported mechanically between lakes.

The most frequently occurring non-native weltand species was purple loosestrife.  Populations of this plant are pervasive throughout the watershed.  Its presence was recorded at 82% of the lakes.  The other non-native wetland plant observed in the Ten Mile River Basin is the common reed grass.  The two non-native wetland species were co-located at five lakes (south basin of Falls Pond in North Attleborough, Fuller Pond in Plainville, Ten Mile Reservation Pond in Attleboro/Pawtucket, RI, Wetherells Pond in Plainville, and Whitings Pond in North Attleborough).

Intentionally left blank

TEN MILE RIVER BASIN

1997 DEP-DWM WATER QUALITY ASSESSMENT REPORT

Figure 1.  1997 DEP DWM water quality, macroinvertebrate, and fish toxics monitoring station data summary in the Ten Mile River Basin.