# REFERENCES

Burns, D.  2000.  Personal Communication.  North Attleborough WWTP and Attleboro WPCF Permit Compliance. Massachusetts Department of Environmental Protection, Bureau of Resource Protection, Southeast Regional Office.  Lakeville, MA

Bode, R. and M. Novak. 1998. Differences in Environmental Preferences of Sister Species of Chironomidae. Presented at the 22nd Annual meeting of the New England Association of Environmental Biologists. Kennebunkport, ME.

Couto, S. 2000.  Letter March 9, 2000 to DeLorenzo/Monte of MA DEP Re: Quarterly Compliance Meeting, March 2, 2000. United States Environmental Protection Agency. Boston, MA.

Dallaire, T.D. 1999.   Water Body Identification. Clean Water Act Section 305(b), Water Body System Database – (WBS).  Foxpro Application.  1999.  Massachusetts Department of Environmental Protection, Division of Watershed Management, Watershed Planning and Permitting Program, Database Manager.  Worcester, MA.

Dallaire, T.D. 2000.   “NPDES Toxicity testing discharge monitoring report summary”. Toxicity Testing Database -- TOXTD.  Dbase Application.  Massachusetts Department of Environmental Protection, Division of Watershed Management, Watershed Planning and Permitting Program, Database Manager.  Worcester, MA.

MDPH.  1998.  Freshwater Fish Consumption Advisory List.  Massachusetts Department of Public Health, Bureau of Environmental Health Assessment.  Boston, MA.

Environmental Law Reporter. 1988. Clean Water Deskbook. Environmental Law Institute. Washington, DC.

EOEA.  1996.  Ten Mile River Watershed Brief.  Executive Office of Environmental Affairs. Boston, MA.

EOEA. 1997. MassGIS statewide 1:25,000 land use datalayer. Environmental Data Center. Executive Office of Environmental Affairs. Boston, MA.

EOEA. 1999. MassGIS datalayers. Environmental Data Center. Executive Office of Environmental Affairs. Boston, MA.

EPA. 1991. Updates from EPA/ORD: Predicting Metal Toxicity in Sediments. U. S. Environmental Protection Agency, Environmental Research Laboratories. Narragansett RI.

EPA. 1997. Guidelines for Preparation of the Comprehensive State Water Quality Assessments (305(b) Reports) and Electronic Updates: Report Contents.  U. S. Environmental Protection Agency.  Assessment and Watershed Protection Division (4503F). Office of Wetlands, Oceans and Watersheds. Office of Water. Washington D.C.

Fennessey, N.M.  1999.  Assessment of Nonpoint Source Pollution in Coles Brook Sub-watershed of the ten Mile River Basin.  Final Report.  University of Massachusetts-Dartmouth, Hydrology and Water Resources group.  Dartmouth, MA.

Halliwell D.B., W.A. Kimball, and A.J. Screpetis.  1982.  Massachusetts Stream Classification Program Part I:  Inventory of Rivers and Streams.  Massachusetts Division of Fisheries and Wildlife, Department of Fisheries, Wildlife, and Recreational Vehicles and Massachusetts Division of Water Pollution Control, Department of Environmental Quality Engineering. Westborough, MA.

Hellyer, G. 1999a. Personal Communication. Ten Mile River Sediment/Water Quality Data .  U. S. Environmental Protection Agency,  Region I Laboratory, Lexington, MA.

Hellyer, G. 1999b. Project Work/QA Plan Ten Mile River Sediment/Water Quality Assessment. U. S. Environmental Protection Agency, Region I Laboratory, Lexington, MA.

Horton, J.  2000.  Personal Communication.  North Attleborough WWTP Implementation of Dechlorination.  North Attleborough WWTP Chief Operator, North Attleborough, MA.

LeVangie, D. 1997.  Water Management Act Database.  1999.  Massachusetts Department of Environmental Protection, Division of Watershed Management, Database Manager.  Boston, MA.

- MA DEM.  1993 June.  Areas of Critical Environmental Concern (ACEC) Program Guide June 1993.  Commonwealth of Massachusetts, Executive Office of Environmental Affairs, Department of Environmental Management, ACEC Program.  Boston, MA.

MA DEP.  1996. (Revision of 1995 report).  Massachusetts Surface Water Quality Standards.  Massachusetts Department of Environmental Protection, Division of Water Pollution Control, Technical Services Branch.  Westborough, MA  (Revision of 314 CMR 4.00, effective June 23, 1996).

MA DEP. 1997.  1997 Synoptic Lake Surveys in the Ten Mile River Basin. Unpublished data.  Massachusetts Department of Environmental Protection, Division of Watershed Management. Worcester, MA.

MA DEP.  1998. Quality Assurance Project Plan for 1997 Water, Fish Tissue, Habitat and Biological Studies in the Ten Mile River Watershed. Massachusetts Department of Environmental Protection, Division of Watershed Management.  Worcester, MA.

MA DEP.  1999a.  Open NPDES and WMA permit files.  Massachusetts Department of Environmental Protection, Division of Watershed Management. Worcester, MA.

MA DEP.  1999b.  Open NPDES permit files.  Massachusetts Department of Environmental Protection, Bureau of Resource Protection. Lakeville, MA.

MA DEQE.  1985.  Ten Mile River Basin 1984 Water Quality Data Part A: River Data. Massachusetts Department of Environmental Quality Engineering, Division of Water Pollution Control, Technical Services Branch. Westborough, MA.

Maietta, R.J. 1998.  Personal Communication. Ten Mile River Basin Field Observations.   Massachusetts Department of Environmental Protection, Division of Watershed Management. Worcester, MA.

Miller, A.  2000.  Personal Communication.  Fortifiber, Corporation Facility Information. Massachusetts Department of Environmental Protection, Bureau of Resource Protection, Southeast Regional Office.  Lakeville, MA.

Nicholson, P.  2000.  Personal Communication. Attleborough WPCF Sewer Service Area and Facility Plan. Attleborough WPCF Chief Operator, Attleborough, MA.

Persaud, D., R. Jaagumagi, and A. Hayton.  1993.  Guidelines for the protection and management of aquatic sediment quality in Ontario.  Water Resources Branch, Ontario Ministry of the Environment.  Queen’s Printer for Ontario. Canada.

Plafkin, J.L., M.T. Barbour, K.D. Porter, S.K. Gross, R.M. Hughes.  1989.  Rapid bioassessment protocols for use in streams and rivers: benthic macroinvertebrates and fish.  U.S. EPA, Washington, DC.  EPA/444/4-89-001.

Rojko, A. M., W. A. Kimball, and A. J. Screpetis.  1995.  Designated Outstanding Resource Waters of Massachusetts 1995.  Massachusetts Executive Office of Environmental Affairs, Department of Environmental Protection, Bureau of Resource Protection, Office of Watershed Management.  Grafton, MA.

Socolow, R.S., Leighton, C.R., Zanca, J.L. and Ramsbey, L.R.  1998.  Water Resources Data for Massachusetts and Rhode Island, Water Year 1997.  U.S. Geological Survey Report MA-RI-97-1.

Szal, G.M.,  1993.  Memorandum for the record: North Attleborough National Fish Hatchery Discharge of Formalin. .  Massachusetts Executive Office of Environmental Affairs, Department of Environmental Protection, Bureau of Resource Protection, Office of Watershed Management.  Grafton, MA.

USEPA. 19 November 1999.  Federal Register Document. [Online]. United States Environmental Protection Agency. http://www.epa.gov/fedrgstr/EPA-WATER/1998/December/Day-10/w30272.htm.

Whitman & Howard, Inc., 1993.  Letter March 31, 1993 to the Board of Water and Sewer Commisioners of Plainville: Prolonged Pumping Test of Well No. 3.  Whitman & Howard, Inc., Wellesley, MA.

Young, D.  2000.  Personal Communication.  Mantrose-Haeuser, Inc. Current Facility Operations.  Mantrose-Haeuser, Inc. Plant Engineer. Attleboro, MA.

# APPENDIX A –DEP DWM QA/QC

Introduction

Quality Assurance/Quality Control (QA/QC) activities were conducted as part of the DEP DWM Ten Mile River Basin Monitoring Survey in 1997.  This QA/QC review was conducted to ensure that the collection and analysis of the monitoring data was of high quality.  The 1997monitoring data subjected to this QA/QC review includes the following: discrete water samples, fish tissue samples and in-situ water quality measurements.  All discrete water sample and fish tissue monitoring data were reviewed independently by the Wall Experiment Station’s (WES) Quality Assurance Program and the Division of Watershed Management’s  (DWM) Quality Assurance Officer and Assessment Coordinator. All in-situ water quality measurements were reviewed independently by DWM’s Hydrolab® Instrument Coordinator and Database Manager.  Data that fell outside established QA/QC acceptance criteria were investigated and may have been subject to censoring. This Quality Assurance/Quality Control appendix is divided into three sections: A.1 field and laboratory data objectives; A.2 QA/QC data; A.3 analytical methods.

A.1 Field and Laboratory QA/QC Objectives

Data collected by DWM in the 1997 Ten Mile River Basin survey was subject to field and laboratory data quality objectives.  Section A.1.1 outlines the field collection objectives and laboratory quality control for discrete water samples.  Section A.1.2 includes fish tissue laboratory quality control methods and Section A.1.3 includes Hydrolab QA/QC procedures.

A.1.1	Discrete Water Sample Data

FIELD

The collection of discrete water sample analytes followed DWM Standard Operating Procedures (1,2).  Four field collection quality control criteria were applied to the Ten Mile River Basin 1997 discrete water sample data:

1.0	Sampling/Analysis Holding Time: Each analyte has a standard holding time that has been established to ensure sample/analysis integrity.  Refer to DWM Standard Operating Procedure Table 1.0 CN# 1.0 (2) for a complete listing.  If the standard holding time was exceeded, this objective is violated.

2.0	Quality Control Sample Frequency: At a minimum, one field blank and one replicate must be collected for every ten samples by any given sampling crew on any given date. If less than one quality control sample per 10 field samples was collected, this objective is violated.

- Field Blank: Field blanks were prepared at the DWM Worcester Office.  Reagent grade water was transported into the field where it was transferred into a sample container and fixed using the same method as its corresponding field sample.   All blanks were submitted to WES laboratory “blind”.  If the field blanks were significantly different (>2 standard deviations (9)) from the detection limit, this data quality objective is violated.

4.0	Field Replicate: Two independent samples were collected from the same location and as close as possible to the same time in the field.  Both samples were submitted to WES laboratory “blind”.  In order for this data quality objective to be met, the results must be:

<20% Relative Percent Difference (RPD) for method detection limits >1mg/L

<30% RPD for method detection limits <1mg/L

A detailed QA/QC summary of the four data quality objectives and additional DWM quality assurance observations for the 1997 Ten Mile River Basin data can be found in the 1997 Watershed QA/QC Assessment Report (8).

LABORATORY

Discrete water sample analysis followed EPA-approved laboratory QA/QC methodologies in accordance with WES Standard Operating Procedures (3).  The quality of data generated at WES was determined by analyzing the results of a variety of quality control procedures including but not limited to:

Low Calibration Standards – Checks the stability of the instrument’s calibration curve. Analyzes the accuracy of an instrument’s calibration within a 5% range.

Reference Standards  – Generally, a second source standard (a standard different from the calibration stock standard) that analyzes the accuracy of an instrument’s calibration within a 5% range.

Laboratory Reagent Blank/Method Blank (LRB) – Reagent grade water (de-ionized) extracted with every sample set to ensure that the system is free of target analytes (< MDL).

Duplicate Sample – Measures the precision (% Relative Percent Difference) of the extraction and analytical process.  The acceptable laboratory %RPD range is typically  25%.

Spike Sample (Laboratory Fortified Blank - LFB, Laboratory Fortified Matrix - LFM)– Measures the accuracy (% Recovery) of an analytical method.  The acceptable laboratory % recovery range is typically between 80 – 120% for LFB samples and 70 –130% for LFM discrete water samples.

The WES Laboratory is solely responsible for the administration of its Quality Assurance Program and Standard Operating Procedures.  The frequency of the laboratory’s quality control procedure was at times inconsistent with their  Quality Assurance Plan (3).   In these circumstances additional quality assurance procedures were used.  Refer to WES’s Quality Assurance Plan (3) for specific laboratory analytical QA/QC criteria.  WES laboratory releases discrete water sample data when their established QA/QC criteria are met or the data are labeled as outside of these criteria.

A.1.2	Fish Tissue Data

Fish were collected and processed according to DWM’s Quality Assurance Project Plan (4). Tissue preparation and analysis strictly adhered to EPA-approved laboratory QA/QC methodologies in accordance with WES Standard Operating Procedures (6,7).  The quality of tissue data generated at WES was determined by incorporating a variety of quality control samples:

Laboratory Reagent Blank/Method Blank (LRB) – Clean clam tissue matrix extracted with every sample set to ensure that the system is free of target analytes (< MDL).

Laboratory Fortified Blank (LFB) – Clean clam tissue matrix spiked with a low concentration of target compounds.  LFB results are used to establish accuracy of system’s performance.  The acceptable laboratory % recovery range is typically 80 – 120%.

Laboratory Fortified Matrix (LFM) – Tissue matrix spiked with a low concentration of a target compound.  LFM results are used to establish accuracy of the extraction and analytical process.  The acceptable laboratory % recovery range is typically between 70 – 130% for metal analysis and 60 –140% for PCB/organochlorine pesticide analysis.

Quality Control Standard (QCS) – A pre-spiked secondary tissue sample.  QCS results are used to establish accuracy in the extraction and test methods.  The acceptable laboratory  % recovery range is typically between 80–120%.

The WES Laboratory is solely responsible for the administration of its Quality Assurance Program and Standard Operating Procedures.  The frequency of the laboratory’s quality control procedure was at times inconsistent with their Quality Assurance Plan (3).   In these circumstances additional quality assurance procedures were used.  Refer to WES’s Quality Assurance Plan (3) for specific laboratory analytical QA/QC criteria.  WES laboratory releases tissue data when their established QA/QC criteria are met or the data are labeled as outside of these criteria.

A.1.3	In-situ Water Quality Analysis

Trained DWM staff members conducted in-situ measurements using a Hydrolab® Multiprobe Series 3 analyzer.  The Hydrolab® Multiprobe Series 3 analyzer measures dissolved oxygen, temperature, pH, conductivity, depth and turbidity and calculates total dissolved solids and % saturation of oxygen.  To ensure the quality of the in-situ data, the following QA/QC steps were taken:

1.0	Pre-Calibration: After each analytical probe on the Hydrolab® analyzer was calibrated, a pre-calibration check was conducted.  A low ionic standard was first analyzed to check the accuracy of the instrument.  Then an instrument check consisting of de-ionized water was analyzed to check the instrument for contamination.  The instrument check criteria is based on de-ionized water that that had been stored and vented to the air for at least three days.  If the pre-calibration check achieved the criteria in Table A.1-1 HL-1 then the instrument was ready for field analysis but if the pre-calibration check failed to achieve the low ionic standard criteria than the instrument was re-calibrated and a second low ionic and instrument check was analyzed.  If the instrument failed to meet the established low ionic standard criteria a second time the Hydrolab® instrument could not be used to collect data and maintenance was scheduled. Refer to the DWM Hydrolab® Standard Operating Procedure (5).

- Post Survey Check: Once the Hydrolab® was returned from field sampling, a post survey check was performed to ensure that no malfunction or damage had occurred to any of the Hydrolab® probes.  The low ionic standard and the instrument check were re-analyzed.  If the post survey check achieved the established criteria in Table A.1-1 HL-1, the data was deemed acceptable and was ready for the data reduction QA/QC step.  If, however, the post calibration failed to meet the criteria, the Hydrolab® Coordinator investigated the cause and recommended censoring of affected data to the Database Manager.

3.0	Data Reduction: The Hydrolab® Coordinator and Database Manager reviewed the Hydrolab® data for instability, instrument malfunction, operator technique and aberrant trends.  If any of these conditions were detected, the data was investigated and may have been recommended for censoring.  The Database Manager electronically tagged all data recommended for censoring in the database.

Table A.1-1.  Hydrolab Multiprobe Series 3 analyzer pre and post calibration specifications.

| Hydrolab® Analyte | Low-Ionic Standard | Instrument Check * |
| --- | --- | --- |
| Dissolved Oxygen | Saturation Chart (dependant on temperature & barometric pressure ) |  |
| pH | 6.90 ±1% | 5.6 ±0.2 units |
| Specific Conductance | 74 ±1% | 1.0 ±1% |
| Turbidity | 0.0 ±5% | 0.0 ±5% |
| Temperature | Ambient ±0.15°C** | Ambient ±0.15°C** |
| Depth | Field Calibrated ±0.45m | Field Calibrated ±0.45m |
| Salinity | Not Applicable | 0.0 ±0.2ppt |
| Redox | Not Applicable | 0.0±20mV |

* Based on Division of Watershed Management’s filtered de-ionized water

** Compared to the DWM laboratory’s wall thermometer

###### REFERENCES

(1)	MA DEP.  1999. CN 1.0 Grab Collection Techniques for DWM Water Quality Sampling 1999.  Massachusetts Department of Environmental Protection, Division of Watershed Management.  Worcester, MA.

(2)	MA DEP.  1999. CN 1.0 Grab Collection Techniques for DWM Water Quality Sampling 1999.  Massachusetts Department of Environmental Protection, Division of Watershed Management.  Worcester, MA.

(3)	MA DEP.  1995.  Laboratory Quality Assurance Plan and Standard Operating Procedures, Appendix B and C.  January 1995. Massachusetts Department of Environmental Protection, Division of Environmental Analysis, Senator William X. Wall Experiment Station.  Lawrence MA.

(4)	MA DEP.  1999.  CN 13.0 Fish Contaminant Monitoring Program Quality Assurance Project Plan, 1999. Massachusetts Department of Environmental Protection, Division of Watershed Management.  Worcester, MA.

(5)	MA DEP.  1999.  CN 4.0 Hydrolab® Multiprobe Series 3 and Appendixes CN 4.1 – 4.5, 1999. Massachusetts Department of Environmental Protection, Division of Watershed Management.  Worcester, MA.

(6)	MA DEP.  1995.  Laboratory Quality Assurance and Standard Operating Procedures, “Wet Tissue Digestion for Metals Analysis by Atomic Absorption Spectroscopy and/or ICP Emission Spectroscopy (Fish, Clams, Mussels, Etc.)”, January 1995. Massachusetts Department of Environmental Protection, Division of Environmental Analysis, Senator William X. Wall Experiment Station.  Lawrence MA.

(7)	MA DEP.  1995.  Laboratory Quality Assurance and Standard Operating Procedures, AOAC Method 983.21 “PCBs and Organochlorine Pesticides in Biological Tissue”, January 1995. Massachusetts Department of Environmental Protection, Division of Environmental Analysis, Senator William X. Wall Experiment Station.  Lawrence MA.

(8)	MA DEP.  1999.  CN 9.0 1997/98 Watershed QA/QC Assessment Report , 1999. Massachusetts Department of Environmental Protection, Division of Watershed Management.  Worcester, MA.

(9)	Clesceri, L.S., A.E. Greenberg, and A.D. Eaton, (editors).  Standard Methods for the Examination of Water and Wastewater, 20th Edition 1998, American Public Health Association, Washington, D.C.  Section 1010B “Statistics”, pg. 1-2 and 1-3.

### A.2 QA/QC Data

Field blank and replicate sampling results for the discrete bacteriological water quality sampling are provided in Tables A.2-1 and A.2-2.  Tables A.2-3 and A.2-4 contain laboratory QA/QC data for organics in tissue analyses and metals in tissue analyses, respectively.

Table A.2-1.  1997 DEP DWM Ten Mile River Basin instream bacteriological QA/QC field blank data.  (Units expressed in colonies/100ml.)

Time	FECAL	E-COLI	ENTEROCOCCUS	AEROMONAS

(24hr)

Field Blank Sample

52-0010	BLANK	07/01/97	6:32	<20	<20	<20	<100

52-0021	BLANK	07/01/97	6:55	<20	<20	<20	<100

52-0049	BLANK	08/05/97	**	**  	--  	--  	--

52-0060	BLANK	08/05/97	**	**  	--  	--  	--

52-0091	BLANK	09/03/97	**	<20	<20	--  	--

52-0102	BLANK	09/03/97	**	<20	<20	--  	--

Table A.2-2. 1997 DEP DWM Ten Mile River Basin instream bacteriological QA/QC field replicate data.  (Units expressed in colonies/100ml,.data log10 transformed).

Time	FECAL	E-COLI	ENTEROCOCCUS	AEROMONAS

(24hr)

TEN MILE RIVER,  Station: TM06

52-0004	52-0005	07/01/97	5:07	2.204	2.000	2.447	6.079

52-0005	52-0004	07/01/97	5:07	2.255	1.778	2.342	5.079

Relative Percent Difference (RPD):	2.3%	11.7%	4.4%	17.9%

52-0043	52-0044	08/05/97	5:13	**  	--  	--  	--

52-0044	52-0043	08/05/97	5:13	**  	--  	--  	--

Relative Percent Difference (RPD):

52-0085	52-0086	09/03/97	5:10	1.778	1.903	--  	--

52-0086	52-0085	09/03/97	5:10	1.778	1.778	--  	--

Relative Percent Difference (RPD):	0.0%	6.8%

TEN MILE RIVER,  Station: TM13

52-0013	52-0014	07/01/97	4:53	2.380	2.079	2.380	6.176

52-0014	52-0013	07/01/97	4:53	2.380	2.000	1.903	6.000

Relative Percent Difference (RPD):	0.0%	3.9%	22.3%	2.9%

52-0052	52-0053	08/05/97	4:40	**  	--  	--  	--

52-0053	52-0052	08/05/97	4:40	**  	--  	--  	--

Relative Percent Difference (RPD):

52-0094	52-0095	09/03/97	5:07	2.380	2.079	--  	--

52-0095	52-0094	09/03/97	5:07	2.380	2.000	--  	--

Relative Percent Difference (RPD):		    0.0%		  3.9%

* = interference           ** = missing/censored data          -- = no data

A.3	Analytical Methods

Discrete Water Sample Analytes			EPA Method*	SM Methods**	Other Methods

Fecal Coliform							SM 9222D

E. Coli, MTEC							SM 9213D

Enterococcus							SM 9230C

Aeromonas Hydrophilia						SM 9260L

Fish Tissue Analytes

PCB Arochlor 1242								AOAC 983.21***

PCB Arochlor	1254									“

PCB Arochlor	1260									“

Chlordane										“

Toxaphene										“

a-BHC											“

b-BHC											“

Lindane											“

d-BHC											“

Hexachlorocyclopentadiene									“

Trifluralin										“

Hexachlorobenzene										“

Heptachlor										“

Heptachlor Epoxide									“

Methoxychlor										“

DDD											“

DDE											“

DDT											“

Aldrin											“

Arsenic	 (STGFAA)				EPA 200.9	SM 3113

Lead (ICP)					EPA 200.7	SM 3120B

Selenium (STGFAA)				EPA 200.9	SM 3113

Cadmium (ICP)					EPA 200.7	SM 3120B

Mercury (cold vapor)				EPA 245.1	SM 3112B

* =  “Methods for Chemical Analysis of Water and Wastes”, Environmental Protection Agency, Environmental Monitoring Systems Laboratory – Cincinnati (EMSL-CI), EPA-600/4-79-020, Revised March 1983 and 1979 where applicable.

** = Standard Methods, Examination of Water and Wastewater, 20th edition

***= PCBs and Organochlorine Pesticides in Biological Tissue, AOAC Official Methods of Analysis, 1990

# APPENDIX B WATER RESOURCE MONITORING

APPENDIX B WATER RESOURCE MONITORING	BError: Reference source not found

LIST OF TABLES	BError: Reference source not found

LIST OF FIGURES	BError: Reference source not found

MATERIALS AND METHODS	BError: Reference source not found

Survey Conditions	BError: Reference source not found

Stream Water Quality Monitoring	BError: Reference source not found

Benthic Macroinvertebrates and Habitat Assessments	BError: Reference source not found

Periphyton	BError: Reference source not found

Fish Population	BError: Reference source not found

Fish Toxics	BError: Reference source not found

Lakes/Ponds	BError: Reference source not found

RESULTS	BError: Reference source not found

Survey Conditions	BError: Reference source not found

Stream Water Quality Monitoring	BError: Reference source not found

Benthic Macroinvertebrates and Habitat Assessments	BError: Reference source not found

Periphyton	BError: Reference source not found

Fish Population	BError: Reference source not found

Fish Toxics	BError: Reference source not found

Lakes/Ponds	BError: Reference source not found

REFERENCES	BError: Reference source not found

LIST OF TABLES

Table B1.   1997 Ten Mile River Basin Surveys DEP-DWM sampling matrix.	B2

Table B2.   1997 DEP DWM Ten Mile River Basin in-situ Hydrolab data.	B8

Table B3.   1997 DEP DWM Ten Mile River Basin bacteria data	B11

Table B4.   1997 DEP DWM Ten Mile River Basin macroinvertebrate biomonitoring station locations	B14

Table B5.   1997 DEP DWM Ten Mile River Basin: Species-level taxa list and counts, functional feeding groups, and tolerance values for macroinvertebrates.	B15

Table B6.   1997 DEP DWM Ten Mile River Basin: RBP III data analysis for macroinvertebrate communities at 9 stations compared to the regional reference station (SM00).	B18

Table B7.   1997 DEP DWM Ten Mile River Basin: RBP III data analysis for macroinvertebrate communities at 9 stations compared to the regional reference station (TM01)..	B19

Table B8.   1997 DEP DWM Ten Mile River Basin: RBP III data analysis for macroinvertebrate communities at 2 stations.  Station TM06a and stationTM06.	B20

Table B9.   1997 DEP DWM Ten Mile River Basin habitat assessment summary.	B20

Table B10. 1997 DEP DWM Ten Mile River Basin fish population data.	B22

Table B11. 1997 DEP DWM Ten Mile River Basin periphyton population and abundance data.	B23

Table B12. 1997 DEP DWM Ten Mile River Basin fish toxics monitoring data.	B23

Table B13. 1997 DEP DWM Ten Mile River Basin lake survey data.	B24

LIST OF FIGURES

Figure B1.  DEP DWM 1997 water quality monitoring stations.  USGS gaging station..	B3

Figure B2.  DEP DWM 1997 benthic macroinvertebrate monitoring stations.  Fish contaminant monitoring stations...	B6

MATERIALS AND METHODS

The DWM sampling began in July 1997 and continued through October 1997.  The DWM sampling matrix is summarized in Table B1.  Sampling components at river stations included: in situ Hydrolab measurements, fecal coliform bacteria sampling, biological community (benthic macroinvertebrate, fish and periphyton) sampling, and toxics in fish flesh.  Synoptic surveys of lakes were conducted during August 1997 to coincide with the maximum extent of macrophyte growth. Each sampling component is described in the sections that follow.

Table B1.  1997 Ten Mile River Basin Surveys DEP-DWM sampling matrix.

| STATION | 1997 July | 1997 August | 1997 September | 1997 October |
| --- | --- | --- | --- | --- |
| TM01 | H, B | H, B | H, B, M |  |
| TM02 |  |  | M, F |  |
| TM04 | H, B | H, B | H, B, M |  |
| TM05 |  |  |  | F |
| F0044 (Falls Pond) |  |  | F2 |  |
| TM06 | H, B | H, B | H, B, M, F |  |
| TM06A |  |  | M, F |  |
| TM07 | H, B | H, B | H, B |  |
| TM08A | H, B | H, B | H, B |  |
| TM11 | H, B | H, B | H, B, M |  |
| TM12 | H ,B | H, B | H, B | F |
| TM13 | H, B | H, B | H, B |  |
| TM14 | H, B | H, B | H, B, M |  |
| SM00 | H, B | H, B | H, B, M, F |  |
| SM01 | H. B | H, B | H, B |  |
| SM02 |  |  |  | F |
| BG01 | H, B | H, B | H, B |  |
| BG02 | H, B | H, B | H, B |  |
| SW01 | H, B | H, B | H, B |  |
| SW01M |  |  | M |  |
| FM01 | H, B | H, B | H, B, QM |  |
| CB01 | H, B | H | H, B |  |
| SB01 |  | H, B |  |  |

B= Bacteria (fecal coliform, E. coli); H= Hydrolab multiprobe meter (pH, temperature, dissolved oxygen, conductivity, total dissolved solids); M= Macroinvertebrate kick sampling and habitat analysis (RBP III) and periphyton sampling; QM= Qualitative macroinvertebrate sampling, F = Fish population sampling via electrofishing, F2 = Toxics in fish tissue (Cd, Pb, Hg, As, Se, % lipids, organochlorine pesticides).

## Survey Conditions

Conditions prior to each synoptic survey were characterized by analyzing precipitation and streamflow data.  One weather station precipitation gage was used to determine precipitation and weather conditions for five days prior to and on the sampling dates: West Street, Attleboro Station #801: data for this station was provided by the DEM Office of Water Resources (MA DEM 1998).  Discharge (hereinafter referred to as streamflow) and duration data was obtained from the only continuous USGS stream gage in the basin (Figure B1), Ten Mile River at Pawtucket Avenue at East Providence, RI (01109403).  The data from this gage was used to calculate streamflow characteristics for the period of record.  These statistical analyses can be found in Water Resources Data Massachusetts and Rhode Island, Water Year 1997 (Socolow et al. 1998), and the Gazetteer of Hydrologic Characteristics of Streams in Massachusetts—Taunton and Ten Mile River Basins and Coastal River Basins of Mount Hope Bay, Narragansett Bay, and Rhode Island Sound (Wandle and Keezer 1984).  The period of record for the Ten Mile River gage is from October 1986 to present.  The provisional 7-day, 10-year (7Q10) low flow was provided by USGS (1998).

Wrentham

Plainville

Foxborough

Mansfield

Attleboro

North

Attleborough

Figure B1.  Location of DEP DWM 1997 water quality monitoring stations and the USGS gaging station in the Ten Mile River Basin.

Seekonk

Rehoboth

Figure 1.  Location of 1997 DEP DWM water quality monitoring stations and the USGS gaging station in the Ten Mile River Basin.

## Stream Water Quality Monitoring

- Water quality sampling was conducted at the stations identified in Figure B1.  Synoptic water quality sampling at these locations included the following: in-situ measurements using a Scout 3 Hydrolab multiparameter meter (water temperature, dissolved oxygen, conductivity, total dissolved solids, pH and in some cases turbidity) and bacteria sampling (fecal coliform, E. coli, Enterococcus and Aeromonas).  Bacteria samples were collected from the Ten Mile, Sevenmile and Bungay rivers, and Coles, Scotts and Fourmile brooks to assess bacterial contamination during low flow conditions.

- Procedures used for water sampling and sampling handling are described in the Basin Program Standard Operating Procedures (MA DEP 1989).  The Wall Experiment Station (WES), the Department’s analytical laboratory, supplied all sample bottles and field preservatives, which were prepared according to the WES Laboratory Quality Assurance Plan and Standard Operating Procedures (MA DEP 1994).  Samples were preserved in the field as necessary, transported on ice to WES, and analyzed according to the WES SOP.  The quality control protocol that was followed for field and equipment blank samples is described in Appendix A of this report (Ten Mile River Basin 1997 Water Quality Assessment Report).  Both quality control samples (field blanks, trip blanks, and split samples) and raw water quality samples were transported on ice to WES on each sampling date; they were analyzed subsequently according to the WES SOP (MA DEP 1994).

## Benthic Macroinvertebrates and Habitat Assessments

Macroinvertebrate biomonitoring was conducted by DEP DWM biologists based on EPA’s Rapid Bioassessment Protocol III (RBP III) (Plafkin et al. 1989). The macroinvertebrate collection procedure utilizes kick sampling, a method of sampling benthic organisms by kicking or disturbing bottom sediments and catching the dislodged organisms downstream with an aquatic net.  An integral component of the RBP III method is the assessment of available habitat quality.  Physical characteristics of the stream substrate, channel morphology, and the structural stability of the stream banks are all evaluated.  RBP III sampling was conducted at a total of 9 stations in the Ten Mile River Basin (Figure B2). Seven stations were located along the mainstem Ten Mile River while one station each was located on the Sevenmile River and Speedway Brook.  Additionally, one station on Fourmile Brook was sampled qualitatively (similar to the RBP I sampling methodology outlined in Plafkin et al. 1989).  Most stations had been sampled previously by DEP: 1984 (Johnson et al. 1986) and/or 1990 (provided as Appendix C of this report).

Methods used to evaluate the benthic macroinvertebrate data followed those outlined in Plafkin et al. (1989).  Habitat assessment methods followed those described in the MA DEP Preliminary Biological Monitoring and Assessment Protocols for Wadeable Rivers and Streams Method 004 Revision #1 (TetraTech, Inc. 1995).

## Periphyton

Periphyton samples were also collected in the vicinity of the macroinvertebrate biomonitoring locations by the DEP DWM biologists during the summer of 1997.  Periphyton samples were collected at a total of ten stations, seven along the mainstem Ten Mile River and one each on the Sevenmile River, Speedway Brook, and Fourmile Brook (Figure B2).  The samples were collected in order to learn more about the biota in the Ten Mile River Basin, to document problem areas, to offer a means of comparing biological communities, to examine community changes and to provide a record of taxa found in Massachusetts.

Periphyton samples were collected by scraping natural substrates (cobble and/or boulders) within the riffle zone and the adjacent run or pool in the vicinity of the macroinvertebrate sampling stations. The algal material was washed into a labeled vial containing site water. Samples were kept on ice and brought back to the laboratory for examination.

Each vial was shaken to get a uniform sample before subsampling.  If filamentous algae comprised most of the sample they were removed first, identified separately and then the remainder of the sample was examined.  Samples were examined using an Olympus microscope with Nomarski optics and identified to genus when possible. Estimation of relative abundance was made according to a method described in Bahls (1993), which assigns categories from rare to very abundant for the algae based on the numbers of cells per field.

With the exception of the designations C and VC being combined and referred to as C, the scheme developed by Bahls for determining abundance is as follows:

R    (rare)		fewer than one cell per field of view at 200x, on the average;

C    (common)		at least one, but fewer than five cells per field of view;

VC  (very common)	between 5 and 25 cells per field;

A    (abundant)		more than 25 cells per field, but countable;

VA  (very abundant)	number of cells per field too numerous to count.

## Fish Population

DEP DWM biologists conducted fish population surveys in the Ten Mile River Basin during September and October 1997.  Five stations were located along the mainstem Ten Mile River and one station was located on the Sevenmile River.  Surveys were conducted using techniques similar to Rapid Bioassessment Protocols V (fish) as described by Plafkin (1989).  Surveys also included a habitat assessment component.

Fish populations were sampled by electroshocking using a Smith Root Model 12 battery powered backpack electrofisher. A reach of approximately 100m was sampled by passing a pole mounted anode ring, side to side through the stream channel and in and around likely fish holding cover.  All fish shocked were netted and held in buckets. Sampling proceeded from an obstruction or constriction, upstream to an endpoint at another obstruction or constriction such as a waterfall or shallow riffle.  Following completion of a sampling run, all fish were identified to species, counted, and released.  Methods used to

evaluate the fish data collected during this survey were similar to those outlined in Protocol V (Plafkin et al. 1989).

## Fish Toxics

Uniform protocols, designed to assure accuracy and prevent cross-contamination of samples, were followed for collecting, processing and shipping fish collected for fish toxics monitoring from the north basin of Falls Pond.  Fish were collected from the north basin of Falls Pond, North Attleborough on 9 September 1997 using a Coffelt electrofishing boat (Figure B2).  Fish were collected as the boat was maneuvered through the littoral habitat in the north basin of Falls Pond and were placed in a live well filled with site water.  Fish included in the sample were removed from the live well, placed in an ice-filled cooler and brought back to the laboratory for sample processing.  The remaining fish were released.

Lengths and weights were measured and fish were visually inspected for tumors, lesions, or other indications of stress or disease.  Fish were then filleted on glass cutting boards, the skin was removed, and samples were prepared for freezing. All equipment used in the filleting process was rinsed in tap water to remove slime, scales, and blood, then re-rinsed twice in de-ionized water before and/or after each individual fish or composite. Fillets targeted for metals analyses were placed in VWR 32 ounce high-density polyethylene cups with covers. The opposite fillets were wrapped in aluminum foil for % lipids, PCB and organochlorine pesticide analysis. In the case of composite samples, three fillets from like-sized individuals of the same species were wrapped together in aluminum foil or stored in a single sample container. Samples were tagged and frozen for subsequent delivery to WES.

Methods used at WES for metals analysis include the cold vapor method using a VGA hydride generator for mercury and Varian 1475 flame atomic absorption for all remaining metals (MA DEP 1994).

PCB/organochlorine pesticide analysis was performed on a gas chromatograph equipped with an electron capture detector. Additional information on analytical techniques used at WES is available from the laboratory.

Mansfield

Attleboro

North

Attleborough

Seekonk

Rehoboth

Figure B2.  Location of DEP DWM 1997 benthic macroinvertebrate and fish contaminant monitoring stations in the Ten Mile River Basin.

Wrentham

Plainville

Foxborough

Mansfield

North Attleborough

Attleboro

Seekonk

Rehoboth

## Lakes/Ponds

Synoptic surveys of 22 lakes, ponds, reservoirs were conducted during July and September 1997.  Synoptic surveys consist of taking observations from at least one access point on each lake (multiple access points on larger lakes).  At each lake, an attempt was made to observe the entire surface area to determine the extent of areal macrophyte cover.

At each observation site the general water quality was noted and all aquatic macrophyteand wetland plant species were recorded along with their general abundance and an estimate of the total percent areal coverage of all species.  Qualitative macrophyte observations were aided by conducting several hauls with a plant "rake," which was constructed by bolting two garden rakes back-to-back, the handles cut to about half length, and then attached to about a 50' length of rope.  Each time the rake was thrown to its maximum extension and then retrieved along the lake bottom.  The rake was thrown into the water several times in different directions from the observation site to provide more thorough coverage.

Where possible (e.g., dam or dock), transparency was measured using a standard 20-centimeter diameter Secchi disc attached to a rope with metric calibrations. When Secchi disc measurements were not feasible, transparency was estimated as being above or below 1.2 meters (based on the 4 foot Secchi disc bathing beach standard).

All observations were recorded on standardized field sheets.  Assessments of trophic status and use impairment were made on site.  Later, the assessments and supporting information will be entered into the US EPA Water Body System database.  Data on the presence of non-native plants were entered into a separate database intended for linking to the Massachusetts Geographic Information System (MassGIS).

RESULTS

## Survey Conditions

To fulfill the assessment guidance, information on precipitation (MA DEM 1998) and stream discharge (Socolow et al. 1998) were analyzed to determine hydrologic conditions during the water quality sampling events.  This review was conducted to determine the streamflow condition in relation to the provisional 7Q10 low flow of 13 cfs at the Ten Mile River USGS gage at East Providence (01109403) (USGS 1998).  Additionally, this review was used to determine whether the fecal coliform bacteria data were representative of “wet” or “dry weather” sampling conditions.  Survey conditions are described below for each DWM sampling event reviewed for the assessment.

1 July 1997—Just under a tenth of an inch of precipitation was recorded five days prior to the sampling event (MA DEM 1998).  The daily mean stream discharge of the Ten Mile River USGS gage at East Providence (01109403) (Socolow et al. 1998) steadily declined over the five-day antecedent period prior to the survey (from 42 to 26 cfs).  Streamflow of the Ten Mile River was approximately two times higher than the estimated 7Q10 flow of 13 cfs.  Data from this sampling event will be interpreted as being representative of dry weather conditions.

5 August 1997—A tenth of inch of precipitation was recorded five days prior to the survey (MA DEM 1998).  Although there was no measurable rainfall during the four days prior to the survey, the daily mean discharge of the Ten Mile River USGS gage at East Providence (01109403) (Socolow et al. 1998) increased from 16 cfs on 3 August to 21 cfs on 5 August.  The reason for this increase is unknown. Streamflow of the Ten Mile River was 1.6 times higher than the estimated 7Q10 flow of 13 cfs. Data from this survey will be interpreted as dry weather conditions.

3 September 1997—Just over a half inch (0.52) of precipitation was measured three days prior to the survey, while 0.15 inches of rain were measured the following day (MA DEM 1998).  Discharge in theTen Mile River responded to the storm event, increasing from baseflow conditions (approximately 22-24 cfs prior to precipitation) to 53 cfs, followed by a decrease in discharge to 34 cfs on the day of sampling (Socolow et al. 1998).  Streamflow of the Ten Mile River was approximately 2.6 times higher than the estimated 7Q10 flow of 13 cfs.  Data from this survey will be interpreted as representative of wet weather conditions.

## Stream Water Quality Monitoring

All DEP water quality data is managed and maintained in an Access Database (Dallaire, 2000).  The Hydrolab in-situ results are provided in Table B2.  Bacterial data are provided in Table B3.

Table B2.  1997 DEP DWM Ten Mile River Basin in-situ Hydrolab data.

Time	Measurement 	Temp	pH 	Cond 	TDS 	DO 	SAT 	Turb

(24hr)	Depth (m)	(°C)	(SU)	(uS/cm)	(g/l)	(mg/l)	(%)	(NTU)

TEN MILE RIVER

Station: TM01,  Mile Point: 22.1

Description: downstream at Fuller Street, Plainville.

52-0001	07/01/97	04:21	<0.3  	22.1  	6.6  	265	0.2	6.5 	74	--

52-0022	07/01/97	14:21	<0.3  	24.1  	6.8  	259	0.2	7.7 	90	6

52-0040	08/05/97	04:19	<0.3  	21.4  	6.8  	209	0.1	5.9 	66	--

52-0061	08/05/97	14:25	<0.3  	20.8  	6.9  	208	0.1	6.7 	74	2

52-0082	09/03/97	04:26	<0.3  	21.0  	6.7  	196	0.1	5.0 	54	--

52-0103	09/03/97	14:29	<0.3  	20.8  	6.7  	205	0.1	5.5 	61	4

TEN MILE RIVER

Station: TM04,  Mile Point: 18.5

Description: upstream at Route 1 (west of inlet to Falls Pond), North Attleborough.

52-0003	07/01/97	04:46	<0.3  	22.1  	6.9  	262	0.2	5.8 	65	--

52-0024	07/01/97	14:40	<0.3  	25.2  	7.1  	256	0.2	7.0 	83	**

52-0042	08/05/97	04:56	<0.3  	19.7  	**  	108	0.07	6.2 	67	--

52-0063	08/05/97	14:47	<0.3  	21.2  	7.0  	148	0.10	8.8 	98	6

52-0084	09/03/97	04:50	<0.3  	22.4  	7.3  	242	0.2	7.7 	86	--

52-0105	09/03/97	15:00	<0.3  	21.4  	7.5  	254	0.2	8.6 	95	**

TEN MILE RIVER

Station: TM06,  Mile Point: 16.5

Description: immediately upstream of Cedar Road, North Attleborough.

52-0004	07/01/97	05:07	<0.3  	21.5  	6.7  	239	0.2	5.8 	64	--

52-0025	07/01/97	14:58	<0.3  	24.8  	6.8  	239	0.2	6.3 	75	7

52-0043	08/05/97	05:13	<0.3  	18.1  	**  	236	0.2	6.9 	72	--

52-0064	08/05/97	15:15	<0.3  	19.3  	6.8  	241	0.2	7.8 	83	9

52-0085	09/03/97	05:10	<0.3  	21.7  	6.9  	234	0.1	5.8 	64	--

52-0106	09/03/97	15:20	<0.3  	21.1  	7.0  	234	0.1	6.9 	76	17

TEN MILE RIVER

Station: TM07,  Mile Point: 15.8

Description: 200 yards downstream of Route 95 (off Woodcock Lane), Attleboro.

52-0006	07/01/97	05:24	<0.3  	18.9  	6.4  	459	0.3	5.2 	55	--

52-0026	07/01/97	15:17	<0.3  	21.8  	6.8  	547	0.3	9.6 	107	6

52-0045	08/05/97	05:28	<0.3  	18.5  	**  	645	0.4	5.5 	58	--

52-0066	08/05/97	15:34	**  	**  	**  	**  	** 	** 	**	**

52-0087	09/03/97	05:27	0.6  	20.8  	6.8  	462	0.3	5.4 	59	--

52-0107	09/03/97	15:43	0.5  	20.7  	7.1  	535	0.3	8.8 	97	**

TEN MILE RIVER

Station: TM08A,  Mile Point: 13.6

Description: approximately 20 yards upstream of Olive Street, Attleboro.

52-0007	07/01/97	05:46	<0.3  	23.0  	6.6  	438	0.3	4.8 	55	--

52-0027	07/01/97	15:37	<0.3  	24.0  	6.7  	441	0.3	6.0 	70	29*

52-0046	08/05/97	05:43	<0.3  	20.9  	**  	**  	** 	5.3 	58	--

52-0067	08/05/97	15:54	**  	**  	**  	**  	** 	** 	**	**

52-0088	09/03/97	05:46	0.5  	22.3  	6.8  	407	0.3	5.3 	60	--

52-0108	09/03/97	16:45	0.3  	21.4  	6.8  	416	0.3	6.6 	74	7

* = outside calibrated range, ** = censored data,  -- = no data

Table B2 (continued).  1997 DEP DWM Ten Mile River Basin in-situ Hydrolab data.

Time	Measurement 	Temp	pH 	Cond 	TDS 	DO 	SAT 	Turb

(24hr)	Depth (m)	(°C)	(SU)	(uS/cm)	(g/l)	(mg/l)	(%)	(NTU)

TEN MILE RIVER

Station: TM11,  Mile Point: 11.5

Description: off the upstream side of the Tiffany Street bridge,  Attleboro.

52-0017	07/01/97	05:58	<0.3  	23.9  	7.0  	401	0.3	6.0 	70	9

52-0035	07/01/97	15:26	<0.3  	25.9  	7.0  	403	0.3	6.8 	82	--

52-0056	08/05/97	05:27	<0.3  	22.8  	7.0  	474	0.3	5.7 	65	3

52-0077	08/05/97	15:23	0.4  	23.4  	7.0  	483	0.3	7.3 	85	--

52-0098	09/03/97	06:11	<0.3  	22.4  	7.0  	424	0.3	6.1 	69	4

52-0116	09/03/97	15:26	0.4  	22.4  	7.2  	418	0.3	7.3 	83	--

TEN MILE RIVER

Station: TM12,  Mile Point: 6.6

Description: 200 yards downstream of Bridge Street (between Old Mill apartment - upstream of railroad  - southeast  of

Read Street), Attleboro.

52-0016	07/01/97	05:37	<0.3  	24.1  	6.8  	359	0.2	6.9 	80	18

52-0034	07/01/97	15:14	<0.3  	24.7  	6.8  	369	0.2	7.0 	83	--

52-0055	08/05/97	05:14	<0.3  	21.7  	6.9  	412	0.3	6.9 	77	2

52-0076	08/05/97	15:10	0.3  	22.2  	6.8  	411	0.3	8.0 	91	--

52-0097	09/03/97	05:51	<0.3  	22.6  	7.0  	402	0.3	7.0 	79	6

52-0115	09/03/97	15:14	0.4  	22.8  	7.2  	408	0.3	7.5 	86	--

TEN MILE RIVER

Station: TM13,  Mile Point: 5.8

Description: off the downstream side of the Pond Street bridge, Seekonk.

52-0013	07/01/97	04:53	<0.3  	23.6  	6.7  	366	0.2	4.8 	55	12

52-0032	07/01/97	14:51	0.3  	27.4  	7.0  	371	0.2	8.5 	106	--

52-0052	08/05/97	04:45	<0.3  	21.8  	6.8  	423	0.3	5.5 	61	4

52-0073	08/05/97	14:45	0.4  	22.8  	6.8  	424	0.3	7.5 	86	--

52-0094	09/03/97	05:07	0.3  	21.7  	6.8  	399	0.3	5.0 	56	3

52-0113	09/03/97	14:52	0.6  	22.6  	7.0  	396	0.3	7.4 	84	--

TEN MILE RIVER

Station: TM14,  Mile Point: 4.3

Description: off the upstream side of the Central Avenue bridge, Pawtucket, Rhode Island.

52-0012	07/01/97	04:19	<0.3  	21.9  	6.8  	542	0.3	5.7 	63	4

52-0031	07/01/97	14:41	<0.3  	25.0  	7.1  	583	0.4	8.8 	105	--

52-0051	08/05/97	04:33	<0.3  	21.1  	6.9  	671	0.4	5.4 	60	3

52-0072	08/05/97	14:31	0.5  	21.1  	7.0  	756	0.5	9.2 	102	--

52-0093	09/03/97	04:51	<0.3  	21.1  	6.8  	538	0.3	5.6 	62	9

52-0112	09/03/97	14:41	0.4  	21.6  	7.1  	599	0.4	8.5 	95	--

COLES BROOK

Station: CB01,  Mile Point: 0.3

Description: upstream/east at Route 152, Seekonk.

52-0011	07/01/97	**  	**	**  	**  	**	**	**	**	**

52-0030	07/01/97	**  	**	**  	**  	**	**	**	**	**

52-0050	08/05/97	--  	             --------------- Not enough flow to take sample--------------	--  	--  	--

52-0071	08/05/97	--  	             --------------- Not enough flow to take sample--------------	--  	--  	--

52-0092	09/03/97	04:32	<0.3  	21.9  	6.6  	94	0.06	5.4 	60	9

52-0111	09/03/97	14:27	0.4  	21.6  	6.7  	97	0.06	5.5 	62	--

SEVENMILE RIVER

Station: SM00,  Mile Point: 5.6

Description: off the downstream/south side of the Draper Avenue bridge, North Attleborough.

52-0019	07/01/97	07:01	<0.3  	18.2  	6.6  	537	0.3	5.8 	60	**

52-0037	07/01/97	15:53	<0.3  	22.0  	6.6  	529	0.3	6.7 	75	--

52-0058	08/05/97	05:53	<0.3  	17.6  	6.5  	235	0.2	5.7 	59	**

52-0079	08/05/97	15:46	<0.3  	19.0  	6.4  	418	0.3	6.2 	66	--

52-0100	09/03/97	06:56	<0.3  	22.1  	6.6  	106	0.07	7.3 	82	16

52-0118	09/03/97	15:49	0.3  	22.3  	6.8  	104	0.07	7.9 	89	--

* = outside calibrated range, ** = censored data,  -- = no data

Table B2 (continued).  1997 DEP DWM Ten Mile River Basin in-situ Hydrolab data.

Time	Measurement 	Temp	pH 	Cond 	TDS 	DO 	SAT 	Turb

(24hr)	Depth (m)	(°C)	(SU)	(uS/cm)	(g/l)	(mg/l)	(%)	(NTU)

SEVENMILE RIVER

Station: SM01,  Mile Point: 0.5

Description: upstream/northwest of County Street, Attleboro.

52-0015	07/01/97	05:16	<0.3  	18.0  	6.4  	321	0.2	6.3 	65	18

52-0033	07/01/97	15:03	0.3  	19.0  	6.4  	321	0.2	6.9 	73	--

52-0054	08/05/97	05:01	<0.3  	17.6  	6.4  	299	0.2	6.0 	62	7

52-0075	08/05/97	14:57	0.4  	17.9  	6.3  	269	0.2	6.5 	68	--

52-0096	09/03/97	05:28	<0.3  	18.4  	6.5  	332	0.2	6.6 	68	5

52-0114	09/03/97	15:03	0.3  	18.0  	6.5  	333	0.2	7.0 	72	--

FOURMILE BROOK

Station: FM01,  Mile Point: 0.4

Description: downstream/south at West Street, Attleboro.

52-0018	07/01/97	06:38	<0.3  	14.5  	6.7  	202	0.1	7.7 	74	10

52-0036	07/01/97	15:41	<0.3  	19.3  	7.0  	203	0.1	8.8 	94	--

52-0057	08/05/97	05:41	<0.3  	23.4  	7.0  	204	0.1	7.4 	85	3

52-0078	08/05/97	15:34	<0.3  	23.8  	6.9  	206	0.1	7.6 	89	--

52-0099	09/03/97	06:38	<0.3  	22.6  	7.0  	201	0.1	7.5 	85	9

52-0117	09/03/97	15:38	<0.3  	21.5  	6.7  	221	0.1	6.3 	70	--

SPEEDWAY BROOK

Station: SW01,  Mile Point: 0.01

Description: off the upstream/east side of the Route 152 bridge, Attleboro.

52-0020	07/01/97	06:17	<0.3  	18.3  	6.7  	395	0.3	4.6 	48	72*

52-0038	07/01/97	16:10	<0.3  	22.2  	6.8  	397	0.3	6.1 	69	--

52-0059	08/05/97	06:12	<0.3  	20.9  	6.6  	172	0.1	4.9 	54	8

52-0080	08/05/97	16:05	<0.3  	20.4  	6.5  	139	0.09	6.2 	68	--

52-0101	09/03/97	07:22	<0.3  	20.9  	7.0  	426	0.3	5.3 	58	**

52-0119	09/03/97	16:08	<0.3  	20.3  	7.1  	424	0.3	5.7 	62	--

BUNGAY RIVER

Station: BG01,  Mile Point: 4.7

Description: approximately 100 feet downstream/south of West Street (Bungay Road), North Attleborough (Two feet

above fish hatchery outfall).

52-0008	07/01/97	06:10	<0.3  	19.3  	6.4  	**  	** 	4.5 	47	--

52-0028	07/01/97	16:01	<0.3  	21.4  	6.5  	**  	** 	7.3 	81	10

52-0047	08/05/97	06:06	<0.3  	18.0  	**  	225	0.1	4.6 	48	--

52-0068	08/05/97	16:14	<0.3  	21.6  	6.7  	227	0.1	8.5 	95	5

52-0089	09/03/97	06:09	<0.3  	18.3  	6.4  	233	0.1	4.7 	48	--

52-0109	09/03/97	16:18	<0.3  	19.9  	6.7  	233	0.1	8.3 	90	16

Pipe/Discharge to BUNGAY RIVER

Station: MA0005398,  Mile Point: 4.69

Description: outlet of North Attleborough National Fish Hatchery discharge pipe, North Attleborough (Two feet below Station

BG01).

52-0126	09/03/97	06:19	<0.3  	12.8  	6.3  	288	0.2	8.7 	81	--

52-0127	09/03/97	16:26	**  	**  	**  	**  	** 	** 	**	**

BUNGAY RIVER

Station: BG02,  Mile Point: 1.2

Description: upstream/north at Holden Street, Attleboro.

52-0009	07/01/97	06:31	<0.3  	24.0  	6.5  	274	0.2	5.2 	60	--

52-0029	07/01/97	16:20	<0.3  	26.1  	7.0  	271	0.2	11.3 	137	11

52-0048	08/05/97	06:27	<0.3  	22.8  	**  	265	0.2	5.8 	66	--

52-0069	08/05/97	16:33	<0.3  	24.7  	6.9  	262	0.2	8.0 	94	6

52-0090	09/03/97	06:35	0.4  	22.1  	6.7  	270	0.2	6.7 	75	--

52-0110	09/03/97	16:01	<0.3  	22.3  	7.0  	273	0.2	9.2 	104	10

* = outside calibrated range, ** = censored data,  -- = no data

Table B2 (continued).  1997 DEP DWM Ten Mile River Basin in-situ Hydrolab data.

Time	Measurement 	Temp	pH 	Cond 	TDS 	DO 	SAT 	Turb

(24hr)	Depth (m)	(°C)	(SU)	(uS/cm)	(g/l)	(mg/l)	(%)	(NTU)

SCOTTS BROOK

Station: SB01,  Mile Point: 0.4

Description: off the upstream/west side of the Broadway bridge, North Attleborough.

52-0002	07/01/97	--  	             --------------- Not enough flow to take sample--------------	--  	--  	--

52-0023	07/01/97	--  	             --------------- Not enough flow to take sample--------------	--  	--  	--

52-0062	08/05/97	--  	             --------------- Not enough flow to take sample--------------	--  	--  	--

52-0041	08/05/97	04:42	<0.3  	19.3  	**  	41	0.03	7.8 	83	--

52-0083	09/03/97	--  	             --------------- Not enough flow to take sample--------------	--  	--  	--

52-0104	09/03/97	--  	             --------------- Not enough flow to take sample--------------	--  	--  	--

* = outside calibrated range, ** = censored data,  -- = no data

Table B3.  1997 DEP DWM Ten Mile River Basin bacteria data.  Units in colonies/100 ml.

Time	FECAL	E-COLI	ENTEROCOCCUS	AEROMONAS

(24hr)

TEN MILE RIVER

Station: TM01,  Mile Point: 22.1

Description: downstream at Fuller Street, Plainville.

52-0001	07/01/97	4:21	40	20	40	1,100,000

52-0040	08/05/97	4:19	**  	--  	--  	--

52-0082	09/03/97	4:26	<20	<20	--  	--

TEN MILE RIVER

Station: TM04,  Mile Point: 18.5

Description: upstream at Route 1 (west of inlet to Falls Pond), North Attleborough.

52-0003	07/01/97	4:46	440	440	740	1,200,000

52-0042	08/05/97	4:56	**  	--  	--  	--

52-0084	09/03/97	4:50	1,200	480	--  	--

TEN MILE RIVER

Station: TM06,  Mile Point: 16.5

Description: immediately upstream of Cedar Road, North Attleborough.

52-0004	52-0005	07/01/97	5:07	160	100	280	1,200,000

52-0005	52-0004	07/01/97	5:07	180	60	220	120,000

52-0043	52-0044	08/05/97	5:13	**  	--  	--  	--

52-0044	52-0043	08/05/97	5:13	**  	--  	--  	--

52-0085	52-0086	09/03/97	5:10	60	80	--  	--

52-0086	52-0085	09/03/97	5:10	60	60	--  	--

TEN MILE RIVER

Station: TM07,  Mile Point: 15.8

Description: 200 yards downstream of Route 95 (off Woodcock Lane), Attleboro.

52-0006	07/01/97	5:24	100	40	140	1,700,000

52-0045	08/05/97	5:28	**  	--  	--  	--

52-0087	09/03/97	5:27	360	220	--  	--

* = interference           ** = missing/censored data          -- = no data

Table B3 (continued).  1997 DEP DWM Ten Mile River Basin bacteria data.  Units in colonies/100 ml.

Time	FECAL	E-COLI	ENTEROCOCCUS	AEROMONAS

(24hr)

TEN MILE RIVER

Station: TM08A,  Mile Point: 13.6

Description: approximately 20 yards upstream of Olive Street, Attleboro.

52-0007	07/01/97	5:46	320	160	220	800,000

52-0046	08/05/97	5:43	**  	--  	--  	--

52-0088	09/03/97	5:46	320	280	--  	--

TEN MILE RIVER

Station: TM11,  Mile Point: 11.5

Description: off the upstream side of the Tiffany Street bridge,  Attleboro.

52-0017	07/01/97	5:58	80	40	80	1,700,000

52-0056	08/05/97	5:22	**  	--  	--  	--

52-0098	09/03/97	6:11	20	20	--  	--

TEN MILE RIVER

Station: TM12,  Mile Point: 6.6

Description: 200 yards downstream of Bridge Street (between Old Mill apartment - upstream of railroad  - southeast  of

Read Street), Attleboro.

52-0016	07/01/97	5:37	120	100	100	1,500,000

52-0055	08/05/97	5:12	**  	--  	--  	--

52-0097	09/03/97	5:49	160	120	--  	--

TEN MILE RIVER

Station: TM13,  Mile Point: 5.8

Description: off the downstream side of the Pond Street bridge, Seekonk.

52-0013	52-0014	07/01/97	4:53	240	120	240	1,500,000

52-0014	52-0013	07/01/97	4:53	240	100	80	1,000,000

52-0052	52-0053	08/05/97	4:40	**  	--  	--  	--

52-0053	52-0052	08/05/97	4:40	**  	--  	--  	--

52-0094	52-0095	09/03/97	5:07	240	120	--  	--

52-0095	52-0094	09/03/97	5:07	240	100	--  	--

TEN MILE RIVER

Station: TM14,  Mile Point: 4.3

Description: off the upstream side of the Central Avenue bridge, Pawtucket, Rhode Island.

52-0012	07/01/97	4:19	360	200	160	1,200,000

52-0051	08/05/97	4:30	**  	--  	--  	--

52-0093	09/03/97	4:51	400	240	--  	--

COLES BROOK

Station: CB01,  Mile Point: 0.3

Description: upstream/east at Route 152, Seekonk.

52-0011	07/01/97	3:51	180	80	10,000	1,000,000

52-0050	08/05/97	             --------------- Not enough flow to take sample--------------	**

52-0071	08/05/97	             --------------- Not enough flow to take sample--------------	**

52-0092	09/03/97	4:32	1,000	1,000	--  	--

* = interference           ** = missing/censored data          -- = no data

Table B3 (continued).  1997 DEP DWM Ten Mile River Basin bacteria data.  Units in colonies/100 ml.

Time	FECAL	E-COLI	ENTEROCOCCUS	AEROMONAS

(24hr)

SEVENMILE RIVER

Station: SM00,  Mile Point: 5.6

Description: off the downstream/south side of the Draper Avenue bridge, North Attleborough.

52-0019	07/01/97	7:01	100	120	280	300,000

52-0058	08/05/97	5:50	**  	--  	--  	--

52-0100	09/03/97	6:56	520	400	--  	--

SEVENMILE RIVER

Station: SM01,  Mile Point: 0.5

Description: upstream/northwest of County Street, Attleboro.

52-0015	07/01/97	5:16	700	400	160	600,000

52-0054	08/05/97	5:00	**  	--  	--  	--

52-0096	09/03/97	5:28	360	300	--  	--

FOURMILE BROOK

Station: FM01,  Mile Point: 0.4

Description: downstream/south at West Street, Attleboro.

52-0018	07/01/97	6:38	40	20	180	700,000

52-0057	08/05/97	5:36	**  	--  	--  	--

52-0099	09/03/97	6:38	40	80	--  	--

SPEEDWAY BROOK

Station: SW01,  Mile Point: 0.01

Description: off the upstream/east side of the Route 152 bridge, Attleboro.

52-0020	07/01/97	6:17	520	280	460	1,300,000

52-0059	08/05/97	6:10	**  	--  	--  	--

52-0101	09/03/97	7:22	720	420	--  	--

BUNGAY RIVER

Station: BG01,  Mile Point: 4.7

Description: approximately 100 feet downstream/south of West Street (Bungay Road), North Attleborough (two feet

above fish hatchery outfall).

52-0008	07/01/97	6:10	260	200	220	690,000

52-0047	08/05/97	6:06	**  	--  	--  	--

52-0089	09/03/97	6:09	240	80	--  	--

BUNGAY RIVER

Station: BG02,  Mile Point: 1.2

Description: upstream/north at Holden Street, Attleboro.

52-0009	07/01/97	6:31	60	40	20	750,000

52-0048	08/05/97	6:27	**  	--  	--  	--

52-0090	09/03/97	6:35	100	40	--  	--

SCOTTS BROOK

Station: SB01,  Mile Point: 0.4

Description: off the upstream/west side of the Broadway bridge, North Attleborough.

52-0002	07/01/97	             --------------- Not enough flow to take sample--------------	**

52-0023	07/01/97	             --------------- Not enough flow to take sample--------------	**

52-0062	08/05/97	             --------------- Not enough flow to take sample--------------	**

52-0041	08/05/97	4:42	**  	--  	--  	--

52-0083	09/03/97	             --------------- Not enough flow to take sample--------------	**

52-0104	09/03/97	             --------------- Not enough flow to take sample--------------	**

* = interference           ** = missing/censored data          -- = no data

## Benthic Macroinvertebrates and Habitat Assessments

A list of the macroinvertebrate biomonitoring station locations for the 1997 Ten Mile River Basin survey is provided in Table B4.  Sampling locations are shown in Figure B2.  The taxonomic list of macroinvertebrates collected at each sampling station is provided in Table B5.  Included in this list are total organism counts, and the functional feeding group (FG) and tolerance value (TV) of each taxon.

Table B4. List of macroinvertebrate biomonitoring station locations for the 1997 Ten Mile River Basin biomonitoring survey, including station number, station description, survey date, and sampling protocol used.

| STATION | SITE DESCRIPTION | SAMPLING DATE | SAMPLING METHOD |
| --- | --- | --- | --- |
| TM01 | Ten Mile River, downstream from Fuller St., Plainville, MA | 9 Sept. 1997 | RBPIII |
| TM02 | Ten Mile River, downstream from West Bacon St., Plainville, MA | 9 Sept. 1997 | RBPIII |
| TM04 | Ten Mile River, upstream from Route 1, N. Attleboro, MA | 9 Sept. 1997 | RBPIII |
| TM06 | Ten Mile River, upstream from N. Attleborough WWTP, Attleboro | 9 Sept. 1997 | RBPIII |
| TM06a | Ten Mile River, downstream from N. Attleborough WWTP, Attleboro | 9 Sept. 1997 | RBPIII |
| TM11 | Ten Mile River, downstream from Tiffany St., Attleboro, MA | 10 Sept. 1997 | RBPIII |
| TM14 | Ten Mile River, downstream from Central Av., Seekonk, MA | 17 Sept. 1997 | RBPIII |
| SM00 | Sevenmile River, upstream from Draper Av., N. Attleborough, MA | 10 Sept. 1997 | RBPIII |
| SW01M | Speedway (Thatcher) Brook, downstream from Dexter St., Attleboro, MA | 17 Sept. 1997 | RBPIII |
| FM01 | Fourmile Brook, between Greenfield and West St., Attleboro, MA | 10 Sept. 1997 | Qualitative |

Table B5. Species-level taxa list and counts, functional feeding groups (FG), and tolerance values (TV) for macroinvertebrates collected from stream sites in the Ten Mile River Basin between 9 and 17 September 1997. Sampling stations were in: Sevenmile River (SM00); Speedway Brook (SW01M); Ten Mile River (TM01, TM02, TM04, TM06, TM06a, TM11, TM14); and Fourmile Brook (FM01).

| TAXON | FG1 | TV2 | SM00 | SW01M | TM01 | TM02 | TM04 | TM06 | TM06A | TM11 | TM14 | FM013 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hydrobiidae | SC | 8 | 5 |  |  |  |  |  |  |  |  |  |
| Lymnaeidae | GC | 6 | 1 |  |  |  |  |  |  |  |  |  |
| Physidae | GC | 8 | 1 |  | 2 |  |  |  | 1 |  |  |  |
| Planorbidae | SC | 6 | 3 |  |  |  |  |  |  |  |  |  |
| Pisidiidae | FC | 6 | 23 | 2 | 4 |  |  |  | 10 | 22 |  |  |
| Lumbricina | GC | 8 |  |  | 1 | 2 |  | 1 |  |  |  | x |
| Tubificidae (w/o capilliform chaetae) | GC | 10 | 2 | 2 |  |  | 33 |  | 9 |  |  |  |
| Tubificidae (with capilliform chaetae) | GC | 10 | 1 | 2 |  |  |  |  |  |  |  |  |
| Aulodrilus pluriseta | GC | 8 | 3 |  |  |  |  |  |  |  |  |  |
| Naididae | GC | 9 |  |  |  |  |  |  |  |  |  | x |
| Nais behningi | GC | 6 |  |  |  |  |  |  |  |  | 1 |  |
| Nais communis | GC | 8 | 1 |  | 13 |  |  |  |  |  |  |  |
| Nais variabilis | GC | 10 | 1 |  |  |  |  |  |  |  |  |  |
| Lumbriculus sp. | GC | 8 | 10 |  | 5 | 1 |  | 6 | 1 |  |  |  |
| Erpobdellidae | PR | 8 |  |  |  |  |  |  | 1 |  |  |  |
| Caecidotea communis | GC | 8 | 1 |  |  |  |  |  | 45 |  |  | x |
| Gammarus sp. | GC | 6 | 6 |  |  | 40 | 23 |  |  |  |  | x |
| Hyalella azteca | GC | 8 | 22 |  | 2 |  |  |  |  | 1 |  |  |
| Hydracarina | PR | 6 | 1 |  |  |  |  |  |  |  |  |  |
| Baetidae | GC | 4 |  |  |  |  |  |  |  |  | 1 |  |
| Baetis sp. | GC | 6 |  |  |  |  |  | 6 |  |  | 2 |  |
| Heptageniidae | SC | 4 |  |  |  |  |  |  |  |  | 2 |  |
| Stenonema sp. | SC | 3 |  |  |  |  |  | 1 |  |  |  |  |
| Eurylophella sp. | GC | 2 |  |  | 1 |  |  |  |  |  |  |  |
| Leptophlebiidae | GC | 2 |  |  |  |  |  |  |  |  | 1 |  |
| Boyeria sp. | PR | 2 |  |  |  |  |  |  |  |  |  | x |
| Calopterygidae | PR | 5 |  |  |  |  |  | 1 |  |  |  |  |
| Coenagrionidae | PR | 9 |  |  |  | 1 |  |  |  |  |  |  |
| Leuctridae | SH | 1 |  |  |  |  | 1 |  |  |  |  |  |
| Sialis sp. | PR | 8 |  |  |  |  |  |  |  |  |  | x |
| Corydalus sp. | PR | 6 |  |  |  |  |  |  |  | 1 |  |  |
| Nigronia sp. | PR | 6 |  |  |  | 1 |  | 2 |  |  |  | x |
| Trichoptera | GC | 5 |  |  | 1 |  |  |  |  |  |  |  |
| Chimarra sp. | FC | 3 | 1 |  |  | 10 |  | 11 |  | 16 | 18 | x |

1Functional Feeding Group (FG) lists the primary feeding habit of each species and follows the abbreviations: SH- Shredder; GC-Gathering Collector; FC-Filtering Collector; SC-Scraper; PR-Predator.

2Tolerance Value (TV) is an assigned value used in the calculation of the biotic index. Tolerance values range from 0 for organisms very intolerant of organic pollution to 10 for organisms very tolerant.

3Sampling at this station was qualitative only. An “x” indicates taxon presence.

Table B5 (continued). Species-level taxa list and counts, functional feeding groups (FG), and tolerance values (TV) for macroinvertebrates collected from stream sites in the Ten Mile River Basin between 9 and 17 September 1997. Sampling stations were in: Sevenmile River (SM00); Speedway Brook (SW01M); Ten Mile River (TM01, TM02, TM04, TM06, TM06a, TM11, TM14); and Fourmile Brook (FM01).

| TAXON | FG1 | TV2 | SM00 | SW01M | TM01 | TM02 | TM04 | TM06 | TM06A | TM11 | TM14 | FM013 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lype sp. | SC | 3 | 1 |  |  |  |  |  |  |  |  |  |
| Hydropsychidae | FC | 4 | 1 | 6 | 2 | 1 | 2 | 2 | 3 |  | 4 |  |
| Cheumatopsyche sp. | FC | 7 | 1 | 1 | 2 | 3 | 14 | 13 | 4 | 2 | 15 | x |
| Hydropsyche betteni gr. | FC | 8 | 1 | 55 | 9 | 10 | 1 | 18 | 3 | 24 | 33 | x |
| Hydropsyche morosa gr. | FC | 6 |  |  |  |  |  |  |  |  | 2 |  |
| Limnephilidae | SH | 4 |  |  |  |  |  |  |  |  |  | x |
| Ceraclea sp. | GC | 3 |  |  |  |  |  |  |  | 5 | 1 |  |
| Oecetis sp. | PR | 6 |  | 1 |  |  |  |  |  |  |  |  |
| Triaenodes sp. | SH | 6 |  |  |  |  |  |  |  |  |  | x |
| Setodes sp. | GC | 1 | 1 |  |  |  |  |  |  |  |  |  |
| Oulimnius sp. | SC | 2 |  |  | 5 |  |  |  |  |  |  |  |
| Oulimnius latiusculus | SC | 2 |  |  | 3 |  |  |  |  |  |  |  |
| Promoresia sp. | SC | 2 | 2 |  |  |  |  |  |  |  |  |  |
| Stenelmis sp. | SC | 5 |  |  | 5 |  |  | 11 | 10 | 23 | 14 |  |
| Stenelmis crenata | SC | 5 |  |  |  |  |  |  |  |  | 2 |  |
| Antocha sp. | GC | 5 |  |  |  |  | 2 |  |  |  | 1 |  |
| Tipula sp. | SH | 8 |  |  |  |  |  |  | 1 |  |  |  |
| Simulium sp. | FC | 4 | 1 |  | 4 | 15 |  | 11 |  |  | 2 |  |
| Tanypodinae | PR | 7 |  |  |  |  |  |  |  | 2 |  |  |
| Conchapelopia sp. | PR | 9 |  | 1 |  |  |  | 3 |  | 1 | 2 |  |
| Krenopelopia sp. | PR | 7 |  |  | 1 |  |  |  |  |  |  |  |
| Larsia sp. | PR | 7 |  |  |  |  | 1 |  |  |  |  |  |
| Natarsia sp. | PR | 10 |  | 1 |  |  |  |  |  |  |  |  |
| Thienemannimyia gr. | PR | 6 |  | 12 | 1 |  |  |  |  |  |  |  |
| Thienemannimyia sp. | PR | 6 |  |  |  |  | 1 |  |  |  |  |  |
| Zavrelimyia sp. | PR | 8 | 1 |  |  |  |  |  |  |  |  |  |
| Diamesinae | GC | 2 |  |  |  |  |  |  | 1 |  |  |  |
| Diamesa sp. | GC | 8 |  |  | 1 |  | 1 |  |  |  |  |  |
| Corynoneura sp. | GC | 6 |  |  | 1 |  |  | 1 |  |  |  |  |
| Cricotopus sp. | SH | 7 |  | 1 |  |  | 4 |  |  |  |  |  |
| Cricotopus bicinctus | SH | 7 |  | 3 |  | 1 | 1 |  |  |  |  |  |
| Cricotopus tremulus | SH | 7 |  |  |  | 1 |  |  |  |  |  |  |
| Diplocladius sp. | GC | 8 |  |  | 1 | 1 |  |  |  |  |  |  |
| Metriocnemus sp. | GC | 5 | 1 |  |  |  |  |  |  |  |  |  |
| Parametriocnemus sp. | GC | 4 |  |  |  |  | 1 | 5 | 1 |  |  |  |
| Rheocricotopus sp. | GC | 6 |  | 2 |  |  |  |  |  |  |  |  |

1Functional Feeding Group (FG) lists the primary feeding habit of each species and follows the abbreviations: SH- Shredder; GC-Gathering Collector; FC-Filtering Collector; SC-Scraper; PR-Predator.

2Tolerance Value (TV) is an assigned value used in the calculation of the biotic index. Tolerance values range from 0 for organisms very intolerant of organic pollution to 10 for organisms very tolerant.

3Sampling at this station was qualitative only. An “x” indicates taxon presence.

Table B5 (continued). Species-level taxa list and counts, functional feeding groups (FG), and tolerance values (TV) for macroinvertebrates collected from stream sites in the Ten Mile River Basin between 9 and 17 September 1997. Sampling stations were in: Sevenmile River (SM00); Speedway Brook (SW01M); Ten Mile River (TM01, TM02, TM04, TM06, TM06a, TM11, TM14); and Fourmile Brook (FM01).

| TAXON | FG1 | TV2 | SM00 | SW01M | TM01 | TM02 | TM04 | TM06 | TM06A | TM11 | TM14 | FM013 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Symposiocladius lignicola | SH | 5 | 1 |  |  |  |  |  |  |  |  |  |
| Tvetenia bavarica gr. | GC | 5 |  |  |  | 2 |  | 2 |  |  |  |  |
| Tvetenia vitracies gr. | GC | 5 |  |  |  |  |  |  |  |  | 2 |  |
| Chironomus sp. | GC | 10 |  | 1 |  |  |  |  |  |  |  |  |
| Microtendipes pedellus gr. | FC | 6 |  |  |  |  |  | 6 |  |  |  |  |
| Phaenopsectra sp. | SC | 7 | 1 |  |  |  | 1 |  |  |  |  |  |
| Polypedilum sp. | SH | 6 | 1 |  |  |  |  | 1 |  |  |  |  |
| Polypedilum convictum | SH | 5 |  |  | 3 |  |  | 2 |  | 8 |  |  |
| Polypedilum fallax gr. | SH | 6 |  | 4 |  |  |  |  |  |  |  |  |
| Polypedilum illinoense | SH | 9 | 2 |  |  |  |  |  | 2 |  |  |  |
| Polypedilum scalaenum | SH | 9 |  | 1 |  |  | 1 |  |  |  |  |  |
| Stictochironomus sp. | GC | 7 | 1 |  |  |  |  |  |  |  |  |  |
| Paratanytarsus sp. | FC | 8 |  |  |  |  | 1 |  |  |  |  |  |
| Rheotanytarsus distinctissimus gr. | FC | 6 |  |  | 6 |  |  |  |  |  |  |  |
| Rheotanytarsus exiguus gr. | FC | 6 |  |  | 13 | 1 | 1 |  | 2 |  |  |  |
| Tanytarsus sp. | FC | 7 | 1 | 1 |  |  |  | 3 |  | 1 |  |  |
| Tanytarsus/Micropsectra sp. | GC | 4 |  |  |  |  | 1 | 1 |  |  |  |  |
| Empididae | PR | 6 |  |  |  |  |  |  |  |  |  | x |
| Hemerodromia sp. | PR | 6 |  |  | 13 | 3 | 1 |  | 1 |  | 2 |  |
| TOTAL |  |  | 98 | 96 | 99 | 93 | 91 | 107 | 95 | 106 | 105 |  |

1Functional Feeding Group (FG) lists the primary feeding habit of each species and follows the abbreviations: SH- Shredder; GC-Gathering Collector; FC-Filtering Collector; SC-Scraper; PR-Predator.

2Tolerance Value (TV) is an assigned value used in the calculation of the biotic index. Tolerance values range from 0 for organisms very intolerant of organic pollution to 10 for organisms very tolerant.

3Sampling at this station was qualitative only. An “x” indicates taxon presence.

Summary tables of the RBP III data analyses, including biological metric calculations, metric scores and impairment scores are included in Tables B6 through B8.  Habitat assessment scores for each station are also included in the data analysis summary tables.  Table B9 provides a more detailed summary of the habitat evaluations for each biomonitoring station.  Scores for individual habitat parameters are included as well as the total habitat assessment score. Results of these analyses are discussed within the segment summaries in the body of this report.

Ten Mile River Basin 1997 Water Quality Assessment ReportAppendix BB18

52append.docDWM CN 18.0

Table B6. Summary of RBP III data analysis for macroinvertebrate communities sampled at 9 stations in the Ten Mile River Basin between 9 and 17 September 1997. Seven biological metrics were calculated and scored (in italics) for taxa collected at each station. Scores were then totaled and compared to the regional reference station (SM00). The percent comparability to the reference station yields a final impairment score for each study site.

| Station # | SM00 |  | SW01M |  | TM01 |  | TM02 |  | TM04 |  | TM06 |  | TM06a |  | TM11 |  | TM14 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stream | Sevenmile River |  | Speedway Brook |  | Ten Mile River |  | Ten Mile River |  | Ten Mile River |  | Ten Mile River |  | Ten Mile River |  | Ten Mile River |  | Ten Mile River |  |
| Habitat Score | 165 |  | 111 |  | 130 |  | 130 |  | 137 |  | 144 |  | 128 |  | 136 |  | 155 |  |
| Taxa Richness | 29 | 6 | 15 | 2 | 21 | 4 | 15 | 2 | 17 | 2 | 18 | 4 | 15 | 2 | 11 | 0 | 15 | 2 |
| Biotic Index | 6.99 | 6 | 7.40 | 6 | 6.14 | 6 | 5.70 | 6 | 7.62 | 6 | 5.86 | 6 | 7.35 | 6 | 5.66 | 6 | 5.90 | 6 |
| EPT Index | 5 | 6 | 3 | 0 | 3 | 0 | 3 | 0 | 3 | 0 | 5 | 6 | 2 | 0 | 4 | 4 | 8 | 6 |
| EPT/Chironomidae | 0.67 | 6 | 2.33 | 6 | 0.56 | 6 | 4.00 | 6 | 1.29 | 6 | 2.13 | 6 | 1.67 | 6 | 3.92 | 6 | 19.75 | 6 |
| Scrapers/Filterers | 0.41 | 6 | 0.00 | 0 | 0.33 | 6 | 0.00 | 0 | 0.05 | 0 | 0.19 | 4 | 0.45 | 6 | 0.35 | 6 | 0.24 | 6 |
| % Dominant Taxon | 23% | 4 | 57% | 0 | 13% | 6 | 43% | 0 | 36% | 2 | 17% | 6 | 47% | 0 | 23% | 4 | 31% | 2 |
| Community Similarity | 100% | 6 | 9% | 0 | 17% | 0 | 12% | 0 | 12% | 0 | 7% | 0 | 10% | 0 | 4% | 0 | 5% | 0 |
| Total Metric Score | 40 |  | 14 |  | 28 |  | 14 |  | 16 |  | 32 |  | 20 |  | 26 |  | 28 |  |
| % Comparability To Reference Station |  |  | 35% |  | 70% |  | 35% |  | 40% |  | 80% |  | 50% |  | 65% |  | 70% |  |
| Biological Condition-Degree Impairment | REFERENCE STATION (regional) |  | MODERATE |  | SLIGHT |  | MODERATE |  | MODERATE |  | NON/SLIGHT |  | MODERATE |  | SLIGHT |  | SLIGHT |  |

Ten Mile River Basin 1997 Water Quality Assessment ReportAppendix BB19

52append.docDWM CN 18.0

Table B7. Summary of RBP III data analysis for macroinvertebrate communities sampled at 9 stations in the Ten Mile River Basin between 9 and 17 September 1997. Seven biological metrics were calculated and scored (in italics) for taxa collected at each station. Scores were then totaled and compared to the regional reference station (TM01). The percent comparability to the reference station yields a final impairment score for each study site.

| Station # | TM01 |  | SM00 |  | SW01M |  | TM02 |  | TM04 |  | TM06 |  | TM06a |  | TM11 |  | TM14 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stream | Ten Mile River |  | Sevenmile River |  | Speedway Brook |  | Ten Mile River |  | Ten Mile River |  | Ten Mile River |  | Ten Mile River |  | Ten Mile River |  | Ten Mile River |  |
| Habitat Score | 130 |  | 165 |  | 111 |  | 130 |  | 137 |  | 144 |  | 128 |  | 136 |  | 155 |  |
| Taxa Richness | 21 | 6 | 29 | 6 | 15 | 4 | 15 | 4 | 17 | 6 | 18 | 6 | 15 | 4 | 11 | 2 | 15 | 4 |
| Biotic Index | 6.14 | 6 | 6.99 | 6 | 7.40 | 4 | 5.70 | 6 | 7.62 | 4 | 5.86 | 6 | 7.35 | 4 | 5.66 | 6 | 5.90 | 6 |
| EPT Index | 3 | 6 | 5 | 6 | 3 | 6 | 3 | 6 | 3 | 6 | 5 | 6 | 2 | 0 | 4 | 6 | 8 | 6 |
| EPT/Chironomidae | 0.56 | 6 | 0.67 | 6 | 2.33 | 6 | 4.00 | 6 | 1.29 | 6 | 2.13 | 6 | 1.67 | 6 | 3.92 | 6 | 19.75 | 6 |
| Scrapers/Filterers | 0.33 | 6 | 0.41 | 6 | 0.00 | 0 | 0.00 | 0 | 0.05 | 0 | 0.19 | 6 | 0.45 | 6 | 0.35 | 6 | 0.24 | 6 |
| % Dominant Taxon | 13% | 6 | 23% | 4 | 57% | 0 | 43% | 0 | 36% | 2 | 17% | 6 | 47% | 0 | 23% | 4 | 31% | 2 |
| Community Similarity | 100% | 6 | 17% | 0 | 15% | 0 | 23% | 0 | 8% | 0 | 31% | 2 | 21% | 0 | 24% | 0 | 22% | 0 |
| Total Metric Score | 42 |  | 34 |  | 20 |  | 22 |  | 24 |  | 38 |  | 20 |  | 30 |  | 30 |  |
| % Comparability To Reference Station |  |  | 81% |  | 48% |  | 52% |  | 57% |  | 90% |  | 48% |  | 71% |  | 71% |  |
| Biological Condition -Degree Impairment | REFERENCE STATION (regional) |  | NON/SLIGHT |  | MODERATE |  | SLIGHT |  | SLIGHT |  | NON |  | MODERATE |  | SLIGHT |  | SLIGHT |  |

Table B8. Summary of RBP III data analysis for macroinvertebrate communities sampled at 2 stations in the Ten Mile River Basin on 9 September 1997. Seven biological metrics were calculated and scored (in italics) for taxa collected at each station. Scores for the downstream study station (TM06a) were then totaled and compared to the upstream reference station (TM06). The percent comparability to the reference station yields a final impairment score for the study site.

| Station # | TM06 |  | TM06a |  |
| --- | --- | --- | --- | --- |
| Stream, description | Ten Mile River, upstream from North Attleborough WWTP |  | Ten Mile River, downstream from North Attleborough WWTP |  |
| Habitat Score | 144 |  | 128 |  |
| Taxa Richness | 18 | 6 | 15 | 6 |
| Biotic Index | 5.86 | 6 | 7.35 | 4 |
| EPT Index | 5 | 6 | 2 | 0 |
| EPT/Chironomidae | 2.13 | 6 | 1.67 | 6 |
| Scrapers/Filterers | 0.19 | 6 | 0.45 | 6 |
| % Dominant Taxon | 17% | 6 | 47% | 0 |
| Community Similarity | 100% | 6 | 22% | 0 |
| Total Metric Score |  | 42 |  | 22 |
| % Comparability To Reference Station |  |  | 52% |  |
| Biological Condition -Degree Impairment | REFERENCE STATION (upstream) |  | SLIGHT/MODERATE |  |

Table B9. Habitat assessment summary for the 10 macroinvertebrate stations sampled during the 1997 Ten Mile River Basin survey. For those primary parameters, scores ranging from 16-20 =optimal; 11-15 =suboptimal; 6-10 =marginal; 0-5 =poor. For those secondary parameters, scores ranging from 9-10 =optimal; 6-8 =suboptimal; 3-5 =marginal; 0-2 =poor.

| STATION | SM00 | SW01M | TM01 | TM02 | TM04 | TM06 | TM06A | TM11 | TM14 | FM01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PRIMARY HABITAT PARAMETER (range is 0-20) |  |  |  |  |  |  |  |  |  |  |
| Instream Cover | 18 | 2 | 5 | 10 | 7 | 14 | 4 | 12 | 14 | 1 |
| Epifaunal Substrate | 16 | 8 | 16 | 16 | 16 | 18 | 11 | 16 | 18 | 11 |
| Embeddedness | 12 | 13 | 18 | 12 | 16 | 11 | 11 | 11 | 8 | 7 |
| Channel Alteration | 15 | 11 | 16 | 15 | 12 | 15 | 17 | 15 | 14 | 12 |
| Sediment Deposition | 11 | 3 | 16 | 11 | 18 | 6 | 6 | 11 | 12 | 15 |
| Velocity-Depth Combinations | 15 | 11 | 3 | 12 | 5 | 17 | 13 | 11 | 14 | 5 |
| Channel Flow Status | 20 | 15 | 1 | 11 | 18 | 19 | 20 | 19 | 19 | 19 |
| SECONDARY HABITAT PARAMETERS (range is 0-10) for each bank (reported left bank, right bank – determine left or right side by facing downstream) |  |  |  |  |  |  |  |  |  |  |
| Bank Vegetative Protection | 10 10 | 10 10 | 10 10 | 10 10 | 10 10 | 10 5 | 10 4 | 9 10 | 10 10 | 2 10 |
| Bank Stability | 10 10 | 9 7 | 10 10 | 10 10 | 10 10 | 9 8 | 10 6 | 7 10 | 8 10 | 5 10 |
| Riparian Vegetative Zone Width | 10 8 | 10 2 | 6 9 | 2 1 | 2 3 | 10 2 | 10 0 | 4 1 | 9 9 | 1 3 |
| Total Score | 165 | 111 | 130 | 130 | 137 | 144 | 122 | 136 | 155 | 101 |

## Periphyton

The periphyton population and abundance data summarized in Table B11 afforded a qualitative assessment of in-stream water quality and habitats (MA DEP 1997a).  The information described is critical for the determination of dominance in-stream.  However, dominance alone does not provide all the information necessary to evaluate the impacts of algal growth on a stream.  Information on the habitat and on the algal coverage is also helpful. Current field collection methods do not include a quantitative assessment of algal cover.  Any indication of the extent of algal cover in a particular reach is based on an estimate made during the habitat assessment.  Areas with extensive algal growth are certainly identified in this manner, but areas in transition maybe overlooked.  This does limit the usefulness of the data; therefore, the analysis is limited to general comments regarding a particular site.  Comparative observations with other streams and habitats may also be described.

## Fish Population

Results from the 1997 fish population survey are presented in Table B10 (MA DEP 1997b).

## Fish Toxics

Sampling in the north basin of Falls Pond resulted in the collection of largemouth bass Micropterus salmoides, white perch Morone americana, and black crappie Pomoxis nigromaculatus. Species, length, and weight data (MA DEP 1997c) is provided in Table B12.  Lead, arsenic and cadmium concentrations were below method detection limits (Table B12) (Pb=<0.140 mg/Kg, As=<0.040 mg/Kg, and Cd=<0.020 mg/Kg) in all samples analyzed.  Selenium and mercury were detected in all samples analyzed. Mercury ranged from 0.145 mg/Kg in a three fish composite of black crappie (Nfpf97-7-9) to 0.300 mg/Kg in a three fish composite of largemouth bass (Nfpf97-1-3). Selenium ranged from 0.147 mg/Kg in the composite of largemouth bass (Nfpf97-1-3) to 0.276 in a three fish composite of white perch (Nfpf97-4-6).  PCB Arochlor 1254 was detected at a concentration of 0.33 mg/Kg in the composite sample of white perch (Nfpf97-4-6).  Organochlorine pesticides were not detected.  This data was sent to MDPH for review.  MDPH did not issue a fish consumption advisory.

Ten Mile River Basin 1997 Water Quality Assessment ReportAppendix BB22

52append.docDWM CN 18.0

Table B10.  1997 Ten Mile River Basin fish population data.  Samples collected at four sites on the Ten Mile River and two sites on the Sevenmile River.

| Station Description | Collection Date | Species1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LMB | GS | YB | CP | WS | B | P | RFP | EBT | BT | CCS | D | observed but not collected |  |
| Ten Mile River (TM02) West Bacon Street, Plainville. (just upstream of Wetherells Pond) | 9/97 |  |  |  |  |  | 3 | 14 | 8 | 6 (2)2 | 1 |  |  | 2 BT approximately 5 large EBT numerous P |  |
| Ten Mile River (TM05) upstream of Cedar Road, North Attleborough. (an approximate 100 meter upstream extension of the reach above the North Attleborough WWTP, Attleboro) | 10/97 | (3) |  |  |  |  |  | (1) | 1 |  |  |  |  |  |  |
| Ten Mile River (TM06) upstream of Cedar Road, North Attleborough. (above the North Attleborough WWTP, Attleboro) | 9/97 | 2 |  |  |  |  |  |  |  |  |  |  |  | 1 unidentified pickerel 1 unidentified sunfish |  |
| Ten Mile River (TM06A) below North Attleborough WWTP, Attleboro. | 9/97 |  |  |  |  |  |  |  |  |  |  |  |  | no fish, some crayfish |  |
| Ten Mile River (TM12) below Bridge Street, Attleboro. | 10/97 | 10 | 1 | 20 | 1 | 4 | 1 | 12 |  |  |  | 7 |  | 2 unidentified pickerel |  |
| Sevenmile River (SM00) upstream of Draper Avenue, North Attleborough. | 9/97 | 1 (2) |  |  |  |  | (12) |  | 4 |  |  |  |  |  |  |
| Sevenmile River (SM02) at Pitas Avenue, Attleboro. | 10/97 |  |  |  |  | 1 |  | 1 | 18 |  |  | 1 (TNTC3) | 35 (TNTC) |  |  |

| 1Species Code | ### Common Name | ### Scientific Name |  | 2 number in parentheses indicate young-of-the-year |
| --- | --- | --- | --- | --- |
| B | bluegill | Lepomis macrochirus |  |  |
| BT | brown trout | Salmo trutta |  | 3 TNTC - too numerous to count |
| CCS | creek chubsucker | Erimyzon oblongus |  |  |
| CP | chain pickerel | Esox niger |  |  |
| D | darters | Etheostoma sp. |  |  |
| EBT | brook trout | Salvelinus fontinalis |  |  |
| GS | golden shiner | Notemigonus crysoleucas |  |  |
| LMB | largemouth bass | Micropterus salmoides |  |  |
| P | pumpkinseed | Lepomis gibbosus |  |  |
| RFP | redfin pickerel | Esox americanus americanus |  |  |
| WS | white sucker | Catostomus commersoni |  |  |
| YB | yellow bullhead | Ameiurus natalis |  |  |

Table B11.  Periphyton population and abundance data collected by DWM at biomonitoring stations in the Ten Mile River Basin between 7 and 17 September 1997.

| Station | Date | Location | Habitat | Genus | Algal grouping | Abundance |
| --- | --- | --- | --- | --- | --- | --- |
| Sevenmile River SM00 | 10-Sep-97 | upstream from Draper Avenue, North Attleborough | riffle | Melosira | diatoms | very abundant |
|  |  |  |  | Nostoc | blue-green | rare |
|  |  |  |  | Lyngbya | blue-green | rare |
|  |  |  |  |  | sewage fungus | rare |
|  |  |  |  | Fragilaria | diatoms | rare |
|  |  |  |  | Synedra | diatoms | rare |
| Speedway (Thatcher) Brook SW01M | 17-Sep-97 | downstream from Dexter St., Attleboro | pool | Spirogyra | green | very abundant |
| Ten Mile River TM01 | 09-Sep-97 | downstream from Fuller Street, Plainville | riffle | Lyngbya | blue-green | very abundant |
|  |  |  |  | Phormidium | blue-green | abundant |
| Ten Mile River TM02 | 09-Sep-97 | downstream from West Bacon St., Plainville | riffle | Lyngbya | blue-green | very abundant |
| Ten Mile River TM04 | 07-Sep-97 | upstream from Rte. 1, North Attleborough | riffle | Spirogyra | green | very abundant |
|  |  |  |  | Mougeotia | green | rare |
| Ten Mile River TM06 |  | 09-Sep-97 | upstream from North Attleborough WWTP | riffle | Spirogyra | green | very abundant |
| Ten Mile River TM06a | 09-Sep-97 | downstream from North Attleboro WWTP | cobble, riffle | Kyliniella | red | common |
|  |  |  |  | unidentified | diatoms | common |
| Ten Mile River TM11 |  | 10-Sep-97 | downstream from Tiffany Street, Attleboro | riffle, run | Vaucheria | chrysophyta | very abundant |
| Ten Mile River TM14 | 17-Sep-97 | downstream from Central Ave., Seekonk, MA/Pawtucket, RI | riffle | Spirogyra | green | very abundant |
|  |  |  |  | Melosira | diatoms | common |
|  |  |  |  | Coleochaete | green | rare |
| Fourmile Brook FM01 |  | 10-Sep-97 | between Greenfield and West Street, Attleboro | riffle, run | Lyngbya | blue-green | abundant |

Table B12.  1997 Ten Mile River Basin Survey.  Fish toxics monitoring data for the north basin of Falls Pond, North Attleborough.  The reporting units are mg/kg unless otherwise noted.  All concentrations are in wet weight.

## Lakes/Ponds

Lake synoptic survey results (MA DEP 1997d) are presented in Table B13.

Table B13.  1997 DEP DWM Ten Mile River Basin lake survey data.

| LAKE and LOCATION | SIZE (Acres) | TROPHIC STATE | OBSERVATIONS and/or OBJECTIONABLE CONDITIONS | POTENTIAL THREATS TO WATER QUALITY |
| --- | --- | --- | --- | --- |
| Cargill Pond, Plainville | 5 | E | Water level very low, heavy turbidity (brown color), low transparency, heavy vegetation, siltation. | Gravel operation runoff |
| Central Pond, Seekonk, MA/Pawtucket/Providence, RI | 139 | H | Dense algal bloom/mats covering approximately 10% of lake area. | Road runoff |
| Chestnut Street Pond, Plainville | 10 | -- | Totally dry. Lythrum salicaria present. | ______ |
| Lake Como, Attleboro | 5 | H | Excessive turbidity, non-native aquatic plant (Cabomba caroliniana), low transparency, dense algal bloom. Lythrum salicaria also present. | Road runoff |
| Dodgeville Pond, Attleboro | 47 | H | Dense algal bloom, moderate turbidity. Lythrum salicaria present. | Railroad, new road construction |
| Falls Pond (North Basin), North Attleborough | 62 | E | Excessive turbidity, low transparency, water level down approximately 6’, algal bloom. Lythrum salicaria present. | Road runoff (erosion and storm water controls present @ Reservoir Road no longer effective), dam repair |
| Falls Pond (South Basin), North Attleborough | 60 | M | Moderate turbidity, algal bloom, Non-native plants (Myriophyllum heterophyllum). Lythrum salicaria present. | Road runoff, power lines, residential development |
| Farmers Pond, Attleboro | 9 | H | Lemna sp. covers most of surface area. Lythrum salicaria present. | Residential development, highway runoff |
| Fuller Pond, Plainville | 4 | E | Moderate turbidity, algal mats, excessive vegetation, Myriophyllum sp. needs confirmation. Lythrum salicaria and Phragmites sp. also present. | Waterfowl population, gravel operation runoff |
| Greenwood Pond, Mansfield/ North Attleborough | 153 | M | Moderate turbidity, slight algal bloom. | Development, road runoff |
| Hebronville Pond, Attleboro | 16 | H | Dense algal bloom/mats. Lythrum salicaria present. | Heavy shoreline development |
| Hoppin Hill Reservoir **, North Attleborough | 30 | U | Moderate turbidity, water level very low, silt. | Possibly dam repair, residential development, water withdrawal |
| Luther Reservoir **, North Attleborough/ Attleboro | 12 | E | Moderate turbidity, algal bloom. Lythrum salicaria present. | Runoff, residential development on shore |
| Manchester Pond Reservoir **, Attleboro | 218 | U | Lythrum salicaria present. | Rte 95 adjacent to Southeast Shore, gulls |
| Mechanics Pond, Attleboro | 9 | H | Moderate algal bloom, silt, surface scum with Lemna sp. and algae, approximately 50% of area covered with Lemna sp. Lythrum salicaria present. | Residential and industrial development |

**  Indicates Class A (water supply) waterbody;  all others are Class B.

Trophic State Codes O= Oligotrophic, M= Mesotrophic, E= Eutrophic, H= Hypereutrophic, U= Undetermined.

TABLE B13 (continued).  1997 DEP DWM Ten Mile River Basin lake survey data.

| LAKE and LOCATION | SIZE (Acres) | TROPHIC STATE | OBSERVATIONS and/or OBJECTIONABLE CONDITIONS | POTENTIAL THREATS TO WATER QUALITY |
| --- | --- | --- | --- | --- |
| Orrs Pond **, Attleboro | 48 | U | Non-native plants (Myriophyllum spicatum). Lythrum salicaria present. | None noted |
| Pawtucket Pond, Seekonk, MA/ Pawtucket, RI | 30 | -- | Pond reduced to a river channel, wetland and terrestrial plant encroachment, moderate turbidity and algal bloom. Lythrum salicaria present. | Runoff, wastewater treatment plant |
| Plain Street Pond, Mansfield | 15 | H | Moderate turbidity and algal blooms/mats, 90% cover of dense aquatic plants (only a channel through center of pond with scant surface cover), silt/muck, Non-native plants (Cabomba caroliniana). Lythrum salicaria also present. | Residential development, road runoff |
| Ten Mile Reservation Pond, Pawtucket, RI | 19 | E | Moderate turbidity, dense algal bloom, silt, approximately 75% pond covered with lilies and algae. Lythrum salicaria and Phragmites sp. also present. | Runoff, erosion, waterfowl, Pawtucket Country Club |
| James V. Turner Reservoir, Seekonk, MA/Providence, RI | 124 | H | Moderate turbidity, dense algal bloom/mats, silt. | Heavy road runoff, bank erosion |
| Wetherells Pond, Plainville | 13 | E | Moderate turbidity, dense algal bloom, low transparency, very heavy algae mats (Lemna sp., Wolffia sp., algae) cover approximately 75% area. Lythrum salicaria and Phragmites sp. also present. | Cemetery, residential |
| Whitings Pond, North Attleborough | 21 | M | Water level appeared low (down approximately 6’), slight to moderate algal bloom. Lythrum salicaria and Phragmites sp. also present. | Dense residential development, road runoff, dam repair |

**  Indicates Class A (water supply) waterbody;  all others are Class B.

Trophic State Codes O= Oligotrophic, M= Mesotrophic, E= Eutrophic, H= Hypereutrophic, U= Undetermined.

REFERENCES

Bahls, L.L. 1993.  Periphyton Bioassessment Methods For Montana Streams.  Water Quality Bureau, Department of Health and Environmental Sciences. Helena, Montana.

Dallaire, T.D. 2000.   “Water Quality Data ”. Water Quality Data Database -- WQD.  Access Application.  1999.  Massachusetts Department of Environmental Protection, Division of Watershed Management, Watershed Planning and Permitting Program, Database Manager.  Worcester, MA.

Johnson, A.S., Jonasch, J.J., Nuzzo, R.M., and Wheeler, M.  The Biological Assessment of Water Pollution in the Ten Mile River Basin – 1984.  Massachusetts Department of Environmental Quality Engineering, Division of Water Pollution Control, Technical Services Branch.  Westborough, MA.

MA DEM. 1998. Open File.  Monthly Precipitation Report. Commonwealth of Massachusetts, Executive Office of Environmental Affairs, Department of Environmental Management. Boston, MA.

MA DEP.  1994.  Laboratory Quality Assurance Plan and Standard Operating Procedures.  Massachusetts Department of Environmental Protection, Division of Environmental Analysis.  Wall Experiment Station, Lawrence, MA.

MA DEP. 1997a. Open File.  1997 Periphyton Data for the Ten Mile River Basin. Massachusetts Department of Environmental Protection, Division of Watershed Management, Watershed Planning and Permitting Program, Worcester, MA.

MA DEP. 1997b. Open File.  1997 Fish Population Survey Data for the Ten Mile River Basin.  Massachusetts Department of Environmental Protection, Division of Watershed Management, Watershed Planning and Permitting Program, Worcester, MA.

MA DEP. 1997c. Open File.  1997 Fish Toxics Monitoring Data for the Ten Mile River Basin.  Massachusetts Department of Environmental Protection, Division of Watershed Management, Watershed Planning and Permitting Program, Worcester, MA.

MA DEP. 1997d. Open File.  1997 Synoptic Lake Surveys Coding Sheets for the Ten Mile River Basin.  Massachusetts Department of Environmental Protection, Division of Watershed Management, Watershed Planning and Permitting Program, Worcester, MA.

Plafkin, J.L., M.T. Barbour, K.D. Porter, S.K. Gross, R.M. Hughes.  1989.  Rapid bioassessment protocols for use in streams and small rivers: benthic macroinvertebrates and fish.  U.S. Environmental Protection Agency.  Washington, D.C.  EPA/444/4-89-001.

Socolow, R.S., C.R. Leighton, J.L. Zanca, and L.R. Ramsbey.  1998.  Water Resources Data for Massachusetts and Rhode Island, Water Year 1997.  U.S. Geological Survey Report MA-RI-97-1.  Water Resources Division.  Marlborough, MA.

TetraTech, Inc.  1995.  Massachusetts DEP Preliminary Biological and Assessment Protocols for Wadeable Rivers and Streams.  Method 004 Revision 1  Owings Mills, MD.

USGS.  1998.  Unpublished Data. Provisional low-flow frequency statistics for gaging stations (3.5” floppy disc).  United States Geological Survey, Water Resources Division. Marlborough, MA.

Wandle Jr., S.W and G.R. Keezer  1984.  Gazetteer of Hydrological Characteristics of Streams in Massachusetts— Taunton and Ten Mile River Basins and Coastal River Basins of Mount Hope Bay, Narragansett Bay, and Rhode Island Sound.  U.S. Geological Survey Water-Resources Investigation Report 84-4283. Boston, MA.

# APPENDIX C – DEP BIOMONITORING TECHNICAL MEMORANDUM

To:		Ten Mile River Watershed Team, David Burns, DEP, SERO

From:		Robert Nuzzo, DWM

Date:		25 March 1997

Subject:	Results of 1990 biomonitoring in the Ten Mile River watershed.

As part of the water quality monitoring program conducted in the Ten Mile River watershed in 1990, benthic macroinvertebrate samples were collected on 17, 18, and 19 September.  The samples were collected from 11 stations within the watershed:

SM00--Located in the Sevenmile River downstream from Draper Avenue, North Attleborough, MA.  This site was intended as one of the two watershed references.

TM01--The most upstream sampling location on the Ten Mile River, it was situated just downstream from Fuller Street (outlet of Fuller Pond), Plainville, MA.  This was also used as a watershed reference station.

TM02--Ten Mile River upstream from West Bacon Street, Plainville, MA

TM04A--Ten Mile River behind trailer park upstream from East Washington St. (Route 1), North Attleborough, MA

TM06--Ten Mile River upstream from Cedar Street, North Attleborough, MA.  This station served as the upstream bracket for the North Attleborough wastewater treatment plant.

TM06A--Ten Mile River downstream from Cedar Street and the wastewater treatment plant discharge, North Attleborough.

TM08A--Ten Mile River downstream from Olive Street, Attleboro, MA.

TM11--Ten Mile River downstream from Tiffany Street, Attleboro, MA.

TM12--Ten Mile River downstream from Hebronville Dam (off Bridge Street) and the railroad bridge, Attleboro.  The station served as the upstream bracket for the Attleboro wastewater treatment plant.

TM14--Ten Mile River downstream from the Attleboro wastewater treatment plant and Central Avenue, Pawtucket, RI.

SW01--Speedway Brook downstream from Route 152, Attleboro, MA.

Laboratory-sorted random 100-organism subsamples were taken from each sample within a few weeks of collection; but no taxonomy was done on these specimens at that time.  We have recently brought the specimens out to do family-level taxonomy in the event the biomonitoring data may be of use to you in planning your information/data needs for the upcoming survey year (1997).

By using the counts of families present, RBP II (Plafkin et al. 1989) metrics can be calculated.  These metrics are then compared against those from a reference station (usually a site upstream on the same stream or a surrogate within the watershed) and used to score the relative status of the health of the aquatic community.  With family-level identifications the categories that can be differentiated are non-impaired, moderately impaired, and severely impaired.  This is a good screen for sites that are in good health and for sites that are in need of urgent attention.  Those sites that score in the moderately impaired category, however, need to be looked at in more depth, in the context of supporting data, or may require additional data collections to clarify their status.  Identifying specimens to genus/species can help remove some ambiguities in the data, and results in scoring categories of non-impaired, slightly impaired, moderately impaired, and severely impaired.  In any event, a result of moderately impaired from family-level identifications indicates a need for some kind of follow-up, such as carrying out the taxonomy beyond family or  collection of additional monitoring data.

SAMPLING AND LABORATORY METHODS

Benthic macroinvertebrates were collected by kicking substrates in front of a D-net while slowly moving upstream (traveling kick-sample technique).  A total of 2 m2 of stream bottom was sampled and composited into a single sample from each site.  Samples were preserved in the field using 95% ethanol and processed at the OWM facility.  Processing involved draining the preservative through a number 30 soil sieve and transferring all sample materials to a gridded pan; a small amount of water was added to aid in distributing the material across the pan.  Grids were selected through a randomized process and organisms were extracted from the grids until a count of approximately 100 had been reached.  The extracted organisms were preserved in 70% ethanol and later identified to family.

RESULTS

The list of taxa and their relative abundance at each station is shown in Table C1.  These data were used to calculate richness (number of different families represented), biotic index, EPT index, EPT/Chironomidae, scrapers/filtering collectors, percent contribution of dominant family, and percent similarity to reference community.  These metrics were compared to the values from a reference to obtain an impairment score.  Two different “reference” comparisons were made: using the most upstream site available on the Ten Mile River (Table C2); and using upstream/downstream pairs bracketing the North Attleborough and Attleboro wastewater treatment plants (Table C3).  For some of the sites--TM01, TM02, and SM00--comparisons were made to 1984 data adapted to the RBP II analysis.  The adapted taxa list based on the 1984 species (Johnson, et al., 1986) list is shown in Table C4, while the corresponding RBP II data summary for the 1990 to 1984 comparisons is in Table C5.

Most of the sites sampled had relatively low habitat scores (perfect score is 135).  In many cases these scores resulted from, or were at least suggestive of, erosional and NPS pollution problems leading to habitat degradation.  While locating and identifying nonpoint sources was not part of the scope of the 1990 assessments, the habitat descriptions often flagged some of these problems or indicated where efforts might be focussed to address them.

For each site a list of upstream influences is provided to lend perspective to benthic invertebrate results.  Although the list is identified as intervening “discharges” I have included other influences I feel may be important--most notably, impoundments and tributaries.   Since nearly all of the discharges have been removed from the river I have included the discontinued discharges noting in parentheses that they are historical.  This information may be useful in trying sort out causes of current ecological conditions, from these and future data.

SM00  Sevenmile River downstream from Draper Avenue, North Attleborough, MA (19 Sept. 1990)

HABITAT

Habitat for fishes and aquatic macroinvertebrates at this station was judged to be very good.  The total habitat score for this site was 116, much higher than all but the two most downstream sampling sites in the watershed.  About 80% of the bottom substrates were cobble and boulder.  No local watershed erosion was noticed, although some potential for nonpoint source (NPS) pollution was identified in conjunction with DPW “dirt” piles.  The eastern bank was densely vegetated with shrubs (Cornus sp.) while the western bank had extensive stands of Phragmites sp.

BENTHOS

SM00 was selected as a reference site for biomonitoring results within the Ten Mile River watershed because there were no NPDES permitted discharges upstream from it and it is included in a reach designated Class A (public drinking water supply) by the Water Quality Standards.  Indeed, the expectation was that the attributes of the macroinvertebrate assemblage would set a high “standard” for comparison of the other sites in the watershed.  Yet when TM01 was used as the reference site SM00 was ranked as moderately impaired. This is surprising because TM01 had some obvious habitat problems, resulting in a habitat score only about two-thirds that of SM00.  Moderately impaired was also the result when SM00 was compared to itself from 1984 data.  This may indicate water quality has been deteriorating in the river.  Given the Class A status and high habitat score of SM00, these results suggest a need for a more thorough investigation of the river corridor to try to identify potential nonpoint sources of pollution, better analyze why this site apparently is not measuring up to its potential, and determine if appropriate practices are being employed to protect the Class A status of the Sevenmile River.

TM01  Ten Mile River downstream from Fuller Pond and Fuller Street, Plainville, MA (17 Sept. 1990)

HABITAT

The habitat at this site scored 76, lower than nearly every other site sampled in this watershed.  The biggest habitat problems appeared to be associated with scour and deposition of fine sediment materials (15 to 20 cm deep in spots).  Field notes indicated that this problem was probably connected to a gravel company upstream from Fuller Pond.  Evidently a heavy rainfall (approx. 18 cm, or seven inches) caused the company’s siltation lagoon to overflow and carry a heavy silt load into Fuller Pond--and, presumably, the Ten Mile River.

BENTHOS

In spite of the habitat problems at TM01, the assemblage of macroinvertebrates indicated better health than at any of the other sampling locations.  Had there been a reference site available in this watershed that was unimpacted by human activities TM01 likely would not have scored well.  Indeed, when compared against data from this same site in 1984 the RBP II analysis of the 1990 data indicated a status of moderately impaired (Table C5).

Upstream “discharges”: Lorusso Corp.; Fuller Pond.

TM02  Ten Mile River upstream from Bacon Street, Plainville, MA (17 Sept. 1990)

HABITAT

The land use along this stretch of the river was characterized as industrial.  Even so, the habitat score was 92, slightly better than at TM01.  The major mark-downs appeared to be related to substrate and flow inadequacies.  No obvious NPS problems were recorded but a parking lot adjacent to the stream bank was identified as a potential source.

### BENTHOS

This site scored as moderately impaired compared against both TM01 in 1990 and TM02 in 1984.  The extreme imbalance in the assemblage and the paucity of, in particular, mayflies and caddisflies make this community appear more unhealthy than its ranking might suggest (i.e., I’m surprised it didn’t score in the severely impaired range).  It is probably a good idea to look carefully at any instream chemistry data, discharge monitoring reports, and sediment data that may be available.  It may also be worthwhile to complete the taxonomy to genus/species on this sample.

Intervening discharges:	Whiting and Davis (historical).

TM04A  Ten Mile River upstream from East Washington Street (Rte. 1) North Attleborough, MA (18 Sept. 1990)

### HABITAT

The habitat score at this site was 69, the lowest score among the sites sampled (still 91% of the reference station habitat score).  The major habitat deficiencies appeared to be related to channelization and flow status.  Some potential for NPS-related problems was noted.  Some oiliness of the sediments was detected, and there was siltation along the stream margins and even around the cobble/gravel substrates.

### BENTHOS

The score for TM04A ranked the site as moderately impaired.  The difference in the habitat score as compared to the reference is not sufficient to account for the impairment ranking.

Intervening “discharges”: Hilsinger Corp. (historical); Cook-Horton (historical); L.S. Peterson Co. (historical); N. Attleborough Taps (historical); Handy & Harmon (historical); L.G. Balfour (historical); Wetherells Pond; Whiting Pond.

TM06 and TM06A  Ten Mile River upstream (TM06) and downstream (TM06A) from Cedar Street and the wastewater treatment plant, North Attleborough, MA (18 Sept. 1990)

HABITAT

These two sites were only about 300 m apart and were selected to bracket the discharge from the North Attleborough wastewater treatment plant.  There was no evidence of local watershed erosion and no potential nonpoint sources of pollution were identified.  The habitat score at TM06 was 82 and at TM06A was 91.

BENTHOS

TM06 ranked as moderately impaired relative to TM01.  TM06A was ranked as moderately impaired against both TM01 and TM06.  Because the habitat score at TM06 was lower than the score downstream at TM06A, and because of how tightly these locations bracketed the wastewater discharge, the degradation in community health evident from the benthos data is likely due to the intervention of the wastewater discharge, and not habitat quality differences.

Intervening “discharges”: between TM04 and TM06--C. Ray Randall (historical), B&J Manufacturing (historical), and Falls Pond; between TM06 and TM06A--North Attleborough wastewater treatment plant.

TM08A  Ten Mile River downstream from Olive Street, Attleboro, MA (17 Sept. 1990)

HABITAT

The habitat score here was 94.  The surrounding land use was characterized as predominantly forest, but with some commercial uses present.  Field notes indicated local watershed erosion was moderate and that some potential for NPS pollution existed in conjunction with a coal or asphalt pile downstream from Olive Street.  Sand deposits were observed instream and filling catch basins.  Chlorine odors wafted through periodically, and the water was slightly turbid and gray (often an indicator of untreated or insufficiently treated sewage).

BENTHOS

The RBP II score for TM08A placed it in the moderately impaired category.  This situation should be investigated further to see if it is attributable to erosion/NPS problems or residual effects of discharges (past and present).  One potentially important influence between TM06A and TM08A is the confluence with the Bungay River.

Intervening “discharges”: Walton and Lonsbury (historical), Mt. Vernon Silver (historical); L.G. Balfour (historical); Foster Metal Products (historical); Lambert Anodizing (historical); Montrose-Heuser (historical); Farmers Pond; Mechanics Pond; Bungay River.

TM11  Ten Mile River downstream from Tiffany Street, Attleboro, MA (19 Sept. 1990)

HABITAT

This habitat score was 90.  Predominant surrounding land use was forest, with some residential.  No local watershed erosion or NPS pollution was evident at the time of sampling.  The water column was clear and colorless and no odors were detected.

BENTHOS

This site fell into the moderately impaired category.  Given the habitat score and the lack of obvious indications of erosion or NPS problems, a better result was expected here.  It is difficult to tell if this result is because of polluting influences downstream from TM08A, or if improvements in water quality between the two locations are simply too slight to detect with this analysis.

Intervening “discharges”: Leavens Corp. (historical); Speedway Brook; Dodgeville Pond.

TM12  Ten Mile River downstream from Hebronville Dam (off Read Street), Attleboro, MA (20 Sept. 1990)

HABITAT

The habitat score was 108, one of the highest among the sampled sites.  The surrounding land uses were a mix of industrial and residential upstream and field and forest downstream.  No evidence of local watershed erosion or NPS problems was detected at the time of sampling.  The water column was clear and colorless.

BENTHOS

This site scored in the moderately impaired category, in spite of a good habitat score and no obvious erosion or NPS problems.

Intervening “discharges”: Handy and Harmon (historical) and Hebronville Impoundment.

TM14  Ten Mile River downstream from Central Avenue, Pawtucket, RI (20 Sept. 1990)

HABITAT

The habitat score was 113, one of the two highest scores among the sites sampled.  The predominant surrounding land use was commercial.  Slight areas of erosion were noticed along the river banks and potential sources of NPS contamination were present.  The water column was clear and colorless at the time of sampling.

BENTHOS

The data from this site produced a rating of moderately impaired when compared against TM01, but non-impaired against TM12.  It appears, then, that the wastewater treatment plant does not cause further degradation in water quality at this point in the river.

Intervening “discharges”: Attleboro wastewater treatment plant; Sevenmile River; unnamed impoundment (upstream from the treatment plant).

SW01  Speedway Brook downstream from route 152, Attleboro, MA (19 Sept. 1990)

HABITAT

The habitat score was 76.  The surrounding land uses were predominantly a mix of forest and field.  There were no indications of erosion or potential NPS problems in the immediate drainage area.  The riffle habitat in this reach was restricted to a small area close to the bridge.  The water column appeared to be clear and colorless--a decided improvement over its appearance during the 1984 survey.

BENTHOS

This site was rated moderately impaired.  In spite of the visual indications of improvements in the water quality of Speedway Brook it may be that more time is required to see significant improvements reflected in instream communities.  A more sensitive analysis (e.g., RBP III) may be able to reveal slight improvements here.

Upstream discharges: Leach and Garner (historical), Swank Inc. (historical), and Robbins Co. (historical).

SUMMARY AND RECOMMENDATIONS

The RBP II analysis showed all sites sampled in this watershed to be moderately impaired when compared against the most upstream site on the Ten Mile River.  Habitat scores at all sites were at least 90% of the reference site habitat score (all but two were higher) suggesting that habitat factors alone are not limiting benthic communities at these sites.  Inasmuch as most of the direct discharges have been eliminated from the river the question arises as to whether there are residual effects from them.  It is also possible that nonpoint source pollution or illegal discharges are causes of the detected impairment, or that they are slowing the recovery of the river since the discharges have been eliminated.

A few sites stand out as particularly high priority in spite of the fact all sites scored the same.  The most obvious of these is the Sevenmile River at SM00.  Its habitat score and Class A status mean that it should have exhibited the healthiest RBP II score in the watershed.  More intensive investigation of this river should be done to corroborate this finding and to scrutinize measures being taken to protect its Class A status.

The field descriptions and resultant habitat score at TM01 document habitat damage caused by deposition of fine sediment materials.  The causes of this problem need to be investigated.  The sand and gravel company should be scrutinized to see if they are taking adequate steps to prevent fine materials from migrating into Fuller Pond, and eventually into the Ten Mile River.  At the same time, however, other land uses upstream from TM01 should also be examined as possible sources of sediment loading.

While TM02 ranked the same as all the remaining sites (moderately impaired) I would place it at a higher priority for follow-up than the others for two reasons.  One is that its habitat score is slightly better than TM01's and the other is that it is only a short distance downstream from TM01.  In this context it should have been the most similar to the reference community and yet it was the least.  Additional biomonitoring work should probably be done here along with sediment sampling and a thorough reconnaissance of the reach upstream to TM01 to identify possible nonpoint sources of pollution.

Depending on resources and other priorities of the team, probably all of the sites used in 1990 should be sampled again for macroinvertebrates, sediment, and water quality.  A more intensive effort should be made to investigate possible problem spots in each reach and identifying the probable causes of ecological impairment based on the data presented here.  Together, past and present data can help sort out to what extent the Ten Mile River is still suffering from past damage and to what extent existing pressures are causing harm.

cc:	A. Johnson

R. McVoy

LITERATURE CITED

Johnson, A.S., J.J. Jonasch, R.M. Nuzzo, M. Wheeler.  1986.  The biological assessment of water pollution in the Ten Mile River Basin--1984.  Massachusetts Dept. Of Environmental Quality Engineering.  Westborough, MA.  84 p.

Plafkin, J.L., M.T. Barbour, K.D. Porter, S.K. Gross, R.M. Hughes.  1989.  Rapid bioassessment protocols for use in streams and rivers: benthic macroinvertebrates and fish.  U.S. Environmental Protection Agency.  EPA/444/4-89-001.  152 p.

LIST OF TABLES

Table C1.  List of macroinvertebrate taxa collected by DEP DWM from stream sites in the Ten Mile River watershed between 17 and 19 September 1990.

Table C2.  Summary of RBP II data analysis for macroinvertebrate communities sampled by DEP DWM at eleven stream sites in the Ten Mile River watershed.

Table C3.  Summary of RBP II data analysis for macroinvertebrate communities sampled by DEP DWM at stream sites bracketing the N. Attleboro (TM06 and TM06A) and Attleboro (TM12 and TM14) wastewater treatment plants in the Ten Mile River watershed.

Table C4.  List of macroinvertebrate families collected by DEP DWM from stream sites in the Ten Mile River watershed between 17 and 20 September 1984.

Table C5.  Summary of RBP II data comparisons for macroinvertebrate communities sampled by DEP DWM at selected stream sites in the Ten Mile River watershed.

Table C1.  List of macroinvertebrate taxa collected by DEP DWM from stream sites in the Ten Mile River watershed between 17 and 19 September 1990.  Sampling sites were in: Ten Mile River (TM01, TM02, TM04A, TM06, TM06A, TM08A, TM11, TM12, TM14); Sevenmile River (SM00); and Speedway Brook (SW01).  All were in Massachusetts except TM14 (Pawtucket, RI).

| TAXA | FFG | TV | TM01 | TM02 | TM04A | TM06 | TM06A | TM08A | TM11 | TM12 | TM14 | SW01 | SM00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Physidae | GC | 8 |  |  |  |  |  | 2 |  |  |  |  |  |
| Ancylidae | SC | 7 |  |  | 1 |  |  |  |  |  |  |  |  |
| Pisidiidae | FC | 6 | 1 |  |  |  |  | 8 | 2 | 4 |  |  | 5 |
| Tubificidae | GC | 10 |  |  | 6 |  |  | 5 |  |  |  |  |  |
| Naididae | GC | 9 | 1 |  |  |  |  |  |  |  |  |  | 10 |
| Lumbriculidae | GC | 7 | 2 |  |  | 4 |  |  |  |  |  |  | 11 |
| Glossophoniidae | PR | 7 |  |  | 1 |  |  |  |  |  |  |  |  |
| Erpobdellidae | PR | 8 |  |  |  |  |  |  |  | 2 |  |  |  |
| Asellidae | GC | 8 |  |  | 8 |  |  | 4 | 1 | 8 |  |  |  |
| Crangonyctidae | GC | 8 |  |  | 1 |  |  |  |  |  |  |  |  |
| Gammaridae | GC | 6 |  | 95 |  |  |  |  |  |  |  |  | 10 |
| Baetidae | GC | 4 |  |  |  |  | 3 |  |  |  |  |  |  |
| Heptageniidae | SC | 4 | 9 | 1 |  | 4 |  |  |  | 1 | 3 |  | 1 |
| Aeschnidae | PR | 3 |  |  |  | 2 |  |  |  |  | 1 |  | 2 |
| Corydalidae | PR | 5 |  |  |  | 3 | 1 |  |  |  | 2 | 6 |  |
| Philopotamidae | FC | 3 | 1 |  |  |  |  |  |  |  |  |  |  |
| Hydropsychidae | FC | 4 | 44 | 2 | 25 | 55 | 21 | 13 | 15 | 19 | 41 | 78 | 12 |
| Hydroptilidae | GC | 4 | 1 |  |  |  |  |  |  |  |  |  | 2 |
| Limnephilidae | SH | 4 |  |  |  | 1 |  |  |  |  |  |  | 1 |
| Elmidae | SC | 4 | 6 |  | 1 | 5 | 3 | 11 | 90 | 62 | 9 | 3 | 2 |
| Tipulidae | SH | 5 |  | 2 |  |  |  |  |  |  |  |  |  |
| Simuliidae | FC | 6 | 30 | 2 | 10 | 5 | 48 | 28 | 3 | 2 | 16 | 2 | 8 |
| Chironomidae | GC | 6 | 4 | 1 | 37 | 29 | 11 | 38 | 3 | 16 | 35 | 19 | 30 |
| Empididae | PR | 6 |  |  | 2 | 1 | 3 |  |  |  |  | 5 | 1 |
| TOTAL |  |  | 99 | 103 | 92 | 109 | 90 | 109 | 114 | 114 | 107 | 113 | 95 |

Ten Mile River Basin 1997 Water Quality Assessment Report         Appendix CC8

52append.doc              DWM CN 18.0

Table C2.  Summary of RBP II data analysis for macroinvertebrate communities sampled by DEP DWM at eleven stream sites in the Ten Mile River watershed. Seven biological metrics were calculated for taxa collected at each station, and scored (in parentheses).  Scores were totaled and compared to station TM01 for reference.  The percent comparability to the reference station yields a final impairment score for each station.

RBP DATA SUMMARY FOR: TEN MILE RIVER WATERSHED       DATE: 1990

| Station # | TM01* |  | TM02 |  | TM04A |  | TM06 |  | TM06A |  | TM08A |  | TM11 |  | TM12 |  | TM14 |  | SW01 |  | SM00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stream | Ten Mile River |  | Ten Mile River |  | Ten Mile River |  | Ten Mile River |  | Ten Mile River |  | Ten Mile River |  | Ten Mile River |  | Ten Mile River |  | Ten Mile River |  | Speedway Brook |  | Sevenmile River |  |
| Habitat Score | 76 |  | 92 |  | 69 |  | 82 |  | 91 |  | 94 |  | 90 |  | 108 |  | 113 |  | 76 |  | 116 |  |
| Taxa Richness | 10 | (6) | 6 | (3) | 10 | (6) | 10 | (6) | 7 | (3) | 8 | (3) | 6 | (3) | 8 | (3) | 7 | (3) | 6 | (3) | 13 | (6) |
| Biotic Index | 4.81 | (6) | 5.92 | (3) | 5.91 | (3) | 4.76 | (6) | 5.39 | (6) | 5.85 | (3) | 4.18 | (6) | 4.74 | (6) | 4.96 | (6) | 4.51 | (6) | 5.99 | (3) |
| EPT Index | 4 | (6) | 2 | (0) | 1 | (0) | 3 | (3) | 2 | (0) | 1 | (0) | 1 | (0) | 2 | (0) | 2 | (0) | 1 | (0) | 4 | (6) |
| EPT/Chironomidae | 13.75 | (6) | 3 | (0) | 0.68 | (0) | 2.07 | (0) | 2.18 | (0) | 0.34 | (0) | 5 | (3) | 1.25 | (0) | 1.26 | (0) | 4.11 | (3) | 0.53 | (0) |
| Riffle Community: Scrapers/Filt. Coll. | 0.20 | (6) | 0.25 | (6) | 0.057 | (3) | 0.14 | (6) | 0.043 | (0) | 0.22 | (6) | 4.5 | (6) | 2.5 | (6) | 0.21 | (6) | 0.038 | (0) | 0.12 | (6) |
| % Contribution (Dominant Family) | 44% | (3) | 92 | (0) | 40% | (3) | 50% | (3) | 53% | (0) | 35% | (3) | 79% | (0) | 54% | (0) | 38% | (3) | 69% | (0) | 32% | (3) |
| Community Similarity | 100% | (6) | 6% | (0) | 43% | (3) | 64% | (3) | 60% | (3) | 49% | (3) | 26% | (0) | 31% | (3) | 66% | (3) | 53% | (3) | 33% | (3) |
| Total Metric Score |  | 39 |  | 12 |  | 18 |  | 27 |  | 12 |  | 18 |  | 18 |  | 18 |  | 21 |  | 15 |  | 27 |
| % Comparability To Reference Station | 100% |  | 31% |  | 46% |  | 69% |  | 31% |  | 46% |  | 46% |  | 46% |  | 54% |  | 38% |  | 69% |  |
| Biological Status- Degree Impairment | Reference |  | Moderately Impaired |  | Moderately Impaired |  | Moderately Impaired |  | Moderately Impaired |  | Moderately Impaired |  | Moderately Impaired |  | Moderately Impaired |  | Moderately Impaired |  | Moderately Impaired |  | Moderately Impaired |  |

*Watershed reference station for all.

Table C3.  Summary of RBP II data analysis for macroinvertebrate communities sampled by DEP DWM at stream sites bracketing the N. Attleboro (TM06 and TM06A) and Attleboro (TM12 and TM14) wastewater treatment plants in the Ten Mile River watershed. Seven biological metrics were calculated for taxa collected at each station, and scored (in parentheses).  Scores were totaled and compared to the upstream station in each pair. The percent comparability to the reference station yields a final impairment score for the downstream station.

RBP DATA SUMMARY FOR: TEN MILE RIVER WATERSHED      DATE: 1990

| Station # | TM061 |  | TM06A |  | TM122 |  | TM14 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stream | Ten Mile River |  | Ten Mile River |  | Ten Mile River |  | Ten Mile River |  |
| Habitat Score | 82 |  | 91 |  | 108 |  | 113 |  |
| Taxa Richness | 10 | (6) | 7 | (3) | 8 | (6) | 7 | (6) |
| Biotic Index | 4.76 | (6) | 5.39 | (6) | 4.74 | (6) | 4.96 | (6) |
| EPT Index | 3 | (6) | 2 | (0) | 2 | (6) | 2 | (6) |
| EPT/Chironomidae | 2.07 | (6) | 2.18 | (6) | 1.3 | (6) | 1.3 | (6) |
| Riffle Community: Scrapers/Filt. Coll. | 0.14 | (6) | 0.043 | (3) | 2.5 | (6) | 0.21 | (6) |
| % Contribution (Dominant Family) | 50% | (3) | 53% | (0) | 54% | (0) | 38% | (3) |
| Community Similarity | 100% | (6) | 45% | (3) | 100% | (6) | 42% | (3) |
| Total Metric Score |  | 39 |  | 21 |  | 36 |  | 36 |
| % Comparability To Reference Station | 100% |  | 54% |  | 100% |  | 100% |  |
| Biological Status- Degree Impairment | Reference |  | Moderately Impaired |  | Reference |  | Non-Impaired |  |

1 Upstream reference station for TM06A.

2 Upstream reference station for TM14.

Table C4.  List of macroinvertebrate families collected by DEP DWM from stream sites in the Ten Mile River watershed between 17 and 20 September 1984.  Sampling sites were in: Ten Mile River (TM01, TM02, TM04, TM06, TM06A, TM08A, TM11, TM12, TM14); Sevenmile River (SM00); and Speedway Brook (SW01).  All were in Massachusetts except TM14 (Pawtucket, RI).   This list was adapted for RBP II analysis from the 1984 species list (Johnson, et al. 1986).

| TAXON | FFG | TOL. VAL. | SM00 | TM01 | TM02 | TM04 | TM06 | TM06A | TM08A | TM11 | TM12 | TM14 | SW01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Physidae | GC | 8 | 5 |  |  |  |  |  |  |  |  |  |  |
| Planorbidae | SC | 6 |  | 1 | 1 |  |  |  |  |  |  |  |  |
| Tubificidae | GC | 10 |  |  | 4 | 8 |  |  | 12 |  | 8 |  | 4 |
| Naididae | GC | 9 | 1 |  |  |  |  |  |  |  |  |  |  |
| Lumbriculidae | GC | 7 |  | 6 |  |  |  |  |  |  |  |  |  |
| Erpobdellidae | PR | 8 |  | 1 |  |  |  |  | 1 |  |  |  |  |
| Asellidae | GC | 8 |  | 1 | 5 | 1 |  |  | 16 |  | 4 | 1 |  |
| Gammaridae | GC | 6 | 6 |  | 1 |  |  |  |  |  |  |  |  |
| Hyalellidae | GC | 8 |  | 1 | 2 |  |  |  |  |  |  |  |  |
| Hydracarina | PR | 6 |  |  | 2 |  |  |  | 1 |  |  |  |  |
| Baetidae | GC | 4 | 1 |  |  |  | 6 | 1 |  | 1 |  |  |  |
| Heptageniidae | SC | 4 | 1 | 29 | 1 |  |  |  |  |  |  |  |  |
| Ephemerellidae | GC | 1 |  |  | 1 |  |  |  |  |  |  |  |  |
| Caenidae | GC | 7 |  |  |  |  |  |  |  |  | 2 |  |  |
| Leptophlebiidae | GC | 2 |  | 13 |  |  |  |  |  |  |  |  |  |
| Aeschnidae | PR | 3 | 3 |  |  |  |  |  |  |  |  |  |  |
| Libellulidae | PR | 9 |  |  |  |  |  |  |  |  | 2 |  |  |
| Calopterygidae | PR | 5 | 5 |  |  |  |  | 1 |  |  |  |  |  |
| Coenagrionidae | PR | 9 |  |  |  | 3 |  |  |  |  | 3 |  |  |
| Sialidae | PR | 8 | 1 |  | 2 | 7 |  |  |  |  |  |  |  |
| Corydalidae | PR | 5 |  | 3 | 5 |  |  | 38 |  |  |  |  |  |
| Philopotamidae | FC | 3 |  | 1 |  |  |  |  |  |  |  |  |  |
| Polycentropodidae | FC | 6 | 4 |  |  |  |  |  |  |  |  |  |  |
| Hydropsychidae | FC | 4 | 26 | 26 | 3 | 1 | 65 | 9 | 14 | 69 | 3 | 86 | 1 |
| Hydroptilidae | GC | 4 | 1 |  |  |  |  |  |  |  |  | 1 |  |
| Phryganeidae | SH | 4 |  |  |  |  |  |  |  |  | 2 |  |  |
| Leptoceridae | PR | 4 |  |  |  |  | 1 | 6 | 1 |  | 7 |  |  |
| Halipildae | SH | 5 | 1 |  |  |  |  |  |  |  |  |  |  |
| Hydrophilidae | PR | 5 |  |  |  |  | 1 |  |  |  | 2 |  |  |
| Elmidae | SC | 4 |  | 1 |  |  |  |  |  |  | 7 |  |  |
| Tipulidae | SH | 5 | 3 | 1 | 2 |  |  |  |  |  |  |  |  |
| Ceratopogonidae | PR | 6 |  |  | 2 | 1 |  |  |  |  |  |  |  |
| Simuliidae | FC | 6 |  |  |  |  | 6 | 1 | 1 | 8 |  | 6 |  |
| Chironomidae | GC | 6 | 41 | 15 | 65 | 79 | 20 | 23 | 20 | 28 | 57 | 2 | 81 |
| Empididae | PR | 6 |  |  |  |  | 3 | 22 | 34 |  |  | 3 | 11 |
| TOTAL |  |  |  | 99 | 99 | 97 | 100 | 102 | 101 | 100 | 106 | 97 | 99 | 97 |

Table C5.  Summary of RBP II data comparisons for macroinvertebrate communities sampled by DEP DWM at selected stream sites in the Ten Mile River watershed. Seven biological metrics were calculated for taxa collected at each station, and scored (in parentheses).  Scores were totaled and the 1990 data set for each station was compared to its 1984 data set for reference.  The percent comparability to the reference station yields a final impairment score for each station.

| Station # | 84-TM01 |  | 90-TM011 |  | 84-SM00 |  | 90-SM002 |  | 84-TM02 |  | 90-TM023 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stream | Ten Mile River |  | Ten Mile River |  | Sevenmile River |  | Sevenmile River |  | Ten Mile River |  | Ten Mile River |  |
| Habitat Score |  |  | 76 |  |  |  | 116 |  |  |  | 92 |  |
| Taxa Richness | 13 | (6) | 10 | (3) | 14 | (6) | 13 | (6) | 14 | (6) | 6 | (3) |
| Biotic Index | 4.4 | (6) | 4.8 | (6) | 5.4 | (6) | 6.0 | (6) | 6.1 | (6) | 5.9 | (6) |
| EPT Index | 4 | (6) | 4 | (6) | 5 | (6) | 4 | (3) | 3 | (6) | 2 | (0) |
| EPT/Chironomidae | 4.6 | (6) | 14 | (6) | 0.80 | (6) | 0.53 | (3) | 0.076 | (6) | 3.0 | (6) |
| Riffle Community: Scrapers/Filt. Coll. | 1.1 | (6) | 0.20 | (0) | 0.033 | (6) | 0.12 | (6) | 0.67 | (6) | 0.25 | (3) |
| % Contribution (Dom. Fam.) | 29 | (6) | 44 | (3) | 41 | (3) | 32 | (3) | 68 | (0) | 92 | (0) |
| Community Similarity | 100% | (6) | 42% | (3) | 100% | (6) | 23% | (0) | 100% | (6) | 7% | (0) |
| Total Metric Score |  | 42 |  | 27 |  | 39 |  | 27 |  | 36 |  | 18 |
| % Compararbility To Reference Station | Reference |  | (27/42) 64 |  | Reference |  | (27/39) 69 |  | Reference |  | (18/36) 50 |  |
| Biological Status--Degree Impairment |  |  | moderately impaired |  |  |  | moderately impaired |  |  |  | moderately impaired |  |

1 Ref.: 84-TM01

2 Ref.: 84-SM00

3 Ref.: 84-TM02