| # 2000 Toxics Use Reduction # Information Release # June 2002 |  |  |
| --- | --- | --- |
| # Commonwealth of Massachusetts # Department of Environmental Protection # Developed in collaboration with: Office of Technical Assistance for Toxics Use Reduction Toxics Use Reduction Institute ### Executive Office of Environmental Affairs |  |  |

# Table of Contents

Executive Summary							Pg. 1

- TURA Progress 1990-2000					Pg. 4

- 2000 TURA Chemical Data					Pg. 12

- 2000 Significant Industrial Sectors				Pg. 22

- 2000 Major TURA Facilities					Pg. 25

V.	2000 Top 5 Chemicals	 				Pg. 30

#### Key TURA Terms						Pg. 35

Executive Summary

The Toxics Use Reduction Act (TURA) Program now has 11 years of toxics use information in Massachusetts.  This information shows that manufacturers and other businesses statewide have reduced their reliance on toxic chemicals dramatically, making Massachusetts the national leader in demonstrable reductions in toxic chemical use and providing clear evidence that the state has made tremendous progress in pollution prevention.

In 2000, 559 facilities reported the use of 192 listed toxic substances.  These facilities fell within certain standard industrial classification (SIC) codes, had ten or more full-time employees, and used listed toxic substances at or above reporting thresholds.  These facilities reported that they used nearly 1.4 billion pounds of listed toxic substances, generated 127.8 million pounds of byproduct (or waste), shipped 424.4 million pounds in or as products, released 10.8 million pounds to the environment, and transferred 42.0 million pounds off-site for further waste management.

One of TURA’s original goals was to reduce the generation of toxic byproducts (or waste) by 50 percent.  That goal was met in 1998 and was surpassed in 1999 and 2000 (see Figure 1).

Figure 1 - Core Group Byproduct Reduction From 1990 to 2000 – Production Adjusted

TURA’s byproduct reduction goal is measured using data normalized for changes in production that is reported by a Core Group of industries that have been subject to reporting since 1990 (this data excludes trade secret data).  In 2000, the Core Group comprised 340 facilities and used 664.4 million pounds, or 57% of the total toxic chemicals reported (i.e., 1.2 billion pounds excluding trade secret data).  Taking into account a 45% increase in production, from 1990 to 2000 the Core Group facilities reduced toxic byproducts by 58%, toxic chemical use by 40%, quantities shipped in product by 47%, on-site releases to the environment by 90%, and transfers off-site for further waste management by 36% (see Figure 2).

Figure 2 – Core Group Toxics Use Reduction Progress From

1990 to 2000 – Production Adjusted

40% 

Even when Core Group data is not adjusted for changes in production, between 1990 and 2000 Core Group filers still decreased their total use of reportable chemicals by 13% (from 761.8 million pounds in 1990 to 664.4 million pounds in 2000), reduced their byproduct generation by 40% (from 99.8 million pounds in 1990 to 66.4 million pounds in 2000), and reduced their on-site releases to the environment by 85% (from 20.5 million pounds in 1990 to 3.1 million pounds in 2000).  This represents significant progress since there has been decreasing toxic chemical use and waste despite substantial increases in production due to economic growth.

2000 was the first year TURA facilities began reporting on chemicals now classified as persistent bioaccumulative toxic (PBT) chemicals by the U.S. Environmental Protection Agency (EPA) under the Toxics Release Inventory (TRI) Program (see Table 1).  The use and release of PBTs and other highly hazardous substances has been receiving increasing attention in recent years.  PBT chemicals are of special concern because they are highly toxic and they remain in the environment for long periods of time, are not readily destroyed, and build up in the food chain.  Due to new lower reporting thresholds, two PBTs now appear in the top 20 chemicals used by TURA facilities (polycyclic aromatic compounds and benzo(g,h,i,)perylene, both of which are contained in fuel oils).  The TURA Program is developing a strategy to focus attention on specific PBTs and other highly hazardous chemicals.  This strategy will challenge Massachusetts facilities to pursue aggressive reductions in these areas.

| Table 1 ### 2000 PBT Summary (in pounds unless otherwise noted) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PBT Chemical/ Chemical Category | Reporting Threshold | # of Facilities that Reported | Total Use | Generated as Byproduct | Shipped in Product | TRI On-Site Releases | TRI Transfers Off-Site |
| Polycyclic Aromatic Compounds (PACs) | 100 lbs. | 140 | 109,492,315 | 7,000 | 31,036 | 59 | 6,676 |
| Benzo(g,h,i)- perylene | 10 lbs. | 105 | 9,618,907 | 70 | 1,227 | 5 | 268 |
| Mercury | 10 lbs. | 10 | 4,927 | 737 | 4,189 | 3 | 2,527 |
| Mercury Compounds | 10 lbs. | 6 | 90,009 | 46,901 | 42,802 | 294 | 97,702 |
| Poly-chlorinated Biphenyls (PCBs) | 10 lbs. | 2 | 118,160 | 118,116 | 44 | 0 | 118,116 |
| Tetrabromo-bisphenol A | 10 lbs. | 1 | 332 | 315 | 17 | 0 | 315 |
| Dioxin and Dioxin-like Compounds | 0.1 Gram | 8 | 12.05 Grams | 11.95 Grams | 0.00 Grams | 11.82 Grams | 0.20 Grams |

The TURA program has achieved its dramatic success through the efforts of Massachusetts industry working with state government to implement the goals of the TURA program. Massachusetts facilities have reduced significant amounts of waste by implementing toxics use reduction techniques, including input substitution, production unit modernization, production unit redesign, improved operation and maintenance, and recycling and reuse of chemicals in their production processes.  They have demonstrated that toxics use reduction not only reduces toxic chemical use and waste, but also saves businesses money over the long term.  In the future, Massachusetts facilities will be challenged to target reductions of PBT and other high hazard chemicals.

## I.    TURA Progress 1990-2000

Since 1990, when the TURA program began, Massachusetts TURA filers have made substantial progress in reducing their use of toxic substances and their generation of toxic byproducts.  In measuring this progress, a number of changes in the TURA reporting universe must be taken into account.

In 1990, only manufacturing firms were required to report to the TURA program.  Then, in accordance with TURA’s phase-in schedule, the reporting universe was expanded to include industries beyond the manufacturing sector.  The list of chemicals subject to reporting also was expanded in reporting years 1991, 1992, and 1993, further enlarging the universe of companies reporting.  In addition, over the years, certain chemicals have been delisted.  For example, effective reporting year 1999, the Administrative Council on Toxics Use Reduction delisted pure copper in solid or molten metal form.

Figure 3 illustrates TURA filing trends over the past eleven years.  Out of 1,422 chemicals listed under TURA, only 192 were reported in 2000.  This is an increase of five chemicals compared to 1999, due to the first year reporting of PBTs at lower thresholds.

The number of facilities reporting under TURA has generally declined over time, from a high of 728 facilities in 1991 and 1992, to 504 in 1999.  The number of reporting facilities increased to 559 in 2000 due in part to the new requirement to report PBTs at lower thresholds.  The number of individual Form Ss declined from a high of 2,661 in 1993, to 2,204 in 1999, and then increased to 2,427 in 2000, again due partly to the reporting of PBTs.

Figure 3 - TURA Filer Trends 1990 –2000

In 2000, 23 facilities left the TURA reporting universe and 79 entered the reporting universe, for a net increase of 56 facilities (see Figure 4).  Of the 79 facilities, 25 were new filers that entered the reporting universe for the first time (11 of these 25 new filers entered due to the first year reporting of PBTs).  Thirty-nine of the new filers previously reported under TURA, but did not report in 1999.  These facilities typically use TURA chemicals at amounts that are very close to the reporting threshold, and in some years they exceed the threshold and in other years they do not (25 of the 39 facilities came back into the reporting universe due to PBT reporting).  Fifteen new filers were businesses that should have previously reported and began reporting in 2000 due to DEP enforcement actions.

Ten facilities left the reporting universe in 2000 due to the delisting of copper in 1999.  Thirteen filers left the reporting universe due to the elimination of a chemical in the production process or the reduction of the use of the chemical below the reporting threshold.

Figure 4 – Reasons for Facilities Entering and Leaving

Reporting Universe in 2000

Core Group Progress

The overall progress of the TURA program is best reflected by toxics use reduction progress within the Core Group of TURA filers.  In order to allow for a consistent picture of TURA progress, a Core Group has been defined, consisting of industries and chemicals that were subject to reporting in 1990 and which remain subject to reporting in 2000.  The Core Group includes any facility whose Standard Industrial Classification (SIC) code is within the manufacturing SIC codes (20 to 39, inclusive), and all chemicals in the 1990 TURA reporting list that have not since been delisted.  The criteria for inclusion in the Core Group do not change. However, there are yearly changes in the Core Group due to chemical delistings and new filers.  The following rules apply to the Core Group data:

- If a chemical is delisted, it is removed from the Core Group for all reporting years.

- New filers are included in the Core Group if their SIC codes and chemicals meet the Core Group criteria.

- If a Core Group facility drops below the reporting threshold, its prior year records remain in the Core Group.

- The Core Group does not include chemicals for which a facility claimed trade secret protection in any year.

The Core Group included 340 (or 61%) of the total number of facilities reporting in 2000 (see Figure 5).  The Core Group used 664 million pounds (or 57%) of the total toxic chemicals reported in 2000 (see Figure 6).

Figure 5 – Number of Facilities: Core Group vs. All TURA Filers

Figure 6 – Amount of Total Use: Core Group vs. All TURA Filers

Core Group Progress – Without Adjusting for Production

Millions of Pounds

Millions of Pounds

Millions of Pounds

The changes in total reported Core Group quantities over the period 1990 to 2000 are shown in Figures 7 and 8.  These quantities have not been adjusted for changes in production.

Year

Year

Year

40% 

23% 

From 1990 to 2000, Core Group filers decreased their total chemical use by 13% (from 761.8 million pounds in 1990 to 664.4 million pounds in 2000), reduced their byproduct generation by 40% (from 99.8 million pounds in 1990 to 66.4 million pounds in 2000), and reduced the quantity of chemicals shipped in product by 23% (from 166.3 million pounds in 1990 to 127.8 million pounds in 2000).

Figure 7 – Core Group Quantities 1990 –2000 (not production adjusted)

00

00

13% 

00

Core Group filers also have been very successful in achieving reductions of on-site releases as defined by the federal Toxics Release Inventory (TRI) program.  These releases have been reduced by 85%, from 20.5 million pounds in 1990 to 3.1 million pounds in 2000.

Finally, Core Group filers reduced their TRI transfers off-site (byproducts that are transferred off-site for energy recovery, recycling, treatment or disposal) by 4%, from 20.4 million pounds in 1991 to 19.7 million pounds in 2000.

Millions of Pounds

Millions of Pounds

Figure 8 – Core Group Quantities 1990-2000 (not production adjusted)

85% 

4% 

Year

Year

00

00

### Core Group Progress - Production Adjusted Data

Between 1990 and 2000, Core Group filers reported a 45% increase in production.  In order to more accurately measure progress, the TURA data is adjusted or normalized to eliminate the effects of changes in production using production ratios reported by the Core Group facilities.

The following example illustrates how data is adjusted to reflect changes in production:

- In year 1, a facility produces 1,000 machine parts, and generates 100 lbs. of byproduct.

- In year 2, the facility produces 25% more machine parts (1,250). Therefore, the production ratio is 1.25.  However, the facility still generates 100 lbs. of byproduct.

- The production adjusted byproduct for year 2 is 100 lbs./1.25 = 80 lbs.

- The production adjusted percent change from year 1 to year 2 is [100-80]/100 = 0.20, or a 20% reduction, while its actual byproduct reduction is 0%.

When the Core Group data is adjusted to account for changes in production since 1990 (see Figures 9 and 10), Core Group filers have reduced their toxic chemical use by 40%, have generated 58% less byproduct, and have shipped 47% fewer chemicals in product.

Figure 9 – Core Group Quantities 1990-2000 (production adjusted)

Millions of Pounds

Millions of Pounds

Millions of Pounds

00

00

00

40% 

58% 

Year

Year

Year

47% 

Core Group filers also have reduced their TRI on-site releases by 90%, and have reduced their TRI transfers off-site by 36%.

Figure 10 - Core Group Quantities 1990-2000 (production adjusted)

90% 

36% 

00

00

Millions of Pounds

Millions of Pounds

Year

Year

Table 2 summarizes TURA data from 1990 to 2000, showing both reported and production adjusted quantities.  For each category, each year’s production adjusted quantity is normalized to the base year production level, thus providing a comparison of production-adjusted quantities to base year quantities.

## Table 2- Core Group Data: 1990 - 2000 Trend Summary *

(Does Not Include Trade Secret Quantities)

# Quantities are in Millions of Pounds

|  | Total Use |  | Byproduct |  | Shipped In Product |  | TRI On-site Releases |  | TRI Transfers Off-site |  | TRI Activity Index |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1990 | 761.8 | 761.8 | 99.8 | 99.8 | 166.3 | 166.3 | 20.5 | 20.5 |  |  |  |
| 1991 | 698.8 | 722.6 | 97.7 | 101.0 | 154.4 | 159.6 | 16.9 | 17.5 | 20.4 | 20.4 | 0.97 |
| 1992 | 642.5 | 674.9 | 94.2 | 99.0 | 141.2 | 148.3 | 14.6 | 15.3 | 23.4 | 23.7 | 0.99 |
| 1993 | 595.9 | 631.9 | 83.6 | 88.7 | 146.3 | 155.2 | 11.4 | 12.1 | 21.8 | 22.2 | 0.99 |
| 1994 | 680.5 | 621.5 | 83.5 | 76.2 | 164.8 | 150.5 | 9.7 | 8.9 | 25.7 | 22.6 | 1.16 |
| 1995 | 638.5 | 588.4 | 80.2 | 73.9 | 129.5 | 119.4 | 8.0 | 7.4 | 27.3 | 24.3 | 0.99 |
| 1996 | 618.2 | 540.4 | 75.1 | 65.6 | 110.0 | 96.1 | 6.4 | 5.6 | 26.8 | 22.6 | 1.05 |
| 1997 | 683.7 | 534.6 | 73.1 | 57.2 | 141.9 | 110.9 | 5.6 | 4.4 | 23.5 | 17.8 | 1.12 |
| 1998 | 651.2 | 473.9 | 71.4 | 51.9 | 123.7 | 90.0 | 4.8 | 3.5 | 24.7 | 17.4 | 1.07 |
| 1999 | 704.4 | 464.9 | 68.5 | 45.2 | 125.3 | 82.7 | 4.3 | 2.8 | 21.0 | 13.5 | 1.10 |
| 2000 | 664.4 | 457.3 | 60.0 | 41.3 | 127.8 | 88.0 | 3.1 | 2.1 | 19.7 | 13.1 | 0.96 |
| Percent Change 1990-2000 | 13% Reduction | 40% Reduction | 40% Reduction | 58% Reduction | 23% Reduction | 47% Reduction | 85% Reduction | 90% Reduction | 4% Reduction | 36% Reduction | 45% Increase |

*   Quantities in shaded boxes are normalized for changes in manufacturing activity (level of production) using the facility-reported TRI Production Ratio/Activity Index.

## 2000 TURA Chemical Data

In 2000, TURA filers manufactured, processed, or otherwise used 1.4 billion pounds of TURA listed chemicals.   TURA defines these terms as follows:

Manufacture – “to produce, prepare, import or compound a toxic or hazardous substance”  (e.g., intentional manufacture of a metal compound or the unintentional manufacture of acid gases during combustion of fossil fuels).

Process – “the preparation of a toxic or hazardous substance, including without limitation, a toxic substance contained in a mixture or trade name product, after its manufacture, for distribution in commerce” (e.g., in the formulation of paints or coatings, any listed toxics are “processed;” in the manufacture of polystyrene, the styrene monomer is “processed”).

Otherwise Use – “any use of a toxic substance that is not covered by the terms manufacture or process and includes use of a toxic substance contained in a mixture or trade name product” (e.g., chemicals used to clean parts, chemicals contained in fuels that are combusted).

Figure 11 summarizes the 2000 data for all TURA filers.  These companies reported using almost 1.4 billion pounds of chemicals and generating 128 million pounds of byproduct.

Figure 11 - How many Tractor-Trailer Trucks would you need to haul…

*Total Use = 1,369,000,000 lbs. = 17,112 Tractor-Trailer Trucks *Total Generated as Byproduct = 128,000,000 lbs. = 1,600 Tractor-Trailer Trucks *Total Shipped in Product = 424,000,000 lbs. = 5,300 Tractor-Trailer Trucks *TRI On-Site Releases = 11,000,000 lbs. = 138 Tractor-Trailer Trucks *TRI Transfers Off-Site = 42,000,000 lbs. = 525 Tractor-Trailer Trucks

*Calculated assuming a tractor-trailer truck carries 80,000 lbs.

Manufactured Chemicals

Figure 12 shows that relatively little manufacturing of TURA chemicals occurs in Massachusetts.  Chemicals reported as “manufactured” accounted for 7% of the total use statewide (or 78 million pounds).  A significant amount of the chemicals reported as manufactured are not manufactured intentionally, but are coincidentally manufactured as a result of some other activity.   Examples include the creation of acid gases from fuel combustion for power generation and the production of nitrate compounds as a result of wastewater treatment.

Processed Chemicals

In Massachusetts, the predominant chemical use is “processing,” which includes incorporating a listed chemical into a product.  Processing of chemicals accounted for 67% of total use (or 790 million pounds).  Styrene, which is used in the production of plastics, accounted for 44% (or 349 million pounds) of total chemicals processed.

Otherwise Used Chemicals

### Chemicals “otherwise used” accounted for 26% of total use (or 304 million pounds).   Chemicals otherwise used include activities such as parts cleaning, waste treatment, and the combustion of fuel oil containing listed chemicals such as polycyclic aromatic compounds and benzo(g,h,i)perylene.

Figure 12 – 2000 Chemical Use (does not include trade secret data) *

Total 2000 Use = 1.2 billion pounds

*    In this Report, when total use is broken down by type of use (i.e., manufactured, processed, or otherwise used), trade secret data is not included.  Thus, the total use in figure 12 is 1.2 billion pounds, rather than 1.4 billion pounds (which includes trade secret data).

### Top 20 Chemicals

In 2000, 192 chemicals were reported out of 1,422 TURA-listed chemicals.  Of the 192, 20 chemicals accounted for 70% of the total use reported statewide, or 953 million pounds (not including trade secret information) (see Table 3).  Styrene monomer was the  chemical with the largest quantity reported in 2000, accounting for 25% of total use reported (or 349 million pounds).   Styrene monomer is the building block for various plastics.

Since the reporting thresholds for persistent, bioaccumulative, toxic (PBT) chemicals were lowered effective reporting year 2000, polycyclic aromatic compounds (PACs) was the chemical category with the second largest quantity of chemical use in 2000, accounting for 8% of total reported use, and benzo(g,h,i)perylene also appears in the top 20 chemicals used.  PACs and benzo(g,h,i)perylene are otherwise used during the combustion of fossil fuels.  Of the 559 facilities that reported, 140 (or 25%) reported PACs and 105 (or 19%) reported benzo(g,h,i)perylene.  Sodium hydroxide was the third highest used chemical with 218 facilities (or 39%) reporting its use, and it had the highest byproduct amount reported statewide.

| Table 3 - 2000 Top 20 Chemicals |  |
| --- | --- |
| Total Use These quantities do not include Trade Secret |  |
| Chemical Name (CAS #) | Total Use (Lbs.) |
| Styrene Monomer (100425) | 348,875,499 |
| Polycyclic Aromatic Compounds | 109,492,315 |
| Sodium Hydroxide (1310732) | 93,456,213 |
| Methanol (67561) | 56,860,236 |
| Hydrochloric Acid (7647010) | 50,737,340 |
| Sulfuric Acid (7664939) | 46,317,517 |
| Toluene (108883) | 43,316,093 |
| Copper (7440508) | 40,190,080 |
| Potassium Hydroxide (1310583) | 18,488,239 |
| Sodium Hypochlorite (7681529) | 17,962,633 |
| Methyl Methacrylate ( 80626) | 17,114,440 |
| Zinc Compounds | 15,785,968 |
| Methyl Ethyl Ketone (78933) | 15,473,817 |
| Phthalic Anhydride (85449) | 13,296,744 |
| Ammonia (7664417) | 12,959,806 |
| Ethyl Acetate (141786) | 12,373,538 |
| Nitrate Compounds | 10,698,924 |
| Phosphoric Acid (7664382) | 10,499,001 |
| Lead Compounds | 9,807,994 |
| Benzo(g,h,i)perylene (191242) | 9,618,907 |
| The following four chemicals would appear in the top 20 chemicals total use list (above) if trade secret quantities were included: Butyraldehyde, Ethylene Glycol, Formaldehyde, Vinyl Acetate. |  |

Table 4 shows the top 20 chemicals generated as byproduct in 2000, which accounted for 84% of the total byproduct generated statewide (or 107 million pounds).

Table 4 also shows the top 20 chemicals shipped in product in 2000, which accounted for 78% of the total shipped in product (or 288 million pounds).

| Table 4 - 2000 Top 20 Chemicals |  |  |  |
| --- | --- | --- | --- |
| Byproduct Generation These quantities include Trade Secret |  | Shipped in Product These quantities do not include Trade Secret |  |
| Chemical Name (CAS #) | Byproduct Generation (Lbs.) | Chemical Name (CAS #) | Chemical Name (CAS #) |
| Sodium Hydroxide (1310732) | 15,036,923 | Methanol ( 67561) | 49,138,652 |
| Toluene (108883) | 14,310,109 | Sodium Hydroxide (1310732) | 43,180,403 |
| Sulfuric Acid (7664939) | 11,988,739 | Copper (7440508) | 39,335,626 |
| Ethyl Acetate (141786) | 11,603,660 | Toluene (108883) | 28,844,547 |
| Methanol (67561) | 7,096,431 | Potassium Hydroxide (1310583) | 14,698,463 |
| Hydrochloric Acid (7647010) | 7,024,367 | Sodium Hypochlorite (7681529) | 12,258,311 |
| Nitrate Compounds | 6,896,220 | Sulfuric Acid (7664939) | 9,851,395 |
| Methyl Ethyl Ketone (78933) | 5,923,889 | Zinc Compounds | 9,217,739 |
| Acetone (67641) | 5,184,504 | Methyl Ethyl Ketone (78933) | 8,934,394 |
| Ammonia (7664417) | 2,994,762 | Chromium (7440473) | 8,697,199 |
| Formaldehyde (50000) | 2,913,268 | Lead Compounds | 8,095,172 |
| Nitric Acid (7697372) | 2,864,013 | Antimony Compounds | 7,653,546 |
| Copper Compounds | 2,474,790 | Ethylene Glycol (107211) | 7,357,560 |
| Potassium Hydroxide (1310583) | 2,161,968 | Nickel (7440020) | 6,776,273 |
| Phosphoric Acid (7664382) | 1,627,557 | Glycol Ethers (1022) | 6,706,279 |
| Dichloromethane (75092) | 1,581,198 | Copper Compounds | 5,821,364 |
| Ethylene Glycol (107211) | 1,532,504 | Phosphoric Acid (7664382) | 5,513,283 |
| Sodium Hypochlorite (7681529) | 1,331,465 | Formaldehyde (50000) | 5,412,324 |
| Dimethylformamide (68122) | 1,331,103 | Ammonia (7664417) | 5,405,254 |
| Acetic Acid (64197) | 1,242,591 | 1-Methyl-2-Pyrrolidone (872504) | 4,918,458 |
|  |  | The following chemicals would appear in the top 20 chemical shipped in product list (above) if trade secret quantities were included: Acetone and Ethyl Acetate. |  |

Table 5 shows the top 20 chemicals reported as TRI on-site releases in 2000, which totaled 93% of the total TRI on-site releases (or 10 million pounds).  Hydrochloric acid had the highest amount of TRI on-site releases reported statewide, accounting for 41% of total TRI on-site releases.  Almost 4 million pounds of hydrochloric acid or 76% of total on-site releases of hydrochloric acid was attributed to power plants.

Table 5 also shows the top 20 chemicals reported as TRI transfers off-site in 2000, which totaled 78%, or 33 million pounds of the total TRI transfers off-site.  Nitrate compounds had the highest TRI transfers off-site reported statewide, accounting for 13% of total TRI transfers off-site.  Nitrate compounds were primarily coincidentally manufactured during neutralization of nitric acid in wastewater treatment.

| Table 5 - 2000 Top 20 Chemicals |  |  |  |
| --- | --- | --- | --- |
| TRI On-Site Releases These quantities include Trade Secret |  | TRI Transfers Off-Site These quantities include Trade Secret |  |
| Chemical Name (CAS #) | On-Site Releases (Lbs.) | Chemical Name (CAS #) | Transfers Off-Site (Lbs.) |
| Hydrochloric Acid (7647010) | 4,437,465 | Nitrate Compounds | 5,633,163 |
| Ammonia (7664417) | 763,555 | Toluene (108883) | 4,125,534 |
| Toluene (108883) | 716,578 | Methanol (67561) | 3,411,699 |
| Acetone (67641) | 592,156 | Formaldehyde (50000) | 2,725,575 |
| Sulfuric Acid (7664939) | 532,353 | Copper Compounds | 2,569,791 |
| Ethyl Acetate (141786) | 415,873 | Ethyl Acetate (141786) | 1,705,885 |
| Hydrogen Fluoride (7664393) | 339,326 | Acetone (67641) | 1,579,211 |
| Methanol (67561) | 326,216 | Dichloromethane (75092) | 1,433,446 |
| Methyl Ethyl Ketone (78933) | 308,577 | Methyl Ethyl Ketone (78933) | 1,394,500 |
| Glycol Ethers (1022) | 267,643 | Sodium Hydroxide (1310732) | 1,300,182 |
| Butyl Alcohol (71363) | 263,148 | Zinc Compounds | 1,115,432 |
| Trichloroethylene (79016) | 242,256 | Copper (7440508) | 851,753 |
| Vanadium Compounds | 225,918 | Butyl Alcohol (71363) | 787,678 |
| Methyl Isobutyl Ketone (108101) | 125,785 | 1-Methyl-2-Pyrrolidone (872504) | 705,947 |
| Nickel Compounds | 108,894 | Ammonia (7664417) | 700,930 |
| Xylene Mixed Isomer (1330207) | 103,561 | Nickel Compounds | 658,293 |
| Formaldehyde (50000) | 96,488 | Ethylene Glycol (107211) | 635,706 |
| Acetic Acid (64197) | 87,654 | Sodium Fluoride (7681494) | 621,765 |
| Hexane (N-Hexane) (110543) | 78,042 | Lead Compounds | 580,563 |
| Dimethylformamide (68122) | 68,643 | Aluminum Sulfate (10043013) | 505,842 |

Persistent, Bioaccumulative, Toxic (PBT) Chemicals

On October 29, 1999, the U.S. Environmental Protection Agency (EPA) published a final rule under section 313 of the Emergency Planning and Community Right-to-Know Act (EPCRA) which lowered the Toxics Release Inventory reporting thresholds for persistent bioaccumulative toxic (PBT) chemicals, and added certain other PBT chemicals to the EPCRA Section 313 Chemical list.  TURA automatically adopts any lower reporting thresholds that are promulgated by EPA.  Toxics use reports for 2000 were the first reports to include PBT chemicals at the new lower reporting thresholds.

PBT chemicals are of particular concern because they are highly toxic and they remain in the environment for long periods of time, are not readily destroyed, and build up or accumulate in body tissue.  Relatively small releases of PBT chemicals can pose human and environmental health threats, and therefore the use and release of these chemicals, even in relatively small amounts, warrant public reporting, and actions should be taken to reduce their use and release.

For 2000, Massachusetts facilities reported the use of seven PBT chemicals/chemical categories (see Table 6).  It should be noted that TURA data is collected only from facilities within certain industrial sectors that have 10 or more full-time employees, and therefore it does not provide a complete picture of the use and emissions of chemicals, whether PBT or non-PBT chemicals.  For instance, TURA data does not include emissions from cars and trucks, nor from the majority of sources of releases of pesticides, volatile organic compounds, fertilizers, and many other non-industrial sources.  It also does not capture the use of toxic chemicals in consumer products that are not manufactured in Massachusetts.

| Table 6 ### 2000 PBT Summary (in pounds unless otherwise noted) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PBT Chemical/ Chemical Category | Reporting Threshold | # of Facilities that Reported | Total Use | Generated as Byproduct | Shipped in Product | TRI On-Site Releases | TRI Transfers Off-Site |
| Polycyclic Aromatic Compounds (PACs) | 100 lbs. | 140 | 109,492,315 | 7,000 | 31,036 | 59 | 6,676 |
| Benzo(g,h,i)- perylene | 10 lbs. | 105 | 9,618,907 | 70 | 1,227 | 5 | 268 |
| Mercury | 10 lbs. | 10 | 4,927 | 737 | 4,189 | 3 | 2,527 |
| Mercury Compounds | 10 lbs. | 6 | 90,009 | 46,901 | 42,802 | 294 | 97,702 |
| Poly-chlorinated Biphenyls (PCBs) | 10 lbs. | 2 | 118,160 | 118,116 | 44 | 0 | 118,116 |
| Tetrabromo-bisphenol A | 10 lbs. | 1 | 332 | 315 | 17 | 0 | 315 |
| Dioxin and Dioxin-like Compounds | 0.1 Gram | 8 | 12.05 Grams | 11.95 Grams | 0.00 Grams | 11.82 Grams | 0.20 Grams |

# The use and release of PBTs and other highly hazardous substances has been receiving increasing attention in recent years.  The TURA Program is beginning to focus more on PBTs and other highly hazardous chemicals.  To be effective, any overall PBT strategy developed will need to consider PBTs beyond TURA facilities.

# Polycyclic Aromatic Compounds (PACs) and Benzo(g,h,i)perylene

In 2000, polycyclic aromatic compounds (PACs) and benzo(g,h,i)perylene were the largest PBT chemical category and chemical used.  140 facilities reported the otherwise use of PACs.  Of these, 105 facilities also reported the use of benzo(g,h,i)perylene.  The primary activity that triggered reporting of these chemicals was combustion of #6 and #4 fuel oils, and to a lesser extent #2 fuel oil.  These fuel oils contain PACs and benzo(g,h,i)perylene (i.e., they are already in the fuel that enters a facility’s boiler to be combusted to generate heat or steam).  Benzo(g,h,i)perylene is an especially toxic polycyclic aromatic compound, and therefore is reported separately from the PACs category and has a lower reporting threshold of 10 pounds (versus a 100 pound reporting threshold for the PACs category).  Table 7 shows a breakdown of PACs use and Table 8 shows a breakdown of benzo(g,h,i)perylene use.

| Table 7 ### 2000 PACs Summary ### (in pounds) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Activity / Facility Type | Number of Facilities | ##### Total Use | ## Byproduct | Shipped in Product | TRI On-site Releases | TRI Transfers Off-site |
| Fuel Combustion | 137 | 109,455,421 | 1240 | 59 | 14 | 751 |
| Power plants | (12) | (107,004,209) | (398) | (0) | (11) | (390) |
| Other facilities | (125) | (2,451,212) | (842) | (59) | (3) | (361) |
| Waste Oil Processing | 1 | 29,820 | 5,670 | 24,150 | 5 | 5,925 |
| Grinding Wheel Manufacturer | 1 | 6,571 | 0 | 6,457 | 0 | 0 |
| Asphalt Shingle Manufacturer | 1 | 503 | 90 | 370 | 40 | 0 |
| Total | 140 | 109,492,315 | 7,000 | 31,036 | 59 | 6,676 |

| Table 8 ### 2000 Benzo(g,h,i)perylene Summary ### (in pounds) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Activity / Facility Type | Number of Facilities | Total Use | ## Byproduct | Shipped in Product | TRI On-site Releases | TRI Transfers Off-site |
| Fuel Combustion | 1036 | 9,617,613 | 9 | 11 | 0 | 8 |
| Power plants | (9) | (9,596,454) | (4) | (0) | (0) | (4) |
| Other facilities | (94) | (21,159) | (5) | (11) | (0) | (4) |
| Waste Oil Processing | 1 | 319 | 61 | 258 | 5 | 260 |
| Grinding Wheel Manufacturer | 1 | 975 | 0 | 958 | 0 | 0 |
| Total | 105 | 9,618,907 | 70 | 1,227 | 5 | 268 |

The 12 power plants that reported PACs (9 of which also reported benzo(g,h,i)perylene) accounted for 98% of total PACs use and over 99% of benzo(g,h,i)perylene use (107,004,209 pounds and 9,596,454 pounds, respectively).  The other facilities that reported due to fuel combustion accounted for 2% of PACs and 1% of benzo(g,h,i)perylene use (2,451,212 pounds and 21,159 pounds, respectively).  The majority of facilities, including the power plants, reported zero byproduct generation, on-site releases, and transfers off-site for these chemicals.  This is because most PACs and benzo(g,h,i)perylene are destroyed in the combustion process.  Some PACs and benzo(g,h,i)perylene are generated as byproduct, but in such small amounts that the amounts round to zero for purposes of reporting.

A handful of facilities burning fuel oil reported PACs and benzo(g,h,i)perylene as byproduct, but this was typically due to one-time events such as boiler cleanouts.  A waste oil processor reported 24,150 pounds of PACs and 258 pounds of benzo(g,h,i)perylene shipped in product as a result of the collection and transfer of waste oil.  The byproduct reported by this processor is comprised largely of sludge removed from the oil.  An asphalt shingle manufacturer reported PACs as a  trace contaminant in asphalt.  A grinding wheel manufacturer reported PACs and benzo(g,h,i)perylene due to its use of creosote as a part of its product formulation.

Mercury and Mercury Compounds

Ten facilities reported the use of mercury, and six facilities reported the use of mercury compounds.  Table 9 shows a breakdown of mercury use by activity.  82% of total mercury use was due to the recycling of fluorescent lamps by a single facility.

| Table 9 2000 Mercury Summary (in pounds) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Activity / Facility Type | Number of Facilities | Total Use | ## Byproduct | Shipped in Product | TRI On-site Releases | TRI Transfers Off-site |
| Lamp / ballast recycler | 1 | 4,055 | 6 | 4,049 | 2 | 4 |
| Manufacturer: incorporated mercury into products | 3 | 473 | 386 | 87 | 0 | 386 |
| Manufacturer: used mercury as sealant for reactor vessels | 1 | 265 | 265 | 0 | 0 | 2 |
| Sand and gravel companies: mercury occurs naturally in Portland cement, and is also in coal combustion fly ash that is mixed with concrete. | 2 | 55 | 2 | 53 | 0 | 0 |
| Power plants: mercury coincidentally generated via combustion and off-site shipment of mercury-containing instruments / light bulbs | 1 | 42 | 41 | 0 | 1 | 40 |
| Hazardous Waste Treatment, Storage, Disposal Facility: receiving mercury-containing waste for recycling/ treatment | 1 | 23 | 23 | 0 | 0 | 2,081 |
| Manufacturer: used mercury in analytical lab | 1 | 14 | 14 | 0 | 0 | 14 |
| Total | 10 | 4,927 | 737 | 4,189 | 3 | 2,527 |

Table 10 shows a breakdown of mercury compounds use.  99% of reported mercury compounds use was due to the handling of mercury-containing wastes by a hazardous waste treatment, storage and disposal facility.  The remaining use was due to the coincidental generation of mercury compounds due to fuel combustion at power plants.

| Table 10 2000 Mercury Compounds Summary (in pounds) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Activity / Facility Type | Number of Facilities | Total Use | ## Byproduct | Shipped in Product | TRI On-site Releases | TRI Transfers Off-site |
| Hazardous Waste Treatment, Storage and Disposal Facility: receiving mercury–containing waste for recycling /treatment | 1 | 89,256 | 46,485 | 42,771 | 0 | 97,593 |
| Power plants: mercury coincidentally generated via combustion | 5 | 753 | 416 | 31 | 294 | 109 |
| Total | 6 | 90,009 | 46,901 | 42,802 | 294 | 97,702 |

Polychlorinated Biphenyls (PCBs)

For 2000, 2 facilities reported the use of polychlorinated biphenyls (PCBs).  Table 11 shows the breakdown of PCBs.  Over 99% of total use of PCBs was attributed to one facility that recycled fluorescent light fixture ballasts.  This facility also accounted for 100% of byproduct, 57% of shipped in product, and 100% of TRI transfers off-site of PCBs. The other facility reported the coincidental generation of PCBs in the manufacture of organic pigments, in accordance with EPA’s guidance document on PCBs.

| Table 11 ### 2000 PCBs Summary ### (in pounds) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Activity / Facility Type | Number of Facilities | Total Use | ## Byproduct | Shipped in Product | TRI On-site Releases | TRI Transfers Off-site |
| Recycler: Lamp / ballast recycling | 1 | 118,141 | 118,116 | 25 | 0 | 118,116 |
| Manufacturer: coincidentally generates PCBs in manufacture of organic pigments | 1 | 19 | 0 | 19 | 0 | 0 |
| Total | 2 | 118,160 | 118,116 | 44 | 0 | 118,116 |

Tetrabromobisphenol A

#### One facility reported the use of tetrabromobisphenol A.  This facility incorporated tetrabromobisphenol A into their product, which was an epoxy flame-retardant molding compound.  The facility’s total use was 332 pounds, their byproduct was 315 pounds, their shipped in product was 17 pounds, and their transfers off-site were 315 pounds.

#### Dioxin and Dioxin-like Compounds

Dioxins are byproducts of chemical and combustion processes, often involving chlorine.  Dioxins have poor solubility in water and thus accumulate in body fat and concentrate in the food chain.  Because dioxin is considered extremely toxic, EPA established a very low reporting threshold of 0.1 gram.

For 2000, 8 facilities reported the use of dioxin and dioxin-like compounds, seven due to the coincidental manufacture of dioxin from combustion (97% of total use) and one due to the bleaching of paper.  Table 12 shows the breakdown of dioxin and dioxin-like compounds.

| Table 12 ### 2000 Dioxin and Dioxin-like Compounds Summary (in grams) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Activity / Facility Type | Number of Facilities | Total Use | ## Byproduct | Shipped in Product | TRI On-site Releases | TRI Transfers Off-site |
| Power plants: dioxin coincidentally generated via combustion | 7 | 11.72 | 11.62 | 0 | 11.73 | 0 |
| Pulp and Paper Manufacturer: dioxin coincidentally generated via paper bleaching | 1 | 0.33 | 0.33 | 0 | 0 .09 | 0.20 |
| Total | 8 | 12.05 | 11.95 | 0 | 11.82 | 0.20 |

- 2000 Significant Industrial Sectors

Under TURA, facilities in the Manufacturing Standard Industrial Classification (SIC) codes (20-39 inclusive) and those in SIC codes 10-14, 40, 44-51, 72, 73, 75 and 76, must report their chemical use if it exceeds certain thresholds.

Figure 13 shows the number of TURA reporting facilities in each industry sector.  The Chemical and Allied Products sector represents approximately 18% (101 facilities) of the number of TURA reporting facilities, and uses approximately 60% of the reportable TURA chemicals (see Figure 14).  This sector is a diverse group of industries, and includes companies that manufacture or formulate adhesives, paints, pharmaceuticals, and plastic materials and resins.  Approximately 42% of the total chemical use for this sector was attributable to the use of styrene monomer, which is used in the manufacture of polystyrene and other plastics.  There was a 39 million pound decrease in styrene chemical use for this sector from 1999 to 2000.  One facility accounted for a 46.7 million pound decrease in the use of styrene, and two facilities accounted for a 7.8 million pound increase in the use of styrene.

Figure 13 - Number of Facilities By Industrial Sector

Figure 14 – 2000 Chemical Use By Industrial Sector

The Electric, Gas and Sanitary Services sector was the second largest chemical user, accounting for 11% of total statewide use.  The 34 firms reporting in this sector are primarily involved in the production of electricity.  In 2000, polycyclic aromatic compounds (PACs) accounted for 74% of total chemical use for this sector and benzo(g,h,i)perylene accounted for 7% of total chemical use for this sector because of the lower thresholds attributed to PBTs.  In 1999, approximately 56% of total chemical use for this sector was attributable to the use of PACs.

The Wholesale Nondurable Goods sector accounted for 10% of chemical use. The activities of this sector involve repackaging of chemicals for sale to other sectors.  Chemical use by facilities in this sector is very efficient with byproduct generated at 0.2% of use.  This translates into a chemical use efficiency of 99.8%.

The Rubber and Plastics sector accounted for 6% of chemical use. The Primary Metals sector accounted for 3% of chemical use.  The Paper and Allied Products and Fabricated Metal sectors accounted for 2% each of chemical use, leaving the balance of statewide use (6%) to a variety of sectors.

Figure 15 shows byproduct generation by industrial sector.  While the Chemical and Allied Products sector accounted for 60% of total statewide use, this sector produced 25% of the total byproduct generated in 2000.  In contrast, the Paper and Allied Products sector, which accounted for 3% of total statewide chemical use, accounted for 14% of the byproduct generated.

The Electric, Gas and Sanitary Services sector accounted for 10% of total byproduct generated.  The Rubber and Plastics sector accounted for 9%. Other major industries that generated byproduct include the Fabricated Metals sector (9%), Textile Mill Products sector (8%), and the Instruments and Related Products sector (6%).  The remaining 19% of byproduct was attributed to a variety of sectors.

Figure 15 – 2000 Byproduct Generation By Industrial Sector

Figure 16 shows TRI on-site releases to the environment.  The Electric, Gas and Sanitary Services sector, which represented 11% of total statewide use, was the largest source of on-site releases, accounting for 56% of all on-site releases.  This sector provides power for Massachusetts businesses and citizens.   Seventy-two percent of on-site releases in this sector were attributed to the coincidental manufacture of hydrochloric acid during combustion.  The Paper and Allied Products sector accounted for 10% of total on-site releases.  The Fabricated Metals sector accounted for 7% of total on-site releases.  The Chemicals and Allied Products sector accounted for 60% of total chemical use and only 7% of total on-site releases.  The Rubber and Plastics sector and the Textile Mill Products sectors, each were responsible for 4% of total on-site releases.

Figure 16 – 2000 TRI On-Site Releases By Industrial Sector

Figure 17 shows TRI transfers off-site by industrial sector.  Chemical and Allied Products accounted for 40%, and Instruments accounted for 10% of transfers off-site.  The third largest sectors in the category, the Fabricated Metals and Primary Metals sectors, each were responsible for 8% of transfers off-site.  The Electronic and Electrical Equipment sector accounted for 7% of total transfers off-site.  The Paper and Allied Products and the Transportation Equipment sectors each were responsible for 6% of total transfers off-site.

Figure 17 – 2000 TRI Transfers Off-Site By Industrial Sector

IV.  2000 Major TURA Facilities

Top 5 Facilities with the Largest Reduction in Byproduct Generation from 1999 to 2000 While Implementing Toxics Use Reduction

Table 13 lists the 5 facilities that showed the largest byproduct reductions from 1999 to 2000 due to toxics use reduction.

Reductions in byproduct generation at these five facilities are best attributed to toxics use reduction.  While individual changes in production varied, these facilities either reported increased production or reported substantially more byproduct reduction than could be attributed to reduced manufacturing activity.  Overall, this group of facilities showed an increase in production levels over 1999.

| # Table 13 # 2000 Top 5 Facilities with the Largest Reduction in Byproduct Generation While Implementing Toxics Use Reduction (1999-2000) |  |  |
| --- | --- | --- |
| ## Company | Reduction in Byproduct (Lbs.) | Toxics Use Reduction Techniques Used |
| - Rexam Image Products (South Hadley) | 3,629,579 (43% decrease) | Product reformulation; improved operation and maintenance; production unit modernization |
| - Crane & Company, Inc., Pioneer Mill (Dalton) | 1,353,509 (31% decrease) | Input substitution; improved operation and maintenance |
| 3. Allegro MicroSystems (Worcester) | 205,063 (26% decrease) | Recycling, reuse, or extended use of toxics for processing operations. |
| 4. MASSPOWER (Indian Orchard) | 201,680 (25% decrease) | Input substitution |
| 5. Saint Gobain Performance Plastics (Worcester) | 171,799 (34% decrease) | Input substitution; production unit modernization; recycling of solvents; improved operation and maintenance |

Rexam Image Products

Rexam Image Products is a global leader in the manufacture of specialty coated paper, film, specialty substrates used in color digital imaging, electronic imaging, component manufacturing, and medical electronic and wound dressing manufacturing technologies.

In the past, Rexam used a solvent coating manufacturing process for production of coated paper, film, and other image media.  Beginning in the late 1980’s and continuing today, Rexam has focused on the development and commercialization of products manufactured by alternative coating technologies, including water borne coatings and ultra violet light cured coatings (100% solids).  Rexam achieved a 43% reduction in byproduct generation  (or 3.6 million pounds) between 1999 and 2000.

Crane & Company

Crane & Company was founded in 1801, and is the oldest continuously run paper manufacturer in North America.  The company’s products include 100% cotton social stationery, commercial printing papers, reprographic papers, synthetic fiber non-wovens, as well as currency and security papers.

Crane & Company’s byproduct reduction was attributed to reduction in use of sulfuric acid and sodium hypochlorite.  The company switched to a different chemistry in the way that it processed off-specification paper.  Sulfuric acid was replaced with liquified carbon dioxide.  Sodium hypochlorite use per ton of pulp processed was reduced by specifying cleaner raw materials, and by optimizing the bleaching chemistry through controlling the temperature and pH of the slurry where the bleach was added.  Crane & Company achieved a 31% reduction in byproduct generation (or 1.3 million pounds) between 1999 and 2000.

##### Allegro MicroSystems, Inc.

Allegro MicroSystems, Inc. is a leading supplier of advanced mixed-signal power semiconductors and Hall-effect sensors targeted toward the computer/office, automotive, industrial, telecommunication and consumer markets.

Allegro MicroSystems’ byproduct reduction was due to the reuse of sulfuric acid and sodium hydroxide for wastewater treatment, after the chemicals had been used for the regeneration of ion exchange columns.  Ion exchange columns are used in the production of ultra high purity water for the production process.  The waste regenerant is pumped directly into tanks for storage, and withdrawn as needed for wastewater treatment.  Allegro Microsystems achieved a 26% reduction in byproduct generation (or 205,000 pounds) between 1999 and 2000.

MASSPOWER

MASSPOWER is a 240-megawatt natural gas fired combined-cycle cogeneration facility.  The facility provides electricity to the New England power grid and also provides process steam to a nearby industrial facility.

MASSPOWER uses sulfuric acid and sodium hydroxide in the production of demineralized water.  The facility changed the water source used in the demineralizer system.  The new water source (city water) reduced the mineral loading to the system, thereby increasing production throughput between resin regeneration.  Increased throughput resulted in a net reduction of pounds of sulfuric acid and sodium hydroxide per gallon of demineralized water produced.  MASSPOWER achieved a 25% reduction  in byproduct generation (or almost 202,000 pounds) between 1999 and 2000.

Saint Gobain Performance Plastics

Saint Gobain Performance Plastics of Worcester, Massachusetts, engages in the application of engineered coatings to rolls of plastic films.  These rolls of coatings are incorporated into a variety of customer products including labels, printing materials, computers, and military decoys.

Saint Gobain was able to significantly reduce their byproduct by first establishing a computerized data collection system.  By using the data collection system, the facility was able to identify and map out toxics use reduction objectives.  Significant byproduct reduction was achieved by implementing ultra violet cured versus chemically cured coatings.  Saint Gobain Performance Plastics achieved a 34% reduction in byproduct generation (or almost 172,000 pounds) between 1999 and 2000.

### Top 20 Facility Lists

Table 14 lists the 20 facilities that used the largest quantity of chemicals.  These 20 facilities used 1 billion pounds, or 74% of total statewide use.

| Table 14 – 2000 Top 20 Facilities (Largest Quantity of Total Use) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Total Use These quantities include ###### Trade Secret |  |  |  |  |  |  |
| Facility Name | Town | Total Use (Lbs.) |  |  |  |  |
| Nova Chemicals Inc | Springfield | 269,207,282 |  |  |  |  |
| Solutia Inc - Indian Orchard Plant | Springfield | 151,640,214 |  |  |  |  |
| American Polymers | Oxford | 84,347,726 |  |  |  |  |
| Borden & Remington | Fall River | 71,549,434 |  |  |  |  |
| USGEN New England Inc | Somerset | 66,459,512 |  |  |  |  |
| Eastman Gelatine Corporation | Peabody | 47,862,673 |  |  |  |  |
| USGEN New England Inc | Salem | 46,973,690 |  |  |  |  |
| Elite Consumer Products | Ludlow | 43,370,742 |  |  |  |  |
| General Cable | Taunton | 39,330,214 |  |  |  |  |
| Holland Company Inc | Adams | 36,002,400 |  |  |  |  |
| Astro Chemicals Inc | Springfield | 27,926,225 |  |  |  |  |
| Allegheny Rodney Strip Division | New Bedford | 20,734,161 |  |  |  |  |
| Fox Packaging Co | Ayer | 17,568,175 |  |  |  |  |
| Tacc International | Rockland | 15,177,017 |  |  |  |  |
| Teknor Apex Co | Attleboro | 14,474,321 |  |  |  |  |
| Houghton Chemical Corp | Boston | 13,352,943 |  |  |  |  |
| Omnova Solutions Inc | Fitchburg | 12,060,465 |  |  |  |  |
| Ashland Distribution Co | Tewksbury | 11,489,089 |  |  |  |  |
| Monson Companies Inc | Leominster | 11,165,012 |  |  |  |  |
| Bostik Findley Inc | Middleton | 10,378,910 |  |  |  |  |

Table 15 lists the 20 facilities that shipped the largest quantity of chemicals in product and the 20 facilities that generated the largest quantity of byproduct in 2000. The 20 facilities with the largest quantity shipped in product shipped 335 million pounds in product, or 79% of total shipped in product statewide. The 20 facilities that generated the largest quantity of byproduct generated 70 million pounds of byproduct, or 55% of total statewide byproduct.

| Table 15 - 2000 Top 20 Facilities (Largest Quantity of Shipped in Product and Byproduct Generation) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Byproduct Generation These quantities include ###### Trade Secret |  |  | Shipped in Product These quantities include ###### Trade Secret |  |  |
| Facility Name | Town | Byproduct Generation (Lbs.) | Facility Name | Town | Shipped in Product (Lbs.) |
| Solutia Inc - Indian Orchard Plant | Springfield | 9,575,755 | Borden & Remington | Fall River | 71,522,855 |
| Flexcon Co Inc | Spencer | 7,330,328 | General Cable | Taunton | 38,904,631 |
| Eastman Gelatine Corporation | Peabody | 6,461,019 | Solutia Inc - Indian Orchard Plant | Springfield | 38,211,810 |
| Rexam Image Products | S. Hadley | 4,844,517 | Astro Chemicals Inc | Springfield | 26,533,997 |
| Polaroid Corporation | Waltham | 4,496,063 | Elite Consumer Products | Ludlow | 26,186,823 |
| Bostik Findley Inc | Middleton | 3,972,140 | Fox Packaging Co | Ayer | 17,565,188 |
| Allegheny Rodney Strip Division | New Bedford | 3,575,565 | Tacc International | Rockland | 15,135,960 |
| Crane & Co Inc Pioneer Mill | Dalton | 2,970,204 | Allegheny Rodney Strip Division | New Bedford | 14,326,779 |
| Chemdesign Corp | Fitchburg | 2,831,045 | Houghton Chemical Corp | Boston | 13,345,079 |
| Venture Tape | Rockland | 2,779,812 | Ashland Distribution Co | Tewksbury | 11,489,089 |
| USGEN New England Inc | Salem | 2,675,167 | Monson Companies Inc | Leominster | 11,154,517 |
| ISP Freetown Fine Chemicals Inc | Assonet | 2,580,079 | Shipley Co Inc | Marlborough | 8,663,990 |
| Mirant New England Inc | Sandwich | 2,376,113 | Callahan Company | Walpole | 7,186,000 |
| Engineered Materials Solutions Inc | Attleboro | 2,277,239 | Vopak Usa Inc | Salem | 6,967,217 |
| Ideal Tape Company | Lowell | 2,233,834 | Engineered Materials Solutions Inc | Attleboro | 5,378,196 |
| Madico Inc | Woburn | 2,125,530 | Webco Chemical Corp | Dudley | 5,095,116 |
| BBA Nonwovens Griswoldville Plant | Colrain | 1,958,973 | Bostik Findley Inc | Middleton | 4,786,807 |
| Sanmina Corp | Wilmington | 1,834,691 | Spalding Sports Worldwide | Chicopee | 4,585,590 |
| Kodak Polychrome Graphics Llc | Holyoke | 1,824,001 | Alphagary | Leominster | 3,932,066 |
| Precision Lithograining Corp | S. Hadley | 1,577,226 | Avecia Inc | Peabody | 3,863,551 |

Table 16 lists the 20 facilities that had the largest quantity of TRI on-site releases and transfers off-site. The 20 facilities with the largest quantity of TRI on-site releases released 8 million pounds, or 71% of total releases statewide.  The top 5 facilities with on-site releases were power plants, accounting for almost 6 million pounds, or 52% of total on-site releases.  Four million pounds, or 76% of power plants’ on-site releases were due to the coincidental manufacture of hydrochloric acid during combustion.  The remainder of the power plants’ on-site releases was due to the coincidental manufacture of the following chemicals during combustion:  metal compounds (8%), hydrogen fluoride (6%), sulfuric acid (5%), and ammonia (5%).

The 20 facilities with the largest quantity of TRI transfers off-site transferred 27 million pounds, or 64% of the total transfers off-site statewide.

| Table 16 – 2000 Top 20 Facilities (Largest Quantity of TRI On-Site Releases and TRI Transfers Off-Site) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| TRI On-Site Releases These quantities include Trade Secret |  |  | TRI Transfers Off-Site These quantities include Trade Secret |  |  |
| Facility Name | Town | On-Site Releases (Lbs.) | Facility Name | Town | Transfers Off-Site (Lbs.) |
| USGEN New England Inc | Salem | 2,293,371 | Solutia Inc - Indian Orchard Plant | Springfield | 7,129,525 |
| USGEN New England Inc | Somerset | 1,466,186 | Polaroid Corporation | Waltham | 3,144,923 |
| Somerset Power LLC | Somerset | 907,227 | Chemdesign Corp | Fitchburg | 2,720,073 |
| Holyoke Water Power Mount Tom Station | Holyoke | 538,610 | Engineered Materials Solutions Inc | Attleboro | 2,269,445 |
| Mirant New England Inc | Sandwich | 440,126 | ISP Freetown Fine Chemicals Inc | Assonet | 2,175,371 |
| Crown Cork & Seal | Lawrence | 392,000 | Sanmina Corp | Wilmington | 1,098,104 |
| Rexam Image Products | South Hadley | 311,934 | Rexam Image Products | S. Hadley | 955,397 |
| Solutia Inc - Indian Orchard Plant | Springfield | 233,560 | Ideal Tape Company | Lowell | 865,039 |
| Ideal Tape Company | Lowell | 185,117 | Duncan Group The | Everett | 655,095 |
| Duro Finishing Corp | Fall River | 101,448 | HC Starck Inc | Newton | 626,607 |
| Alliance Leather Inc | Peabody | 96,744 | Clean Harbors Environmental Services Inc | Braintree | 617,528 |
| Hollingsworth & Vose Company | West Groton | 94,849 | Brittany Dyeing & Printing Corp | New Bedford | 590,609 |
| Trigen Boston Energy Corporation | Boston | 92,521 | Munters Corp | Amesbury | 554,214 |
| Proma Technologies Inc | Franklin | 86,676 | Waters Corp | Taunton | 528,077 |
| Flexcon Co Inc | Spencer | 86,536 | Borregaard Synthesis Inc | Newburyport | 523,549 |
| Lucent Technologies Inc | North Andover | 78,540 | Flexcon Co Inc | Spencer | 522,133 |
| Metalor Technologies USA | Attleboro | 78,428 | Shipley Co Inc | Marlborough | 508,990 |
| ANP-Operations Company - Milford Power | Milford | 77,686 | Allegheny Rodney Strip Division | New Bedford | 485,229 |
| MacDermid Graphic Arts | Adams | 76,550 | Genzyme Corp | Boston | 432,998 |
| Adden Furniture Inc | Lowell | 75,632 | Kodak Polychrome Graphics Llc | Holyoke | 396,053 |

## V.	2000 Top 5 Chemicals

This section highlights the 5 chemicals with the largest quantity used in Massachusetts under the TURA program, including information on each chemical’s use, exposure routes, health effects, and fate in the environment.  The five substances highlighted are: styrene, polycyclic aromatic compounds, sodium hydroxide, methanol, and hydrochloric acid.

### STYRENE

Styrene (also called vinyl benzene) is a flammable, oily liquid. It is clear to yellowish in color and has a strong odor.  Styrene is not found in nature.

Styrene is produced in very large amounts to make plastics, synthetic rubber, resins, and insulators.  Styrene plastics are used in business machines, luggage, construction materials, automotive and household goods, and packaging material.

In 2000, fifteen TURA facilities (3% of total facilities) reported styrene monomer:

- 348,875,499 pounds used  (25% of the total use reported, highest use of any chemical);

- 94,263 pounds generated as byproduct (less than 0.01% of the total generated as byproduct reported);

- 1,590,687 pounds shipped in product (less than 1% of the total shipped in product  reported);

- 40,770 pounds released on-site (less than 1% of total on-site releases); and

- 48,857 pounds transferred off-site (0. 1% of the total transfers off-site).

While styrene monomer is one of the top 5 chemicals with the largest quantity used in Massachusetts, only 18 facilities reported the chemical in 2000.  The largest SIC group is 28, Chemicals and Allied Products, which includes 7 facilities that reported styrene.  These facilities used styrene to make resin, gel coats, and polystyrene.  Styrene is also repackaged from large containers to smaller containers.

Even though styrene monomer was the highest quantity used TURA chemical in Massachusetts, a relatively small amount of byproduct was generated. This is largely due to the efficiency of the process by which styrene monomer is turned into plastic polymers.  For styrene monomer, byproduct as a percent of total use is less than 0.01%, reflecting efficiency in use of greater than 99.9%.

Exposure to styrene can occur in the workplace, as well as in the environment following releases to air, water, land, or groundwater.  Styrene is less likely to be absorbed through skin contact.  Styrene does not remain in the body for a long period of time.

Styrene evaporates when exposed to air, but dissolves only slightly when mixed with water.  Most releases of styrene to the environment are to air; once in air, styrene breaks down to other chemicals. Microorganisms that live in water and in soil also can break down styrene. Because it is a liquid that does not bind well to soil, styrene can move through the ground and enter groundwater.  Plants and animals are not likely to store styrene in their tissue.

Styrene vapor can irritate the eyes, the nose, and the throat.  Styrene vapor also can  adversely affect the human nervous system, causing adverse eye effects.  However, these effects are not likely to occur at levels of styrene that are normally found in the environment.  Human health effects associated with breathing small amounts of styrene over long periods of time in the workplace include alterations in vision, hearing loss, and decreased reaction times.  Other human health effects associated with these types of exposures are not known.  EPA is currently reviewing the potential for styrene to cause cancer in humans, and to affect reproduction.  Long-term animal exposures to high styrene levels have resulted in liver damage; however, this effect has not been seen in humans.

POLYCYCLIC AROMATIC COMPOUNDS

Polycyclic Aromatic Compounds (PACs) are a group of 21 different chemicals that are contained in fossil fuels and also formed during the incomplete burning of coal, oil, gas, garbage or other organic substances like tobacco or charbroiled meat.

PACs are classified as persistent, bioaccumulative, toxic chemicals (PBTs). PBTs are toxic, long-lasting substances that can build up in the food chain to levels that are harmful to human and ecosystem health. They are associated with a range of adverse human health effects, including effects on the nervous system, reproductive and developmental effects and cancer. PBTs have the ability to travel long distances, to transfer easily among air, water, and land, and to linger for generations in people and the environment.

In 2000, 140 TURA facilities (25% of total facilities) reported PACs to the TURA program:

- 109,492,315 pounds used (8% of total use reported, second in reported use);

- 7,000 pounds generated as byproduct (less than 0.01% of the total generated as byproduct reported);

- 31,036 pounds shipped in product (less than 0.01% of the total shipped in product reported);

- 59 pounds released on-site (less than 0.001% of the total on-site releases reported); and

- 6,676 pounds transferred off-site (less than 0.1% of total transfers off-site reported).

Fossil fuel combustion for heat and power generation is the primary source of PACs.  In Massachusetts, the vast majority of PACs are other wise used in the combustion process by facilities in the 49 SIC group, Electric, Gas and Sanitary Services.  Forty facilities in SIC 49 either burn fuel oils or coal to generate electricity.  Non-power facilities also use fuel containing PACs to produce power and steam in order to operate their facilities.

PACs can enter surface water through discharges from industrial plants and wastewater treatment plants, and they can be released to soils at hazardous waste sites if they escape from storage containers.  They are present in air as vapors or stuck to the surfaces of small solid particles.  Like most PBTs, they can travel long distances before they return to earth in rainfall or particle settling. PACs bioaccumulate in plants and animals at a much higher rate than when deposited in soil or water.

People may be exposed to PACs in soil near areas where coal, wood, gasoline, or other products have been burned.  PACs have been found in some drinking water supplies in the United States. For some people, the primary exposure to PACs occurs in the workplace.  Workers may be exposed to PACs by using products that contain PACs in a variety of industries such as mining, oil refining, metalworking, chemical production, transportation, and the electrical industry.  PACs have also been found in other facilities where petroleum, petroleum products, or coal are used or where wood; cellulose, corn or oil is burned.

PACs can be harmful to human health.  Studies show that individuals exposed by breathing or skin contact for long periods to mixtures and compounds that contain PACs can develop cancer.  Studies in animals have shown that PACs can cause harmful effects on skin, body fluids, and the body’s system for fighting disease after both short and long-term exposure.  These effects have not been reported in people.

SODIUM HYDROXIDE

Sodium hydroxide is a white, odorless solid that is used in water solutions in a wide range of industrial and chemical processes.  Water-based solutions of sodium hydroxide are known as soda lye.

Sodium hydroxide is used to neutralize acids, make sodium salts, rayon, plastics, paper and cellophane, in reclaiming rubber, and in the manufacture of laundering, bleaching, and dishwashing materials.

Exposure to sodium hydroxide can occur in the workplace or in the environment following releases to air, water, land, or groundwater.

In 2000, 218 facilities (39% of total facilities) reported sodium hydroxide:

- 93,456,213 pounds used  (7% of the total use reported, #3 in reported use);

- 15,036,923 pounds generated as byproduct (12% of the total byproduct reported, highest byproduct reported);

- 43,180,403 pounds shipped in product (10% of the total shipped in product reported, #2 in reported shipped in product);

- 46,006 pounds released on-site (less than 1% of the total on-site releases reported); and

- 1,300,182 pounds transferred off-site (3% of total transfers off-site reported, #10 in reported transfers off-site).

Sodium hydroxide is primarily used by facilities in SIC group 28, Chemicals and Allied Products.  The 46 facilities that reported sodium hydroxide in SIC 28 include chemical manufacturers, chemical repackagers, biotechnology firms, cocoa processors, fatty acid manufacturers, and food processors.

Small quantities of sodium hydroxide will slightly raise the pH of water in aquatic ecosystems, but larger quantities can raise the pH for extended periods of time. Sodium hydroxide is highly soluble in water and, when dissolving, generates considerable heat. Sodium hydroxide is very toxic to aquatic life at high levels; effects may include the death of animals, birds, or fish, and death or low growth rate in plants.

Breathing sodium hydroxide can irritate the mouth, nose, and throat.  Exposure to higher levels may irritate the lungs, causing coughing and/or shortness of breath.  Still higher exposure can cause a buildup of fluid in the lungs (pulmonary edema), causing death.  Sodium hydroxide is a corrosive solid or liquid and can cause severe burns of the eyes and skin on contact.  It has not been tested for its ability to cause cancer, nor has it been tested for its ability to adversely affect reproduction.

##### METHANOL

Methanol (also known as methyl alcohol and wood alcohol) is a colorless liquid with a strong odor that may explode when exposed to an open flame.  It occurs naturally in wood and in volcanic gases.  Methanol also is a product of decaying organic material.

Methanol is used in a variety of industrial applications. It is used in the production of formaldehyde, acetic acid, chloromethanes, methyl methacrylate, methylamines, dimethyl terephthalate. Companies use methanol as a solvent or antifreeze in the manufacturing of paint stripper, aerosol spray paints, wall paints, carburetor cleaners, and car windshield washer compounds.  It is used in the packaging and repackaging of chemicals, as a solvent in the quality testing of pharmaceutical products, and in the treatment of wastewater.  Methanol also is a gasoline additive, and in some cases, a gasoline substitute for use in automobiles and other small engines.  It is considered an ideal hydrogen carrier for fuel cell vehicles.

In 2000, 49 facilities (9% of total facilities) reported methanol:

- 56,860,236 pounds used  (4% of the total use reported, #4 in reported use);

- 6,574,377 pounds generated as byproduct (5% of the total byproduct, #5 in reported generated as byproduct);

- 49,138,652 pounds shipped in product (12% of the total shipped in product reported, #1 in reported shipped in product);

- 314,806 pounds released on-site (3% of the total on-site releases reported, #8 in reported on-site releases); and

- 2,910,684 pounds transferred off-site (7% of the total transfers off-site reported, #3 in reported transfers off-site).

The vast majority of methanol users are in SIC group 28, Chemicals and Allied Products.  These facilities make adhesives, various papers, resin, steam, paint and shampoo.

Exposure to methanol can occur in the workplace or in the environment following releases to air, water, land, or groundwater.  Methanol releases are primarily to the atmosphere. It contributes to the formation of photochemical smog when it reacts with other volatile organic carbon substances in air.  However, approximately 20% of methanol releases are direct discharges to the soil, groundwater or surface water.

In the home or workplace, exposure can occur when people use certain paint strippers, aerosol spray paints, wall paints, windshield wiper fluid, and small engine fuel. Methanol enters the body when breathed in with contaminated air or by passing through the skin. Breathing methanol can irritate the nose, mouth, and throat, causing coughing and wheezing. Repeated or prolonged exposure can cause dryness and cracking of the skin.  Workers repeatedly exposed to methanol have experienced effects ranging from headaches, sleep disorders, and gastrointestinal problems, to optic nerve damage.

The effects of drinking even small amounts of methanol range from headaches to a lack of coordination similar to that associated with drunkenness. Delayed effects such as severe abdominal, leg, and back pain can follow the inebriation effects of methanol.  People have died from drinking large amounts of methanol.  Methanol is classified as a Teratogen, a chemical that might interfere with the normal development of the fetus and result in the loss of a pregnancy, a birth defect, or a pregnancy complication.

Methanol use is expected to increase exponentially with the demand for fuel cell technology for transportation.  Given this expected increase in the production, transportation, storage, and use of methanol, the potential for accidental releases to the environment will increase. Because it is a liquid that does not bind well to soil, methanol that makes its way into the ground can enter groundwater.  However, methanol will dissolve and dilute to very low concentrations in the event of a surface water spill. Similarly, groundwater methanol concentrations will likely fall to low levels once complete dissolution has occurred.

### HYDROCHLORIC ACID

Hydrochloric acid is also known as hydrogen chloride. Hydrochloric acid occurs as a colorless, nonflammable liquid or gas with an irritating, pungent odor.

Hydrochloric acid is used in the production of chlorides, for refining ore in the production of tin and tantalum, for pickling and cleaning of metal products, in electroplating, in removing scale from boilers, for the neutralization of basic systems, as a laboratory reagent, as a catalyst and solvent in organic syntheses, in the manufacture of fertilizers and dyes, for hydrolyzing starch and proteins in the preparation of various food products, and in the textile and rubber industries.  Hydrochloric acid also is emitted during combustion, and during petroleum refining.  It is used to maintain pH balance in swimming pools, spas, etc.  It also is registered as an antimicrobial, a bactericide, and a fungicide. Hydrochloric acid is used as a general antimicrobial to disinfect bathrooms, kitchens and food preparation areas, and other areas in commercial and industrial buildings, hospitals, nursing homes, and in and around household dwellings.

In 2000, 73 facilities (13% of total facilities) reported hydrochloric acid:

- 50,737,340 pounds used  (4% of the total use reported, #5 in reported use);

- 7,024,367 pounds generated as byproduct (5% of the total byproduct reported, #6 in reported generated as byproduct);

- 4,319,037 pounds shipped in product (1% of the total shipped in product reported);

- 4,437,465 pounds released on-site (41% of the total on-site releases, #1 in reported on-site releases); and

- 118,591 pounds transferred off-site (less than 1% of the total transfers off-site reported).

The vast majority of hydrochloric acid reporters are in SIC group 28, Chemicals and Allied Products.  The 15 facilities that reported hydrochloric acid in SIC 28 make automotive pressure disks, ceramic parts, edible gelatin, fungicides, industrial chemical intermediates, metal parts that are plated, and pen caps.

Exposure to hydrochloric acid can occur in the workplace or in the environment following releases to air, water or groundwater.

Hydrochloric acid released into the atmosphere as a gas will be readily incorporated into clouds, rain, and fog.  Hydrochloric acid is also soluble in alcohol, benzene, methanol, ethanol, and ether.  It is incompatible with most metals.

Hydrochloric acid is corrosive to the eyes, skin, and mucous membranes.  Short-term inhalation exposure may cause coughing, hoarseness, inflammation and ulceration of the respiratory tract, chest pain, and pulmonary edema.  Short-term oral exposure may cause corrosion of the mucous membranes, esophagus, and stomach, with nausea, vomiting, and diarrhea.  Skin contact may produce severe burns, ulceration, and scarring.

Long-term occupational exposure to hydrochloric acid has been reported to cause gastritis, chronic bronchitis, dermatitis, and photosensitization in workers.  Prolonged exposure to low concentrations may also cause dental discoloration and erosion. Hydrochloric acid has not been classified with respect to potential carcinogenicity, nor has it been tested for its ability to adversely affect reproduction.

Page 35

This section contains the definitions of key TURA terms.  Additional information regarding TURA and TRI, as well as general chemical information, can be obtained from the Internet web sites noted on the next page.

TURA – Massachusetts Toxics Use Reduction Act of 1989 (MGL 21I)

TRI – federal EPA Toxics Release Inventory

TRADE SECRET – the information identified as confidential by TURA filers.  To protect confidentiality claims by Trade Secret filers, all trade secret data in this information release is presented in aggregated form.  Aggregated data does not include the names and amounts of chemicals subject to claims of confidentiality.

The terms and definitions below have been arranged in order of inputs and outputs.  Chemicals that are used by companies are brought into the facility and are manufactured, processed or otherwise used.  As a result of using these chemicals, a company has outputs that can include a product that is created for sale, or a byproduct or waste. The calculation of use and waste of chemicals is known as ‘Mass Balance’. Generally the inputs equal the outputs, but there are circumstances where a chemical is used in ways that result in an imbalance between inputs and outputs.  These circumstances are most often the result of: 1) chemicals are recycled on-site, 2) the product was held in inventory, 3) chemical is consumed or transformed, or 4) the chemical is a compound.

## VI. Key TURA Terms

### INPUTS

### OUTPUTS

##### Mass Balance

PRODUCT – a product, a family of products, an intermediate

product, family of intermediate products, or a desired result or a

family of results.  “Product” also means a byproduct that is used

as a raw material without treatment.

SHIPPED IN PRODUCT – the quantity in pounds of the chemical that leaves the facility as product.

BYPRODUCT – all non-product outputs of reportable substances generated by a production unit prior to handling, treatment, and release.

TOTAL USE – the total quantity in pounds of TURA chemicals reported d as manufactured, processed and otherwise used.

MANUFACTURE – to produce, prepare, import or compound a toxic or hazardous substance.

OTHERWISE USE – any use of a toxic substance that is not covered by the terms “manufacture” or “process” and includes use of a toxic substance contained in a mixture or trade name product.

PROCESS – the preparation of a toxic or hazardous substance, including without limitation, a toxic substance contained in a mixture or trade name product, after its manufacture, for distribution in commerce:  a) in the same form or physical state, or in a different form or physical state, from that in which it was received by the toxics user so preparing such substance; or b) as part of an article containing the toxic or hazardous substance.

ON-SITE RELEASES– all byproducts that are released to the air, discharged to surface waters, released to land and underground injection wells.

TRANSFERS OFF-SITE – byproducts that are transferred off-site for energy recovery, recycling, treatment and disposal.

The Following Web Sites Contain Information Regarding Chemicals, TURA and Pollution Prevention:

Massachusetts Department of Environmental Protection, Toxic Use Reduction Program

http://www.mass.gov/dep/bwp/dhm/tura

Toxics Use Reduction Institute (TURI)

http://www.turi.org

http://www.turi.org/turadata/index.html

Office of Technical Assistance for Toxics Use Reduction (OTA)

http://www.mass.gov/ota

Agency for Toxic Substances and Disease Registry

http://www.atsdr.cdc.gov/toxprofiles/

CambridgeSoft Chemfinder gives basic physico-chemical data and molecular structures for chemicals, and provides links to international data sources addressing specific chemicals.
http://chemfinder.camsoft.com/

Envirofacts, US EPA

http://www.epa.gov/enviro/html/emci/chemref/index.html

Environmental Defense Fund (EDF) Scorecard

http://www.scorecard.org/chemical-profiles/

Global Information Network on Chemicals (GINC)
http://www.nihs.go.jp/GINC/

Integrated Risk Management Information System (IRIS) is a database of human health effects that may result from exposure to various substances found in the environment US EPA

http://www.epa.gov/iris/

Top of Form 1

Bottom of Form 1

Top of Form 2

Top of Form 3

University of Akron Hazardous Chemicals Database provides physical data on chemicals and links to Department of Transportation safety guides that are valuable for emergency response
http://ull.chemistry.uakron.edu/erd/

Massachusetts Department of

Environmental Protection

One Winter Street

Boston, MA 02108-4746

Commonwealth of Massachusetts

Jane Swift, Governor

Executive Office of Environmental Affairs

Bob Durand, Secretary

Department of Environmental Protection

Lauren A. Liss, Commissioner

Data shown for years prior to 2000 have been adjusted to reflect changes in the Core Group of TURA facilities, and therefore may differ slightly from the same information reported in the 1999 Toxics Use Reduction Information Release.

A separate Form S is required for each chemical reported by a facility; the Form S is the form used to report chemical information.

Trends are measured from 1991 due to a change in the definition of Transfers Off-Site that year.

Trends are measured from 1991 due to a change in the definition of transfers off-site that year.

A ratio of reporting production by comparing current year to prior year production.

The majority of reporting facilities included information identifying fuel combustion as the reason for reporting PACs; however, some facilities lacked this information but are assumed to be reporting due to fuel combustion.  The fuel combustion facilities have been sub-categorized into power plants and other facilities.  The numbers shown below in parenthesis reflect this sub-categorization and should not be added to the totals.