WORKER PROTECTION STANDARD Respirator Checklist

Date:

Instructor:

Facility Name:

Massachusetts Department of Agricultural Resources

Pesticide Division

251 Causeway Street

Boston, MA 02114

| Employee Name: |  |
| --- | --- |
| REQUIREMENTS 170.507(B)(10)(iii) |  |
| Respirator specified by product labeling? | Y N |
| Fit test using respirator specified on the pesticide label in manner that conforms with 29 CFR 1910.134? | Y N |
| Training Handler in use of respirator specified on the pesticide label in manner that conforms with 29 CFR 110.134(k)(1)(i) | Y N |
| Medical evaluation by licensed physician/licensed health care professional that conforms with 29 CFR 1910.34? | Y N |
| FIT TEST PROCEDURES 29 CFR 1910.134 Appendix A |  |
| Allow employee to pick acceptable respirator that fits user correctly | Y N |
| Employee shall be shown proper wear of respirator: | Y N |
| Positioning on face | Y N |
| How to set strap tension | Y N |
| Mirror available to assist employee in proper fitting of respirator | Y N |
| Employee will be informed that he/she is being asked to select suitable respirator | Y N |
| Employee shall be instructed to hold each chosen face piece up to the face to eliminate those with an unacceptable fit | Y N |
| Most acceptable face piece is donned and worn for at least five (5) minutes to assess comfort | Y N |
| Employee will don mask a few times to become familiar with setting proper tension on the straps | Y N |
| Assessment of comfort: | Y N |
| Position of mask on the nose | Y N |
| Room for Eye Protection | Y N |
| Room to talk | Y N |
| Position of mask on face and cheeks | Y N |
| Criteria to determine adequacy of respirator fit: | Y N |
| Chin properly place | Y N |
| Adequate strap tension, not overly tightened | Y N |
| Fit across nose bridge | Y N |
| Respirator of proper size to span distance from nose to chin | Y N |
| Tendency of respirator to slip | Y N |
| Self-observance in mirror to evaluate fit and respirator position | Y N |
| Employee conduct user seal check | Y N |
| If employee has difficulty breathing during test refer to a physician to see if employee can wear respirator | Y N |
| Employee shall be given description of fit test prior to fit test | Y N |
| Employee shall applicable safety equipment that may be during actual respirator use when doing the fit test | Y N |
| Employee must perform test exercises: | Y N |
| Normal breathing (in a normal standing position, without talking) | Y N |
| Deep breathing (in a normal standing position breathing slowly and deeply being cautious not to hyperventilate | Y N |
| Turning head side to side (standing in place, slowly moving head up and down inhaling) | Y N |
| Talking (talk out loud, slowly and loud enough for instructor to hear) | Y N |
| Grimace (smile or frown) | Y N |
| Bending over (bend at waist as if touching toes) | Y N |
| Normal breathing ***Each test shall be performed for one (1) minute except for grimace exercise which shall be performed for fifteen (15) seconds | Y N |
| FIT TEST RECORD REQUIREMENTS 40 CFR 170.507(b)(10) & 29 CFR 19.10.134(m)(2) |  |
| Name of handler tested |  |
| Type of fit test performed |  |
| Make, model and size of respirator tested |  |
| Date of fit test |  |
| Results of fit test |  |
| Records kept for 2 years Y N |  |
| TRAINING MATERIALS |  |
| www.osha-safety-training.net Compliance |  |
| www.osha.gov 29 CFR 1910.134 (fit test procedures) |  |
| www.epa.gov Worker Protection Standards (WPS) |  |
| NOTES |  |